| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0206-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0182 (EE1-1.5: multiscale blocks in LOP3 and VLOP2 filters) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | T. Dumas | Email: | thierry.dumas@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

The contribution reports the results of the training cross-check of EE1-1.5, sub-tests 1.5.1, 1.5.2, 1.5.3, and 1.5.4. EE1-1.5 proposes to introduce a multiscale component in the backbone of the LOP3/VLOP2 loop-filter. Using this multiscale component, the stack of feature maps at the input to a residual block is processed at two different scales. In this training cross-check, for each of the four sub-tests 1.5.1, 1.5.2, 1.5.3, and 1.5.4, the multiscale block features a number of channels being multiple of 16.

The following results are reported.

- Sub-test EE1-1.5.1

  - ANCHOR = NNVC-10.0 with LOP3 Variant 1 in 16-bit signed integer from the proponents and the NN-based intra prediction.

  - TEST = NNVC-10.0 with LOP3 Variant 1 in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

    - AI  : 0.02%   -1.17%   -1.27%

    - RA : xxx%   xxx%     xxx%

- Sub-test EE1-1.5.2

  - ANCHOR = NNVC-10.0 with LOP3 Variant 2 in 16-bit signed integer from the proponents and the NN-based intra prediction.

  - TEST = NNVC-10.0 with LOP3 Variant 2 in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

    - AI   : 0.03%   0.66%    0.82%

    - RA : xxx%     xxx%     xxx%

- Sub-test EE1-1.5.3

  - ANCHOR = NNVC-10.0 with VLOP2 Variant 1 in 16-bit signed integer from the proponents and the NN-based intra prediction.

  - TEST = NNVC-10.0 with VLOP2 Variant 1 in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

    - AI   : 0.05%   -0.12%   -0.26%

    - RA : xxx%    xxx%      xxx%

- Sub-test EE1-1.5.4

  - ANCHOR = NNVC-10.0 with VLOP2 Variant 2 in 16-bit signed integer from the proponents and the NN-based intra prediction.

  - TEST = NNVC-10.0 with VLOP2 Variant 2 in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

    - AI   : -0.02%   0.27%   1.08%

    - RA : xxx%      xxx%    xxx%

The results from the training cross-check match those from the proponents, despite small differences in the luma/chroma balancing.

# Introduction

EE1-1.5 [1] modifies the architecture of the residual blocks of the backbone in LOP3/VLOP2 to support multiscale processing at two different scales. The differences between the original backbone block, Variant 1 of the multiscale backbone block, and Variant 2 of the multiscale backbone block are shown in Figure 1, Figure 2, and Figure 3.

Figure 1: original backbone block.

Figure 2: Variant 1 of multiscale backbone block.

Figure 3: Variant 2 of multiscale backbone block.

# Experiments

The source of EE1-1.5 picked from the EE1 branch “EE1-1.5” is used to carry out the training cross-check of EE1-1.5. The training cross-check follows the AHG11 training CTC [2]. For both LOP3 and VLOP2, the training cross-check starts from Stage 3.

## Sub-test EE1-1.5.1

### Cross-checked model in floats

ANCHOR = NNVC-10.0 with LOP3 Variant 1 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with LOP3 Variant 1 (channels multiple of 16) in floats from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.1 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.06% | -1.18% | 0.12% | -0.01% | -1.06% | -0.62% | 134% | 356% | 100% |
| Class A2 | -0.04% | -1.26% | -0.93% | 0.10% | -1.37% | -0.98% | 127% | 340% | 100% |
| Class B | 0.01% | -1.32% | -1.45% | 0.06% | -1.64% | -1.86% | 126% | 332% | 100% |
| Class C | -0.10% | -0.76% | -1.08% | -0.04% | -0.74% | -1.29% | 120% | 303% | 100% |
| Class E | 0.00% | -1.28% | -1.24% | 0.12% | -1.64% | -1.54% | 224% | 697% | 100% |
| Overall | -0.02% | -1.16% | -0.99% | 0.04% | -1.30% | -1.33% | 139% | 374% | 100% |
| Class D | -0.17% | -0.60% | -1.37% | -0.05% | -0.89% | -1.95% | 119% | 300% | 100% |
| Class F | -0.14% | -0.74% | -1.99% | -0.16% | -0.44% | -2.01% | 129% | 348% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.1 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.06% | -0.62% | -0.82% | 0.12% | -0.62% | -0.86% | 133% |  | 100% |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.92% | -0.65% | -0.01% | -1.02% | -0.87% | 129% | 396% | 100% |
| Class C | -0.05% | -0.49% | -1.08% | -0.10% | 0.84% | -0.48% | 130% | 387% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0.06% | -0.52% | -0.81% | 0.01% | 0.34% | -0.08% | 123% | 366% | 100% |
| Class F | -0.01% | -0.61% | -1.54% | 0.10% | 0.14% | -2.09% | 142% | 399% | 100% |

### Cross-checked models after the quantization

ANCHOR = NNVC-10.0 with LOP3 Variant 1 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with LOP3 Variant 1 (channels multiple of 16) in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.1 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |  |
| Class A1 | -0.07% | -1.50% | -1.79% | 0.15% | -1.61% | -1.68% | 125% | 123% | 100% |  |
| Class A2 | 0.07% | -1.24% | -0.86% | 0.17% | -1.39% | -0.98% | 121% | 124% | 100% |  |
| Class B | 0.03% | -1.26% | -1.35% | 0.07% | -1.60% | -1.83% | 122% | 127% | 100% |  |
| Class C | -0.01% | -0.77% | -1.17% | -0.02% | -0.70% | -1.32% | 118% | 128% | 100% |  |
| Class E | 0.10% | -1.14% | -1.18% | 0.20% | -1.57% | -1.63% | 215% | 263% | 100% |  |
| Overall | 0.02% | -1.17% | -1.27% | 0.10% | -1.36% | -1.52% | 133% | 142% | 100% |  |
| Class D | -0.08% | -0.51% | -1.39% | -0.03% | -0.85% | -2.02% | 118% | 128% | 100% |  |
| Class F | -0.05% | -0.63% | -1.99% | -0.09% | -0.33% | -2.06% | 127% | 132% | 100% |  |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.1 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0.06% | -0.43% | -0.86% | 0.11% | -0.52% | -0.88% | 119% |  | 100% |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.03% | -0.76% | -0.49% | -0.04% | -1.04% | -0.81% | 116% | 125% | 100% |
| Class C | 0.07% | -0.40% | -0.64% | -0.01% | 0.70% | -0.18% | 121% | 131% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0.10% | 0.12% | -0.66% | 0.10% | 0.59% | -0.07% | 115% | 131% | 100% |
| Class F | 0.06% | -0.50% | -1.47% | 0.20% | 0.36% | -2.10% | 122% | 135% | 100% |

## Sub-test EE1-1.5.2

### Cross-checked models in floats

ANCHOR = NNVC-10.0 with LOP3 Variant 2 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with LOP3 Variant 2 (channels multiple of 16) in floats from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.2 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.04% | 1.32% | 1.45% | -0.12% | 1.90% | 0.60% | 222% | 359% | 100% |
| Class A2 | -0.07% | 0.37% | 0.19% | 0.11% | 0.20% | 0.47% | 219% | 357% | 100% |
| Class B | -0.04% | 0.48% | 1.02% | -0.01% | 0.89% | 1.78% | 217% | 357% | 100% |
| Class C | -0.08% | 0.54% | 0.85% | -0.02% | 0.96% | 1.33% | 213% | 354% | 100% |
| Class E | -0.06% | 0.68% | 0.78% | 0.05% | 0.72% | 1.37% | 218% | 355% | 100% |
| Overall | -0.04% | 0.65% | 0.88% | 0.00% | 0.93% | 1.20% | 218% | 356% | 100% |
| Class D | -0.13% | 0.61% | 0.54% | -0.10% | 0.51% | 0.96% | 211% | 359% | 100% |
| Class F | -0.03% | -0.13% | -0.28% | 0.02% | -0.19% | -0.46% | 236% | 358% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.2 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.14% | 0.48% | 0.72% | -0.12% | 0.93% | 1.17% | 222% | 375% | 100% |
| Class C | -0.14% | 0.62% | 0.85% | -0.09% | 0.89% | 0.78% | 219% | 372% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0.09% | 1.33% | 0.93% | 0.07% | 0.45% | 1.44% | 213% | 362% | 100% |
| Class F | -0.01% | 0.45% | 0.02% | 0.21% | 0.04% | -0.16% | 233% | 353% | 100% |

### Cross-checked models after the quantization

ANCHOR = NNVC-10.0 with LOP3 Variant 2 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with LOP3 Variant 2 (channels multiple of 16) in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.2 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.06% | 1.35% | 1.34% | -0.06% | 1.85% | 0.51% | 215% | 249% | 100% |
| Class A2 | 0.03% | 0.47% | 0.22% | 0.17% | 0.23% | 0.49% | 216% | 252% | 100% |
| Class B | 0.02% | 0.50% | 0.99% | 0.04% | 0.84% | 1.70% | 214% | 254% | 100% |
| Class C | 0.00% | 0.50% | 0.76% | 0.03% | 0.94% | 1.30% | 211% | 258% | 100% |
| Class E | 0.02% | 0.67% | 0.72% | 0.12% | 0.69% | 1.26% | 214% | 255% | 100% |
| Overall | 0.03% | 0.66% | 0.82% | 0.06% | 0.90% | 1.14% | 214% | 254% | 100% |
| Class D | -0.02% | 0.46% | 0.47% | -0.05% | 0.44% | 0.91% | 210% | 262% | 100% |
| Class F | 0.06% | -0.11% | -0.20% | 0.12% | -0.13% | -0.34% | 235% | 250% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.2 with LOP3 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.12% | 0.51% | 0.86% | -0.10% | 0.90% | 1.33% | 215% | 247% | 100% |
| Class C | -0.02% | 0.55% | 1.01% | -0.09% | 0.96% | 0.87% | 214% | 247% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0.11% | 1.05% | 0.82% | 0.18% | 0.32% | 1.17% | 209% | 244% | 100% |
| Class F | 0.02% | 0.43% | 0.04% | 0.17% | -0.09% | -0.42% | 221% | 228% | 100% |

## Sub-test EE1-1.5.3

### Cross-checked models in floats

ANCHOR = NNVC-10.0 with VLOP2 Variant 1 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with VLOP2 Variant 1 (channels multiple of 16) in floats from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.3 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.08% | 1.16% | 1.75% | -0.17% | 2.00% | 1.14% | 209% | 501% | 100% |
| Class A2 | -0.02% | -0.24% | -0.44% | -0.09% | 0.08% | -0.31% | 209% | 479% | 100% |
| Class B | 0.00% | -0.27% | -0.32% | 0.01% | -0.21% | -0.32% | 206% | 460% | 100% |
| Class C | 0.00% | -0.54% | -0.43% | 0.03% | -0.40% | -0.50% | 201% | 406% | 100% |
| Class E | 0.06% | -0.49% | -0.36% | -0.03% | -0.22% | -0.06% | 207% | 479% | 100% |
| Overall | 0.02% | -0.12% | -0.03% | -0.04% | 0.16% | -0.07% | 206% | 460% | 100% |
| Class D | -0.06% | -0.29% | -0.46% | -0.02% | 0.30% | -0.50% | 211% | 478% | 100% |
| Class F | 0.02% | -0.19% | 0.32% | 0.14% | -0.84% | 0.61% | 222% | 468% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.3 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0.06% | 1.06% | 0.19% | -0.26% | 1.67% | 0.08% | 131% | 312% | 100% |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.35% | -0.51% | 0.02% | -0.09% | -0.54% | 125% | 299% | 100% |
| Class C | -0.14% | -0.79% | -0.96% | -0.02% | -0.29% | -0.77% | 123% | 308% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0.31% | -0.51% | -1.07% | -0.06% | 0.48% | -1.29% | 119% | 297% | 100% |
| Class F | -0.06% | -0.17% | -0.16% | 0.01% | -0.25% | 0.14% | 128% | 260% | 100% |

### Cross-checked models after the quantization

ANCHOR = NNVC-10.0 with VLOP2 Variant 1 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with VLOP2 Variant 1 (channels multiple of 16) in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.3 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.09% | 0.94% | 0.33% | -0.11% | 1.88% | 0.35% | 215% | 257% | 100% |
| Class A2 | 0.01% | -0.14% | -0.41% | -0.09% | 0.14% | -0.27% | 215% | 261% | 100% |
| Class B | 0.01% | -0.21% | -0.31% | 0.02% | -0.16% | -0.33% | 214% | 263% | 100% |
| Class C | 0.09% | -0.50% | -0.45% | 0.05% | -0.32% | -0.49% | 212% | 263% | 100% |
| Class E | 0.08% | -0.50% | -0.34% | -0.04% | -0.26% | -0.09% | 214% | 263% | 100% |
| Overall | 0.05% | -0.12% | -0.26% | -0.02% | 0.18% | -0.20% | 214% | 262% | 100% |
| Class D | 0.10% | -0.29% | -0.43% | 0.03% | 0.34% | -0.42% | 209% | 267% | 100% |
| Class F | 0.23% | -0.13% | 0.34% | 0.35% | -0.76% | 0.61% | 235% | 255% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.3 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0.01% | 1.04% | 0.10% | -0.16% | 1.77% | 0.01% | 126% |  | 100% |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.35% | -0.52% | 0.06% | -0.18% | -0.50% | 121% | 126% | 100% |
| Class C | -0.06% | -0.81% | -0.87% | -0.03% | -0.42% | -0.76% | 120% | 128% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | -0.61% | -0.85% | -0.01% | 0.47% | -1.06% | 116% | 126% | 100% |
| Class F | 0.12% | -0.21% | -0.14% | 0.19% | -0.28% | 0.15% | 122% | 111% | 100% |

## Sub-test 1.5.4

### Cross-checked models in floats

ANCHOR = NNVC-10.0 with VLOP2 Variant 2 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with VLOP2 Variant 2 (channels multiple of 16) in floats from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.4 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | 0.02% | 1.10% | 1.29% | 0.23% | 1.75% | 1.82% | 213% | 274% | 100% |
| Class A2 | -0.07% | 0.13% | 0.79% | -0.14% | 0.77% | 1.23% | 213% | 278% | 100% |
| Class B | -0.01% | 0.17% | 1.10% | -0.01% | 0.42% | 1.97% | 211% | 276% | 100% |
| Class C | 0.00% | -0.06% | 0.75% | 0.00% | 0.50% | 1.43% | 208% | 279% | 100% |
| Class E | -0.05% | 0.17% | 1.56% | -0.01% | 0.55% | 2.20% | 210% | 278% | 100% |
| Overall | -0.02% | 0.27% | 1.08% | 0.01% | 0.74% | 1.74% | 211% | 277% | 100% |
| Class D | -0.03% | 0.07% | 0.59% | -0.03% | 0.64% | 1.24% | 205% | 285% | 100% |
| Class F | -0.05% | 0.64% | 1.11% | -0.10% | 1.60% | 2.32% | 224% | 279% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.4 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.12% | 0.33% | 1.21% | 0.10% | 0.91% | 2.22% | 118% | 151% | 100% |
| Class C | 0.03% | -0.08% | 0.62% | 0.02% | 1.13% | 2.08% | 113% | 138% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0.07% | -0.22% | 0.34% | 0.24% | 1.09% | 1.74% | 115% | 139% | 100% |
| Class F | -0.09% | 0.50% | 0.63% | 0.11% | 2.01% | 2.02% | 118% | 131% | 100% |

### Cross-checked models after the quantization

ANCHOR = NNVC-10.0 with VLOP2 Variant 2 (channels multiple of 16) in 16-bit signed integer from the proponents and the NN-based intra prediction. The figures are extracted from [1].

TEST = NNVC-10.0 with VLOP2 Variant 2 (channels multiple of 16) in 16-bit signed integer from the training cross-check and the NN-based intra prediction.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.4 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -0.07% | 1.16% | 1.32% | 0.22% | 1.75% | 1.83% | 209% | 237% | 100% |
| Class A2 | -0.04% | 0.13% | 0.82% | -0.13% | 0.72% | 1.21% | 210% | 244% | 100% |
| Class B | 0.00% | 0.18% | 1.08% | 0.00% | 0.40% | 1.92% | 208% | 244% | 100% |
| Class C | 0.02% | -0.08% | 0.75% | 0.00% | 0.45% | 1.41% | 206% | 248% | 100% |
| Class E | -0.05% | 0.17% | 1.55% | -0.01% | 0.52% | 2.14% | 208% | 245% | 100% |
| Overall | -0.02% | 0.27% | 1.08% | 0.01% | 0.71% | 1.71% | 208% | 244% | 100% |
| Class D | -0.01% | 0.08% | 0.57% | -0.01% | 0.63% | 1.21% | 204% | 249% | 100% |
| Class F | -0.04% | 0.69% | 1.15% | -0.09% | 1.62% | 2.33% | 221% | 243% | 100% |

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Sub-test EE1-1.5.4 with VLOP2 (proponents, 16-bit signed integer) and the NN-based intra prediction |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | 0.38% | 1.13% | 0.02% | 0.88% | 2.11% | 118% | 128% | 100% |
| Class C | 0.02% | -0.06% | 0.68% | -0.04% | 1.21% | 2.17% | 113% | 119% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | 0.08% | -0.29% | 0.24% | 0.22% | 0.72% | 1.65% | 114% | 118% | 100% |
| Class F | -0.05% | 0.42% | 0.57% | 0.13% | 1.74% | 1.88% | 116% | 113% | 100% |

# Conclusion

The results from the training cross-check match those from the proponents, despite small differences in the luma/chroma balancing. Suck kinds of variations in the luma/chroma balancing have already been observed when testing re-trained LOP3 and re-trained VLOP2.

# Reference

- R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, M. Hannuksela. “EE1-1.1.5: multiscale blocks in LOP3 and VLOP2 filters”. JVET-AJ0182. 36th JVET meeting, Kemer, TR, 1-8 November 2024.

- JVET-AF2016, “JVET common test conditions and evaluation procedures for neural network-based video coding technology”, E. Alshina, R. L. Liao, S. Liu, A. Segall, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Interdigital does not have any current or pending patent rights relating to the technology described in this contribution.