| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0211-v1 |
| --- | --- |

| Title: | EE2-6.1-6.2: Entropy coding extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Franck Galpin Fabrice Le Leannec Federico Lo Bianco Charles Salmon-Legagneur Muhammet Balcilar Pavel Nikitin Muhammed Coban Marta Karczewicz Vadim Seregin | Email: | franck.galpin@interdigital.com pnikitin@qti.qualcomm.com |
| Source: | Interdigital, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the results of EE2-6.1 and EE2-6.2 tests, where the new type of bin coding was introduced.

There are seven tests related to the proposal. In the first the Cabac retraining for ECM 14.0 is done. In the second test EP bins are allowed to be used for context coding bins. In the third test all coding bins can be use either CABAC or EP bins. In the fourth test CABAC/NEP/EP is allowed for all elements. In the fifth and sixth tests the number of bins using CABAC/NEP is kept the same as ECM14.0, allowing CABAC/NEP/EP for all the bins. In Test 6.2b the complexity of NEP is considered 0.5 of complexity of CABAC, and in test 6.2c the complexity of NEP is considered equal to the complexity of CABAC. In test 6.2d only ECM14.0 bins are allowed to use NEP/CABAC.

Test 6.1a:

AI: -0.01% 0.06% 0.06% EncT: 10X.X% DecT: 10X.X%

RA: -0.02% 0.07% 0.08% EncT: 10X.X% DecT: 10X.X%

LB: 0.09% 0.31% -0.78% EncT: 10X.X% DecT: 10X.X%

Test 6.1b:

AI: XX% XX% XX% EncT: 10X.X% DecT: 10X.X%

RA: XX% XX% XX% EncT: 10X.X% DecT: 10X.X%

LB: XX% XX% XX% EncT: 10X.X% DecT: 10X.X%

Test 6.1c:

AI: -0.06% -0.17% -0.11% EncT: 10X.X% DecT: 10X.X%

RA: -0.16% -0.12% -0.14% EncT: 10X.X% DecT: 10X.X%

LB: -0.08% 0.33% -0.45% EncT: 10X.X% DecT: 10X.X%

Test 6.2a:

AI: -0.06% -0.14% -0.10% EncT: 10X.X% DecT: 10X.X%

RA: -0.17% -0.21% -0.15% EncT: 10X.X% DecT: 10X.X%

LB: -0.07% -0.03% -0.43% EncT: 10X.X% DecT: 10X.X%

Test 6.2b:

AI: -0.06% -0.11% -0.09% EncT: 10X.X% DecT: 10X.X%

RA: -0.16% -0.017% -0.11% EncT: 10X.X% DecT: 10X.X%

LB: -0.09% 0.11% -0.85% EncT: 10X.X% DecT: 10X.X%

Test 6.2c:

AI: -0.06% -0.15% -0.15% EncT: 10X.X% DecT: 10X.X%

RA: -0.17% -0.28% -0.09% EncT: 10X.X% DecT: 10X.X%

LB: -0.03% 0.25% -0.67% EncT: 10X.X% DecT: 10X.X%

Test 6.2d:

AI: -0.01% 0.05% 0.06% EncT: 10X.X% DecT: 10X.X%

RA: -0.02% 0.03% 0.06% EncT: 10X.X% DecT: 10X.X%

LB: 0.07% 0.11% -0.80% EncT: 10X.X% DecT: 10X.X%

# Introduction

# Results

In these tests the training is performed using all CTC sequences including class TGM encoder in AI, RA, LB and LP configurations.

  - Test 6.1a: Context initialization retraining

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.02% | 0.01% | 110.3% | 109.9% | 98.9% | 101.7% |
| Class A2 | 0.00% | 0.11% | -0.02% | 110.9% | 111.7% | 100.4% | 102.5% |
| Class B | -0.01% | 0.01% | 0.14% | 106.9% | 109.3% | 102.7% | 100.2% |
| Class C | 0.00% | -0.04% | -0.03% | 109.5% | 110.2% | 100.8% | 100.1% |
| Class E | -0.04% | 0.29% | 0.19% | 110.1% | 108.8% | 102.0% | 100.2% |
| Overall | -0.01% | 0.06% | 0.06% | 109.2% | 109.9% | 101.1% | 100.8% |
| Class D | -0.02% | 0.09% | -0.08% | 106.9% | 108.0% | 99.9% | 100.0% |
| Class F | -0.08% | -0.18% | -0.13% | 108.7% | 109.7% | 99.9% | 100.2% |
| Class TGM | -0.01% | 0.00% | -0.02% | 108.8% | 109.1% | 100.6% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.03% | -0.02% | 81.9% | 81.3% | 99.9% | 99.8% |
| Class A2 | -0.01% | 0.16% | -0.02% | 77.6% | 76.3% | 100.3% | 100.1% |
| Class B | -0.02% | 0.00% | 0.15% | 101.6% | 98.8% | 99.9% | 100.2% |
| Class C | -0.04% | 0.18% | 0.14% | 101.5% | 99.3% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.07% | 0.08% | 92.2% | 90.4% | 100.0% | 100.1% |
| Class D | 0.00% | -0.25% | -0.02% | 100.9% | 98.9% | 100.1% | 100.0% |
| Class F | -0.06% | -0.25% | -0.06% | 101.1% | 98.8% | 99.9% | 100.0% |
| Class TGM | 0.03% | 0.04% | 0.08% | 101.5% | 99.7% | 100.3% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.08% | -0.63% | 96.3% | 97.5% | 100.0% | 100.3% |
| Class C | 0.09% | 0.43% | -0.25% | 111.4% | 112.3% | 100.0% | 100.0% |
| Class E | 0.19% | 0.54% | -1.73% | 107.9% | 108.4% | 101.0% | 100.1% |
| Overall | 0.09% | 0.31% | -0.78% | 104.0% | 105.0% | 100.3% | 100.1% |
| Class D | 0.02% | 0.73% | 0.27% | 108.2% | 105.6% | 100.0% | 100.0% |
| Class F | 0.07% | 0.29% | 0.26% | 101.0% | 100.9% | 99.7% | 100.1% |
| Class TGM | -0.21% | -0.12% | -0.27% | 95.9% | 91.1% | 99.8% | 99.9% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.04% | 0.15% | 100.2% | 97.5% | 100.1% | 99.9% |
| Class C | 0.05% | -0.03% | -0.85% | 103.6% | 100.0% | 100.0% | 100.0% |
| Class E | -0.19% | -1.16% | 0.22% | 104.8% | 103.0% | 96.1% | 99.8% |
| Overall | -0.06% | -0.32% | -0.17% | 102.4% | 99.7% | 99.0% | 99.9% |
| Class D | 0.05% | -0.68% | -0.33% | 104.7% | 101.2% | 100.7% | 100.0% |
| Class F | 0.40% | -0.65% | -0.17% | 96.6% | 95.7% | 100.2% | 99.9% |
| Class TGM | 0.10% | 0.04% | -0.03% | 90.2% | 89.6% | 100.0% | 99.6% |

  - Test 6.1b: Context bins coding is redistributed between context and EP coding

  - Test 6.1c: Bins coding is redistributed between context and EP coding

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.19% | -0.13% | 109.9% | 111.1% | 99.0% | 101.6% |
| Class A2 | -0.09% | -0.23% | -0.23% | 110.1% | 111.5% | 100.4% | 102.9% |
| Class B | -0.05% | -0.30% | -0.23% | 107.5% | 109.7% | 102.4% | 100.0% |
| Class C | -0.03% | -0.03% | -0.08% | 109.6% | 110.7% | 100.9% | 100.1% |
| Class E | -0.07% | -0.04% | 0.17% | 110.1% | 109.2% | 101.9% | 100.1% |
| Overall | -0.06% | -0.17% | -0.11% | 109.2% | 110.4% | 101.0% | 100.8% |
| Class D | -0.01% | 0.19% | -0.04% | 106.3% | 106.9% | 99.9% | 100.0% |
| Class F | -0.12% | -0.13% | -0.24% | 109.8% | 109.5% | 99.9% | 100.0% |
| Class TGM | -0.33% | -0.39% | -0.38% | 108.7% | 108.7% | 101.9% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14% | -0.13% | 0.00% | 81.5% | 81.8% | 100.0% | 100.0% |
| Class A2 | -0.07% | 0.06% | -0.18% | 77.1% | 75.6% | 100.0% | 99.9% |
| Class B | -0.27% | -0.09% | -0.27% | 101.8% | 98.4% | 100.0% | 100.2% |
| Class C | -0.09% | -0.29% | -0.04% | 101.3% | 98.7% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.16% | -0.12% | -0.14% | 92.0% | 90.0% | 100.0% | 100.1% |
| Class D | -0.14% | -0.25% | -0.39% | 101.0% | 98.2% | 100.3% | 100.0% |
| Class F | -0.10% | -0.31% | -0.21% | 101.5% | 99.5% | 100.0% | 99.9% |
| Class TGM | -0.98% | -0.98% | -1.02% | 102.1% | 100.6% | 100.6% | 99.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.14% | -0.02% | -0.49% | 96.4% | 96.8% | 100.0% | 100.2% |
| Class C | -0.10% | 0.27% | -0.37% | 111.3% | 113.9% | 100.0% | 100.1% |
| Class E | 0.03% | 0.99% | -0.50% | 108.1% | 106.7% | 101.1% | 99.8% |
| Overall | -0.08% | 0.33% | -0.45% | 104.1% | 104.7% | 100.3% | 100.0% |
| Class D | -0.07% | 0.42% | -0.06% | 106.9% | 104.4% | 99.6% | 100.1% |
| Class F | -0.12% | -0.54% | 0.79% | 102.9% | 101.6% | 100.0% | 100.0% |
| Class TGM | -1.25% | -1.32% | -1.42% | 95.4% | 92.4% | 99.7% | 99.9% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.15% | -0.33% | -0.64% | 99.7% | 97.1% | 100.0% | 99.9% |
| Class C | -0.08% | 0.18% | 0.01% | 103.3% | 101.2% | 100.1% | 100.0% |
| Class E | -0.23% | -1.83% | -0.41% | 102.7% | 100.1% | 95.2% | 99.7% |
| Overall | -0.15% | -0.53% | -0.36% | 101.6% | 99.2% | 98.8% | 99.9% |
| Class D | -0.02% | -1.08% | -1.02% | 103.8% | 101.3% | 100.3% | 100.1% |
| Class F | 0.28% | 0.20% | -0.30% | 97.2% | 95.1% | 99.9% | 99.9% |
| Class TGM | -1.04% | -1.10% | -1.15% | 90.6% | 89.0% | 99.6% | 99.6% |

  - Test 6.2a: Bins redistribution between context, NEP and EP with no constraint on the number of arithmetic coded bins.

Note: times are not reliable but complexity is not expected to be changed.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.07% | -0.14% | 110.4% | 110.3% | 98.8% | 102.3% |
| Class A2 | -0.09% | -0.24% | -0.25% | 111.1% | 112.4% | 100.4% | 102.3% |
| Class B | -0.04% | -0.25% | -0.12% | 107.5% | 111.0% | 101.4% | 100.2% |
| Class C | -0.03% | -0.02% | -0.12% | 109.9% | 110.9% | 102.7% | 100.1% |
| Class E | -0.07% | -0.05% | 0.17% | 110.4% | 108.8% | 104.1% | 100.3% |
| Overall | -0.06% | -0.14% | -0.10% | 109.6% | 110.7% | 101.5% | 100.9% |
| Class D | -0.01% | 0.13% | -0.11% | 107.0% | 106.4% | 99.9% | 100.0% |
| Class F | -0.12% | -0.11% | -0.18% | 109.1% | 110.3% | 98.8% | 100.1% |
| Class TGM | -0.33% | -0.36% | -0.38% | 109.0% | 108.9% | 99.9% | 100.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14% | -0.28% | 0.07% | 81.5% | 81.2% | 99.9% | 100.2% |
| Class A2 | -0.10% | -0.04% | -0.18% | 77.1% | 75.2% | 100.4% | 99.8% |
| Class B | -0.25% | -0.24% | -0.29% | 101.9% | 100.1% | 99.9% | 100.2% |
| Class C | -0.13% | -0.23% | -0.12% | 100.9% | 98.4% | 100.4% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.17% | -0.21% | -0.15% | 91.9% | 90.2% | 100.1% | 100.1% |
| Class D | -0.13% | 0.03% | -0.15% | 100.8% | 98.0% | 100.2% | 100.0% |
| Class F | -0.14% | -0.47% | -0.51% | 101.2% | 99.1% | 100.0% | 100.1% |
| Class TGM | -0.94% | -0.93% | -0.79% | 100.7% | 99.0% | 100.1% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | -0.23% | -0.79% | 97.8% | 99.8% | 100.0% | 100.4% |
| Class C | 0.05% | 0.19% | -0.22% | 113.1% | 118.2% | 100.0% | 100.0% |
| Class E | -0.15% | 0.02% | -0.11% | 109.3% | 109.0% | 101.1% | 100.0% |
| Overall | -0.07% | -0.03% | -0.43% | 105.6% | 107.9% | 100.3% | 100.2% |
| Class D | 0.04% | 0.48% | 0.03% | 108.4% | 108.3% | 100.1% | 100.1% |
| Class F | -0.21% | -0.62% | 0.36% | 101.8% | 102.3% | 100.0% | 100.0% |
| Class TGM | -1.34% | -1.50% | -1.42% | 96.7% | 94.3% | 99.9% | 100.0% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.19% | -0.10% | -0.52% | 101.9% | 99.8% | 100.1% | 99.9% |
| Class C | 0.00% | 0.11% | -0.45% | 102.6% | 104.0% | 100.1% | 100.1% |
| Class E | -0.04% | -1.68% | 0.39% | 102.8% | 102.9% | 95.2% | 99.8% |
| Overall | -0.09% | -0.43% | -0.27% | 102.4% | 102.0% | 98.8% | 99.9% |
| Class D | 0.05% | -1.01% | -0.18% | 103.5% | 103.7% | 99.9% | 100.0% |
| Class F | 0.28% | -0.38% | 0.01% | 96.0% | 97.2% | 100.1% | 99.9% |
| Class TGM | -1.04% | -1.00% | -1.11% | 91.5% | 92.6% | 99.3% | 99.5% |

  - Test 6.2b: Bins redistribution between context , NEP and EP, with a constraint on the number of arithmetic coded bins (alpha=0.5)

Note: times are not reliable but complexity is not expected to be changed.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.03% | -0.09% | 110.3% | 109.8% | 98.9% | 101.8% |
| Class A2 | -0.10% | -0.10% | -0.21% | 111.0% | 110.9% | 100.4% | 103.1% |
| Class B | -0.05% | -0.36% | -0.17% | 107.9% | 110.3% | 100.8% | 100.1% |
| Class C | -0.03% | 0.01% | -0.06% | 109.4% | 110.2% | 100.9% | 100.1% |
| Class E | -0.05% | 0.05% | 0.16% | 110.1% | 109.5% | 103.4% | 100.1% |
| Overall | -0.06% | -0.11% | -0.09% | 109.5% | 110.1% | 100.9% | 100.9% |
| Class D | -0.01% | 0.06% | -0.11% | 107.0% | 107.8% | 100.2% | 100.0% |
| Class F | -0.13% | -0.08% | -0.18% | 108.8% | 110.8% | 97.7% | 100.1% |
| Class TGM | -0.30% | -0.38% | -0.36% | 109.0% | 109.2% | 101.0% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.16% | -0.22% | 0.04% | 81.2% | 81.1% | 100.0% | 100.6% |
| Class A2 | -0.10% | -0.13% | -0.23% | 77.0% | 75.3% | 100.1% | 99.6% |
| Class B | -0.26% | -0.17% | -0.25% | 102.1% | 99.8% | 99.9% | 100.2% |
| Class C | -0.09% | -0.15% | 0.05% | 100.6% | 98.0% | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.16% | -0.17% | -0.11% | 91.8% | 90.1% | 100.1% | 100.1% |
| Class D | -0.13% | 0.11% | -0.21% | 100.1% | 98.4% | 100.2% | 100.0% |
| Class F | -0.07% | -0.41% | -0.18% | 101.8% | 99.8% | 100.0% | 99.9% |
| Class TGM | -0.92% | -0.92% | -0.90% | 101.7% | 101.0% | 100.4% | 99.5% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.38% | -0.88% | 95.6% | 96.8% | 100.0% | 100.2% |
| Class C | -0.06% | -0.20% | -0.57% | 113.2% | 116.1% | 99.7% | 100.0% |
| Class E | -0.11% | 1.33% | -1.17% | 110.3% | 111.1% | 100.8% | 100.1% |
| Overall | -0.09% | 0.11% | -0.85% | 104.8% | 106.5% | 100.1% | 100.1% |
| Class D | -0.09% | 0.88% | 0.12% | 112.0% | 114.2% | 100.0% | 100.0% |
| Class F | -0.13% | -0.46% | -0.04% | 100.1% | 99.7% | 100.4% | 99.8% |
| Class TGM | -1.34% | -1.33% | -1.32% | 94.5% | 88.2% | 100.1% | 99.6% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.19% | -0.08% | -0.29% | 101.6% | 98.7% | 100.0% | 100.0% |
| Class C | 0.04% | 0.24% | -0.40% | 104.8% | 102.8% | 100.0% | 99.9% |
| Class E | -0.28% | -1.19% | 0.05% | 103.1% | 103.0% | 96.1% | 99.8% |
| Overall | -0.14% | -0.25% | -0.24% | 103.0% | 101.2% | 99.0% | 99.9% |
| Class D | -0.12% | -0.19% | -0.28% | 104.7% | 101.8% | 100.4% | 100.0% |
| Class F | 0.08% | -0.16% | -0.22% | 98.1% | 95.8% | 100.3% | 100.0% |
| Class TGM | -1.00% | -1.04% | -1.11% | 91.1% | 88.5% | 99.3% | 99.3% |

  - Test 6.2c: Bins redistribution between context , NEP and EP, with a constraint on the number of arithmetic coded bins (alpha=1.0)

Note: times are not reliable but complexity is not expected to be changed.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.20% | -0.19% | 110.0% | 110.5% | 98.9% | 101.7% |
| Class A2 | -0.11% | -0.14% | -0.21% | 110.4% | 110.7% | 100.4% | 102.8% |
| Class B | -0.04% | -0.31% | -0.23% | 107.4% | 109.7% | 99.2% | 100.2% |
| Class C | -0.02% | -0.10% | -0.05% | 109.6% | 110.9% | 102.3% | 100.1% |
| Class E | -0.06% | 0.10% | -0.08% | 109.0% | 107.6% | 103.0% | 100.1% |
| Overall | -0.06% | -0.15% | -0.15% | 109.1% | 109.9% | 100.6% | 100.8% |
| Class D | -0.02% | -0.01% | 0.09% | 106.1% | 106.3% | 100.2% | 100.0% |
| Class F | -0.15% | -0.09% | -0.07% | 109.8% | 110.2% | 99.8% | 100.0% |
| Class TGM | -0.30% | -0.36% | -0.33% | 109.7% | 108.5% | 100.7% | 100.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | -0.27% | 0.09% | 81.7% | 81.9% | 100.0% | 100.4% |
| Class A2 | -0.11% | -0.02% | -0.21% | 77.5% | 76.0% | 100.1% | 99.7% |
| Class B | -0.27% | -0.47% | -0.18% | 102.0% | 100.4% | 99.9% | 99.8% |
| Class C | -0.13% | -0.23% | -0.04% | 99.6% | 98.6% | 100.4% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.17% | -0.28% | -0.09% | 91.8% | 90.7% | 100.1% | 100.0% |
| Class D | -0.12% | 0.08% | -0.29% | 100.4% | 97.9% | 100.2% | 100.0% |
| Class F | -0.13% | -0.41% | -0.65% | 100.9% | 98.3% | 100.0% | 99.9% |
| Class TGM | -0.86% | -0.86% | -0.90% | 103.2% | 100.9% | 100.3% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.10% | -0.68% | 95.4% | 96.9% | 100.0% | 100.4% |
| Class C | 0.04% | 0.25% | -0.30% | 110.7% | 112.7% | 100.0% | 100.2% |
| Class E | -0.01% | 0.84% | -1.13% | 108.0% | 107.5% | 101.2% | 99.9% |
| Overall | -0.03% | 0.25% | -0.67% | 103.4% | 104.6% | 100.3% | 100.2% |
| Class D | 0.01% | 0.54% | 0.26% | 103.7% | 103.3% | 100.3% | 100.0% |
| Class F | -0.45% | -0.85% | -0.27% | 102.4% | 101.1% | 100.2% | 99.9% |
| Class TGM | -1.34% | -1.35% | -1.41% | 94.8% | 90.4% | 99.7% | 99.7% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.17% | 0.02% | -0.38% | 100.1% | 98.1% | 100.1% | 100.1% |
| Class C | 0.01% | 0.09% | -0.73% | 101.5% | 100.8% | 100.1% | 100.1% |
| Class E | -0.32% | -1.54% | 0.01% | 100.7% | 98.8% | 94.2% | 99.9% |
| Overall | -0.15% | -0.35% | -0.40% | 100.7% | 99.2% | 98.6% | 100.0% |
| Class D | -0.08% | -0.88% | -0.43% | 100.6% | 99.1% | 100.2% | 100.0% |
| Class F | 0.23% | 0.03% | 0.18% | 97.0% | 95.9% | 100.1% | 99.9% |
| Class TGM | -0.92% | -0.81% | -1.05% | 89.3% | 88.9% | 99.7% | 99.4% |

  - Test 6.2d: Context bin coding is redistributed between context and NEP

Note: times are not reliable but complexity is not expected to be changed.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.02% | 0.01% | 110.0% | 110.8% | 98.9% | 102.2% |
| Class A2 | 0.00% | 0.10% | -0.02% | 110.1% | 112.4% | 100.2% | 102.6% |
| Class B | -0.01% | 0.01% | 0.14% | 106.8% | 109.8% | 100.9% | 100.2% |
| Class C | 0.00% | -0.05% | -0.04% | 109.1% | 111.0% | 101.7% | 100.1% |
| Class E | -0.04% | 0.29% | 0.20% | 109.4% | 107.9% | 103.2% | 100.1% |
| Overall | -0.01% | 0.05% | 0.06% | 108.8% | 110.4% | 101.0% | 100.9% |
| Class D | -0.02% | 0.09% | -0.08% | 106.1% | 106.3% | 100.1% | 100.0% |
| Class F | -0.08% | -0.17% | -0.12% | 109.4% | 109.1% | 99.9% | 100.0% |
| Class TGM | -0.01% | 0.01% | -0.02% | 109.0% | 108.3% | 100.6% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.13% | 0.08% | 81.4% | 81.2% | 99.9% | 100.0% |
| Class A2 | 0.01% | 0.03% | 0.01% | 77.0% | 75.2% | 100.2% | 99.8% |
| Class B | -0.02% | 0.05% | 0.06% | 101.0% | 99.0% | 100.0% | 100.0% |
| Class C | -0.05% | 0.14% | 0.07% | 101.3% | 99.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.03% | 0.06% | 91.7% | 90.1% | 100.0% | 100.0% |
| Class D | 0.01% | -0.33% | 0.02% | 101.0% | 97.9% | 100.0% | 100.0% |
| Class F | -0.04% | -0.26% | -0.08% | 101.6% | 98.4% | 100.0% | 100.0% |
| Class TGM | 0.03% | 0.08% | 0.06% | 102.3% | 101.0% | 100.1% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.04% | 0.20% | -0.82% | 96.6% | 97.0% | 100.0% | 100.1% |
| Class C | 0.05% | 0.19% | -0.31% | 110.8% | 112.8% | 100.0% | 100.1% |
| Class E | 0.15% | -0.12% | -1.43% | 108.6% | 108.9% | 101.0% | 100.1% |
| Overall | 0.07% | 0.11% | -0.80% | 104.1% | 105.0% | 100.3% | 100.1% |
| Class D | 0.06% | 0.41% | 0.36% | 108.5% | 106.2% | 100.1% | 100.0% |
| Class F | 0.11% | 0.60% | 0.53% | 103.4% | 102.8% | 99.8% | 100.0% |
| Class TGM | -0.15% | -0.08% | -0.33% | 95.2% | 90.9% | 100.1% | 100.0% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.00% | 0.00% | 99.9% | 96.5% | 100.0% | 99.7% |
| Class C | 0.08% | -0.03% | -0.83% | 104.8% | 103.1% | 100.0% | 99.9% |
| Class E | -0.13% | -0.57% | 0.38% | 103.7% | 103.0% | 96.1% | 99.9% |
| Overall | -0.02% | -0.15% | -0.18% | 102.5% | 100.3% | 99.0% | 99.8% |
| Class D | 0.00% | -0.28% | -0.02% | 105.3% | 102.7% | 100.5% | 100.0% |
| Class F | 0.43% | 0.22% | 0.02% | 98.5% | 97.6% | 100.1% | 99.9% |
| Class TGM | 0.15% | 0.12% | 0.18% | 91.5% | 91.4% | 99.4% | 99.4% |

# Additional analysis on complexity

xxx

# Reference

[1] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 14 (ECM 14),” JVET-AI2025, July 2024.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, July 2024.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).