| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0211-v4 |
| --- | --- |

| Title: | EE2-6.1-6.2: Entropy coding extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Franck Galpin Fabrice Le Leannec Federico Lo Bianco Charles Salmon-Legagneur Muhammet Balcilar Pavel Nikitin Muhammed Coban Marta Karczewicz Vadim Seregin | Email: | franck.galpin@interdigital.com pnikitin@qti.qualcomm.com |
| Source: | Interdigital, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the results of EE2-6.1 and EE2-6.2 tests, where the new type of bin coding was introduced.

There are seven tests related to the proposal. In the first the Cabac retraining for ECM 14.0 is done. In the second test EP bins are allowed to be used for context coding bins. In the third test all coding bins can be use either CABAC or EP bins. In the fourth test CABAC/NEP/EP is allowed for all elements. In the fifth and sixth tests the number of bins using CABAC/NEP is kept the same as ECM14.0, allowing CABAC/NEP/EP for all the bins. In Test 6.2b the complexity of NEP is considered 0.5 of complexity of CABAC, and in test 6.2c the complexity of NEP is considered equal to the complexity of CABAC. In test 6.2d only ECM14.0 bins are allowed to use NEP/CABAC. Additional results showing complexity/gains trade-offs are also reported.

Test 6.1a:

AI: -0.01% 0.06% 0.06% EncT: 10X.X% DecT: 10X.X%

RA: -0.02% 0.07% 0.08% EncT: 10X.X% DecT: 10X.X%

LB: 0.09% 0.31% -0.78% EncT: 10X.X% DecT: 10X.X%

Test 6.1b:

AI: -0.01% 0.06% 0.06% EncT: 103.4% DecT: 100.2%

RA: -0.01% 0.06% 0.06% EncT: 100.3% DecT: 100.5%

LB: 0.07% 0.21% -0.58% EncT: 102.1% DecT: 99.9%

Test 6.1c:

AI: -0.06% -0.17% -0.11% EncT: 10X.X% DecT: 10X.X%

RA: -0.16% -0.12% -0.14% EncT: 10X.X% DecT: 10X.X%

LB: -0.08% 0.33% -0.45% EncT: 10X.X% DecT: 10X.X%

Test 6.2a:

AI: -0.06% -0.14% -0.10% EncT: 10X.X% DecT: 10X.X%

RA: -0.17% -0.21% -0.15% EncT: 10X.X% DecT: 10X.X%

LB: -0.07% -0.03% -0.43% EncT: 10X.X% DecT: 10X.X%

Test 6.2b:

AI: -0.06% -0.11% -0.09% EncT: 10X.X% DecT: 10X.X%

RA: -0.16% -0.017% -0.11% EncT: 10X.X% DecT: 10X.X%

LB: -0.09% 0.11% -0.85% EncT: 10X.X% DecT: 10X.X%

Test 6.2c:

AI: -0.06% -0.15% -0.15% EncT: 10X.X% DecT: 10X.X%

RA: -0.17% -0.28% -0.09% EncT: 10X.X% DecT: 10X.X%

LB: -0.03% 0.25% -0.67% EncT: 10X.X% DecT: 10X.X%

Test 6.2d:

AI: -0.01% 0.05% 0.06% EncT: 10X.X% DecT: 10X.X%

RA: -0.02% 0.03% 0.06% EncT: 10X.X% DecT: 10X.X%

LB: 0.07% 0.11% -0.80% EncT: 10X.X% DecT: 10X.X%

# Introduction

# Results

In these tests the training is performed using all CTC sequences including class TGM encoder in AI, RA, LB and LP configurations.

  - Test 6.1a: Context initialization retraining

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.02% | 0.01% | 110.3% | 109.9% | 98.9% | 101.7% |
| Class A2 | 0.00% | 0.11% | -0.02% | 110.9% | 111.7% | 100.4% | 102.5% |
| Class B | -0.01% | 0.01% | 0.14% | 106.9% | 109.3% | 102.7% | 100.2% |
| Class C | 0.00% | -0.04% | -0.03% | 109.5% | 110.2% | 100.8% | 100.1% |
| Class E | -0.04% | 0.29% | 0.19% | 110.1% | 108.8% | 102.0% | 100.2% |
| Overall | -0.01% | 0.06% | 0.06% | 109.2% | 109.9% | 101.1% | 100.8% |
| Class D | -0.02% | 0.09% | -0.08% | 106.9% | 108.0% | 99.9% | 100.0% |
| Class F | -0.08% | -0.18% | -0.13% | 108.7% | 109.7% | 99.9% | 100.2% |
| Class TGM | -0.01% | 0.00% | -0.02% | 108.8% | 109.1% | 100.6% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.03% | -0.02% | 81.9% | 81.3% | 99.9% | 99.8% |
| Class A2 | -0.01% | 0.16% | -0.02% | 77.6% | 76.3% | 100.3% | 100.1% |
| Class B | -0.02% | 0.00% | 0.15% | 101.6% | 98.8% | 99.9% | 100.2% |
| Class C | -0.04% | 0.18% | 0.14% | 101.5% | 99.3% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.07% | 0.08% | 92.2% | 90.4% | 100.0% | 100.1% |
| Class D | 0.00% | -0.25% | -0.02% | 100.9% | 98.9% | 100.1% | 100.0% |
| Class F | -0.06% | -0.25% | -0.06% | 101.1% | 98.8% | 99.9% | 100.0% |
| Class TGM | 0.03% | 0.04% | 0.08% | 101.5% | 99.7% | 100.3% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.08% | -0.63% | 96.3% | 97.5% | 100.0% | 100.3% |
| Class C | 0.09% | 0.43% | -0.25% | 111.4% | 112.3% | 100.0% | 100.0% |
| Class E | 0.19% | 0.54% | -1.73% | 107.9% | 108.4% | 101.0% | 100.1% |
| Overall | 0.09% | 0.31% | -0.78% | 104.0% | 105.0% | 100.3% | 100.1% |
| Class D | 0.02% | 0.73% | 0.27% | 108.2% | 105.6% | 100.0% | 100.0% |
| Class F | 0.07% | 0.29% | 0.26% | 101.0% | 100.9% | 99.7% | 100.1% |
| Class TGM | -0.21% | -0.12% | -0.27% | 95.9% | 91.1% | 99.8% | 99.9% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.04% | 0.15% | 100.2% | 97.5% | 100.1% | 99.9% |
| Class C | 0.05% | -0.03% | -0.85% | 103.6% | 100.0% | 100.0% | 100.0% |
| Class E | -0.19% | -1.16% | 0.22% | 104.8% | 103.0% | 96.1% | 99.8% |
| Overall | -0.06% | -0.32% | -0.17% | 102.4% | 99.7% | 99.0% | 99.9% |
| Class D | 0.05% | -0.68% | -0.33% | 104.7% | 101.2% | 100.7% | 100.0% |
| Class F | 0.40% | -0.65% | -0.17% | 96.6% | 95.7% | 100.2% | 99.9% |
| Class TGM | 0.10% | 0.04% | -0.03% | 90.2% | 89.6% | 100.0% | 99.6% |

  - Test 6.1b: Context bins coding is redistributed between context and EP coding

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.02% | 0.01% | 103.2% | 99.5% | 100.8% | 100.0% |
| Class A2 | 0.00% | 0.11% | -0.02% | 104.7% | 103.7% | 100.0% | 99.9% |
| Class B | -0.01% | 0.01% | 0.14% | 103.3% | 99.5% | 102.0% | 100.1% |
| Class C | 0.00% | -0.04% | -0.03% | 103.3% | 100.0% | 101.4% | 100.0% |
| Class E | -0.04% | 0.29% | 0.19% | 102.5% | 98.6% | 100.0% | 100.1% |
| Overall | -0.01% | 0.06% | 0.06% | 103.4% | 100.2% | 101.0% | 100.0% |
| Class D | -0.02% | 0.10% | -0.08% | 104.0% | 103.2% | 99.8% | 100.0% |
| Class F | -0.08% | -0.18% | -0.13% | 108.7% | 118.1% | 100.0% | 99.8% |
| Class TGM | -0.01% | 0.00% | -0.02% | 110.8% | 119.5% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.02% | -0.01% | 0.13% | 99.6% | 97.6% | 100.0% | 99.7% |
| Class C | -0.02% | 0.17% | 0.08% | 101.2% | 98.2% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.00% | -0.26% | -0.03% | 100.8% | 98.0% | 100.2% | 100.0% |
| Class F | -0.05% | -0.25% | -0.07% | 104.8% | 119.8% | 100.0% | 99.9% |
| Class TGM | 0.03% | 0.03% | 0.07% | 106.2% | 128.9% | 99.7% | 99.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | 0.08% | 0.33% | -0.25% | 101.8% | 99.4% | 99.9% | 99.9% |
| Class E | 0.16% | -0.36% | -1.16% | 102.3% | 99.5% | 100.8% | 100.4% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | 0.06% | 0.93% | 0.49% | 104.3% | 103.1% | 99.8% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

  - Test 6.1c: Bins coding is redistributed between context and EP coding

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.19% | -0.13% | 109.9% | 111.1% | 99.0% | 101.6% |
| Class A2 | -0.09% | -0.23% | -0.23% | 110.1% | 111.5% | 100.4% | 102.9% |
| Class B | -0.05% | -0.30% | -0.23% | 107.5% | 109.7% | 102.4% | 100.0% |
| Class C | -0.03% | -0.03% | -0.08% | 109.6% | 110.7% | 100.9% | 100.1% |
| Class E | -0.07% | -0.04% | 0.17% | 110.1% | 109.2% | 101.9% | 100.1% |
| Overall | -0.06% | -0.17% | -0.11% | 109.2% | 110.4% | 101.0% | 100.8% |
| Class D | -0.01% | 0.19% | -0.04% | 106.3% | 106.9% | 99.9% | 100.0% |
| Class F | -0.12% | -0.13% | -0.24% | 109.8% | 109.5% | 99.9% | 100.0% |
| Class TGM | -0.33% | -0.39% | -0.38% | 108.7% | 108.7% | 101.9% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14% | -0.13% | 0.00% | 81.5% | 81.8% | 100.0% | 100.0% |
| Class A2 | -0.07% | 0.06% | -0.18% | 77.1% | 75.6% | 100.0% | 99.9% |
| Class B | -0.27% | -0.09% | -0.27% | 101.8% | 98.4% | 100.0% | 100.2% |
| Class C | -0.09% | -0.29% | -0.04% | 101.3% | 98.7% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.16% | -0.12% | -0.14% | 92.0% | 90.0% | 100.0% | 100.1% |
| Class D | -0.14% | -0.25% | -0.39% | 101.0% | 98.2% | 100.3% | 100.0% |
| Class F | -0.10% | -0.31% | -0.21% | 101.5% | 99.5% | 100.0% | 99.9% |
| Class TGM | -0.98% | -0.98% | -1.02% | 102.1% | 100.6% | 100.6% | 99.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.14% | -0.02% | -0.49% | 96.4% | 96.8% | 100.0% | 100.2% |
| Class C | -0.10% | 0.27% | -0.37% | 111.3% | 113.9% | 100.0% | 100.1% |
| Class E | 0.03% | 0.99% | -0.50% | 108.1% | 106.7% | 101.1% | 99.8% |
| Overall | -0.08% | 0.33% | -0.45% | 104.1% | 104.7% | 100.3% | 100.0% |
| Class D | -0.07% | 0.42% | -0.06% | 106.9% | 104.4% | 99.6% | 100.1% |
| Class F | -0.12% | -0.54% | 0.79% | 102.9% | 101.6% | 100.0% | 100.0% |
| Class TGM | -1.25% | -1.32% | -1.42% | 95.4% | 92.4% | 99.7% | 99.9% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.15% | -0.33% | -0.64% | 99.7% | 97.1% | 100.0% | 99.9% |
| Class C | -0.08% | 0.18% | 0.01% | 103.3% | 101.2% | 100.1% | 100.0% |
| Class E | -0.23% | -1.83% | -0.41% | 102.7% | 100.1% | 95.2% | 99.7% |
| Overall | -0.15% | -0.53% | -0.36% | 101.6% | 99.2% | 98.8% | 99.9% |
| Class D | -0.02% | -1.08% | -1.02% | 103.8% | 101.3% | 100.3% | 100.1% |
| Class F | 0.28% | 0.20% | -0.30% | 97.2% | 95.1% | 99.9% | 99.9% |
| Class TGM | -1.04% | -1.10% | -1.15% | 90.6% | 89.0% | 99.6% | 99.6% |

  - Test 6.2a: Bins redistribution between context, NEP and EP with no constraint on the number of arithmetic coded bins.

Note: times are not reliable but complexity is not expected to be changed.

Note: ECM-14.0 has 5096 context bin + 1360 “context” EP bins (without counting bins related to residual coding).

| Model |  |  | Number of contexts |
| --- | --- | --- | --- |
| EP |  |  | 84 |
| NEP |  |  | 606 |
| CABAC |  |  | 5766 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.07% | -0.14% | 110.4% | 110.3% | 98.8% | 102.3% |
| Class A2 | -0.09% | -0.24% | -0.25% | 111.1% | 112.4% | 100.4% | 102.3% |
| Class B | -0.04% | -0.25% | -0.12% | 107.5% | 111.0% | 101.4% | 100.2% |
| Class C | -0.03% | -0.02% | -0.12% | 109.9% | 110.9% | 102.7% | 100.1% |
| Class E | -0.07% | -0.05% | 0.17% | 110.4% | 108.8% | 104.1% | 100.3% |
| Overall | -0.06% | -0.14% | -0.10% | 109.6% | 110.7% | 101.5% | 100.9% |
| Class D | -0.01% | 0.13% | -0.11% | 107.0% | 106.4% | 99.9% | 100.0% |
| Class F | -0.12% | -0.11% | -0.18% | 109.1% | 110.3% | 98.8% | 100.1% |
| Class TGM | -0.33% | -0.36% | -0.38% | 109.0% | 108.9% | 99.9% | 100.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14% | -0.28% | 0.07% | 81.5% | 81.2% | 99.9% | 100.2% |
| Class A2 | -0.10% | -0.04% | -0.18% | 77.1% | 75.2% | 100.4% | 99.8% |
| Class B | -0.25% | -0.24% | -0.29% | 101.9% | 100.1% | 99.9% | 100.2% |
| Class C | -0.13% | -0.23% | -0.12% | 100.9% | 98.4% | 100.4% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.17% | -0.21% | -0.15% | 91.9% | 90.2% | 100.1% | 100.1% |
| Class D | -0.13% | 0.03% | -0.15% | 100.8% | 98.0% | 100.2% | 100.0% |
| Class F | -0.14% | -0.47% | -0.51% | 101.2% | 99.1% | 100.0% | 100.1% |
| Class TGM | -0.94% | -0.93% | -0.79% | 100.7% | 99.0% | 100.1% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | -0.23% | -0.79% | 97.8% | 99.8% | 100.0% | 100.4% |
| Class C | 0.05% | 0.19% | -0.22% | 113.1% | 118.2% | 100.0% | 100.0% |
| Class E | -0.15% | 0.02% | -0.11% | 109.3% | 109.0% | 101.1% | 100.0% |
| Overall | -0.07% | -0.03% | -0.43% | 105.6% | 107.9% | 100.3% | 100.2% |
| Class D | 0.04% | 0.48% | 0.03% | 108.4% | 108.3% | 100.1% | 100.1% |
| Class F | -0.21% | -0.62% | 0.36% | 101.8% | 102.3% | 100.0% | 100.0% |
| Class TGM | -1.34% | -1.50% | -1.42% | 96.7% | 94.3% | 99.9% | 100.0% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.19% | -0.10% | -0.52% | 101.9% | 99.8% | 100.1% | 99.9% |
| Class C | 0.00% | 0.11% | -0.45% | 102.6% | 104.0% | 100.1% | 100.1% |
| Class E | -0.04% | -1.68% | 0.39% | 102.8% | 102.9% | 95.2% | 99.8% |
| Overall | -0.09% | -0.43% | -0.27% | 102.4% | 102.0% | 98.8% | 99.9% |
| Class D | 0.05% | -1.01% | -0.18% | 103.5% | 103.7% | 99.9% | 100.0% |
| Class F | 0.28% | -0.38% | 0.01% | 96.0% | 97.2% | 100.1% | 99.9% |
| Class TGM | -1.04% | -1.00% | -1.11% | 91.5% | 92.6% | 99.3% | 99.5% |

  - Test 6.2b: Bins redistribution between context , NEP and EP, with a constraint on the number of arithmetic coded bins (alpha=0.5)

Note: times are not reliable but complexity is not expected to be changed.

| Model | Number of contexts |
| --- | --- |
| EP | 829 |
| NEP | 1062 |
| CABAC | 4565 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.03% | -0.09% | 110.3% | 109.8% | 98.9% | 101.8% |
| Class A2 | -0.10% | -0.10% | -0.21% | 111.0% | 110.9% | 100.4% | 103.1% |
| Class B | -0.05% | -0.36% | -0.17% | 107.9% | 110.3% | 100.8% | 100.1% |
| Class C | -0.03% | 0.01% | -0.06% | 109.4% | 110.2% | 100.9% | 100.1% |
| Class E | -0.05% | 0.05% | 0.16% | 110.1% | 109.5% | 103.4% | 100.1% |
| Overall | -0.06% | -0.11% | -0.09% | 109.5% | 110.1% | 100.9% | 100.9% |
| Class D | -0.01% | 0.06% | -0.11% | 107.0% | 107.8% | 100.2% | 100.0% |
| Class F | -0.13% | -0.08% | -0.18% | 108.8% | 110.8% | 97.7% | 100.1% |
| Class TGM | -0.30% | -0.38% | -0.36% | 109.0% | 109.2% | 101.0% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.16% | -0.22% | 0.04% | 81.2% | 81.1% | 100.0% | 100.6% |
| Class A2 | -0.10% | -0.13% | -0.23% | 77.0% | 75.3% | 100.1% | 99.6% |
| Class B | -0.26% | -0.17% | -0.25% | 102.1% | 99.8% | 99.9% | 100.2% |
| Class C | -0.09% | -0.15% | 0.05% | 100.6% | 98.0% | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.16% | -0.17% | -0.11% | 91.8% | 90.1% | 100.1% | 100.1% |
| Class D | -0.13% | 0.11% | -0.21% | 100.1% | 98.4% | 100.2% | 100.0% |
| Class F | -0.07% | -0.41% | -0.18% | 101.8% | 99.8% | 100.0% | 99.9% |
| Class TGM | -0.92% | -0.92% | -0.90% | 101.7% | 101.0% | 100.4% | 99.5% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.38% | -0.88% | 95.6% | 96.8% | 100.0% | 100.2% |
| Class C | -0.06% | -0.20% | -0.57% | 113.2% | 116.1% | 99.7% | 100.0% |
| Class E | -0.11% | 1.33% | -1.17% | 110.3% | 111.1% | 100.8% | 100.1% |
| Overall | -0.09% | 0.11% | -0.85% | 104.8% | 106.5% | 100.1% | 100.1% |
| Class D | -0.09% | 0.88% | 0.12% | 112.0% | 114.2% | 100.0% | 100.0% |
| Class F | -0.13% | -0.46% | -0.04% | 100.1% | 99.7% | 100.4% | 99.8% |
| Class TGM | -1.34% | -1.33% | -1.32% | 94.5% | 88.2% | 100.1% | 99.6% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.19% | -0.08% | -0.29% | 101.6% | 98.7% | 100.0% | 100.0% |
| Class C | 0.04% | 0.24% | -0.40% | 104.8% | 102.8% | 100.0% | 99.9% |
| Class E | -0.28% | -1.19% | 0.05% | 103.1% | 103.0% | 96.1% | 99.8% |
| Overall | -0.14% | -0.25% | -0.24% | 103.0% | 101.2% | 99.0% | 99.9% |
| Class D | -0.12% | -0.19% | -0.28% | 104.7% | 101.8% | 100.4% | 100.0% |
| Class F | 0.08% | -0.16% | -0.22% | 98.1% | 95.8% | 100.3% | 100.0% |
| Class TGM | -1.00% | -1.04% | -1.11% | 91.1% | 88.5% | 99.3% | 99.3% |

  - Test 6.2c: Bins redistribution between context , NEP and EP, with a constraint on the number of arithmetic coded bins (alpha=1.0)

Note: times are not reliable but complexity is not expected to be changed.

| Model | Number of contexts |
| --- | --- |
| EP | 1360 |
| NEP | 242 |
| CABAC | 4854 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | -0.20% | -0.19% | 110.0% | 110.5% | 98.9% | 101.7% |
| Class A2 | -0.11% | -0.14% | -0.21% | 110.4% | 110.7% | 100.4% | 102.8% |
| Class B | -0.04% | -0.31% | -0.23% | 107.4% | 109.7% | 99.2% | 100.2% |
| Class C | -0.02% | -0.10% | -0.05% | 109.6% | 110.9% | 102.3% | 100.1% |
| Class E | -0.06% | 0.10% | -0.08% | 109.0% | 107.6% | 103.0% | 100.1% |
| Overall | -0.06% | -0.15% | -0.15% | 109.1% | 109.9% | 100.6% | 100.8% |
| Class D | -0.02% | -0.01% | 0.09% | 106.1% | 106.3% | 100.2% | 100.0% |
| Class F | -0.15% | -0.09% | -0.07% | 109.8% | 110.2% | 99.8% | 100.0% |
| Class TGM | -0.30% | -0.36% | -0.33% | 109.7% | 108.5% | 100.7% | 100.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.13% | -0.27% | 0.09% | 81.7% | 81.9% | 100.0% | 100.4% |
| Class A2 | -0.11% | -0.02% | -0.21% | 77.5% | 76.0% | 100.1% | 99.7% |
| Class B | -0.27% | -0.47% | -0.18% | 102.0% | 100.4% | 99.9% | 99.8% |
| Class C | -0.13% | -0.23% | -0.04% | 99.6% | 98.6% | 100.4% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.17% | -0.28% | -0.09% | 91.8% | 90.7% | 100.1% | 100.0% |
| Class D | -0.12% | 0.08% | -0.29% | 100.4% | 97.9% | 100.2% | 100.0% |
| Class F | -0.13% | -0.41% | -0.65% | 100.9% | 98.3% | 100.0% | 99.9% |
| Class TGM | -0.86% | -0.86% | -0.90% | 103.2% | 100.9% | 100.3% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.10% | -0.68% | 95.4% | 96.9% | 100.0% | 100.4% |
| Class C | 0.04% | 0.25% | -0.30% | 110.7% | 112.7% | 100.0% | 100.2% |
| Class E | -0.01% | 0.84% | -1.13% | 108.0% | 107.5% | 101.2% | 99.9% |
| Overall | -0.03% | 0.25% | -0.67% | 103.4% | 104.6% | 100.3% | 100.2% |
| Class D | 0.01% | 0.54% | 0.26% | 103.7% | 103.3% | 100.3% | 100.0% |
| Class F | -0.45% | -0.85% | -0.27% | 102.4% | 101.1% | 100.2% | 99.9% |
| Class TGM | -1.34% | -1.35% | -1.41% | 94.8% | 90.4% | 99.7% | 99.7% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.17% | 0.02% | -0.38% | 100.1% | 98.1% | 100.1% | 100.1% |
| Class C | 0.01% | 0.09% | -0.73% | 101.5% | 100.8% | 100.1% | 100.1% |
| Class E | -0.32% | -1.54% | 0.01% | 100.7% | 98.8% | 94.2% | 99.9% |
| Overall | -0.15% | -0.35% | -0.40% | 100.7% | 99.2% | 98.6% | 100.0% |
| Class D | -0.08% | -0.88% | -0.43% | 100.6% | 99.1% | 100.2% | 100.0% |
| Class F | 0.23% | 0.03% | 0.18% | 97.0% | 95.9% | 100.1% | 99.9% |
| Class TGM | -0.92% | -0.81% | -1.05% | 89.3% | 88.9% | 99.7% | 99.4% |

  - Test 6.2d: Context bin coding is redistributed between context and NEP

Note: times are not reliable but complexity is not expected to be changed.

Note: ECM-14.0 has 5096 context bin in total.

| Model | Number of contexts |
| --- | --- |
| EP | 0 |
| NEP | 95 |
| CABAC | 5001 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.02% | 0.01% | 110.0% | 110.8% | 98.9% | 102.2% |
| Class A2 | 0.00% | 0.10% | -0.02% | 110.1% | 112.4% | 100.2% | 102.6% |
| Class B | -0.01% | 0.01% | 0.14% | 106.8% | 109.8% | 100.9% | 100.2% |
| Class C | 0.00% | -0.05% | -0.04% | 109.1% | 111.0% | 101.7% | 100.1% |
| Class E | -0.04% | 0.29% | 0.20% | 109.4% | 107.9% | 103.2% | 100.1% |
| Overall | -0.01% | 0.05% | 0.06% | 108.8% | 110.4% | 101.0% | 100.9% |
| Class D | -0.02% | 0.09% | -0.08% | 106.1% | 106.3% | 100.1% | 100.0% |
| Class F | -0.08% | -0.17% | -0.12% | 109.4% | 109.1% | 99.9% | 100.0% |
| Class TGM | -0.01% | 0.01% | -0.02% | 109.0% | 108.3% | 100.6% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.13% | 0.08% | 81.4% | 81.2% | 99.9% | 100.0% |
| Class A2 | 0.01% | 0.03% | 0.01% | 77.0% | 75.2% | 100.2% | 99.8% |
| Class B | -0.02% | 0.05% | 0.06% | 101.0% | 99.0% | 100.0% | 100.0% |
| Class C | -0.05% | 0.14% | 0.07% | 101.3% | 99.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.02% | 0.03% | 0.06% | 91.7% | 90.1% | 100.0% | 100.0% |
| Class D | 0.01% | -0.33% | 0.02% | 101.0% | 97.9% | 100.0% | 100.0% |
| Class F | -0.04% | -0.26% | -0.08% | 101.6% | 98.4% | 100.0% | 100.0% |
| Class TGM | 0.03% | 0.08% | 0.06% | 102.3% | 101.0% | 100.1% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.04% | 0.20% | -0.82% | 96.6% | 97.0% | 100.0% | 100.1% |
| Class C | 0.05% | 0.19% | -0.31% | 110.8% | 112.8% | 100.0% | 100.1% |
| Class E | 0.15% | -0.12% | -1.43% | 108.6% | 108.9% | 101.0% | 100.1% |
| Overall | 0.07% | 0.11% | -0.80% | 104.1% | 105.0% | 100.3% | 100.1% |
| Class D | 0.06% | 0.41% | 0.36% | 108.5% | 106.2% | 100.1% | 100.0% |
| Class F | 0.11% | 0.60% | 0.53% | 103.4% | 102.8% | 99.8% | 100.0% |
| Class TGM | -0.15% | -0.08% | -0.33% | 95.2% | 90.9% | 100.1% | 100.0% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | 0.00% | 0.00% | 99.9% | 96.5% | 100.0% | 99.7% |
| Class C | 0.08% | -0.03% | -0.83% | 104.8% | 103.1% | 100.0% | 99.9% |
| Class E | -0.13% | -0.57% | 0.38% | 103.7% | 103.0% | 96.1% | 99.9% |
| Overall | -0.02% | -0.15% | -0.18% | 102.5% | 100.3% | 99.0% | 99.8% |
| Class D | 0.00% | -0.28% | -0.02% | 105.3% | 102.7% | 100.5% | 100.0% |
| Class F | 0.43% | 0.22% | 0.02% | 98.5% | 97.6% | 100.1% | 99.9% |
| Class TGM | 0.15% | 0.12% | 0.18% | 91.5% | 91.4% | 99.4% | 99.4% |

# Additional analysis on complexity

In test 6.2 an additional entropy model, NEP, is introduced. This model uses the same initial probability flexibility as CABAC, but remove the probability update, as for a EP bin coding, to lower the complexity of decoding. In test 6.2, results are presented with several tradeoffs in terms of contexts number using the CABAC coding. This contribution shows more results where the tradeoffs was computed in terms of throughput instead of number of contexts. All corresponding CABAC parameters are available in the branch EE1-6.2 of the repository. The throughput computation is also available using a macro available in the same branch.

## Overall results

In the following, the tests were performed on top of ECM-14.0, for RA configuration.

Two configurations were tested:

- In configuration “EP”, available entropy coding models are CABAC and EP,

- In configuration “NEP”, available entropy coding models are CABAC, NEP and EP.

For the 2 configurations, several points in terms of throughput are used to show the gains/complexity tradeoffs.

In above figures, we show the results of the trade-off between throughput and gains over ECM-14.0:

- EP: the throughput is computed as the number of bins using the CABAC coding, w.r.t.  the total number of bins using CABAC and EP, in ECM-14.0 RA CTCs. That is, the throughput metric used here reflects the complexity of entropy decoding in current RA CTCs. For ECM, the best gains are obtained when about 91% of bins are CABAC coded. As a reference, in VTM, in the same conditions, 78% of bins are CABAC coded.

- NEP: in addition to CABAC coding, NEP is added. We count the “complexity” as half the one of CABAC as a reference (see later for complexity assessment).

At a given gain, for example a loss of 0.07% on top of ECM-14.0 (see below for more trade-offs details)

  - EP+CABAC: about 81% of the bins are CABAC coded (and the rest is EP coded)

  - EP+NEP+CABAC: about 61% of the bins are CABAC coded, 25% NEP coded (and the rest is EP coded).

## Complexity assessment

### Memory usage

The table shows the memory usage per contexts. Currently in ECM-14.0, there are 6432 contexts (4 types x 1608 contexts).

| Model | Working memory | ROM |
| --- | --- | --- |
| EP | 0 | 0 |
| NEP | 10 bits | 6 bits |
| CABAC | 64 bits | 36 bits |

Note: we do not count the memory used for computation here.

### Operations

| Model | Multiplication | Addition | Comparison | shift | masking |
| --- | --- | --- | --- | --- | --- |
| EP | 0 | 2 | 1 | 1 | 0 |
| NEP | 1 | 4 | 1 | 4 | 0 |
| CABAC | 3 | 16 | 5 | 17 | 10 |

Note that the bits fetching operations are not counted in the complexity.

## Detailed results

### Tests ecm14, 6.1c (CABAC+EP), 6.2a(CABAC+NEP+EP),

| Model | % coded bins |  |  |
| --- | --- | --- | --- |
|  | Ecm14 | 6.1c | 6.2a |
| EP | 12.6 | 9.7 | 8.2 |
| NEP | 0 | 0 | 0.3 |
| CABAC | 87.4 | 90.3 | 91.5 |

### EP+CABAC

| Model | % coded bins |
| --- | --- |
| EP | 9.5 |
| NEP | 0 |
| CABAC | 90.5 |

#### 90% CABAC

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | -0.14% | -0.13% | 0.00% |
| Class A2 | -0.07% | 0.06% | -0.18% |
| Class B | -0.27% | -0.09% | -0.27% |
| Class C | -0.09% | -0.29% | -0.04% |
| Class E |  |  |  |
| Overall | -0.16% | -0.12% | -0.14% |
| Class D | -0.14% | -0.25% | -0.39% |
| Class F | -0.10% | -0.31% | -0.21% |
| Class TGM | -0.98% | -0.98% | -1.02% |

#### 80% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 18.7 |
| NEP | 0 |
| CABAC | 81.3 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.21% | -0.25% | 0.23% |
| Class A2 | 0.15% | 0.17% | -0.04% |
| Class B | -0.05% | -0.22% | -0.08% |
| Class C | 0.06% | -0.23% | 0.05% |
| Class E |  |  |  |
| Overall | 0.07% | -0.15% | 0.02% |
| Class D | 0.04% | 0.19% | -0.27% |
| Class F | 0.02% | -0.21% | -0.07% |
| Class TGM | -0.59% | -0.51% | -0.62% |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 1.65% | 0.36% | 0.97% |
| Class A2 | 1.08% | 0.44% | 0.51% |
| Class B | 0.61% | 0.26% | 0.38% |
| Class C | 0.60% | 0.08% | 0.30% |
| Class E |  |  |  |
| Overall | 0.91% | 0.27% | 0.50% |
| Class D | 0.60% | -0.08% | -0.02% |
| Class F | 0.47% | 0.16% | 0.52% |
| Class TGM | -0.01% | 0.05% | -0.03% |

#### 70% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 30.1 |
| NEP | 0 |
| CABAC | 69.9 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 3.23% | 3.84% | 2.36% |
| Class A2 | 2.07% | 2.22% | 1.68% |
| Class B | 1.61% | 1.65% | 1.41% |
| Class C | 1.31% | 1.20% | 1.39% |
| Class E |  |  |  |
| Overall | 1.94% | 2.08% | 1.65% |
| Class D | 1.24% | 0.97% | 0.91% |
| Class F | 1.04% | 1.01% | 0.85% |
| Class TGM | 0.40% | 0.48% | 0.54% |

#### 60% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 39.3 |
| NEP | 0 |
| CABAC | 60.7 |

#### 50% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 39.3 |
| NEP | 0 |
| CABAC | 60.7 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 4.55% | 6.03% | 5.11% |
| Class A2 | 3.29% | 3.63% | 3.36% |
| Class B | 2.80% | 3.17% | 3.21% |
| Class C | 2.45% | 2.16% | 2.66% |
| Class E |  |  |  |
| Overall | 3.16% | 3.57% | 3.47% |
| Class D | 2.25% | 2.02% | 1.99% |
| Class F | 2.01% | 2.09% | 2.23% |
| Class TGM | 1.54% | 1.91% | 1.88% |

### EP+NEP+CABAC

#### 90% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 9.4 |
| NEP | 0.4 |
| CABAC | 90.2 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | -0.14% | -0.28% | 0.07% |
| Class A2 | -0.10% | -0.04% | -0.18% |
| Class B | -0.25% | -0.24% | -0.29% |
| Class C | -0.13% | -0.23% | -0.12% |
| Class E |  |  |  |
| Overall | -0.17% | -0.21% | -0.15% |
| Class D | -0.13% | 0.03% | -0.15% |
| Class F | -0.14% | -0.47% | -0.51% |
| Class TGM | -0.94% | -0.93% | -0.79% |

#### 80% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 11.7 |
| NEP | 8.5 |
| CABAC | 79.8 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | -0.13% | -0.43% | 0.01% |
| Class A2 | -0.04% | -0.02% | -0.22% |
| Class B | -0.21% | -0.37% | -0.15% |
| Class C | -0.10% | -0.14% | 0.03% |
| Class E |  |  |  |
| Overall | -0.13% | -0.25% | -0.09% |
| Class D | -0.06% | -0.48% | 0.01% |
| Class F | -0.12% | -0.29% | -0.21% |
| Class TGM | -0.83% | -0.82% | -0.77% |

#### 70% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 13.0 |
| NEP | 16.5 |
| CABAC | 70.6 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.04% | -0.35% | 0.18% |
| Class A2 | 0.04% | 0.09% | -0.06% |
| Class B | -0.13% | -0.05% | -0.10% |
| Class C | -0.02% | -0.04% | 0.21% |
| Class E |  |  |  |
| Overall | -0.03% | -0.08% | 0.05% |
| Class D | -0.08% | -0.23% | -0.34% |
| Class F | 0.00% | -0.35% | -0.29% |
| Class TGM | -0.82% | -0.67% | -0.74% |

#### 60% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 14.1 |
| NEP | 24.9 |
| CABAC | 60.9 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.30% | 0.00% | 0.30% |
| Class A2 | 0.07% | -0.14% | -0.11% |
| Class B | -0.05% | -0.11% | -0.07% |
| Class C | 0.04% | -0.23% | 0.06% |
| Class E |  |  |  |
| Overall | 0.07% | -0.13% | 0.03% |
| Class D | 0.02% | -0.02% | -0.38% |
| Class F | -0.03% | -0.34% | 0.00% |
| Class TGM | -0.72% | -0.72% | -0.65% |

#### 50% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 16.5 |
| NEP | 31.9 |
| CABAC | 51.6 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 0.81% | 0.01% | 0.29% |
| Class A2 | 0.25% | 0.20% | 0.19% |
| Class B | 0.09% | -0.10% | -0.06% |
| Class C | 0.24% | -0.12% | 0.32% |
| Class E |  |  |  |
| Overall | 0.31% | -0.03% | 0.16% |
| Class D | 0.19% | -0.26% | 0.01% |
| Class F | 0.17% | 0.01% | 0.05% |
| Class TGM | -0.50% | -0.49% | -0.43% |

#### 30% CABAC

| Model | % coded bins |
| --- | --- |
| EP | 29.1 |
| NEP | 44.6 |
| CABAC | 26.3 |

|  | Y | U | V |
| --- | --- | --- | --- |
| Class A1 | 2.50% | 1.75% | 1.09% |
| Class A2 | 1.50% | 1.61% | 1.13% |
| Class B | 1.39% | 0.45% | 0.61% |
| Class C | 1.02% | 0.45% | 0.52% |
| Class E |  |  |  |
| Overall | 1.54% | 0.94% | 0.79% |
| Class D | 1.00% | 0.16% | 0.16% |
| Class F | 0.98% | 0.50% | 0.84% |
| Class TGM | 0.69% | 0.86% | 0.99% |

# Reference

[1] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 14 (ECM 14),” JVET-AI2025, July 2024.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, July 2024.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).