| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0217-v3 |
| --- | --- |

| Title: | Non-EE2: Template-based intra interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuan Zhou Zhuoyi Lv Guiqi Wang | Email: | chuan.zhou@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes a template-based intra interpolation filter (TMIF) method. A list filled with filter candidates is constructed. The template cost is used to select the best filter candidate among them.

On top of the ECM14.0, simulation results are reported as below:

AI: -0.03% (Y), 0.04% (U), -0.05% (V),  100.9% (EncT),  100.5% (DecT)

Introduction

In current ECM, for intra prediction, a 2-tap interpolation filter with [1 2 1]/4 filter is applied for reference samples, and one of three interpolation filters is utilized to generate prediction samples. The options include a 6-tap DCT-based interpolation filter (DCTIF) and two 4-tap/6-tap smoothing interpolation filters (SIF). The selection of the filter depends on the intra mode and block size.

Proposed method

The proposed TMIF method includes the following steps:

- Constructing a candidate list of filters, including 6-tap/4-tap SIF and 6-tap DCTIF.

- Calculating the template cost between the reconstructed samples and the prediction samples generated using the evaluated filters in the list.

- Selecting the candidate with the lowest template cost for prediction of the current block.

Specifically, for step 1, if the filter obtained according to the existing method in ECM is an SIF filter, the candidate list is { 4-tap SIF, 6-tap SIF, DCTIF } or { 6-tap SIF, 4-tap SIF, DCTIF }.

Simulation results

The proposed method is tested on top of ECM14.0 under the common test conditions [1].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | 0.02% | -0.03% | 101.8% | 101.0% | 100.2% | 100.2% |
| Class A2 | -0.02% | -0.12% | -0.14% | 101.5% | 100.4% | 99.8% | 100.1% |
| Class B | -0.01% | -0.03% | -0.01% | 100.8% | 100.6% | 99.3% | 100.0% |
| Class C | -0.07% | -0.13% | -0.07% | 100.1% | 100.0% | 100.0% | 100.0% |
| Class E | -0.05% | 0.09% | -0.02% | 100.8% | 100.4% | 101.1% | 100.0% |
| Overall | -0.03% | -0.04% | -0.05% | 100.9% | 100.5% | 100.0% | 100.1% |
| Class D | -0.02% | 0.15% | -0.14% | 99.4% | 99.6% | 99.8% | 100.0% |
| Class F | -0.06% | -0.04% | -0.15% | 101.1% | 95.5% | 98.4% | 100.0% |
| Class TGM | 0.01% | 0.02% | -0.03% | 101.8% | 101.0% | 100.2% | 100.2% |

References

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AI2017, Jul. 2024.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).