| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0249-v1 |
| --- | --- |

| Title: | EE2-2.20_2.21: Neural network-based intra prediction with DIMD mode derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Urban, T. Dumas, F. Galpin, E. François (Interdigital) S. Eadie, P. Garus, P. Nikitin, M. Coban, V. Seregin, M. Karczewicz (Qualcomm) | Email: thierry.dumas@interdigital.com seadie@qti.qualcomm.com |  |
| Source: | Interdigital, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results from the combined EE2 tests of 2.20 and 2.21. These EE2 tests both proposed neural network-based intra prediction; given their similarities, the proponents have converged to a single harmonized design, as reported in this contribution. This combined test replaces the equivalent intra mode predictor in the neural network from 2.20, with DIMD derivation. The following results are reported:

AI:  -0.58%, -0.40%, -0.40% BD-rate gains for Y, U and V, respectively, with 101.6% encoder and 109.8% decoder run times.

RA:  -0.22%, -0.21%, -0.17% BD-rate gains for Y, U and V, respectively, with 99.0% encoder and 99.8% decoder run times.

# Proposed Method

The neural network-based intra prediction algorithm and models are described in JVET-AH2019 [1] and JVET-AH0156 [2].

- SADL for inference

- 16-bit integer for model weights and intermediate results, using 32-bit integer for internal computation

- 6 models depending on the block size (4x32 from JVET-AH0156 has been removed)

- sparse weights enforced at training time for complexity reduction. SADL supports a simple sparse matrix storage (compressed row method) and sparse matrix multiplication algorithm, exploiting the matrices sparsity. Sparsity constraints have been strengthened compared to JVET-AH0156.

Additionally, only the intra prediction output is retained, and the equivalent intra mode is derived using DIMD. The model complexities are summarized in Table 1.

Table 1: Model complexity for the different neural network models.

| Model | Parameter count (k) | kMAC/pixel | Memory Footprint (MB) |
| --- | --- | --- | --- |
| 4x4 | 50.823 | 3176 | 0.195 |
| 8x4 | 61.886 | 1934 | 0.224 |
| 16x4 | 123.420 | 1928 | 0.450 |
| 8x8 | 132.260 | 2066 | 0.472 |
| 16x8 | 178.754 | 1396 | 0.590 |
| 16x16 | 215.376 | 841 | 0.596 |

Number of parameters for the whole intra prediction mode: 762519.

# Experiment results

The simulations of the proposed NN-based intra prediction mode are run following the JVET CTC [3] and reported in 2 and 3.

Table 3: Proposed intra prediction mode over ECM-14.0 in All Intra configuration.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.99% | -0.84% | -0.87% | 102.7% | 108.6% | 100.7% | 100.7% |
| Class A2 | -0.40% | -0.31% | -0.30% | 102.7% | 108.1% | 102.6% | 100.9% |
| Class B | -0.45% | -0.25% | -0.25% | 102.4% | 109.8% | 101.1% | 100.6% |
| Class C | -0.47% | -0.43% | -0.38% | 99.2% | 112.7% | 101.5% | 100.9% |
| Class E | -0.74% | -0.27% | -0.30% | 101.1% | 108.7% | 99.3% | 100.8% |
| Overall | -0.58% | -0.40% | -0.40% | 101.6% | 109.8% | 101.1% | 100.8% |
| Class D | -0.55% | -0.31% | -0.77% | 99.8% | 114.1% | 100.5% | 100.9% |
| Class F | -0.28% | -0.34% | -0.31% | 100.6% | 106.2% | 97.4% | 100.7% |

Table 4: Proposed intra prediction mode over ECM-14.0 in Random Access configuration.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.33% | -0.51% | -0.52% | 98.8% | 100.0% | 100.3% | 100.8% |
| Class A2 | -0.13% | -0.14% | -0.14% | 99.3% | 99.3% | 100.3% | 99.6% |
| Class B | -0.20% | -0.06% | -0.06% | 99.1% | 99.8% | 100.2% | 100.5% |
| Class C | -0.21% | -0.21% | -0.07% | 98.8% | 100.0% | 100.4% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.22% | -0.21% | -0.17% | 99.0% | 99.8% | 100.3% | 100.4% |
| Class D | -0.21% | -0.27% | -0.62% | 98.5% | 100.2% | 101.0% | 100.8% |
| Class F | -0.01% | -0.41% | -0.10% | 99.0% | 99.8% | 100.2% | 100.5% |

# Cited References

- JVET-AH2019, “Algorithm description for neural network-based video coding (NNVC-9.1)”, F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang. 34th Meeting, Rennes, FR, 17–24 April 2024.

- JVET-AH0156, “AhG12: neural network-based intra prediction”, F. Urban, T. Dumas, F. Galpin, E. François, 34th Meeting, Rennes, FR, 17–24 April 2024.

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).