| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0252-v1 |
| --- | --- |

| Title: | AHG9: On SPTI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Tomás M. Borges Robert Skupin Cornelius Hellge Thomas Schierl | Tel: Email: | [phone contact number] [name.surname@fraunhofer.hhi.de] |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document, two issues with the SPTI SEI message are identified as follows:

- A simplification introduced at a previous meeting (conditioning presence of spti_max_sublayers_minus1 on the value of spti_persistence_flag) precludes using the current design for certain coding structures.

- Using temporally interleaved constituent pictures (frame packing arrangement type 5) leads to redundant signalling under the current design since an SPTI SEI message with spti_persistence_flag equal to 1 cannot apply to several pictures.

In order to address the first issue, it is proposed to allow indicating the temporal distance between an IDR picture and its leading picture when the spti_persistence_flag is equal to 0.

The second issue is fixed by allowing that the temporal distance indicated in a SPTI SEI message applies to each constituent picture separately, thereby enable its use with spti_persistence_flag equal to 1 to apply to several pictures when a frame packing arrangement SEI message indicates that the content is temporally interleaved stereo content with alternating first and second constituent pictures.

# Introduction

The source picture timing information (SPTI) SEI message, which is part of the draft of VSEI version 4, indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding.

Two issues have been identified in the current design of the SPTI SEI message as described in the following with proposed solutions.

## First issue

The presence of spti_max_sublayers_minus1 is conditioned on the value of spti_persistence_flag, since it was considered that when spti_persistence_flag is equal to 0, i.e., for the case that the SPTI SEI message only applies to the current picture, there is no need of signalling any information for sublayers higher than the TemporalId of the current access unit. It was motivated by the fact that it saved the 3 bits required for the syntax element spti_max_sublayers_minus1, while considering that signalling spti_max_sublayers_minus1 was unnecessary. This was based on the assumption that there is always a previous picture that has the same or a lower TemporalId than that of the current picture unit, which the SEI message applies to.

However, while reviewing the text, it has been identified that there are coding structures for which this change is undesirable. Concretely, when a CVS is encoded with the first picture unit being an IDR with associated RADL pictures (IDR_W_RADL). In such a case, there may be no picture previous to the IDR picture in output with TemporalId equal to 0, as exemplified in Figure 1.

Figure 1 – Example scenario where the temporal distance between the leading pictures and the IDR picture cannot be indicated, with the current design.

In such a case, the current design does not allow to indicate the temporal distance between the leading pictures and the IDR picture.

This seems to be an oversight, and it is asserted that although this issue does not occur for some coding structures, saving 3 bits should not result in precluding being able to indicate the temporal distance for within other coding structures.

## Second issue

When the CVS including the SPTI SEI message contains temporally interleaved stereo content with alternating first and second constituent pictures, as illustrated in Figure 2, the current design does not allow sending timing information in an SPTI SEI message that applies to more than one picture (spti_persistence_flag equal to 1). In fact, it would require sending at each and every picture an SPTI SEI message, since every second constituent frame has the same timing as the first constituent frame, i.e., their temporal distance is equal to 0.

Figure 2 – Example of temporally interleaved stereo stream indicated by a frame packing arrangement SEI message with frame_packing_arrangement_type = 5, where green and blue colors represent the different views.

# Proposed solutions

## First issue

The solution to the first problem consists of removing the conditioning of the syntax element spti_max_sublayers_minus1 on spti_persistence_flag and inferring the value of sptiMinTemporalSublayer as being equal to TemporalId when spti_persistence_flag is equal to 0. In addition, since a single value of spti_sublayer_synthesized_picture_flag is required for the case that the SPTI SEI message applies only to the current picture (spti_persistence_flag is equal to 0), in such case, spti_sublayer_synthesized_picture_flag is present only for the temporal level equal to TemporalId.

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_source_type_present_flag | u(1) |
| if( spti_source_type_present_flag ) |  |
| spti_source_type | u(16) |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_interval | u(32) |
| if( spti_persistence_flag ) |  |
| spti_max_sublayers_minus1 | u(3) |
| for( i = sptiMinTemporalSublayer; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_interval_scale_factor[ i ] | ue(v) |
| if(spti_persistence_flag ¦¦ i == sptiMinTemporalSublayer) |  |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

…

spti_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers for which picture interval scale factor (spti_sublayer_interval_scale_factor[ i ]) and synthesized flag (spti_sublayer_synthesized_picture_flag[ i ]) information is signalled. When spti_max_sublayers_minus1 is not present, it is inferred to be equal to TemporalId. The value of spti_max_sublayers_minus1 shall be equal to or greater than TemporalId of the SPTI SEI message.

The variable sptiMinTemporalSublayer is set to as follows:

–	If spti_persistence_flag is equal 1, sptiMinTemporalSublayer is equal to 0.

–	Otherwise, sptiMinTemporalSublayer is equal to spti_max_sublayers_minus1TemporalId.

## Second issue

Two alternatives are proposed in the following to solve the second issue, and allow using an SPTI SEI message with spti_persistence_flag equal to 1:

- Explicit signaling that the SPTI message applies to each constituent picture separately.

- Specifying that by the presence of a frame packing SEI message with fp_arrangement_type equal to 5 (temporal interleaving of constituent pictures as coded pictures), the SPTI message applies to each constituent picture separately.

Option 1:

| source_picture_timing_info( payloadSize ) { | Descriptor |
| --- | --- |
| spti_cancel_flag | u(1) |
| if( !spti_cancel_flag ) { |  |
| spti_persistence_flag | u(1) |
| spti_source_timing_equals_output_timing_flag | u(1) |
| if( !spti_source_timing_equals_output_timing_flag ) { |  |
| spti_constituent_picture_timing_flag | u(1) |
| spti_source_type_present_flag | u(1) |
| if( spti_source_type_present_flag ) |  |
| spti_source_type | u(16) |
| spti_time_scale | u(32) |
| spti_num_units_in_elemental_interval | u(32) |
| if( spti_persistence_flag ) |  |
| spti_max_sublayers_minus1 | u(3) |
| for( i = sptiMinTemporalSublayer; i <= spti_max_sublayers_minus1; i++ ) { |  |
| spti_sublayer_interval_scale_factor[ i ] | ue(v) |
| spti_sublayer_synthesized_picture_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

spti_constituent_picture_timing_flag equal to 1 specifies that the source picture timing information, in this SEI message applies to each constituent picture separately.

When the value of spti_constituent_picture_timing_flag is equal to 1 for a SPTI SEI message associated to a picture, there shall be a frame packing arrangement SEI message with fp_arrangement_type equal to 5 associated with that picture.

Option 2:

## 8.32.2 Source picture timing information SEI message semantics

The source picture timing information (SPTI) SEI message indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding. For example, for camera-captured content, the temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order. The information provided by the SPTI SEI message pertains only for picture(s) starting from the picture in the current layer in the access unit that contains the SPTI SEI message and all subsequent pictures of the current layer in output order based on its persistence.  [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.]. When a frame packing arrangement SEI message with fp_arrangement_type equal to 5 (temporal interleaving of constituent pictures as coded pictures) applies to the decoded pictures, the source picture timing information applies to each constituent picture separately.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).