| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0254-v1 |
| --- | --- |

| Title: | AHG8: Pre-analysis based adaptive temporal resampling algorithm for machine vision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang Jie Chen Yan Ye Binzhe Li (Alibaba Group) Shiqi Wang (City University of Hong Kong) | Tel: Email: | shurun.wsr, jiechen.cj, yan.ye, libinzhe.lbz,}@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Alibaba Group, City University of Hong Kong |  |  |

_____________________________

- Abstract

This contribution proposes a pre-analysis based adaptive temporal resampling algorithm for machine vision. According to the results of the experiments conducted under common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content, the proposed method achieves 3.13% (RA) and 18.45% (LD) BD-rate savings for object detection task on SFU-HW dataset, and 5.64% (RA) and 31.25% (LD) BD-rate savings for object tracking task on TVD dataset.

Algorithm description

Temporal resampling framework

Figure 1 shows the coding framework with the proposed pre-analysis based adaptive temporal resampling. The video data {Fi} is first input into a pre-analysis module to determine the resampling ratio f and then a temporal down-sampling module is applied on the input data {Fi} according to the resampling ratio f (i.e., removing f-1 frames per f frames). The down-sampled data {Fi}D is then encoded and decoded with VVC codec. After decoding, the decompressed data {Fi}D' is up-sampled by a temporal up-sampling module by duplicating each frame f-1 times. Finally, we get reconstructed up-sampled video data  {Fi}' and feed it to the machine analysis task. The resampling ratio f can be signaled using the encoder optimization information (EOI) SEI message [1]. The value of syntax elements can be set as in Table 1.

Table 1 An example setting of the syntax element values of EOI SEI for proposed temporal resampling

| Syntax | Value | Syntax | Value |
| --- | --- | --- | --- |
| eoi_cancel_flag | 0 | eoi_type | 0x02 |
| eoi_persistence_flag | 1 | eoi_temporal_resampling_type_flag | 0 |
| eoi_for_human_viewing_idc | 0 | eoi_num_int_pics | f-1 |
| eoi_for_machine_analysis_idc | 3 |  |  |

Figure 1 The pre-analysis based adaptive temporal resampling compression framework

Pre-analysis based adaptive algorithm

Motivated by the diversity of video content, the pre-analysis based adaptive temporal resampling algorithm proposed in this contribution is based on the temporal complexity for machine vision.

First, a metric named mean intersection of union (MIOU) is introduced. There are two MIOUs in this contribution. The first one is frame level MIOU, denoted as Mf, and the second one is the sequence level MIOU, denoted as Ms. The sequence level MIOU is derived based on the frame level MIOU. In different configurations, such as random access (RA) and low delay (LD), the calculation methods of Ms are different. The derivation of these two MIOUs is described as follow.

Frame-level MIOU Mf

Given a frame pair (F1,F2), object detection is conducted on 2 frames. The detection results of each frame are noted as B1={(x1,i,y1,i,w1,i,h1,i,c1,i,s1,i)} and B2={(x2,i,y2,i,w2,i,h2,i,c2,i,s2,i)} respectively, where i denotes the index of the bounding boxes. Herein, x and y are the horizontal and vertical coordinates of the top-left corner of the bounding box, w and h are the width and height of the bounding box, c is the class index of the bounding box and s is the confidence score in range from 0 to 1. Herein, the boxes with confidence score under 0.5 are removed. By comparing the intersection of union (iou) for the bounding boxes in B1 and B2, the matched box pairs can be achieved, noted as P={(x1,i,y1,i,w1,i,h1,i,c1,i,s1,i;x2,i,y2,i,w2,i,h2,i,c2,i,s2,i;ioui)}, where c1,i=c2,i.

In order to distinguish the temporal complexity difference of various object scales, the frame-level MIOU is measured at 3 spatial scales. Based on the definition in [2], the paired bounding boxes are classified into 3 groups: small group (object area is smaller than 32×32), medium group (object area is smaller than 96×96 and larger than 32×32) and large group (object area is larger than 96×96), noted as Pk, k=s, m, l for small, medium and large groups respectively. For each spatial group Pk, the bounding boxes are grouped based on object class indexes, noted as Pk=s1,k,i,j;s2,k,i,j;iouk,i,jj, j=1, 2, …, nck, where nck is the number of object class indexes in Pk. The frame-level MIOU of group Pk is defined as Mf,k=jnckiouk,jnck, where iouk,j=igjiouk,i,j(s1,k,i,j+s2,k,i,j)igj(s1,k,i,j+s2,k,i,j), indicating the IOU value of class index j in group k, and gj is the number of detected results for class index j. On this basis, the frame-level MIOU is defined as Mf=(Mf,k)=(Mf,s,Mf,m,Mf,l). Specifically, the bounding box number for small, medium and large groups are represented with ns, nm, nl. For object tracking task, only the meaningful and moving object classes would be considered, such as person, car and bus, etc.

Sequence-level MIOU Ms

The sequence-level MIOU Ms is calculated based on frame-level MIOU Mf. For simplicity, only the first 33 frames of video sequence are calculated. The sequence-level MIOU is coding configuration dependent, in order to accommodate with the coding structure.

For RA, if resampling ratio is 2, the frame pairs are selected with interval 8, 16 and 32. The frame pairs are {F0,F8,F8,F16,F16,F24,F24,F32interval 8,  F0,F16,F16,F32interval 16,F0,F32interval 32}. For each interval h, the averaged frame-level MIOU can be calculated, noted as {Mf,8,Mf,16,Mf,32}. The sequence-level MIOU for resampling ratio 2 is Ms=0.5Mf,32+0.3Mf,16+0.2Mf,8. Similarly, if resampling ratio is 4, the frame pairs are selected with interval 16 and 32 (i.e., {F0,F16,F16,F32interval 16,F0,F32interval 32}) and sequence-level MIOU for resampling ratio 4 is Ms=0.625Mf,32+0.375Mf,16.

For LD, if resampling ratio is 2, the frame pairs are selected with interval 8 (i.e., {F0,F8,F8,F16,F16,F24,F24,F32})and sequence-level MIOU is Ms=Mf,8. If resampling ratio is 4, the frame pairs are selected with interval 16 (i.e., {F0,F16,F16,F32}) and sequence-level MIOU is Ms=Mf,16.

The sequence-level MIOU Ms is a three-dimensional vector, noted as Ms=(Ms,s,Ms,m,Ms,l).

Adaptive temporal resampling ratio decision

For sequence-level MIOU Ms, the decision metric MD is defined as (nsMs,s+nmMs,m+nlMs,l)(ns+nm+nl). Here, the sum of confidence score for Pk is nk,  nk=jnckigj(s1,k,i,j+s2,k,i,j), k=s, m, l. Given a video sequence and coding configuration, the decision metric for resampling ratio 2 and 4 are calculated, noted as MD,2 and MD,4. The decision algorithm is as follow.

If MD,2>0.5 and MD,4>0.6, resampling ratio is 4.

If MD,2>0.5 and MD,4≤0.6, resampling ratio is 2.

Otherwise, resampling ratio is 1. Temporal resampling is not performed.

Performance

We evaluate the performance of the proposed pre-analysis based adaptive temporal resampling algorithm following the common test conditions specified in JVET-AI2031 [3]. The overall performance for detection and tracking is shown in Table 2.

Table 2 Overview of BD-rate results under VCM CTC

| Task | Dataset | RA (Mono-mAP) | LD (Mono-mAP) |
| --- | --- | --- | --- |
| Object Detection | SFU-HW (Average) | -3.13% | -18.45% |
| Object Tracking | TVD (Average) | -5.64% | -31.25% |

The per-class performance on SFU-HW dataset for object detection is shown in Table 3 and the per-sequence performance on TVD dataset for object tracking is given in Table 4. (Please note “/” in the tables indicates the performance cannot be calculated due to the nonmonotonic machine task performance). The results show that the proposed algorithm achieves significant performance improvement under CTC.

Table 3 Performance of post-processing algorithm on SFU-HW dataset under RA and LD configurations

| SFU-HW | Metric | BD-rate (%) |  | BD-mAP |
| --- | --- | --- | --- | --- |
| Config | Class Name | Mono-mAP | mAP |  |
| RA | Class A | 0.00% | 0.00% | 0.00 |
|  | Class B | -4.28% | / | 1.20 |
|  | Class C | 1.61% | -0.06% | -0.31 |
|  | Class D | -7.49% | -6.18% | 1.01 |
|  | Average | -3.13% | / | 0.59 |
| LD | Class A | -12.73% | -12.57% | 2.63 |
|  | Class B | -17.14% | / | 5.38 |
|  | Class C | -14.60% | -15.06% | 2.65 |
|  | Class D | -25.03% | -23.15% | 4.19 |
|  | Average | -18.45% | / | 3.96 |

Table 4 Performance of post-processing algorithm on TVD dataset under RA, LD and AI configurations.

| TVD | Metric | BD-rate (%) |  | BD-MOTA |
| --- | --- | --- | --- | --- |
| Config | Sequence Name | Mono-MOTA | MOTA |  |
| RA | TVD-01-1 | 0.00% | / | 0.00 |
|  | TVD-01-2 | 0.00% | 0.00% | 0.00 |
|  | TVD-01-3 | -8.24% | / | 0.79 |
|  | TVD-02-1 | -2.48% | / | -0.62 |
|  | TVD-03-1 | -4.89% | -2.44% | 0.60 |
|  | TVD-03-2 | -26.24% | / | 2.05 |
|  | TVD-03-3 | 2.40% | -1.39% | -0.47 |
|  | Average | -5.64% | / | 0.34 |
| LD | TVD-01-1 | -13.44% | / | 1.46 |
|  | TVD-01-2 | -33.47% | -33.99% | 3.09 |
|  | TVD-01-3 | -42.49% | -46.17% | 4.16 |
|  | TVD-02-1 | -38.06% | / | 1.57 |
|  | TVD-03-1 | -23.12% | -24.69% | 3.99 |
|  | TVD-03-2 | -41.75% | -40.53% | 3.92 |
|  | TVD-03-3 | -26.38% | -27.33% | 2.07 |
|  | Average | -31.25% | / | 2.89 |

Conclusion

A pre-analysis based adaptive temporal resampling algorithm is proposed in this contribution to improve the coding efficiency of video coding for machines. Simulation shows that proposed algorithm is effective to improve compression performance of VVC under various coding configurations. Thus, it is suggested to include the implementation in AHG8 software branch. It is also suggested to add the following highlighted sentence to section 7.3 of the next draft of technical report (TR) on optimization of encoders and receiving systems for machine analysis of coded video content and include the algorithm description (section 1) to the annex of TR.

“In some use cases, for example when the frame rate is high, a way to reduce the bit rate without a strong negative impact on the machine consumption performance is to skip certain frames and encode the video at a lower frame rate. One example is to remove every other frame from the input video and encode the video at half frame rate. This can be done in a dynamic manner, for example by evaluating the motion between two or more frames (e.g., evaluating the mean intersection of union (MIOU) based on an object detection pre-analysis module) and if there is only little motion, a frame can be removed. Motivated by dynamic motion of various sequences, the frame resampling ratio can be different and the resampling ratio can be signaled using the encoder optimization information (EOI) SEI message. In some cases, if the receiving system requires a specific frame rate, a corresponding post-processing technology that up-samples to full frame rate needs to be applied.”

References

J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang, “Additional SEI messages for VSEI version 4 (Draft 3),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AI2006, 35th Meeting, Sapporo, JP, 12–19 July 2024 (available online at https://jvet-experts.org/).

Lin, Tsung-Yi, et al. "Microsoft coco: Common objects in context." Computer Vision–ECCV 2014: 13th European Conference, Zurich, Switzerland, September 6-12, 2014, Proceedings, Part V 13. Springer International Publishing, 2014.

S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AI2031, 35th Meeting, Sapporo, JP, 12–19 July 2024 (available online at https://jvet-experts.org/).

Patent rights declaration(s)

Alibaba Group and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).