| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0264-v1 |
| --- | --- |

| Title: | Non-EE2: Multiple transform set selection for LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

The MTSS (multiple transform set selection) method allows CUs coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP to select one LFNST/NSPT transform set out of 2 candidate sets. When the current block is coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP in combination with LFNST/NSPT, one more bin is employed to indicate whether the 1st or the 2nd candidate transform set is selected. In this contribution, it is proposed to apply MTSS to only LFNST coded blocks.

On top of ECM-14.0, simulation results are reported as follows:

AI {-0.08 % Y, -0.01 % U, -0.01 % V, 100.8 % EncT, 100.7 % DecT}

RA {-0.06 % Y, -0.05 % U, 0.04 % V, 100.7 % EncT, 99.9 % DecT}

Introduction

The multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT was proposed in JVET-AG0062 [1]. The proposed method was further investigated in AG EE2-4.1 and AH EE2-4.4. The proposed MTSS method allows CUs coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP to select one LFNST/NSPT transform set out of 2 candidate sets. When the current block is coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP in combination with LFNST/NSPT, one more bin is employed to indicate whether the 1st or the 2nd candidate transform set is selected.

Proposed method

It is proposed to apply MTSS to only LFNST coded blocks, not NSPT coded blocks. In Figure 1, the blocks using NSPT are marked in gray, the blocks using LFNST are marked in green.

Compared with the previous MTSS EE, encoding fast algorithms are applied to MTSS.

Figure 1

Simulation results

The proposed method is implemented on top of the ECM-14.0 software [2] and tested based on the common test condition [3]. The test results are summarized below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.18% | -0.06% | -0.11% | 102.0% | 100.5% | 99.3% | 100.2% |
| Class A2 | -0.07% | 0.01% | -0.03% | 100.6% | 100.4% | 100.2% | 100.2% |
| Class B | -0.06% | 0.09% | 0.00% | 100.7% | 100.4% | 101.6% | 100.1% |
| Class C | -0.03% | -0.06% | -0.09% | 100.4% | 101.5% | 100.5% | 100.0% |
| Class E | -0.07% | -0.08% | 0.18% | 100.3% | 100.6% | 100.9% | 99.9% |
| Overall | -0.08% | -0.01% | -0.01% | 100.8% | 100.7% | 100.6% | 100.1% |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | -0.39% | -0.13% | 101.1% | 100.0% | 100.1% | 100.0% |
| Class A2 | -0.02% | 0.03% | -0.02% | 100.7% | 99.7% | 100.7% | 100.1% |
| Class B | -0.03% | 0.06% | 0.07% | 100.7% | 100.0% | 100.1% | 100.1% |
| Class C | -0.07% | 0.01% | 0.15% | 100.5% | 100.0% | 100.1% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.05% | 0.04% | 100.7% | 99.9% | 100.2% | 100.1% |
| Class D |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Conclusions

It is recommended to investigate the proposed method in EE.

References

- ECM-14.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-14.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AI2017, July 2024

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).