| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0279-v1 |
| --- | --- |

| Title: | AHG8: Joint teleconference with WG 4 AHG on Video Coding for Machines |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Shan Liu Chris Rosewarne | Email: | shanl@global.tencent.com Chris.Rosewarne@canon.com.au |
| Source: | AHG co-chairs |  |  |

_____________________________

# Abstract

A teleconference meeting was held on September 20th to discuss potential improvement and alignment of common test conditions, as well as procedure for performance evaluation and comparison. Experts from JVET and WG4 attended this meeting.

# Meeting notes

# CTC study

## m69437 [VCM][CTCS-related] On VCM CTC configuration [Junxi Zhang, Zhenzhong Chen (Wuhan University)]

Reviewed in the AhG call on September 20, 2024.

This contribution proposes to align settings between the e2e and inner configs as much as possible, and asserts that the current definitions include differences that seem not well justified.

RoiAccumulationWindow, RoIAccumulationWindowExtension in LD_inner would be set to 1,32. RoI retargeting mode would be “dynamic+pad” for all six configs.

RoiRegargeingMaxNumRoIs would be set to 11 across all configs.

Support was expressed for generally increasing the amount of alignment across different configs but commented that changes in RoI accumulation window were set based on config requirements. Regarding the dynamic pad and dynamic fit, the motivation for each of these was described when they were proposed. Essentially the difference was due to specifics of the AI mode, for which ‘dynamic+fit’ is preferable. The “dynamic+pad” introduces a lot of gray region in the frame which is not handled well by AI (coded for every frame).

It was noted that at present no results had been included in the contribution. However, the RoiRetargetingMaxNumRoIs arguably could be set to a constant value (perhaps 11 but that was selected for the ‘inner’ scenarios) across all configs.

If dynamic+fit were used in the RA/LD, the downside would be sending a larger image size than necessary.

It was commented that intra block copy might help with the gray area issue in the dynamic+fit mode if used in AI.

It was commented that these settings could be tested in CE0 if the group agrees, and that selecting which ones would be tested would be the first step as there are many.

It was commented that increasing alignment across configs should not be at the expense of coding efficiency.

It was suggested that the table of modifications would be a good input to this study and the amount of tested combinations could be driven by available simulation bandwidth.

Recommendation: CE0 to test config modifications based on m69437 (discretion for finalizing exact modifications to be tried is given and amount of testing is best effort).

# Activity comparison

## m69442 [VCM] Suggestion on VCM/FCM/AhG8 activity comparison [C. Rosewarne, R. Nguyen (Canon)]

Reviewed in the AhG call held on September 20, 2024.

This document presents a suggested plan for lining up test conditions between WG4 VCM and FCM activities, and WG5 (JVET) AHG8 activities. The evaluation methodologies of these three projects are observed similar, but not entirely identical w.r.t. datasets, QP points and configurations. It was identified that the difference between WG4/VCM and WG5/AhG8 is relatively small, however as WG5/AhG8 uses VTM as the anchor and the software branches hosted by AHG8 git are on volunteer basis (i.e. some of them are not adopted or crosschecked) it is not obvious which software should be used for performance comparison. One expert suggested to use the most stable and clean branch of WG5/AhG8 for performance evaluation, however it was countered by the argument that none of the branch at this moment represents "TR adoptions" and further study is needed. Regarding the WG4/FCM it was reported that further work needs to be done on inner codec configuration for AI, LD (conventional, no 1s IP), and RA. WG4/FCM experts reported that they are going to continue this work and WG4/VCM experts volunteered to help with cross-checking when the configuration is ready.

Recommendations:

1) Encourage to continue this study.

2) WG5/AhG8 is suggested to work on software for the performance evaluation and comparison.

3) WG4/FCM is suggested to continue the work on producing FCM reference point following the proposed plan.

4) WG4/VCM is suggested to cross-check the FCM reference points produced by WG4/FCM when it is finished.