| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0283-v1 |
| --- | --- |

| Title: | Crosscheck of Test 3.4a and Test 3.4e in JVET-AJ0107 (EE2-3.4: GPM mode extension) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ke Jia | Tel: Email: | feijia.jk@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of Test 3.4a and Test3.4e in JVET-AJ0107 (EE2-3.4), where the number of GPM partitioning modes is extended with the additional angles and distances. It is confirmed that the implementation matches the proposal and the partial results match with the proponent’s.

Introduction

The methods are implemented on top of ECM-14.0 and are tested under CTC for enhanced compression tool testing with random access and lowdelay B configurations.

Table 1 Crosscheck results of Test 3.4a vs. ECM14.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.20% | -0.10% | 102.5% | 100.5% | 101.4% | 100.2% |
| Class A2 | -0.04% | 0.05% | -0.01% | 102.4% | 101.4% | 101.5% | 99.8% |
| Class B | -0.02% | -0.04% | 0.03% | 101.8% | 100.7% | 103.5% | 99.8% |
| Class C | -0.05% | -0.10% | 0.14% | 102.7% | 101.0% | 108.8% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.07% | 0.02% | 102.3% | 100.9% | 104.0% | 99.9% |
| Class D | -0.03% | -0.18% | -0.18% | 102.0% | 101.7% | 112.1% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.12% | -0.48% | 103.6% | 101.8% | 103.8% | 99.8% |
| Class C | 0.02% | -0.03% | 0.17% | 103.4% | 101.5% | 109.9% | 100.1% |
| Class E | 0.08% | 0.43% | -0.22% | 103.1% | 100.6% | 107.5% | 99.9% |
| Overall | 0.03% | 0.05% | -0.20% | 103.4% | 101.4% | 106.7% | 100.0% |
| Class D | 0.15% | 0.84% | 0.22% | 103.6% | 100.8% | 112.8% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Table 2 Crosscheck results of Test 3.4e vs. ECM14.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.18% | -0.07% | 101.7% | 101.3% | 101.4% | 100.3% |
| Class A2 | -0.11% | 0.05% | -0.10% | 101.6% | 101.5% | 101.5% | 100.1% |
| Class B | -0.06% | -0.02% | -0.05% | 101.1% | 100.5% | 103.6% | 99.8% |
| Class C | -0.08% | -0.03% | 0.04% | 101.3% | 100.9% | 108.8% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.04% | -0.04% | 101.4% | 101.0% | 104.1% | 100.0% |
| Class D | 0.01% | -0.02% | 0.12% | 100.9% | 100.9% | 111.9% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.06% | 0.06% | 0.23% | 102.1% | 100.8% | 109.8% | 99.9% |
| Class E | -0.13% | 0.02% | -0.48% | 101.0% | 100.1% | 107.5% | 99.8% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.07% | 0.40% | 0.34% | 101.8% | 100.4% | 112.7% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).