| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0291-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0056 (EE1-4.1 – Wavelet transform for super-resolution loss function) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chaoyi Lin | Email: | linchaoyi.cy@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for EE1-4.1.2, which uses wavelet transform in loss funtion for training super resolution. Both training and inference crosschecks are succeed. For training crosscheck, the model trained by crosschecker shows maximum 0.03% luma coding performance difference compared to the results provided by JVET-AJ0056. For inference crosscheck, the results exactly match with JVET-AJ0056.

# Introduction

This document reports the crosscheck results of EE1-4.1.2 (JVET-AJ0056), which proposed to use wavelet transform in loss function when training super resolution model. The DWT transform of the ground truth image and the SR image output from the network is conducted before calculating the loss.

# Training crosscheck

For EE1-4.1.2, crosschecker trained the model from scratch with the training scripts provided by proponents. The training data is generated with LOP2 and NNintra enabled. Both 2x and 1.5x data are generated and used for training. The model is trained with 60 epochs. After training, the model is quantized to integer model for inference.

The training crosscheck results are presented in the following tables.

Table 6 Results of EE1-4.1.2 in RA configurations

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -3.02% | 1.78% | 2.83% |
| Class A2 | -0.78% | 8.13% | 7.88% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| Overall | -0.76% | 1.98% | 2.14% |

Table 7 Results of EE1-4.1.2 in AI configurations

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -1.67% | 4.64% | 4.52% |
| Class A2 | -3.09% | 10.76% | 10.80% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| Overall | -0.79% | 2.57% | 2.55% |

# Inference crosscheck

Table 6 inference crosscheck results under RA configurations

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -3.18% | 2.77% | 4.03% |
| Class A2 | -0.79% | 8.18% | 7.78% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E |  |  |  |
| Overall | -0.79% | 2.19% | 2.36% |

Table 7 inference crosscheck results under AI configurations

|  | Y-PSNR | U-PSNR | V-PSNR |
| --- | --- | --- | --- |
| Class A1 | -1.79% | 5.81% | 5.39% |
| Class A2 | -3.15% | 11.38% | 11.20% |
| Class B | 0.00% | 0.00% | 0.00% |
| Class C | 0.00% | 0.00% | 0.00% |
| Class E | 0.00% | 0.00% | 0.00% |
| Overall | -0.82% | 2.86% | 2.76% |

# Conclusion

Training and inference crosscheck were performed. Training crosscheck is passed where minor difference is observed during training crosscheck. Besides, the results of inference crosschecks are totally matched.