| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ0299-v3 |
| --- | --- |

| Title: | AHG12: IntraTMP improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | M. Radosavljević, F. Le Léannec, K. Naser, T. Dumas | Tel: Email: | milos.radosavljevic@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes several modifications to Intra TMP. First, the Intra TMP merge mechanism is enriched with the use of HMVP candidates. Second, maximum distance for non-adjacent merge candidates is increased. Thirdly, CABAC context model to signal LIC flag is enriched.

The proposed aspects reportedly lead to the following performances over ECM-14.0.

- AI (Y / Cb / Cr) : -0,08% / -0,07% / -0,03%	102,3%	(EncT) 103,4% (DecT)

# Proposed Method

The following Intra TMP modification are introduced on top of ECM-14.0.

HMVP merge mode in Intra TMP

When constructing Intra TMP merge BV candidates, block vectors from HMVP list are used. HMVP candidates are added after spatial (local and non-local) candidates and before the ARBVP candidate construction. Up to 25 candidates from HMVP list are added to the IntraTMP merge candidates.

Increased distance for non-adjacent Intra TMP merge candidates

The distance for non-adjacent spatial merge candidates is increased from 4 to 7, see Figure 1. Specifically, up to 7 rows/columns of spatial neighboring blocks (in terms of the CU height/width) of the current CU can be used in Intra TMP merge BV candidate construction. This is achieved for camera captured content only. Total number of BV candidates is limited to 92.

Figure 1: Adjacent and non-adjacent spatial merge candidates.

Improved CABAC context for Intra TMP-LIC flag signaling

A second CABAC context is introduced for the signaling of Intra TMP LIC flag. That is, 2 contexts are used instead of 1 as in ECM-14.0. The context used for signaling LIC flag is chosen conditionally to the value of Intra TMP fusion flag.

# Experimental Results

The proposed aspects were tested over ECM-14.0, according to ECM common test conditions [1]. Following Table 1 shows the performances of proposed Intra TMP modifications, over ECM-14.0.

Table 1: performance of proposed Intra TMP modification over ECM-14.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,03% | 0,02% | -0,05% | 101,5% | 101,3% | 99,0% | 101,1% |
| Class A2 | -0,07% | -0,02% | 0,00% | 101,9% | 102,6% | 101,4% | 103,3% |
| Class B | -0,09% | -0,05% | 0,00% | 101,7% | 102,8% | 100,3% | 100,2% |
| Class C | -0,08% | -0,10% | -0,12% | 102,9% | 105,4% | 100,8% | 100,1% |
| Class E | -0,12% | -0,21% | 0,03% | 103,7% | 104,8% | 103,4% | 100,0% |
| Overall | -0,08% | -0,07% | -0,03% | 102,3% | 103,4% | 100,9% | 100,8% |
| Class D | -0,04% | 0,03% | -0,20% | 102,4% | 104,2% | 99,8% | 100,0% |
| Class F | -0,14% | -0,13% | -0,05% | 100,9% | 102,2% | 98,7% | 100,0% |
| Class TGM | -0,08% | -0,08% | -0,13% | 100,7% | 102,0% | 99,9% | 100,2% |

# Conclusion

This contribution proposes several modifications to Intra TMP, which are HMVP merge candidates usage, increase non-adjacent merge candidates maximum distance, and enriched CABAC context to signal LIC flag.

The proposed aspects lead to -0.08% coding gain over ECM-14.0. It is proposed to study the proposed method in next round of JVET EE2.

# Cited References

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).