| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0306-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0110 (EE2-3.1: Dual merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fangjun Pu | Tel: Email: | fpu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

In EE2 Test 3.1, the dual merge mode is evaluated. In the dual merge mode, a dual merge candidate list is constructed. Each dual merge candidate in the list has two motion information which is derived from previous coded blocks, and each motion information can be either uni-predicted or bi-predicted motion. The prediction for this mode is generated by averaging the two predictions which are separately predicted using the two motion information. This contribution reports the crosschecking results of EE2 Test 3.1. It is confirmed that the implementation matches the proposal and crosschecking results for random access and low delay B configurations match with the proponent’s test results.

# Introduction

In ECM, only one motion information of the merge modes, which comprises inter prediction direction, motion vectors, reference picture indices, BCW index, LIC parameters and MHP information, is inherited from previous coded blocks. To improve the prediction efficiency, a dual merge mode which inherits two motion information from previous coded blocks was proposed and studied in EE2 Test 3.1.

For a CU coded in merge mode, a dual merge flag is signaled after the regular merge flag to indicate whether the proposed dual merge mode is used to predict the CU. When the CU is coded in dual merge mode, a dual merge list is constructed as follows:

- derive dual merge candidates from spatial, temporal and non-adjacent spatial neighboring blocks if the neighboring blocks are coded in the dual merge mode;

- if the dual merge list is not full, construct a regular merge list using a process similar to that of the regular merge mode, and candidates in the regular merge list are paired and added to the dual merge list;

- perform ARMC to dual merge candidates.

Each dual merge candidate in the dual merge list has two motion information, and each motion information can be either uni- or bi-predicted motion information. An index is signaled to indicate which one of dual merge candidates is selected. The two selected motion information are used to generate two prediction blocks using the same motion compensation process as normal inter modes. Then, the two prediction blocks are averaged to generate the final prediction for the CU. To store the two motion information for the dual merge coded blocks, the size of motion storage buffer is doubled. When deriving merge candidates from a dual merge coded block for other merge modes, the two motion information are treated as two separate merge candidates. In addition, both motion for two GPM partitions are stored for the blending area of GPM mode. It is noted that the MHP, DMVR, BDOF, LIC and OBMC are not applied to the dual merge mode.

# Crosscheck results

EE2 Test 3.1 is implemented on top of ECM-14.0. This crosscheck report evaluates the performance of EE2 Test 3.1 with random access and low delay B configurations under the common test conditions (CTC) and evaluation procedures for ECM. Results are provided in Table 1 and Table 2.

Table 1. Crosscheck results over ECM-14.0 for random access configuration

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.12% | -0.15% | 100.7% | 101.7% | 103.5% | 103.9% |
| Class A2 | 0.00% | 0.07% | -0.01% | 101.2% | 102.1% | 103.6% | 103.8% |
| Class B | -0.06% | -0.10% | 0.02% | 100.8% | 101.7% | 102.2% | 102.2% |
| Class C | -0.03% | 0.07% | 0.14% | 101.5% | 100.9% | 101.3% | 102.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.03% | 0.01% | 101.1% | 101.6% | 102.5% | 102.9% |
| Class D | -0.02% | -0.15% | 0.04% | 100.3% | 101.1% | 100.9% | 100.6% |
| Class F | -0.03% | -0.10% | -0.04% | 101.0% | 101.9% | 101.5% | 100.9% |

Table 2. Crosscheck results over ECM-14.0 for low delay B configuration

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.07% | 0.12% | -0.07% | 100.4% | 100.6% | 100.9% | 101.0% |
| Class E | -0.04% | 0.59% | -0.19% | 100.1% | 100.1% | 100.0% | 101.7% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.47% | 0.50% | 101.4% | 100.3% | 100.8% | 101.0% |
| Class F | 0.26% | 0.31% | 0.57% | 100.1% | 100.8% | 100.5% | 100.5% |

# Patent rights declaration(s)

Dolby Laboratories, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.