| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ00309 |
| --- | --- |

| Title: | EE1 related: EE1-2.3 Block based QP information in NN loop filters additional results |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Franck Galpin, Tangi Poirier Thierry Dumas Philippe Bordes | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This document reports additional results to EE1-2.3. It shows the impact of using a dataset composed of both flat QP and adaptive QP per block during the training stage. The training process has been adapted to be similar to the one of HOP5. On top of NNVC-10 using HOP5 model, the following results are reported compared to using the QP slice as an input:

- {xxx} for YUV in RA configuration.

- {-xxxx} for YUV in AI configuration.

# Introduction

In NNVC, several loop filters and post-filters use the QP slice information. However, in real encoding, adaptive QP is often used to control the quality inside the slice and allow an increase of the subjective quality. We report the results of using a model trained on a dataset containing both flat QP and adaptive QP per block.

# Simulation

## Training description

A new dataset was created by merging the official HOP dataset and the dataset described in JVET-AJ0124. The same process as in CTC for training HOP is used but the dataset is twice as large, the number of epochs for training has been cut in half (10n epochs in total). The model is called HOP5-AQ+Flat in the following.

## HOP5 results

### AQ mode

The inference is performed on top of NNVC where the following options are used “--AdaptiveQP -aqr 12 --MaxCuDQPSubdiv=2”.

#### NNVC-10.0 HOP5 vs NNVC-10.0 HOP5-AQ+flat QP [AQ mode]

The model is compared to the one trained only with flat QP (anchor NNVC-10).

|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #DIV/0! |
| Class B | -1.06% | -0.20% | 0.06% | -1.40% | -0.01% | 0.91% | 108% | #NUM! |
| Class C | -0.73% | -0.49% | -0.21% | -1.50% | -0.17% | -0.16% | 107% | 115% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | -0.65% | -0.51% | -0.82% | -1.51% | -1.44% | -2.13% | 106% | 114% |
| Class F | -0.63% | -1.10% | -0.80% | -0.66% | -0.34% | -0.94% | 111% | #NUM! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class B | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1.30% | -0.78% | -0.52% | -1.25% | -1.06% | -0.94% | 102% | #NUM! |
| Class F | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |

#### NNVC-10.0 HOP5-AQ vs NNVC-10.0 HOP5-AQ+flat QP [AQ mode]

The model is compared to the one trained only on AQ mode.

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT |  | DecT CPU |  |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |  | #DIV/0! |  |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! |  | #DIV/0! |  |
| Class B | -0.11% | -1.31% | -0.11% | -0.03% | -1.28% | 0.83% | 100% |  | #NUM! |  |
| Class C | -0.09% | -1.08% | -1.10% | 0.05% | -1.42% | -1.34% | 101% |  | 102% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |  | #DIV/0! |  |
| Class D | -0.23% | -0.97% | -0.57% | -0.21% | -1.13% | -1.40% | 100% |  | 101% |  |
| Class F | -0.17% | -1.37% | -0.58% | -0.01% | -2.96% | -1.08% | 100% |  | #NUM! |  |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |  |
| Class A1 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |  |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |  |
| Class B | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |  |
| Class C | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |
| Class E | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |
| Class D | 0.01% | -0.74% | -0.24% | 0.07% | -0.99% | -0.52% | 102% | #NUM! |  |
| Class F | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |  |

### Flat mode

The inference is performed on top of NNVC using CTC configuration.

#### NNVC-10.0 HOP5 vs NNVC-10.0 HOP5-AQ+flat QP [flat mode]

The model is compared to the one trained only with flat QP (anchor NNVC-10).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #NUM! |
| Class B | 0.06% | 0.12% | 0.26% | -0.22% | 0.30% | 0.90% | 73% | #NUM! |
| Class C | 0.02% | -0.79% | -0.28% | -0.46% | -0.47% | -0.19% | 109% | #NUM! |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.20% | -0.29% | -0.78% | -0.12% | -1.23% | -1.89% | 107% | 123% |
| Class F | 0.10% | -0.54% | -0.48% | -0.08% | -0.23% | -0.81% | 73% | #NUM! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.15% | 0.00% | 0.29% | 0.08% | 0.11% | 0.34% | 101% | 117% |
| Class E | 0.15% | 0.70% | 0.66% | 0.11% | 0.32% | 1.03% | 105% | 116% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.09% | -0.56% | -0.54% | 0.08% | -0.80% | -0.85% | 100% | 115% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

#### NNVC-10.0 HOP5-AQ vs NNVC-10.0 HOP5-AQ+flat QP [AQ mode]

The model is compared to the one trained only on AQ mode.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class A2 | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | 100.00% | #NUM! | #DIV/0! |
| Class B | -0.19% | -1.05% | -0.54% | -0.13% | -0.82% | 0.30% | 79% | #NUM! |
| Class C | -0.22% | -1.00% | -1.07% | -0.12% | -1.55% | -1.34% | 101% | #NUM! |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | -0.29% | -0.89% | -0.74% | -0.18% | -1.28% | -2.03% | 101% | 99% |
| Class F | -0.29% | -1.56% | -0.87% | -0.21% | -2.88% | -1.20% | 67% | #NUM! |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -100.00% | -100.00% | -100.00% | -100.00% | -100.00% | -100.00% | #DIV/0! | #DIV/0! |
| Class E | -100.00% | -100.00% | -100.00% | -100.00% | -100.00% | -100.00% | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -100.00% | -100.00% | -100.00% | -100.00% | -100.00% | -100.00% | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

In JVET_AG0124, it is proposed to replace the QP slice of the models by the QP of the block. These additional results show that adapting the training process to consider both flat QP and Adaptive QP is even more beneficial and allows to increase gains for the current model. It is suggested that for further exploration to consider these constraints when the official dataset will be renewed.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).