| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0312-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0158 (EE2-3.9: Sub-block inter mode improvement) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Zhipin Deng | Tel: Email: | zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-AJ0158. The simulation results reportedly match those provided by the proponents.

# Introduction

JVET- AJ0158 [1] investigates the sub-block inter mode improvement.

Four subtests are arranged:

Test 3.9a: Affine CPMV search for affine DMVR

Test 3.9b: High precision MV for affine motion compensation

Test 3.9c: Additional combined merge candidates

Test 3.9d: Test 3.9a + Test 3.9b +Test  3.9c

We verified that

–	The source code implements match with what was described in the proposal.

–	Simulation results match with those provided in the proposal.

The coding results under the ECM CTC are summarized in Table 1.

Table 1: Performance of Test 3.9a over ECM-14.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.03% | -0.15% | 99.8% | 101.3% | 99.9% | 100.1% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.05% | -0.07% | 100.5% | 100.6% | 100.0% | 99.9% |
| Class C | -0.03% | -0.01% | -0.05% | 100.4% | 104.3% | 100.4% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | -0.03% | 0.01% | 97.9% | 100.8% | 100.0% | 100.0% |
| Class F | -0.01% | -0.03% | 0.01% | 100.0% | 100.3% | 99.8% | 100.0% |
| Class TGM | -0.02% | 0.02% | -0.03% | 99.8% | 100.4% | 100.6% | 100.2% |

Table 2: Performance of Test 3.9b over ECM-14.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.04% | -0.05% | 100.6% | 100.8% | 0.01% | -0.04% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.01% | -0.09% | 101.1% | 101.6% | 100.0% | 100.0% |
| Class C | -0.06% | -0.10% | 0.18% | 100.0% | 101.7% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.09% | -0.15% | -0.14% | 100.7% | 99.0% | 100.3% | 100.0% |
| Class F | -0.01% | -0.05% | -0.02% | 101.9% | 101.6% | 99.9% | 99.9% |
| Class TGM | -0.05% | 0.01% | -0.03% | 100.2% | 100.3% | 100.6% | 100.5% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | 0.23% | -0.35% | 102.3% | 103.5% | 100.0% | 100.1% |
| Class C | -0.08% | 0.14% | -0.16% | 101.4% | 100.8% | 100.0% | 100.0% |
| Class E | -0.10% | 0.44% | -1.20% | 107.1% | 102.5% | 99.8% | 100.3% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.08% | -0.32% | 0.25% | 103.2% | 116.0% | 100.0% | 100.0% |
| Class F | -0.23% | -0.01% | 0.09% | 100.0% | 99.5% | 99.8% | 99.9% |
| Class TGM | -0.14% | -0.06% | 0.00% | 101.2% | 99.0% | 100.0% | 99.8% |

Table 3: Performance of Test 3.9c over ECM-14.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.02% | -0.03% | 99.8% | 101.4% | 100.0% | 100.1% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.03% | -0.09% | 0.06% | 100.2% | 105.0% | 100.4% | 100.3% |
| Class C | -0.02% | 0.07% | 0.19% | 100.1% | 104.8% | 101.3% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.07% | 0.02% | 0.00% | 100.8% | 103.1% | 101.2% | 100.9% |
| Class F | 0.05% | 0.03% | 0.06% | 99.9% | 101.7% | 100.5% | 100.5% |
| Class TGM | 0.03% | 0.05% | 0.00% | 100.1% | 103.8% | 100.4% | 100.6% |

Table 4: Performance of Test 3.9d over ECM-14.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.10% | -0.07% | 0.01% | 103.6% | 103.8% | 100.9% | 100.8% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.12% | -0.24% | -0.02% | 104.2% | 103.4% | 101.3% | 100.9% |
| Class F | -0.06% | -0.10% | -0.09% | 102.0% | 101.9% | 100.5% | 100.6% |
| Class TGM | -0.05% | -0.01% | -0.05% | 101.1% | 101.9% | 100.5% | 100.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | 0.23% | -0.35% | 102.3% | 103.5% | 100.0% | 100.1% |
| Class C | -0.08% | 0.14% | -0.16% | 101.4% | 100.8% | 100.0% | 100.0% |
| Class E | -0.10% | 0.44% | -1.20% | 107.1% | 102.5% | 99.8% | 100.3% |
| Overall | -0.09% | 0.25% | -0.50% | 103.1% | 102.4% | 100.0% | 100.1% |
| Class D | -0.08% | -0.32% | 0.25% | 103.2% | 116.0% | 100.0% | 100.0% |
| Class F | -0.23% | -0.01% | 0.09% | 100.0% | 99.5% | 99.8% | 99.9% |
| Class TGM | -0.14% | -0.06% | 0.00% | 101.2% | 99.0% | 100.0% | 99.8% |

The available BD-points are matched with the results of the proposal [1]. Note that timing results may not be accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

J.-L. Lin, Y. Zhang, H. Huang, Z. Zhang, P.-H. Lin, C.-C. Chen, V. Seregin, M. Karczewicz, “EE2-3.9: Sub-block inter mode improvement”, JVET-AJ0158, November 2024.