| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0327-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0165 (EE1-1.2 LOP with residual groups and Attention) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Jacob Ström | Tel: Email: | jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document is a training crosscheck of JVET-AJ0165, which proposes a new architecture for LOP. The reported results for the model trained by the crosschecker over LOP3 are

RA: 	-0.69%	(Y) 	1.82% (U)		1.29% (V)
AI: 		-0.61%	(Y) 	1.04% (U) 		0.71% (V)
LDB: 	-0.51% (Y)		0.89% (U)		2.44% (V)

The reported results for the model trained by the crosschecker over the model trained by the proponents are

RA: 	-0.26%	(Y) 	1.86% (U)		0.95% (V)
AI: 		-0.23%	(Y) 	1.18% (U) 		0.34% (V)
LDB: 	-0.15% (Y)		0.49% (U)		2.29% (V)

It is the opinion of the crosschecker that the difference in BDR performance between the model trained by the crosschecker and the one by the proponents is well within training noise.

It was also reported that the training time needed to complete the training is longer for the proposed model (2h35min per epoch) compared to the LOP3 model (1h50min per epoch). This amounts to about a 40% increase in training time.

# Introduction

Summary of crosscheck

- Inference of the int16 model from the proponents was partially tested with matching BDR results.

- A stage3 training was carried out for the proponent’s model, and a full inference was created from that.

If comparing the BDR performance of the crosschecker-trained model with that of the proponent-trained model, the luma is better and the chroma is worse:

RA: 	-0.26%	(Y) 	1.86% (U)		0.95% (V)
AI: 		-0.23%	(Y) 	1.18% (U) 		0.34% (V)
LDB: 	-0.15% (Y)		0.49% (U)		2.29% (V)

Assuming that an average chroma BDR-loss is worth about 1/10 BDR-gain in luma, it looks like the crosschecker’s trained model performs slightly better than that of the proponents. In any case the training crosscheck is within the training noise and it is the crosschecker’s opinion that the training crosscheck is a match.

It was noticed that the time to train the model of the proponents was considerably longer than for LOP3. Whereas one epoch of LOP3 takes 1h50min, one epoch of training the proponents model took 2h35min, an increase of about 40%. About 43 epochs could be trained in the same time as the 60 epochs for LOP.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).