| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ0339-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AJ0138 (EE2-5.1a/b/c: Adaptive Mantissa Precision for ALF Coefficients) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-AJ0138. The simulations were carried out based on the common test condition. The partial results are matched in BD-rate except some RA class A and B sequences. The overall BD-rate difference between AJ0339 and AJ0138 is negligible. In the crosscheck report, decoding time is reliable, but encoding time is not.

# Introduction

In ECM-14.0, ALF coefficients are clipped to the range [-2BALF-1+1, 2BALF-1-1], where BALF is the coefficient precision. The parameter BALF is called the precision. Coefficients are separated into two groups based on the position in the filter. Then coefficients are signalled by Golomb codes with adaptive orders for each group.  In JVET-AJ0138, ALF coefficients are further limited to the elements of the set CBALF,BM={m⋅2b}, where m and b are integers, m∈[-2BM+1, 2BM-1] and b∈[0, BALF-1], moreover, m⋅2b belongs to the interval [-2BALF-1, 2BALF-1]. The coefficients are further mapped to indices in the alphabet array. The mapped indices are then coded with Huffman codes. For each allowed tuple (channel, BALF, BM, coefficients group), the Huffman table is predefined.

In test 5.1a, ALF coefficients are restricted to limited to the elements of the set CBALF,BM={m⋅2b} and Huffman codes are used to signal the mapped indices.

In test 5.1b, ALF coefficients are restricted to limited to the elements of the set CBALF,BM={m⋅2b} and modified Golomb codes are used to signal the mapped indices.

In test 5.1c, a simulated annealing-based coefficients search method is applied to improve the coding efficiency at encoder.

# Simulation Results

The proposed method was implemented on top of ECM-14.0 and tested following common test condition described in JVET-AI2017.

Table 1. Simulation results of EE2-5.1a.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.00% | 0.06% | 62.6% | 100.6% | 103.5% | 99.9% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.06% | -0.01% | 66.4% | 99.4% | 98.8% | 100.0% |
| Class C | -0.02% | 0.08% | 0.13% | 66.4% | 98.6% | 100.5% | 100.0% |
| Class E | -0.02% | 0.12% | 0.05% | 64.5% | 98.7% | 101.8% | 100.1% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | -0.21% | -0.41% | 66.2% | 102.8% | 100.5% | 100.0% |
| Class F | -0.05% | -0.14% | -0.17% | 64.9% | 98.9% | 101.8% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.24% | -0.18% | 68.5% | 101.9% | 100.1% | 100.5% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.16% | -0.09% | 66.9% | 100.6% | 100.1% | 101.2% |
| Class C | 0.03% | -0.16% | 0.12% | 65.1% | 101.7% | 100.3% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.06% | -0.24% | -0.05% | 64.2% | 100.7% | 100.5% | 100.1% |
| Class F | 0.01% | -0.09% | -0.08% | 64.9% | 98.5% | 100.4% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.03% | -0.33% | -0.32% | 63.3% | 101.6% | 100.8% | 100.3% |
| Class E | 0.03% | -0.26% | -0.70% | 66.0% | 98.1% | 100.5% | 100.4% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.12% | 0.33% | -0.14% | 63.8% | 98.7% | 101.1% | 100.1% |
| Class F | -0.15% | -0.03% | 0.94% | 65.4% | 99.4% | 99.4% | 100.5% |
| Class TGM |  |  |  |  |  |  |  |

Table 2. Simulation results of EE2-5.1b.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.02% | 0.11% | 63.5% | 100.2% | 103.5% | 100.0% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.00% | 0.07% | 65.1% | 98.5% | 99.8% | 99.8% |
| Class C | 0.02% | 0.17% | 0.22% | 65.3% | 100.0% | 101.2% | 100.0% |
| Class E | 0.06% | 0.13% | 0.09% | 66.6% | 99.4% | 104.1% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.05% | -0.10% | 65.5% | 101.8% | 100.6% | 100.0% |
| Class F | 0.02% | 0.02% | -0.01% | 63.1% | 98.4% | 102.1% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.04% | -0.12% | -0.03% | 66.9% | 100.3% | 100.1% | 99.8% |
| Class C | 0.08% | -0.04% | 0.23% | 64.7% | 101.0% | 100.3% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.11% | -0.21% | 0.05% | 64.5% | 100.3% | 100.4% | 100.1% |
| Class F | 0.08% | 0.08% | 0.01% | 65.4% | 98.4% | 100.4% | 100.2% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.19% | 0.12% | 0.01% | 64.3% | 101.2% | 101.2% | 99.8% |
| Class E | 0.11% | -0.39% | -0.70% | 65.4% | 98.9% | 99.5% | 99.8% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.26% | 0.96% | 1.00% | 64.8% | 98.8% | 101.1% | 100.0% |
| Class F | 0.11% | -0.76% | -0.27% | 64.8% | 100.1% | 101.6% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |

Table 3. Simulation results of EE2-5.1c

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.01% | -0.02% | 62.7% | 100.3% | 100.0% | 100.0% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.02% | -0.02% | 64.9% | 98.3% | 99.3% | 100.1% |
| Class C | -0.01% | 0.00% | 0.00% | 66.5% | 102.1% | 100.2% | 100.0% |
| Class E | -0.02% | 0.00% | 0.02% | 68.4% | 98.7% | 100.0% | 100.1% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.02% | 0.00% | -0.01% | 65.7% | 102.7% | 100.0% | 100.0% |
| Class F | -0.02% | 0.00% | -0.04% | 66.2% | 98.3% | 100.0% | 100.0% |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.07% | 0.01% | 65.9% | 100.1% | 100.0% | 100.0% |
| Class C | -0.01% | -0.10% | 0.11% | 65.7% | 101.0% | 100.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.06% | -0.09% | -0.18% | 63.7% | 99.8% | 99.9% | 100.0% |
| Class F | 0.02% | -0.03% | 0.12% | 65.9% | 98.5% | 99.8% | 100.3% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.05% | 0.11% | -0.05% | 64.2% | 103.2% | 99.0% | 99.9% |
| Class E | 0.02% | 0.04% | -0.39% | 65.6% | 96.5% | 98.0% | 99.8% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.04% | -0.37% | -0.09% | 62.9% | 102.8% | 100.5% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |