| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ1011-v2 |
| --- | --- |

| Title: | White paper on HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | White paper |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jens-Rainer Ohm Gary J. Sullivan Ye-Kui Wang | Email: | benjamin.bross@hhi.fraunhofer.de ohm@ient.rwth-aachen.de gary.sullivan@dolby.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This is an update of the white paper on HEVC which had previously been published as MPEG output document WG 11 N 15075 in February 2015, providing a summary of the HEVC standard and related developments, now updated to November 2024.

# Introduction

High Efficiency Video Coding (HEVC) is a video compression standard developed jointly by ITU-T VCEG and ISO/IEC MPEG through their Joint Collaborative Team on Video Coding (JCT-VC) and their Joint Collaborative Team on 3D Video (JCT-3V), published as twin text in ISO/IEC 23008-2 and ITU-T Rec. H.265. Since 2020, ongoing developments in HEVC are conducted by the Joint Video Experts Team (JVET) of ISO/IEC JTC 1/SC 29 and ITU-T SG16 (which is now becoming SG21 following the ITU’s reorganization for 2025–2028). The first version of HEVC was finalized by the JCT-VC in January 2013, and subsequent editions provided additional functionality. The HEVC project was launched with an intent to enable a major savings relative to the bit rate needed by the widely used AVC [1] standard (for equivalent visual quality). Especially for high-resolution video, where such additional compression is the most essential to enabling the applications, implementations of the standard are clearly meeting that targeted goal. This white paper includes a review of the architecture and building blocks of HEVC, which were carefully selected with regard to compression capability versus complexity and to enable parallelism for the signal processing operations.

Given the benefits that HEVC provides, it has been very widely adopted into industry products and services that use video compression. It has been a major factor in the successful deployment of several other recent advances in video technology, including the introduction of ultra-high definition (UHD) resolution, 10-bit video precision, high-dynamic range (HDR) with wide colour gamut (WCG), high frame rates, and streaming video services.

The HEVC standard has become the primary video broadcast format in various countries (such as Croatia, Czech Republic, Germany, Italy, and Netherlands), and it has been adopted into the industry specifications of many other organizations in addition to those of ITU, ISO and IEC, including 3GPP, the 8K Association, ANSI/SCTE, ARIB, ATSC, BD ROM, CTA/CEA, Digital UK, ETSI/DVB, ETSI/EBU, IETF, NATO, MISB, and the Ultra HD Forum. Low-power decoders are available from basically all video chip makers, and its encoding is supported in nearly all professional video production workflows.

HEVC is now supported in nearly all currently produced televisions, personal computers, mobile handsets, and web browsers, and it is very widely used by streaming service providers, television broadcasters, video recording devices, and video security systems. A recent (2023) publication estimated that more than 96% of current consumer video products have HEVC playback capability, amounting to 2.3 billion devices annually, with that volume continuing to rise substantially. The tremendous importance of efficient video compression technology becomes apparent when noting that about 80% of the data traffic on the world's communication networks now consists of video.

In recognition of the impact that the HEVC standard on the technology of television, the development of the HEVC standard was recognized by a Primetime Emmy Engineering Award of the Academy of Television Arts and Sciences in October 2017.

# Background

The history of video compression standardization has been dominated by efforts to maximize compression capability, where an increasing variety of services, the growing popularity of HD video, and the emerging beyond-HD formats (e.g. 4K×2K resolution) are creating even stronger needs for compression capabilities. Mobile devices and tablet PCs will now need to receive and display HD video as well. The advent of stereo and multi-view displays (and corresponding camera systems) further increases the amount of data. As increasing video traffic is seen as a major cause of the “spectrum crunch” that could occur in wireless networks, improved video compression can be a factor to mitigate this as well. Moreover, video now comprises a majority of Internet data traffic worldwide, and its percentage of that traffic is continually increasing.

HEVC has been designed to address essentially all existing applications of previous standards and to particularly focus on two key issues: increased video resolution and increased use of parallel processing architectures. The syntax of HEVC is generic, and its design elements could also be attractive for other application domains that have not used the preceding standards.

# Technology and key features

The video coding layer of HEVC employs essentially the same “hybrid” approach (inter- / intra-picture prediction and 2D transform coding) used in all video compression standards since H.261. A prediction residual is computed from previously decoded information (either previously decoded pictures for inter-picture motion compensated prediction or previously decoded samples from the same picture for intra-picture spatial prediction). The residual is then processed by a block transform, and the transform frequency coefficients are quantized and entropy coded. Side information data such as motion vectors and mode switching parameters are also encoded and transmitted.

Although much of this is similar to the design in prior standards, there are key differences that enable the enhanced compression capability of HEVC, which we discuss below. More detailed descriptions of the key technical features can be found in references [2] through [9], and reports about performance and complexity analysis were given in [10] and [11], respectively. The following paragraphs a-h describe features that were defined in edition 1 and are also common (in some cases extended) with later editions, whereas features described in i-j and k-l were newly added in editions 2 and 3, respectively. An overview about the current deployment status of HEVC can be found in [12].

a) Coding Tree Units and Coding Tree Block structure: The core of the coding layer in previous standards was the macroblock, consisting of a 16×16 block of luma samples and, in the usual case of “4:2:0” colour sampling, two corresponding 8×8 blocks of chroma samples and associated syntax elements. In contrast, the analogous structure in HEVC is the coding tree unit (CTU). The CTU consists of coding tree blocks (CTBs) for luma and chroma. The size L×L of a luma CTB can be chosen with L = 16, 32, or 64 samples, with the larger sizes typically enabling better compression. CTBs are then partitioned into coding blocks (CBs), signalled via a quadtree structure, as illustrated in Fig. 1. Usually, luma and chroma CBs are both split together, where it depends on the colour sampling format whether they have the same size or not. A luma CB can be as small as 8×8 (accompanied by two 4×4 chroma CBs in case of 4:2:0 colour sampling, for example). One luma CB together with the two corresponding chroma CBs and associated syntax elements is referred to as a coding unit (CU).

Below the CU level, there is additional partitioning into prediction units (PUs) and a tree of transform units (TUs). The decision whether to encode a picture area by inter-picture (motion compensated) or intra-picture prediction is made at the CU level. The CB shapes are always square. The luma and chroma prediction blocks (PBs) within a PU are also always square in the case of intra-picture prediction; however, the shapes used for inter-picture prediction can also be chosen as non-square rectangular blocks.

Fig. 1. Subdivision of a 64x64 luma CTB into CBs and TBs. Solid lines indicate CB boundaries and dotted lines indicate TB boundaries. Left: the CTB with its partitioning, right: the corresponding quadtree. In this example, the leaf nodes are each 8×8 in size – although, in general, a TB can actually be as small as 4×4.

b) Transform Units and Transform Blocks: The prediction residual is coded using block transforms. A transform unit (TU) tree structure has its root at the CU level, where the CBs may be further split into smaller transform blocks (TBs). Integer basis functions approximating the discrete cosine transform (DCT) are defined for dyadic TB sizes from 4×4 to 32×32 (whereas the largest transform size is 8x8 in AVC). For the 4×4 transform of intra-picture prediction residuals, an integer transform derived from the discrete sine transform (DST) is additionally specified, which better matches the statistics of directional intra-picture prediction residual signals. The quantization and coding of transform coefficients is similar to AVC. Coding and decoding of non-zero coefficients is performed by grouping them into 4×4 coefficient groups and scanning the coefficients in each group using a scanning order that is usually diagonal, but changes to horizontal or vertical in the case of small TBs (8x8 and smaller) that are coded using certain directional modes of intra-picture prediction. The position of the last non-zero coefficient in the scanning order is encoded, followed by a “significance map” to identify which coefficients have non-zero values, and then the sign and magnitude of the significant coefficients are signalled.

c) Motion compensation: Motion compensation uses quarter-sample precision, and 7-tap or 8-tap filters are used for interpolation of any fractional-sample positions (compared to 6-tap filtering of half-sample positions followed by averaging between two values to form quarter-sample positions in AVC). Similar to AVC, multiple reference pictures are used – with up to 6 stored pictures (or, when using reduced picture resolutions, sometimes up to 16). Per PB, either one or two motion vectors can be transmitted, resulting in uni-predictive or bi-predictive coding, respectively. For the latter case, either ordinary averaging or weighted prediction can be employed. Advanced motion vector (MV) coding is used, including derivation of several most probable candidates based on data from adjacent PBs and the co-located region in a reference picture. A new merge mode for MV coding is also defined, signalling the inheritance of MVs from one of the neighbouring PBs within the same CTU or from the reference picture. Moreover, compared to AVC, improved “skip” and “direct” motion inference is specified – and in these cases no selection is signalled and the most probable candidate is simply selected and used without modification.

d) Intra-picture prediction: Decoded boundary samples from adjacent blocks are used as prediction reference data for spatial prediction in PB regions when inter-picture prediction is not performed. Intra-picture prediction uses 33 directional modes (compared to 8 such modes in AVC), plus DC (flat overall averaging) and planar (surface fitting) prediction modes. Chroma prediction is similar, but uses a simplified selection between fewer modes (horizontal, vertical, planar, DC, the same mode used for luma, or left-downward diagonal). The different intra-picture prediction modes are encoded by deriving most probable modes (e.g. the prediction directions) based on those of previously-decoded neighbouring PBs.

e) Entropy coding: Five generic binarization schemes are defined for symbol encoding, and it is specified which of these is applied to each type of syntax element. Context-adaptive binary arithmetic coding (CABAC) is then used for entropy coding. The basic method is similar to the CABAC scheme in AVC, but has undergone a number of improvements, especially in regard to reducing the number of adaptive coding contexts, increasing the use of fast “bypass” coding, and improving the capability for parallel processing to increase the throughput.

f) In-loop filtering: One or two filtering stages can be optionally applied (within the inter-picture prediction loop) before writing the reconstructed picture into the decoded picture buffer. A deblocking filter (DBF) is used that is similar to the one in AVC; however the DBF design has been simplified with regard to its decision making and filtering processes and also has been made more friendly to parallel processing. The number of luma edges to be filtered by the DBF in the worst case has been cut in half relative to the filtering in AVC, by limiting the filtering operations of the DBF to operate only on an 8×8 rather than 4×4 block boundary grid. The second stage, called the sample adaptive offset (SAO) filter, is a non-linear amplitude mapping. The goal of SAO is to improve the reconstruction of the signal amplitudes by a lookup table mapping that is controlled by the encoder. Two types of SAO operation can be selected for each CTB – the band offset and edge offset modes. Using band offsets, banding artifacts can be reduced by dividing the signal amplitude range into segments and adding offset values that depend on the amplitude category of the value of each reconstructed sample. Edge offsets have a directional “de-ringing” effect, and are controlled by an encoder selection between a vertical, horizontal, downward-diagonal, or upward-diagonal filtering direction. The value of each sample is checked in relation to its directional neighbours to identify regions that have a local minimum or maximum or a local edge – and then an offset value is added that depends on this classification.

g) Slices, tiles and wavefronts: A slice is a series of CTUs that can be decoded independently from other slices of the same picture (except for in-loop filtering of the edges of the slice, and except for the case of “dependent slices” as described below). A slice can either be an entire picture or a region of a picture. One of the main purposes of slices is re-synchronization after data losses. An example partitioning of a picture into a slice structure is shown in Fig. 2a.

To enable parallel processing and localized access to picture regions, the encoder can partition a picture into rectangular regions called tiles. Fig. 2b shows an example. Tiles are also independently decodable but can share some header information when multiple tiles are used within a slice. Alternatively, for finer-granularity packetization of data, an encoder can use multiple slices within a tile (but is not allowed to mix these two behaviors within the same picture). An additional supported form of enabling parallelism is for the encoder to use wavefront parallel processing (WPP), in which a slice is divided into rows of CTUs. With WPP, the encoding or decoding of CTUs of each row can begin before processing most of the CTUs of the previous row – thus enabling different processing threads to work on different rows of the picture at the same time, as shown in Fig. 2c. (To minimize the difficulty of making decoders, encoders are prohibited from using WPP when using multiple tiles per picture.) Additionally, to enable packetization at a fine level of granularity, dependent slices are also defined, which allows the header information for multiple sequences of CTUs to share the same slice header information, each starting at a tile or wavefront entry point.

Fig. 2. Subdivision of a picture into (a) slices and (b) tiles, and illustration of wavefront parallel processing (c).

h) High-level syntax: Above the coding layer, many of the high-level syntax features of AVC have been retained or extended. Parameter sets contain information that can be shared for the decoding of several pictures or regions of the decoded video. The parameter set structure provides a robust mechanism for conveying data that are essential to the decoding process. Each syntax structure is placed into a logical data packet called a network abstraction layer (NAL) unit. Depending on the content of a two-byte NAL unit header, it is possible to readily identify the purpose of the associated payload data, e.g. parameter sets, data for decoding random accessible pictures, etc. Further elements of high-level syntax are specifications of access structures, management of coded and decoded picture buffers, signalling of video usability information (VUI) and supplemental enhancement information (SEI).

i) Extended format and quality ranges: Version 1 of HEVC allowed only using 4:2:0 colour sampling, with components represented by a maximum bit depth of 10. Version 2 enables using other colour sampling schemes such as 4:2:2 and 4:4:4, and allows representing up to 16 bit video. In order to achieve this efficiently in the range of high bit rates, also some elements of entropy coding are modified when used in this context. Further tools are also added for more efficient lossless and near-lossless coding, whereas version 1 only supported simple lossless coding of CUs by invoking bypass of transform, quantization and loop filtering mechanisms.

j) Multi-view and scalable coding: Version 2 introduced more sophisticated layering mechanisms in the high-level syntax, allowing to establish hierarchical bitstream structures also associated with dependency structures between parts of information associated with the layers, as well as the related access mechanism. This can be used for the purpose of stereo/multi-view coding, where a decoder of a dependent view can refer to previously decoded pictures from another view (usually captured by a different camera) to save bit rate by utilizing inter-view redundancy; herein, motion compensation tools can be identically used to perform disparity compensated prediction from a reference picture captured at same time instance in a different view. Another usage is scalable coding, where an enhancement layer decoder can take reference to a lower layer, e.g. the same picture decoded by lower resolution or quality. In both multi-view and scalable coding existing version 1 decoder implementations can be re-used for reconstructing the pictures associated to the different layers; in the case of scalability with varying spatial resolution between the layers, filters for performing the re-sampling are specified for the additional inter-layer processing. As a specific type of layers, auxiliary pictures are furthermore defined, which could for example be used to represent alpha maps or depth maps.

k) 3D coding tools: For better compression of multi-view sequences accompanied by depth maps, more specific tools were added later which allow utilizing mutual dependencies between video and depth. For example, a motion vector that had already been coded/decoded for the video texture can be re-used for a collocated depth map, or the depth information can be used for improving the inter-view prediction. Tools can utilize information about camera positions (when available) and are better adapted to the specific characteristics of depth data. Though depth maps can also be compressed as auxiliary pictures in a video stream, the extension nicknamed 3D-HEVC provides better bitrate reduction for this type of data.

l) Screen content coding tools: Characteristics of computer-generated content are quite different from those of video captured by cameras. For example, this type of data is less noisy, has sharper edges and areas with more constant colour. Furthermore, repetitions of identical structures can occur more frequently in computer-generated content. To reach better compression for these cases, new tools were added, particularly intra-block copy (allowing motion compensation to take reference to the current picture), and palette coding, using a lookup table to signal the values of samples. Beyond the fact that computer generated content is becoming more and more prominent in the production of moving pictures, support for wireless displays is another foreseen application area.

# Profiles, levels, tiers, and version history

Similar to previous video compression standards, HEVC defines conformance points in terms of profiles (combinations of decoding tools and associated bitstream syntax that is expected to be interpreted by a decoder), and levels (typically maximum sizes of pictures and frame rates, maximum bit rate, buffer capacity etc.). Currently, a total of 17 levels, ranging in support for typical picture sizes as small as VGA at the low end, up to 16K×8K at the high end, are specified. For more flexibility, HEVC also defines two tiers in some of the levels, basically defining two different limits in terms of the maximum number of bits per second that a conforming decoder should be able to process.

As of the time of preparation of this white paper, ten editions of the HEVC standard have been published by ITU-T and five have been published in ISO/IEC. Generally, some modifications were first published as amendments in ISO/IEC that described differences relative to the most recent previous ISO/IEC edition, whereas ITU-T has issued only complete editions. However, the technical content of the standards has been maintained in strong technical alignment with a “twin text” approach.

The first edition, published by ISO/IEC in 2013-12, contained three profiles (all supporting only 4:2:0 colour sampling):

- The Main profile (8 bit);

- The Main 10 profile (10 bit, otherwise same as Main);

- The Main Still Picture profile (8 bit, only intra coding tools from Main).

The second ISO/IEC edition, published in 2015-05, additionally specified a whole bundle of profiles, collectively referred to as the format range extensions profiles, which extend the applicability of the main and main still picture profiles in various regards of colour sampling and bit depth:

- The Monochrome, Monochrome 12 and Monochrome 16 profiles;

- The Main 12 profile;

- The Main 4:2:2 10 and Main 4:2:2 12 profiles;

- The Main 4:4:4, Main 4:4:4 10, and Main 4:4:4 12 profiles;

- The Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 Intra, Main 4:4:4 10 Intra, Main 4:4:4 12 Intra, and Main 4:4:4 16 Intra profiles;

- The Main 4:4:4 Still Picture and Main 4:4:4 16 Still Picture profiles;

- The High Throughput 4:4:4 16 Intra profile.

In the second ISO/IEC edition, the following profiles were also added:

- The Multiview Main profile, enabling layered dependent coding of multiple views (8 bit, 4:2:0 colour sampling);

- The Scalable Main (8 bit) and Scalable Main 10 profiles (both with 4:2:0 colour sampling over all layers).

One amendment of the second edition was also published in 2015, and it was then integrated into the third edition.

The third ISO/IEC edition, published in 2017-10, added profiles supporting 3D-HEVC and screen content coding, among others, more specifically:

- The 3D-HEVC profile, enabling layered dependent coding of multiple views plus depth;

- Seven SCC profiles, namely the Screen-Extended Main, Screen-Extended Main 10, Screen-Extended Main 4:4:4, Screen-Extended Main 4:4:4 10, Screen-Extended High Throughput 4:4:4, Screen-Extended High Throughput 4:4:4 10, and Screen-Extended High Throughput 14 profiles, enabling efficient coding of variety of screen content;

- Four scalable range extensions profiles, namely the Scalable Monochrome, Scalable Monochrome 12, Scalable Monochrome 16, and Scalable Main 4:4:4 profiles, enabling layered dependent coding of monochrome content of bit depth up to 16 bits as well as 4:4:4 content;

- Three additional high throughput profiles, namely the High Throughput 4:4:4, High Throughput 4:4:4 10, and High Throughput 4:4:4 14 profiles, enabling high throughput decoding capability of 4:4:4 content with bit depth less than 16 bits (high throughput decoding capability of 16-bit 4:4:4 content was already enabled via the High Throughput 4:4:4 16 Intra profile in the 2nd edition).

Three amendments of the standard were published by ISO/IEC in 2018, and these were consolidated into the fourth edition published in 2020-08.

A fifth ISO/IEC edition was then published in 2023-10.

The features that were added in the fourth and fifth ISO/IEC HEVC editions include:

- The Monochrome 10 profile and the Main 10 Still Picture profile, enabling 10-bit coding of monochrome and still picture content;

- Five additional SEI messages, namely 1) the SEI manifest SEI message, 2) the SEI prefix SEI message, 3) the fisheye video information SEI message, 4) the annotated regions SEI message, the shutter interval information SEI message;

- Four levels, namely levels 6.3, 7, 7.1, and 7.2, for all profiles, enabling coding of picture sizes up to 16Kx8K at the high end;

- One level, i.e., level 8.5, only for the video profiles (i.e., those profiles that are not still picture profiles), enabling coding of video content with potentially unlimited capabilities for various convenience purposes such as product development, testing, and debugging.

# Performance

The JCT-VC carried out various formal subjective verification tests of different HEVC profiles, where HEVC encoding was typically compared to its predecessor AVC where roughly 50% bit rate saving was typically found, or to a previous profile without the corresponding functionality. To ensure that the design was not overly customized to a particular test set, none of these tests used video sequences that had previously been part of the test set during standard development:

- A comparison of the HEVC Main Profile against the AVC High Profile can be found in JCTVC-Q1011.

- A comparison of various HEVC RExt Profiles versus corresponding AVC profiles, as well as a comparison of the two standards in coding of interlaced video is reported in JCTVC-U1003.

- Scalable HEVC profiles are compared against HEVC profiles without such functionality (using  simulcast) in JCTVC-W1004 and JCTVC-X1004, reporting 40-60% bit rate savings for different application scenarios.

- The HEVC Multi-view Profile is compared against the AVC Multi-View Profile and against HEVC simulcast of views in JCT3V-N1001, indicating roughly 50% bit rate saving against the predecessor, and roughly 33% against HEVC main profile without such functionality.

- For the case of video-plus depth coding targeting a viewpoint synthesis scenario, the 3D-HEVC profile is shown to outperform the multi-view profile in JCT3V-M1001, indicating roughly 20% additional bit rate saving.

- The scenario of using the HEVC Main 10 Profile for coding HDR/WCG video is presented in the white paper N17664. Corresponding subjective quality results are reported in JCTVC-Y1018.

- The compression performance of the HEVC SCC Profile is reported in JCTVC-AA1006 for cases of graphics-only content and mixed content, reporting 60-90% bit rate savings against AVC, and 40-80% over HEVC without dedicated SCC tools.

# Ongoing developments

The sixth ISO/IEC edition of HEVC was approved by JVET in April 2024, and is expected to be published soon. The corresponding ITU edition (version 10) was published in October 2024. This recent development has added:

- Six additional profiles, namely 1) the Multiview Extended profile, 2) the Multiview Extended 10 profile, 3) the Multiview Monochrome profile, 4) the Multiview Monochrome 10 profile, 5) the Multiview Monochrome 12 profile, and 6) the Multiview Monochrome 16 profile;

- Three additional colour type identifiers in VUI;

- Three additional SEI messages, namely the neural network post-filter characteristics (NNPFC) SEI message, the neural-network post-filter activation (NNPFA) SEI message, and the phase indication SEI message, all through referencing to Rec. ITU-T H.274 | ISO/IEC 23002-7.

# Resources

All JCT-VC, JCT-3V and JVET documents are publicly available from https://jvet-experts.org/, and the HM reference software repository is available at http://hevc.hhi.fraunhofer.de/.

In addition to the HM, software codebases have also been developed for the various extensions of functionality. These include:

- The “SHM” (scalable HEVC model)

- The “HTM” (HEVC 3-D test model) supporting the multiview and 3D extensions

- The “SCM” (screen content model) supporting the screen content coding extensions

The various reference software codebases have been formally published as twin text in ISO/IEC 23008-5 and ITU-T Rec. H.265.2, which have been updated periodically along with the core coding standard.

Conformance tests have also been published as twin text in ISO/IEC 23008-8 and ITU-T Rec. H.265.1.

Extensive reviews of the deployment status for the standard have periodically been submitted to the JCT-VC and JVET, typically as document number 20 of a meeting. These have also included listings of industry specifications outside of ITU, ISO and IEC in which the standard is supported, and also listings of some available bitstream analysers and conformance test sets. As of the time of preparation of this document, the latest of these deployment surveys is found in [12].

# References

- ITU-T and ISO/IEC JTC 1, “Advanced Video Coding for generic audio-visual services,” ITU-T Rec. H.264 and ISO/IEC 14496-10 (AVC), May 2003 (and subsequent editions).

- G. J. Sullivan, J.-R. Ohm, W.-J. Han, and T. Wiegand, “Overview of the high efficiency video coding (HEVC) standard,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, Dec. 2012, pp. 1649–1668.

- V. Sze, M. Budagavi, and G. J. Sullivan, High Efficiency Video Coding (HEVC): Algorithms and Architectures (Integrated Circuits and Systems), ISBN-13: 978-3319068947, ISBN-10: 3319068946, Springer, 375 pages, Aug. 2014.

- M. Wien, High Efficiency Video Coding: Coding Tools and Specification (Signals and Communication Technology), ISBN-13: 978-3662442753, Springer, 339 pages, Sep. 2014.

- D. Flynn, D. Marpe, M. Naccari, T. Nguyen, C. Rosewarne, K. Sharman, J. Sole, J. Xu: “Overview of the Range Extensions for the HEVC Standard: Tools, Profiles, and Performance”, IEEE Trans. Circuits Syst. Video Technol., vol. 26, Jan. 2016, pp. 4–19

- J. M. Boyce, Y. Ye, J. Chen, A. K. Ramasubramonian: “Overview of SHVC: Scalable Extensions of the High Efficiency Video Coding Standard”, IEEE Trans. Circuits Syst. Video Technol., vol. 26, Jan. 2016, pp. 20–34

- G. Tech, Y. Chen, K. Müller, J.-R. Ohm, A. Vetro, Y.-K. Wang: “Overview of the Multiview and 3D Extensions of High Efficiency Video Coding”, IEEE Trans. Circuits Syst. Video Technol., vol. 26, Jan. 2016, pp. 35–49

- J. Xu, R. Joshi, R. A. Cohen: “Overview of the Emerging HEVC Screen Content Coding Extension”, IEEE Trans. Circuits Syst. Video Technol., vol. 26, Jan. 2016, pp. 50–62

- E. François, C. Fogg, Y. He, X. Li, A. Luthra, A. Segall: “High Dynamic Range and Wide Color Gamut Video Coding in HEVC: Status and Potential Future Enhancements”, IEEE Trans. Circuits Syst. Video Technol., vol. 26, Jan. 2016, pp. 63–75

- T. K. Tan, R. Weerakkody, M. Mrak, N. Ramzan, V. Baroncini, J.-R. Ohm, G. J. Sullivan: “Video Quality Evaluation Methodology and Verification Testing of HEVC Compression Performance”, IEEE Trans. Circuits Syst. Video Technol., vol. 26, Jan. 2016, pp. 76–90

- F. Bossen, B. Bross, K. Sühring, D. Flynn, “HEVC Complexity and Implementation Analysis,” IEEE Trans. Circuits Syst. Video Technol., vol. 22, Dec. 2012, pp 1685–1696.

- G. J. Sullivan: “Deployment status of the HEVC standard”, document JVET-AI0020, July 2024