| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 1–8 November 2024 | Document: JVET-AJ2023-v1 |
| --- | --- |

| Title: | Exploration Experiments on Neural Network-based Video Coding (EE1) |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, R. Chang, F. Galpin, Y.Li, D. Rusanovskyy, M. Santamaria, J. Ström, and Z. Xie | Email: | elena.alshina@huawei.com renjiechang@tencent.com franck.galpin@interdigital.com yue.li@bytedance.com dmytror@qti.qualcomm.com maria.santamaria_gomez@nokia.com jacob.strom@ericsson.com xiezhihuang@oppo.com |
| Source: | EE Coordinators |  |  |

_____________________________

# Abstract

This document summarizes Exploration Experiment 1 (EE1) tests to be performed between the JVET-AJ and JVET-AK meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performances and complexity aspects.

# Introduction

Code base for the test should be NNVC11.0, anchor is default configuration of NNVC-11.0 (NN-Intra and LOP-4 filter enabled). NNVC common test conditions [1], results and complexity reporting template must be used.

For proposals in-loop filter, NN-inter and super-resolution categories, proponents are encouraged to use existing AhG11 training set [1]. Proponents in NN-Inter category should specify details of training set if it is different from NNVC common test condition (DIV2K, BVI, TVD).  In all tests, training set must be clearly specified in the contribution.

For tests competing with technologies in NNVC-11.0 it was agreed to configure the proposed solution targeting close to existing NNVC tool complexity, but not exceeding it:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- the number of channels (both input and output) in neural network modules which are modified must be keep multiple of 16,

- Number of Parameters EE1 test  Number of Parameters NNVC (if possible).

If it is not possible to respect both constrains 1) and 2) at the same time then two sub tests need to be performed: first test respecting the constrain 1) only – in order to prove the value of the proposed change, and the second test respecting constrain 2) with minor violation of 1)   - in order to provide software and hardware friendly design.

In order to simplify EE1 tests overview the proponents are requested to provide the model analysis according to the example given in the section 7.

Proposed neural network architecture must be specified in exploration experiment description and kept unchanged. Adjustment for the number of channels and the number of repletion of back bone blocks can be done later in order to respect constrains listed above. Exact parameters settings must be announced by proponents by 2nd AhG11/14 teleconference on December 20.

Two teleconferences are planned to be conducted in order to discuss the NNVC reference software integration and EE1 tests setup.

Inference cross-check is required for all EE1 tests. Candidates for adoption to NNVC are required to undergo training cross-check, implementation must be compatible with SADL. The layers, not supported by majority of AI accelerators, must be explained in the contribution with details of implementation (including quantization).

# List of tests

This round of EE1 tests includes:

- EE1-1: LOP and VLOP in-loop filter

  - EE1-1.1 – Partial Convolution and Over-Parameterization JVET-AJ0080

    - Tester: UESTC, cross-check: Qualcomm

  - EE1-1.2 – EE1-1.1 and LOP with residual groups and Attention JVET-AJ0165

    - Tester: Qualcomm, cross-check: Nokia

  - EE1-1.3 - Multiscale blocks JVET-AJ0182

    - Tester: Nokia, cross-check: UESTC

  - EE1-1.4 Cross-component enhanced LOP filter JVET-AJ0068

    - Tester: Bytedance, cross-check: Sharp (?)

  - EE1-1.5 Adaptive skip of LOP filtering based on boundary strength partitions JVET-AJ0141

    - Tester: Hanyang University and ETRI, cross-checker: Huawei

  - EE1-1.6 Reduced complexity input feature extraction for LOP & VLOP JVET-AJ0066

    - Tester: Qualcomm, cross-checker: ???

- EE1-2: HOP in-loop filter

  - EE1-2.1 – Attention block with transformers JVET-AI0176

  - E1-2.2 Transformer-Based Reference Frame Synthesis for VVC Inter Coding JVET- JVET-AJ0148

- EE1-3: NN-inter prediction

  - E1-3.1 Transformer-Based Reference Frame Synthesis for VVC Inter Coding JVET- JVET-AJ0148

  - E1-3.2 – RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding JVET-AJ0099

    - Tester: Xidian University, cross-checker: Nokia (???)

- EE1-4: NN-based super resolution

  - EE1-4.1 – A Neural Network Downscaling Filter for RPR JVET-AJ0093

    - Tester: HUSTUniversity, cross-checker: ???

# Test description

## EE1-1: LOP and VLOP in-loop filter

### EE1-1.1 – Partial Convolution and Over-Parameterization JVET-AJ0080

We tested the LOP network structure using partial convolution and reparameterization in AJ0080. Compared to LOP3, there is a gain in luma, and the decoding time is reduced by nearly 10%. In the following EE, we will complete the following test:

- Test on new NNVC LOP/VLOP anchor, plan to use 8/16 channels convolution to accelerate network inference gain and obtain better performance at the same time.

ADD NN DIAGRAM/DESCRIPTION

- Sub-test on reduced training time (aligned to anchor training time).

- Joint test Nokia/ Qualcomm

Training: Following LOP/VLOP training strategy with stage 3 LOP dataset.

Inference: NNVC 11.0 software or later on NNVC CTC with LOP/VLOP anchor configuration

Proponent: UESTC, Transsion

### EE1-1.2 – EE1-1.1 and LOP with residual groups and Attention JVET-AJ0165

LOP with residual group and Attention was tested in JVET-AJ0165. The inference results show a significant Luma coding gain compared to LOP3. In this EE, the following test will be conducted:

- Test on QC design on top of NNVC LOP/VLOP anchor (adjustment of the architecture may be flexible to improve the performance).

ADD NN DIAGRAM/DESCRIPTION

- Sub-test on reduced training time (aligned to anchor training time).

- Joint test Nokia/UETSC

Training: Following LOP/VLOP training strategy with stage 3 LOP dataset.

Inference: NNVC 11.0 software or later on NNVC CTC with LOP/VLOP anchor configuration

Proponent: Qualcomm

Crosschecker: TBD.

### EE1-1.3 - Multiscale blocks JVET-AJ0182

This test will study the combination of aspects from EE1-1.2 with multiscale blocks in LOP and VLOP filters.

One subtest will study the use of aspects from EE1-1.2 in the luma branch and aspects of multiscale blocks in the chroma branch. The following figure shows this subtest.

One or more other subtests will study the combination of aspects from EE1-1.2 with aspects of multiscale blocks in both the luma branch and chroma branch. The following figure shows such subtest(s). Architectural details of the combination will be refined and communicated later.

Training: Following LOP/VLOP training strategy, Stage 3 LOP dataset.

Inference: NNVC 11.0 software or later, NNVC CTC, following LOP/VLOP anchor settings.

Proponent: Nokia

Crosschecker: TBD.

### EE1-1.4 Cross-component enhanced LOP filter JVET-AJ0068

LOP3 filter employs a shared heading structure but separate backbone branches for luma and chroma components. The cross-component information may not be fully utilized in LOP3 filter, since luma and chroma only interact in shallow layers. Besides, the capacity of chroma branch is limited compared to the luma branch.

To fully leverage the cross-component correlation and enhance chroma filtering process, a cross-component enhanced LOP filter was proposed in JVET-AJ0068, introducing deep features from the luma branch into the chroma branch, as shown in the figure below. This EE test will study the usage of luma features from different positions for cross-component enhancement.

Fig. X. (a) Existing LOP3 filter, (b) Proposed cross-component enhanced LOP filter.

### EE1-1.5 Adaptive skip of LOP filtering based on boundary strength partitions JVET-AJ0141

    - JVET-AJ0141, proponents: H. Kwon, J. Seo, H. Ko (HYU), D. Kim, S.-C. Lim (ETRI)

- EE1-1.5.1: Proposed adaptive skip of LOP filtering with different lambda sets

- EE1-1.5.2: Non-normative implementation of EE1-1.5.1

In this test, boundary strength (BS) partition based adaptive skip is tested to reduce decoding time induced by the LOP filter. In the proposed method, LOP filtering is skipped if the number of BS partitions in a filtering block is lower than a BS partition threshold. To determine optimal thresholds for each slice, a novel scheme called time-distortion optimization (TDO) is integrated into the NN in-loop filter encoding process.

An example of the number of BS partitions in a filtering block.

In test EE1-1.5.1, the proposed adaptive skip of LOP filtering will be tested. In EE1-1.5.1.1 subtest, the lambda value used in JVET-AJ0141 will be tested. In the remaining subtests, different lambda sets will be tested to evaluate the trade-off between decoder runtime reduction and coding performance.

In test EE1-1.5.2, a non-normative implementation of the proposed adaptive skip of LOP filtering will be tested and compared with the original normative method used in EE1-1.5.1.

Inference: Experiments will be tested on NNVC 11.0 using the NNVC CTC.

Proponent: Hanyang University and ETRI

Crosscheck: Huawei

### EE1-1.6 Reduced complexity input feature extraction for LOP & VLOP JVET-AJ0066/ JVET-AI0204

A reduced complexity input features extraction was proposed in JVET-AI0204 and some of its aspects (removal of the DCT over sparse data) was tested in JVET-AJ0066, Sub-test 1. In EE1-1.6, the second aspect of the original proposal (reduction of the input data volume by combining planes of the sparse input data types) will be tested.

ADD NN DIAGRAM/DESCRIPTION

### EE1-2.1 – Attention block with transformers

JVET-AI0176, proponents: Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)

- EE1-2.1Integer implementation of the transformer of JVET-AI00176

The architecture to be modified to achieve target complexity levels for HOP and better performance/complexity trade-off. Training to follow HOP training strategy and Stage 3 HOP dataset to be used. Training process may be modified to reflect change in the architecture and improve performance.

Training: Stage 3 HOP dataset.

Inference: NNVC 11.0 software or latter, NNVC CTC, following HOP anchor settings.

Proponent: Qualcomm

Crosscheck: TBD.

ADD NN DIAGRAM/DESCRIPTION

## EE1-3: NN-inter prediction

### E1-3.1 – RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding JVET-AJ0099

ADD DESCRIPTION

### E1-3.2 Transformer-Based Reference Frame Synthesis for VVC Inter Coding JVET-AJ0148

ADD DESCRIPTION

## EE1-4: NN-based super resolution

### EE1-4.1 – A Neural Network Downscaling Filter for RPR JVET-AJ0093

    - JVET-AJ0093, proponents: Yiqing Zhu, Wenxuan He, Xu Li, Jiedong Ye, Qiong Liu (HUST), Zhuoyi Lv (vivo)

- EE1-4.1.1: Proposed method tested on the latest NNVC (combining with NNSR).

- EE1-4.1.2: Joint retrain the model with NNSR using DWT loss.

- EE1-4.1.3: Redesign model for 1.5x downsampling.

RPR with a Neural Network Downscaling Filter was tested in JVET-AJ0093. In the test, the RPR down-sampled frames were enhanced by the proposed method. And there are gains in YUV components over the NNVC8.0 with default RPR.

Illustration of the integration for the CNN-based downscaling filter for RPR.

JVET-AJ0093 downscaling network architecture.

In test EE1-1.4.1, the proposed 2x down-sampling filter will be tested on the latest NNVC combining with NNSR.

In test EE1-1.4.2, training data will be regenerated using proposed down-scaling model. Within training progress, we’ll fix the weights of NNSR and finetune downscaling model in co-training way with the latest loss function (including DWT loss proposed in JVET-AJ0056).

In test EE1-1.4.3, based on the proposed method, we will redesign a 1.5x down-sampling filter and test it on the latest NNVC combining with NNSR.

Inference: NNVC 11.0 software using the NNVC CTC.

Proponent: Huazhong University of Science and Technology and vivo

Crosschecker: TBD.

# Expected EE1 Timeline

T1 – 2.5 weeks after JVET-AI meeting (28 -November-2024): 1st teleconference. LOP changes adopted to NNVC training is finished, results ready to make final decision on NNVC-11.0. NNVC-11.0 performance results are discussed during teleconference.

T2 – 3 weeks after JVET-AI meeting (29 - November -2024) EE description (JVET-AJ2023) finalized and uploaded. NNVC-11.0 software is available, including anchor performance.

T3 – 6 weeks after JVET-AI meeting (20 -December-2024) 2nd teleconference. Final setting for parameters to be announced, partial results discussed, combinational tests (if any) decided.

T4 – 2 weeks before T6 (02-January -2025): EE1 software is frozen (both inference and training), write access closed, proponents are recommended to notify crosschecked about commit to be used.

T5 – 3 days before T6 (11- January -2025): Cross-checkers report status to EE1 coordinators (sending e-mail).

T6 –14- January -2025: EE1 summary is uploaded as input contribution

# SW location

https://vcgit.hhi.fraunhofer.de/jvet-aj-ee1/VVCSoftware_VTM/-/tree/EE1-X.X

# NNVC models analysis

The analysis [2] was performed using the “debug_model” software provided in the SADL repository, parsing the [ANALYZE] logs of the software.

The software is available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl/-/merge_requests/115

## VLOP2

Figure 1. LOP and VLOP filters in NNVC-10

| Add |  |  | Conv2D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| size | count |  | size IN | size OUT | count |  |
| 2 | 1 |  | 1 | 1 | 2 | input |
| 4 | 1 |  | 4 | 1 | 1 | input |
| 8 | 2 |  | 6 | 2 | 1 | input |
| 16 | 8 |  | 6 | 4 | 1 | input |
| 32 | 24 |  | 6 | 8 | 1 | input |
| 48 | 1 |  | 16 | 8 | 1 | output |
| 64 | 3 |  | 16 | 32 | 2 |  |
| 96 | 7 |  | 16 | 16 | 5 |  |
| 20736 | 3 |  | 17 | 64 | 1 | comb inputs |
| 165888 | 1 |  | 32 | 32 | 15 |  |
| 497664 | 6 |  | 32 | 16 | 3 |  |
|  |  |  | 32 | 96 | 7 |  |
|  |  |  | 64 | 48 | 1 |  |
|  |  |  | 64 | 64 | 2 |  |
|  |  |  | 96 | 32 | 7 |  |

## LOP3

Figure 1. LOP and VLOP filters in NNVC-10

| Add |  |  | Conv2D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| size | count |  | size IN | size OUT | count |  |
| 2 | 3 |  | 1 | 2 | 2 | input |
| 4 | 1 |  | 4 | 2 | 1 | input |
| 8 | 2 |  | 6 | 16 | 1 | input |
| 16 | 2 |  | 6 | 4 | 1 | input |
| 32 | 62 |  | 6 | 8 | 1 | input |
| 64 | 4 |  | 32 | 16 | 1 |  |
| 128 | 5 |  | 32 | 8 | 1 | output |
| 144 | 15 |  | 32 | 144 | 15 |  |
| 663552 | 4 |  | 32 | 32 | 42 |  |
| 746496 | 14 |  | 32 | 128 | 5 |  |
|  |  |  | 34 | 64 | 1 |  |
|  |  |  | 64 | 64 | 3 |  |
|  |  |  | 128 | 32 | 5 |  |
|  |  |  | 144 | 32 | 15 |  |

## HOP5

Figure 4 HOP filter architecture with attention block

| Add |  |  | Conv2D |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| size | count |  | size IN | size OUT | count |  |
| 6 | 1 |  | 1 | 16 | 3 | input |
| 16 | 4 |  | 3 | 16 | 1 | input |
| 32 | 24 |  | 3 | 192 | 1 | input |
| 48 | 24 |  | 3 | 32 | 1 | input |
| 64 | 48 |  | 32 | 80 | 23 |  |
| 80 | 23 |  | 48 | 64 | 24 |  |
| 192 | 24 |  | 64 | 6 | 1 | output |
| 262144 | 25 |  | 64 | 64 | 15 |  |
|  |  |  | 64 | 192 | 23 |  |
|  |  |  | 64 | 32 | 23 |  |
|  |  |  | 64 | 48 | 23 |  |
|  |  |  | 272 | 64 | 23 |  |
|  |  |  | 288 | 48 | 1 |  |

## Intra NN

## Super resolution models

| Add |  |  |  |
| --- | --- | --- | --- |
| size | count |  |  |
| 2 | 1 |  |  |
| 4 | 1 |  |  |
| 8 | 3 |  |  |
| 16 | 61 |  |  |
| 32 | 10 |  |  |
| 82944 | 12 |  |  |
| Conv2D |  |  |  |
| size IN | size OUT | count |  |
| 1 | 8 | 3 | input |
| 3 | 16 | 1 | input |
| 16 | 2 | 1 | output |
| 16 | 4 | 1 | output |
| 16 | 16 | 51 |  |
| 16 | 32 | 9 |  |
| 32 | 32 | 1 |  |
| 32 | 16 | 8 |  |
| 40 | 16 | 1 | comb. Inputs |

# Reference

- F.Galpin, E. Alshina, R.-L. Liao, S. Liu, A. Segall Common test conditions and evaluation procedures for neural network-based video coding technology JVET-AJ2016

- F.Galpin, E.Alshina AhG11/AhG14: Current NNVC models analysis JVET-AJ0361