| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 36th Meeting, Kemer, TR, 01–08 November 2024 | Document: JVET-AJ2030-v1 |
| --- | --- |

| Title: | Optimization of encoders and receiving systems for machine analysis of coded video content (draft 7) |  |  |
| --- | --- | --- | --- |
| Status: | Output document by JVET |  |  |
| Purpose: | Committee draft technical report |  |  |
| Author(s) or Contact(s): | Shan Liu Jie Chen Jacob Ström | Email: | shanl@global.tencent.com jiechen.cj@alibaba-inc.com jacob.strom@ericsson.com |
| Source: | Editors |  |  |

_____________________________

Items added to this version (draft 7):

- Text on chroma processing (to clause 8) JVET-AJ0168 (done)

- Post-processing algorithm (to annex A.3) JVET-AJ0178 (done)

- Combinations to new annex (to annex B) JVET-AJ0181, JVET-AJ0232, JVET-AJ0233 (done)

- Editorial change on RPR and aggressive downsampling (to clause 7.4) JVET-AJ0232 (done)

- Editorial change on useful combinations in the annex (to clause 5.1) JVET-AJ0233 (done)

- Non-monotonicity issues and curve fitting (to clause 6) JVET-AJ0235 (done)

- Temporal subsampling (to annex A.4) JVET-AJ0254 (done)

- Description of MIOU (to clause 6) JVET-AJ0254 (done, but put in Annex A.4)

ISO 23888-3:202#(1)

ISO/IEC JTC 1/SC 29/WG 5

Date: 2024-12-31

Information technology — Artificial intelligence for multimedia — Part 3: Optimization of encoders and receiving systems for machine analysis of coded video content

CDTR stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

A model document of an International Standard (the Model International Standard) is available at:
https://www.iso.org/drafting-standards.html

© ISO 2024

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	vi

Introduction	vii

1	Scope	1

2	Normative references	1

3	Terms and definitions	1

4	Abbreviations and acronyms	1

5	Overview	2

5.1	General overview	2

5.2	Background	3

5.3	Use cases and applications	3

6	Evaluation methodology	3

6.1	Bit rate	4

6.2	PSNR	4

6.3	mAP	4

6.4	MOTA	4

6.5	BD-rate	5

7	Pre-processing technologies	5

7.1	Region of interest-based methods	5

7.2	Foreground and background processing	6

7.3	Temporal subsampling	6

7.4	Spatial subsampling	7

7.5	Noise filtering	7

8	Encoding technologies	7

8.1	RoI-based quantization parameter adaption	7

8.2	Quantization step adjustment for temporal layers	8

9	Post-processing technologies	9

9.1	Temporal resampling	9

9.2	Spatial resampling	9

9.3	Enhancement post-filtering	9

10	Metadata	9

10.1	Neural-network post-filter SEI	9

10.2	Annotated regions SEI	10

10.3	Object mask information SEI	10

10.4	Encoder optimization information SEI	10

Annex A (informative)  Software implementation examples	11

A.1	Region of interest-based adaptive QP	11

A.1.1	Introduction	11

A.1.2	Analyser	11

A.1.3	Encoder	11

A.2	Pre-processing of foreground and background	11

A.2.1	Introduction	11

A.2.2	Pre-analysis	12

A.2.3	Pre-processing	12

A.3	Enhancement post-filtering	12

A.3.1	Introduction	12

A.3.2	Network structure	13

A.4	Temporal resampling	14

A.4.1	Introduction	14

A.4.2	Pre-analysis	14

A.4.2.1	Frame-level MIOU Mf	14

A.4.2.2	Sequence-level MIOU Ms	15

A.4.2.3	Adaptive temporal resampling ratio decision	16

A.4.3	Down-sampling	16

A.4.4	Up-sampling	16

Annex B (informative) Combined software implementation examples	17

Bibliography	18

Foreword

ISO (the International Organization for Standardization) is a worldwide federation of national standards bodies (ISO member bodies). The work of preparing International Standards is normally carried out through ISO technical committees. Each member body interested in a subject for which a technical committee has been established has the right to be represented on that committee. International organizations, governmental and non-governmental, in liaison with ISO, also take part in the work. ISO collaborates closely with the International Electrotechnical Commission (IEC) on all matters of electrotechnical standardization.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of ISO documents should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

ISO draws attention to the possibility that the implementation of this document may involve the use of (a) patent(s). ISO takes no position concerning the evidence, validity or applicability of any claimed patent rights in respect thereof. As of the date of publication of this document, ISO [had/had not] received notice of (a) patent(s) which may be required to implement this document. However, implementers are cautioned that this may not represent the latest information, which may be obtained from the patent database available at www.iso.org/patents. ISO shall not be held responsible for identifying any or all such patent rights.

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Joint Technical Committee ISO/IEC JTC 1, Information technology, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

A list of all parts in the ISO/IEC 23888 series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Introduction

This document provides a description of optimization techniques of video encoders and receiving systems for machine analysis tasks performed on coded video content. It provides a concept-level overview of recent practices and references to technical articles that describe further details. It also provides comments on some technical aspects and identifies situations where caution is recommended to be taken when interpreting the results.

Information technology — Artificial intelligence for multimedia — Part 3: Optimization of encoders and receiving systems for machine analysis of coded video content

# Scope

This document provides summary information about optimizations for encoders and receiving systems for conducting machine analysis tasks on coded video content using video coding standards developed in the ITU-T SG 16 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/SC 29 Moving Picture Experts Group (MPEG) communities. This document does not represent an authoritative recommendation for how encoders and receiving systems can be optimized. It merely describes technologies that have recently been studied and demonstrated benefits to coding efficiency for a set of machine analysis tasks being tested. The described algorithms might not be applicable or appropriate for all possible machine analysis tasks.

# Normative references

There are no normative references in this document.

# Terms and definitions

For the purposes of this document, the following terms and definitions apply.

ISO and IEC maintain terminology databases for use in standardization at the following addresses:

- ISO Online browsing platform: available at https://www.iso.org/obp

- IEC Electropedia: available at https://www.electropedia.org/

3.1

machine consumption

Applying a machine analysis task such as object detection, instance segmentation or object tracking, etc. on coded video content

# Abbreviations and acronyms

For the purposes of this document, the following abbreviations and acronyms apply.

| AVC | Advanced Video Coding (Rec. ITU-T H.264 ¦ ISO/IEC 14496-10) |
| --- | --- |
| BD-rate | Bjøntegaard delta bit rate |
| CfE | call for evidence |
| CfP | call for proposals |
| CTU | coding tree unit |
| HEVC | High Efficiency Video Coding (Rec. ITU-T H.265 ¦ ISO/IEC 23008-2) |
| JVET | Joint Video Experts Team (of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29) |
| mAP | mean average precision |
| MOTA | multiple object tracking accuracy |
| MPEG | Moving Picture Experts Group (of ISO/IEC JTC 1/SC 29) |
| NNPF | neural-network post-filter |
| NNPFA | neural-network post-filter activation |
| NNPFC | neural-network post-filter characteristics |
| OMI | object mask information |
| PSNR | peak signal-to-noise ratio |
| QP | quantization parameter |
| RoI | region of interest |
| RPR | reference picture resampling |
| SEI | supplemental enhancement information |
| TID | temporal identifier |
| URI | uniform resource identifier |
| VCEG | Video Coding Experts Group (of ITU-T SG 16 WP 3) |
| VSEI | Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (Rec. ITU-T H.274 ¦ ISO/IEC 23002-7) |
| VVC | Versatile Video Coding (Rec. ITU-T H.266 ¦ ISO/IEC 23090-3) |
| Y′CBCR | colour space representation commonly used for video/image distribution, also written as YUV |
| YUV | colour space representation commonly used for video/image distribution, also written as Y′CBCR |

# Overview

## General overview

Most video processing systems consist of four main processing steps, as shown in Figure 1. This document describes technologies for optimization of encoders and receiving systems, such as pre-processing, encoding and post-processing for machine consumption. The decoding process, on the other hand, is fully specified in the respective Versatile Video Coding (VVC)[1], High Efficiency Video Coding (HEVC)[2] and Advanced Video Coding (AVC)[3] video coding standards, amongst others, and a decoder must fully comply to the intended profile and level to properly decode and output the reconstructed video samples from a given input bitstream.

input video

pre-processing

encoding

decoding

post-processing

machine consumption

General video coding and processing pipeline

An overview of the commonly used practices for evaluating encoder optimization technologies for machine consumption in the JVET and MPEG communities can be found in clause 6. Descriptions of pre-processing technologies can be found in clause 7. Encoder optimization technologies are described in clause 8 and post-processing technologies are described in clause 9. Metadata that is useful for machine consumption is described in clause 10.

It is noted that depending on specific use cases, the technologies outlined in this document can be implemented individually or in combination to optimize machine consumption performance within the constraints of the system capabilities. When employing multiple technologies simultaneously, it is important to consider that certain combinations can be impractical or infeasible due to inherent methodological constraints. Tested combinations of two or more technologies are listed in Annex B.

## Background

In July 2019 the requirements subgroup of ISO/IEC JTC 1/SC 29/WG 11 (MPEG) started exploration activities in the area of video coding for machines (VCM), with the initial goal of defining use cases, requirements and an evaluation framework. This work, which was continued by ISO/IEC JTC 1/SC 29/WG 2, resulted in a formal call for evidence (CfE) in January 2021. As the responses to this CfE showed evidence of improvements over existing coding standards, a call for proposals (CfP) was issued in April 2022.

Some responses to the CfP contained elements that can be used without modifications to existing video coding standards, and one response produced bitstreams compliant to the VVC standard with pure encoder optimization. To further study the use of existing video coding standards on a set of machine tasks, JVET decided to conduct a study on optimization of encoders and receiving systems for machine consumption in October 2022. The outcome of this study is reported in this document.

## Use cases and applications

There are various use cases and applications using encoded video that benefit from optimizing both encoders and receiving systems for machine consumption. Some of them are highlighted below:

- Surveillance: Considerable amount of bandwidth is needed to transmit high volume of data generated by large number of sensors. The number of sensors also has an impact on the computational load on the server side, as having to analyse the input from many sensors can become a huge burden. This can be eased by distributing the computation to the front-end devices.

- Intelligent transportation: A key aspect for vehicular applications is interoperability between not only vehicles from different vendors, but also the infrastructures of various locations. Connected vehicles are expected to play a significant role in future transport systems and the tremendous number of vehicles emphasizes the need of reducing the amount of data being transmitted between them to avoid overloading the network.

- Intelligent industry: One example in this area is visual content analysis, checking and screening. Machine automation is desirable for enhancing the efficiency.

A more detailed description of use cases can be found in subclause 6.5.2 of ISO/IEC TR 23888-1 [4] .

# Evaluation methodology

A set of assessment metrics are used for the evaluation of encoder and receiving systems optimization technologies for machine consumption. An overview evaluation framework is shown in Figure 2. Here the input video is encoded to generate a bitstream. This bitstream is then decoded, and the decoded video is used for the machine consumption. Note that in this diagram the “encoder” includes both pre-processing and encoding steps as in Figure 1, and the “decoder” includes both decoding and post-processing steps shown in Figure 1.

input video

encoder

decoder

output video

bit rate

ground truth

YUV PSNR calculation

machine consumption

task performance calculation

Evaluation framework and points of measurement

## Bit rate

The bit rate is determined based on the encoded bitstream and parameters of the input video such as frame rate and the number of total frames. The following equation (1) is applied to calculate the bit rate:

bitrate=8 × fileSizeInBytes × fpsnumFrames ×1000	(1)

## PSNR

Encoding for video distribution is ordinarily performed in the Y′CBCR domain (nicknamed YUV herein for brevity and ease of typing). For standard-dynamic range video, the distortion metric primarily used in the video coding standardization community has been the Peak Signal to Noise Ratio (PSNR). The following equations (2) and (3) is applied to calculate PSNR:

MSE=1m*ni=0n-1j=0m-1xi,j-yi,j2	(2)

PSNR=10*log10255≪bitdepth-82MSE	(3)

where bitdepth is the bit depth of the input video.

## mAP

The performance of object detection and instance segmentation tasks are measured by mean average precision (mAP). This metric indicates what percentage of objects are correctly identified by having sufficient overlap between the detected object and the ground truth as well as being assigned the correct object class. Then the share of correctly identified objects for each class is determined, and finally the score for each class is averaged. The calculation of mAP is as follows:

mAP=1numOverlapsi=1numOverlaps1numClassesii=1numClassescorrectObjectsiitotalObjectsii	(4)

Some commonly used variants of this metric are:

- mAP@0.5: For an object to be counted as correctly identified, the Intersection over Union (IoU) between the detected bounding box and the ground truth bounding box needs to be at least 0.5. Sometimes this variant of the mAP metric is also referred to as mAP50.

- mAP@[0.5:0.05:0.95]: In this variant a total of ten mAP scores with increasing IoU thresholds are calculated. The IoU threshold starts at 0.5 and increases by 0.05 after each iteration, until it reaches to the upper bound value 0.95. Once all ten scores are determined, the average of these scores is calculated to produce the final mAP.

## MOTA

Object tracking performance is measured by Multiple Object Tracking Accuracy (MOTA). This metric accounts for all object configuration errors made by the tracker, false positives, misses (true negative), mismatches, overall frames. The calculation of MOTA is as follows:

MOTA=1-tFNt+FPt+mmettgt	(5)

where FNt, FPt, mmet and gt are the number of false negatives, the number of false positives, the number of mismatch error (ID Switching between 2 successive frames), and the number of objects in the ground truth respectively at time t.

## BD-rate

To compare the performance of a technology against the reference, the well-known Bjøntegaard delta rate (BD-rate) metric [5] is used. Instead of using PSNR as the distortion metric as in human vision performance evaluation, machine consumption distortion metrics, e.g., mAP and MOTA, are used in machine BD-rate calculation.

The distortion measurement of machine consumption (e.g., mAP and MOTA) can sometimes be non-monotonic to the bit rate due to the characteristics of the machine analysis task and possible limitations of machine networks. Polynomial curve fitting is applied to ensure rate-distortion monocity and thus valid BD-rate calculation.

fx= b0x3+ b1x2+ b2x + b3	(4)

For a given polynomial function in (4), b0, b1, b2, and b3 are coefficients of the function, x is the input (bit rate) and f(x) is the output (quality). The following two constraints are invoked to ensure its monotonicity and convexity:

- the first order derivative of the polynomial (5) is positive in the given x range

f'x= 3b0x2 + 2b1x + b2 	(5)

- the second order derivative of the polynomial (6) is negative in the given x range

f''x= 6b0x + 2b1 	(6)

Parameters (b0,b1,b2,b3) in the polynomial function (4) are solved by Sequential Least Squares Programming (SLSQP) and applied to curve fitting.

NOTE: The minimal quality value of the fitted curve shall be no smaller than the minimal quality value of the original curve and the maximum quality value of the fitted curve shall be no greater than the maximum performance value of the original curve.

# Pre-processing technologies

## Region of interest-based methods

One often-used optimization method is region of interest (RoI)-based coding. Here the input video is analysed in some way and then the encoder can optimize the encoding towards machine consumption based on the analysis results. The analysis can be done using various methods, e.g., neural networks. An example of a pipeline that can be used for RoI-based approaches is shown in Figure 3.

Input video

Analysis

Encoder

Decoder

Machine consumption

Pipeline for RoI-based systems

In one implementation example, an object detection network is used to analyse the input data. This network produces a list of objects that can be found in the current picture. The information needed to describe each object includes the index of the picture in which the object can be found and the position of the object in the picture. Some networks might provide more information than this and the encoder might choose to select a subset of all objects by filtering based on for example the class of an object or the estimated likelihood of an object of the described class being at the described position. In a similar approach, a segmentation network can be used where the object is not described by a bounding box but by a segmentation mask indicating exactly which pixels belong to the object. The list produced during the analysis can then be used by the encoder, for example, to separate foreground and background with the purpose of encoding the foreground at a better quality and the background at a lower quality. One such encoding method is described in clause 8.1. In this example, the analysis does not change the input video, but directly forward it to the encoder.

In other RoI-based methods, the pre-processing might change the input video, for example, by applying different pre-processing methods on the foreground and background, or specific parts of the video, such as subsampling background area of the input video.

In one implementation example, an object segmentation network is used to analyse the input data first. The network produces a list of objects segmented with the object shapes in the current picture. The object shapes and positions could be represented, for example, by segmentation masks. More information such as the object class or the estimated likelihood of the object segment could also be provided by the network to identify the objects. Based on the object information, spatial complexity and temporal complexity can be derived and then RoI-based pre-processing on the input video can be adapted based on the spatial and temporal complexity. The spatial complexity here indicates the averaged object size which can be calculated by dividing the percentage of the area covered by the objects by the total number of the objects. Temporal complexity indicates the content changes between two pictures which can be calculated by various methods, for example, by taking the mean absolute difference of the collocated pixels in two pictures.

## Foreground and background processing

After pre-analysis that determines the foreground and background areas, one straightforward way to handle the background that is less critical to machine consumption is to “eliminate” it by setting the corresponding sample values to a constant value. However, some portions of the background samples, for example those immediately surrounding the foreground area, could still be useful for machine consumption. Therefore, the background regions relevant to the machine consumption can be preserved to a certain extent with low-pass filtering, such as a Gaussian filter with a sliding window, where the window size can be set based on the input video resolution.

Moreover, extracted features can reveal importance information of the input video. In other words, compared with binary classification of foreground and background, these extracted features can provide importance information at a finer granularity. Therefore, such extracted features can be used to determine how to process foreground and background differently. In one implementation example, a feature map is extracted by a feature extraction network and based on the feature map, the parameters of a Gaussian smooth filter are adapted and then the adaptive filtering is applied on the picture. As the background area and foreground area have different features and even within the background or foreground area, different regions can have different features, the Gaussian smooth filter can be controlled at a finer granularity, which finally results in a more efficient pre-processing.

An implementation example with more detailed description can be found in Annex A.2.

## Temporal subsampling

In some use cases, for example when the frame rate is high, a way to reduce the bit rate without a strong negative impact on the machine consumption performance is to skip certain frames and encode the video at a lower frame rate. One example is to remove every other frame from the input video and encode the video at half frame rate. This can be done in a dynamic manner, for example by evaluating the motion between two or more frames and if there is only little motion, a frame can be removed. In some cases, if the receiving system requires a specific frame rate, a corresponding post-processing technology that up-samples the video to the full frame rate can be applied.

An implementation example with more detailed description can be found in Annex A.4.

## Spatial subsampling

If the analysis of a video shows that it contains primarily large objects, one way to improve the BD-rate performance is to perform spatial subsampling on the input video. This will result in fewer pixels in the subsampled frames to be encoded, and thus likely lead to faster encoding and bit rate savings. The optimum downscaling factor is content dependent. It is possible that the machine consumption performance drops when subsampling is too aggressive. Therefore, it is advisable to apply spatial subsampling adaptively, for example, based on the characteristics of the video content (such as the averaged object spatial area and the number of objects) and the target bit rate. Moreover, the spatial subsampling can also be dependent on the picture types. For example, depending on whether the input video is captured by regular camera as natural scenes, or is captured by infrared sensor as thermal images, different spatial subsampling methods can be applied.

One tool specified in the VVC standard that can be used for the purpose of spatial resampling is called reference picture resampling (RPR). This tool allows the encoder to choose to encode some pictures of the input video at different resolutions. For example, based on the analysis of keyframes, without encoding an intra picture at the specified changed resolution, inter predictions can be made from all allowed pictures regardless of their resolution.

In one implementation example, the RPR tool can be used at the frame level, where the input video can be analysed as described in clause 7.1. In this case, the unmodified input video is forwarded to the encoder with a scale factor list generated by the analyser. Specifically, an object detection network is used to analyse the input video in both full resolution and at least one spatially subsampled resolution. This network produces a list of objects with object information that can be found in the current picture and the spatially resampled picture. The object information describes the position and size of the detected objects for  both the current picture and the spatially resampled picture. Based on the object information, an object occupancy distribution (i.e., the distribution of the ratio of object size to the corresponding picture resolution) can be generated for both the current picture and the spatially resampled picture. The scale factor can be derived for the current picture by comparing the correlation of object occupancy distributions, and then the list of all scale factors is passed on to the encoder for utilization of the RPR tool.

## Noise filtering

Under some circumstances, various types of noise can be present in the video contents. Denoising filters can be applied on these contents to reduce unnecessary bit rate increases and avoid machine consumption performance degradation by filtering out the undesirable noise from the video content while preserving information important to machine consumption. Various types of denoising filters can be applied according to the characteristics of the noise. The strength of the filter can be adjusted based on the noise, and the filter can adaptively be enabled and disabled for either an entire picture or sequence, or only a part thereof.

It is noted that some existing denoising filters such as bilateral or anisotropic diffusion filters can preserve local details during denoising. For applications that need such local details, filtering entire picture using the same strength needs to be carefully considered.

# Encoding technologies

NOTE: The quality of chroma versus luma may need to be addressed when encoding in 4:2:0 colour format as many machine analysis tasks are performed on 4:4:4 colour format data and thus performance may be negatively impacted by reduced chroma quality. The use of a negative chroma QP offset may improve performance of 4:2:0 encoding.

## RoI-based quantization parameter adaption

One method that is available in many video coding standards is adaptive quantization parameter (QP). Here the encoder can change the QP value at a sub-picture level, for example the coding tree unit (CTU) level, to optimize the encoding for the application. Due to this versatility, adaptive QP can be used in many different use cases to improve performance without any changes to the decoding process.

The decision on where to change the QP value and by how much can be made by the encoder based on an analysis of the input video. Another option is to utilize the output of an external analyser such as described in clause 7.1. In this case, the encoder receives information about the positions and sizes of objects in each frame to make a differentiation between foreground and background.

One option is to use the base QP value of the picture for areas that contain objects, i.e., foreground areas, and an increased QP value for background areas, resulting in fewer bits being used to encode the background. As the background is not critical to the machine consumption, this is a straight-forward way to reduce the bit rate. As an extension, it can also be beneficial to encode large objects with slightly higher QP values. As it is generally easier to detect larger objects, reducing the bit rate for large objects usually does not reduce the performance of the machine consumption.

However, it is noted that when utilizing the analysis, complexity can be traded against bit rate. If a light-weight neural network is used to perform the analysis, the encoder is recommended to take into account that not all relevant objects might be found and thus it could be detrimental to reduce the quality for the background too much as there might be objects that the initial analysis missed. These objects might still be important for the machine consumption and if the background is encoded in sufficient quality, the machine consumption network has a chance of detecting objects in the background even if they are coded in lower quality than the foreground area. On the other hand, if the encoding system has a lot of resources, it can employ a neural network of higher complexity for the analysis. With a better and more certain analysis, the bit rate for the background can be reduced more as there are likely fewer objects that have been missed in the initial analysis.

A more detailed description with a link to an implementation can be found in Annex A.1.

## Quantization step adjustment for temporal layers

It is a common practice that the encoder places different pictures on different temporal layers, i.e., assigning different temporal identifiers (TID). This has the purpose to create hierarchical structures that indicate from which previously coded pictures the encoder can create predictions for the current picture. One aspect of these hierarchical structures is that pictures cannot be referenced by other pictures on a higher temporal layer. This way, not every decoded picture needs to be stored in the decoded picture buffer. Another aspect of the hierarchical structure is that pictures on higher temporal layers can be encoded with higher QP values, i.e., lower quality, as they will be not or more rarely used as references by other pictures. An example of a hierarchical structure is shown in Figure 4. The display order is left-to-right, and the numbers specify the order of the coded pictures in the bitstream.

An example hierarchical referencing structure in a VVC random access bitstream

This hierarchy of pictures can be exploited by encoding pictures on higher temporal layers using higher QP values, i.e., reducing the number of bits spent on these pictures in total. This is also done in the case of the common test conditions for standard definition range for VVC. In the use case of coding video content for machine consumption, this characteristic can further be exploited by increasing the QP value for pictures on higher temporal layers more. Taking advantage of motion compensation, many bits can be saved when compressing pictures in high temporal layers while these pictures are still able to be reconstructed with high quality.

# Post-processing technologies

## Temporal resampling

When the receiving system requires a specific frame rate that is different from the frame rate of the decoded sequences, temporal resampling can be applied on the decoded video by utilizing conventional temporal filters (e.g., motion compensated interpolation filters) or neural network-based filters, or just frame repetition. An implementation example with more detailed description can be found in Annex A.4.

## Spatial resampling

Spatial resampling can for example be applied on the decoded video by utilizing conventional spatial filters (e.g., VVC interpolation filter or VVC reference picture resampling filter) or neural network-based filters.

## Enhancement post-filtering

Video coding introduces quality degradation to the original video, which can consequently reduce the performance of the machine consumption. To improve machine consumption performance, the receiver can introduce a machine-oriented post-filtering network to enhance the decoded video before feeding it to the machine consumption networks. Such a machine-oriented post-filtering network can be fixed at the receiver side, i.e., without requiring any signalling in the coded bitstream or metadata. An illustration of this process is shown in Figure 5.

decoder

enhancement post-filter

machine consumption

bitstream

decoded video

filtered video

The processing order for an enhancement post-filter without additional signaling

Alternatively, the information of the post-filtering network can be signalled as metadata, for example using the SEI messages described in clause 10.1. In this case, the encoder can train the post-filtering network specifically for the machine consumption conducted in the decoder side if the machine consumption network is known on the encoder side. An illustration of this process is shown in Figure 6. An example of an enhancement post-filter is described in Annex A.3.

bitstream

decoder

enhancement post-filter

machine consumption

decoded video

filtered video

decoded metadata

The processing order for an enhancement post-filter with metadata signaling

# Metadata

## Neural-network post-filter SEI

The neural-network post-filter (NNPF) characteristics (NNPFC) SEI message and neural-network post-filter activation (NNPFA) SEI message are specified in version 3 of VSEI [6]. These SEI messages can be used for the purpose of machine consumption.

The NNPFC SEI message describes the characteristics of a neural network that can be used to filter pictures after decoding and such filtering can be beneficial for machine consumption. This SEI message might contain an indication of the purpose of the post-filter such as general visual quality improvement, chroma up-sampling, resolution resampling, picture rate resampling, bit depth up-sampling, or colorization. Note that the specific design of post-filters is not restricted by the SEI messages and post-filters that are suitable for the desired purposes can be used and described by this SEI message. The filter parameters of the neural network can be signalled in the NNPFC SEI message itself or could be identified by a uniform resource identifier (URI).

The NNPFA SEI message is used to enable or disable a post-filter specified by a referenced NNPFC SEI message on a particular picture.

Several different post-filters can be specified with multiple NNPFC SEI messages. In this case, the NNPFA SEI message can be used to indicate which of the available post-filters is to be applied on the current picture. By using several NNPFA SEI messages, the usage of different post-filters on the same picture can be indicated.

## Annotated regions SEI

The annotated regions SEI message specified in VSEI [6], HEVC [2] and AVC [3] can be used to signal parameters of the bounding boxes to identify annotated regions containing the detected objects. Attributes about the detected objects can also be optionally signalled, including a label for the object and the confidence level associated with the tracking and detection. This SEI message also contains a flag that indicates whether a coded bitstream is not optimized for human viewing but encoded with machine consumption as the target, and a flag that indicates if the motion information (motion vectors, modes) was selected in order to accurately track object motion rather than to optimize coding efficiency. As object detection and tracking is computationally expensive, the annotated regions SEI message signalled with the coded bitstream allows the receiving systems to separate the objection detection or tracking functionality from the decoding of video.

## Object mask information SEI

The object mask information (OMI) SEI message specified in version 4 of VSEI [6] can be used to signal information about the shape of detected objects; this contrasts with the annotated regions SEI message described in clause 10.2, which can only signal rectangular bounding boxes of detected objects. When using the OMI SEI message to signal the object shape information, the object shapes are signalled as auxiliary pictures using the capability provided by VVC multilayer coding. The primary pictures contain the coded video content, and the auxiliary pictures associated with the respective primary pictures contain the coded object masks that indicate the shapes of the respective objects. The OMI SEI message provides information about the mask identifier and the sample values for each of the object masks in the auxiliary pictures. Similar to the annotated regions SEI message, attributes about the object can be optionally signalled in the OMI SEI message, including a label and the depth for the object and the confidence level associated with tracking and detection. As object detection and tracking are computationally expensive tasks, the OMI SEI message and the associated auxiliary pictures signalled with the coded primary pictures can alleviate the receiving systems of the computational burden to detect and track the object based on the decoded video.

## Encoder optimization information SEI

The encoder optimization information SEI message specified in version 4 of VSEI [6] can be used to indicate if the video has been optimized for machine consumption and which types of optimizations have been applied in pre-processing or encoding. Different optimizations can be indicated with this SEI message, for example object-based optimizations, temporal or spatial resampling optimizations, temporal quality optimizations causing a quality fluctuation temporally, spatial quality optimizations, or optimizations regarding privacy aspects. It is also possible to indicate that combinations of these optimizations have been applied.

- (informative)

Software implementation examples

  - Region of interest-based adaptive QP

    - Introduction

An implementation example of the method described in this section can be found in the branch “JVET-AB0275” in a software repository hosted by JVET. The software codes can be found in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm/-/tree/JVET-AB0275?ref_type=heads.

This implementation example consists of two main parts: the analyser that is run before the encoder, and the modified VTM encoder.

    - Analyser

The analyser is an object detection network that evaluates each frame to determine the positions of objects. In this implementation, the YOLOv3 network has been used for practicality reasons, but it is asserted that the same method can be applied using nearly any other object detection network. The output of the network is dumped into a text file, which is then used as an additional input to the encoder.

    - Encoder

The default VTM encoder has been extended with a new module to handle the generation of the adaptive QP map that indicates what offset to use for which CTU. In this module the information provided by the analyser is evaluated and for each CTU the overlap with objects is determined. Based on the overlap, a CTU can be assigned to one of three possible categories:

- No overlap: if the CTU has no overlap with any objects, it is treated as a background CTU. This allows the encoder to increase the QP offset by a large value as there are no objects of interest in this CTU and thus no information gets lost by reducing the quality.

- Overlap only with large objects: objects are pre-sorted into large and small objects, which is determined by comparing the size of the area covered by the bounding box of the object to a pre-set threshold. If all objects the CTU overlaps with are large objects, the QP offset can be increased by an intermediate value as large objects are generally easy to detect and thus the detection does not suffer by reducing the quality slightly.

- Overlap with at least one small object: using the same sorting based on the size of the bound box as mentioned above, if a CTU has an overlap with at least one small object, the quality needs to be preserved as small objects in many cases are quite challenging for machine consumption. In this implementation the QP value is not increased for this category of CTUs.

Each category of CTUs corresponds to a value for the QP offset. This offset is then signalled using the delta QP mechanism that is part of the VVC standard.

  - Pre-processing of foreground and background

    - Introduction

An implementation example of the method described in this section can be found in the branch “JVET-AC0086” in a software repository hosted by JVET. The software codes can be found in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm/-/tree/JVET-AC0086?ref_type=heads.

This implementation example consists of two main parts: the pre-analysis and the pre-processing modules, as shown in Figure A.1.

    - Pre-analysis

The pre-analysis is an analysis module to extract the critical information of input pictures for machine consumption. Specifically, in this implementation, an instance segmentation network is adopted to evaluate each input picture I and generate the masks of instances. Then the instance masks are merged as a merged mask M. Meanwhile, in order to achieve a fine-grained machine-oriented importance map, the feature maps F in the instance segmentation network are extracted and fused (e.g., averaged and normalized) with those from all other channels to form an average feature map Fa. The merge mask M and the average feature map Fa output by the pre-analysis module are sent to the following pre-processing module.

Instance segmentation network

Input picture I

Feature map Fa

Merge mask M

IBlur

Block mask MB

IBlur, Block

Pre-analysis

Pre-processing

The framework of implementation example of the pre-processing of foreground and background

    - Pre-processing

According to the merge mask M and the average feature map Fa, the pre-processing module further processes the input pictures. Blurring the background is an intuitive method to remove redundancy and maintain the machine consumption performance. Herein, the blurred picture IBlur is obtained by blurring the background of the input picture I sample by sample with a Gaussian filter, of which the standard deviation is set to the corresponding sample value in the average feature map Fa. Moreover, a block mask MBlock is derived from M, indicating the important regions to the machine consumption. The representation compactness can be further improved by removing the unimportant regions based on MBlock. As such, the final result is formulated as IBlur,Block=MB⨀IBlur, where ⨀ indicates the element-wise multiplication with MB across the channels of IBlur.

For pre-processing of video data, a group of pictures can be processed using the same feature map Fa and the same mask M and MBlock in order to provide temporal stability.

  - Enhancement post-filtering

    - Introduction

An implementation example of the method described in this section can be found in the branch “JVET-AJ0178” in a software repository hosted by JVET. The software codes can be found: https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm/-/tree/JVET-AJ0178?ref_type=heads.

    - Network structure

The structure of the post-filter network in shown as Figure A.2, in which a U-Net architecture with parallel mirrored skip connections and multi-resolution modules is adopted to support multi-granularity post-processing. The network consists of feature extraction module and feature recovery module.

The feature extraction module is used to extract the features of the input picture Ir. Three base blocks connected with each other through a down-sampling module are used to sequentially output features F'd1,F'd2, and F'd3. Each base block is composed of four convolution layers, a depth-separable convolution layer, the Gaussian error linear units (GELU), layer normalization, channel attention layer and shortcuts. The detailed structure of the base block is given in Figure A.2. And the channel number of the base block and the down-sampling module is also marked in the Figure A.2. After feature extraction, the extracted feature {F'd1,F'd2,F'd3} are fed to feature recovery module. In the feature recovery module, {F'd1,F'd2,F'd3} are merged with channel-wise concatenation to output the processed picture Ip.

The structure of implementation example of the enhancement post-filter network

  - Temporal resampling

    - Introduction

An implementation example of the method described in this section can be found in the branch “JVET-AJ0254” in a software repository hosted by JVET. The software codes can be found in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm/-/tree/JVET-AJ0254?ref_type=heads.

This implementation example consists of three main parts: the pre-analysis, down-sampling and up-sampling. The framework of the implementation is shown in Figure A.3. The video data {Fi} is first input into a pre-analysis module to determine the resampling ratio f (f is a positive integer number) and then a temporal down-sampling module is applied on the input data {Fi} according to the resampling ratio f. The down-sampled data {Fi}D is then encoded and decoded with VVC codec. After decoding, the decompressed data {Fi}D' is up-sampled by a temporal up-sampling module to get up-sampled video data {Fi}'. The resampling ratio f can be signalled with the encoder optimization information (EOI) SEI message mentioned in section 10.4.

The framework of implementation example of the temporal resampling

    - Pre-analysis

The pre-analysis module is used to pre-analysis the input videos to adaptively determine the temporal resampling ratio f based on the temporal complexity of the input videos.

To describe the pre-analysis algorithm, a metric named mean intersection of union (MIOU) is introduced. There are two MIOUs used in the algorithm. The first one is frame level MIOU, denoted as Mf, and the second one is the sequence level MIOU, denoted as Ms. The sequence level MIOU is derived based on the frame level MIOU. The derivation of these two MIOUs is described in the following sections.

      - Frame-level MIOU Mf

Given a frame pair (F1,F2), the object detection task is respectively conducted on the two frames of this pair to get two results, denoted as

B1=b1,i=x1,i1,y1,i1,w1,i1,h1,i1,c1,i1,s1,i1  i1 is the index of the detected bounding box in frame F1 }

B2=b2,i=x2,i2,y2,i2,w2,i2,h2,i2,c2,i2,s2,i2  i2 is the index of the detected bounding box in frame F2},

where x and y are the horizontal and vertical coordinates of the top-left corner of the bounding box, w and h are the width and height of the bounding box, c is the class index of the object covered by the bounding box and s is the confidence score with range of 0 to 1. Herein, the bounding boxes with confidence score under 0.5 are removed.

Then, for each detected bounding box b1,i in B1, a matched bounding box b2,  j in B2 is identified by checking the object class and the value of intersection of union (IOU) which is defined as the ratio of intersection area of the two bounding boxes to the union area of the two bounding boxes. Herein, for a bounding box b1,i in B1, if there is no matched bounding box found in B2, b1,i is removed. By doing this, the bounding boxes in B1 are paired with the bounding boxes in B2, which can be denoted as

P=(b1,i, b2,  j)  b1,i∈B1, b2,  j∈B1 and b1,i is matched with b2,  j}

Based on the area of the bonding box (for simplicity, only the area of the first bounding box is checked), the paired bounding box P can be classified into three groups which can be denoted as

P=P0∪P1∪P2

P0=b1,i, b2,  j  area of b1,i<32×32

P1=b1,i, b2,  j 96×96>area of b1,i≥32×32

P2=b1,i, b2,  j area of b1,i≥96×96

For each group of the paired bounding box Pl (l =0, 1 or 2), the weighted averaged IOU is first calculated for each object class as follows

IOUk,l=b1,i, b2,  j∈Pl(IOU of b1,i, b2,  j) ×(s1,i+s2,j)b1,i, b2,  j∈Pl(s1,i+s2,j)

where b1,i, b2,  j is the bounding box pair of the object with class index k in Pl, s1,i is confidence score of bounding box b1,i and s1,i is confidence score of bounding box b2,  j.

And then the mean of IOU (MIOU) is calculated by averaging the weighted averaged IOU of each object class k as follows

Ml=averageIOUk,l

where Ml is the MIOU for paired bounding box group Pl (l =0, 1 or 2). Herein, average(x) is a function to get the averaged value of variables x.

Finally, the frame-level MIOU Mf ,which is defined as a three-dimension vector, is obtained as follows

Mf=(M0, M1, M2)

      - Sequence-level MIOU Ms

In order to accommodate with different coding structures and temporal resampling ratios, the sequence-level MIOU Ms is encoding configuration and resampling ratio dependent. First, frame pairs are selected according to coding structure and temporal resampling ratio, and then frame-level MIOU Mf is calculated for each of selected frame pair. And finally, based on the frame-level MIOU Mf, the sequence-level MIOU Ms is derived. As Mf is a three-dimension vector, the Ms is also a three-dimension vector. Herein, the calculations described in this section are vector operations.

For random access configuration, if resampling ratio is 2, the frame pairs are selected with POC distance 8, 16 and 32. In this implementation example, the selected frame pairs are F0,F8, F8,F16, F16,F24 and F24,F32 for POC distance 8, F0,F16 and F16,F32 for POC distance 16, and F0,F32 for POC distance 32. For each POC distance, the averaged frame-level MIOU Mf can be calculated, denoted as Mf,8, Mf,16, and Mf,32. That is, Mf,8 is the averaged frame-level MIOU of frame pair F0,F8, F8,F16, F16,F24 and F24,F32; Mf,16 is the averaged frame-level MIOU of frame pair F0,F16 and F16,F32; Mf,32 is the frame-level MIOU of frame pairF0,F32. Then, the sequence-level MIOU Ms is derived as Ms=0.5Mf,32+0.3Mf,16+0.2Mf,8. If resampling ratio is 4, the frame pairs are selected with POC distance 16 and 32. In this implementation example, the selected frame pairs are F0,F16, F16,F32 for POC distance 16 and F0,F32 for POC distance 32. The sequence-level MIOU Ms is derived as Ms=0.625Mf,32+0.375Mf,16.

For low-delay configuration, if resampling ratio is 2, the frame pairs are selected with POC distance 8. In this implementation example, the selected frame pairs are F0,F8, F8,F16, F16,F24 and F24,F32. The averaged frame-level MIOU of these frame pairs is calculated, denoted as Mf,8. The sequence-level MIOU Ms is set equal to Mf,8. If resampling ratio is 4, the frame pairs are selected with POC distance 16. In this implementation example, the selected frame pairs are F0,F16 and F16,F32. The averaged frame-level MIOU of these two frame pairs is calculated, denoted as Mf,16, and the sequence-level MIOU Ms is set equal to Mf,16.

For all intra configuration, if resampling ratio is 2, the frame pairs are selected with POC distance 2. In this implementation example, the selected frame pairs are F0,F2,F2,F4,…,F28,F30 and F30,F32. The averaged frame-level MIOU of these frame pairs is calculated, denoted as Mf,2. And then the sequence-level MIOU Ms is set equal to Mf,2. If resampling ratio is 4, the frame pairs are selected with POC distance 4. In this implementation example, the selected frame pairs are F0,F4,F4,F8,…,F24,F28 and F28,F32. The averaged frame-level MIOU of these frame pairs is calculated, denoted as Mf,4, and then the sequence-level MIOU Ms is set equal to Mf,4.

      - Adaptive temporal resampling ratio decision

After derivation of the sequence-level MIOU Ms=(Ms0, Ms1, Ms2), the decision metric MD is derived as follows.

MD=n0Ms0+n1Ms1+n2Ms2n0+n1+n2

where nl is the sum of confidence score of each matched bounding box pair in the respective paired bounding box group Pl (l =0, 1 or 2).

Given an encoding configuration, the decision metric for resampling ratios 2 and 4 are calculated respectively, denoted as MD,2 and MD,4. If MD,2 is larger than 0.5 and MD,4 is larger than 0.6, the resampling ratio is determined as 4; if MD,2 is larger than 0.5 and MD,4 is less than or equal to 0.6, the resampling ratio is determined as 2; otherwise, the resampling ratio is determined as 1 which means temporal resampling is not applied.

    - Down-sampling

In this implementation example, the down-sampling is performed by directly removing the frames. Suppose the resampling ratio is f, then for each f consecutive frames, f-1 frames are removed from the video sequence.

    - Up-sampling

In this implementation example, the up-sampling is performed by duplicating the frames. Suppose the resampling ratio is f, then each frame is duplicated f-1 times.

- (informative)
Combined software implementation examples

Some tool combinations have been tested and demonstrated complimentary coding benefits. These tool combination examples are listed in Table 1.

Combined software implmentaton examples

| Technology 1 | Technology 2 | Technology 3 |
| --- | --- | --- |
| Adaptive QP (Clause 8.1 and Annex A.1) | Temporal layer QP offset (Clause 8.2) |  |
| De-noising filter (Clause 7.5) | Temporal layer QP offset (Clause 8.2) |  |
| Pre-processing (Clause 7.2 and Annex A.2) | NNPF (Clause 9.3 and Annex A.3) |  |
| 4:4:4 coding (Clause 8) | Reduced resolution (Clauses 7.4 and 9.2) |  |
| Pre-processing (Clause 7.2 and Annex A.2) | NNPF (Clause 9.3 and Annex A.3) | Adaptive QP (Clause 8.1 and Annex A.1) |
| NNPF (Clause 9.3) | Adaptive QP (Clause 8.1 and Annex A.1) |  |
| NNPF (Clause 9.3) | Reduced resolution (Clauses 7.4 and 9.2) |  |

Bibliography

- Rec. ITU-T Recommendation H.266 | ISO/IEC 23090-3, Information technology — Coded representation of immersive media — Part 3: Versatile video coding

- Rec. ITU-T Recommendation H.265 | ISO/IEC 23008-2, Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 2: High efficiency video coding

- Rec. ITU-T Recommendation H.264 | ISO/IEC 14496-10, Information technology — Coding of audio-visual objects — Part 10: Advanced video coding

- ISO/IEC TR 23888-1, Information technology — Artificial Intelligence for multimedia — Part 1: Vision and scenarios

- ISO/IEC TR 23002-8 | ITU-T HSTP-VID-WPOM, Information technology — MPEG video technologies — Part 8: Working practices using objective metrics for evaluation of video coding efficiency experiments

- Rec. ITU-T Recommendation H.274 | ISO/IEC 230002-7, Information technology — Versatile supplemental enhancement information messages for coded video bitstreams