| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 42nd Meeting, Santa Eulària, ES, 24 April – 1 May 2026 | Document: JVET-AP_notes_d6 |
| --- | --- |

| Title: | Meeting Report of the 42nd Meeting of the Joint Video Experts Team (JVET), Santa Eulària, ES, 24 April – 1 May 2026 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contacts): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-second meeting during 24 April – 1 May 2026 at Palacio de Congresos de Ibiza, Santa Eulària, Spain. The meeting was held as a hybrid meeting, where remote participation was provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twenty-third meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0900 CEST (0700 UTC) on Friday 24 April 2026. Meeting sessions were held on all days including the weekend days of Saturday and Sunday 25 and 26 April 2026, until the meeting was closed at approximately XXXX hours CEST on Friday 1 May 2026. Approximately XXX people attended the JVET meeting, and approximately XXX input documents (not counting crosschecks, reports, and summary documents), 18 JVET AHG reports, 1 report of the Joint AHG on Gaussian splat coding, 2 EE summary reports, X BoG report(s), and X incoming liaison document(s) were discussed. The meeting took place in coordination with a meeting of various SC29 Working Groups and Advisory Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body, under whose auspices this JVET meeting was held (also mention co-located meeting of WP3/21). The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of the joint EE on Gaussian splat coding, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development. In the latter context, the development of the Final Draft Joint Call for Proposals on video compression with capability beyond VVC was a major activity at his meeting.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the forty-first JVET meeting in producing the following documents:

a) JVET documents

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AO1016 AVC with extensions and corrections (Draft 5)

JVET-AO1017 Support for additional VSEI messages in AVC (Draft 5)

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23)

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8)

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 388

JVET-AO2027 Common Test Conditions (CTC) for gaming applications

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10), also issued as WG 5 2nd DTR N 387

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11)

JVET-AO2040 Complexity reporting template for coding algorithms and tools

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026

b) documents produced as WG 5 documents only:

WG 5 N 384 Preliminary disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 385 Disposition of comments received on DIS ISO/IEC 23090-15:202X (3rd edition)

WG 5 N 386 Disposition of comments received on ISO/IEC DTR 23888-3

WG 5 N 390 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 391 Liaison statement to SMPTE on requesting SEI mechanism for new SMPTE ST 2094-60 standard

As main results, the JVET produced XX output documents from the current meeting (update):

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AO1016 AVC with extensions and corrections (Draft 5)

JVET-AO1017 Support for additional VSEI messages in AVC (Draft 5)

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23)

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8)

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 388

JVET-AO2027 Common Test Conditions (CTC) for gaming applications

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10), also issued as WG 5 2nd DTR N 387

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11)

JVET-AO2040 Complexity reporting template for coding algorithms and tools

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026

- The following 5 documents were produced as WG 5 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 384 Preliminary disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 385 Disposition of comments received on DIS ISO/IEC 23090-15:202X (3rd edition)

WG 5 N 386 Disposition of comments received on ISO/IEC DTR 23888-3

WG 5 N 390 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 391 Liaison statement to SMPTE on requesting SEI mechanism for new SMPTE ST 2094-60 standard

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. Another Joint AHG on Gaussian splat coding was established together with WG 4 and WG 7. At this meeting, X Exploration Experiments (EE) on video coding aspects, and a Joint EE on Gaussian splat coding (together with WG 4 and WG 7) were defined. The next ten JVET meetings were planned for 7 – 15 July 2026 under ITU-T SG21 auspices in Geneva, CH; during 17 – 23 October 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN; during 13 – 22 January 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU; during 17 – 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN; during January 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; during 25 March – 5 April 2028 under ITU-T SG21 auspices in Geneva, CH; during July 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during October 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. The most recent versions of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site, whereas all uploaded versions are also available from http://wftp3.itu.int/av-arch/jctvc-site/ and http://wftp3.itu.int/av-arch/jct3v-site/, respectively. JVET documents are also provided via http://wftp3.itu.int/av-arch/jvet-site/, but as this is currently not maintained as a mirror with the jvet-experts.org site, documents are typically only made available with a delay of two or three meeting cycles, to avoid that late uploads of output documents would be missing.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-second meeting during 24 April – 1 May 2026 at Palacio de Congresos de Ibiza, Santa Eulària, Spain. The meeting was held as a hybrid meeting, where remote participation was provided on best-effort basis for experts who were unable to travel.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

- Joint exploration (with WG 4 and WG 7 of SC 29) on Gaussian splat coding

JVET had been tasked by the parent bodies to develop a call for proposals on video compression with capability beyond existing standards, with plan to issue a first draft call from the current meeting.

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, consisting of two parts, (B1) in-person attendees as recorded by a sign-in sheet circulated in meeting rooms, (B2) remote attendees as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2026_04_AP_SantaEularia/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the forty-first JVET meeting in producing the following output documents:

a) JVET documents

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

JVET-AO1016 AVC with extensions and corrections (Draft 5)

JVET-AO1017 Support for additional VSEI messages in AVC (Draft 5)

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23)

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8)

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 388

JVET-AO2027 Common Test Conditions (CTC) for gaming applications

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10), also issued as WG 5 2nd DTR N 387

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11)

JVET-AO2040 Complexity reporting template for coding algorithms and tools

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026

b) documents produced as WG 5 documents only:

WG 5 N 384 Preliminary disposition of comments received on ISO/IEC 14496-10:2025/DAM 1

WG 5 N 385 Disposition of comments received on DIS ISO/IEC 23090-15:202X (3rd edition)

WG 5 N 386 Disposition of comments received on ISO/IEC DTR 23888-3

WG 5 N 390 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 391 Liaison statement to SMPTE on requesting SEI mechanism for new SMPTE ST 2094-60 standard

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the UTC time, except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair, or other experts that were appointed to chair parallel sessions. It is indicated who took the notes in review of a given category (or individual documents în a category), and by which day and time the review was conducted. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 17 April 2026. Any documents uploaded after 1159 hours Paris/Geneva time on Saturday 18 April 2026 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AP0236 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AP0XXX (a proposal on …), uploaded 04-XX,

- …

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AO0054 (a contribution on …), uploaded 04-XX,

- …

Almost all cross-verification reports at this meeting (except for JVET-AP0XXX, …) were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AP0141, JVET-AP0198, JVET-AP0203, JVET-AP0206, JVET-AP0208, JVET-AP0209, JVET-AP210, JVET-AP0258, JVET-AP0259, ….

The following cross-verification report(s) were still missing by the end of the meeting, but were uploaded later: JVET-AP0XXX, …. The following report(s) had not become available yet three weeks after the end of the meeting: JVET-AP0XXX, … . These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete (paragraph kept for future use).

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did not apply to any contributions. (did apply to document(s) JVET-AP0XXX, being initially empty of results and flagged as late in the list above, based on the time of the first reasonable document upload | sentence kept for future use).

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AO1000, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AO1004, the AVC with extensions and corrections (draft 5) JVET-AO1016, the Support for additional SEI messages in AVC (draft 5) JVET-AO1017, the Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) JVET-AO2002, the VTM and HM common test conditions and evaluation procedures for HDR/WCG video JVET-AO2011, the Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC) JVET-AO2019, the Verification test plan for VVC multilayer coding (update 8) JVET-AO2021, the Description of the EE on Neural Network-based Video Coding JVET-AO2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AO2024, the Draft Joint Call for Proposals on video compression with capability beyond VVC JVET-AO2026, the Common Test Conditions (CTC) for gaming applications JVET-AO2027, the Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10) JVET-AO2030, the Technologies under consideration for future extensions of VSEI (version 11) JVET-AO2032, the Complexity reporting template for coding algorithms and tools JVET-AO2040, and the Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026 JVET-AO2041, had been completed and those which were available were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible (kept for future use). The software implementations of VTM version 23.14 and 24.0, ECM version 19.1, and NNVC version 16.0 were also approved.

It was further noted that some editorial modifications were necessary on the WG 5 versions of standard deliverables, to be compliant with the ISO/IEC rules.

Only minor editorial issues were found in the meeting report JVET-AO1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annexes B1 and B2 of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts, and through the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- It was pointed out the according to ISO rules, any recording or transcription of zoom sessions is not permitted. Participants who are performing recording will be removed from the session unless they stop recording upon request.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Approval of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities, and Joint AHG on Gaussian splat coding

Report of exploration experiment on neural-network-based video coding (EE1)

Report of exploration experiment on enhanced compression beyond VVC capability (EE2)

Report of joint EE on Gaussian splat coding (JEE 6.9)

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on software development in exploration activities

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on gaming content compression

Consideration of contributions on generative face video compression

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on assessment of implementation complexity of video coding technology

Consideration of contributions on aspects of ultra-low latency and packet loss resilience in the context of video compression

Consideration of contributions and planning of next steps towards a Call for Proposals on future video coding standardization, development of a Final Draft Call for Proposals, and definition of criteria for technology assessment

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting (see under section 2.15). Typical meeting hours were expected to be 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the online JVET calendar in advance as far as possible, but it was agreed that some activities (such as breakout sessions) could be held at short notice.

## ISO, IEC and ITU-T Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

Participants were also reminded of the UN Codes of Conduct, applying to work under ITU-T, found at https://www.un.org/management/sites/www.un.org.management/files/un-system-model-code-conduct.pdf

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that also the most recent versions of JCT-VC and JCT-3V documents can be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) has also been provided via the ITU ftp site, providing a “documents” subfolder in the directory of each respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1389 (as of 23 April 2026). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – video coding standard developed by JVT, formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BIF: Bilateral filter

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardization

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted for study of coding technology

- CfE: Call for Evidence

- CfP: Call for Proposals

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CICP: Coding-independent code points (Rec. ITU-T H.273 | ISO/IEC 23091-2)

- CIIP: Combined inter/intra prediction

- CL-RAS: Cross-layer random-access skip

- CNN: Convolutional neural network

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DRF: Deep reference frame (for NNVC inter prediction)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EE: Exploration experiment – a coordinated experiment conducted for study of coding technology prior during exploration work prior to establishment of a formal project (EEn for some number n is exploration experiment number n)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase and test model algorithm for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FGRC: Film grain regions characteristics

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- GS: Gaussian splatting

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a software codebase and video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for NNLF.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC 3D video extensions)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (originally joint video exploration team, initially for VVC, later expanded in scope)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- kMAC: thousands of multiply-accumulate operations – used as a complexity metric

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LB-CCP: Local-boosting CCP

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for NNLF.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MM-CCCM: Multi-model CCCM

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MOS: Mean opinion score (a metric for subjective quality evaluation)

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MSE: Mean-squared error, a distortion metric

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNLF: Neural network loop filter

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display)

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code

- operation point: A temporal subset of an OLS

- PDPC: Position-dependent (intra) prediction combination

- PERP: Padded equirectangular projection (a 360° projection format)

- PH: Picture header

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format)

- PMMVD: Pattern-matched motion vector derivation

- POC: Picture order count

- PoR: Plan of record

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set

- PSNR: Peak signal-to-noise ratio, a fidelity metric derived from MSE

- PTL: Profile/tier/level combination

- QM: Quantization matrix

- QP: Quantization parameter (sometimes conflated with quantization step size)

- QT: Quadtree

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD)

- RADL: Random-access decodable leading (type of picture)

- RASL: Random-access skipped leading (type of picture)

- R-D: Rate-distortion

- RDO: Rate-distortion optimization

- RDOQ: Rate-distortion optimized quantization

- RDPCM: Residual DPCM

- ROI: Region of interest

- ROT: Rotation operation for low-frequency transform coefficients

- RPL: Reference picture list

- RPLM: Reference picture list modification

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC

- RPS: Reference picture set

- RQT: Residual quadtree

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q)

- RVM: Rate variation measure

- SAD: Sum of abolute differences, a measurement of distortion

- SADL: Small adhoc deep learning librar

- SAO: Sample-adaptive offset

- SBT: Subblock transform

- SbTMVP: Subblock based temporal motion vector prediction

- SCIPU: Smallest chroma intra prediction unit

- SCM: Screen content model – a software codebase and test model algorithm for the HEVC screen content coding extensions

- SD: Slice data; alternatively, standard-definition

- SDH: Sign data hiding

- SDT: Signal-dependent transform

- SE: Syntax element

- SEI: Supplemental enhancement information

- SGPM: Spatial geometric partition mode

- SH: Slice header

- SHM: Scalable HM – a software codebase and test model algorithm for SHVC

- SHVC: Scalable high efficiency video coding

- SIF: Switchable (motion) interpolation filter

- SIMD: Single instruction, multiple data

- SMVD: Symmetric MVD

- SPO: SEI processing order

- SPS: Sequence parameter set

- SR: Super-resolution

- STMVP: Spatial-temporal motion vector prediction

- STRP: Short-term reference picture

- STSA: Step-wise temporal sublayer access

- TALF: Temporal adaptive loop filter

- TBA/TBD/TBP: To be announced/determined/presented/public

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content

- TIMD: Template-based intra mode derivation

- TM: Template matching

- TMRL: Template-based multiple reference line prediction

- TMVP: Temporal motion vector prediction

- TS: Transform skip

- TSRC: Transform skip residual coding

- TT: Ternary tree

- TuC: Technology under consideration

- UGC: User-generated content

- UWP: Unequal weight prediction

- VCEG: Visual coding experts group (ITU-T Q.6/21, the relevant rapporteur group in ITU-T WP3/21, which is one of the two parent bodies of the JVET)

- VCM: Video coding for machines

- VLOP: Very low-complexity operating point for NNLF

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy

- VQA: Visual quality assessment

- VT: Verification testing

- VTM: VVC Test Model – a software codebase and test model algorithm for the VVC standard

- VUI: Video usability information (part of an SPS, similar to an SEI message that applies to a video sequence)

- VVC: Versatile Video Coding, video coding standard developed by JVET, formalized by ITU-T as Rec. ITU-T H.266 and by ISO/IEC as ISO/IEC 29090-3

- WCG: Wide colour gamut

- WG: Working group, a group of technical experts (usually used to refer to the WGs of ISO/IEC JTC 1/SC 29)

- WPP: Wavefront parallel processing (usually synonymous with ECS)

- Block and unit names in MPEG-2 and AVC:

  - MB: Macroblock – a 16x16 array for the luma component and corresponding 8x8 arrays for the chroma components (basically the same as a CTU in later standards, but with a fixed size)

  - MB pair: A pair of vertically contiguous macroblocks in a frame that is coupled for use in macroblock-adaptive frame/field decoding

  - Sub-MB: One quarter of the samples of a macroblock, i.e., an 8x8 luma block and two corresponding chroma blocks of which one corner is located at a corner of the macroblock for a picture that has three sample arrays or an 8x8 luma block of which one corner is located at a corner of the macroblock for a monochrome picture or a picture that is coded using three separate colour planes.

  - Sub-MB partition: A block of luma samples and two corresponding blocks of chroma samples resulting from a partitioning of a sub-macroblock for inter prediction for a picture that has three sample arrays or a block of luma samples resulting from a partitioning of a sub-macroblock for inter prediction for a monochrome picture or a picture that is coded using three separate colour planes

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU)

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions)

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape

  - PB: Prediction block, a luma or chroma block of a PU

  - PU: Prediction unit, has the same size as a CU in the VVC context

  - TB: Transform block, a luma or chroma block of a TU

  - TU: Transform unit, has the same size as a CU in the VVC context

## Standards, TRs, supplements and technical papers approval and publication status (update)

- MPEG-2 | H.262 (the video coding specification is common text and not publicly available without cost per ISO special request)

  - ITU-T H.262 V3 was approved in 2012-02; its Amd 1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

    - Cor 1:2007, Cor 2:2009, Cor 3:2012 (publication availability unclear, as these do not appear to be directly available from the ISO website) may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) final ballot closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13 (ITU work programme link)

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15

    - ITU-T H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13

    - ISO/IEC 14496-10:2025 (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, FDIS ballot issued 2025-01-14, FDIS ballot closed 2025-03-23; published 2025-07-10

  - ISO/IEC 14496-10:2025/DAM 1 to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, ready to issue FDAM or FDIS at current meeting if desired, but it was agreed during the meeting to defer such action to a future meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting, targeting consent in 2026-07 (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004 (Ed. 2), including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software (twin text)

    - ITU-T H.264.2 V6 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (with shutter interval information SEI message and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM1 issued 31st meeting 2023-07, DAM issued with new (multiview) profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - ITU-T H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2:2025 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, FDIS ballot issued 2024-12-30, FDIS ballot closed 2025-02-24, published 2025-03-20

  - ISO/IEC 23008-2:2025/DAM 1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consultation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.265 (V11) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 during the current meeting (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software (twin text)

    - ITU-T H.265.2 V3 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, FDIS issued from 2016-02 meeting, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, FDAM issued from 2017-04 meeting, FDAM ballot opened 2017-08-10, closed 2017-10-07, published 2017-11-09

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initiated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

    - H.266 (2023) Erratum 1 published 2025-05 to correct errors in a few figures

  - ISO/IEC 23090-3:2024/DAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.266 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22 during the current meeting (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

    - ISO/IEC 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, ready for FDIS or to proceed directly to publication at current meeting; it was agreed during the meeting to proceed directly to publication – see Annex C (ISO Projects link)

    - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 during the current meeting (ITU work prgramme link)

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - ITU-T H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:2025 (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, closed 2025-06-01, approved to proceed to publication without an FDIS at 29th meeting 2025-07, public availability requested 2025-07, published 2025-11-19, not available without cost. (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12

  - ISO/IEC 23002-7:202x (Ed. 4) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.274 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending pre-publication (ITU work programme link)

- Post-meeting note: Pre-publication occurred on 2026-02-04.

- CICP (twin text)

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07

  - ISO/IEC 23091-2:2025 (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, FDIS ballot issued 2024-12-03, FDIS ballot closed 2025-01-28, published 2025-02-25

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - ITU-T H-Series Supplement H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - ITU-T H-Series Supplement H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08

- Usage of video signal type code points (twin text)

  - ITU-T H-Series Supplement H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4:2021 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - ITU-T H-Series Supplement HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8:2021 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T H-Series Supplement H.Sup21 V1 (ex H.Sup-FGST), Agreement 2025-01-24 by ITU-T SG21, pre-publication 2025-03-26, published 2025-05-21

  - ISO/IEC TR 23002-9 (Ed. 2) Edition 2 planned but not yet in formal work programme of ISO/IEC

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC TR 23888-3 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closed 2025-11-03, summary of voting in m75361, ready for action at current meeting if desired, but it was agreed during the meeting to issue a second DTR ballot – See Annex C (ISO Projects link)

  - ITU-T provisional name H.Sup.MACVC (ITU work programme link)

- Freely available ISO/IEC standards (without cost) in the ISO Webstore and IEC Webstore include

  - ISO/IEC 13818-4:2004 Conformance testing for MPEG-2 (confirmed available 2025-10-31)

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed available 2025-10-31)

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2 (confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-4:2004 specify conformance for AVC (base text confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) specify reference software for AVC (base text confirmed available 2025-10-31)

- Intended to be freely available (without cost) but unavailable (without cost) standards and technical reports include the following. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 14496-10:2025 (Ed. 11) AVC, published 2025-07-10 (public availability request to be confirmed)

  - ISO/IEC 23002-7:2024 (Ed. 3) VSEI, published 2024-10-30 (public availability requested 2023-07)

  - ISO/IEC 23008-2:2025 (Ed. 6) HEVC, published 2025-03 (public availability requested 2024-04)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01 (public availability requested 2016-02, reiterated 2025-10)

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09 (public availability not requested 2017-01, 2017-04, 2017-07, 2017-10, 2018-01, public availability requested 2025-10)

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06 (public availability requested 2018-10, reiterated 2025-10)

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15 (public availability requested 2025-10)

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04

  - ISO/IEC 23090-16:202x (Ed. 2) Reference software for VVC, not yet published, public availability requested 2025-07

  - ISO/IEC 23091-2:2025 (Ed. 3) CICP, published 2025-02-25 (public availability requested 2024-04, reiterated 2025-10)

  - ISO/IEC TR 23008-14:2018 (Ed. 1) Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics, published 2018-08-06, public availability requested 2025-07

  - ISO/IEC TR 23008-15:2018 (Ed. 1) Signalling, backward compatibility and display adaptation for HDR/WCG video coding, published 2018-08, public availability requested 2025-07

  - ISO/IEC 23090-16:2025 (Ed. 2) Reference software for VVC, public availability requested 2025-07

  - ISO/IEC TR 23091-4:2021 (Ed. 3) Usage of video signal type code points, published 2021-05-23, public availability requested 2025-07

  - ISO/IEC TR 23002-8:2021 (Ed. 1) Working practices using objective metrics for evaluation of video coding efficiency experiments, published 2021-05-20, public availability requested 2025-07

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Film grain synthesis technologies for video applications, published 2024-07-24 (not requested 2024-04), public availability requested 2025-07

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that public availability for some was never requested, e.g. for those that were produced by JCT-3V.

## Draft standards progression status for active work items (update)

- AVC ISO/IEC 14496-10:2025/DAM 1 to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, ready to issue FDAM or FDIS at current meeting if desired, but it was agreed during the meeting to defer such action to a future meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting, targeting consent in 2026-07 (ITU work programme link)

- HEVC ISO/IEC 23008-2:2025/DAM 1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.265 (V11) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 (ITU work programme link)

- VVC ISO/IEC 23090-3:2024/DAM 1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

- Post-meeting note: FDIS registered for formal approval 2026-03-09

  - ITU-T H.266 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-22 (ITU work programme link)

- VVC conformance ISO/IEC DIS 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, pending closure 2025-06-30 during the current meeting, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in m75347, ready for FDIS or to proceed directly to publication at current meeting; it was agreed during the meeting to proceed directly to publication – see Annex C (ISO Projects link)

  - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pre-published 2026-01-19 (ITU work programme link)

- VSEI ISO/IEC 23002-7:2024 (Ed. 4) Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, DIS approved for registration as FDIS 2026-01-15, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.274 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending pre-publication (ITU work programme link)

- Post-meeting note: Pre-publication occurred on 2026-02-04.

- ISO/IEC TR 23002-9 (Ed. 2) Film grain synthesis technology for video applications – Edition 2 planned but not yet in formal work programme of ISO/IEC

- H.Sup.MACVC | ISO/IEC TR 23888-3 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC TR 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closes 2025-11-03, summary of voting in m75361, ready for action at current meeting if desired, but it was agreed during the meeting to issue a second DTR ballot – See Annex C (ISO Projects link)

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the ongoing work items when they become finalized.

## Opening remarks

Remarks during the opening session of the meeting Friday 24 April at 0930-XXXX CEST were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was slightly higher than in last meeting (approximately 200 vs. 185 by the time of opening), where an increase is observed in SEI related contributions, whereas other categories have decreased in number (in particular, EE2 has significantly less)

  - Parallel sessions were considered to be necessary (SEI and core coding aspects), starting Wednesday evening

  - Aspects of common interest (final draft CfP, non-SEI HLS, GS SEI, plans for VSEIv5) to be reviewed and discussed at plenary level

- Future non-virtual meetings will be held as hybrid meetings with best-effort remote access (when under ISO/IEC SC 29 auspices) or as physical meetings with remote participation (when under ITU-T SG21 auspices). In terms of practical arrangements, this is identical.

- Plans for subsequent non-virtual meetings were reviewed: July 2026 (Geneva), October 2026 (Hangzhou), January 2027 (Brisbane), April 2027 (Geneva), July 2027 (Tampere), October 2027 (Shenzhen), April 2028 (Geneva); meeting location for January 2028 is still to be determined.

- An AHG17 interim meeting was conducted in Aachen, DE, 25-27 February 2026. Good progress was made in preparation of CfP test conditions, with some follow-up investigation at current meeting – viewing planned to be conducted on Saturday, discussion this afternoon about material to be viewed.

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings, or plan for AHG meeting days prior to the regular meeting (e.g., for analysis of CfP submissions)

- It was mentioned that a problem occurred with the jvet.org site in early March, which caused the necessity of resetting all passwords of registered users. The security against attacks was improved in that context, but further actions to make sure that only active JVET members would be able to login for document registration and upload might become necessary.

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AO1000 were reviewed (which was finalized some days before the meeting, but sufficiently mature drafts had been available before in the ITU ftp site). Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- At the current meeting, on-site attendance was recorded via the traditional sign-in sheet. Participants were asked to correct their affiliation and email in case these changed. In the previous meeting, a few cases were found (mostly for those mentioned in the previous bullet point) where the affiliation was outdated. Remote attendance (of those not present in person) will be recorded via the zoom records. It is therefore important to follow the conventions of naming as mentioned before. Participants who cannot be correctly identified will not appear in the attendance sheet.

- There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a significant number of documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions. As for now, the JVET chair took action of correcting those cases in the document registry. Several documents that had been registered without title were marked as withdrawn by the chair when he intended to start categorization in preparing the meeting notes.

- Some general principles:

  - Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

  - Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site, as WG 5 docs (as that feature of the DMS site has not been working properly).

  - Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles such as “EEx contribution” shall be avoided.

- The following ballot result had become available through the SC 29 secretariat:

  - Summary of Voting on ISO/IEC 2nd DTR 23888-3 Optimization of encoders and receiving systems for machine analysis of coded video content mXXXXX

- The FDIS ballots for next editions of HEVC, VVC and VSEI will be started soon (all synchronized in timing).

- The FDIS of a new edition for 14496-10 should be produced at the current meeting, and also submission for ITU-T consent on the next versions of H.264 should be conducted, to be submitted to WP3/21 who meet on May 1. A preliminary DoCR WG 5 N 384 had already been produced from the last meeting. It might be good to start the FDIS ballot soon, to have still an overlap in timing with the ballots on HEVC, VVC and VSEI

- If the ballot on the 2nd DTR 23888-3 does not contain comments for technical changes again, it will be forwarded for publication, with editorial updates to be made by ISO editors. No DoCR would be necessary. To be clarified if the corresponding ITU supplement could be submitted from the current meeting, or if it would be better to wait until July to allow better alignment with 23888-3.

- The 3rd edition of VVC conformance 23090-15 was submitted for IS publication.

- Plans for generating the next versions of reference software for AVC, HEVC and VVC (which would need to support new SEI messages from VSEIv4) need to be developed. Earliest target date for ITU-T consent would be July 2026 – should requests be made for ISO? For AVC, it might further be useful to generate a new part of ISO/IEC 14496 containing the entire newest JM software at least, and cancel the corresponding outdated amendments of 14496-5.

- The primary goals of the meeting were:

  - Prepare AVC next version/edition

  - Final Draft CfP preparation, test conditions to be frozen, some alignment on VVC anchors/encodings, complexity analysis and other issues to be improved

  - TuC on VSEI (and CICP?)

  - Non-SEI HLS aspects

  - Gaussian splats – work of Joint AhG (meets Sunday), and Joint EE (was established only at an interim JAhG meeting)

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

  - Liaison communication – any?

  - VVC white paper update (remove VSEI, some update of the 2021 version)

- Joint meetings were expected with MPEG AG 5 (on matters involving visual quality assessment), and with MPEG WG 2 Requirements and ITU-T VCEG on future video standardization. Another joint meeting was expected Gaussian splatting with VCEG, WGs 2, 4 and 7.

- As a follow-up to communication conducted previously, parent bodies are making further considerations about future JVET management structures. As a step forward, SC 29 decided during its 48th meeting in February to appoint Mathias Wien as WG 5 convenor for the period 2027-2029. This means that (provided SG 21 would agree on this appointment), starting from January 2027 JVET will have two chairs (J.-R. Ohm from SG 21 side and M. Wien from SC 29 side).

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.15.

## Scheduling of discussions

The times of the meeting sessions followed the needs of the onsite meeting arrangements, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were in the range of 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were noted to potentially be necessary (sessions could start 0830 at earliest and end by 2100 when the rooms need to be closed). Sessions were announced in the JVET calendar as far as possible in advance, although it was acknowledged that some activities (such as breakout sessions) might be held at short notice.

Coffee breaks were regularly scheduled at 1030 and 1600.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Fri. 24 April, 1st day

  - Morning sessions:

    - 0900–1015 Opening remarks, review of practices, agenda, IPR policy reminder, scheduling

    - 1015–1230 Reports of AHGs 1-9,12

  - Afternoon sessions:

    - Plenary: main track:

      - 1400-1515 Reports of AHGs 10-11, 13-18

      - 1515–1615 Planning viewing of CfP test sequences

      - 1645–1745 Discussion about VSEIv5 plans

      - 1800–1945 Main track: Review of EE2 5.2.1/5.2.2/5.2.3

    - 1800-2000 Separate track on HLS: XXX (Chaired by J. Boyce / S. Deshpande)

- Sat. 25 April, 2nd day

  - Morning sessions:

    - Main track: EE2 5.2.4, EE1 5.1.1/5.1.2 etc.

    - HLS track: XXX

  - Afternoon sessions:

    - Main track: Remaining EE1 5.1.2 ff.

    - HLS track: XXX

- Sun. 26 April, 3rd day

  - Morning sessions:

    - 0900–1330 Viewing sessions on CfP sequences

    - 0900–1330 No JVET sessions during Joint AhG on Gaussian splats (AHG meets in JVET main room)

    - …

  - Afternoon sessions:

    - 1500–1800 Main track: Complexity assessment 4.8, 4.15; 4.16.2 first results of viewing

    - 1500–2000 HLS track: XXX

- Mon. 27 April, 4th day

  - 0900–1230 MPEG information sharing session

  - Afternoon sessions:

    - 1400–1545 JVET plenary: Coordination, report and open issues from tracks, scheduling/planning, plenary-level doc review 4.x

    - 1600–1800 Updates of CfP test conditions and text

    - 1600–1800 Joint meeting with WG 4, WG 7 and VCEG on Gaussian splats (Room P2+P3)

    - 1800–1930 Joint meeting with VCEG (until 1845) and AG 5: 4.4 metrics, 4.5 subjective quality / verification testing

    - 1800–2000 HLS track: XXX

- Tue. 28 April, 5th day

  - Morning sessions:

    - 0900-–1215 Main track: Updates of CfP

    - HLS in parallel

  - Afternoon sessions:

    - 1400-–1800 Main track: review of 4.12, 4.11, 4.14, 4.17. 4.10, updates of CfP rate points,  (from 1700 chaired by F. Bossen) 4.3, 4.7, 4.10

    - 1600–1800 Joint meeting with WG 4, WG 7 and VCEG on Gaussian splats (Room P2+P3)

    - HLS in parallel

- Wed. 29 April, 6th day

  - 0900–1030 MPEG information sharing session

  - Morning sessions:

    - 1045–1230 JVET Plenary:  Coordination, output planning, liaison review, issues from tracks, review remaining 4.x, AOB

    - …

  - No meeting 1230–1430 (WG 3 Systems demo in C2)

  - Afternoon sessions:

    - 1400–XX HLS

    - 1430–1800 4.17 Updates of CfP rate points, sequences, VVC encoding configurations;

    - 1600–1800 Joint meeting with WG 4, WG 7 and VCEG on Gaussian splats (Room P2+P3)

    - 1800–1900: Joint meeting with WG 2, VCEG, AG 5: Requirements for next generation video standardization & CfP presentation

- Thu. 30 April, 7th day

  - Morning sessions:

    - 0830–1030 Joint meeting with WG 4, WG 7 and VCEG on Gaussian splats: Define CTC and anchores, review AhG mandates and JEE descriptions (Room P2+P3)

    - 10:45–1230  Remaining doc review 4.18, liaison review, EE1 revisits, and main track revisits

    - HLS in parallel

  - Afternoon sessions:

    - 1400-1700 TBD

    - HLS

- Fri. 1 May, 8th day

  - 0900–1200 JVET plenary:

    - Remaining revisits

    - EE document review and approval

    - List of HLS adoptions

    - Establishment of AHGs

    - Approval of output docs (including review as needed)

    - Review of WG 5 meeting recommendations

    - Software timeline

    - Future planning, a.o.b.

  - 1400–XXXX MPEG information sharing session

  - XXXX–XXXX WG 5 approval of meeting recommendations, closing of meeting

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (19) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (2)

  - AHG2: Text development and errata reporting (0)

  - AHG3: Software development (3)

  - AHG3: Test conditions and metrics (1)

  - AHG4: Subjective quality testing and verification testing (2)

  - AHG4: Test and training material (0)

  - AHG5: Conformance test development (1)

  - AHG7: Tool assessment (2)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

  - AHG10: Encoding algorithm optimization (1)

  - AHG13: Film grain technology (1)

  - Implementation studies (2)

  - Profile/tier/level specification (2)

  - AHG15: Gaming content compression (2)

  - AHG16: Hardware implementation complexity (4)

  - AHG17: CfP on video coding technology beyond VVC (14)

  - AHG18: Ultra-low latency and error resilience (3)

  - Video CICP, generic aspects of metadata (2) TBP

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (24) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (17) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics (update numbers)

  - Aspects of SEI messages in VSEI, VVC, HEVC and AVC (4) (section 6.1)

  - Study SEI messages in TuC for VSEI (95) (section 6.2)

  - Potential needs for SEI messages on other topics (6) (section 6.3)

  - Usage of SEI messages for Gaussian splatting (14) (section 6.4)

  - SEI software and showcases (2) (section 6.5)

  - Non-SEI HLS aspects (0) (section 6.6)

- Joint meetings, plenary discussions, BoG reports (X) liaison (X), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (19)

These reports were discussed during 1015–1230 and 1400–1515 on Friday 24 April 2026 (chaired by JRO).

JVET-AP0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice chair)]

The reflector used for discussions by the JVET and all of its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

The number of subscribers (on the day before the beginning of the current meeting) was 1389 (compared to 1384 by the time of the previous meeting). Furthermore, the previous lists of joint teams (which were still kept open as archives) had the following number of subscribers:

- JCT-VC – 1162 subscribers

- JCT-3V – 680 subscribers

- JVT-experts – 2082 subscribers

It is likely that many subscriptions of these latter three reflectors would have become obsolete, as no emails are being sent over them any more, and therefore automatic removal of addresses causing permanent bounces by being invalid (as it happens with the main JVET reflector) cannot be executed by the reflector management system.

- Goals and activity

The work of the JVET overall had proceeded well in the interim period with increased number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET ftp site (http://wftp3.itu.int/av-arch/jvet-site/2026_01_AO_Virtual/). JCT-VC and JCT-3V documents can also be accessed directly via the JVET site. Previous meeting documents of JVT, JCT-VC, JCT-3V and JVET are also available from the ITU-based ftp site, where sub-folders of the different groups can be found under https://www.itu.int/wftp3/av-arch/, and sub-folders ‘./documents/’ were created in the folders of the respective meetings (it is noted that, for practicality reasons, for JVET this is only done after all meeting documents including output documents are completely available in all versions, such that there may be some delay in availability). All those files come with the original time stamps by which the original versions of documents were uploaded.

The list of output documents produced since the last meeting included the following:

- JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Posted 2026-02-27]

- JVET-AO1016 AVC with extensions and corrections (Draft 5) [Posted 2026-04-24]

- JVET-AO1017 Support for additional VSEI messages in AVC (Draft 5) [Posted 2026-03-16]

- JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) [Posted 2026-03-06]

- JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [Posted 2026-01-30]

- JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC) [Posted 2026-04-20]

- JVET-AO2021 Verification test plan for VVC multilayer coding (update 8) [Posted 2026-02-05, last update 2026-03-03]

- JVET-AO2023 Exploration experiment on neural network-based video coding (EE1) [Posted 2026-01-22, last update 2026-02-10]

- JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [Posted 2026-01-23, last update 2026-01-30]

- JVET-AO2026 Draft Joint Call for Proposals on video compression with capability beyond VVC, also issued as WG 5 N 388 [Posted 2026-02-05]

- JVET-AO2027 Common Test Conditions (CTC) for gaming applications [Posted 2026-01-23, last update 2026-01-29]

- JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10), also issued as WG 5 2nd DTR N 387 [Posted 2026-02-06]

- JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 11) [Posted 2026-03-13, last update 2026-04-13]

- JVET-AO2040 Complexity reporting template for coding algorithms and tools [Posted 2026-02-14, last update 2026-02-27]

- JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026 [Posted 2026-02-05]

The eighteen ad hoc groups had made progress, and reports from those activities had been submitted. A report from the Joint AHG on Gaussian splat coding (together with WG 4 and WG 7) is expected during the meeting. Numerous teleconferences had been held by AHGs. Furthermore, two exploration experiments (EE) on neural network-based video coding and on enhanced compression beyond VVC capability, as well as a joint EE on Gaussian splat coding (under the JAHG) were conducted.

The arrangements for the 42nd meeting had been announced in the JVET reflector, in the JVET logistics document https://www.itu.int/wftp3/av-arch/jvet-site/2026_04_AP_SantaEularia/JVET-AP_Logistics.docx, and in the WG 5 calling notice (N 393) and agenda (N 394) for the 23rd WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

More than 195 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, maintenance, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC and JCT-3V topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 42nd meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2026_04_AP_SantaEularia/JVET-AP_notes_d0.docx.

As a follow-up to communication conducted previously, parent bodies are making further considerations about future JVET management structures. As a step forward, SC 29 decided during its 48th meeting in February to appoint Mathias Wien as WG 5 convenor for the period 2027-2029. This means that (provided SG 21 would agree on this appointment), starting from January 2027 JVET will have two chairs (J.-R. Ohm from SG21 side and M. Wien from SC 29 side).

The WG 5 chair thanked experts for the very positive post-meeting feedback submitted to ISO.

- Recommendations

- The AHG recommends its continuation.

- It is recommended to appoint Mathias Wien as additional AHG1 co-chair.

JVET-AP0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice chairs)]

Output documents produced

- JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, and Video CICP, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AO meeting:

- For VVC:

  - Removed existing errata items, which have been addressed in VVCv4 texts.

  - In D.2.1, change "text_description( payloadSize )" to "text_description_info( payloadSize )".

  - JVET-AO0280: In D.2.2, added the following sentence: "The value of SeiExtensionBitsPresentFlag may be updated within a specific SEI payload syntax structure, e.g., film_grain_characteristics( payloadSize )."

  - Item 3 of JVET-AO0081: Added a constraint on po_sei_payload_type[ i ] and its relation to SeiProcessingOrderSeiList.

  - Item 1 of JVET-AO0101: Added the payloadType value of the TDI SEI message (219) into SeiProcessingOrderSeiList.

  - JVET-AO0280: Added a sentence to clarify that the value of SeiExtensionBitsPresentFlag may be updated within a specific SEI payload syntax structure, e.g., film_grain_characteristics( payloadSize ).

- For VSEI:

  - Item 1 of JVET-AO0081: Corrected reversed semantics of gfv_matrix_pred_flag and gefv_matrix_pred_flag.

  - Item 2 of JVET-AO0081: Modified the persistence scope of the TDI SEI message to replace "decoded picture" language with "picture unit and its decoded picture".

  - Item 3 of JVET-AO0081: Remove the constraint on po_sei_payload_type[ i ] and its relation to SeiProcessingOrderSeiList.

  - Item 5 of JVET-AO0081-v2: Added a NOTE for clarification of AURR signalling in processing chains and sub-chains of SPO SEI messages.

  - JVET-AO0280: Updated the NOTE in VSEI v4 saying that SEI messages extended in certain manner are not supported in AVC, such that that statement only applies to AVC versions until the extension mechanism is supported for newly specified SEI payload types.

  - In 8.36.1.1, added a missing space.

  - In 8.36.1.2, updated the semantics of gfv_matrix_present_flag.

  - In 8.36.2.2, updated the semantics of gefv_matrix_present_flag.

- For HEVC:

  - Item 3 of JVET-AO0081: Added a constraint on po_sei_payload_type[ i ] and its relation to SeiProcessingOrderSeiList.

- JVET-AO1016 AVC with extensions and corrections (Draft 5)

[not available at the time of preparing this AhG report]

- JVET-AO1017 Support for additional SEI messages in AVC (Draft 5)

This document contains the draft text for changes to the Advanced Video Coding (AVC) standard (Rec. ITU-T H.264 | ISO/IEC 14496-10). The changes include the support of five new SEI messages, namely the source picture timing information, modality information, digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7).

- Changes that have been integrated for the 41st JVET meeting in January 2026:

- JVET-AO0081 AHG9: Proposed corrections for miscellaneous aspects on VSEI V4

- JVET-AO0212 Extension mechanism for new AVC SEI

- DAM ballot comments

- JVET-AO0280: In D.2.2, added the following sentence: "The value of SeiExtensionBitsPresentFlag may be updated within a specific SEI payload syntax structure, e.g., film_grain_characteristics( payloadSize )."

- Related input contributions

Input contributions were noted as relevant to the work of this ad hoc group:

- JVET-AP0058 AHG2/AHG9: Some errata items on VVC, VSEI, HEVC, and/or AVC

- Remaining VVC spec tickets

Closed since JVET-AO0002 was reported:

- (none)

Carried over (strikethough means addressed in the indicated errata report and can be closed):

- #1629 mtt_split_cu_vertical_flag context uses undefined variable chType. (Discussed in JVET-AI1004)

- #1650 Incorrect indexing for the h location component in ALF process. (Discussed in JVET-AJ1004)

- #1654 Typo in derivation process for GPM motion vectors.

New (since JVET-AO0002 was reported)

- (none)

- Remaining HEVC spec tickets

Closed since JVET-AO0002 was reported:

- (none)

Carried over:

- (none)

New (since JVET-AO0002 was reported):

- (none)

- Recommendations

The AHG recommends to:

- Approve JVET-AO1004, JVET-AO1016, and JVET-AO1017 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2-related inputs and act on them if found to be necessary.

JVET-AP0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs)]

The software model versions prior to the start of the meeting were:

- VTM 24.0 (April 2026)

- VTM-22.2-TuC4.0 (Feb. 2025)

- HM-18.0 (Apr. 2023)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.1 (Apr. 2023)

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.26 (August 2025)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

- Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

Short before the 42nd JVET meeting denial of service was observed on the Fraunhofer HHI web servers, which required blocking large IP spaces. This may also affect legitimate access.

- VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued using the GitLab server. VTM version 23.14 was tagged on February 5, 2026, VTM version 24.0 was tagged on April 8, 2026.

VTM 23.14 was tagged on February 5, 2026. Changes include:

- JVET-AN0348: On reporting metrics for RPR

- Update CI pipeline settings

- Revert RPR side effects introduced by !2685

- Remove unnecessary load

- Fix and unify full-size allocation condition

- JVET-AN0058: On the NNPF SEI messages

- JVET-AN0060: On the OMI SEI message

- JVET-AN0237: film grain analysis

- JVET-AN0062 item 1: Generative AI usage restrictions

- PRI SEI: clean up code and fix picture output order

- Add 2026 to copyright date range

- fix: Deactivate HashMe for same POC with inter-layer reference (with scaleRatio = 1)

- Improved check for sps_palette_enabled_flag

- JVET-AO0288: Port temporal filter and BIM improvement from JVET-AN0267

- Fix DSC decoding for second verification period

- Fix a gcc-15.2 compiling issue

- fix Slice::decodingRefreshMarking() when encoding multilayer

- Fix WPSNR calculation

- Update version number to 23.14

VTM 24.0 was tagged on April 8, 2026. Changes include:

- Fix an issue with chroma QP table for CbCr

- Bug fix missing initialization in Shutter Interval SEI message

- Macro cleanup for VTM 23.14

- Update version to VTM-24.0 and PDF of software manual

The following merge requests were merged on top of VTM 24.0. A new version will be tagged as necessary:

- Fix deblocking corner case involving palette and dual-tree

- fix memory growth due to unused DecLib::m_nalUnitInfo

  - CTC Performance

In SDR CTC and high performance test conditions VTM 23.14 shows an improvement in coding performance compared to VTM 23.13, stemming mainly from the inclusion of “JVET-AO0288: Port temporal filter and BIM improvement”. No changes in coding time were observed.

For the HDR CTCs, coding performance of VTM 24.0 compared to VTM 23.13 are reported in the table below. Gains are observed from the fix made on the chroma QP table for CbCr which impacts class H1 and the temporal filter changes mentioned above.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM23.13 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.64% | -0.25% | -0.21% | -0.09% | 0.57% | -0.24% | -0.09% | 0.69% | 100% | 101% |
| Class H2 |  |  |  |  |  | -1.36% | -0.76% | -0.67% | 98% | 100% |
| Overall | -0.64% | -0.25% | -0.21% | -0.09% | 0.57% | -0.65% | -0.33% | 0.20% | 99% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM23.13 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.70% | -0.02% | -0.02% | -1.75% | -1.31% | -0.02% | -1.66% | -1.20% | 100% | 100% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | -0.70% | -0.02% | -0.02% | -1.75% | -1.31% | -0.01% | -1.06% | -0.76% | 100% | 101% |

For high bit depth CTCs, coding performance of VTM 24.0 compared to VTM 23.13 with the standard QP range is shown in the table below As with HDR CTCs, gains are observed. For the low QP range, VTM 23.14 produced identical coding results to VTM23.13, and partial results of VTM 24.0 show similarly identical results to VTM23.13.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM 24.0 over VTM 23.13 - Standard QP range |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.17% | -0.15% | -0.13% | 0.32% | 1.19% | -0.16% | 0.05% | 0.94% | 100% | 99% |
| Class H2 |  |  |  |  |  | -1.40% | -0.18% | -0.83% | 99% | 98% |
| Overall | -0.17% | -0.15% | -0.13% | 0.32% | 1.19% | -0.78% | -0.07% | 0.05% | 99% | 99% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | VTM 24.0 over VTM 23.13 - Standard QP range |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.94% | -0.02% | -0.03% | -2.13% | -1.69% | -0.03% | -2.14% | -1.78% | 101% | 102% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | -0.94% | -0.02% | -0.03% | -2.13% | -1.69% | -0.01% | -1.07% | -0.89% | 100% | 101% |

Coding performance of VTM 24.0 compared to HM 18.0 are reported in the table below.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -40.01% | -37.83% | -36.91% | -55.10% | -48.62% | -33.94% | -49.50% | -40.68% | 248% | 84% |
| Class H2 |  |  |  |  |  | -32.82% | -58.00% | -63.53% | 211% | 78% |
| Overall | -40.01% | -37.83% | -36.91% | -55.10% | -48.62% | -33.53% | -52.59% | -48.99% | 234% | 82% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over HM18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.86% | -27.33% | -26.81% | -58.52% | -53.52% | -24.01% | -53.77% | -45.81% | 1481% | 104% |
| Class H2 |  |  |  |  |  | -21.76% | -47.23% | -50.60% | 1186% | 95% |
| Overall | -41.86% | -27.33% | -26.81% | -58.52% | -53.52% | -23.19% | -51.39% | -47.55% | 1366% | 101% |

  - Issues in VTM affecting conformance

The following issues in VTM master branch may affect conformance:

- Missing HLS features (see sections below)

  - Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

  - SEI TuC software

Per the decision during the 32nd JVET meeting, an SEI TuC repository was created based on VTM-22.2. The repository is located at:

https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

It should be noted that some of the technologies for which merge requests were submitted to TuC, got moved into the working draft. The merge requests were closed, when appropriate merge requests for VTM were available.

The following merge requests were merged:

- Fix build on macos arm

- JVET-AK0153: Photosensitive content SEI messages

- Remove Packed Regions SEI (moved to VTM)

- JVET-AJ0245: Multi-layer support for Constituent Rectangles SEI

- JVET-AG0328: FGR SEI message

- JVET-AK0142: Display Rectangles SEI

The following merge requests were submitted and are pending:

- JVET-AM0086: Photosensitive content additional information (pending review)

- JVET-AL0219: Colour mapping information SEI (pending minor proponent updates)

- JVET-AI0340: Implementation of AI-restrictions usage SEI message (pending proponent updates)

VTM-22.2-TuC-5.0 is expected to be tagged during the 42nd JVET meeting.

- HM related activities

There was no new HM version tagged during this meeting cycle.

The following MRs were merged:

- JVET-AE0101: implement phase indication SEI message

- Update auto-build from VTM build definition

- Update copyright headers to include 2025

- JVET-AK0107: Modality Information SEI

- JVET-AK0194: Digitally Signed Content SEI messages

- Fix building on ARM

- Enable build on macOS/arm

- Modify semantics of IntraPeriod to enable autosetting based on frame rate

- Fix yuv output when bitdepth is changed between sequence

- Add missing initialization in TComPPS and TComScalingList (fix asan/msan errors)

- Avoid reading SEI content when payloadSize = 0

- Modified CMake for ARM variations

- Update build pipeline from VTM

- JVET-AK2006:Add support for SPTI SEI message

- Remove output picture marked as non reference from the DPB

- Fix build after merging !82

- JVET-AL0059: Packed regions information SEI

- JVET-AL0061: Import encoder optimization information SEI Message from VTM

- JVET-AN0237: Film Grain Analysis

- JVET-AL0148: implementation of GFV and GFVE SEI messages

- JVET-AL0339: Spatial resolution for film grain characteristics SEI message.

- Add 2026 to copyright date range

- Fix erroneously skipped pictures when decoding

- Fix condition to reset PicOrderCntMsb (could happen on CRA)

- Mark all pictures as to not be output when encountering a CRA with noRaslOutputFlag = 1

- Fix bumping process (add conditions on PicLatencyCnt, fixes for EOS before CRA cases)

- JVET-AL0062: AI usage restrictions SEI message

- Add support for NNPF SEI messages

- Merge DSC SEI code from VTM up to VSEIv4 syntax

The no MRs are pending for the main branch.

A new version of HM is pending to be tagged during or after the 42th JVET meeting. CTC need to be run to verify coding performance.

A new branch was created for 4:4:4/Multiview profile support:

https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tree/dev-multiview444?ref_type=heads

The following MRs were merged into the dev-multiview444 branch:

- JVET-AN0292-0293: Multiview HEVC encoding and decoding including 4:4:4 profiles

- Remove restriction of MR target branch for CI

The following MRs are pending:

- MV-HEVC 4:4:4 multi-view profile support and decoder compatibility fixes (pending review)

- HRD Initial CPB removal delay handling (based on above MR pending review)

- JVET-AO0227: Multivew 444 fixes (pending review)

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

Otherwise the HEVC bug tracker lists:

- 44 tickets for “HM”, (most of which were created before 2019; 1 created in last year),

- 1 ticket for “HM RExt” (created in 2020),

- 9 tickets for “HM SCC” (most of which were created before 2018),

- 1 ticket for “RExt Text” (created in 2015),

- 1 ticket for “SCC Text” (created in 2016),

- 8 tickets for text (6 created before 2021; 2 created in the last year),

- 2 tickets for encoder description (created in 2020 and 2014)

Help to address these tickets would be appreciated.

- 360Lib related activities

There had not been any further developments to 360Lib during this meeting cycle.

- SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

- SHM related activities

An update to HM 18.0 and some smaller changes were merged. A new tag is planned.

- HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

One merge request is was merged:

- JVET-AE0295: MV Main 10 profile support (waiting for proponent response)

The next release will also include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

- HDRTools related activities

There had not been any further developments to HDRTools during this meeting cycle.

- JM, JSVM, JMVM related activities

There had not been any further developments to JSVM or JMVM during this meeting cycle.

There was no new JM version tagged during this meeting cycle, although merge requests were submitted for SEI messages in VSEI.

The following MRs were merged:

- Update build servers aligned with HM

- JVET-AK0107: Modality Information SEI

- Update CI & CMake configuration to include current build setups

The following MRs are pending [with status indicated]:

- JVET-AK2006: NNPFC and NNPFA SEI message (in review)

- JVET-AL0062: AI usage restrictions SEI message (in review)

- JVET-AK2006: SPTI SEI Message (in review)

- JVET-AL0061: Encoder Optimization Information SEI Message (in review)

- JVET-AE0101: implement phase indication SEI message (in review)

- JVET-AL0148: Add support for Generative Face Video (GFV) and Generative Face Video Enhancement (GFVE) SEI messages(in review)

For the previous release of JM only basic testing was performed to confirm that the code still works, but performance testing is pending. Performance changes are unlikely but may occur due to bug fixes. A possible encoder/decoder mismatch was reported privately, but needs more investigation.

- Bug tracking

The creation of accounts to the Trac based bug trackers was closed due to excessive spam account registrations. It is planned to move to GitLab integrated bug trackers and keep the old ones only read only.

GitLab based tracking was tested for TuC and VSEI specification texts, which seems to work well.

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain.

https://hevc.hhi.fraunhofer.de/trac/hevc

New account creation was disabled due to a large amount of spam registration. Extensive maintenance of the Trac bug tracker would be required to keep it secure. It should be considered to move bug tracking to GitLab, although no automatic conversion of existing bugs is available.

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

- CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-AL2010		VTM/HM 4:2:0 test conditions

JVET-AN2011		VTM/HM HDR test conditions

JVET-AA2018	VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100	HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009	SHVC test conditions

JVET-AC1015	SCM test conditions

JVET-AE1013		3DV test conditions

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

- Guidelines for reference software development

Current versions of the software guidelines are:

- Guidelines for JM software development are included with the software

- JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

- JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

- Related proposals

JVET-AP0243 proposes bug fixes and profile support for MV-HEVC software.

- Recommendations

The AHG recommends to:

- Continue to develop reference software.

- Improve documentation, especially the software manual.

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

- Design and add configuration files to the VTM software for testing of HLS features.

- Review VTM-related contributions and determine whether features should be added (or removed) from the software.

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

It was suggested to develop a plan for the next version of HEVC reference software, supporting the new multiview profiles (except 4:4:4 which is not in the standard at this moment), and the new VSEI messages. Multiview profile support is still in a separate branch. Preferentially, a new version of H.265.2 might be submitted for ITU consent in the July meeting.

JVET-AP0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

- Activities

- Preparations for the Joint Call for Evidence on video compression with capability beyond VVC

AHG4 supported the activities of AHG17 and prepared for the on-site testing activity.

  - VVC multilayer verification tests

The visual tests for the VVC multilayer verification tests have been completed. The results are reported in JVET-AP0268.

- Test sequences

The set of JVET test sequences is hosted at https://vqa.lfb.rwth-aachen.de. A mirror of this site is available at https://datacloud.hhi.fraunhofer.de with the same login credentials. The directory structure of the previous ftp server has been maintained. The test sequences used for CfP/CTC are available in directory “/ctc”.

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e., members of ISO/IEC MPEG and ITU-T VCEG). Members of JVET may contact the JVET chair for login information. The password for the site is updated after each meeting with the Zoom password for that meeting. The tentative schedule for the changes is as follows:

- After the April meeting, password changes on 1st of June

- After the July meeting, password changes on 1st of September

- After the October meeting, password changes on 1st of December.

- After the January meeting, password changes on 1st of March.

- Related contributions

(listing document directly related to AHG4 as well as documents related to testing activities and test material)

| JVET number | Title | Source |
| --- | --- | --- |
| JVET-AP0054 | [AHG4/AHG17] VQQ: An Interactive Display Assessment Tool for Subjective Tests | J. Liu, Z. Chen (Wuhan Univ.) |
| JVET-AP0186 | AHG17/AHG4: A Generic Full-Reference Objective Quality Assessment Method for Compressed Video | J. Wang, X. Zhuang, J. Zhang, L. Yu (Zhejiang Univ.) |
| JVET-AP0268 | Viewing test results of verification test for VVC multilayer coding | M. Wien, V. Baroncini |

- Recommendations

The AHG recommends:

- To take into account the test sequences proposed to AHG17 in the preparation of the CfP on video compression with capability beyond VVC.

- To prepare a verification test report for multilayer VVC based on JVET-AP0268.

- To continue to collect new test sequences available for JVET with licensing statement.

- To collect volunteers to conduct further verification tests and subjective quality tests.

- To collect volunteers to actively contribute to the verification test development.

Joint meeting with AG 5 on the three documents planned for next week.

It was suggested to add a recommendation that JVET thanks the companies who supported the multi-layer verification test.

JVET-AP0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice chairs)]

- Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between 41st and 42nd meetings.

- Timeline

The progress on the Conformance testing specification is proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15

  - ISO/IEC FDIS 23090-15:202x 2nd edition text output of 2023-01, preparation delayed to 2023-09, FDIS ballot opened 2024-04-08, FDIS ballot closed 2024-06-03, published 2024-07-04

  - H.266.1 V2 Consent 2023-07, Last Call began 2023-08-16, Approved 2023-09-13, pre-published 2023-10-06, published 2023-10-19

- VVCv3 conformance:

  - CD: 2025-04-15

  - DIS: 2025-07-25

  - DIS ballot closed 2025-12-22

  - FDIS: 2026-02-20

  - IS: 2026-06-30

  - H.266.1 V3 Consent 2025-10-17, Last Call began 2025-12-01, Last Call ended 2026-01-12, Approved 2026-01-16, pending prepublication.

- Status on bitstream submission

The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - No changes between the 41st and 42nd meeting.

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 128 bitstreams of 57 identified categories have been provided, cross-checked and made available

  - No changes between the 41st and 42nd meeting.

- additional conformance bitstreams for VVC Multilayer:

  - 3 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 7 total bitstreams have been provided, cross-checked, and made available

  - No changes between the 41st and 42nd meeting.

- conformance bitstreams for new HEVC Multiview profiles

  - 2 HEVC Multiview Extended 10 bitstreams have been provided, cross-checked, and made available, no changes between the 41st and 42nd meeting.

  - 2 HEVC Multiview Extended bitstreams have been provided, cross-checked, and made available, no changes between the 41st and 42nd meeting.

  - 2 HEVC Multiview Main 10 bitstreams have been provided, cross-checked and made available, no changes between the 41st and 42nd meeting.

- Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 3.

VVC activities:

The streams are available at ​ https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

VVC operation range extensions activities:

The streams are available at ​ https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

VVC Multilayer activities:

The streams are available at ​ https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

HEVC Multiview supporting extended bit depth activities:

The 6 packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/  No change in bitstreams and/or packages.

The HM MR https://vcgit.hhi.fraunhofer.de/jvet/HM/-/merge_requests/98 has been merged in the HM dev-multiview444 branch (https://vcgit.hhi.fraunhofer.de/jvet/HM/-/tree/dev-multiview444). The code changes in this MR extend the HM encoder and decoder to support all the HEVC multiview profiles - Multiview Main, Multiview Main 10, Multiview Extended, Multiview Extended 10, and the Multiview Monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16, collectively referred to as the multiview format range extensions profiles) - and to enable HM to support multiview encoding with other chroma formats, such as 4:4:4.

The HM dev-multiview444 branch can be used to generate and cross-check Multiview Monochrome profiles conformance streams, and to validate the MR extensive code changes (23k+ lines) in preparation for merging them into the HM main branch.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

- Contributions

JVET-AP0243 Bug Fixes and Profile Support for MV-HEVC Reference Software (HM) [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)].

JVET-AP0244 MV-HEVC Conformance Bitstreams for Multiview Extended Profiles [A. M. Tourapis, D. Podborski, J. Kim (Apple)].

- Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it was previously necessary to set the minimum TLS level to 1.0. Since around January 2024, this configuration issue went away, and ITU disabled the use of TLS 1.0 and 1.1 on its servers.

- Recommendations

The AHG recommends the following:

- Review the related contributions.

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AM1008, and continue the generation, cross-checking, and documentation of the conformance streams for the HEVC Multiview profiles supporting extended bit depth, the HEVC Multiview Main 10 profile, and the HEVC Multiview monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles, collectively referred to as the multiview format range extensions profiles).

It was suggested to develop a plan for the next version of HEVC reference software, supporting the new multiview profiles. Preferentially, a new version of H.265.1 might be submitted for ITU consent in the July meeting.

JVET-AP0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs)]

- Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following changes were integrated into ECM-19.1:

JVET-AO0145: Modification of the CTU size and MaxTT size for Class B sequences in RA configuration.

ECM-19.1 was tagged on January 30, 2026.

  - CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AI2017 are summarized.

ECM-19.1 performance over ECM-19.0 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class B | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class C | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Overall | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class B | -0.02% | -0.63% | -0.65% | 99.2% | 109.0% | 130.6% | 103.8% |
| Class C | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.21% | -0.22% | 99.7% | 102.9% | 109.3% | 101.3% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class C | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Overall | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class C | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Overall | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100.0% | 100.0% | 100.0% | 100.0% |

The below tables show ECM-19.1 performance comparing to VTM-11.0ecm19.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -14.87% | -16.35% | -27.31% | 1183.3% | 578.9% | 237.6% | 326.7% |
| Class A2 | -21.54% | -25.02% | -29.24% | 1117.5% | 618.4% | 238.9% | 326.5% |
| Class B | -15.11% | -23.02% | -20.90% | 1050.8% | 646.9% | 178.3% | 355.8% |
| Class C | -15.30% | -12.03% | -13.20% | 1030.4% | 649.6% | 122.4% | 394.7% |
| Class E | -19.69% | -23.36% | -21.40% | 1005.3% | 659.8% | 141.4% | 381.4% |
| Overall | -16.95% | -19.86% | -21.73% | 1070.2% | 633.0% | 173.8% | 358.0% |
| Class D | -13.04% | -8.61% | -9.49% | 993.9% | 692.2% | 110.1% | 414.3% |
| Class F | -31.03% | -35.19% | -35.37% | 616.5% | 656.3% | 125.9% | 374.4% |
| Class TGM | -43.73% | -49.29% | -48.58% | 505.2% | 645.6% | 150.3% | 351.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -27.70% | -25.43% | -37.71% | 1226.1% | 1275.1% | 153.4% | 247.6% |
| Class A2 | -30.98% | -35.54% | -40.94% | 1181.5% | 1462.9% | 153.2% | 247.5% |
| Class B | -25.59% | -35.20% | -32.29% | 1028.7% | 1367.6% | 189.2% | 278.6% |
| Class C | -27.32% | -24.94% | -25.55% | 1155.1% | 1555.8% | 117.4% | 321.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -27.55% | -30.58% | -33.31% | 1129.8% | 1414.7% | 153.1% | 276.0% |
| Class D | -28.00% | -24.79% | -26.22% | 1084.2% | 1651.9% | 105.4% | 360.4% |
| Class F | -33.69% | -37.31% | -38.21% | 807.9% | 865.5% | 121.6% | 296.0% |
| Class TGM | -43.03% | -48.96% | -48.99% | 721.6% | 745.4% | 125.8% | 280.9% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -22.69% | -41.03% | -35.94% | 1048.5% | 1184.3% | 167.3% | 266.0% |
| Class C | -25.22% | -29.20% | -31.15% | 1016.4% | 1236.8% | 125.8% | 316.7% |
| Class E | -22.82% | -29.50% | -27.98% | 1033.7% | 732.0% | 140.2% | 295.6% |
| Overall | -23.57% | -34.21% | -32.35% | 1034.0% | 1065.4% | 145.5% | 289.4% |
| Class D | -26.68% | -30.77% | -32.61% | 1040.5% | 1416.4% | 111.1% | 352.8% |
| Class F | -31.47% | -41.72% | -42.29% | 818.4% | 855.4% | 130.1% | 294.8% |
| Class TGM | -41.32% | -51.82% | -51.52% | 707.0% | 643.7% | 120.8% | 282.6% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -20.61% | -49.22% | -44.58% | 922.2% | 1169.2% | 172.3% | 293.6% |
| Class C | -23.22% | -37.72% | -39.26% | 863.4% | 1172.2% | 127.1% | 344.9% |
| Class E | -20.96% | -38.14% | -36.76% | 935.2% | 737.6% | 144.1% | 318.4% |
| Overall | -21.56% | -42.61% | -40.85% | 905.3% | 1042.9% | 148.9% | 316.1% |
| Class D | -25.94% | -41.54% | -42.37% | 863.5% | 1382.6% | 111.6% | 374.7% |
| Class F | -29.50% | -47.20% | -47.58% | 765.2% | 793.5% | 130.5% | 318.5% |
| Class TGM | -39.51% | -54.10% | -53.63% | 739.6% | 668.2% | 126.8% | 294.8% |

- ECM memory consumption

ECM encoder memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below as maximum class-wise consumption rounded up to GiB.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1 & A2) | 9 | 16 |  |
| class B | 4 | 6 | 5 |
| class C | 2 | 3 | 3 |
| class D | 2 | 2 | 2 |
| class E | 3 |  | 3 |
| class F | 4 | 6 | 5 |
| class TGM | 4 | 8 | 7 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

- Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.

It is noted that the newest VTM may be somewhat better than the VTM version used in the comparison.

JVET-AP0007 JVET AHG report: Tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice chairs)]

- Group off tests

During this meeting cycle, ECM only had a minor release ECM-19.1. The difference between ECM-19.0 and ECM-19.1 is RA configuration change for class B. After discussion, AHG members decided to not perform new tests during this meeting cycle. The ECM-19 group off test results are attached as an Annex of this report.

- Issues

  - Open issues

- #91, MaxTU setting

- #80, Compilation Issue ECM14.0 when disabling GPM reordering

- #78, Encoder crash in RA with --LMChroma=0

- #71, Encode/decode mismatch when using single tree

- #66, Difference in encoding results of ECM-12.0 in AhG7 group 2 tool off test

- #65, Different encoding results of ECM12 in AHG7 group1-4 off tests

- #64, Encode/decode mismatch and decoder crash when inter-CCCM is disable

- #53, Decoding mismatch was observed when AMVR is off

- Input contributions

| JVET-AP0170 | m76381 | 2026-04-17 12:35:37 | 2026-04-17 13:17:09 | 2026-04-17 13:17:09 | AHG7: Performance counters for reliable decoding time measurement | S. Hong, Y. Tokumo, T. Ikai (Sharp) |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AP0207 | m76438 | 2026-04-17 18:43:47 | 2026-04-17 18:54:55 | 2026-04-17 18:54:55 | AHG7: On Per-class BD-Rate Calculation | X. Li (Google) |

- Recommendations

- Continue and improve tool assessment

- Resolve identified software issues related to the tool assessment

- Review all the input documents

JVET-AP0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs)]

- Activities

The AHG 8 used the main JVET reflector, jvet@lists.rwth-aachen.de, for email discussion. Between the last and this meeting, there was no AHG8 email sent to the main reflector. There is no AHG8 input contribution to this meeting. The current focus is on finalization of TR (v1).

- Software and Common Test Conditions

AHG 8 related software and documents can be accessed at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects: common test conditions, reporting templates with anchor results, evaluation scripts and task networks are available in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/ofm-ctc, and software implementation examples are hosted in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm. For this meeting cycle, common test conditions remain unchanged as described in output document JVET-AI2031.

- Technical Report

Suggested changes on Draft ISO/IEC TR 23888-3 (JVET-AO0292) based on the comments received on the first Draft Technical Report (DTR) was discussed during the last meeting. Following that the 2nd DTR was submitted in February for ISO process and ballot.

- Input contributions

There is no other input contributions related to AHG 8 mandates except this report.

- JVET-AP0008	JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8)

- Recommendations

The AHG recommends to:

- Review received comments.

- Proceed to finalization.

Consider to close this AHG, or reduce the mandates such that the remaining work is editors’ business to generate the final versions for ISO and ITU. At this moment, no activity is foreseen to produce a second edition.

JVET-AP0009 JVET AHG report: SEI message studies (AHG9) [J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs)]

- Related contributions

As of Monday April 20th, a total of 116 contributions (not including this AHG report) were identified relating to the mandates of AHG9. Some contributions relate to more than one mandate and/or SEI message.

  - Study the SEI messages in VSEI, VVC, HEVC and AVC (4)

JVET-AP0058, AHG2/AHG9: Some errata items on VVC, VSEI, HEVC, and/or AVC, Y.-K. Wang, J. Xu (Bytedance)

JVET-AP0090, AHG9: On the cancel_flag related semantics in the FP, PRI and CTI SEI messages in VSEI v4, J. Xu, Y.-K. Wang, L. Zhang (Bytedance)

JVET-AP0091, AHG9: On semantics related to persistence_flag and cancel_flag in various SEI messages in VSEI v4 and VSEI TuC, J. Xu, Y.-K. Wang (Bytedance)

JVET-AP0217, AHG9: On FGC SEI message payloadType code points in AVC, R. Skupin, Y. Sanchez, C. Hellge, T. Schierl (HHI)

  - Study JVET-AO2032 TuC for VSEI (92)

    - General (12)

JVET-AP0069, AHG9: On VSEI RBSP, M. M. Hannuksela, J. Boyce, D. Fortin (Nokia)

JVET-AP0076, AHG9: SW for floating point syntax fl(n) and fl(w,t), L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)

JVET-AP0137, AHG9: On payload sizes for versatile SEI messages, M. Pettersson, M. Damghanian, S. Harribey, R. Sjöberg (Ericsson)

JVET-AP0138, AHG9: On SEI messages presence, M. Damghanian, S. Harribey, M. Pettersson, R. Sjöberg (Ericsson)

JVET-AP0160, AHG9: On signaling of identifiers and numbers of layers for SEI messages in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0161, AHG9: On presence of SEI messages in multiple layers, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0162, AHG9: On conformance cropping windows for SEI messages in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0163, AHG9: On inference values of syntax elements in SEI messages in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0196, AHG9: Working draft for VSEI v5, J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)

JVET-AP0216, AHG9: On SEI extensions for legacy devices, R. Skupin, Y. Sanchez, T. M. Borges, C. Hellge, T. Schierl (HHI)

JVET-AP0229, AHG9: Signaling options for alpha plane metadata in non-auxiliary picture context in AVC, HEVC and VVC, E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)

JVET-AP0230, AHG9: Signaling options for depth map metadata in non-auxiliary picture context in AVC, HEVC and VVC, E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)

    - New extensions or constraints of SEI messages specified in VSEI v4 (4)

JVET-AP0083, AHG9: On OMI, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AP0110, AHG9: On signaling of AI usage type for AURR SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0113, AHG9: On signaling content moderation information in TDI SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0147, AHG9: On the Alpha Channel Information SEI message, C. H. Demarty, E. François, P. de Lagrange, F. Urban, N. Caramelli (InterDigital)

JVET-AP0252, AHG9: On multiple extensions of SEI messages, H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE) [Late]

    - Scalability Dimension Information SEI message (3)

JVET-AP0094, AHG9: On the scalability dimension information SEI message in VSEI TuC, J. Xu, Y.-K. Wang (Bytedance)

JVET-AP0098, AHG9: On the SDI confidence map, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AP0167, AHG9: Fixes to the Scalability Dimension Information SEI message, J. Chen, Y. Ye (Alibaba)

    - Shutter Interval SEI message (2)

JVET-AP0073, AHG9: Proposed Shutter Interval SEI message extension to rolling shutter for version 5 of VSEI, L. Kerofsky, M. Karczewicz (Qualcomm), J. Xu, Y.-K. Wang (Bytedance)

JVET-AP0175, AHG9: On shutter interval information SEI message, X. Xu, M.Tang, S. Liu (Tencent)

    - Neural-network Post-filter Characteristics / related URIs (NNPFC / NNPFA) SEI message (8)

JVET-AP0060, AHG9: On nnpfc_uri and nnpfc_tag_uri, S. Deshpande (Sharp)

JVET-AP0068, AHG9: Additional tag URIs for the NNPFC SEI message, M. M. Hannuksela, J. Boyce, F. Cricri (Nokia)

JVET-AP0081, AHG9: On Multiple Inferences for Neural-network Post-filter, S. Deshpande (Sharp)

JVET-AP0092, AHG9: On redundant bits related to string signalling in the CR and NNPFA SEI messages in VSEI TuC, J. Xu, Y.-K. Wang (Bytedance)

JVET-AP0130, AHG3/AHG9: Public source code release of real-time NNPF implementation in VVdeC, J. Funnell, M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, S. Schwarz (Nokia)

JVET-AP0171, AHG9: On indication of intermediate data application in EOI, SPO and NNPFC SEI messages, X. Xu, S. Wenger, S. Liu (Tencent) (Also relates to SPO, EOI)

JVET-AP0176, AHG9: Bug fixes of NNPF SEI messages in TuC document, X. Xu, S. Liu (Tencent)

JVET-AP0226, AHG9/AHG18: Missed Picture Generation purpose for NNPF SEI message, S. Ikonin, M. Sychev, K. Malyshev, E. Alshina (Huawei)

    - SEI processing order (SPO) and processing order nesting (PON) SEI messages (1)

JVET-AP0215, AHG9: On detection of SEI message extensions in SPO, T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)

    - Encoder Optimization Information (EOI) SEI message (8)

JVET-AP0067, AHG9: On the encoder optimization information (EOI) SEI message, M. M. Hannuksela, J. Boyce (Nokia)

JVET-AP0111, AHG9: On spatial quality optimization in EOI SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0150, AHG9: Suggested Modifications for EOI SEI Message, Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)

JVET-AP0152, AHG9: Extension of Temporal Extrapolation Support for EOI SEI Message, Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)

JVET-AP0165, AHG9: On temporal resampling information in EOI SEI message, J. Chen, Y. Ye (Alibaba)

JVET-AP0166, AHG9: On depth information extension in EOI SEI message, J. Chen, Y. Ye (Alibaba)

JVET-AP0172, AHG9: On temporal sublayer limitation in EOI SEI message, X. Xu, S. Liu (Tencent)

JVET-AP0173, AHG9: On temporal extrapolation semantics in EOI SEI message, X. Xu, S. Liu (Tencent)

    - Modality Information SEI message (2)

JVET-AP0095, AHG9: On the modality information SEI message extension, J. Xu, Y.-K. Wang (Bytedance)

JVET-AP0121, AHG9: On MI SEI Extension to Indicate Sample Value Usage, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

    - Digitally signed content (DSC) SEI messages (5)

JVET-AP0136, AHG9: Fixes to subpicture signing for digitally signed content SEI messages, M. Pettersson, R. Sjöberg, M. Damghanian, S. Harribey (Ericsson)

JVET-AP0142, AHG9: On subpicture-based DSC SEI messages and subpicture extraction, H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0164, AHG9: Miscellaneous aspects of digitally signed content SEI messages in TuC, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0188, AHG9: TuC DSC subpicture improvements, I. Sodagar (Dolby)

JVET-AP0231, AHG9: On DSC SEI support for subpictures, J. Boyce, M. M. Hannuksela (Nokia)

    - Packed regions information (PRI) SEI message (1)

JVET-AP0102, AHG9: On the PRI SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

    - Film Grain Regions Characteristics (FGRC) SEI message (5)

JVET-AP0055, AHG9: Editorial changes for the FGRC SEI message, Y.-K. Wang, J. Xu (Bytedance)

JVET-AP0056, AHG9: Bug fixes for the FGRC SEI message, Y.-K. Wang, J. Xu, K. Zhang (Bytedance)

JVET-AP0057, AHG9: Some potential improvements for the FGRC SEI message, Y.-K. Wang, J. Xu (Bytedance)

JVET-AP0061, AHG9/AHG13: On Film Grain Regions Characteristics SEI message, S. Deshpande, J. Samuelsson-Allendes (Sharp)

JVET-AP0062, AHG9: On the FGRC SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

    - Constituent rectangles SEI message (3)

JVET-AP0092, AHG9: On redundant bits related to string signalling in the CR and NNPFA SEI messages in VSEI TuC, J. Xu, Y.-K. Wang (Bytedance)

JVET-AP0099, AHG9: On the CR SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AP0159, AHG9: Miscellaneous aspects of constituent rectangles and enhanced colour format information SEI messages, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

    - Quality Metrics (QM) SEI message (5)

JVET-AP0088, AHG9: On the QM SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AP0135, AHG9: On circular reference in the QM SEI message, C. H. Demarty, A. Ak, R. James (InterDigital)

JVET-AP0139, AHG9: On the QM SEI message, C. H. Demarty, A. Ak, R. James (InterDigital)

JVET-AP0154, AHG9: On the quality metrics SEI message, Y. Gao, P. Wu, S. Xie, S. Wang, Y. Bai (ZTE)

JVET-AP0174, AHG9: On quality metrics SEI message, X. Xu, S. Wenger, S. Liu (Tencent)

    - Lens Optical Correction SEI message (1)

JVET-AP0075, AHG9: Proposed Lens Optical Correction SEI message for version 5 of VSEI, L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou, S. Wenger (Tencent)

    - Display Overlays Information (DOI) SEI message (6)

JVET-AP0103, AHG9: On the DOI SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AP0104, AHG9: On the DOI alpha component, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AP0114, AHG9: On miscellaneous aspects of DOI SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0115, AHG9: On viewer advisory overlay in the DOI SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0131, AHG9: On display overlays information SEI message, K. Abe, T. Nishi (Panasonic)

JVET-AP0158, AHG9: On signaling of target picture in display overlays information SEI message, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

    - Enhanced colour format information (ECFI) SEI message (3)

JVET-AP0106, AHG9: On signalling packed channels in ECFI SEI message, H. Tan, C. Kim, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0116, AHG9: On single-layer and chroma format support in the ECFI SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0159, AHG9: Miscellaneous aspects of constituent rectangles and enhanced colour format information SEI messages, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

    - Photosensitive content information (PCI) SEI message (1)

JVET-AP0129, AHG9: On PCI SEI, T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)

    - Display Rectangles (DR) SEI message (3)

JVET-AP0119, AHG9: On display rectangles SEI message, J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)

JVET-AP0126, AHG9: On DR SEI, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AP0155, AHG9: On the display rectangles SEI message, S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)

    - Picture segmentation information (PSI) SEI message (1)

JVET-AP0089, AHG9: On the PSI SEI message, Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)

    - Danmu Information (DI) SEI message (6)

JVET-AP0097, AHG9: Implementation and showcase for the danmu information SEI message, J. Xu (Bytedance)

JVET-AP0117, AHG9: On danmu information SEI messages, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0127, AHG9: On Danmu SEI, T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AP0132, AHG9: On Danmu Information SEI, S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AP0151, AHG9: On the default parameters in the DI SEI message, S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)

JVET-AP0153, AHG9: Enable the use of segmentation planes with the DI SEI message, S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)

    - Colour Mapping Information (CMI) SEI message (4)

JVET-AP0071, AHG9: On the colour mapping information SEI message, J. Samuelsson-Allendes, S. Deshpande (Sharp)

JVET-AP0096, AHG9: On the colour mapping information SEI message, J. Xu, Y.-K. Wang, K. Zhang (Bytedance)

JVET-AP0122, AHG9: On CMI, S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)

JVET-AP0128, AHG9: On CMI SEI, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)

    - Auxiliary sampling information SEI message (1)

JVET-AP0148, AHG9:On Auxiliary sampling alignment information SEI, S. Wang, Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)

    - Localization and Mapping (LAM) SEI message (5)

JVET-AP0074, AHG9: Proposed Localization and Mapping SEI message for version 5 of VSEI, L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou, S. Wenger (Tencent)

JVET-AP0093, AHG9: On group cancellation for the localization and mapping SEI message, J. Xu, Y.-K. Wang (Bytedance)

JVET-AP0112, AHG9: On miscellaneous aspects of LAM SEI message, C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0157, AHG9: On localization and mapping SEI message with multiple cameras, J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)

JVET-AP0204, AHG9: Rotation precision in LAM SEI message, Y. Sanchez, R. Skupin, P. Helle, T. Hinz, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)

    - Sample Interleaving SEI message (3)

JVET-AP0077, AHG9: On Sample Interleaving, L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)

JVET-AP0134, AHG9: On the Sample Interleaving SEI message, C. H. Demarty, A. Ak, R. James (InterDigital)

JVET-AP0193, AHG9: Syntax for the sample interleaving SEI message, J. Boyce, S. He, T. Biatek, M. M. Hannuksela (Nokia)

  - Collect software and showcase information for SEI messages (3)

JVET-AP0097, AHG9: Implementation and showcase for the danmu information SEI message, J. Xu (Bytedance)

Also in 2.2.20

JVET-AP0130, AHG3/AHG9: Public source code release of real-time NNPF implementation in VVdeC, J. Funnell, M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, S. Schwarz (Nokia)

Also in 2.2.5

JVET-AP0146, AHG9: Software implementation of the Green SEI message and demonstration of the use of attenuation map information metadata for display adaptation, C. H. Demarty, N. Caramelli, F. Aumont (InterDigital)

  - Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (6)

JVET-AP0078, AHG9: Support of Infrared Auxiliary Picture (AUX_IR) and IR Information SEI for Sample Interpretation, S. Zhao, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)

JVET-AP0084, AHG9: Support of Gain Map Auxiliary Picture Type and Gain Map SEI Message, S. Zhao, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)

JVET-AP0085, AHG9: Subject Recognition Information(SRI) SEI message, Q. Zhang (BOE)

JVET-AP0177, AHG9: Transcoding history information SEI message, X. Xu, S. Wenger, S. Liu (Tencent), Johan Pardo, Alexander Karabutov, Hongjie You, Atanas Boev, Johannes Sauer, Timofey Solovyev, Elena Alshina (Huawei)

JVET-AP0179, AHG9: Historical Information SEI message, J. Pardo, A. Karabutov, H. You, A. Boev, J. Sauer, T. Solovyev, E. Alshina (Huawei), X. Xu, S. Liu (Tencent)

JVET-AP0197, AHG9: Shot summarization information SEI message, J. Boyce, M. M. Hannuksela (Nokia)

  - Coordinate with the joint AHG on Gaussian splat coding about aspects related to the usage of SEI messages (13)

JVET-AP0079, AHG9: Triplane video-based implicit Gaussian splatting with a scaffold-based anchor point framework, B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)

JVET-AP0100, AHG9: Gaussian splatting information SEI message, Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent), J. Xu, Y.-K. Wang, L. Zhang (Bytedance)

JVET-AP0101, AHG9: GSI SEI message workflow and packing examples, Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent), J. Xu, Y.-K. Wang, L. Zhang (Bytedance)

JVET-AP0108, AHG9: On Gaussian Splatting Information SEI message, J. Do, H. Lee, G. Bang (ETRI)

JVET-AP0109, AHG9: Enhancements to the Gaussian Splatting Information SEI message, H. Lee, J. Kang, J. Do, G. Bang (ETRI)

JVET-AP0194, AHG9: Common picture format for Gaussian splats coding, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AP0195, AHG9: Splatting information SEI with patches, J. Boyce, M. M. Hannuksela (Nokia)

JVET-AP0205, AHG9: On the GSI SEI message for spatial random access, Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent)

JVET-AP0213, AHG9: On implicit representations with the Gaussian splatting information SEI message, S. Lee, S. Sasse, Y. Berendsohn, Y. Sanchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (HHI)

JVET-AP0220, AHG9: Showcase for Real-time Decoding using Video-based Implicit Representations for 4DGS, S. Lee, S. Sasse, Y. Berendsohn, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)

JVET-AP0223, AHG9: Software implementation and results of the GSI SEI message with HM, HM-SCM, and VTM, J. Ricard, G. Teniou, S. Wenger (Tencent)

JVET-AP0225, AHG9: Extended gaussian splatting information SEI message with SH non-uniform quantization and 2D transforms, J. Ricard, G. Teniou, S. Wenger (Tencent)

JVET-AP0227, AHG9: Content-adaptive packing order optimization for 3D Gaussian splatting video coding, J. Ricard, G. Teniou, S. Wenger (Tencent)

- Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. No emails with [AHG9] were exchanged.

Git projects were set up for reporting and tracking of specification bugs for VSEI and the TuC, at the links below. It is encourage to submit an issue to report any specification bugs.

- VSEI: https://vcgit.hhi.fraunhofer.de/jvet/publications/VSEI

- TuC: https://vcgit.hhi.fraunhofer.de/jvet/publications/VSEI-TuC

- Recommendations

The AHG recommends to:

- Review all related contributions

- Continue SEI messages studies

It was requested to identify which contributions of Gaussian splatting have/have-not duplicate submissions to the JAhG.

JVET-AP0010 JVET AHG report: Encoding algorithm optimization (AHG10) [K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs)]

- Related contributions

Following contributions were identified relating to AHG10 and summarized in the following sections.

  - JVET-AP0070: AHG10: Additional random-access configuration with reduced delay

This document suggests adding an additional random-access configuration file based on a length 8 hierarchical GOP structure. The document also suggests updating the reference software with some additional parameters to control the QP every N:th frame to mimic quantization for a larger hierarchical GOP structure. The overall impact on BDR-rate compared to CTC using a length 32 hierarchical GOP structure is reported to be +7.88%/+13.08%/13.35% (Y/U/V) with encoding time of 99.7% and decoding time of 100.3%. Additionally, results for also enabling GOPbasedRPR and BIM=2 reports an overall BDR impact of +6.02%/+10.84%/+10.92% with encoding time 98.1% and decoding time 92.9%. The additional results indicates that both GOPbasedRPR and BIM also works together with the hierarchical GOP of length 8.

  - JVET-AP0191: [AHG16] VTM software extension for counting and constraining per-CU RDOs

This document recommends creating a AHG16 git repository for development of the complexity metrics as well as the hardware encoding mimicking simulation platform, e.g., with per-CU RDO number counting and constraints. The aim is to make the next-generation video coding more hardware friendly for both encoding and decoding.

In comparison to CTC (VTM-23.9) for random-access configuration the overall BDR impact of a limitation of the number of RDO per CU for intra/inter is reported to be:

- 10/10: 1.93%/1.20%/1.66% (Y/U/V) with encoding time of 88% and decoding time of 100%.

- 5/5:     4.80%/6.88%/7.31% (Y/U/V) with encoding time of 78% and decoding time of 100%.

  - JVET-AP0192: [AHG19] Evaluating VTM under HW constraints

This document recommends establishing a test configuration that considers hardware encoding constraints, for the development of next-generation standard. The GOP8 random-access configuration file proposed in the contribution is recommended to be a starting point.

The overall BDR impact of all tested encoding constraints in the document compared to CTC is 24.38%/35.22%/36.55% (Y/U/V) with encoding time of 15% and decoding time of 99% compared to VTM CTC (VTM-23.9). The test is using a GOP 8 random-access configuration with a limitation to 2 reference pictures, cfg/alternative-addon/reduced_runtime3.cfg, and a limitation to the number of intra and inter RDO’s to 10 respectively.

In comparison to an anchor using cfg/alternative-addon/reduced_runtime3.cfg the overall BDR impact of the GOP 8 random-access configuration with a limitation to 2 reference pictures is 12.12%/19.97%/19.69% (Y/U/V) with encoding time of 98% and decoding time of 101%.

  - JVET-AP0237: AHG17: Improved VTM configuration for 0.2x runtime target

This document reports results for a modified configuration for 0.2x runtime target with BDR rate impact of 3-4% on top of VTM-24.0 for the CfE test set and it also asserts visual improvements compared to the current 0.2x configuration. The document also provides rate matched settings for the VTM default anchor, the VTM RPR comparison point with BIM=2, the old 0.2x runtime target and the proposed 0.2x runtime target. The proposed modifications for the 0.2x runtime target apply for both random-access and low-delay configuration and are applied on top of the corresponding default configurations (Note: LMCSEnable settings are overridden by HDR specific settings):

Random-access:

MergeRdCandQuotaRegularSmallBlk : 3, MergeRdCandQuotaRegular : 3, MaxNumGeoCand : 4,

UseNonLinearAlfLuma : 0, AffineAmvp : 0, MaxNumAffineMergeCand : 4, SplitPredictAdaptMode : 2,

AdaptBypassAffineMe : 1, MergeRdCandQuotaGpm : 3, TransformSkip : 0, MaxMTTHierarchyDepth : 1, MaxMTTHierarchyDepthISliceL : 2, MaxMTTHierarchyDepthISliceC : 2, MTS : 4, SBT : 0, LFNST : 1, MMVD : 0, MaxNumMergeCand : 6, BCW : 0, Geo : 1, AllowDisFracMMVD : 0, AffineAmvr : 0, LMCSEnable : 0, MIP : 0, SMVD : 0, ChromaTS : 0, MaxMergeRdCandNumTotal : 4, AffineAmvrEncOpt : 0

Low-delay:

AffineAmvp : 0, UseNonLinearAlfLuma : 0, MergeRdCandQuotaGpm : 5, SplitPredictAdaptMode : 2,

AdaptBypassAffineMe : 1, TransformSkip : 0, TemporalFilter : 0, MaxMTTHierarchyDepth : 1, SBT : 0,

BCW : 0, AllowDisFracMMVD : 0, ChromaTS : 0, MaxMergeRdCandNumTotal : 6

  - Recommendations

The AHG recommends that the related input contributions are reviewed, and to further continue the study of encoding algorithm optimizations in JVET.

JVET-AP0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs)]

  - Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on February 20th and March 20th. In those teleconferences, the following topics were discussed:

- NNVC-16.0 software integration status and anchor performance.

- Clarification on training data sets.

- Framework for externally E2E AI coded picture.

- Operational bit-exact reproducibility.

- EE1 tests final design and cross-check status.

Details are summarized in the AHG11 & AHG14 teleconference report JVET-AP0048.

  - Common Test Conditions

Anchor for the NN-based video coding activity was made available though the Git repository used for the AHG activity:

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC_anchor_performance_NNVC-16-VTM_vs_NNVC-16.xlsm

Anchor performance data also distributed by AhG14 in JVET-AO0014 and announced during intermediate AhG meeting.

  - Training data

A revision of training sets has been conducted JVET-AP0049. It is asserted that DIV2K data set (images) which was used for NN-Intra and NN-based loop filters training is available for academic research purposes only. JVET has received a response to the call for training materials JVET-AP0228  which proposes to use JPEG AI training data set (which could be a replacement for DIV2K). If group will decide to re-train NNVC tools then wider training sets (with higher portion of UHD video content) identified in JVET-AP0049 can be used.

  - Bit-exact reproducibility

So far there is no mathematical proof of bit-exact reproducibility for any tool in NNVC. Stable convolution approach was studied in EE1 and two new AhG11 proposals aim to address this issue: JVET-AP0219 (dynamic range analysis for stable convolutions) and JVET-AP0234 (proposes overflow-aware quantized variant of NN-Intra).

  - Regarding complexity

The number of multiplications, reported in kMAC/pxl, and number of model parameters are primarily used for complexity assessment. CPU run time is also reported. For one of EE1 tests with substantial architectural change compared to LOP7 we observe same kMAC/pxl but 30% higher CPU decoding run-time. Obviously, the level of optimization and possibility of further speed-up need to be taken into account while we compare NN-based tools.

  - EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available in JVET-AP0023.

EE1 related contributions and cross-checks are summarized in EE1 summary report.

  - Performance Evaluation

The performance and complexity of NN-based tools available in NNVC SW is summarized in the table below. All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital, except content adaptive tools results that are from Nokia.

There were no changes in default configuration of NNVC SW. The main reason for increase of decoding run time relatively to NNVC configured as VTM (with all NNVC tools disabled) is the removal of redundant operations in NNVC-VTM (as was agreed at the last meeting). Changes compared to the last meeting are should in blue font. More detailed historical records about NNVC tools performance can be found in attached to this report excel.

| Test vs NNVC (configured as VTM) | Random Access cfg |  |  |  |  | kMAC/pxl |  |  |  | Param (Mprm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR or DRF | Total | Filter | Intra | SR or DRF |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 (LOP7) | -8.2% | -15.1% | -13.9% | 1.2 | 23 | 21.3 | 16.5 | 4.8 | 0 | 1.5 | 0.24 | 1.3 | 0 |
| NNVC-15.0 (LOP6) | -8.0% | -14.9% | -13.5% | 1.2 | 39 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-14.0 (LOP6) | -8.0% | -14.9% | -13.5% | 1.2 | 27 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 (HOP5) | -14.1% | -19.8% | -20.1% | 3.4 | 1180 | 471 | 466 | 4.8 | 0 | 2.7 | 1.41 | 1.3 | 0 |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 (VLOP4) | -6.1% | -7.6% | -6.1% | 1.2 | 15 | 9.9 | 5.1 | 4.8 | 0 | 1.3 | 0.07 | 1.3 | 0 |
| NN-Intra & LOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC 16.0 caLOP4 | -8.5% | -18.3% | -17.8% | 3.3 | 21 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NNVC 15.0 caLOP4 | -8.5% | -18.0% | -17.5% | 2.3 | 38 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NNVC 14.0 caLOP4-decp | -8.6% | -15.7% | -16.3% | 2.3 | 25 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NN-Intra & VLOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 caVLOP4 | -6.9% | -8.5% | -7.2% | 3.2 | 11 | 10.2 | 5.4 | 4.8 | 0 | 1.3 | 0.02 | 1.3 | 0 |
| NNVC-15.0 caVLOP4 | -6.9% | -8.5% | -7.2% | 2.3 | 18 | 10.2 | 5.4 | 4.8 | 0 | 1.3 | 0.02 | 1.3 | 0 |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 NNSR | -8.8% | -13.7% | -12.8% | 1.1 | 18 | 26.1 | 16.8 | 4.8 | 4.7 | 1.6 | 0.05 | 1.3 | 0.2 |
| NNVC-14.0 NNSR | -8.6% | -13.7% | -12.5% | 1.1 | 24 | 26.1 | 16.8 | 4.8 | 4.7 | 1.6 | 0.05 | 1.3 | 0.2 |
| NNVC-16.0 RPR | -8.7% | -14.4% | -13.4% | 1.1 | 18 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-14.0 RPR | -8.6% | -13.7% | -12.6% | 1.1 | 24 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NN-Intra & LOP filter & DRF (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-16.0 lDRF | -9.6% | -14.8% | -15.2% | 1.7 | 869 | 83 | 16.6 | 4.8 | 62 | 5.2 | 0.25 | 1.3 | 3.6 |
| NNVC-16.0 hDRF | -10.6% | -16.9% | -15.1% | 1.7 | 869 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |
| NNVC-15.0 DRF | -10.0% | -16.2% | -15.4% | 1.7 | 869 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |
| NNVC-14.0 DRF | -9.8% | -15.9% | -14.2% | 1.7 | 604 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |

BD-rate performance relatively to VTM depending on encoder run time for some NNVC tools combinations is shown on a plot below. Size of the bubble indicates decoder operations in kMAC/pxl.

More details and analysis for tools and tools combination are provided in AhG14 report.

  - End-to-end AI coded reference picture

Multi-layer and single-layer framework for externally E2E AI coded picture was studied in EE1. So far, the best performance is demonstrated by multi-layer framework with frame level RDO decision for inclusion E2E AI coded picture. But single layer framework in average shows 10% faster encoder and decoder CPU run time. Statistics for use of E2E AI image codec in multi-layer framework, depending on QP and resolution is reported in EE1-related contribution JVET-AP0144.

Improvement for external E2E AI coded picture framework was found in JVET-AP0140 by better mapping between in hybrid codec QP and quality index of E2E AI codec.

  - NNVC implementation aspects

One contribution (JVET-AP0190) provides very detailed analysis of power consumption and run-time for CPU neural-network accelerator. Operations count (kMAC/pxl) still remains relevant complexity measure, but power consumption and execution time are very dependent on tensor size and number of channels.

  - New tools proposals

Deep Reference Frame (DRF) technology, a recent addition to the NNVC toolbox, has been drawing considerable attention. There are two proposals aiming at performance improvement of DRF JVET-AP0052 , JVET-AP0063.

NNVC has content adaptive post- and in-loop filters, content adaptivity is extended to super-resolution in JVET-AP0178 . Solution is based on NNPF SEI.

  - List of related contributions

In total 29 NNVC related contributions submitted for this meeting. Among them 3 reports, 10 documents describing EE1 tests, 4 cross-checks, 5 contributions of bit-exact reproducibility and implementation aspects, 5 new proposals improving or modifying NNVC tools (2 of them are specifically about external E2E AI coded picture handling), 2 contributions are on training materials.

| JVET number | Title | Source |
| --- | --- | --- |
| Reports (3) |  |  |
| JVET-AP0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) |
| JVET-AP0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators) |
| JVET-AP0048 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| EE1 contributions (10) |  |  |
| JVET-AP0051 | EE1-3.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement | T. Shu, X. Chen, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.) |
| JVET-AP0066 | EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AP0080 | EE1-4: DCVC-RT as a Learned End-to-End Intra Frame Codec | M. Aderdor, T. Solovyev, E. Alshina (Huawei), F. Urban, F. Galpin, E. FranÃ§ois (InterDigital) |
| JVET-AP0182 | EE1-4.1: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AP0183 | EE1-4.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AP0184 | EE1-4.3: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | A. Hallapuro, N. Zou, A. B. Koyuncu, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AP0185 | EE1-4.4: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | A. Hallapuro, N. Zou, A. B. Koyuncu, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AP0201 | EE1-2.1: VLOP4 with new backbone block based on Spatial-Channel Mixing (SCM) | H. Cho, S. Bahk, J. Kim, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI) |
| JVET-AP0218 | EE1-5: operational bit-exact reproducibility | H. Zhang, N. Le, F. Cricri, N Zou, A. B. Koyuncu, L. Murn (Nokia) |
| JVET-AP0232 | EE1-4.1.3 and EE1-4.2.3: JPEG-AI as a Learned End-to-End Intra Frame Codec | A. Karabutov, P. Jia, E. Alshina (Huawei) |
| Externally E2E AI coded picture (2) |  |  |
| JVET-AP0140 | EE1-related: QP to QI mapping for intra-frame coding with DCVC | V. Rizzello, J. StrÃ¶m, P. Wennersten, M. Damghanian, D. Liu (Ericsson) |
| JVET-AP0144 | EE1-related: Statistics on multilayer frame level RDO using DCVC-RT | M. Aderdor, T. Solovyev, E. Alshina (Huawei), F. Urban, F. Galpin, E. FranÃ§ois (InterDigital) |
| Training materials (2) |  |  |
| JVET-AP0049 | [AHG11] On NNVC training sets | E. Alshina, N. Bhaskar, V. Khamidullin (Huawei), F. Galpin, T. Dumas (InterDigital), J. N. Shingala (Ittiam) |
| JVET-AP0228 | [AHG11] Response to Call for training materials | E. Alshina, J. Ascenso, T. Ebrahimi |
| New NNVC proposals (3) |  |  |
| JVET-AP0052 | AHG11: Improved H-DRF with Weighted Fusion and Optimized YUV Processing | X. Chen, J. Zhang, Z. Chen (Wuhan Univ.) |
| JVET-AP0063 | AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation | P. Bordes, F. Galpin, F. LoBianco, M. Paquiry (InterDigital) |
| JVET-AP0178 | AHG11: Neural Network Super Resolution as Postfilter with Content Adaptation | K. Pham-Dinh, F. Cricri, M. Santamaria, R. Yang, M. M. Hannuksela (Nokia) |
| Implementation and bit-exact reproducibility (3) |  |  |
| JVET-AP0190 | [AHG11] Energy Efficiency of Neural Accelerators | S. Cizel, R. Mullakhmetov, F. Galpin (InterDigital) |
| JVET-AP0219 | [AHG11] Additional Information on Stable Float Method for Neural Network Inference on GPU devices | H. Zhang, F. Cricri, N. Le, N. Zou, A. B. Koyuncu, L. Murn (Nokia) |
| JVET-AP0234 | AHG11: Dynamic Quantization and Hardware Optimization for NNIP | A. Akhtar, S. Esenlik, Y. Matsuba, M. Karczewic (Qualcomm) |
| Cross-checks (6) |  |  |
| JVET-AP0189 | Crosscheck of JVET-AP0066 (EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering) | N. J. Gadgil, S. Mishra, R. N. Gadde, W. I. Choi, K. P. Choi (Samsung) |
| JVET-AP0255 | Crosscheck of JVET-AP0066 (EE1-1.1.1 and EE1-1.1.2: Dynamic convolution for LOP7 neural in-loop filtering) | Y. Li (Qualcomm) |
| JVET-AP0272 | Crosscheck of JVET-AP0051 (EE1-3.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) | N. Bhaskar (Huawei) |
| JVET-AP0274 | Cross-check of JVET-AP0219 ([AHG11] Additional Information on Stable Float Method for Neural Network Inference on GPU devices) | A. Karabutov, E. Alshina (Huawei) F. Galpin (Interdigital), L. Kerofsky S. Esenlik, (Qualcomm) |
| JVET-AP0277 | Crosscheck of JVET-AN0201 (EE1-2.1: VLOP4 with new backbone block based on Spatial-Channel Mixing (SCM)) | J. Chi, X. Wang, J. Liu, C. Zhu, L. Luo, H. Guo(UESTC), Y. Huo, Y. Liu, Z. Zhang, J. Ca(Transsion) |
| JVET-AP0278 | Crosscheck of JVET-AP0218: “EE1-5: operational bit-exact reproducibility” | J. Ström, N. Stegmaier (Ericsson) |

  - Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Continue study of the framework for hybrid codec combined with End-to-End AI coded pictures.

- Analyze device interoperability and bit-exact reconstruction for all NNVC algorithms.

- Recommend promising technologies for the next EE1 round.

JVET-AP0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

- Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AN2024). The combined improvements of the ECM-19.1 over VTM-11.0ecm19.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.87% | -16.35% | -27.31% | 1183.3% | 578.9% |
| Class A2 | -21.54% | -25.02% | -29.24% | 1117.5% | 618.4% |
| Class B | -15.11% | -23.02% | -20.90% | 1050.8% | 646.9% |
| Class C | -15.30% | -12.03% | -13.20% | 1030.4% | 649.6% |
| Class E | -19.69% | -23.36% | -21.40% | 1005.3% | 659.8% |
| Overall | -16.95% | -19.86% | -21.73% | 1070.2% | 633.0% |
| Class D | -13.04% | -8.61% | -9.49% | 993.9% | 692.2% |
| Class F | -31.03% | -35.19% | -35.37% | 616.5% | 656.3% |
| Class TGM | -43.73% | -49.29% | -48.58% | 505.2% | 645.6% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -27.70% | -25.43% | -37.71% | 1226.1% | 1275.1% |
| Class A2 | -30.98% | -35.54% | -40.94% | 1181.5% | 1462.9% |
| Class B | -25.59% | -35.20% | -32.29% | 1028.7% | 1367.6% |
| Class C | -27.32% | -24.94% | -25.55% | 1155.1% | 1555.8% |
| Class E |  |  |  |  |  |
| Overall | -27.55% | -30.58% | -33.31% | 1129.8% | 1414.7% |
| Class D | -28.00% | -24.79% | -26.22% | 1084.2% | 1651.9% |
| Class F | -33.69% | -37.31% | -38.21% | 807.9% | 865.5% |
| Class TGM | -43.03% | -48.96% | -48.99% | 721.6% | 745.4% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -22.69% | -41.03% | -35.94% | 1048.5% | 1184.3% |
| Class C | -25.22% | -29.20% | -31.15% | 1016.4% | 1236.8% |
| Class E | -22.82% | -29.50% | -27.98% | 1033.7% | 732.0% |
| Overall | -23.57% | -34.21% | -32.35% | 1034.0% | 1065.4% |
| Class D | -26.68% | -30.77% | -32.61% | 1040.5% | 1416.4% |
| Class F | -31.47% | -41.72% | -42.29% | 818.4% | 855.4% |
| Class TGM | -41.32% | -51.82% | -51.52% | 707.0% | 643.7% |

The performance of the ECM between these two meetings remained practically unchanged since the only modification was related to Class B of Random-Access configuration.

- Contributions

In addition to 8 EE2 contributions, 8 (comparing to 13 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

  - Cross Component Prediction and Filtering (1)

JVET-AP0105, "Non-EE2: A bugfix of the subsampling filter for certain CCLM/CCCM tools", T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI))

  - Intra (5)

JVET-AP0133, "EE2-1.1: Improvement on chroma MPM", Z. Li, W. Niu, X. Zeng, M. Jia, Y. Bai (ZTE)

JVET-AP0143, "EE2-related: Combination of JVET-AO0092 and EE2-1.2b", P.-K. Liu, C.-Y. Liu, S.-P. Wang, C.-C. Lin, C.-L. Lin (ITRI), Y.-T. Liu, Z.-H. Zhang, Y.-K. Huo, Z.-H. Zhang, J.-Y. Cai (Transsion)

JVET-AP0180, "Non-EE2: Fix for MDIP", W. Niu, Z. Li, M. Jia, X. Zeng, S. Xie, Y. Bai (ZTE)

JVET-AP0181, "Non-EE2: Enhancement of Chroma MPM List Construction", W. Niu, Z. Li, M. Jia, X. Zeng, S. Xie, Y. Bai (ZTE)

JVET-AP0233, "Non-EE2: Improvements on IBC-AMVP candidate list construction", H. Jeong, B. Jeon (SKKU)

  - Geometric Partitioning Mode (1)

JVET-AP0156, "Non-EE2: Adaptive GPM blending weights with linear and Hermite profiles", K. Kishimoto, H. Kato, Y. Kidani (KDDI)

  - Entropy Coding (1)

JVET-AP0235, "Non-EE2: Temporal CABAC and CTPI at Different CTU Locations", Z. Xiang, R. Chernyak, H. Huang, S. Li (Tencent)

- Recommendations

The AHG recommends:

- To review all the related contributions.

It was commented that CTC JVET-AI2017 should be kept valid and frozen as reference, but likely not be used as basis for future standard test conditions. A note should be added on that below JVET-AI2017.

JVET-AP0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

From the previous meetings, the following two items should be in the second edition:

- Updated Technical Report (TR) text to fix errors and clumsy wording

- Updated tools for the TR

- Video test sequences specific to film grain that were added to the draft report were removed after conferring with the other chairs due to the sequences not being located in a permanent location.

- Related contributions

Three contributions related to AHG13 were identified as of 04/23/26.

- One contribution was the AHG report:

  - JVET-AP0013 JVET AHG report: Film grain technologies (AHG13)

- Four other contributions were uploaded at the time of the report drafting:

  - JVET-AP0061 AHG9/AHG13: On Film Grain Regions Characteristics SEI message

  - JVET-AP0187 AHG13: Proposal to update the technical report on film grain synthesis technology for video applications

  - Contributions

There were two contributions registered other than the AHG report.  All were uploaded as of 04/23/26.

    - JVET-AP0061 AHG9/AHG13: On Film Grain Regions Characteristics SEI message

This contribution proposes modifications to film grain regions characteristics (FGRC) SEI message, which is included in the Technologies under consideration for future extensions of VSEI. Following is proposed:

- Proposal 1: It is proposed to signal the flag fgr_film_grain_enabled_flag[ i ] only when fgr_active_regions_number_minus1 is greater than 0 and infer its value otherwise.

- Proposal 2: It is proposed to normatively specify the processing logic for applying film grain at a sample location which falls within multiple regions.

Proposal 3: An asserted simplification is proposed to specify the for loop which signals the film grain models using a ternary conditional operator instead of using a derived variable.

    - JVET-AP0187 AHG13: Proposal to update the technical report on film grain synthesis technology for video applications

This contribution proposes updates to the technical report on film grain synthesis technology for video applications. The updates include clarifications and refinements related to the frequency filtering model, particularly for film grain analysis based on this model. In addition, a new subsection describing learning-based approaches to film grain analysis is introduced. The updated document is attached to this contribution.

- Recommendations

The AHG recommends:

- Continue editing the second edition for the TR;

- the related input contributions (non-AHG9) to be reviewed;

- any liaisons to be reviewed;

- to continue conformance discussion;

- to consider any newly proposed SEI message extensions;

- to discuss the possibility of creating, as appropriate, documents providing Engineering Guideline (EG) or recommended practices (RP) relating to film grain technologies and their applications; and

- to continue the study of film grain technologies in JVET.

It should be discussed during the meeting whether a more official stage of the 2nd version of the TR should be started.

JVET-AP0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs)]

- Software development

  - Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM in the NNVC branch.

NNVC software is based on latest VTM, VTM-23.14 at the time of software integration.

Legacy version (up to NNVC-13.0) can be found in branch VTM-11.0_nnvc and contains history of development of tools integrated up to version 13.0.

NNVC-16.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc is used for NNVC performance evaluation.

  - Software changes

    - Current NNVC

Several commits were merged in the NNVC repository. The following changes were integrated:

| Status | MR | Topic | Contributions |
| --- | --- | --- | --- |
| done | 435 | fix | log and JVET-AN0348 |
| done | 432 | fix | fix CANNLF |
| done | 431 | fix | fix NNSR and GOPbasedRPR interaction |
| done | 430 | fix | fix (non CTC) when NNIP dumper is on |
| done | 428 | fix | fix CI andl flake8 policy |
| done | 429 | upgrade | SADL v16 |
| done | 427 | fix | fix issues build issues and missing code |
| done | 426 | fix | fix ALOP with new interface |
| done | 425 | fix | Fix RPR and NNSR related to the VTM 23.14 |
| done | 422 | fix/feat | fix non LOP7 inference and allow legacy models inference |
| done | 420 | hybrid | Fix distortion weighting from gopsize |
| done | 419 | hybrid | fix POC setting during "encodePrep" using "getBaseLayerTemporalSubsampleRatio" |
| done | 418 | fix | remove unnecessary processings in nnpf and nnip when not used |
| done | 424 | VTM 23.14 | remainings VTM commits |
| done | 423 | VTM 23.14 | RPR aspects |
| done | 417 | VTM 23.14 | cherry pick up to VTM-23.14 |
| done | 414 | training scripts/models | EE1-1.1.2 : LOP with Overlapped Feature Integration JVET-AO0066 |
| done | 416 | training scripts/models | EE1-5.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM) JVET-AO0128 (KHU, ETRI). |
| done | 415 | training scripts | JVET-AO0127 AHG11/AHG14: Improving Training Reproducibility in NNVC [H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI), T. Yang, W.-X. He, Q. Liu (HUST), J. Chi, L. Luo, Ce Zhu (UESTC)] |
| done | 407 | fix | fix intra BL bitrate because of temporal subsampling ratio and enhancement layer frames encoded |
| done | 409 | fix | fix SEI skipping in parcat for external base layer without NN coded slice |
| done | 408 | fix | Deactivate HashMe for same POC with external base layer reference |
| done | 412 | inference | EE1-4.1:4.1.1 + 4.1.3 Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization JVET-AO0108 |
| done | 411 | training scripts | EE1-4.1:4.1.1 + 4.1.3 Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization JVET-AO0108 |
| done | 411 | models | EE1-4.1:4.1.1 + 4.1.3 Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization JVET-AO0108 |
| done | 413 | fix | added a user time clock at decoder to include time outside the main process |
| done | 406 | fix | fix bug in adaptive vlop4 |

SADL changes:

| Status | MR | Topic | Contributions |
| --- | --- | --- | --- |
| done | 214 | fix | fix issue with speedup and NNSR |
| done |  | feature | add a parameter to onnx converter to put the model info directly into the model |
| done | 203 | new layer | JVET-AO0056 AHG14: GlobalAveragePool layer for SADL [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)] |
| done | 212 | feature | JVET-AO0057 AHG14: SADL viewer module using Netron [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)] |
| done | 204 | fix | JVET-AO0058 AHG14: Bug fix for Conv2D sampling grid misalignment in SADL [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)] |
| done | 205 | fix | JVET-AO0073/JVET-AO0217 fix saturation |
| pending | 207 | conv 3x3 | JVET-AO0217 AhG14: Improved SADL implementation [D. Rusanovskyy, J. Lainema (Nokia), D. Tolmachov, D. Shevchenko, I. Zaporozhets (Aimprosoft)] |
| pending | 209 | biasadd | JVET-AO0217 AhG14: Improved SADL implementation [D. Rusanovskyy, J. Lainema (Nokia), D. Tolmachov, D. Shevchenko, I. Zaporozhets (Aimprosoft)] |
| pending | 206 | conv1x1 | JVET-AO0217 AhG14: Improved SADL implementation [D. Rusanovskyy, J. Lainema (Nokia), D. Tolmachov, D. Shevchenko, I. Zaporozhets (Aimprosoft)] |
| pending | 208 | peeling ixj | JVET-AO0217 AhG14: Improved SADL implementation [D. Rusanovskyy, J. Lainema (Nokia), D. Tolmachov, D. Shevchenko, I. Zaporozhets (Aimprosoft)] |
| done | 211 | fix nnintra | fix corner case in load sparsify for float models |
| done | 200 | 1.1 mix conv | JVET-AO0073 AHG14: Decoding time reduction in SADL using SIMD acceleration [J. Han, C. Jung, Q. Qin (Xidian Univ.)] |
| done | 202 | 1.3 peel simd | JVET-AO0073 AHG14: Decoding time reduction in SADL using SIMD acceleration [J. Han, C. Jung, Q. Qin (Xidian Univ.)] |
| done | 201 | 1.2 conv split | JVET-AO0073 AHG14: Decoding time reduction in SADL using SIMD acceleration [J. Han, C. Jung, Q. Qin (Xidian Univ.)] |
| done | 197 | improvement | JVET-AO0100 SIMD for slice |
| done | 196 | improvement | JVET-AO0100 SIMD biasadd |
| done | 195 | improvement | JVET-AO0100 SIMD concat |
| done | 194 | improvement | JVET-AO0100 better SIMD mul |
| done | 193 | improvement | JVET-AO0100 better SIMD add |
| done | 189 | new layer | JVET-AN0222: clip operation |
| done | 192 | new layer | JVET-AN0222: BitShift operator |
| done | 190 | new layer | JVET-AN0222 New Depth2Space layer |
| done | 191 | new layer | JVET-AN0222 New Abs layer |
| done | 177 | new layer | o MR 177? JVET-AN0197 New Gather layer |
| done | 176 | new layer | o MR 176? JVET-AN0197 New GatherElements layer |

The RPR policy between VTM and NNVC is now synchronized for GOP based RPR. In RA a configuration to emulate legacy NNSR policy is also provided.

  - Software overview

The framework for Hybrid end-to-end coding of intra slice has been improved for flexibility of testing for EE1. Tests can now be done on clusters in parallel using external encoder/decoder.

The master branch is kept synchronized with the upstream repository (VTM).

  - NNVC-16

  - Software version

NNVC 16.2 was tagged 17th April 2026 (contains VTM-23.14, LOP7, LDRF/HDRF, NNSR2)

NNVC 16.1 was tagged 18th March 2026 (contains VTM-23.14, LOP7, LDRF/HDRF, NNSR2)

NNVC 16.0 was tagged 24th February 2026 (contains VTM-23.14, LOP7, LDRF/HDRF, NNSR2)

NNVC 15.1 was tagged 19th December 2025 (contains VTM 23.13)

NNVC-15.0 was tagged 14th November 2025.

NNVC-14.1 was tagged 28th August 2025

NNVC-14.0 was tagged July 28th, 2025 (contains VLOP4, DRF, hybrid e2e)

NNVC-13 was tagged April 28th, 2025 (contains LOP6)

NNVC-12 was tagged February 6th, 2025 (containing final LOP5 model)

NNVC-12rc1 was tagged February 5th, 2025 (containing new intermediate LOP5 model)

NNVC-11.0 was tagged December 10th, 2024 (bit accurate with 11rc)

NNVC-11.0rc was tagged December 3rd, 2024

NNVC-10.0 was tagged August 9th, 2024

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

- CTC performance

All performances are changed compared to NNVC-15 because VTM 23.14 made some changes (improved MCTF, policy changes for RPR). Some unnecessary processing have been removed from VTM mode, hence the decoding complexity of NNVC is increased with respect to anchor (even if the absolute complexity is not changed).

Note: see configurations section for naming convention.

  - Comparison to VTM

    - NNVC-15.0 VTM vs NNVC-16 VTM

Note: NNVC-16 is now based on VTM-23.14.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.41% | -1.47% | -1.01% | -0.51% | -1.57% | -1.20% | 95% | 96% |
| Class A2 | -0.63% | 0.11% | 0.21% | -0.58% | -0.43% | -0.57% | 97% | 98% |
| Class B | -0.72% | 0.77% | 0.05% | -1.01% | -0.11% | -0.67% | 96% | 92% |
| Class C | 0.06% | 1.44% | 1.39% | -0.05% | -0.20% | 0.57% | 98% | 94% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.43% | 0.37% | 0.23% | -0.57% | -0.49% | -0.43% | 96% | 95% |
| Class D | 0.17% | 1.76% | 1.71% | -0.16% | 0.08% | 0.35% | 97% | 95% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 91% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.05% | 0.24% | 0.33% | 0.09% | 0.17% | 0.49% | 99% | 96% |
| Class C | 0.18% | 0.60% | 0.47% | 0.15% | 0.48% | -0.02% | 98% | 95% |
| Class E | 0.08% | -0.19% | -0.40% | 0.07% | 0.12% | 0.33% | 99% | 94% |
| Overall | 0.10% | 0.25% | 0.19% | 0.11% | 0.26% | 0.28% | 99% | 95% |
| Class D | 0.19% | 0.42% | 0.36% | 0.26% | 0.68% | 0.50% | 98% | 95% |
| Class F | 0.04% | 0.04% | 0.04% | 0.06% | 0.05% | 0.05% | 99% | 95% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 95% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 95% | 95% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 95% | 91% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 90% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 93% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 96% |

Notes: results by InterDigital, crosschecked by xxx.

    - NNVC-16.2 VTM vs NNVC-16.2 anchor

Anchor: NNVC in VTM mode.

Test: NNLF LOP7 + NNIP.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9,29% | -13,23% | -15,33% | -9,53% | -15,16% | -17,37% | 117% | 2244% |
| Class A2 | -8,54% | -14,99% | -10,30% | -8,74% | -15,13% | -10,22% | 115% | 2173% |
| Class B | -7,90% | -16,44% | -14,84% | -7,83% | -17,92% | -17,18% | 118% | 2391% |
| Class C | -7,61% | -14,94% | -14,29% | -7,91% | -15,06% | -15,67% | 115% | 2385% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8,23% | -15,11% | -13,88% | -8,37% | -16,05% | -15,42% | 117% | 2314% |
| Class D | -8,12% | -14,13% | -13,57% | -6,99% | -15,22% | -15,31% | 110% | 2231% |
| Class F | -4,42% | -8,65% | -7,81% | -5,47% | -11,60% | -11,45% | 121% | 1356% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -6,48% | -12,39% | -11,75% | -7,33% | -10,33% | -11,85% | 104% | 2311% |
| Class E | -6,52% | -6,57% | -6,50% | -7,88% | -4,91% | -8,13% | 111% | 2720% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -7,04% | -9,84% | -8,90% | -6,75% | -5,72% | -9,32% | 100% | 2157% |
| Class F | -3,87% | -6,93% | -4,79% | -5,86% | -7,59% | -8,09% | 110% | 1463% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6,84% | -14,59% | -13,29% | -6,78% | -15,01% | -18,77% | 113% | 2369% |
| Class C | -6,90% | -14,91% | -14,59% | -7,44% | -14,72% | -16,34% | 106% | 2374% |
| Class E | -7,69% | -8,48% | -7,30% | -8,97% | -8,07% | -10,81% | 114% | 3074% |
| Overall | -7,07% | -13,17% | -12,22% | -7,55% | -13,18% | -15,97% | 111% | 2530% |
| Class D | -7,61% | -11,51% | -12,60% | -6,72% | -7,25% | -16,11% | 101% | 2220% |
| Class F | -4,16% | -8,76% | -6,21% | -6,01% | -11,45% | -12,27% | 114% | 1523% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9,38% | -15,09% | -17,28% | -9,34% | -18,59% | -19,25% | 171% | 1888% |
| Class A2 | -7,86% | -15,13% | -11,88% | -8,47% | -15,77% | -11,60% | 172% | 1624% |
| Class B | -7,77% | -15,05% | -15,93% | -7,91% | -17,51% | -18,00% | 171% | 1732% |
| Class C | -7,88% | -13,34% | -13,84% | -8,14% | -16,81% | -17,02% | 159% | 1421% |
| Class E | -11,56% | -17,12% | -18,46% | -11,86% | -17,56% | -19,45% | 166% | 1941% |
| Overall | -8,71% | -15,03% | -15,44% | -8,95% | -17,25% | -17,17% | 168% | 1695% |
| Class D | -7,76% | -11,42% | -12,74% | -7,27% | -15,69% | -16,24% | 152% | 1240% |
| Class F | -5,66% | -9,45% | -8,92% | -6,16% | -12,52% | -13,16% | 138% | 1397% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-16 VTM mode vs NNVC-16 CALOP4

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-16 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.09% | -15.81% | -21.22% | -13.28% | -19.40% | -22.89% | 340% | 2170% |
| Class A2 | -8.50% | -18.29% | -15.49% | -8.22% | -19.47% | -15.71% | 327% | 2057% |
| Class B | -8.02% | -22.66% | -19.69% | -6.46% | -24.55% | -21.45% | 339% | 2217% |
| Class C | -6.97% | -14.81% | -14.52% | -7.10% | -16.47% | -16.82% | 329% | 2145% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.45% | -18.32% | -17.78% | -8.35% | -20.35% | -19.36% | 334% | 2156% |
| Class D | -7.00% | -13.46% | -12.82% | -6.65% | -14.77% | -14.35% | 315% | 1697% |
| Class F | -4.09% | -9.55% | -8.74% | -5.06% | -11.94% | -12.51% | 353% | 1287% |

Note: Results from Nokia, crosschecked by xxx.

    - NNVC-16 VTM mode vs NNVC-16 CAVLOP4

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-16 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.31% | -6.90% | -8.74% | -11.29% | -8.47% | -10.41% | 333% | 1137% |
| Class A2 | -6.68% | -8.98% | -6.01% | -7.03% | -11.04% | -6.07% | 321% | 1089% |
| Class B | -6.70% | -10.43% | -7.57% | -5.86% | -13.09% | -9.16% | 332% | 1184% |
| Class C | -5.31% | -6.93% | -6.27% | -6.01% | -8.33% | -7.82% | 326% | 1149% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.85% | -8.50% | -7.15% | -7.22% | -10.49% | -8.43% | 328% | 1146% |
| Class D | -5.14% | -5.58% | -5.43% | -5.47% | -7.30% | -7.46% | 314% | 957% |
| Class F | -3.18% | -3.89% | -3.96% | -4.16% | -5.16% | -5.62% | 340% | 739% |

Note: Results from Nokia, crosschecked by xxx.

    - NNVC-16 VTM mode vs NNVC-16 VTM GOPRPR

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 16 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.21% | -3.61% | -3.73% |  |  |  | 86% | 62% |
| Class A2 | -0.51% | 4.29% | 3.87% |  |  |  | 88% | 73% |
| Class B | -0.09% | -0.41% | -1.06% |  |  |  | 99% | 91% |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  | 100% | 98% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.77% | 0.00% | -0.33% |  |  |  | 94% | 82% |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  | 102% | 96% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-16 VTM mode vs NNVC-16 GOPRPR

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 16 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.49% | -15.54% | -17.05% |  |  |  | 100% | 1225% |
| Class A2 | -8.73% | -9.34% | -5.32% |  |  |  | 100% | 1375% |
| Class B | -7.92% | -16.42% | -15.26% |  |  |  | 117% | 2071% |
| Class C | -7.61% | -14.94% | -14.29% |  |  |  | 113% | 2176% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.71% | -14.43% | -13.37% |  |  |  | 109% | 1741% |
| Class D | -8.12% | -14.13% | -13.57% |  |  |  | 109% | 2059% |
| Class F | -4.42% | -8.65% | -7.81% |  |  |  | 122% | 1279% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-16.2 VTM mode vs NNVC-16.2 GOPRPR HDRF

Anchor: NNVC 16.2 VTM mode

Test: NNVC 16.2 NNLF LOP7 + NNIP + HDRF + GOP based RPR

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -13,80% | -15,92% | -17,71% | -16,55% | -20,71% | -23,11% | 135% | 56685% |
| Class A2 | -11,74% | -11,23% | -7,15% | -14,84% | -15,10% | -11,54% | 136% | 61825% |
| Class B | -10,00% | -17,82% | -15,66% | -10,99% | -20,42% | -19,94% | 170% | 97397% |
| Class C | -10,87% | -18,01% | -16,93% | -12,04% | -18,80% | -19,22% | 153% | 92900% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -11,34% | -16,17% | -14,71% | -13,15% | -18,98% | -18,70% | 151% | 78810% |
| Class D | -11,70% | -16,92% | -15,93% | -10,80% | -18,58% | -18,51% | 146% | 81832% |
| Class F | -5,55% | -9,72% | -8,65% | -7,12% | -13,03% | -12,85% | 232% | 158341% |

Note: Results from Interdigital, crosschecked by xxx.

  - Comparison to NNVC-15 anchor

    - NNVC-15 anchor vs NNVC-16 anchor

Anchor: NNVC 15 NNLF LOP6 + NNIP

Test: NNVC 16 NNLF LOP7 + NNIP

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 15 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.78% | -0.78% | -1.13% | -1.29% | -0.67% | -1.18% | 94% | 56% |
| Class A2 | -0.89% | 0.13% | -0.56% | -0.92% | -0.03% | -1.31% | 95% | 55% |
| Class B | -0.93% | 0.33% | -0.02% | -1.07% | 0.35% | -0.16% | 96% | 54% |
| Class C | -0.11% | 0.99% | 0.95% | -0.34% | -0.15% | 0.21% | 97% | 54% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.67% | 0.24% | -0.09% | -0.89% | -0.06% | -0.50% | 96% | 55% |
| Class D | -0.01% | 1.48% | 0.68% | -0.29% | 0.32% | -1.43% | 97% | 57% |
| Class F | -0.07% | -0.08% | -0.56% | -0.07% | -0.56% | -0.05% | 96% | 57% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 15 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.23% | -0.58% | -0.61% | -0.16% | 0.14% | -2.81% | 97% | 54% |
| Class C | -0.28% | 0.04% | -1.35% | -0.49% | 0.66% | -3.11% | 97% | 54% |
| Class E | -0.03% | -1.98% | 1.59% | -0.21% | -0.18% | -1.52% | 93% | 54% |
| Overall | -0.20% | -0.73% | -0.31% | -0.28% | 0.23% | -2.59% | 96% | 54% |
| Class D | -0.08% | 0.41% | -1.79% | -0.20% | 0.38% | -5.45% | 97% | 55% |
| Class F | -0.25% | -0.83% | 0.42% | -0.27% | -0.34% | 0.41% | 96% | 55% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 15 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.06% | 0.92% | 0.06% | -0.29% | 0.96% | 0.64% | 97% | 59% |
| Class A2 | -0.13% | 0.32% | -0.62% | -0.22% | 0.49% | -0.12% | 99% | 62% |
| Class B | -0.11% | 0.08% | -0.14% | -0.12% | 0.35% | 0.50% | 99% | 63% |
| Class C | -0.10% | 0.03% | -0.68% | -0.16% | 0.17% | 0.01% | 99% | 67% |
| Class E | -0.28% | -0.17% | -0.10% | -0.36% | 0.01% | 0.47% | 99% | 62% |
| Overall | -0.13% | 0.21% | -0.30% | -0.21% | 0.38% | 0.31% | 99% | 62% |
| Class D | -0.09% | 0.19% | -0.59% | -0.18% | 0.48% | 0.65% | 99% | 69% |
| Class F | -0.13% | 0.02% | -0.72% | -0.17% | 0.21% | -0.74% | 99% | 61% |

Note: Results from Interdigital, crosschecked by xxx.

  - Comparison to NNVC-16 anchor

    - NNVC-16.2 anchor vs NNVC-16.2 GOP based RPR

Anchor: NNLF LOP7 + NNIP

Test: NNLF LOP7 + NNIP + Gop based RPR

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.46% | -2.98% | -2.07% |  |  |  | 86% | 55% |
| Class A2 | -0.15% | 5.67% | 5.28% |  |  |  | 88% | 64% |
| Class B | -0.03% | -0.04% | -0.55% |  |  |  | 100% | 88% |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  | 100% | 98% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.53% | 0.52% | 0.46% |  |  |  | 95% | 77% |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  | 100% | 95% |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  | 100% | 97% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-16.2 anchor vs NNVC-16.2 SR1

Anchor: NNLF LOP7 + NNIP

Test: NNLF LOP7 + NNIP + SR1

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -2.53% | 1.82% | 1.63% | -5.05% | -5.18% | -5.11% | 89% | 54% |
| Class A2 | -0.40% | 4.79% | 4.34% | -3.02% | 1.74% | 1.02% | 89% | 63% |
| Class B | -0.10% | 0.46% | -0.06% | -0.83% | -0.77% | -1.85% | 99% | 86% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.62% | 1.48% | 1.17% | -1.89% | -0.94% | -1.44% | 95% | 77% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 99% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-16.2 anchor vs NNVC-16.2 HDRF

Anchor: NNLF LOP7 + NNIP

Test: NNLF LOP7 + NNIP + HDRF

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1,61% | -0,37% | -0,29% | -1,76% | -1,35% | -1,45% | 159% | 4256% |
| Class A2 | -3,00% | -2,38% | -1,98% | -3,29% | -3,42% | -2,95% | 147% | 3946% |
| Class B | -2,19% | -1,68% | -0,42% | -2,50% | -2,45% | -1,79% | 154% | 4574% |
| Class C | -3,51% | -3,62% | -3,08% | -4,49% | -4,46% | -4,27% | 135% | 4081% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2,59% | -2,08% | -1,42% | -3,04% | -2,96% | -2,62% | 148% | 4246% |
| Class D | -3,88% | -3,29% | -2,78% | -4,09% | -3,98% | -3,73% | 132% | 3747% |
| Class F | -1,21% | -1,18% | -0,91% | -1,79% | -1,66% | -1,65% | 190% | 11971% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-16.2 anchor vs NNVC-16.2 LDRF

Anchor: NNLF LOP7 + NNIP

Test: NNLF LOP7 + NNIP + LDRF

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.25% | 3.43% | -1.16% | -1.44% | 0.07% | -1.43% | 117% | 1202% |
| Class A2 | -2.01% | 0.57% | -1.65% | -2.49% | -1.86% | -2.12% | 111% | 1130% |
| Class B | -1.13% | 0.28% | -1.14% | -1.69% | -1.16% | -1.67% | 114% | 1300% |
| Class C | -1.56% | -1.36% | -1.94% | -2.73% | -2.10% | -2.77% | 108% | 1177% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.44% | 0.53% | -1.46% | -2.08% | -1.31% | -2.01% | 112% | 1212% |
| Class D | -1.23% | -1.07% | -1.63% | -2.18% | -2.14% | -2.56% | 107% | 1067% |
| Class F | -0.61% | -0.42% | -0.62% | -1.19% | -0.75% | -1.16% | 125% | 3258% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-16.2 anchor vs NNVC-16.2 VLOP4

Anchor: NNLF LOP7 + NNIP

Test: NNLF VLOP4 + NNIP

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.56% | 7.29% | 11.12% | 0.45% | 8.36% | 12.91% | 99% | 67% |
| Class A2 | 2.50% | 9.52% | 6.42% | 2.07% | 8.81% | 6.11% | 98% | 63% |
| Class B | 2.42% | 9.81% | 9.74% | 1.62% | 10.60% | 11.14% | 99% | 65% |
| Class C | 2.70% | 9.43% | 9.37% | 1.89% | 8.53% | 9.78% | 99% | 64% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 2.34% | 9.15% | 9.25% | 1.55% | 9.24% | 10.13% | 99% | 65% |
| Class D | 3.34% | 10.12% | 9.73% | 1.73% | 9.84% | 9.79% | 101% | 61% |
| Class F | 1.51% | 5.54% | 4.46% | 1.55% | 7.34% | 7.06% | 98% | 69% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-16.2 anchor vs NNVC-16.2 HOP5

Anchor: NNLF LOP7 + NNIP

Test: NNLF HOP5 + NNIP

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4,21% | -12,30% | -15,36% | -6,01% | -14,07% | -14,60% | 330% | 7956% |
| Class A2 | -4,83% | -16,31% | -25,90% | -3,88% | -15,64% | -23,50% | 301% | 7338% |
| Class B | -4,72% | -22,25% | -16,22% | -3,22% | -19,41% | -13,33% | 307% | 7518% |
| Class C | -5,73% | -15,16% | -15,60% | -3,68% | -11,87% | -11,70% | 237% | 7149% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4,91% | -17,18% | -17,82% | -4,03% | -15,58% | -15,18% | 290% | 7466% |
| Class D | -6,27% | -16,37% | -16,91% | -3,21% | -10,74% | -9,41% | 229% | 7050% |
| Class F | -3,55% | -10,65% | -10,52% | -2,81% | -9,10% | -7,76% | 424% | 7274% |

- Contributions

4 contributions were submitted for AhG14, and 1 telco report.

| JVET-AP0048 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| --- | --- | --- |
| JVET-AP0053 | AhG14: SIMD Improvements of Operators in SADL Library | X. Chen, J. Zhang, Z. Chen (Wuhan Univ.) |
| JVET-AP0072 | [AHG14] NNLF compact dataset | A. Kherchouche, F. Galpin (InterDigital) |
| JVET-AP0211 | AHG14: New memory management policy in SADL | C. Hallam, F. Galpin, J. Pohl, S. Cizel (Interdigital) |
| JVET-AP0212 | AHG14: SADL Update | F. Galpin (InterDigital) |

- Configurations

The following configurations are used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration |
| --- | --- | --- |
| NNVC VTM mode | None | encoder_xxx_vtm.cfg |
| NNVC Anchor/EE1 | Intra Pred + LOP.6 | encoder_xxx_nnvc.cfg |
| NNVC. HOP | Intra Pred + HOP.5 | encoder_xxx_nnvc.cfg + nn-based/HOP5.cfg |
| NNVC. VLOP | Intra Pred + VLOP.4 | encoder_xxx_vtm.cfg + nn-based/vlop4.cfg |
| NNSR | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| RPR | Anchor RPR | nn-based/rpr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| NNPF | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |
| CALOP | Intra pred+adaptive LOP | encoder_randomaccess_nnvc.cfg+nn-based/intra.cfg+nn-based/calop.cfg |
| NNVC DRF | NNVC anchor + DRF | encoder_xxx_nnvc.cfg + drf.cfg |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Keep synchronized with VTM upstream

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

Encourage people to submit merge requests fixing identified bugs.

JVET-AP0015 JVET AHG report: Gaming content compression (AHG15) [S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs)]

- Software repositories

Software developed by AHG15 consists of forks of VTM/ECM that have support for reading auxiliary data that is provided for sequences of classes G1/G3. A third repository contains scripts for the verification of the auxiliary information. The repositories can be found at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gcc.

- Excel template

The updated excel templates for reporting results on Class Gaming are provided with this report.

- Test results

Test results were obtained according to the latest Gaming CTC document JVET-AO2027. The list of sequences/classes is copied for convenience below:

| Class | Sequence name | Frame count | Low delay frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 (SDR) | Level1_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | Darktree_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | ARPG2_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | DesertTown3_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | SunTemple3_SDR | 600 | 300 | 60 | 10 | M | M | M |
| G2 | GTAV | 600 | 300 | 60 | 8 | M | M | M |
|  | Minecraft | 600 | 300 | 60 | 8 | M | M | M |
|  | JianlingTemple | 600 | 300 | 60 | 8 | M | M | M |
|  | BaoleiYard | 300 | - | 60 | 8 | M | M | - |
|  | Wukong2 | 300 | - | 60 | 10 | M | M | - |
|  | Wukong3 | 300 | - | 60 | 10 | M | M | - |
| G3 (HDR) | ARPG2_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | DesertTown3_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | SunTemple3_HDR | 600 | 300 | 60 | 10 | O | O | O |

- VTM-11.0ecm19.1 vs ECM-19.1

- SDR

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -12.0% | -24.7% | -19.5% | 1213% | 941% | 359.2% |
| Class G2 | -15.5% | -23.1% | -26.2% | 1259% | 994% | 347.3% |
| Overall | -13.9% | -23.8% | -23.1% | 1238% | 970% | 352.7% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -28.1% | -46.9% | -38.3% | 991% | 2674% | 284.7% |
| Class G2 * | -25.4% | -37.3% | -38.4% | 1107% | 2214% | 278.3% |
| Overall * | -26.9% | -42.7% | -38.3% | 1041% | 2459% | 281.8% |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -21.3% | -55.4% | -44.3% | 1106% | 1606% | 274.7% |
| Class G2 | -18.9% | -50.8% | -49.6% | 1195% | 1839% | 280.2% |
| Overall | -20.4% | -53.7% | -46.3% | 1138% | 1689% | 276.8% |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -19.7% | -60.7% | -51.5% | 1025% | 1663% | 299.2% |
| Class G2 | -16.7% | -57.6% | -57.0% | 996% | 1524% | 307.5% |
| Overall | -18.6% | -59.6% | -53.6% | 1014% | 1610% | 302.3% |

* QP22 for Wukong2 and QP22/27 for Wukong3 are still missing for ECM. Summary is without those.

- HDR

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19.1 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -15.7% | -57.7% | -55.9% | -15.6% | -57.9% | -55.5% | 1246% | 685% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm20.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -34.4% | -71.9% | -71.0% |  |  |  | 839% | 1424% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm20.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -25.9% | -75.2% | -73.1% | -26.2% | -75.9% | -73.0% | 840% | 1645% |
|  | Low Delay P |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  |  |  | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 |  |  | -25.4% | -75.4% | -72.0% | -25.7% | -76.0% | -72.0% | 867% | 1353% |

- Input contributions

Three contributions listed below have been identified relating to the mandates of AHG15.

| Report |  |  |
| --- | --- | --- |
| JVET-AP0044 | AhG17/AhG15: New gaming sequences with auxiliary information captured from Carla driving Simulator | Z. Li, P. Jia, J. Sauer, T. Solovyev, E. Alshina (Huawei) |
| JVET-AP0118 | AhG15: Derivation of camera parameters for auxiliary picture reconstruction using Vision Transformers | V. Zakharchenko (Nokia) |
| JVET-AP0120 | AhG15: Depth map information derivation from reconstructed visual bitstreams consistency evaluation | V. Zakharchenko (Nokia) |

- Recommendations

The AHG recommends to:

- Review input documents on gaming content compression

- Identify if/how auxiliary information can be used for coding of gaming content

Was presented by V. Zakharchenko, due to zoom participants could not be heard in the JVET meeting room.

Currently, no plan to modify CTC JVET-Ax2027 again. Carla sequences might be considered in new CTC after CfP.

JVET-AP0016 JVET AHG report: Hardware implementation complexity (AHG16) [Y. Zhao, J. Park, I. Moccagatta (co-chairs), H. Huang, T. Ikai, X. Li, K. Naser, N. Song, G. Verba (vice chairs)]

- Input contributions

Contributions listed below have been identified relating to the mandates of AHG16.

| Category | JVET number | Title | Source |
| --- | --- | --- | --- |
| RDO counting and restriction | JVET-AP0169 | AHG16: Further report on weighted sum of all RDOs | Y. Tokumo, S. Hong, T. Ikai (Sharp) |
|  | JVET-AP0191 | [AHG16] VTM software extension for counting and constraining per-CU RDOs | K. Naser, F. Le Léannec, F. Galpin, P. de Lagrange (InterDigital), Y. Zhao, E. Alshina (Huawei), M. Wu, J. Yu (HiSilicon), Y. Tokumo, T. Ikai (Sharp) |
|  | JVET-AP0253 | AHG16: Comparative Study on RDO count and EncT under tool off test | Y. Tokumo, T. Ikai, S. Hong, K.-W. Liang (Sharp) |
| Simulation configuration for hardware encoding | JVET-AP0192 | [AHG16] Evaluating VTM under HW constraints | K. Naser, F. Le Léannec, F. Galpin, P. de Lagrange, Y. Zhao, E. Alshina (Huawei), Y. Tokumo, T. Ikai (Sharp) |

One AHG10 contribution on GOP8 Hierarchical B structure for Random Access configuration (i.e., JVET-AP0070) has been identified to be related to JVET-AP0192. It is suggested to discuss JVET-AP0070 and JVET-AP0192 together.

| JVET-AP0070 | AHG10: Additional random-access configuration with reduced delay | K. Andersson (Ericsson) |
| --- | --- | --- |

- Recommendations

The AHG recommends to:

- Review related input contributions.

In the context of reviewing documents submitted to AHG7 and AHG16, it should be identified if further update of the complexity reporting template JVET-AO2040 would be necessary (possible BoG activity).

JVET-AP0017 JVET AHG report: Preparation of Call for Proposals (AHG17) [J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs)]

- Activities

Three online AHG calls were held on 2026-02-03, 2026-03-10, and 2026-03-26. A hybrid AHG meeting was held on 2026-02-25/27 in Aachen, Germany. The reports of these meetings are available in JVET-AP0041.

Based on the discussions at these meetings, the proposed draft CfP document JVET-AP0047 was generated as input to the 42nd meeting, departing from the CfE document by reflecting proposed modifications as discussed during the 41st JVET meeting and in the AHG telcos.

- Related contributions

| JVET number | Title | Source |
| --- | --- | --- |
| JVET-AP0041 | AHG17: AhG meeting notes | M. Wien, J.-R. Ohm, F. Bossen |
| JVET-AP0042 | AHG17: Suggested editorial changes to the Draft Joint Call for Proposals on Video Compression with Capability beyond VVC | R. Chernyak, S. Liu (Tencent), T. Solovyev, E. Alshina (Huawei) |
| JVET-AP0043 | AHG17: Suggestion to enable BIM for the GOP-based RPR comparison point | K. Andersson, P. Wennersten (Ericsson) |
| JVET-AP0044 | AhG17/AhG15: New gaming sequences with auxiliary information captured from Carla driving Simulator | Z. Li, P. Jia, J. Sauer, T. Solovyev, E. Alshina (Huawei) |
| JVET-AP0045 | AHG17: Chroma sample location for CfP sequences | T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI) |
| JVET-AP0046 | AHG17: Report on dry-run expert viewing for test sequences under consideration for the CfP | M. Wien |
| JVET-AP0047 | AHG17: Updates of Draft Joint CfP text suggested by the AHG | J.-R. Ohm, M. Wien, F. Bossen |
| JVET-AP0050 | AHG17: Proposal of target bitrates for the HDR and UHD Gaming sequences for the upcoming CfP | C. Lehmann, A. Wieckowski, B. Bross, J. Pfaff (Fraunhofer HHI), Z. Li, J. Sauer, T. Solovyev, E. Alshina (Huawei) |
| JVET-AP0054 | [AHG4/AHG17] VQQ: An Interactive Display Assessment Tool for Subjective Tests | J. Liu, Z. Chen (Wuhan Univ.) |
| JVET-AP0064 | AHG17: Liverpool test sequences with updated chroma sample location and target bitrates | H. Dou, A. Foster, D. Collado, J. Calic (BBC) |
| JVET-AP0149 | AHG17: Additional sequence evaluation results for the upcoming CfP | D. Li, V. Rufitskiy, A. Filippov, H. Qin (TCL) |
| JVET-AP0186 | AHG17/AHG4: A Generic Full-Reference Objective Quality Assessment Method for Compressed Video | J. Wang, X. Zhuang, J. Zhang, L. Yu (Zhejiang Univ.) |
| JVET-AP0202 | AHG17: On per-frame data reporting in CfP | F. Bossen, F. Galpin, F. Le Léannec, K. Naser, E. François (InterDigital) |
| JVET-AP0237 | AHG17: Improved VTM configuration for 0.2x runtime target | F. Bossen |

It is noted that multiple documents have partial allocation to AHG17 and other AHGs. This includes JVET-AP0044, JVET-AP0054, JVET-AP0186.

- Recommendations

The AHG recommends:

- To further progress the development of the draft CfP document.

- To study the proposed new test sequences and rate points for the CfP and to assess the results corresponding expert viewing tests to be conducted during the meeting for the determination of the final CfP test set.

- To review the remaining input contributions related to AHG17 during the meeting.

A short verbal summary of the results of interim AHG meetings (JVET-AP0041) was also given.

A suggestion for viewing at the current meeting is in document JVET-AP0279.

JVET-AP0018 JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) [S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs)]

- Software development

Adoptions of last JVET meeting (JVET-AN2039: ULL CTC implementation, JVET-AO0208: GDR LDB fixes) have been merged to ‘ull-master’ branch: https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM/-/tree/ull-master.

- Related contributions

A total of 3 contributions (not including this AHG report) are identified relating to the mandates of AHG18. They are listed below. One contribution also related to the work of AHG9.

| Proposals |  |  |
| --- | --- | --- |
| JVET-AP0200 | AHG18: Random Access GOP4 Configuration for Live-streaming Applications evaluated in ULL test conditions | S. Ikonin, I. Gribushin, M. Sychev, K. Malyshev, V. Khamidullin, B. Shevchenko, X. Ma, E. Alshina (Huawei), Z. Deng (Bytedance) |
| JVET-AP0222 | AHG18: Decoder side NN-based picture generation for lost picture substitution | K. Malyshev, M. Sychev, S. Ikonin, E. Alshina (Huawei) |
| JVET-AP0226 | AHG9/AHG18: Missed Picture Generation purpose for NNPF SEI message | K. Malyshev, M. Sychev, S. Ikonin, E. Alshina (Huawei) |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Discuss test conditions and evaluation methodology.

- Collect test cases and specific requirements.

- Continue development of simulation software.

- Continue the study of ultra-low latency and packet loss resilience technologies in JVET.

(Report of Joint AhG on Gaussian splats JVET-AP0019 to be added and presented after the AhG meets on Sunday)

# Project development (42)

## AHG1: Deployment and advertisement of standards (2)

Contributions in this area were discussed during 1530–1545 on Monday 27 April 2026 (chaired by JRO).

JVET-AP0020 Deployment status of the HEVC standard [G. J. Sullivan]

Add content

JVET-AP0021 Deployment status of the VVC standard [G. J. Sullivan]

Add content

## AHG2: Text development and errata reporting (0)

This section is kept as a template for future use.

JVET-AP0058 in 6.1 is related.

## AHG3: Software development (3)

Contributions in this area were discussed during 1705–1745 on Tuesday 28 April 2026 (chaired by F. Bossen).

JVET-AP0130 AHG3/AHG9: Public source code release of real-time NNPF implementation in VVdeC [J. Funnell, M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, S. Schwarz (Nokia)]

This contribution reports a newly released fork of VVdeC that implements functionality to parse NNPFC and NNPFA SEI messages and apply NNPF in real time using OpenVino: a popular inference library with support for Intel NPUs.

For information.

Reports average gain of about 4% on SRH sequences (without weight update overhead). Gain drops to 3% with weight update overhead. Decoder is assumed to have access to a base model.

JVET-AP0243 Bug Fixes and Profile Support for MV-HEVC Reference Software (HM) [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)] [late]

This contribution documents bug fixes and enhancements to the MV-HEVC branch of the HEVC reference software (HM). i.e. the dev-multiview444 branch. The changes enable correct encoding and decoding of auxiliary picture bitstreams with various chroma format and profile combinations, including the Multiview Monochrome profiles defined in Annex G.11.1.3 of HEVC..

The implementation new profiles of MV-HEVC is feature-complete with these changes.

It was noted that the described merge request has already been merged into the dev-multiview444 branch.

JVET-AP0245 HDRTools software update for the Enhanced Colour Format Information and Picture Segmentation Information SEI messages [A. M. Tourapis, D. Podborski, J. Kim (Apple)] [late]

This document describes modifications to the HDRTools software package (version 0.27) to add support for the Enhanced Colour Format Information (ECFI, SEI payload type 228) and Picture Segmentation Information (PSI, SEI payload type 229) SEI messages as described in JVET-AN2032. Two new tools are added: ECFIConvert for chroma format conversion between 4:4:4 and 4:2:0 with separate chroma enhancement files, and PictureSegmentation for mask-based picture segmentation operations. The document also describes extensions to the HDRTools common library, including new Frame class methods and a JSON parser/writer for SEI metadata file support.

For information.

## AHG3: Test conditions and metrics (1)

Contributions in this area were discussed during 1910–1935 on Monday 27 April 2026 (chaired by JRO and MW).

JVET-AP0186 AHG17/AHG4: A Generic Full-Reference Objective Quality Assessment Method for Compressed Video [J. Wang, X. Zhuang, J. Zhang, L. Yu (Zhejiang Univ.)]

Recent advances in block-based hybrid video codecs and neural network-based codecs for next-generation compression, alongside the widespread adoption of high dynamic range (HDR) and wide color gamut (WCG) video content, pose significant challenges for existing objective quality metrics. These metrics often fail to maintain consistent correlation with human perception across different codec types, distortion patterns, and HDR/WCG content characteristics.

In this contribution, we propose FDIM, a generic full-reference objective quality assessment method with high correlation to human perception, strong generalization across traditional and learning-based codecs, and high robustness across SDR and HDR/WCG content, to support the objective assessment of next-generation video coding technologies.

It is recommended to consider FDIM as a candidate quality metric beyond PSNR, e.g., for studying perceptual coding algorithms, evaluating CfP proposals, etc.

The source code for FDIM has been released at https://github.com/MCL-ZJU/FDIM.

FDIM is a hybrid approach, averaging a learning based metric with traditional metric, as shown below:

Training data are obtained by crowd sourcing

FDIM has already being considered in AG 5, where investigation of metrics for better matching visual quality, this is an ongoing work item. In an activity such as the CfP, visual tests are necessary, and it would not be appropriate to over-burden proponents by computing various metrics. Further discussion was recommended in AG 5, which might also use the subjective results from the CfP to further investigate various metrics.

Was cross-checked in an AG 5 document

## AHG4: Subjective quality testing and verification testing (2)

Contributions in this area were discussed during 1815–1910 on Monday 27 April 2026 (Joint meeting chaired by JRO, MW, GJS).

JVET-AP0054 [AHG4/AHG17] VQQ: An Interactive Display Assessment Tool for Subjective Tests [J. Liu, Z. Chen (Wuhan Univ.)]

Due to inherent variations in panel characteristics and display-side processing, the same video content exhibit noticeable appearance differences across displays even after calibration. These discrepancies would lead to inconsistent subjective test results across different displays and laboratories. In this proposal, we contribute an interactive qualification tool, called Visual Quality Qualification (VQQ), for assessing perceptual consistency across different displays.

It commented that it might be difficult to get a proof that after calibration the visual results would be better aligned.

The tool could be valuable. It was suggested to further investigate offline (by responsible test coordinators) if it can be used for JVET or AG 5. This might also be useful for remote visual quality testing rather than labs who know how to calibrate their environment.

It was commented that it might be difficult to use the tool when video boards (rather than graphics boards) are used for video output.

JVET-AP0268 Viewing test results of verification test for VVC multilayer coding [M. Wien, V. Baroncini] [late]

(include abstract/results)

High-level summary of results shown below.

## AHG4: Test and training material (0)

This section is kept as a template for future use.

Related contributions in section 4.16.2 on test materials related to CfP, and JVET-AP0049/JVET-AP0228 in section 5.1.3 on training for neural-network based coding.

## AHG5: Conformance test development (1)

Contributions in this area were discussed during 1745–1755 on Tuesday 28 April 2026 (chaired by F. Bossen).

JVET-AP0244 MV-HEVC Conformance Bitstreams for Multiview Extended Profiles [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)] [late]

This document provides a set of conformance bitstreams for Multiview Extended, Multiview Extended 10, and Multiview Monochrome profiles specified in Annex G and Annex H of Rec. ITU-T H.265 | ISO/IEC 23008-2. These bitstreams are intended for inclusion in the HEVC conformance bitstream set (document w16061). Ten bitstreams are provided, covering inter-view prediction, simulcast, auxiliary picture types (depth and unspecified), mixed chroma formats, mixed bit depths, mixed resolutions, and monochrome layers. The bitstream specifications follow the format of w16061 to facilitate direct integration by the editors.

Bitstreams have not been cross-checked yet. I. Moccagatta volunteered to cross-check.

AM1008 Document contains description of multiview conformance bitstreams.

Decision: Issue new version of conformance document JVET-AP1008 with sufficient editing period, including these additional bitstreams.

## AHG7: Tool assessment (2)

Contributions in this area were discussed during 1510–1610 on Sunday 26 April 2026 (chaired by JRO).

JVET-AP0170 AHG7: Performance counters for reliable decoding time measurement [S. Hong, Y. Tokumo, T. Ikai (Sharp)]

In standard development, decoder execution time has long been recognized as unreliable due to various noise factors. As part of AHG7's mandate, there is a request to “Develop methodology of more reliable runtime measurement”. JVET-AO0131 proposed perf-cpp usage in reference software. This contribution uses perf as external tool and reported that experimental results confirm a strong correlation between decoder execution time and instruction counts. It is also reported that the overhead enabling performance counters measure is about 6 to 8 % in decoding time. It is recommended to include instruction counts as an option in the reporting template.

It was commented that there are various ways to reduce number of instructions in a software which people might use to reduce the number of instructions. Therefore, it might be undesirable to include this in the reporting template. It may also be difficult to interpret when comparing different software bases.

The information that instruction count gives may also be incomplete, e.g. depending on parallelization. Counting number of cycles might better reflect the runtime.

No agreement on taking action.

JVET-AP0207 AHG7: On Per-class BD-Rate Calculation [X. Li (Google)]

This contribution proposes a per-class BD-rate calculation based on average bitrate and MSE-based PSNR. This method prioritizes higher-bitrate sequences, reportedly offering an evaluation metric that better aligns with practical deployment scenarios.

It is shown with some examples that the per-class BD rate gain becomes higher if the highest rate sequence has most gain, and the other way round (highest bit rate gains least, or lowest birate gains most).

It was commented that the approach is equivalent to concatenating all sequences in a class and measure all together. This might even be extended, by considering rate used in the different QP points.

It was commented that a large divergence between the current method and the proposed method would occur in classes that contain sequences with largely different characteristics (e.g., some which only require very low rate).

One expert commented that it is very welcome to care more about high rate cases, but the proposed method might not be an ultimate solution to achieve this. Also another expert expressed support.

It is generally agreed that the problem that average BD rates are not sufficient to get the entire picture about benefit in compression, and that inhomogeneous behaviour must often be identified by looking at individual sequence results.

Further study in AHG on those aspects. Some approach of identifying inhomogeneous behaviour (could also be by a graphical representation, or some number that identifies this, and also considering at which rates most gain is achieved. This could be helpful to detect inconsistencies in results.

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

This section is kept as a template for future use, or can be removed after finalization of the TR/Sup.

## AHG10: Encoding algorithm optimization (1)

Contributions in this area were discussed during 1755–1805 on Tuesday 28 April 2026 (chaired by F. Bossen).

JVET-AP0070 AHG10: Additional random-access configuration with reduced delay [K. Andersson (Ericsson)]

This contribution suggests adding a hierarchical GOP configuration with length 8 for random-access to have a configuration with reduced delay compared to the current available random-access configurations (hierarchical GOP of lengths 16 and 32) for the VVC reference software. It is also suggested to update the software to support additional quantization settings for inter pictures every 16th and every 32th picture to mimic a larger hierarchical GOP.

The BD-rate impact of that compared to CTC using the latest VTM version 23.14:

RA Y/U/V: 7,90%*/13,08%*/13,37%* EncTime:100%* DecTime:100%* (to be updated)

Uses fewer reference pictures than the common size 32 GOP configuration used in CTC.

It was pointed out that having adaptive GOP size could be desirable.

Decision (SW): include proposed configuration (without additional QP changes) into next VTM release (non-CTC).

## AHG13: Film grain technology (1)

Contributions in this area were discussed during 1430–1445 on Tuesday 28 April 2026 (chaired by JRO).

JVET-AP0187 AHG13: Proposal to update the technical report on film grain synthesis technology for video applications [M. Radosavljević, Z. Ameur (InterDigital)]

This contribution proposes updates to the technical report on film grain synthesis technology for video applications. The updates include clarifications and refinements related to the frequency filtering model, particularly for film grain analysis based on this model. In addition, a new subsection describing learning-based approaches to film grain analysis is introduced. The updated document is attached to this contribution.

It was agreed to merge the proposed text (which is based on the ITU text) with JVET-AN2020, and issue this as a new version JVET-AP2020.

It was agreed to start work on second edition by issuing a request in WG 5. It would not make much sense to already go for CDTR, as that might be under ballot during the next meeting. For the ITU supplement, no action is possible at the current meeting, anyway.

## Implementation studies (2)

Contributions in this area were discussed during 1400–1430 on Tuesday 28 April 2026 (chaired by JRO).

JVET-AP0124 Further evaluation of Ali266 for real-time VVC encoding on smartphones [L. Wang, J. Li, J. Liang, K. Xie, S. Fang, X. Zhao, J. Chen, Y. Ye (Alibaba)]

This contribution presents results of further evaluation of the Ali266 software encoder for real-time Versatile Video Coding (VVC) encoding on smartphones. Building upon the results reported in JVET-AO0221, this work extends the assessment to encompass additional test scenarios and devices. We report the rate-distortion performance and encoding speed of Ali266 on the Xiaomi 17 smartphone using five Class B 1080p sequences from the JVET Common Test Conditions (CTC). Furthermore, we evaluate its performance on User-Generated Content (UGC) sequences captured from real-world short-video applications, tested on both the Xiaomi 17 and the Xiaomi 14 Pro. The results demonstrate that Ali266 achieves significant BD-rate improvements over the on-device HEVC hardware encoders while sustaining real-time encoding speeds across diverse mobile platforms and content types. Additionally, we provide a live demonstration of on-device real-time VVC encoding and decoding using a dual-smartphone setup including the Xiaomi 14 Pro.

BD rate gain compared to HEVC hardware encoder is >50%. 1080p RT encoding >60 fps possible in almost all cases on the newer model, >50 fps on the older one.

Power consumption was not measured, but may be less relevant for encoding.

Compared to VTM, compression performance is >25% worse.

Decision: Adopt JVET-AP0242 (new version of HEVC extensions JVET-AP1006)

JVET-AP0214 i266 decoder: Towards global VVC deployment on mobile devices [J. R. Arumugam, J. N. Shingala, M. B. Muthukrishnan (Ittiam)]

This informative contribution provides the performance results of i266 decoder. This software decoder is developed from scratch and optimized for ARMv8.x architectures to decode 8bit 4:2:0 VVC compliant bitstreams. The i266 software decoder is specifically designed for power efficient real time playback on a broad spectrum of mobile devices to assist mass market VVC adoption. Current performance shows 1080p60 real-time playback using single core for current generation flagship mobile devices and 720p30/60 real-time playback using 1 or 2 cores on 5-year-old budget friendly mobile devices. Comparative performance against vvdec shows that the i266 decoder is faster by roughly 30 to 50% for 1080p on client devices.

It was requested to provide the presentation deck.

## Profile/tier/level specification (2)

Contributions in this area were discussed during 1225–1245 on Wednesday 29 April 2026 during JVET plenary (chaired by JRO).

JVET-AP0242 On the Creation of Multiview 4:4:4 Profiles for HEVC [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri, S. Choi, W. Zia (Apple)] [late]

This contribution renews the proposal, first presented in JVET-AN0293, for the creation of the Multiview 4:4:4 10 and Multiview 4:4:4 12 profiles in Rec. ITU-T H.265 | ISO/IEC 23008-2. The original proposal demonstrated the need for multiview profiles capable of handling higher chroma sampling formats to support professional applications, consumer screen casting, and video-based 3D gaming. Since that proposal, a comprehensive MV-HEVC reference software implementation has been completed and submitted as a companion contribution, demonstrating full encoder and decoder support for multilayer 4:4:4 coding along with extensive bug fixes and enhancements to the HM codebase. The availability of this complete, tested software implementation removes a key barrier to adoption and provides a reference platform for conformance testing. It is requested that work on defining these profiles begin as soon as possible.

JVET-AP0246 HEVC Profile Signalling Reference [A. M. Tourapis, D. Podborski, J. Kim, W. Zia (Apple)] [late]

This document provides a comprehensive and consolidated reference for the profile, tier, and level signalling mechanisms in Rec. ITU-T H.265 | ISO/IEC 23008-2 High Efficiency Video Coding (HEVC). The definitions and characteristics of the numerous HEVC profiles are distributed across multiple annexes and clauses of the specification, requiring significant effort from implementers to interpret correctly. This contribution consolidates this scattered information into a single, accessible guide. It covers the profile identification mechanism, constraint flag signalling, per-profile characteristics, backward compatibility requirements, and the distinct constraint scoping rules for multi-layer profiles, with the goal of reducing implementation ambiguity and providing a clear reference for the HEVC community.

A significant portion of the data presented in the tables within this document was extracted and cross-referenced from the ITU-T H.265 specification (01/2026) with the assistance of Google's Gemini 2.5 Pro large language model.

For possible consideration in future edition of HEVC, requires further study. Proponents are asked uploading a new version with a patent statement.

## AHG15: Gaming content compression (2)

Contributions in this area were discussed during 1445–1510 on Tuesday 28 April 2026 (chaired by JRO).

JVET-AP0118 AhG15: Derivation of camera parameters for auxiliary picture reconstruction using Vision Transformers [V. Zakharchenko (Nokia)]

Gaming video content typically exhibits geometric structure defined by graphics rendering processes, including explicitly defined camera intrinsic and extrinsic parameters. While such camera parameters may be available at content generation time, they are not guaranteed to be preserved in practical delivery scenarios such as re-transmission, streaming, and intermediate re-encoding. This contribution investigates derivation of camera intrinsic and extrinsic parameters directly from reconstructed video content, without reliance on externally signaled auxiliary information. A learning-based approach employing a vision transformer architecture with separate prediction heads for intrinsic and extrinsic camera parameters is described. Sequences in classes G1/G3 of the JVET-AO2027 Common Test Conditions (CTC) for gaming applications are accompanied with auxiliary data, including depth map and camera parameters. The goal of this contribution is to support camera-aware processing for gaming content under AhG15 common test conditions when graphics rendering information is unavailable.

The sequences with “true” auxiliary parameters have camera homography and scene homography parameters, whereas the estimation algorithm that is used has only camera parameters which start at an assumed world origin which is the camera position at frame zero. Currently, the estimated results are not matching – further investigation necessary.

It is further mentioned that the algorithm was not trained with gaming data but with real camera data (to avoid training with test data), which might be part of the problem. It was suggested that the gaming engine of the Carla sequences might be used to generate training material out of the test set.

JVET-AP0120 AhG15: Depth map information derivation from reconstructed visual bitstreams consistency evaluation [V. Zakharchenko (Nokia)]

This contribution is a continuation of AO0151 and studies the influence of compression quantization noise on the quality of depth maps derived from decoded pictures. A visual transformer based depth estimation network, as described in AO0151, is applied independently per frame to generate a reference depth map from the original input YUV picture. The same network is then applied to reconstructed low-delay B pictures decoded at QP values 22, 27, 32, and 37 corresponding to the AhG15 CTC operating points. The resulting depth maps are compared against the reference depth maps to quantify degradation as a function of QP and content.

Depth maps are estimated from decoded pictures, such that they could be used at the decoder without transmitting them. If needed for post processing, this could save data.

It was commented that using a depth map estimated at the decoder for better compression of video might be critical, if the depth estimation would be overly complex and be part of the decoding loop.

Parties interested in investigating the model should contact the proponents.

## AHG16: Hardware implementation complexity (4)

Contributions in this area were discussed during 1610–1735 on Sunday 26 April 2026 (chaired by JRO).

JVET-AP0169 AHG16: Further report on weighted sum of all RDOs [Y. Tokumo, S. Hong, T. Ikai (Sharp)]

This contribution presents a further report comparing the weighted sum of all RDOs (TotalSumAllRDOS) in the AHG16 software and the non weighted RDO count proposed by JVET-AO0114. The experiment confirms that weighted sum in the AHG16 software has similar or more linearity between EncT and RDO count in CTC and reduced runtime configurations. It is recommended to use the AHG16 software metric and consider potential usage in the reference software in the next standard development.

What kind of weights are used? Block size.

It was clarified that both weighted and unweighted RDO counts are already in the software, and this contribution is just providing further analysis on their impact on runtime.

JVET-AP0191 [AHG16] VTM software extension for counting and constraining per-CU RDOs [K. Naser, F. Le Léannec, F. Galpin, P. de Lagrange (InterDigital), Y. Zhao, E. Alshina (Huawei), M. Wu, J. Yu (HiSilicon), Y. Tokumo, T. Ikai (Sharp)]

This contribution provides software implementation on top of VTM23.9 that measures and constrains the maximum number of per-CU RD cost computations, in addition to a weighted sum of all RDOs (Sum-RDO) that is correlated to the encoder time. It is based on the previous SW proposed in JVET-AO0075 with further added a feature of constraining the maximum allowed number of RD cost computations per CU for intra coding modes and inter coding modes.

An RDO is classified as an encoder decision that requires forward transform (and likely, entropy coding for rate estimation)

Individual restriction is possible for intra RDOs and inter RDOs.

The following results are obtained:

Test 1: (intra RDO: 10 & inter RDO: 10):

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.22% | 1.62% | 2.60% | 88% | 101% |
| Class A2 | 1.61% | 1.50% | 1.36% | 88% | 100% |
| Class B | 1.89% | 1.31% | 1.68% | 88% | 99% |
| Class C | 1.99% | 0.53% | 1.13% | 87% | 100% |
| Class E |  |  |  |  |  |
| Overall | 1.93% | 1.20% | 1.66% | 88% | 100% |
| Class D | 1.69% | 1.41% | 1.84% | 86% | 98% |
| Class F | 3.35% | 3.91% | 4.38% | 90% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.21% | 0.49% | 0.71% | 91% | 100% |
| Class C | 1.12% | 0.53% | 0.71% | 89% | 98% |
| Class E | 0.86% | -0.22% | 0.20% | 97% | 100% |
| Overall | 1.09% | 0.33% | 0.58% | 91% | 99% |
| Class D | 1.36% | 0.78% | 0.39% | 88% | 100% |
| Class F | 2.23% | 1.66% | 1.96% | 94% | 104% |

In terms of MaxRDOs, the average value is provided in the table below:

|  | Anchor |  |  | Test |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MaxRDO Joint | MaxRDO Luma | MaxRDO Chroma | MaxRDO Joint | MaxRDO Luma | MaxRDO Chroma |
| RA | 111.5 | 58.9 | 65.5 | 20.0 | 10.0 | 10.0 |
| LDB | 74.0 | 19.6 | 27.0 | 20.0 | 10.0 | 10.0 |

Test 2: (intra RDO: 5 & inter RDO: 5):

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-23.9 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5.22% | 6.52% | 7.56% | 78% | 102% |
| Class A2 | 4.09% | 6.59% | 5.37% | 79% | 100% |
| Class B | 4.57% | 7.30% | 7.90% | 79% | 99% |
| Class C | 5.31% | 6.86% | 7.84% | 75% | 99% |
| Class E |  |  |  |  |  |
| Overall | 4.80% | 6.88% | 7.31% | 78% | 100% |
| Class D | 5.14% | 8.15% | 8.45% | 74% | 99% |
| Class F | 9.63% | 14.37% | 15.93% | 79% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-23.9 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.72% | 4.01% | 4.33% | 81% | 99% |
| Class C | 3.25% | 4.52% | 4.72% | 76% | 103% |
| Class E | 2.31% | 4.50% | 4.31% | 93% | 94% |
| Overall | 2.80% | 4.30% | 4.46% | 82% | 99% |
| Class D | 4.45% | 6.77% | 5.61% | 74% | 102% |
| Class F | 7.37% | 8.64% | 10.03% | 85% | 105% |

The software is used in the contribution JVET-AP0192.

The limitation is same for all CU sizes (theoretically, smaller CUs should be restricted more).

It was asked if typical hardware encoders really restrict maximum number per CU. It was confirmed that in certain encoders that was the case, but other encoders may rather use CTU size for certain restrictions.

It is understood that the software implementation for mimicking behaviour of hardware encoders is just a starting point, and more functionality would be welcome.

See further notes under JVET-AP0192.

JVET-AP0192 [AHG16] Evaluating VTM under HW constraints [K. Naser, F. Le Léannec, F. Galpin, P. de Lagrange, Y. Zhao, E. Alshina (Huawei), Y. Tokumo, T. Ikai (Sharp)]

This contribution studies the VTM encoder under HW constraints. Specifically, VTM software is tested with constrained number of RDOs, smaller GOP size and reduced number of reference pictures. This setting was recommended in JVET-AO0053.

Using VTM software with RDO extension from JVET-AP0191, the tests were carried out under RA cfg with VTM CTC. The following tests are performed:

Test 1: Impact of small GOP and few references on reduced time environment

- Anchor: VTM-23.9 + RR3 cfg

- Test1-a: Anchor + GOP8 + 2 Ref

- Test1-b: Anchor + GOP8 + 1 Ref

Test 2 : Impact of small GOP and few references on reduced time environment and constrained RDOs

- Anchor: VTM-23.9 + RR3 + MaxAllowedIntraRDOs = 10 + MaxAllowedInterRDOs = 10

- Test2-a: Anchor + GOP8 + 2 Ref

- Test2-b: Anchor + GOP8 + 1 Ref

Test 3: Impact of small GOP and few references, reduced run time and constrained RDOs on VTM encoder

- Anchor: VTM-23.9

- Test3-a: VTM-23.9 + RR3 + GOP8 + 2 Ref + MaxAllowedIntraRDOs = 10 + MaxAllowedInterRDOs = 10

- Test3-b: VTM-23.9 + RR3 + GOP8 + 1 Ref + MaxAllowedIntraRDOs = 10 + MaxAllowedInterRDOs = 10

Test 4: Impact of small GOP and few references, reduced run time and more constrained RDOs on VTM encoder

- Anchor: VTM-23.9

- Test4-a: VTM-23.9 + RR3 + GOP8 + 2 Ref + MaxAllowedIntraRDOs = 5 + MaxAllowedInterRDOs = 5

- Test4-b: VTM-23.9 + RR3 + GOP8 + 1 Ref + MaxAllowedIntraRDOs = 5 + MaxAllowedInterRDOs = 5

The following results are obtained:

Test1-a: Anchor: VTM-23.9 + RR3 cfg, Test1-a: Anchor + GOP8 + 2 Ref

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-23.9 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.44% | 13.70% | 14.10% | 99% | 103% |
| Class A2 | 7.98% | 15.94% | 13.41% | 98% | 102% |
| Class B | 11.15% | 19.50% | 20.02% | 99% | 104% |
| Class C | 11.08% | 18.20% | 18.03% | 101% | 110% |
| Class E |  |  |  |  |  |
| Overall | 9.55% | 17.28% | 16.99% | 99% | 105% |
| Class D | 10.78% | 18.22% | 17.68% | 100% | 104% |
| Class F | 14.23% | 15.79% | 16.16% | 104% | 107% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-23.9 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 6.47% | 8.80% | 8.55% | 82% | 97% |
| Class C | 10.84% | 14.15% | 13.28% | 83% | 101% |
| Class E | 6.04% | 6.30% | 7.37% | 82% | 99% |
| Overall | 7.82% | 9.96% | 9.83% | 82% | 99% |
| Class D | 7.95% | 10.80% | 9.82% | 81% | 100% |
| Class F | 10.03% | 11.23% | 12.88% | 90% | 102% |

Test1-b: Anchor: VTM-23.9 + RR3 cfg, Test1-a: Anchor + GOP8 + 1 Ref

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-23.9 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.68% | 14.21% | 14.72% | 89% | 102% |
| Class A2 | 8.01% | 16.22% | 13.60% | 89% | 102% |
| Class B | 12.30% | 20.66% | 21.20% | 91% | 104% |
| Class C | 12.05% | 19.30% | 19.01% | 93% | 109% |
| Class E |  |  |  |  |  |
| Overall | 10.25% | 18.12% | 17.80% | 91% | 105% |
| Class D | 11.38% | 18.72% | 18.39% | 92% | 103% |
| Class F | 15.00% | 16.46% | 16.71% | 99% | 108% |

From this test, it can be observed that the impact of setting GOP size to 8 and reducing the number of reference frames to 2 is about 10% BD rate loss. Reducing further the number of reference frames to 1 comes with less than 1% BD rate loss.

Test2-a:

Anchor: VTM-23.9 + RR3 + MaxAllowedIntraRDOs = 10 + MaxAllowedInterRDOs = 10, Test: Anchor + GOP8 + 2 Ref

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.40% | 14.00% | 13.88% | 98% | 102% |
| Class A2 | 7.96% | 15.83% | 13.66% | 97% | 102% |
| Class B | 11.19% | 19.40% | 19.87% | 98% | 104% |
| Class C | 11.15% | 18.52% | 18.12% | 99% | 108% |
| Class E |  |  |  |  |  |
| Overall | 9.57% | 17.37% | 16.96% | 98% | 104% |
| Class D | 10.83% | 18.14% | 17.84% | 99% | 104% |
| Class F | 14.41% | 15.77% | 15.94% | 103% | 109% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 6.40% | 8.65% | 8.73% | 81% | 101% |
| Class C | 10.90% | 14.03% | 13.39% | 81% | 106% |
| Class E | 6.16% | 7.14% | 7.74% | 80% | 95% |
| Overall | 7.84% | 10.07% | 10.03% | 81% | 101% |
| Class D | 7.92% | 10.71% | 10.66% | 80% | 99% |
| Class F | 10.17% | 12.96% | 12.87% | 89% | 99% |

Test2-b:

Anchor: VTM-23.9 + RR3 + MaxAllowedIntraRDOs = 10 + MaxAllowedInterRDOs = 10, Test: Anchor + GOP8 + 1 Ref

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.61% | 14.16% | 14.70% | 88% | 102% |
| Class A2 | 7.97% | 16.01% | 13.74% | 87% | 102% |
| Class B | 12.37% | 20.56% | 21.22% | 89% | 105% |
| Class C | 12.15% | 19.50% | 18.91% | 90% | 108% |
| Class E |  |  |  |  |  |
| Overall | 10.28% | 18.09% | 17.80% | 89% | 104% |
| Class D | 11.60% | 18.54% | 17.98% | 90% | 103% |
| Class F | 15.21% | 16.51% | 16.73% | 97% | 109% |

From this test, the same conclusion can be drawn. The impact of setting GOP size to 8 and reducing the number of reference frames to 2 is about 10% BD rate loss. Reducing further the number of reference frames to 1 comes with less than 1% BD rate loss.

Test3-a:

Anchor: VTM-23.9, Test3-a: VTM-23.9 + RR3 + GOP8 + 2 Ref + MaxAllowedIntraRDOs = 10 + MaxAllowedInterRDOs = 10

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 18.77% | 28.50% | 31.70% | 14% | 105% |
| Class A2 | 22.89% | 33.70% | 31.20% | 16% | 105% |
| Class B | 22.43% | 35.45% | 37.54% | 15% | 106% |
| Class C | 21.37% | 29.88% | 31.29% | 14% | 105% |
| Class E |  |  |  |  |  |
| Overall | 21.51% | 32.22% | 33.44% | 15% | 105% |
| Class D | 19.10% | 28.84% | 29.91% | 14% | 98% |
| Class F | 29.25% | 31.71% | 33.76% | 22% | 111% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 16.99% | 23.25% | 27.15% | 14% | 98% |
| Class C | 19.77% | 24.90% | 25.49% | 13% | 95% |
| Class E | 18.60% | 21.71% | 22.92% | 18% | 92% |
| Overall | 18.32% | 23.41% | 25.54% | 15% | 96% |
| Class D | 15.42% | 17.86% | 20.93% | 14% | 90% |
| Class F | 27.57% | 32.27% | 33.47% | 21% | 98% |

Test3-b:

Anchor: VTM-23.9, Test3-a: VTM-23.9 + RR3 + GOP8 + 1 Ref + MaxAllowedIntraRDOs = 10 + MaxAllowedInterRDOs = 10

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 19.01% | 28.68% | 32.69% | 13% | 105% |
| Class A2 | 22.91% | 33.93% | 31.31% | 14% | 105% |
| Class B | 23.78% | 36.82% | 39.13% | 14% | 107% |
| Class C | 22.49% | 31.00% | 32.22% | 13% | 104% |
| Class E |  |  |  |  |  |
| Overall | 22.31% | 33.06% | 34.43% | 14% | 106% |
| Class D | 19.95% | 29.32% | 30.08% | 13% | 98% |
| Class F | 30.19% | 32.60% | 34.74% | 21% | 110% |

From this test, the impact of the imposing the HW constraints on the VTM encoder leads to 21% BD rate loss. With additionally reducing the number of reference frames to 1, less than 1% coding loss is observed.

Test4-a:

Anchor: VTM-23.9, Test3-a: VTM-23.9 + RR3 + GOP8 + 2 Ref + MaxAllowedIntraRDOs = 5 + MaxAllowedInterRDOs = 5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 21.64% | 33.41% | 36.91% | 13% | 107% |
| Class A2 | 25.51% | 39.85% | 35.67% | 14% | 106% |
| Class B | 25.35% | 42.55% | 45.04% | 14% | 106% |
| Class C | 25.19% | 36.83% | 38.97% | 13% | 102% |
| Class E |  |  |  |  |  |
| Overall | 24.60% | 38.66% | 39.92% | 13% | 105% |
| Class D | 23.01% | 36.68% | 37.70% | 12% | 98% |
| Class F | 37.23% | 43.98% | 47.36% | 19% | 109% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 18.49% | 26.93% | 30.90% | 12% | 99% |
| Class C | 21.88% | 28.47% | 29.29% | 12% | 97% |
| Class E | 20.21% | 25.11% | 24.83% | 17% | 95% |
| Overall | 20.05% | 26.99% | 28.85% | 13% | 97% |
| Class D | 17.78% | 22.21% | 24.99% | 12% | 91% |
| Class F | 33.82% | 39.59% | 42.17% | 19% | 98% |

Test4-b:

Anchor: VTM-23.9, Test3-a: VTM-23.9 + RR3 + GOP8 + 1 Ref + MaxAllowedIntraRDOs = 5 + MaxAllowedInterRDOs = 5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over Test |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 21.92% | 33.88% | 37.78% | 11% | 107% |
| Class A2 | 25.58% | 40.26% | 35.91% | 13% | 104% |
| Class B | 26.71% | 44.14% | 46.59% | 13% | 106% |
| Class C | 26.31% | 37.83% | 39.75% | 11% | 105% |
| Class E |  |  |  |  |  |
| Overall | 25.42% | 39.63% | 40.87% | 12% | 106% |
| Class D | 23.70% | 37.19% | 38.22% | 11% | 99% |
| Class F | 38.07% | 44.54% | 48.02% | 18% | 109% |

From this test, the impact of the imposing the HW constraints with further reducing the number of RDOs to 5 leads to 24% BD rate loss. With additionally reducing the number of reference frames to 1, less than 1% coding loss is observed.

Summary of the results:

|  | GOP |  | Num Ref |  | Add cfg |  | MaxRDOs |  | Y | EncT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Anchor | Test | Anchor | Test | Anchor | Test | Anchor | Test |  |  |
| Test1-a | 32 | 8 | Def | 2 | RR3 | RR3 | Def | Def | 9.55% | 99% |
| Test1-b | 32 | 8 | Def | 1 | RR3 | RR3 | Def | Def | 10.25% | 91% |
| Test2-a | 32 | 8 | Def | 2 | RR3 | RR3 | 10 | 10 | 9.57% | 98% |
| Test2-b | 32 | 8 | Def | 1 | RR3 | RR3 | 10 | 10 | 10.28% | 89% |
| Test3-a | 32 | 8 | Def | 2 |  | RR3 | Def | 10 | 21.51% | 15% |
| Test3-b | 32 | 8 | Def | 1 |  | RR3 | Def | 10 | 22.31% | 14% |
| Test4-a | 32 | 8 | Def | 2 |  | RR3 | Def | 5 | 24.60% | 13% |
| Test4-b | 32 | 8 | Def | 1 |  | RR3 | Def | 5 | 25.42% | 12% |

It was agreed to include the software from JVET-AP0191 in the AHG16 software for further experimentation. Contributions from other companies with experience in hardware encoder design are more than welcome to include more possible approaches of mimicking constraints imposed by hardware encoders (including possibly more sophisticated ones that cause less loss in compression).

It was commented that at this moment it would be premature to define a starting point for restrictions in software of the next generation standard, as it cannot be known what that software would be, and in an initial phase after January it would be most important to establish a stable code base for experimentation.

JVET-AP0253 AHG16: Comparative Study on RDO count and EncT under tool off test [Y. Tokumo, T. Ikai, S. Hong, K.-W. Liang (Sharp)] [late]

This contribution presents comparative study of the RDO count, calculated as the weighted sum of all RDOs, and EncT under tool-off testing using AHG16 software. The experiment indicates that, for some tools, the RDO count and EncT values are similar, while for others, they exhibit different behavior. This suggests that using RDOs as well as EncT can be helpful for evaluating the complexity characteristics of each tool. It is recommended to use the AHG16 software metrics and consider their potential application in the reference software for the next standard development.

TO test is performed on per-tool basis. There are cases where the number of RDOs increases, perhaps because more residual needs to be encoded. In some cases, the number of RDOs increases but encoding time decreases when a tool is switched off. This might indicate that number of RDOs might not be useful as a standalone criterion for the benefit of a tool.

It was concluded that none of the contributions in 4.8 and 4.15 provided new aspects to be included in the complexity reporting template – a new version JVET-AP2040 is not needed.

## AHG17: CfP on video coding technology beyond VVC (14)

JVET-AP0041 AHG17: AhG meeting notes [M. Wien, J.-R. Ohm, F. Bossen]

No need for separate presentation – part of AHG report.

### Proposed draft CfP text (4)

Contributions in this area were discussed as noted below.

JVET-AP0042 AHG17: Suggested editorial changes to the Draft Joint Call for Proposals on Video Compression with Capability beyond VVC [R. Chernyak, S. Liu (Tencent), T. Solovyev, E. Alshina (Huawei)]

No need for presentation – was reviewed during the Aachen meeting (see JVET-AP0041), and partially included in JVET-AP0047 versions of edited CfP.

JVET-AP0043 AHG17: Suggestion to enable BIM for the GOP-based RPR comparison point [K. Andersson, P. Wennersten (Ericsson)]

This was reviewed during the Aachen meeting (see JVET-AP0041). Viewing was performed there, and it had been concluded that it was not harmful using the method, but there was no evidence that visual quality would be improved when BIM=2 would be used (confidence intervals overlapping), compared to RPR encodings that use BIM=0 (see report JVET-AP0041), but no final recommendation about using it was made.

Short presentation of the document was given Wednesday 29 April at 1645.

After short discussion, it was agreed to use BIM=2 for the additional VTM encodings employing RPR (not in anchors), which have the main purpose of getting better understanding how much benefit comes from proposals using some kind of adaptive resolution coding. It is assessed that this is in compliance with the general coding conditions of the CfP; this encoder-side tool is also properly documented in JVET-AO2002, and its is usage is documented in the configuration files which will be made available. No need for change in draft CfP.

JVET-AP0047 AHG17: Updates of Draft Joint CfP text suggested by the AHG [J.-R. Ohm, M. Wien, F. Bossen on behalf of AHG]

On Monday 27 April 1600-1800 JVET-AO2026-v3 was reviewed (chaired by M. Wien). Modifications were uploaded in JVET-AP0047.zip as JVET-AO2026-v4 (look at changemarks by M. Wien). Further discussion was conducted Tuesday 28 April during 0900-1215 (chaired by M. Wien and JRO), changes agreed documented in JVET-AO2026-v5. (note: For the changes in  -v6, see notes under JVET-AP0285)

Follow-up discussion was conducted on Wednesday 29 April 1430-1630, results are documented in -v7. The main purpose of that session was about finalization of the sequence selection, as documented below.

  - It was agreed to replace Driving POV by TravellerSwim in SRH category (already in -v6)

The following suggestions/comments were made for HDR-PD:

- Keep all 5 sequences in HDR-PQ, and add two new in HDR-HLG for symmetry

- Remove Meridian

- Remove Fashion Lady

- Remove Sparks Welding

- Remove Neptun Fountain

For reasons of symmetry PQ/HLG, it would be good to keep Fashion Lady, as then both categories would have two cropped 8K, and either 2 or 3 4K sequences.

Among the proposals above, consensus was reached to remove Neptun Fountain.

The following suggestions/comments were made for HDR-HLG (one sequence to be added):

- Seawalk has best characteristics to view artifacts, but low in rate.

- Crowdwalk may be too similar to Crowdrun in terms of content; it has medium rate, and has a significant variety of dark content, has highest dynamic range, and also appropriate to see artifacts in various.

- Towerplants has highest rate, but the bit distribution over the picture may have large impact on the visual appearance, may not be ideal for viewing.

Towerplants is least favored generally. Among the other two, more preference was expressed for Seawalk which was finally selected.

The following suggestions/comments were made for Gaming UHD Wukong (one out of three sequences to be added):

- Preference was expressed for Wukong2 during the Aachen meeting, and again in the current session, being most representative in terms of content.

- In terms of characteristics for viewing, all Wukong are similar. Wukong 2 is more challenging than Wukong 1, and only slightly less than Wukong 3.

Consensus was reached to use Wukong 2.

The following suggestions/comments were made for Gaming UHD Carla (one out of three sequences to be added):

- In terms of viewing characteristics, Carla 3 is least favored.

- Carla 4 has more variation in the scene than Carla 5, whereas Carla 5 has more texture on the road and in the buildings, and the higher consistency/less changes may be better for evaluation, also the rendering artifacts are less than Carla 4 (note that as a side remark, opinions were expressed that in general Carla sequences are much better than the first versions, but still have rendering problems).

Consensus was reached to use Carla 5.

Camellia should be kept, representing highest quality in detail, and Sports-Iceball is excellent for viewing. Among the other three, consensus was reached to remove Vertical-Walk, as it is not optimum for detecting artifacts. On the other hand, Hobby and Vertical-Snow were preferred to be kept both, due to their different scene characteristics (text overlay in Hobby, fast moving camera in Snow). Landscape-Night is not optimum for viewing and was agreed to be removed.

Revisit: A work plan needs to be established, providing configuration files, identifying volunteers, etc., to generate VTM anchors and additional VTM encodings which are due end of May according to CfP timeline. F. Bossen was asked to generate an Excel sheet with all streams that need to be generated, such that volunteering experts/companies can be filled in.

JVET-AP0202 AHG17: On per-frame data reporting in CfP [F. Bossen, F. Galpin, F. Le Léannec, K. Naser, E. François (InterDigital)]

Abstract:

The draft CfP requires reporting of bit counts for each frame of each submitted bitstream. It is proposed to extend the reporting requirement to include per-frame PSNR values, such as to enable the computation of alternative combined YUV metrics.

It was agreed:

  - to include the additional PSNR information as proposed

  - Precision of reported metrics to stay at the precision reported by HDRTools by default.

### Test sequences and conditions (10)

Contributions in this area were discussed as noted below.

JVET-AP0054 and JVET-AP0186 in 4.5 are also related to AHG17.

JVET-AP0044 AhG17/AhG15: New gaming sequences with auxiliary information captured from Carla driving Simulator [Z. Li, P. Jia, J. Sauer, T. Solovyev, E. Alshina (Huawei)]

No need for presentation – was reviewed in Aachen meeting (see JVET-AP0041) – updates of sequences are included in JVET-AP0279.

JVET-AP0045 AHG17: Chroma sample location for CfP sequences [T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI)]

No need for presentation – was reviewed in Aachen meeting (see notes in JVET-AP0041).

JVET-AP0046 AHG17: Report on dry-run expert viewing for test sequences under consideration for the CfP [M. Wien]

No need for presentation – was reviewed in Aachen meeting (see notes in JVET-AP0041).

JVET-AP0050 AHG17: Proposal of target bitrates for the HDR and UHD Gaming sequences for the upcoming CfP [C. Lehmann, A. Wieckowski, B. Bross, J. Pfaff (Fraunhofer HHI), Z. Li, J. Sauer, T. Solovyev, E. Alshina (Huawei)]

Sequences included in JVET-AP0279.

For the earlier versions of this document no need for presentation was foreseen, but -v6 had substantial changes including a suggestion to modify the CfP. This was presented and discussed on Wednesday 29 April 1730-1805.

Abstract:

In this proposal, the proponents propose and update a set of UHD SDR and HD HDR gaming sequences to be evaluated as additional candidates for the upcoming CfP. The contribution comprises three distinct sets of synthetic sequences. The Wukong sequences, which have already been evaluated and recommended as candidates, will be presented as an update with rate-matched results for the latest VTM. The Carla sequences have been adapted based on feedback from the subjective viewings in AHG17. Additionally, three new HD HDR gaming sequences, already part of the Gaming CTC, are introduced for evaluation for the next-generation video coding standard. After subsequent viewing of the sequences at the 42nd meeting, it was agreed to add Wukung2 and Carla5 into the CfP draft and do not evaluate the new HD HDR gaming sequences subjectively. HDR gaming, especially with the ground truth auxiliary metadata, is a relevant use case that should be covered and evaluated in some way at the CfP as well. Hence, it is proposed to evaluate this HD HDR gaming category only objectively as for the submitted R5 rate points. Furthermore, we consider streaming of gaming content in a RA configuration a relevant use case and propose to also evaluate this category objectively as well. A proposed modified draft CfP is attached to this document.

From the discussion:

It was commented that the main problem in the gaming category is the missing variety of content (as concluded in the meeting in Aachen), which is an important aspect for future standard CTC, and might also be looked at in the selection of the supplementary set (not necessarily from the set of sequences that have been known for a long time), where also RA configurations of gaming sequences could be conducted. Adding more sequences of Wukong and Carla might not be beneficial in this context.

It was commented that the aspect of using metadata obtained from gaming sequence rendering is still under investigation in AHG15. Auxiliary data are only available only for some sequences, investigation about potentially generating them artificially by estimation is ongoing (see section 4.14), and it was already concluded in the last meeting that it would be premature to attach them to the CfP sequence set, even if they were available for some sequences. It is understood that this could become an important topic during standard development. Also, supplemental information about benefits by using metadata of gaming sequences would be highly welcome in the context of a proposal.

No action at this stage.

JVET-AP0064 AHG17: Liverpool test sequences with updated chroma sample location and target bitrates [H. Dou, A. Foster, D. Collado, J. Calic (BBC)]

No need for presentation – sequences included in JVET-AP0279.

JVET-AP0149 AHG17: Additional sequence evaluation results for the upcoming CfP [D. Li, V. Rufitskiy, A. Filippov, H. Qin (TCL)] [late]

No need for presentation – sequences included in JVET-AP0279.

JVET-AP0237 AHG17: Improved VTM configuration for 0.2x runtime target [F. Bossen] [late]

The VTM reference for the upcoming CfP includes a “fast” 0.2x reference point. Modified VTM configurations are proposed for this reduced encoder run-time reference point. It is asserted that the proposed configurations provide a BD-rate improvement of about 3-4% on average under CfP conditions as well as a visual improvement.

Version 2 contains the following corrected results: the rate 2 for SRH2_DrivingPOV4 was corrected from 350 to 300 kbps and the BBC sequences with corrected chroma location were used (see JVET-AP0064).

Was presented Wednesday 29 April 1705.

Agreed to use the proposed configurations for the 0.2x VTM encodings in CfP.

No changes in configurations of the other two VTM encodings in the <1x range (in the CfE, roughly 0.4x and 0.75x were generated)

JVET-AP0279 AHG17: Candidates for expert viewing tests [M. Wien]

This document provides a collection of the candidate test sequences with associated rate points for subjective evaluation in expert viewing tests at the 42nd meeting. It is recommended to include the VTM anchor configuration for all candidates in these tests. It is suggested to add relevant additional points e.g. for the RPR and/or 0.2x configurations in order to confirm the selection choice in terms of test sequences and target bitrates for the final CfP test set.

Presentation and discussion on Friday 24 April 1515-1615.

It was agreed to also include rate point 4 suggested in JVET-AP0149 for Fashion Lady in the test.

Adding gaming sequences with HD+HDR would need to introduce an eighth category (otherwise SDR and HDR would be mixed), with only 3 sequences proposed in JVET-AP0050. Mixing HD content with UHD in the same viewing session for HDR would also be problematic.

The number of sequences in the CfP should not be extended. Also, some of the gaming sequences proposed in HDR versions (from G3 gaming CTC) were investigated as SDR versions (G1) before, and other sequences were preferred in terms of content characteristics suitability for subjective viewing.

At this moment, it was concluded that it appears better to defer the extension of gaming content for new standard development for the time after the CfP (seeking for even higher variety), and at this moment consider adding one Wukong and one Carla sequences, as suggested by AHG17.

It might also be desirable to include more gaming sequences in the additional set of sequences to be defined after submission of subjective-viewing CfP materials.

It was agreed to test the set of sequences as suggested in JVET-AP0279 (only rates 1-4 in the visual test), plus a second variant of R4 for Fashion Lady.

Also the sequences from JVET-AP0050 should be viewed, but not as new candidates for the CfP.

For the new sequences (BBC, Carla, AP0050), also RPR versions should be included.

JVET-AP0280 AHG17: Proposal of target bitrates for the new HD-RA-SDR sequence TravellerSwim [M. Abdoli, A. Tissier (Xiaomi)] [late]

No need for presentation – sequences included in JVET-AP0279

JVET-AP0285 AHG17: Report on 2nd dry-run expert viewing for test sequences under consideration for the CfP [M. Wien, J. Calic, C. Lehmann, P. de Lagrange]

A first presentation of the expert viewing test results was given on Sunday 26 April 1735-1800.

Results are assessed to be appropriate to make final corrections in the rate points later during the meeting. More discussion was planned after availability of the detailed results.

A side activity (M. Wien, B. Bross, J. Calic, K. Andersson, F. Bossen, E. Alshina, P. de Lagrange, C. Lehmann, meeting held Tuesday 1430-1530) was given the task to further analyse the results, and propose which rate points should be used in the final draft CfP

  - For sequences that were already in the draft CfP (investigated during the AHG meeting in Aachen, but tested now with modified rates), to suggest whether the previous rates or the newly tested rates should be used

  - For the newly proposed sequences, make a proposal whether further adjustments of the rate range and intermediate rate points would be necessary.

A verbal report of the side activity was given Tuesday 1545. It was noted that generally, it appears as if the experts at the current meeting assessed the quality more critical than in Aachen. Based on the suggestions made by the side activity, confirmation of rate points and final discussion about sequence selection to be conducted in JVET as follows.

The following suggestions were made (updates made during the discussion documented below):

  - Confirm new rate points from JVET-AP0279 for SRU3 and SRU4

  - Confirm new rate points from JVET-AP0279 for SRH2

  - Confirm new rate points from JVET-AP0279 for Traveller Swim

  - Confirm new rate points from JVET-AP0279 for SLH1, SLH4

  - HPQ2 requires further modification: 250, 650, 1700, 4500

  - Confirm new rate points from JVET-AP0279 for HPQ5

  - Confirm new rate points from JVET-AP0279 for URH1

  - Rate points for Carla3 from JVET-AP0279 are OK

  - All rate points for Carla4 should be lower than in JVET-AP0279, new range 800…10000 (factor 2.3)

  - Rate points for Carla5 from JVET-AP0279 are OK

  - Rate points 1,3,4 for Wukong1 should be lower than in JVET-AP0279, 700, 1800, 4000, 8500

  - Rate points for Wukong2 should be higher than in JVET-AP0279, 1000, 2400, 6000, 14000

  - Rate points 3 and 4 for Wukong3 should be higher than in JVET-AP0279, 800, 2000, 6000, 18000

  - Rate points 2, 3 and 4 for Crowdwalk should be lower than in JVET-AP0279, 500, 900, 1800, 3600

  - All rate points for Seawalk should be lower than in JVET-AP0279, 200, 400, 800, 1800

  - Rate points 1-3 for Towerplants should be lower than in JVET-AP0279, e.g. 800, 2000, 4000, 12000

No objection was raised against those changes during the meeting on Thursday.

M. Wien was tasked to integrate the changes above in the draft CfP document and make this available as JVET-AO2026-v6 in the JVET-AP0047.zip container. For follow-up discussion, see under JVET-AP0047.

## AHG18 Ultra-low latency and packet loss resilience (3)

Contributions in this area were discussed during 1515–1545 on Tuesday 28 April 2026 (chaired by JRO).

JVET-AP0226 in 6.2.4 is also related.

JVET-AP0200 AHG18: Random Access GOP4 Configuration for Live-streaming Applications evaluated in ULL test conditions [S. Ikonin, I. Gribushin, M. Sychev, K. Malyshev, V. Khamidullin, B. Shevchenko, X. Ma, E. Alshina (Huawei), Z. Deng (Bytedance)]

This contribution reports evaluation results of random access GOP4 configuration suggested by contribution JVET-AM0238 in low latency scenario. Under ULL test conditions described in JVET-AN2039 with latency restriction modified to the range of 70 to 130 ms it was observed that temporal layer 0 is delivered in most of the cases confirming the benefits of temporal scalability in low latency scenario. At the same time temporal layers 1 and 2 are often missed due to network transmission issues leading to instant FPS drop clearly noticeable during video playback. The ULL CfE response published as contribution JVET-AN0080 was also evaluated in the same conditions, and at QP22 demonstrated 2.1 dB of average Y-PSNR improvement, and 28% to 16% of freeze ratio reduction for transmission priority channel, and 0.8 dB average Y-PSNR improvement and 31% to 27% freeze ratio reduction on a channel without transmission priorities, with 1.9% BD-Rate overhead of Y-PSNR. The modifications required for evaluation of RA configuration on top of AHG18 simulation software are also described.

It was agreed to include this as an option for further study into the AHG software.

It was commented that this is not “ultra” low latency, unlike the other scenarios/configurations investigated so far, but a higher variety of cases is probably welcome.

JVET-AP0222 AHG18: Decoder side NN-based picture generation for lost picture substitution [K. Malyshev, M. Sychev, S. Ikonin, E. Alshina (Huawei)]

This document describes an experiment of applying NNFP functionality for generation of lost pictures unavailable due to network transmission issues in ultra-low latency scenario, and proposes modifications to the AHG18 software to enable this functionality based on existing NNPF SEI mechanism. The proposed implementation is controlled via the NNPF interface and utilizes the additional syntax proposed in JVET-AP0226. The generated pictures can be used both for output to display, and for lost reference frames substitution controlled by specific flag. Being tested according to ULL CTC JVET-AN2039, this picture generation technique allows to reduce video freezes significantly, and noticeable improves subjective quality for short interval freeze substitution. At the same time long distance extrapolation remains challenging. The contribution provides a starting point for further investigation and improvement in various aspects of missed picture generation functionality.

Complexity? Not precisely described, run on GPU.

It was asked if also other information related to the lost picture (beyond reconstructed pixels) would be generated? No

Same basic architecture as for the error-correcting loop filter proposed before, also presented in CfE. Kind of non-normative version of it.

Agreed to be included in AHG software (same branch as for loop filter).

JVET-AP0286 On ultra-low latency and packet loss resilience functionality [S. Ikonin, X. Ma, E. Alshina (Huawei)] [late]

In order to insure the ultra-low latency and packet loss resilience requirement will be satisfied, this contribution suggests to address this functionality during collaborative phase of next generation video coding standard, and use the test conditions designed by JVET-AhG18 and approved by JVET as document JVET-AN2039, for assessment the technologies and finding proper balance of loss in coding efficiency and benefits achieved by error resilience at the coding layer.

This document is a mirroring contribution also submitted to WG 02 as m76950 and VCEG-CA03, and suggested for joint discussion.

Was presented and discussed in the joint meeting with VCEG and WG 2 Requirements on Wednesday 29 April 1830-1915 (see further notes in section 7.3.3).

It was commented that the purpose of this document was not fully clear, as basically the requirements that are targeted were already included in the draft requirements document that had been issued from the April 2025 MPEG/VCEG meetings. Further defining test conditions as requested is not typically an objective of a requirements document.

It was commented that the achievements made in AHG18 are highly appreciated, and JVET has an initial testbed for this purpose.

It was confirmed that further development of CTC related to low latency and error resilience is highly welcome, including more variety of network types, protocols and transmission aspects with focus on future development, relevant use cases, efficient error concealment, and metrics, such that this could be used early enough in the collaborative phase of future standard development.

## Video CICP, generic aspects of metadata (3)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0221 Proposed definitions for alpha planes in Video CICP [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

TBP

JVET-AP0224 Proposed definitions for depth map in Video CICP [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

TBP

JVET-AP0283 On the Placement of Metadata and Coordination with File Format [D. Podborski, A. M. Tourapis, J. Kim (Apple Inc)] [late]

No review as objection was raised against discussing this late document.

# Low-level tool technology proposals (41)

## AHG11/AHG14: Neural network-based video coding (24)

### Summary and BoG reports

Contributions in this area were discussed during 1015–1300 on Saturday 25 April 2026 (chaired by JRO).

JVET-AP0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)]

Code base for EE1 tests was NNVC16.0, the anchor for the majority of EE1 tests is default configuration of NNVC-16.0 (NN-Intra and LOP7 filter enabled). NNVC common test conditions [1], results and complexity reporting template were used.

For majority of proposals AhG11 training set, which consists of DIV2K [2], BVI-DVC [3] and TVD [4] was used. NN-based Inter tools were trained using Vimeo 90K triplet [5]. Comparison is done between tests which use the same sub-set of training data.

It was agreed for tests competing with technologies in NNVC to configure the proposed solution targeting the complexity close to existing NNVC tool, but not exceeding it:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- the number of channels (both input and output) in neural network modules which are modified must be keep multiple of 16,

- Number of Parameters EE1 test  Number of Parameters NNVC (if possible).

According to the EE description only tests with results provided to quantized (int 16) model are considered for adoption to NNVC, float point model results are provided just for information.

Inference cross-check is required for all EE1 tests. Candidates for adoption to NNVC are required to undergo training cross-check, training scripts (including model quantizing algorithm) must be made available, implementation must be compatible with SADL.

Exact parameters settings were announced by proponents by 2nd AhG11/14 teleconference on March 20 [6].

List of tests in this round of exploration experiment includes:

- EE1-1: LOP in-loop filter

  - EE1-1.1 – Dynamic convolution for LOP7 neural in-loop filtering JVET-AP0066 (KHU, KBS).

  - EE1-1.2 – AHG11: Enhancing LOP7 with Re-Exploited Boundary Strength Guidance withdrawn  (to be confirmed)

- EE1-2: VLOP in-loop filter

  - EE1-2.1 – VLOP with new backbone block based on Spatial-Channel Mixing JVET-AP0201 (KHU, ETRI).

- EE1-3 NN-Inter

  - EE1-3.1 – Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement  JVET-AP0051 (Wuhan Uni)

- EE1-4: Framework for externally coded pictures

  - EE1-4.1 - Multi-layer framework

  - EE1-4.2 - Multi-layer framework with frame level control

  - EE1-4.3 - Single-layer framework with different E2E AI codec

  - EE1-4.4 - Single-layer framework with frame level control

JVET-AP0080 (Huawei, InterDigital), JVET-AP0182, JVET-AP0183, JVET-AP0184, JVET-AP0185 (all Nokia), JVET-AP0232 (Huawei)

- EE1-5: Operational bit-exact reproducibility

  - EE1-5.1 –  [AHG11] Stable float convolution for neural network inference  JVET-AP0218  (Nokia)

EE1-1: LOP in-loop filter

Anchor is NNVC-16, LOP7 filter and NN-Intra enabled.

Comparison point NNVC LOP7: kMAC/pxl = 16.5,  NumParameters =0.24 M

  - EE1-1.1 – Dynamic convolution for LOP7 neural in-loop filtering JVET-AP0066 (KHU, KBS).

    - EE1-1.1.1 – SADL int16 model generation for the proposed dynamic convolution in LOP

    - EE1-1.1.2 – kMAC/pxl reduction of dynamic convolution (make it lower than LOP7)

kMAC/pxl = 16.59, NumParameters = 0.84 M

    - EE1-1.1.3 – Parameter reduction of dynamic convolution in LOP7

kMAC/pxl = 16.52, NumParameters = 0.83 M

    - EE1-1.1.4 – Optimal combination of kMAC/pxl and parameter reduction for dynamic convolution with coding-efficiency trade-off

In this test fixed-kernel static convolution layers in the LOP7 in-loop filter are replaced with dynamic convolution. Unlike static convolution, which uses a single fixed kernel for all inputs, dynamic convolution generates a per-sample effective kernel as a weighted combination of multiple expert kernels, where the weights are derived from the input feature via a lightweight router as shown in Figure 1.

Changes compared to LOP7 are illustrated on Figure 2: Blue-shaded CONV blocks are replaced with dynamic convolution (DynConv); the yellow-shaded CONV block indicates the chroma input convolution whose output channel count is d₂ (the parameter varied across sub-tests). Subtests use different number of expert kernels and different chroma input channels (d2).

The decoder run time increase in this test (130% of decoding with LOP7) is associated with the dynamic kernels derivation.

Training time for the model (only stage 3 as agreed in EE1) is 10 days (same training for LOP7 requires 7 days).

Figure 1 Difference between static and dynamic convolutions.

Figure 2 Modifications compared to LOP7

Table 1 EE1-1.1 tests summary

| Test | #ExpertKernels | d2 | kMAC/pixel | # Pars | Mem., MB | RA cfg, BD-Rate (PSNR) |  |  | RunTime |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Y | U | V | Enc | Dec |
| LOP7 | 1 | 6 | 16.54 | 242 | 0.51 | 0.0% | 0.0% | 0.0% | 100% | 100% |
| EE1-1.1.1 | 4 | 6 | 16.59 | 836 | 1.76 | -0.6% | -5.1% | -4.3% | 101% | 131% |
| EE1-1.1.2 | 4 | 4 | 16.52 | 835 | 1.78 | -0.6% | -3.7% | -3.1% | 99% | 130% |
| EE1-1.1.3 | 3 | 6 | 16.58 | 641 | 1.41 | -0.2% | -3.9% | -2.9% | 102% | 133% |
| EE1-1.1.4 | 3 | 4 | 16.51 | 640 | 1.40 | -0.4% | -2.6% | -2.5% | 102% | 131% |

Proponents conclude that test EE1-1.1.2 provide the best complexity-performance trade-off (EE1-1.1.1 and EE1-1.3 are not fulfilling criteria of max number of kMAC/pixel).

Feedback from the cross-checkers: cross-check of inference is done, training is partially finished. There is perfect match in inference, tests after re-training are expected to be ready during meeting (second week), so far maximum deviation between original and re-trained model is within 0.1% BD-rate.

It is expected that the decoding run time can be further reduced by optimizing the implementation

It is noted that the number of parameters is significantly increased, which may have some memory usage aspect. This is not a criterion in the exploration, but might be relevant in standard development. EE1-1.4 has lower number of parameters, but also lower gain.

1.1.3 and 1.1.4 were also cross-checked (only inference).

It was asked and confirmed that learning rate was not changed relative to the one used for LOP7 training.

Revisit: EE1-1.1.2 is candidate for adoption, provided that full cross-check would be finished.

EE1-2: VLOP in-loop filter

EE1-2.1 – VLOP with new backbone block based on Spatial-Channel Mixing JVET-AP0201 (KHU, ETRI).

The key idea of Spatial-Channel mixing is Fusion of depth-wise 3×3 convolution and skip connection during inference. Inspired by inspired by the RepViT, but batch normalization is not used, GeLU was replaced with PReLU.

Complexity:

- Parameter counts: 72.98k (VLOP4: 72.72k)

- MACs/pixel: 5.06k (VLOP4: 5.09k)

Two variants were tested

- a non-re-parameterized model (training = inference), and

- a re-parameterized model (training ≠ inference), where the skip connections are removed only during inference.

Table 2 EE1-2.1 tests summary

| Design | Test cfg | BD-Rate (PSNR) vs NNVC with VLOP4 |  |  | RunTime |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Enc | Dec |
| non-re-parameterized model | All Intra | 0.1% | 0.5% | 0.9% | 100% | 95% |
|  | Random access | 0.1% | 0.2% | 0.3% | 100% | 93% |
|  | Low-delay B | 0.0% | -0.6% | -2.0% | 100% | 90% |
| Re-parameterized model | All Intra | 0.1% | 0.5% | 0.9% | 100% | 94% |
|  | Random access | 0.1% | 0.2% | 0.3% | 100% | 88% |
|  | Low-delay B | 0.0% | -0.5% | -1.9% | 100% | 86% |

Changes relatively to VLOP4 (depicted on Figure 3) include:

- : Replacement VLOP4 blocks by the SCM-based backbone block

- : Channel Reduction in 1×1 convolution via SCM-based backbone block (complexity reduction)

- : Tail convolution optimization for SCM-based backbone block (complexity reduction)

- : Components removed when applying reparameterization (complexity reduction)

Figure 3 Modifications compared to VLOP4

Feedback from the cross-checkers: cross-check of inference and training is on-going. There is perfect match in inference, tests after re-training can be ready during meeting.

Figure 4 Re-parametrized model: training (left) and inference (right).

It was commented that the benefit of the channel spatial mixing is not obvious from the results – the benefit in complexity/runtime reduction rather comes from other changes in the backbone blocks.

It was commented that higher loss occurs for campfire, by tendency all UHD sequences have losses, while some lower resolution sequences have more gains.

Not obvious that the complexity advantage is significant enough to justify the losses in compression, in particular for the higher resolution classes.

EE1-3 NN-Inter

EE1-3.1 – Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement  JVET-AP0051 (Wuhan Univ.)

Several variants for very-small DRFG network have been studied. Difference between EE1-3.1.1 and EE1-3.1.2 is the number of channels is made multiple of 16, so decoder speed-up is observed.

In test EE1-3.1.3 the DRFG tool (same design as EE1-3.1.2) alone (w/o LOP7 and NN-Intra) was tested vs NNVC configured as VTM.

Feedback from the cross-checkers: no information about cross-check at the moment report was created

Figure 5 Very small DRFG

Table 3 EE1-3.1 tests summary (RA cfg)

| RA | 16? | kMAC/pixel | Pars,K | BD-Rate (PSNR) |  |  | BD-Rate (MS-SSIM) |  |  | RunTime |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Y | U | V | Y | U | V | Enc | Dec |
| DRFG | Y | 62.0 | 3600 | -1.4% | 0.5% | -1.5% | -2.1% | -1.3% | -2.0% | 112 | 1212% |
| EE1-3.1.1 | N | 8.8 | 243 | -0.4% | -0.3% | -0.6% | -0.8% | -0.7% | -0.8% | 115% | 450% |
| EE1-3.1.2 | Y | 9.7 | 257 | -0.5% | -0.5% | -0.5% | -0.8% | -0.8% | -0.9% | 112% | 372% |
| EE1-3.1.3* | Y | 9.7 | 257 | -0.9% | -0.9% | -1.1% | -1.2% | -1.1% | -1.2% | 103% | 4379% |

Standalone gain (without other NNVC tools) is higher according to EE1-3.1.3

It was commented that the extra reference picture requires more memory. This may not be too relevant for the exploration of the technology.

EE1-3.1.2 would be the most preferred solution, according to proponents.

It was commented that the runtime numbers found by cross-checkers would be lower.

From the numbers in the table above, decoder runtime of NNVC default would increase 3.7x. This might also impact assessment of other tool modifications.

3.1.2 is candidate for adoption, revisit after completion of cross-check.

EE1-4: Framework for externally coded pictures

Four different configurations for externally E2E AI coded picture have been tested:

  - EE1-4.1 - 'Multi-layer framework (I-frame is replaced by E2E AI picture + residual + in-loop filters + multi-layer HLS, unconditionally)

  - EE1-4.2 - Multi-layer framework with frame level control (I-frame is replaced by E2E AI picture + residual + in-loop filters + multi-layer HLS, based on encoder choice)

  - EE1-4.3 - Single-layer framework with different E2E AI codec (I-frame is replaced by E2E AI picture unconditionally)

  - EE1-4.4 - Single-layer framework with frame level control (I-frame is replaced by E2E AI picture, based on encoder choice)

Experiments were performed with different E2E AI image codec. Test results for DCVC-RT as E2E AI image codec are summarized in Table 4.  In Random access cfg test the best BD-rate performance (1.7% Luma gain) was demonstrated by multi-layer framework with frame level RDO decision for inclusion or not E2E AI coded picture into the reference picture list. Frame level RDO increases CPU encoder run time by 6…8% (in average) but reduces CPU decoder run time roughly by factor 2 since E2E AI decoding is not used only in 60% cases.  Frame level RDO improves BD-rate performance by 0.1% in presence of residual (multi-layer framework), and by 0.4% in absence of residual (single layer framework).

For single layer framework encoding run time is 4..6% faster than for multilayer frame work (no encoder search at enhancement layer). Also, there is no high-level-syntax overhead in single layer framework, so in all-intra test configuration single layer framework shows better performance compared to the multi-layer. But in random access cfg  (more relevant to video coding) residuals and in-loop filters of enhancement layer improve BD-rate performance by 1.4..1.7%.

Table 4 Random access test results for EE1-4 test with DCVC-RT as E2E AI image codec.

| Residual + filters + HLS | Always use E2E AI coded picture |  |  |  |  | Conditionally (RDO) use E2E AI coded picture |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate (PSNR) |  |  | Run Time (CPU) |  | BD-Rate (PSNR) |  |  | Run Time (CPU) |  |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| Yes | -1.6% | -14.3% | -11.1% | 96% | 1111% | -1.7% | -13.7% | -10.2% | 102% | 527% |
| No | 0.1% | -12.8% | -9.0% | 90% | 1088% | -0.3% | -13.0% | -9.3% | 98% | 529% |

Table 5 Random access test results for EE1-4 test with NLIC as E2E AI image codec.

| Residual + filters + HLS | Always use E2E AI coded picture |  |  |  | Conditionally (RDO) use E2E AI coded picture |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate (PSNR) |  |  | Run Time (CPU) | BD-Rate (PSNR) |  |  | Run Time (CPU) |
|  | Y | U | V | Dec | Y | U | V | Dec |
| Yes | -0.3% | -4.3% | -2.7% | 2435% | -0.5% | -4.1% | -2.4% | 863% |
| No | 0.5% | -2.0% | 0.0% | 2443% |  |  |  |  |

Similar tendency is observed in NLIC (E2E AI image codec by Nokia).  Partial test results for NLIC as E2E AI image codec are summarized in Table 5. Full results are not available by the time of this report preparation. Partial data show slightly lower BD-rate gain and roughly twice higher CPU decoding run time compared to DCVC-RT.

None of the E2E AI image codec in this test has full SADL implementation, but for some E2E AI image codec SADL implementation and quantizing to int 16 is ongoing.

Participants of this EE1 category cross-checked inference of each other. More details can be found in JVET-AP0080 (Huawei, InterDigital), JVET-AP0182, JVET-AP0183, JVET-AP0184, JVET-AP0185 (all Nokia), JVET-AP0232 (Huawei).

Statistic for use of E2E AI image codec in multi-layer framework, depending on QP and resolution is reported in EE1-related contribution JVET-AP0144.

Conditional encoding of AI intra in table 4 shows benefit (this is not currently implemented in NNVC16 SW)

Results above are only for RA, corresponding complete tables for DCVC-RT are shown below:

It is asserted from the row “no” that single layer would not give benefit at least in combination with DCVC-RT. Corresponding complete single-layer results for NLIC are not available yet.

From the current results, it can be concluded that implementing frame-level switching in the multi-layer framework gives benefit in improving compression, and reducing decoding run time. This was tested and cross-checked with both AI intra codecs.

Decision(SW): Adopt the RDO based multi-layer frame-level switching used in JVET-AP0080 and JVET-AP0183.

Further investigation in EE appears necessary on identifying potential benefits of single-layer approach.

EE1-5: Operational bit-exact reproducibility

  - EE1-5.1 –  [AHG11] Stable float convolution for neural network inference  JVET-AP0218  (Nokia)

This test uses the mechanism known as "stable float" for neural network-based video codec inference implemented floating-point data. This technique was claimed to ensure bit-exact reproducibility by converting operands involved in convolution operations to integers, with the integer values represented as floating-point numbers.  Implementation requires multi-pass (at least double pass) over latent tensor: 1) compute in and max of input data 2) re-scale and round 3) perform convolution 4) apply inverse scaling 5) add bias.

Compared to float point SADL 16 bit integer implementation for LOP7 (default in NNVC) there is minor BD-rate performance deviation in convolutions are replaced by stable convolutions. Roughly 2 increase of CPU decoder run-time, but almost twice faster decoding using GPU was demonstrated.

Table 6 Summary of EE1-5 test results (stable convolutions in NNLF) compared to int 16 SADL (default NNVC).

| Test cfg | BD-rate (PSNR) |  |  | Runtime |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc CPU | Dec CPU | Dec GPU |
| RA | 0.0% | -0.1% | -0.4% | 102% | 222% | 55% |
| LDB | 0.0% | 0.0% | -1.4% | 102% | 248% | 68% |
| All Intra | -0.0% | -0.1% | 0.1% | 96% | 180% | 69% |

Bit-exact reproducibility is ensured for ‘traditional video’ coding tools by design (typically assuming no more than 16 bits precision for multipliers and no more than 32 bits for register).  The comparison of stable convolution and other well-known video coding tools (like in-loop filters and transforms) is shown in the Table 7.

Table 7 Stable convolution in comparison with traditional video coding tools.

| Aspect | Other video coding tools | Stable convolutions |
| --- | --- | --- |
| De-scaling factor value | Power of 2 | Any |
| De-scaling factor derivation method | Off-line trained, ‘written in a specification’ | Content adaptive, requires additional pass |
| Reliable bits in a register | 32 bits | 24 bits |
| Clipping | Upper and low bound are clipped | Upper bound ensured by on-a-fly de-scaling parameters derivation, low bound by rounding |
| Implementation ‘input 1* input 2 + register’ | ‘int16*int16+int32 (32)’ | ‘fp32*fp32+fp32‘ Specific float point implementation |

Feedback from the cross-checkers:

- Match between CPU and GPU encoding / decoding for NNVC CTC was reported by one cross-checker (JVET-AP0278).

- Bit-exactness is only guaranteed with a full control of the environment and implementation of the inferring backend (JVET-AP0274).

- For LOP7 model parameters and intermediate buffer (between convolution) are both 16 bits. For convolutions with 33 kernel and just 16 channel the worst-case register accumulation is 39 bits.

- Based on specification float point format used in this implementation bit-exact operations are ensured if the overall dynamic range of accumulator is within 23 bits. Proponent confirmed that in the worst-case stable convolution would be equivalent to quantizing the model parameters and intermediate buffer to 7 bits (6 bits magnitude, one sign).

It is asserted that more investigation is necessary to resolve the problem. Integer implementation is stable, but not typically supported by GPUs so far, and FP is implemented differently on different GPUs. To be further investigated in AhG11/14 (clarify in mandates), based on the software that is available in the EE branch.

JVET-AP0048 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

### EE1 contributions: Neural network-based video coding (10)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AP0023. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AP0051 EE1-3.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement [T. Shu, X. Chen, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.)]

JVET-AP0272 Crosscheck of JVET-AP0051 (EE1-3.1: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) N. Bhaskar (Huawei) [late]

JVET-AP0066 EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

JVET-AP0189 Crosscheck of JVET-AP0066 (EE1-1.1: Dynamic convolution for LOP7 neural in-loop filtering) [N. J. Gadgil, S. Mishra, R. N. Gadde, W. I. Choi, K. P. Choi (Samsung)] [late]

JVET-AP0255 Crosscheck of JVET-AP0066 (EE1-1.1 and EE1-1.2: Dynamic convolution for LOP7 neural in-loop filtering) [Y. Li (Qualcomm)] [late]

JVET-AP0281 Crosscheck of JVET-AP0066 (EE1-1.1.1 and EE1-1.1.2: Dynamic convolution for LOP7 neural in-loop filtering) [V. Khamidullin, S. Ikonin, E. Alshina (Huawei)] [late]

JVET-AP0080 EE1-4: DCVC-RT as a Learned End-to-End Intra Frame Codec [M. Aderdor, T. Solovyev, E. Alshina (Huawei), F. Urban, F. Galpin, E. François (InterDigital)]

JVET-AP0182 EE1-4.1: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0183 EE1-4.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0184 EE1-4.3: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [A. Hallapuro, N. Zou, A. B. Koyuncu, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0185 EE1-4.4: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [A. Hallapuro, N. Zou, A. B. Koyuncu, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AP0201 EE1-2.1: VLOP4 with new backbone block based on Spatial-Channel Mixing (SCM) [H. Cho, S. Bahk, J. Kim, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)]

JVET-AP0277 Crosscheck of JVET-AN0201 (EE1-2.1: VLOP4 with new backbone block based on Spatial-Channel Mixing (SCM)) [J. Chi, X. Wang, J. Liu, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang, J. Ca (Transsion)] [late]

JVET-AP0218 EE1-5: operational bit-exact reproducibility [H. Zhang, N. Le, F. Cricri, N Zou, A. B. Koyuncu, L. Murn (Nokia)]

JVET-AP0278 Crosscheck of JVET-AP0218: “EE1-5: operational bit-exact reproducibility” [J. Ström, N. Stegmaier (Ericsson)] [late]

JVET-AP0232 EE1-4.1.3 and EE1-4.2.3: JPEG-AI as a Learned End-to-End Intra Frame Codec [A. Karabutov, P. Jia, E. Alshina (Huawei)]

### EE1 related and beyond-EE contributions: Neural network-based video coding (10)

Contributions in this area were discussed during 1430–1815 on Saturday 25 April 2026 (chaired by JRO).

JVET-AP0049 [AHG11] On NNVC training sets [E. Alshina, N. Bhaskar, V. Khamidullin (Huawei), F. Galpin, T. Dumas (InterDigital), J. N. Shingala (Ittiam)]

This document clarifies the situation regarding the training sets available for JVET and specifies which sequences were used for each NNVC tools training.

Training sets, their location and permission to be used for standardization

| Data set | can use? | Location |
| --- | --- | --- |
| BVI-DVC | yes | https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn |
| TVD | yes | https://multimedia.tencent.com/resources/tvd |
| UGC | not clear | https://media.withyoutube.com/ |
| BVI-AOM | yes | http://download.opencontent.netflix.com.s3.amazonaws.com/index.html?prefix=bvi_aom_dataset/ |
| vimeo90K-triplet | not clear | http://toflow.csail.mit.edu/index.html#triplet |
| vimeo90K-septuplet | not clear | http://toflow.csail.mit.edu/index.html#septuplet |
| RWTH | yes | https://vqa.lfb.rwth-aachen.de |
| DIV2K_train | Research purpose only | http://data.vision.ee.ethz.ch/cvl/DIV2K/DIV2K_train_HR.zip |
| DIV2K_valid |  | http://data.vision.ee.ethz.ch/cvl/DIV2K/DIV2K_valid_HR.zip |
| JVET-AH0298 | yes | https://vqa.lfb.rwth-aachen.de/index.php/apps/files/files/550079?dir=/jvet/ctc/gaming/G1. |
| JVET-AH0120 | yes | https://vqa.lfb.rwth-aachen.de/index.php/apps/files/files/551240?dir=/jvet/ahg/trainingset/JVET-AH0120 |

DIV2K appears critical, as it is only made available for research purposes.

Suggestions made in the document:

For now training materials are hosted by third parties and there is a risk materials will be lost or modified. For the next generation video coding standard development or even before CfP it would be good to copy all materials JVET has permission to be used in standardization process to https://vqa.lfb.rwth-aachen.de.

In order to simplify re-training only stage 3 of NN-based loop filter it is suggested to make stage 3 materials available on to https://vqa.lfb.rwth-aachen.de. Currently HOP and LOP use different set of training materials for stage 3 (LOP and VLOP use the same). It is suggested to unify and use the only one (from previous study in AhG11 difference would be small). Streams, reconstructions, patches – all materials needed for stage 3 to which simplify data loading will take about 4 TB.

For AhG11 and EE1 proposals it must be very clearly mentioned which data set / sub-set was used for training and if training of NN-filter was stage 3 only or filter was re-trained from the scratch.

It is suggested to up-date the NNVC training sets, includes two missed BVI-DVC sequences, 66 unused before TVD sequences, 9 sequences proposed in  JVET-AH0120 and 239 sequences from BVI-AOM data set. DIV2K is available for research purposes only. JVET might consider excluding it from NNVC training set. Training of DRF could benefit if vimeo90K-septuplet will be used additionally to vimeo90K-triplet.

It was commented that copying the material and making it available on the JVET server would not be allowed for most of the material. Storing processed material (e.g. for retraining stages of loop filters) should be possible.

No immediate action possible at this moment, but the aspects of availability of training material usable for standard development will have high impact on investigations and decisions after CfP.

The information contained in the table should be included in the CTC document where the training material and procedures are described.

JVET-AP0052 AHG11: Improved H-DRF with Weighted Fusion and Optimized YUV Processing [X. Chen, J. Zhang, Z. Chen (Wuhan Univ.)]

This contribution builds on H-DRF (DRF at high operational point) from NNVC-16.0 by introducing a HardSigmoid-based weighted fusion layer, performing optical-flow computation directly in the YUV domain, optimizing quantization for model conversion, and replacing bilinear down/up sampling with Lanczos for YUV420/YUV444 conversion.

The coding gains of the proposed method are reported as follows:

1)	Int16, SADL implementation, RA, compared to NNVC-15.0 anchor (NNIntra+LOP6):

Overall: -2.76% (Y), -2.98% (U), -3.09% (V) EncT: 142% DecT: 2420%

2)	Int16, SADL implementation, RA, compared to NNVC-15.0 (H-DRF from NNVC-16.0 + NNIntra + LOP6):

Overall: -0.28% (Y), -1.79% (U), -3.33% (V) EncT: 100% DecT: 101%

It was reported that complexity is practically identical to current H-DRF in NNVC SW, but gives gain (see 2) above).

It was asked if there would be a chanc to avoid the conversion 4:2:0/4:4:4 and back? This might be beneficial in terms of complexity, and improve performance.

Investigate in EE. A similar approach should also be investigated for the lower-complexity DRF versions.

JVET-AP0273 Crosscheck of JVET-AP0052 (AHG11: Improved H-DRF with Weighted Fusion and Optimized YUV Processing) N. Bhaskar (Huawei) [late]

JVET-AP0063 AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement with motion compensation [P. Bordes, F. Galpin, F. LoBianco, M. Paquiry (InterDigital)]

This contribution reports results of NN-based inter prediction enhancement using Deep Reference Frame Generation (HDRF) where the input patches are motion compensated. The results against the NNVC 16.1 LOP7 anchor are summarized as follows:

Patches 256x256 with borders=8:

RA: {–2.97%, –2.23%, –1.33%}, EncT: 147%, DecT: 4235%

Patches 256x256 with borders=32:

RA: {–3.09%, –2.32%, –1.39%}, EncT: 178%, DecT: 6959%

Gain compared to NNVC anchror+H-DRF is approx. 0.4% for border=8, almost the same gain also possible for L-DRF. For border=32, gain is 0.5%, but encoding and decoding time also increases.

At picture boundary, padding is used when MV points outside of the picture.

Investigate in EE. The aspect of increasing the border size should be investigated separately, also combination with JVET-AP0052 should be investigated.

JVET-AP0140 EE1-related: QP to QI mapping for intra-frame coding with DCVC [V. Rizzello, J. Ström, P. Wennersten, M. Damghanian, D. Liu (Ericsson)]

This contribution presents the effects of using a different quantization parameter (QP) to quantization index (QI) mapping for I-frame coding with DCVC-RT. The contribution JVET-AO0194 based on DCVC-FM and DCVC-RT reported the gains in terms of BD-rate of neural network-based external I-frame coding over NNVC 15 configured as VTM 23.13. This contribution reports the BD-rate of DCVC-RT using a different QP to QI mapping compared to the one considered in JVET-AO0194. The CTC results with DCVC-RT as external I-frame coder and different configurations are reported to be the following:

-	no chroma scaling:

AI:	 	-0.08% (Y)		0.78% (U)		0.55% (V)

RA: 	-0.31% (Y)		-1.24% (U)		-1.01% (V)

-	no chroma scaling, chroma QP offset equal to 2:

AI: 		-0.05% (Y)		0.48% (U)		0.41% (V)

RA: 	-0.27% (Y)		-1.13% (U)		-0.95% (V)

The QP to QI mapping is currently implemented in an external script for the case of DCVC-RT (as encoding parameter), not part of the interface. It was suggested that for experimentation with DCVC-RT this modification should be used to get best possible performance.

JVET-AP0144 EE1-related: Statistics on multilayer frame level RDO using DCVC-RT [M. Aderdor, T. Solovyev, E. Alshina (Huawei), F. Urban, F. Galpin, E. François (InterDigital)]

This contribution includes information about the multilayer framework used for EE1-4.2, as well as statistics on encoder choices when deciding to insert a DCVC-RT I-frame using frame level RDO.

The statistics include overall, per class, per QP and per sequence statistics. Some biases can be observed. It should be noted that residual coding and in-loop filtering are not considered for the E2E I-frames during RDO. This additional refinement could influence encoder choices on which frame to insert as an I-frame.

It is proposed to make the functionality of frame level switching from EE1-4.2 available on NNVC 17.

Software was already included from EE results.

The QP dependent results seem to indicate that this AI based codec was optimized for a certain QP range.

JVET-AP0178 AHG11: Neural Network Super Resolution as Postfilter with Content Adaptation [K. Pham-Dinh, F. Cricri, M. Santamaria, R. Yang, M. M. Hannuksela (Nokia)]

This contribution reports experiments on a neural network–based super-resolution (NNSR) method used as a post-filter. The super-resolution model is reported to use input tensors compatible with the neural-network post-filter (NNPF) SEI messages, to perform a 2× upsampling operation within the network, and to operate across multiple QPs. The contribution also reports experiments on content adaptation to overfit the model, where the weight updates are coded using the MPEG NNR standard and the resulting bitrate overhead is included in the BD-rate computation. The BD-rates of the proposed content-adaptive NNSR, when using NNVC 16 default as the anchor, are reported to be as follows:

RA: -0.64% 3.56%	-1.43%	EncT 81%	DecT 53%

AI:	-1.34% 7.03%	-1.43%	EncT 60%	DecT 61%

Results are sequence/class dependent, here for RA:

It was commented that the reduction in runtime is mainly due to adaptive resolution coding, which might also be used by enabling RPR in the anchor (as well as gain in the high-resolution classes). The gain of RPR is also homogeneous, giving also gain in class A2.

More adaptivity in general usage of LR coding (such as in adaptive RPR) might be better, avoiding losses in A2.

Which downsampling filter was used? Bilinear.

How would it be performing compared to a fixed model? Comparison vs. NNVC+NN-based SR would be desirable. When comparing, using a similar approach of adaptation (in terms of enabling LR coding) would be relevant.

Further study on these aspects – not mature yet for EE.

Generally interesting, as also NNSR in NNVC is plain post processing and could hypothetically be implemented via SEI message (as a fixed post processing).

JVET-AP0190 [AHG11] Energy Efficiency of Neural Accelerators [S. Cizel, R. Mullakhmetov, F. Galpin (InterDigital)]

This informative contribution investigates whether dedicated neural-network accelerators provide a more energy-efficient execution path than CPU-based inference. By evaluating representative model configurations across multiple execution modes, it shows that accelerator-backed execution can offer clear efficiency advantages when workloads are effectively mapped to the target architecture. It further discusses the roles of dispatch behavior, workload shape, and sustained utilization in determining the observed gains.

It is agreed that this is rather important to have a realistic implementability. Also in the past, parameters like typical cache size, external memory bandwidth etc. of CPUs have been used, and already in EE2, parameters such as number of channels, kMAC, number of parameters are used. Necessity of parameter reload (switching models) could also be important. The data paths of GPU, NPU etc. architectures are somewhat different from CPUs.

It was requested to investigate whether relevant parameters are missing, also w.r.t. the complexity reporting template JVET-AN2040. Further consideration of these aspects is also relevant in the context of AHGs 16 and 7 (where possibly a BoG would be installed).

JVET-AP0219 [AHG11] Additional Information on Stable Float Method for Neural Network Inference on GPU devices [H. Zhang, F. Cricri, N. Le, N. Zou, A. B. Koyunku, L. Murn (Nokia)]

A mechanism known as "stable float" was introduced in JVET-AO0178 where it was reported to facilitate inference on GPU devices for neural network-based coding tools using a floating-point data format. The stable float approach is reported to enable cross-device reproducibility across various computing environments by using integer components within convolution operations while utilizing floating-point numbers for other operations. It is reported that several concerns regarding the stable float method and IEEE 754 standard were raised during the JVET-AO meeting review of the proposal. This document is reported to address these concerns and to provide the proof of the overflow-free property of the proposed method.

V2: editorial changes and more information on IEEE 754 standards on rounding operations.

Just for information - no need for presentation according to proponents

JVET-AP0274 Cross-check of JVET-AP0219 ([AHG11] Additional Information on Stable Float Method for Neural Network Inference on GPU devices) A. Karabutov, E. Alshina (Huawei) [late]

JVET-AP0228 [AHG11] Response to Call for training materials [E. Alshina, J. Ascenso, T. Ebrahimi]

In response to the Call for training materials this document proposes to consider JPEG AI training data set for neural network-based video coding (NNVC) tool development.

Training data set of JPEG AI consists of camera captured and synthesized images.

The JPEG AI dataset has the following characteristics:

• Format – PNG images (sRGB color space);

• Spatial resolution – from 256×256 to 8K (8 bit);

• Training/validation/test dataset:

camera captured images: 5264/350/50 image.

synthetic images: 3029/202/36 images.

The table in JVET-AP0049 also contains this data set.

It was suggested to clarify whether the material could be made available in the JVET content server.

Experts were requested to investigate possible existence of more materials that could be used for training.

JVET-AP0234 AHG11: Dynamic Quantization and Hardware Optimization for NNIP [A. Akhtar, S. Esenlik, Y. Matsuba, M. Karczewic (Qualcomm)]

In NNVC-16.1 and ECM-19.1 neural network based intra prediction tool is implemented to utilize 16-bit activations, 16-bit weights and 32-bit accumulators. The current design presents two major issues, firstly the accumulators can reportedly overflow which would cause indeterministic behavior across different processing units. Secondly, due to the bit-depth of the multiplications, hardware area required to implement NNIntra is estimated to be very high. The proposed solution addresses both of the problems by reducing the activation bitwidth to 10 bits, and weight bitwidth to 8 bits. Furthermore, reportedly the accumulator overflow is proven to be mathematically avoided.

When implemented in ECM‑19.1, the proposed design results in BD‑rate changes of +0.01% (Y), +0.04% (U), and +0.03% (V) for Random Access, and +0.01% (Y), +0.02% (U), and +0.01% (V) under the All Intra configuration. When implemented in NNVC‑16.1, the BD‑rate changes are +0.01% (Y), −0.12% (U), and −0.10% (V) for Random Access, and +0.02% (Y), +0.04% (U), and +0.01% (V) for All Intra.

It was commented that one more cycle would be needed, which might also be critical.

Several experts expressed interest.

Investigate in EE.

JVET-AP0282 Cross-check of JVET-AP0234 (AHG11: dynamic quantization and hardware optimization for NNIP) [T. Dumas, F. Galpin (InterDigital)] [late]

### SADL and NNVC implementation, CTC (4)

Contributions in this area were discussed during 1815–1845 on Saturday 25 April 2026 (chaired by JRO).

JVET-AP0053 AhG14: SIMD Improvements of Operators in SADL Library [X. Chen, J. Zhang, Z. Chen (Wuhan Univ.)]

This contribution presents the SIMD improvements of operators in the Small AdHoc Deep Learning (SADL) library from Wuhan University. The optimized layers include Conv2D, Conv2DTranspose, and GridSample. The implementation utilizes the AVX2 instruction set. Experimental results show that the proposed implementation reduces encoding and decoding time.

According to proponents, the modification gives bit-exact results:

Decision(SW): Adopt JVET-AP0053 (all three improvements)

JVET-AP0072 [AHG14] NNLF compact dataset [A. Kherchouche, F. Galpin (InterDigital)]

This contribution proposes a method to reduce by half the size of the dataset used for Neural Network Loop Filters (NNLF) training while preserving the same informational content. The approach relies on identifying and removing redundancy within the dataset without altering its statistical properties. As a result, the reduced dataset is expected to provide equivalent training information. This enables lower storage and handling complexity for NNLF development.

Decision(SW): Integrate the approach from JVET-AP0072 into the training script

It may be used in the next EE (if that includes any re-training of NNLF)

JVET-AP0211 AHG14: New memory management policy in SADL [C. Hallam, F. Galpin, J. Pohl, S. Cizel (InterDigital)]

This contribution presents an alternative memory management for Neural Network inference in SADL. The new memory management allows to decrease significantly the memory usage in some cases. It is proposed as an alternative memory policy for models with large intermediate tensors or long chain of layers. The new memory policy is deactivated by default, and the behavior is then similar to the legacy memory manager. While the legacy memory management is usually faster for small memory footprint models, the proposed new policy can reduce significantly the memory footprint for large models and even speed-up the inference in that case.

The method is asserted to have benefit for large models in particular.

No noticeable impact on runtime.

Decision(SW): Adopt JVET-AP0211 as an alternative method for memory allocation (up to the choice of users to select this method or the currently existing method)It was pointed out that it might be relevant to report in the context of experiments which of the memory allocation methods was used, or ask proponents for that.

JVET-AP0212 AHG14: SADL Update [F. Galpin (InterDigital)]

Modifications in version v16:

  - Improvements

  - MR 197 JVET-AO0100 SIMD for slice

  - MR 196 JVET-AO0100 SIMD biasadd

  - MR 195 JVET-AO0100 SIMD concat

  - MR 194 JVET-AO0100 better SIMD mul

  - MR 193 JVET-AO0100 better SIMD add

  - MR210/MR201: JVET-AO0073 speed-up

  - New

  - MR 190 JVET-AN0222 New Depth2Space layer

  - MR 191 JVET-AN0222 New Abs layer

  - MR 189 JVET-AN0222 New Clip layer

  - MR 192 JVET-AN0222 New BitShift layer

  - MR 177 JVET-AN0197 New Gather layer

  - MR 176 JVET-AN0197 New GatherElements layer

  - MR 212: new SADL models visualizer

  - Fixes

  - MR214: Fix issues with new speed. Was only affecting NNSR models

  - MR211: fix issues for sparse Tensor. Was only affecting float sparse Tensor.

- New proposed in version v17

Several MR in the queue:

- MR217: SIMD for gridsample

- MR216: SIMD for Conv2DTranspose

- MR215: SIMD for Conv 3x3 striude 2

- MR213: fix Clang build

- MR206/207/208/209: SIMD speed-up – alternative postponed

- MR218: new memory manager

Several contributions improving SADL:

| JVET-AP0053 | AhG14: SIMD Improvements of Operators in SADL Library | X. Chen, J. Zhang, Z. Chen (Wuhan Univ.) |
| --- | --- | --- |
| JVET-AP0211 | AHG14: New memory management policy in SADL | C. Hallam, F. Galpin, J. Pohl, S. Cizel (Interdigital) |

No need for presentation – information only.

## AHG6/AHG12: Enhanced compression beyond VVC capability (17)

### Summary and BoG reports

Contributions in this area were discussed during 1800–1945 on Friday 24 April 2026 (chaired by JRO).

JVET-AP0024 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, D. Buğdayci Sansli, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1a | Add more chroma DIMD modes into the MPM list | Z. Li (ZTE) | JVET-AP0275 Z. Xie (OPPO) |
| 1.1b | Modify the reordering strategy | Z. Li (ZTE) | JVET-AP0275 Z. Xie (OPPO) |
| 1.1c | Test 1.1a + Test 1.1b | Z. Li (ZTE) | JVET-AP0265 Y.Liu (Transsion) |
| 1.2a | EIP with modified H-filter and V-filter | Y. Liu (Transsion) | JVET-AP0239 H. Qin (TCL) JVET-AP0249 W. Niu (ZTE) |
| 1.2b | Test 1.2a + modified S-filter | Y. Liu (Transsion) | JVET-AP0239 H. Qin (TCL) JVET-AP0249 W. Niu (ZTE) |
| 1.3 | CCCM clipping operations with clipping range adjustment | P. Onno (Canon) L.-C. Xu (SYSU) | JVET-AP0240 Y. Ahn (Qualcomm) |
| 2 Inter prediction |  |  |  |
| 2.1a | Modified GPM partition mode with refined offset direction derivation | Y. Kidani (KDDI) H. Zhang (OPPO) | JVET-AP0251 H.-J. Jhu (Kwai) |
| 2.1b | Modified GPM partition mode with enabling block-shaped adaptive angle selection in LDB | Y. Kidani (KDDI) | JVET-AP0251 H.-J. Jhu (Kwai) |
| 2.1c | Test 2.1a + Test 2.1b | Y. Kidani (KDDI) H. Zhang (OPPO) | JVET-AP0251 H.-J. Jhu (Kwai) |
| 3 Transform and coefficients coding |  |  |  |
| 3.1a | Quantization borders optimization for URQ (non-CTC) (encoder only) | M. Le Pendu (InterDigital) | JVET-AP0270 Y. Sun (Huwaei) |
| 3.1b | Unbiasing URQ (non-CTC) | M. Le Pendu (InterDigital) | JVET-AP0247 Y. Yu (OPPO) |
| 3.1c | Unbiasing RDOQ (non-CTC) | M. Le Pendu (InterDigital) | JVET-AP0247 Y. Yu (OPPO) |
| 3.2a | QP adaptive dead-zone size for DQ | M. Balcilar (Ofinno) | JVET-AP0248 Y. Yu (OPPO) |
| 3.2b | Dead-zone size optimization for DQ (encoder only) | M. Balcilar (Ofinno) | JVET-AP0271 C. Zhou (Vivo) |
| 3.2c | QP adaptive dead-zone size plus QCS offset for DQ | M. Balcilar (Ofinno) | JVET-AP0276 M. Coban (Qualcomm) |
| 3.2d | Using QCS offset as a dead-zone offset for DQ | M. Balcilar (Ofinno) | JVET-AP0276 M. Coban (Qualcomm) |
| 3.2e | QP adaptive dead-zone size for RDOQ (non-CTC) | M. Balcilar (Ofinno) | JVET-AP0241 K. Y. Kim (WILUS) |
| 3.2f | Dead-zone size optimization for RDOQ (non-CTC) | M. Balcilar (Ofinno) | JVET-AP0271 C. Zhou (Vivo) |
| 3.2g | QP adaptive dead-zone size plus QCS offset for RDOQ (non-CTC) | M. Balcilar (Ofinno) | JVET-AP0250 Z. Zhang (Alibaba) |
| 3.2h | Using QCS offset as a dead-zone offset for RDOQ (non-CTC) | M. Balcilar (Ofinno) | JVET-AP0250 Z. Zhang (Alibaba) |
| 3.3a | Intra LFNST index prediction | Y. Wang (Tencent) | JVET-AP0264 C. Hollmann (TCL) JVET-AP0266 K. Naser (InterDigital) |
| 3.3b | Test 3.3a with sign prediction disabled (non-CTC) | Y. Wang (Tencent) | JVET-AP0264 C. Hollmann (TCL) JVET-AP0266 K. Naser (InterDigital) |
| 4 In-loop filtering |  |  |  |
| 4.1 | Simplification for TALF | L. Xu (OPPO) | JVET-AP0254 P. Onno (Canon) |
| 5 Entropy coding |  |  |  |
| 5.1a | CABAC contexts retraining | Z. Xiang (Tencent) | JVET-AP0125 P. Nikitin (Xiaomi) |
| 5.1b | Counter-based temporal probability initialization | Z. Xiang (Tencent) | JVET-AP0125 P. Nikitin (Xiaomi) |
| 5.1c | Test 5.1b + Test 5.1a | Z. Xiang (Tencent) | JVET-AP0125 P. Nikitin (Xiaomi) |
| 5.1d | Using quotient C1/CN as initialization probability | Z. Xiang (Tencent) | JVET-AP0125 P. Nikitin (Xiaomi) |

Intra prediction

Test 1.1: Improvement on chroma MPM (JVET-AP0133)

In the Test 1.1a, more chroma DIMD modes are added to the current MPM candidate list after the adjacent chroma modes and before the DBV modes.

In Test 1.1b, the costs of the second and third modes are considered during the pre-sorting process, when the cost of the first mode in the MPM candidate list is lower than that of the second mode, an additional cost comparison between the second mode and the third mode is performed. Only when the cost of the first mode is lower than that of the second mode and the cost of the second mode is lower than that of the third mode, the top 6 or top 7 modes in the MPM candidate list are selected to form the final MPM candidate list; otherwise, it is necessary to further execute the process of reordering all candidates in the MPM candidate list to construct the final MPM candidate list.

Test 1.1a: Add more chroma DIMD modes into the MPM list.

Test 1.1b: Modify the reordering strategy.

Test 1.1c: Test 1.1a + Test 1.1b

Test 1.2: Modification of EIP filter shapes (JVET-AP0145)

Two modifications of EIP filter shapes are tested.

In ECM, EIP filter shapes are shown in the next figure.

Test 1.2a modifies the horizontal and vertical filters: in the horizontal filter, one input sample is replaced by the above sample, and in the vertical filter, one input sample is replaced by the left sample.

Test 1.2b further modifies the square filter on top of Test 1.2a, in the square filter, one input sample is replaced by the nearest spatial input sample in the diagonal prediction order.

The corresponding filter shapes are shown in the figure below.

Test 1.2a: EIP with modified H-filter and V-filter

Test 1.2b: Test 1.2a + modified S-filter

Test 1.3: Clipping operation refinements in CCCM modes (JVET-AP0168)

This test modifies clipping operation in CCCM as follows:

- Apply a margin to the minimum and maximum values of the chroma samples from the reference area to derive the final clipping range for the chroma sample prediction.

- Introduction of a minimum gap between the minimum and maximum clipping values which are obtained from the minimum and maximum values of the chroma reference samples. Typically, a minimum gap corresponding of certain percentage of the maximum possible chroma range is applied.

- Apply the above two adaptations for small chroma PUs having less than 32 samples.

- Apply the above adaptations to all CCCM modes (called GLM, BVG, CFL) defined in the ECM.

- Apply the above minimum gap according to model type and whether the model is converted from CCP candidate (for Decoder-derived CCP and CCP merge mode).

Test 1.3: CCCM clipping operations with clipping range adjustment.

Results AI/RA

Test 1.1x: Some additional complexity by adding modes, no good tradeoff in terms of gain – no action.

Test 1.2x: No significant benefit from EE results. EE related document JVET-AP0143 was presented in this context, but the additional benefit is still too small for taking action.

Test 1.3: Some additional processing necessary to derive the range, but it gives some gain in chroma (in particular for intra, somewhat diverging results for RA). Several experts expressed support.

Decision: Adopt JVET-AP0168 Test 1.3.

Inter prediction

Test 2.1: Modified GPM partition mode (JVET-AP0086)

In ECM, GPM supports 32 angles, where a variable shiftHor defined below specifies the offset direction from the center of the CU.

shiftHor = (angleIdx % 16 = = 8 | | (angleIdx % 16 != 0 && nH >= nW ) ) ? 0 : 1

where nH and nW represent the height and width of the current block, respectively.

In the test, two modifications are evaluated. In the first aspect, the offset direction is derived by identifying whether the partitioning angle is in a proximity of the horizontal or vertical direction, which is done based on a comparison between the block aspect ratio with the aspect ratio of the GPM partition angle, as well as the relative dimensions of the block, when the partitioning angle is close to 45 degrees.

The GPM offset direction is controlled by a variable “shiftHor” as follows:

nearHor = g_angle2mask[angleIdx]==0 || g_angle2mask[angleIdx]==1|| g_angle2mask[angleIdx]==2

nearVer = g_angle2mask[angleIdx]==6 || g_angle2mask[angleIdx]==7||g_angle2mask[angleIdx]==8

blockRatio = blockWidth / blockHeight = nW / nH;

gpmRatio   = angle_w / angle_h;

shiftHor = nearHor ? 1 : (nearVer ? 0 : ((gpmRatio > blockRatio|| (gpmRatio == blockRatio&&nH>=nW)) ? 0 : 1))

In the second aspect, block-shaped adaptive angle selection proposed by JVET-AJ0107 is enabled in the LB configuration.

Test 2.1a: Modified GPM partition mode with refined offset direction derivation.

Test 2.1b: Modified GPM partition mode with enabling block-shaped adaptive angle selection in LDB.

Test 2.1c: Test 2.1a + Test 2.1b.

Reults AI/RA/LB

Cross-checkers confirm results and support adoption of 2.1a. Also some other expert expressed support.

Decision: Adopt JVET-AP0086 Test 2.1a.

Transforms and coefficient coding

Test 3.1: Dead-zone adjustments and unbiasing of scalar quantizers (JVET-AP0065)

In ECM, in uniform quantization (RDOQ off, DQ off), an offset β is added at encoder side, the value of β is inherited from HEVC as β=171/512≈1/3 for Independent Random-Access Point (IRAP) pictures, and β=85/512≈1/6 for the rest of the pictures.

Consequently, the reconstruction points are not in the middle of their quantization intervals, resulting in a bias controlled by β.

In ECM, the quantization center shifting method (QCS) further shifts the reconstruction points in the dequantization process, where the dequantized coefficient t’ is computed from the quantization level q and a lookup table T of QCS offsets.

The tests are targeting the removal of the reconstruction bias.

Two offsets β* (applied in tests 3.1a and 3.1b) are introduced replacing the current β encoder offsets, which are set to 161/512 in AI configuration, 191/512 for IRAP and 120/512 for non-IRAP pictures in RA and LB configurations.

In Test 3.1c, two offsets at encoder are introduced with enabled RDOQ, they are set as 16/512 for AI, 0 for IRAP and 32/512 for non-IRAP pictures in RA and LB configurations.

In Test 3.1a, those new encoder offsets are applied.

In Test 3.1b, a flag is signalled indicating the QCS offset T is replaced with the signaled offset O for all quantization levels. In the implementation, the offset table T is replaced with the signalled offset O as follows T’[0]=0 and T’[i]=O for i>0. This test is performed with disabled RDOQ and DQ.

In Test 3.1c, the signaled offset O is added to the existing QCS offset T, and this test is performed for enabled RDOQ.

Test 3.1a: Modified encoder offsets (encoder only) (RDOQ off, DQ off).

Test 3.1b: Test 3.1a with signalled offset for inverse quantization replacing QCS (RDOQ off, DQ off).

Test 3.1c: Modified encoder offsets with signalled offset for inverse quantization in addition to QCS (RDOQ on, DQ off).

Test 3.2: On the dead-zone in quantization (JVET-AP0059)

In ECM, there is no offset in quantization, but an offset inversely proportional to quantization index is applied in inverse quantization. This offset is kept in a lookup table shown by T in ECM.  In this test, a new QP adaptive quantization offset ρ is introduced. This new offset is a first order linear function of QP as in following equation.

ρ=a×QP+b

The coefficients of this linear function can be different for slice and channel types. The optimal parameters are set as SPS level syntax elements for each encoding mode. This syntax elements are signalled if inverse quantization applies this offset in the test.

In the tests, the encoder quantization offsets and the offset applied for the inverse quantization at decoder are summarized in the following table for the tested configurations.

| Codebase | Quantization Offset | De-quantization Offset | Test Conditions on Quantizers |
| --- | --- | --- | --- |
| ECM-19.1 | 0 / β | T | RDOQ, DQ / URQ |
| Test 3.1a | β* | T | RDOQ off, DQ off |
| Test 3.1b | β* | O | RDOQ off, DQ off |
| Test 3.1c | O | T + O | RDOQ on, DQ off |
| Test 3.2a | ρ | T+ρ | DQ for RRC, RDOQ for TSRC (ctc) |
| Test 3.2b | ρ | T | DQ for RRC, RDOQ for TSRC (ctc) |
| Test 3.2c | T+ρ | T+ρ | DQ for RRC, RDOQ for TSRC (ctc) |
| Test 3.2d | T | T | DQ for RRC, RDOQ for TSRC (ctc) |
| Test 3.2e | ρ | T+ρ | RDOQ for all residual coding |
| Test 3.2f | ρ | T | RDOQ for all residual coding |
| Test 3.2g | T+ρ | T+ρ | RDOQ for all residual coding |
| Test 3.2h | T | T | RDOQ for all residual coding |

Test 3.2a: Modified encoder offset with signalled inverse quantization offset in addition to QCS

Test 3.2b: Modified encoder offset (encoder only)

Test 3.2c: Modified encoder and signalled inverse quantization offsets, both in addition to QCS

Test 3.2d: Using QCS offset at encoder (encoder only)

Test 3.2e: Test 3.2a (RDOQ on, DQ off)

Test 3.2f: Test 3.2b (RDOQ on, DQ off) (encoder only)

Test 3.2e: Test 3.2c (RDOQ on, DQ off)

Test 3.2h: Test 3.2d (RDOQ on, DQ off) (encoder only)

Test 3.3: Intra LfnstIdx prediction (JVET-AP0107)

In the test, the boundary discontinuity used in sign prediction is applied to signal LFNST indices. The dequantized coefficients are tested using all possible inverse transform kernels as shown in the next figure. Then, the boundary continuity cost is calculated for each possible inverse transform kernel, and the inverse transform kernel index with the smallest cost is used as the LFNST index prediction. A flag is signalled whether the LFNST index is equal to the prediction, otherwise the LFNST index remainder is signalled.

This method is not applied if DIMD, TIMD, MIP, IntraTMP, SGPM, or EIP is used for the prediction and is used when transform coefficient sign prediction is not utilized.

Test 3.3a: Intra LFNST index prediction.

Test 3.3b: Test 3.3a with sign prediction disabled (non-CTC).

Results AI/RA/LB

3.1/3.2/3.5: Only 3.2a gives gain (most is -0.1% in RA and -0.13% LB) compared to CTC. Some other tests demonstrate that benefit by signalling inverse quantization offset becomes larger under non-CTC (e.g. disabling DQ).

Some concern is raised whether the degree of interaction between encoder and decoder optimization is sufficiently understood, and would justify the normative change, also considering that the gain is not large. No support by other experts.

Test 3.3: Sign prediction is replaced for cases of regular intra mode in 3.1a. Small gain over CTC is shown in AI in test 3.1a (no gain in RA). This would not justify introduction/switching of an additional tool. Test 3.3b completely replaces sign prediction (which may be some complexity advantage), but does not show gain in CTC. Not relevant enough for taking action.

In-loop filtering

Test 4.1: Simplification on TALF (JVET-AP0087)

In ECM, Temporal adaptive loop filter (TALF) uses the reconstructed pixels in the reference pictures, and the motion-compensation padded samples located in the padded area of the reference. TALF has four uni-filtering modes and two bi-filtering modes. The uni-filtering process is given by the following equation,

o=ci×Krx'+ui,y'+vi-saoLumax,y,b+Krx'-ui,y'-vi-saoLumax,y,b,

and the bi-filtering process is given by the following equation,

o=ci×K(r0x'+ui,y'+vi+r1x'+ui,y'+vi≫1)-saoLumax,y,b+K(r0x'-ui,y'-vi+r1x'-ui,y'-vi≫1)-saoLumax,y,b

In the above equation,

- o is the offset generated by TALF filter and added to the luma ALF output;

- c(i,j) is one of the positions within the filter window;

- r0 represents the reference samples from the reference picture 0

- r1 represents the reference samples from the reference picture 1

- (x,y) is the current filtering position in the current picture;

- (x',y')  is the collocated position or the MV guided position in the reference picture;

- K(d,b) is a clipping function which restricts the value d within the range from -b to b;

- saoLuma represents the output samples after the SAO filtering.

Two filter shapes are employed in TALF.

In the test, the filtering process is modified by replacing the SAO value with the sample value from the collocated position. The uni-filtering process is modified as follows,

o=ci×Krx'+ui,y'+vi-centerVal,b+Krx'-ui,y'-vi-centerVal,b,

where centerVal= rx',y',

and the bi-filtering process is modified as follows,

o=ci×K(r0x'+ui,y'+vi+r1x'+ui,y'+vi≫1)-centerVal,b+K(r0x'-ui,y'-vi+r1x'-ui,y'-vi≫1)-centerVal,b,

where centerVal= (r0x',y'+r1x',y')≫1.

Here, the proposed changes are highlighted in yellow.

With the above modifications, the TALF offset o is generated only from the reconstructed samples from the reference pictures. The dependency on the SAO output samples is removed. In addition, the number of filter coefficients is reduced from 13 to 12.

Results RA/LB:

Small gain, and further simplification of ECM (which has many more complicated elements) is not of high importance at this stage. No action at this moment.

Entropy coding

Test 5.1: Counter-based temporal probability initialization (JVET-AP0082)

In ECM, temporal CABAC is used for the context initialization by using the probability states of the previously coded slice with the same slice type, slice QP, and temporalID, if available.

In the test, counter based method for the context initialization is introduced. Two counters CN and C1 are introduced to each context. The counter CN tracks the total number of bins coded, and the counter C1 tracks the total number of “1” coded. The quotient of the two counters (i.e., C1/CN) is used to adjust initialization probability as follows

ProbabilityS=ProbablityE'+C1E'CNE'/2,

where ProbablityE'is the probability from the temporal CABAC initialization.

The counters are accumulated across slices with the same slice type, slice QP, and temporalID. To prevent overflow, the counters are not accumulated if a certain threshold value is reached.

The probability is adjusted if the total counter CN is greater than a predefined threshold, which is set to 32 for the low delay conditions, and to 48, otherwise.

Test 5.1a: ECM-19.1 CABAC context retraining.

Test 5.1b: Counter-based temporal probability initialization.

Test 5.1c: Test 5.1b + Test 5.1a.

Test 5.1d: Using quotient C1/CN as initialization probability.

Results AI/RA/LB:

It was commented that it is remarkable to further improve temporal probability initialization by a modification. However, some additional memory might be necessary to implement, and it might be questionable if it would be practical in a real implementation. Not of high importance at this stage. No action at this moment.

### EE2 contributions: Enhanced compression beyond VVC capability (9)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AP0024. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AP0059 EE2-3.2: On the Dead-Zone in Quantization [M. Balcilar, M. Blestel, P. Andrivon (Ofinno)]

JVET-AP0241 Crosscheck of JVET-AP0059 (EE2-3.2e: On the Dead-Zone in Quantization) [K. Y. Kim, J.-Y. Kim, J.-H. Son, J.-S. Kwak (WILUS)] [late]

JVET-AP0248 Crosscheck of JVET-AP0059 (EE2-3.2a: On the Dead-Zone in Quantization) [Y. Yu (OPPO)] [late]

JVET-AP0250 Crosscheck of JVET-AP0059 (EE2-3.2g/h: On the Dead-Zone in Quantization) [Z. Zhang (Alibaba)] [late]

JVET-AP0257 Crosscheck of JVET-AP0059 (EE2-3.2e: On the Dead-Zone in Quantization) [T. N. Canh (Dolby)] [late]

JVET-AP0269 Crosscheck report of JVET-AP0059: EE2-3.2e [M. G. Sarwer (Google)] [late]

JVET-AP0271 Crosscheck of JVET-AP0059 (EE2-3.2b/f: On the Dead-Zone in Quantization) C. Zhou (vivo) [late]

JVET-AP0276 Crosscheck of JVET-AP0059 (EE2-3.2c/d: On the Dead-Zone in Quantization) [M. Coban (Qualcomm)] [late]

JVET-AP0065 EE2-3.1a/b/c: Dead-Zone Adjustments and Unbiasing of Scalar Quantizers [M. Le Pendu, F. Le Léannec (InterDigital)]

JVET-AP0247 Crosscheck of JVET-AP0065 (EE2-3.1b/c: Dead-Zone Adjustments and Unbiasing of Scalar Quantizers) [Y. Yu (OPPO)] [late]

JVET-AP0270 Crosscheck of JVET-AP0065 (EE2-3.1a: Dead-Zone Adjustments and Unbiasing of Scalar Quantizers) [Y. Sun (Huawei)] [late]

JVET-AP0082 EE2-5.1: Counter-based Temporal Probability Initialization (CTPI) [Z. Xiang, R. Chernyak, H. Huang, S. Liu (Tencent)]

JVET-AP0125 Crosscheck of JVET-AP0082 (EE2-5.1: Counter-based Temporal Probability Initialization (CTPI)) [P. Nikitin (Xiaomi)]

JVET-AP0086 EE2-2.1a/b/c: Modified GPM partition mode [H. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO), Y. Kidani, H. Kato, T. Chujoh, K. Kawamura (KDDI)]

JVET-AP0238 Crosscheck of JVET-AP0086 (EE2-2.1a/b/c: Modified GPM partition mode) [X. Li (Alibaba)] [late]

JVET-AP0251 Crosscheck of JVET-AP0086 (EE2-2.1a/b/c: Modified GPM partition mode) [H.-J. Jhu (Kwai)] [late]

JVET-AP0267 Crosscheck of JVET-AP0086 Test 2.1a (EE2-2.1a/b/c: Modified GPM partition mode) [Z. Deng (Bytedance)] [late]

JVET-AP0087 EE2-4.1: Simplification on TALF [L. Xu, N. Song, Y. Yu, H. Yu, J. Gan, D. Wang (OPPO)]

JVET-AP0254 Crosscheck of JVET-AP0087 (EE2-4.1: Simplification on TALF) [P. Onno (Canon)] [late] [miss]

JVET-AP0107 EE2-3.3 Intra LfnstIdx Prediction [Y. Wang, R. Chernyak, H. Huang, S. Liu (Tencent)]

JVET-AP0264 Crosscheck report of JVET-AP0107 (EE2-3.3 Intra LfnstIdx Prediction) [C. Hollmann (TCL)] [late]

JVET-AP0266 crosscheck of JVET-AP0107: EE2-3.3 Intra LfnstIdx Prediction [K. Naser (InterDigital)] [late]

JVET-AP0133 EE2-1.1: Improvement on chroma MPM [Z. Li, W. Niu, X. Zeng, M. Jia, Y. Bai (ZTE)]

JVET-AP0265 Crosscheck of JVET-AP0133 (EE2-1.1c: Improvement on chroma MPM) [Y. Liu (Transssion)] [late]

JVET-AP0275 Crosscheck of JVET-AP0133 (EE2-1.1: Improvement on chroma MPM) Test a & b [Z. Xie (OPPO)] [late]

JVET-AP0145 EE2-1.2: Modification of EIP filter shapes [Y. Liu, Z. Zhang, Y. Huo, Z. Zhang, J. Cai (Transsion)]

JVET-AP0239 Crosscheck of JVET-AP0145 (EE2-1.2: Modification of EIP filter shapes) [H. Qin (TCL)] [late]

JVET-AP0249 Crosscheck of JVET-AP0145 (EE2-1.2: Modification of EIP filter shapes) [W. Niu (ZTE)] [late]

JVET-AP0168 EE2-1.3: Clipping operation refinements in CCCM modes [P. Onno, G. Laroche, B. Galmiche (Canon), L. Xu, C. Liao, Y. Lu, F. Liang (SYSU)]

JVET-AP0240 Crosscheck of JVET-AP0168 (EE2-1.3: Clipping operation refinements in CCCM modes) [Y. Ahn (Qualcomm)] [late]

### EE2 related contributions (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0143 EE2-related: Combination of JVET-AO0092 and EE2-1.2b [P.-K. Liu, C.-Y. Liu, S.-P. Wang, C.-C. Lin, C.-L. Lin (ITRI), Y.-T. Liu, Z.-H. Zhang, Y.-K. Huo, Z.-H. Zhang, J.-Y. Cai (Transsion)]

JVET-AP0260 Crosscheck of JVET-AP0143 (EE2-related: Combination of JVET-AO0092 and EE2-1.2b) [H. Qin (TCL)] [late]

### ECM modifications and software improvements beyond EE2 (7)

Contributions in this area were discussed during 0910–1010 on Saturday 25 April 2026 (chaired by JRO).

JVET-AP0105 Non-EE2: A bugfix of the subsampling filter for certain CCLM/CCCM tools [T. Chujoh, H. Kato, Y. Kidani, K. Kawamura (KDDI)]

In this contribution, a 4:4:4-to-4:2:0 subsampling filter is improved for certain CCLM/CCCM tools in the ECM-19.1 software. At the Geneva meeting in October 2025, experiments on adaptive subsampling filter selection were conducted; however, they didn't include the case where ChromaLocType is equal to 2 (top-left position) for HDR sequences. Therefore, this contribution improves it and reports the experimental results. As a result, the coding efficiency for HDR sequences has been improved without increasing the encoding and decoding times.

This would modify the implementation of some existing tools such that they would work as intended for ChromaLocType equal to 2, and improve performance for HDR sequences.

Decision (BF/SW): Adopt JVET-AP0105. (no change in CTC, HDR is not part of CTC).

JVET-AP0199 Crosscheck of JVET-AP0105 (Non-EE2: A bugfix of the subsampling filter for certain CCLM/CCCM tools) [P. Bordes, F. Galpin (InterDigital)] [late]

JVET-AP0123 Matrix based MPM prediction [Y. Gao, H. Tian (Shandong Univ.), S. Li, J. Lei, B. Li, F. Xing, P. Han (Hisense)] [late]

In EE2, the MPM list is constructed to find the best RD mode for the current block based on the best RD mode of neighbors. That is because the best RD mode of the current block is related to the best mode of neighbors. These best mode is related to each other, yet not necessarily exactly the same. Thus a matrix based MPM prediction is presented with, where the relationship between the best mode of the current block and the neighbors is explored. And no additional complexity is introduced.

On top of ECM-19.0, results of the proposed method are:

AI:  -0.01%	-0.07% 	-0.04% , EncT: % , DecT: %.

Partial results in RA show losses.

It was suggested to not use CTC sequences for training (currently, CTC sequences and BVI have been used).

Further investigation may be necessary on handling non-angular modes.

No action at this moment.

JVET-AP0156 Non-EE2: Adaptive GPM blending weights with linear and Hermite profiles [K. Kishimoto, H. Kato, Y. Kidani (KDDI)]

This contribution proposes adaptive Geometric Partitioning Mode (GPM) blending weights with linear and Hermite profiles. In VVC and the current ECM design, GPM blending weights are derived from a linear function of the sample-to-boundary distance. While this design is simple and decoder-friendly, it does not always fit smoothly varying sample values around the partition boundary. More specifically, the conventional linear profile is retained for the narrowest blending-width candidate, while wider candidates use cubic or quintic Hermite profiles according to the selected blending width. The resulting weights are clipped to the valid range, converted to the integer blending domain, and then used to combine the two conventional GPM predictors. On top of ECM-19.1, the reported results for all JVET CTC sequences are as follows {Y, U, V, EncT, DecT}:

-	AI: -0.01%, 0.00%, 0.00%, EncT 100.2%, DecT 100.2%;

-	RA: -0.05%, 0.08%, 0.05%, EncT 100.0%, DecT 99.9%;

-	LDB: 0.00%, 0.31%, 0.12%, EncT 99.9%, DecT 100.0%.

Loss in LDB chroma. No gain in AI. The relative small gain in RA would not justify switching between two blending methods, where the new one proposed requires additional computation.

JVET-AP0180 Non-EE2: Fix for MDIP [W. Niu, Z. Li, M. Jia, X. Zeng, S. Xie, Y. Bai (ZTE)]

In ECM-19.1, it is asserted that MDIP has problems in deriving 2 intra prediction modes from left template area. The problem is fixed in the contribution and test results on top of ECM-19.1 are summarized as follows:

AI(Y/U/V): 0.00%/-0.02%/0.00%, EncT 99.5%, DecT 99.6%

The proponents were requested to clarify with the SW coordinator if it is relevant to implement this fix. It does not seem to have much impact on the performance of MDIP.

JVET-AP0261 Crosscheck of JVET-AP0180 (Non-EE2: Fix for MDIP) [L. Xu (OPPO)] [late]

JVET-AP0181 Non-EE2: Enhancement of Chroma MPM List Construction [W. Niu, Z. Li, M. Jia, X. Zeng, S. Xie, Y. Bai (ZTE)]

This proposal proposes to enhance the chroma MPM list construction, specifically, to modify the MPM list construction for non-CCP mode. The simulation results on top of ECM-19.1 are summarized as follows:

AI : {-0.02%, 0.01%, -0.03%, 100.0x%, 100.8x%}

Gain is asserted to be too small to justify introduction of more candidates, which also increases decoding time.

JVET-AP0262 Crosscheck of JVET-AP0181 (Non-EE2: Enhancement of Chroma MPM List Construction) [L. Xu (OPPO)] [late]

JVET-AP0233 Non-EE2: Improvements on IBC-AMVP candidate list construction [H. Jeong, B. Jeon (SKKU)]

This contribution proposes a TM-cost-based pre-filtering stage for non-adjacent and history candidates in regular IBC-AMVP candidate list construction. When no adjacent candidate is listed, up to two non-adjacent candidates with the lowest TM costs are added if such candidates are available; when adjacent candidate is already enlisted, only one non-adjacent candidate with the lowest TM cost is added. For the history candidates, only the candidate with the lowest TM cost is added. The subsequent clustering process remains unchanged. The proposed method achieves BDBR gain compared to ECM-19.1 as follows:

Class F: -0.03% (Y) / -0.13% (Cb) / -0.09% (Cr), Enc: 99.5%, Dec: 99.2%.

No gain, and increase of encoding time in class TGM. Also in class F, two sequences have loss.

Not relevant to be considered.

JVET-AP0235 Non-EE2: Temporal CABAC and CTPI at Different CTU Locations [Z. Xiang, R. Chernyak, H. Huang, S. Liu (Tencent)]

Temporal probability information is used to improve initialization probability of inter coded slices in ECM. Such methods include Temporal CABAC (JVET-Z0133) and Counter-based Temporal Probability Initialization (CTPI). This proposal studies the performance of Temporal CABAC and CTPI at different CTU locations over ECM-19.1.

Summary of experimental results are as follows:

Anchor: ECM 19.1 (No Temporal CABAC); Test: ECM 19.1 (Temporal CABAC @ Last CTU)

RA: [Class B] -0.44%(Y)	0.13%(U)	-0.13%(V)

[Class C] -0.41%(Y)	-0.08%(U)	-0.24%(V)

LDB: -1.16% (Y)	-0.80%(U)	-0.88%(V)

Anchor: ECM 19.1 (No Temporal CABAC); Test: ECM 19.1 (CTPI @ Last CTU)

RA: [Class B] -0.47%(Y)	0.13%(U)	-0.07%(V)

[Class C] -0.44%(Y)	-0.14%(U)	-0.19%(V)

LDB: -1.20%(Y)	-0.69%(U)	-0.73%(V)

Anchor: ECM 19.1 (No Temporal CABAC); Test: ECM 19.1 (Temporal CABAC @ Half CTU)

RA: [Class B] -0.41%(Y)	0.13%(U)	-0.03%(V)

[Class C] -0.33%(Y)	-0.05%(U)	-0.23%(V)

LDB: -0.95% (Y)	-0.55%(U)	-0.61%(V)

Anchor: ECM 19.1 (No Temporal CABAC); Test: ECM 19.1 (CTPI @ Half CTU)

RA: [Class B] -0.44%(Y)	0.14%(U)	-0.03%(V)

[Class C] -0.35%(Y)	-0.08%(U)	-0.13%(V)

LDB: -1.00%(Y)	-0.57%(U)	-0.36%(V)

Related to EE2-5.1. Interesting additional information which aspects could give further gain in temporal probability initialization.

No specific action.

### CTC for EE2 and software aspects (including non-ECM SW) (0)

This section is kept as a template for future use.

# High-level syntax (HLS) and related proposals (118)

## AHG9: Study of SEI messages in VSEI, VVC, HEVC and AVC (4)

Contributions in this area were discussed during 1700–1950 on Friday 24 April 2026 (chaired by J. Boyce).

JVET-AP0058 AHG2/AHG9: Some errata items on VVC, VSEI, HEVC, and/or AVC [Y.-K. Wang, J. Xu (Bytedance)]

This contribution reports some asserted text bugs in the specifications of VVC, VSEI, HEVC, and/or AVC and proposes changes for fixing them.VVC

- In 7.4.3.4, change "CVLS" to "CLVS" in the following sentence: "sps_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is enabled for the CVLS".

- In 7.4.3.5, replace "When sps_res_change_in_clvs_allowed_flag equal to 0" (two instances) with "When sps_res_change_in_clvs_allowed_flag is equal to 0" (adding "is").

- In D.12.7, replace '–' in both bullet items with the correct minus sign, i.e., '−'.

- In D.12.20, replace "sps_pic_width_in_luma_samples" with "sps_pic_width_max_in_luma_samples", and "sps_pic_height_in_luma_samples" with "sps_pic_height_max_in_luma_samples" (adding "_max)".

VSEI, AVC, HEVC: several small fixes.

Delegate to editor.

JVET-AP0090 AHG9: On the cancel_flag related semantics in the FP, PRI and CTI SEI messages in VSEI v4 [J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

This document proposes the following items:

Modify the semantics of fp_upsampled_aspect_ratio_flag according to the value of fp_arrangement_cancel_flag in the frame packing arrangement (FP) SEI message.change the “shall” to “should”.

Agreed for VSEI with changing “shall” to “should”. Further study for HEVC.

- Modify the semantics of ListOfPriAssociatedLayers and related constraints according to the value of pri_cancel_flag in the packed regions information (PRI) SEI message.

Agreed for VSEI errata.

- Modify a constraint related to colour_transform_id according to the value of colour_transform_cancel_flag in the colour transform information (CTI) SEI message.

Agreed for VSEI errata.

JVET-AP0091 AHG9: On semantics related to persistence_flag and cancel_flag in various SEI messages in VSEI v4 and VSEI TuC [J. Xu, Y.-K. Wang (Bytedance), H. Tan (LGE)]

This document proposes the following items:

- Change the semantics related to persistence_flag of various SEI messages to not be conditioned on the output status of the picture. The related SEI messages include the FGC, CCV, ERP, GCMP, SR, RWP, AR, OV, SARI, ACI, DO, PRI, CTI, NNPFA, PI, EOI, SPTI, OMI, MI, TDI, AURR, IFM, FGR, LOC, GRI, PCI, DOI, DR, CMI, ASAI, and LAM SEI messages.

It was suggested to add a note for the SEI messages in VSEI v4, AVC, and HEVC. Agreed to add a note.

For SEI messages in the TuC, a change can be made. A constraint could be made.

Further discussion requested after offline discussion.

2) Change the semantics related to cancel_flag of various SEI messages for clarification, especially for the cases related to multi-layer bitstreams. The related SEI messages include the ERP, GCMP, SR, RWP, AR, OV, DO, NNPFA, EOI, MI, AURR, IFM, DOI, PCI, DR, DI, and LAM SEI messagesIt was questioned if any changes were needed. It was questioned if there are any implications if an SEI message is nested in a scalable nesting SEI message.

Agreed for VSEI errata and the TuC.

Further discussed at 1500 on Wednesday 29 April 2026 chaired by J. Boyce.

A -v3 version has been uploaded with a new author added.

Agreed to the -v3 version.

JVET-AP0217 AHG9: On FGC SEI message payloadType code points in AVC [R. Skupin, Y. Sanchez, C. Hellge, T. Schierl (HHI)]

The contribution asserts that adding support for the VSEI FGC SEI message in AVC leads to issues with interaction of persistence signalling between the then two payloadType code points for FGC SEI messages. Further an issue with contradicting information in applicable FGC SEI messages is identified.

It is proposed to

- add clarifying language to the persistence related semantics of the FGC SEI message in AVC and VSEI

add bitstream constraints related to the PON-nesting and content of VSEI FGC SEI messages to Annex D.2.1 of AVC. the language needs to be fixed for because payload type is not defined in VSEI.

Agreed to add AVC items to AVC spec, and to add VSEI items to errata as reflected in a -v2 version of the document.

## AHG9: Study JVET-AO2032 TuC for VSEI (95)

Discussion about table of SEI messages in TuC

Google sheet created. Proponents should fill in for their SEI messages. Be concise. Follow same order as in the TuC document.

Plan to discuss Saturday first thing after lunch. Want to finalized by end of Sunday, so is available for JVET plenary on Monday.

Use bug tracking for simple bug fixes.

VSEI: https://vcgit.hhi.fraunhofer.de/jvet/publications/VSEI

TuC: https://vcgit.hhi.fraunhofer.de/jvet/publications/VSEI-TuC

Need to be efficient.

Set a timer for initial presentation for 10 minutes. Multi-part contributions may require more time.

Move to offline discussion when relevant

Only have long discussion when other participants support having more time.

Can defer contributions to later meeting.

Discussion about table describing SEI messages during 1400-1505 on Saturday 25 April 2026.

It was suggested that software availability column should indicate whether processing is implemented or just syntax parsing. Some SEI messages that are descriptive may not need processing.

In terms of timeline for a new standard, it is noted that new versions of the video coding standards that reference it would also be needed.

A participant suggested that a first WD or PWD be output at next meeting rather than this meeting so there is time to make SW available.

At JVET plenary yesterday, it was noted that an ITU-T SG21 meeting is planned for April 2028, and that April 2028 may be an appropriate time for consent of VSEI v5.

HLS Track recommends that we plan to output a WD or preliminary WD for VSEI v5 at the July 2026 meeting

The current spreadsheet table will be included in a contribution document by Monday for discussion in JVET plenary.

It was suggested that proponents bring contributions to the next meeting with proposals of including particular SEI messages in the WD.

JVET-AP0290 Summary of VSEI TuC status [J. Boyce]

Was presented 1130 during the plenary on Wednesday 29 April

About potential additions to the table, and information to be requested from proponents who desire inclusion from TuC in a future WD, or new messages in TuC:

- Report the number of companies interested in that message (in terms of contributing content of that message) – companies requesting for inclusion in WD should also report about this.

- More detail about the nature of the available/necessary software (only writing/parsing, or also implementation of extraction or output processing (which may not apply to all messages).

- Consideration about potential desirability of normative output processing.

For SEI messages already in the table, it would also be important to add that information (e.g. it would also be useful to make an assessment for which SEI messages additional processing would be desirable (e.g., if the intent is improvement of compression)).

Further improvement of the table as mandate of the AhG.

Agreed Include the table from JVET-AP0290 as additional file in JVET-AP2032.zip.

### General (12)

Contributions in this area were discussed during 1950–2000 on Friday 24 April and 0900 1230 and 1505 –1615 on Saturday 25 April 2026 (chaired by J. Boyce).

JVET-AP0069 AHG9: On VSEI RBSP [M. M. Hannuksela, J. Boyce, D. Fortin (Nokia)]

Chaired by S. Deshpande on 25 April 2026 from 9:00-9:50 CEST

This contribution proposes the following changes to the VSEI RBSP design of the VSEI TuC:

- The following asserted bug fix related to the inferring the SEI message payload size is proposed:

When VSEI RBSP includes only one SEI message and vsei_payload_size_length is equal to 0, vsei_payload_size is inferred to be equal to the number of bytes in the RBSP minus 1 (used for VSEI RBSP header) minus the number payload type bytes minus 1 (used for the trailing byte in the VSEI RBSP caused by RBSP trailing bits).

This items is related to JVET-AP0137 which has some similar intention.

See additional discussion there. There was more support for the approach of counting bytes as proposed here.

Decision: Agreed to JVET-AP0069 item 1.

- The following asserted bug fixes related to referring an unspecified vsei_payload( ) syntax structure are proposed:

  - It is proposed that versatile_sei_message( ) references sei_payload( ) rather than vsei_payload( ).

  - It is proposed to add sei_payload( ) into the VSEI TuC with the following indicated changes relative to VVC:

- The SEI payloads for the VSEI RBSP with vsei_position equal to 0 are the same as those for prefix SEI NAL units.

- The SEI payloads for the VSEI RBSP with vsei_position equal to 1 are the same as those for suffix SEI NAL units.

It was commented that sei_payload is codec specific. It was commented that what is currently in the TuC is broken.

Decision: Agreed.

- It is proposed to omit the byte alignment at the end of the last SEI message payload of the VSEI RBSP so that the extra trailing byte caused by rbsp_trailing_bits( ) is avoided. The asserted benefit of this proposal is a one-byte saving in each VSEI RBSP for which either of the following is true:

The last SEI message in the VSEI RBSP is extended relative to its original syntax (SeiExtensionBitsPresentFlag is equal to 1).last SEI message in the VSEI RBSP uses its original syntax (SeiExtensionBitsPresentFlag is equal to 0) but does not end at a byte-aligned position.

This can save one byte if SEI did not end on a byte-aligned position. This design was implemented and confirmed to work by the proponents internally. It was asked if this works for extension bits. The proponent will check if this has been tested regarding extension bits aspect.

Decision: Agreed

Discussed again on 29 April 2026 from 15:40-15:45 CEST chaired by S. Deshpande.

The proponents reported verifying the requested extension functionality as correctly handled for a decoder which understands extension and also for a decoder which ignores the extension.

JVET-AP0076 AHG9: SW for floating point syntax fl(n) and fl(w,t) [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

Several SEI messages use floating point syntax. While implementing SW for some SEI messages, (LAM, LOC) implementation of floating-point was required.  This contribution proposes SW for floating point types which may be used in SW implementation for various SEI messages. Implementation of fl(16), fl(32) and fl(w,t) for w,t less than 33 are provided as part of those SEI implementations.

Agreed to add SW.

JVET-AP0137 AHG9: On payload sizes for versatile SEI messages [M. Pettersson, M. Damghanian, S. Harribey, R. Sjöberg (Ericsson)]

This contribution proposes updates to the signaling and inference of the payload size of the versatile SEI message in the TuC for future extension of VSEI. Two independent proposals are suggested where the first proposal is an asserted bug fix for inferring the payload size when it is not present and the second proposal proposes an updated extendable signalling of the payload size. The following is proposed:

- When the payload size for an SEI message is not present, infer the payload size to be equal to NumBytesInNalUnit – 3 – vseiPayloadType/256, instead of inferring it to be equal to NumBytesInNalUnit – 4 as in the TuC.

Chaired by S. Deshpande on 25 April 2026 from 9:15-9:95 CEST
This was discussed together with item 1 of JVET-AP0069. It was asked if the division should be by 255 instead of 256.

It was also commented that we should not assume 2 byte NAL unit header which this contribution does. JVET-AP0069 is more general about the NAL unit header size.

- Signal the payload size with an extendable number of bytes for each SEI message. Two alternatives are proposed:

  - Replace vsei_payload_size with two new syntax elements vsei_payload_7bits and vsei_payload_extension_flag and derive vseiPayloadSize according to:

| vseiPayloadSize = 0 |  |
| --- | --- |
| do { |  |
| vsei_payload_7bits | u(7) |
| vseiPayloadSize = (vseiPayloadSize << 7) + vsei_payload_7bits |  |
| vsei_payload_extension_flag | u(1) |
| } while (vsei_payload_extension_flag) |  |

  - Signal the vsei_payload_size using a new eu(n) descriptor exercising the method of alternative A. The solutions in alternatives A and B are asserted to produce bit exact output. The proponents claim that the eu(n) descriptor may be beneficial for other syntax elements of the codec with large values.

Both alternatives also propose to replace the vsei_payload_size_byte syntax element with a vsei_payload_size_flag syntax element, that specifies whether syntax elements for the payload size are present or not.

It is asserted by the proponents that the extendable signalling provides a more flexible and future proof solution for signalling the payload size for individual SEI messages.

It is proposed to update the solution in the TuC for future extension of VSEI with the two proposals.

Proposal 2 chaired by J. Boyce.

Proposes extendable syntax for payload size.

The proposed syntax saves 3 bytes for large SEI message. It was suggested that it adds a byte for some other SEI message lengths.  No action on proposal 2.

JVET-AP0138 AHG9: On SEI messages presence [M. Damghanian, S. Harribey, M. Pettersson, R. Sjöberg (Ericsson)]

This contribution proposes new text to be added to VSEI v4 (errata) and VSEI TuC, to specify a preferred decoder action for the case that presence of an SEI message for the first picture of a CLVS (or CVS) is mandated when there is an SEI message of that type present for pictures other than the first picture in that CLVS (or CVS), and the SEI message for the first picture of the CLVS (or CVS) is not present (e.g., dropped or lost). In such a case, the proponents claim that it is not specified or suggested what action a decoder supporting the SEI message should take. The proponents state that a decoder supporting the SEI message could be configured for instance a) to act as if the SEI message for the first picture of the CVS were present, b) to ignore the SEI message, etc. The proponents claim that a decoder may benefit from a described preferred action to improve interoperability. Three options are proposed that can be applied to single SEI messages or a group of SEI messages for which an SEI message for the first picture of a CLVS, or CVS, is mandated.

It was commented that if a MANE were to drop a first SEI message in a CLVS, it would likely drop subsequent occurrences as well.

This proposal can address the network loss of an SEI messages. We have not previously considered error concealment for SEI messages. The video coding standards also don’t recommend behavior for lost pictures and slices.

It was considered dangerous to make such a change for existing standards.

It was suggested that a TR could address non-conforming bitstreams.

JVET-AP0160 AHG9: On signaling of identifiers and numbers of layers for SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to signaling of identifiers and numbers of layers in constituent rectangles (CR), display overlays information (DOI), bitdepth range information (BRI), and enhanced colour format information (ECFI) SEI messages. The proposed modifications are as follows:

(Related to CR SEI message)

1	Specify value range of cr_layer_id[ lIdx ] in VVC.

2	Specify value range of cr_vui_layer_id[ i ] in VVC.

3	Specify value range of cr_num_layers_minus1 in VSEI and VVC.

(Related to DOI SEI message)

4	Specify value range of doi_layer_id[ i ] in VVC.

5	Specify value range of doi_num_layers_minus1 in VSEI and VVC.

(Related to BRI SEI message)

6	Specify value range of bri_layer_id[ i ] in VVC.

(Related to ECFI SEI message)

7	Specify value range of ecfi_layer_id[ i ] in VVC.

8	Specify value range of ecfi_vui_layer_id in VVC.

9	Change descriptor of ecfi_num_layers_minus1 and specify its value range in VSEI and VVC.

The proponents note that the proposed text is similar to that already in PRI SEI in VSEI v4.

Agreed.

JVET-AP0161 AHG9: On presence of SEI messages in multiple layers [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to handling constituent rectangles (CR), display overlays information (DOI), bitdepth range information (BRI), and enhanced colour format information (ECFI) SEI messages when the SEI messages are present in multiple layers. The proposed modifications are as follows:

1	On CR, BRI, and ECFI SEI messages

–	Specify that the SEI message shall be present in the lowest layer among the layers included in the SEI message and there shall be no repetition of the same SEI in other layer.

–	Specify that all the SEI messages that are included in the same CVS but different CLVSs shall associate with the mutually exclusive sets of the layers.

2	On DOI SEI message

–	Specify that the SEI message with a particular value of doi_id shall be present in the lowest layer among the layers included in the SEI message with the same value of doi_id and there shall be no repetition of the same SEI message with the same value of doi_id in other layer.

Agreed on requiring that the SEI message shall be present in the lowest layer among the layers included in the SEI message.

It was questioned why the constraint on duplicating the SEI message in a higher layer is problematic. It was suggested that duplicating might be useful for extraction.proponents assert that duplicating the SEI message in a higher layer may cause a problem for SPO.

It was noted that the CR SEI wouldn’t likely be in the SPO anyway because it is a property not a process.

Further study on the constraint.

JVET-AP0162 AHG9: On conformance cropping windows for SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to handling cropping windows for constituent rectangles (CR), display overlays information (DOI), bitdepth range information (BRI), and enhanced colour format information (ECFI) SEI messages. The proposed modifications are as follows:

Option 1: modify the semantics such that partitioning is based on the cropped picture.

–	Specify that the variables PicWidthInLumaSamples[] and PicHeightInLumaSamples[] indicate the width and height of the cropped picture.

–	The cropped picture size is calculated based on the coded picture size minus the conformance window size / crop window size that may be signalled in the parameter set of codec that uses this SEI message.

–	Modify the term ‘coded picture’ to ‘cropped picture’ in the SEI message semantics.

–	Modify / introduce the interface text for SubPicTopLeftX[][], SubPicTopLeftY[][], SubPicWidth[][], and SubPicHeight[][] such that a region indicated by those variables doesn’t include any samples outside the conformance window.

Option 2: specify a constraint in each of VVC interface such that those SEI messages shall not associate with the layers that contain the cropped regions.

In the first revision of this contribution, the following updates were made:

–	Editorial update to description of proposed option 2.

–	Move the text implementation of both proposed options to the two attachment files.

The derivation of SubPicWidth and SubPicHeight have an unnecessary “– 1” at the end. Agreed to remove it.

A key question to consider is if each of the SEI messages intends to operated on the decoded picture or the output picture.

Revisit.

Further discussed at 1645 on Wednesday 29 April chaired by J. Boyce.

Agreed to option 1 for CR, DOI, ECFI, and BRI SEI messages.

JVET-AP0163 AHG9: On inference values of syntax elements in SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to handling inference values of syntax elements in constituent rectangles (CR), display overlays information (DOI), and localization and mapping (LAM) SEI messages. The proposed modifications are as follows:

(Related to CR SEI message)

1	On subpicture partitioning

–	Modify the semantics such that the values of syntax elements that are related to subpicture partitioning are inferred only when they are needed.

(Related to DOI SEI message)

2	On partition and layer indexes

–	Remove the sentences for inference of the doi_partition_idx[ j ] and doi_alpha_partition_idx[ j ] values.

–	Modify the sentence for inference of the doi_alpha_layer_idx[ j ] value such that this value inferred when they are not present and doi_alpha_present_flag[ j ] is equal to 1.

(Related to LAM SEI message)

3	On principle point

–	Modify the conditions of the inference so that lam_principal_point_x_times100 and lam_principal_point_y_times100 are inferred when they are not present and lam_intrinsic_param_presence_flag equal to 1.

4	On focal length

–	Option 1: specify that lam_focal_length_times100 is inferred when not present.

–	Option 2: update the signalling of lam_focal_length_times100 such that it is present regardless the value of lam_camera_model_presence_flag.

5	On polynomial coefficient for radial distortion model

–	Option 1: introduce gating flags for lam_rd_poly3_k1, lam_rd_poly5_k1 and lam_rd_poly5_k2.

–	Option 2: remove the sentences that specify the inference values of lam_rd_poly3_k1, lam_rd_poly5_k1 and lam_rd_poly5_k2.

Agreed on item 1, item 2, item 3,  item 4 option 2, item 5 option 2.

JVET-AP0196 AHG9: Working draft for VSEI v5 [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

JVET-AP0216 AHG9: On SEI extensions for legacy devices [R. Skupin, Y. Sanchez, T. M. Borges, C. Hellge, T. Schierl (HHI)]

When encountering unknown payload beyond the expected SEI message payload, legacy decoders shall ignore the unknown payload bits and process the SEI message payload up to the unknown extension bits.

This contribution argues that this behaviour may not always be desirable and proposes to add a mechanism that allows encoders to indicate whether SEI payload extension bits are required to correctly process the SEI message by a legacy device that cannot parse a particular SEI message payload extension.

It is proposed to consider adding the signalling to the draft text for the new version of AVC and VSEI v5 as well as for future versions of HEVC and VVC and future codecs.

The mechanism consists of adding a flag (sei_payload_ext_indispensable_flag) to the sei_payload structure() in the codec specifications and a corresponding flag (sei_ext_indispensable_flag) to future SEI message extensions in VSEI, controlled by presence variables to accommodate legacy devices.

The value of SeiExtIndispensablePresntFlag would be done by the codec specification for each SEI message.

It was noted that the SEI manifest message can indicate essentiality of a full SEI message.

It was suggested that a new SEI message can be defined if an extension is indispensable.

A question was raised  about the need for identifying indispensable in particular extensions in multiple extensions.

Further study.

JVET-AP0229 AHG9: Signaling options for alpha plane metadata in non-auxiliary picture context in AVC, HEVC and VVC [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

This contribution analyzes signaling options for alpha metadata in non-auxiliary picture contexts across AVC, HEVC, VSEI, and VVC specifications. Three primary approaches are considered: extending the existing alpha channel information SEI message, creating a new non-auxiliary alpha information SEI message, and adding alpha information to VUI parameters. It is reported that each option has different compatibility implications across the video coding standards, requiring varying levels of specification changes and standardization effort. It is asserted that creating a new SEI message in VSEI would be preferable approach to avoid breaking existing auxiliary picture implementations and to maintain clear separation between auxiliary and non-auxiliary alpha signaling.It was suggested that using VUI would require an extension to the SPS which could mean new profiles would be required.

It was noted that if a new SEI message was created, it would be possible to add some additional features.

Option 2 for a new SEI message was preferred.

It was suggested that a new SEI message and the existing SEI message could have a common syntax structure, with potentially additional syntax elements for the new SEI message.

For the proposed semantics, it was questioned why VUI colour primaries is mentioned in the first sentence, and it was suggested to remove that mention.

The use of “another picture’ in the semantics is somewhat misleading, and should be reworded or removed. Could perhaps use “colour plane”.

Further study is suggested for use of this new SEI message with the CR and ECFI SEI message.

It was noted that bit depth may differ from the bit depth of the coded picture and the semantics should be updated to explain how to interpret a differing value in terms of suggested operations on the decoded pictures to adjust the bit depth.

It was noted that a new SEI message could potentially replace the existing SEI message, which may lead to market fragmentation and confusion.

Revisit.

JVET-AP0230 AHG9: Signaling options for depth map metadata in non-auxiliary picture context in AVC, HEVC and VVC [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

This contribution presents three options for signaling depth map metadata in non-auxiliary picture contexts. The options are analyzed based on compatibility with existing AVC, HEVC, VSEI, and VVC specifications. Three primary approaches are considered: extending the existing depth representation info SEI message, creating a new non-auxiliary depth information SEI message, and adding depth information to VUI parameters. It is reported that each option has different compatibility implications across the video coding standards, requiring varying levels of specification changes and standardization effort. It is asserted that creating a new SEI message in VSEI would be preferable approach to avoid breaking existing auxiliary picture implementations and to maintain clear separation between auxiliary and non-auxiliary depth signaling.

The proposal removes the disparity depth representation type, but still support inverse depth. It is noted that use of disparity requires information about the other view used for the calculation, which is considered to not be appropriate for including in a new SEI message for use in a single layer bitstream for which any additional views are externally provided.

The semantics language has similar problems identified in JVET-AP02229.

It was suggested that an identifier could be added.

It was noted that with a new message additional functionality could be removed.

Should consider the case where both the earlier SEI message and the proposed new SEI message were present in the same bitstream. May want to consider if constraints should be placed.

Revisit.

### New extensions or constraints of SEI messages specified in VSEI v4 (7)

Contributions in this area were discussed during 1640–1805 on Saturday 25 April 2026 (chaired by J. Boyce).

JVET-AP0073 AHG9: Proposed Shutter Interval SEI message extension to rolling shutter for version 5 of VSEI [L. Kerofsky, M. Karczewicz (Qualcomm), J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes adding an extension of the Shutter Interval Information (SII) SEI message to support rolling shutter cameras into the working draft of version 5 of VSEI.  The extended SI SEI message is meant to be available for HEVC and VVC.

Software implementing the extended SII SEI message is available and will be provided for integration with the VTM if this message is adopted into a VSEIv5 draft.

Notes that SW is available.

JVET-AP0083 AHG9: On OMI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes a deterministic derivation method that first identifies the set of matching masks and then selects a single mask identifier using depth information, confidence information, or mask identifier ordering.

No action.

JVET-AP0110 AHG9: On signaling of AI usage type for AURR SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes an extension to the AURR SEI message to enable signaling of AI usage type, which refers to the type of AI usage characterized by the nature of the information derived from the picture and the purpose of the usage. The current design is asserted to not distinguish different AI usage types when the same AI processing is applied, even though such usage types may have significantly different risk and regulatory implications. The proposed extension claims that it would enable recognization of such usage types, and allow more precise interpretation of requested restrictions.

Further discussion requested after offline discussion.

Further discussed at 1515 on Wednesday 29 April 2026.

Further study.

JVET-AP0113 AHG9: On signaling content moderation information in TDI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes to enable the signaling of content moderation information using the TDI SEI

Agreed to add a new TDI purpose.

JVET-AP0147 AHG9: On the Alpha Channel Information SEI message [C. H. Demarty, E. François, P. de Lagrange, F. Urban, N. Caramelli (InterDigital)]

The current specification allows multiple SEI messages to reference auxiliary layers of type ALPHA_MAP, including the ACI, FGRC, and Green SEI messages. These SEI messages may assign different semantics and processing roles to the same auxiliary layer, which can lead to ambiguous or conflicting decoder behaviour.

This contribution identifies configurations in which multiple SEI messages reference the same alpha plane and highlights potential inconsistencies arising from incompatible interpretations or non-commutative processing operations.

To address these issues, constraints are proposed to clarify decoder behaviour in the presence of multiple SEI messages referencing alpha planes.

In v2 version, the specification text proposals are slightly changed.

It was suggested that a new codepoint for alpha_channel_use_idc could be defined for the FGRC SEI message, rather than using the unspecified codepoint. The proposed constraint could use the new codepoint value.

Where would the most appropriate place to impose the constraint? Candidates are the ACI SEI, FGRC, and SDI SEI message. It was suggested that the SDI SEI message may the appropriate location.

It was suggested that the codepoint itself be for generic film grain blending and not specific to the FGRC SEI message.

Further discussion requested on item 1 after new text reflecting the above is available.

No action on item 2.

In v3 version, the proposal is modified to reflect suggestions from the group in clause 2.4.

Further discussed at 1520 on Wednesday 29 April 2026 chaired by J. Boyce.

It was suggested that nesting SEI messages in the SPO as an alternative.

Agreed to the v3 version. This change should also be reflected in the new alpha SEI message for primary layers.

JVET-AP0175 AHG9: On shutter interval information SEI message [X. Xu, M. Tang, S. Liu (Tencent)]

Presented not requested.

JVET-AP0252 AHG9: On multiple extensions of SEI messages [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)] [late]

It is asserted that in the design of VSEI, it is not clear how to handle SEI message that have been exended multiple times. While it is asserted that the assumption seems to be that it is expected for older decoders to process the SEI up to the syntax element in the extension that is specified in VSEI version that they conform to and ignore further bits that they do not know because those bits are only specified in future extension, such assumption is not clearly specified.

In this contribution, it is proposed to clarify this issue by including text in VSEI to specify that when payload extension is present, decoders that conform to the current version of VSEI Specification shall do both of the following:

- They shall not ignore syntax elements that are specified in the payload extension part in the current version of Specification.

- They shall ignore extension bits that are present after the last syntax element in the payload extension part in the current version of Specification

It was suggested that the existing design already addresses this issue. No action.

### Scalability dimension information (SDI) SEI message (3)

Contributions in this area were discussed during 1805–1925 on Saturday 25 April 2026 and 1710-1720 on Sunday 26 April 26 (chaired by J. Boyce).

JVET-AP0094 AHG9: On the scalability dimension information SEI message in VSEI TuC [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following items to the scalability dimension information SEI message described in the latest VSEI TuC:

1)	Add a constraint for sdi_aux_id[ i ] in the range of 128 to 159, inclusive.

2)	Add a constraint on the associated layer of a confidence map.

3)	Modify the semantics of the confidence value considering the signal range and monotonicity.

It was suggested to allocate some Reserved values of sdi_aux_id[ i ]  to unspecified while allowing association with a non-primary layer.

Agreed on item 1.

It was noted that alpha or depth can be primary layers. action on item 2.

Item 3 chaired by S. Deshpande.

For item 3, it was suggested that it is awkward to have to refer to the VUI to be able to interpret the SEI message. It was suggested that an option would be to signal a min and max value in the SEI message.

Agreed to add final proposed bullet of item, and to remove the two middle bullets, and add an editor’s note to study the min and max values.

Further study encouraged about determination of min and max, especially for determination of full confidence.

JVET-AP0098 AHG9: On the SDI confidence map [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm), J. Chen, Y. Ye (Alibaba Group)]

This contribution proposes the following changes to the confidence map in TuC.

- Propose text description in the SDI SEI message extension to differentiate multiple confidence maps associated with the same layer

- Propose the confidence map resolution constraint

- Propose the confidence map sample persistence

It was suggested that a note could be added to suggest external means.

No action on item 1.

Item 2 has two proposed options. Option 1 uses resampling. Option 2 require matching picture resolutions.

Item 2 is related to JVET-AO0167, which proposes that the confidence map picture dimensions shall be less than or equal to the pic width.

It was noted that the ASAI SEI in the TuC can associate sample positions between pictures with different dimensions.

Further discussion requested how to constrain and/or associate samples between the confidence map and its associated layer.

For item 3, it was suggested that a confidence map should not persist, and it should only apply if there is a confidence map present, and when not present is undefined.

Further study if some additional clarification in the text is required to express that.

Further discussed at 1530 on Wednesday 29 April 2026 chaired by J. Boyce

v2 proposed option C of item 2 and added co-authors.

Agreed to the -v2 version.

JVET-AP0167 AHG9: Fixes to the Scalability Dimension Information SEI message [J. Chen, Y. Ye (Alibaba)]

This contribution is to fix the identified issues in the scalability dimension information SEI message extension for confidence map. The following three fixes are proposed:

- Refine the value range of sdi_num_associated_layers[i] depending on sdi_aux_id[ i ] to make it more precise.

- Define interface arrays for the width and height of each layer, and constrain the confidence map layer not to exceed its associated layer in width or height.

- Define an interface variable for the bit depth of the confidence map layer.

For item 1, it was suggested to also address the unspecified values of sdi_aux_id[ i ].

Agreed for item 1, with addition for unspecified values.

For item 2, see notes for JVET-AP0098.

### Neural-network post-filter / related URIs (NNPFC / NNPFA) SEI messages (7)

Contributions in this area were discussed during 1500–1710 on Sunday 26 April 2026 (chaired by J. Boyce).

JVET-AP0130 in 4.3 is also related.

JVET-AP0060 AHG9: On nnpfc_uri and nnpfc_tag_uri  [S. Deshpande (Sharp)]

Following is proposed related to nnpfc_uri and nnpfc_tag_uri:

- Proposal 1: For supporting in-band transmission of neural network data in formats other than ISO/IEC 15938-17, a data URL scheme (IETF RFC 2397) is specified to be used for use in nnpfc_uri.

- Proposal 2: It is proposed to define additional nnpfc_tag_uri values for widely used/ deployed standards for specifying neural network information.

It is proposed to add the proposed approaches to the TuC.

For proposal 1, it is noted that a data URL can already be used. Adding some clarifying information is proposed.

It was suggested to change the first “shall be” to “is”.

It was suggested that a Note could be used instead.

It was suggested to follow up with some IETF experts on use of the data URL scheme, especially relative to potentially large data.

Further discussion requested onproposal 1.

For proposal 2, see notes under JVET-AP0068.

Further discussed at 1535 on Wednesday 29 April 2026 chaired by J. Boyce.

In v2 an alternative option for proposal 1 is proposed.

Agreed to the alternative option for proposal 1 in v2.

JVET-AP0068 AHG9: Additional tag URIs for the NNPFC SEI message [M. M. Hannuksela, J. Boyce, F. Cricri (Nokia), S. Deshpande (Sharp)]

Chaired by Y-K. Wang.

This contribution proposes to specify tag URIs for ONNX and PyTorch neural network frameworks. These tag URIs can be used in the nnpfc_tag_uri syntax element of the neural-network post-filter characteristics (NNPFC) SEI message to indicate the framework used in the neural network identified by nnpfc_uri.

Version 2 of the contribution adds a discussion on the choice of the tagging entity part of the proposed tag URIs. Inspired by JVET-AP0060 item 2, version 2 of this contribution also adds the version of the neural network framework into the proposed tag URIs.

Version 3 proposes shorter strings for the proposed tag URIs and uses the same phrasing for the new tag URIs as used for the NNC/NNR tag URI since VSEI v3 for consistency.

It was suggested to refer to the ONNX operator set version rather than the library version.

Agreed.

JVET-AP0081 AHG9: On Multiple Inferences for Neural-network Post-filter [S. Deshpande (Sharp)]

Following is proposed related to multiple inferences for Neural-network Post-filter (NNPF) in VSEI TuC:

- Asserted bug-fixes are proposed in StoreOutputTensors process for four nnpfc_output_order_idc values, related to multiple inferences, to the calculation of side-by-side packed temporal extrapolation X co-ordinates.

It is proposed to add the proposed bug-fixes to the TuC.

Agreed.

JVET-AP0092 AHG9: On redundant bits related to string signalling in the CR and NNPFA SEI messages in VSEI TuC [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following items:

- Save byte alignment when there is no type description for a layer in the constituent rectangles (CR) SEI message.

- Save a redundant bit for signalling nnpfa_prompt_update_flag in the neural-network post-filter activation (NNPFA) SEI message in some cases.

Item 2 discussed.

It was noted that the syntax of the NNPFA extension in the TuC has v4 backwards compatibility issue.

JVET-AP0176 is related to backwards compatibility issue.

Revisit.

JVET-AP0171 AHG9: On indication of intermediate data application in EOI, SPO and NNPFC SEI messages [X. Xu, S. Wenger, S. Liu (Tencent)]

In this contribution, addition of a syntax element in the TuC document is suggested, to extend the usage of video content, beyond the existing human viewing and machine analysis coverage. Such a change will impact the current EOI, NNPFC and SOP SEI messages.

The usefulness of the proposed additional information is not clear.

No action.

JVET-AP0176 AHG9: Bug fixes of NNPF SEI messages in TuC document [X. Xu, S. Liu (Tencent), Y.-K. Wang (Bytedance)]

In this contribution, bug fixes in the TuC document are suggested. Such changes will impact the current NNPFC and NNPFA SEI messages. In NNPFC and NNPFA SEI messages, newly added syntax elements do not follow the extension mechanism of VSEI V4.

In v2, areas of fixes are summarized.

The TuC text has syntax element name overlap between the base NNPFA SEI and the extension, nnpfa_prompt and nnpfa_prompt[ i ].

Delegate to editors to change syntax element name in the extension.TuC includes new syntax elements in the base NNPFA which cause problems with VSEI v4 backwards compatibility.

Revisit after offline activity.

JVET-AP0226 AHG9/AHG18: Missed Picture Generation purpose for NNPF SEI message [K. Malyshev, M. Sychev, S. Ikonin, E. Alshina (Huawei)]

This contribution proposes further extension of NNPF SEI messages to support a new purpose for applying NNPF specifically for missed picture generation. The proposal introduces a new MissedGenerationFlag within the nnpfc_purpose syntax element, allowing to dynamically select missed picture handling method. This addition includes the following aspects:

- A new bit in nnpfc_purpose is dedicated for missed picture generation.

- When nnpfc_purpose indicates missed picture generation, nnpfc_missed_generation_mode specifies the method, with mode 0 relying on closest picture replacement and mode 1 choosing between interpolation (picture rate upscaling) or temporal extrapolation.

- A new flag nnpfc_absent_generation_flag for picture rate upsampling NNPF, which removes restrictions to allow generating absent input pictures

- A new signaling for temporal extrapolation NNPF, that clarifies temporal positions of the extrapolated pictures, eliminating uncertainty.

This contribution signals recommended post-processing that a decoder would apply when the decoder detects a missing picture, e.g. using NNPF for error concealment.

Previously NNPF operation is encoder driven via the NNPFA SEI message. This proposed a fundamental change makes the operation become decoder driven, based on missing pictures.

For the absent generation flag, the proposed language says “the NNPF shall generated at least one new picture…”, which is problematic language for an SEI message.

It was asked if the NNPFC SEI message would be lost when the picture was lost. questioned if an NNPFA SEI message is sent. The proponents suggested that it is possible for the NNPFA SEI message to be inserted by the decoder.

It was asked if the generated picture would be for post-processing, or if it would also impact the DPB? Proponents say it is only for post-processing.

It was questioned if NNPF is the appropriate framework for this.

Proposal 3 proposes more explicit positioning of temporal position of extrapolation.proposed syntax for proposal 3 is not structured in a backward compatible manner.

Additional justification of the use case that requires this additional functionality would be useful.

### SEI processing order and processing order nesting (SPO/PON) SEI messages (1)

Contributions in this area were discussed during 1710–1720 on Sunday 26 April 2026 (chaired by J. Boyce).

JVET-AP0215 AHG9: On detection of SEI message extensions in SPO [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)]

SPO includes provisions for the fact that a decoder system may not be able to understand certain SEI messages, e.g. by indicating payload types and prefix bits in the SPO SEI message. However, no dedicated provision in SPO allows decoder systems to recognize presence of SEI message extensions in an efficient way.

This document proposes an extension for the SPO SEI message to add provisions allowing decoder systems to recognize the presence of required SEI message extensions bits in the SEI messages of the indicated processing chains during processing of the SPO SEI message.

The proposal adds a loop over the indicated SEI message types in the SPO SEI message extension to transmit a flag (po_sei_ext_indespensable_flag[ i ]) that indicates whether an SEI message type contains an SEI message payload extension that is required to correctly process the SEI message.

Is related to JVET-AP0216.

Further discussion requested.

Further discussed at 1645 on Wednesday 29 April 2026 chaired by J. Boyce.

A scenario has been described about v6 bitstream handling by a v5 decoder.

This is related to general extensibility in JVET-AP0216. It is requested to study the use cases where the current syntax can not address that requires the message to be the SPO chain.

Further study.

### Encoder optimization information (EOI) SEI message (8)

Contributions in this area were discussed during 1725–1930 on Sunday 26 April 2026 and during 1520-XXXX on Tuesday 28 April 2026 (chaired by J. Boyce).

JVET-AP0067 AHG9: On the encoder optimization information (EOI) SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

Chaired by S. Deshpande on 18:00-18:30 CEST on 26 April 2026.This contribution proposes the following changes to the extension of the encoder optimization information (EOI) SEI message specified in the VSEI TuC:

- It is proposed to remove eoi_num_sublayers and eoi_min_tid and related semantics, since it is asserted that they are not compatible with VEI version 4 decoders. To support the use cases that were presented in JVET-AG0086 to motivate eoi_num_sublayers and eoi_min_tid, it is instead proposed to include eoi_max_human_viewing_tid_plus1 in the extension of the EOI SEI message. When equal to 0, it indicates that it is unknown if any subset of temporal sublayers is suitable for human viewing. When greater than 0, it indicates the subset of temporal sublayers that is suitable for human viewing.

The main issue is we do not allow multiple EOI SEIs in the same PU, which is what the original proposal was designed for.

For backward compatibility reasons it is agreed to remove eoi_num_sublayers and eo_min_tid syntax elements and the semantics which depend on them.

Regarding addition of eoi_max_human_viewing_tid_plus1 syntax: which is a metadata only and has no relation to the persistence.was commented that the inference to 0 may not be correct and should be changed. V2 version of the document will be uploaded regarding the inference.

Agreed

Discussed again on 29 April 2026 from 15:45-15:50 CEST chaired by S. Deshpande.

Interface text in v2 regarding inference was reviewed. 3 alternative options were included in v2 and were discussed.

There was more support for option 2. Option 2 is agreed.

- To improve the clarity of eoi_pic_quality_idc, it is proposed to change the semantics of eoi_pic_quality_idc to an option originally proposed in JVET-AM0126 that specifically indicates whether the associated pictures are suitable for human viewing.

Current semantics are too vague and may not be useful.

Agreed.

- It is proposed to change the syntax related to extending the EOI SEI message to be identical to that used to extend the film grain characteristics and neural-network post-filter activation SEI messages.

Editorial bug-fix.

It is proposed to move the EOI SEI message extensions with the proposed changes from the VSEI TuC to a working draft of VSEI version 5.

JVET-AP0111 AHG9: On spatial quality optimization in EOI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes additional signaling for spatial quality optimization in the encoder optimization information (EOI) SEI message. The proposed aspects are as follows:

- Item 1: To indicate the type of spatial quality optimization

- Item 2: To indicate the component(s) affected by spatial quality optimization

- Item 3: To indicate the impact level on spatial characteristics

In version 2, the name of the syntax proposed in Item 3 was incorrectly specified and has been corrected.

For item 1, it was suggested that the proposed semantics language is vague.

The existing semantics language doesn’t specify

For item 2, it was questioned why u(8) is used for bit masks for 3 colour components, and if the chroma components would be optimized separately.

For item 3, the quality indication is to describe pre-processing applied prior to encoding.

The decoder side impact of having this information is not clear.

JVET-AP0150 AHG9: Suggested Modifications for EOI SEI Message [Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)]

No need to present because of action on JVET-AP0067.

JVET-AP0152 AHG9: Extension of Temporal Extrapolation Support for EOI SEI Message [Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)]

The following changes are proposed to the extension of the EOI SEI message:

- Remove the constraint eoi_num_int_pics > 0 from the conditional statement if( !eoi_temporal_resampling_type_flag && eoi_num_int_pics > 0 ).

- Add eoi_max_num_int_pics_minus1 to indicate the maximum number of pictures that the encoding system excluded between each pair of coded pictures in output order.

v2 provides the updated semantics of eoi_temporal_extrapolation_dir_flag based on the above changes.

There are some editorial issues with derivation and naming of NumoPics.

JVET-AN0165 aspect 4 is related to proposal 1.

There was support for removing eoi_num_int_pics from the condition.

It is not clear how the proposed new syntax elements to indicate the max num of int pics would be used by the decoder.

Agreed to Proposal 1 on removing eoi_num_int_pics from the condition.

JVET-AP0165 AHG9: On temporal resampling information in EOI SEI message [J. Chen, Y. Ye (Alibaba)]

The EOI SEI message was extended in TuC by introducing temporal resampling information in last meeting. This contribution raised several issues in the current design of the temporal resampling information, and the following items are proposed to address them.

- Aspect 1: either impose a constraint on the value of eoi_temporal_extrapolation_flag and eoi_temporal_exrapolation_dir_flag such that both flags remain constant within one CLVS, or revise the current semantics to clarify the suggested decoder behavior when these two flags change value.

- Aspect 2: signal eoi_temporal_clvs_flag only when eoi_temporal_extrapolation_flag is equal to 1, and infer eoi_temporal_clvs_flag to be equal to 1 when eoi_temporal_extrapolation_flag is equal to 0.

- Aspect 3: Remove four constraints

- Aspect 4: Allow the signaling of temporal resampling information when eoi_num_int_pics is equal to 0.

For aspect 1, it was suggested that the proposed constraint is already effectively imposed by other constraints.

It was noted that there is a current constraint that two pictures are required for extrapolation which may not be appropriate. JVET-A0173 is related.

It was suggested that the proposed constraint in aspect 2 seems overlay strict, and could either be made less strict or not imposed.

Aspects 1, 2 were discussed in JVET-AP0292. See notes there.

Agreed to aspect 3 remove the four constraints.

For aspect 4, refer to notes in JVET-AP0152.

JVET-AP0166 AHG9: On depth information extension in EOI SEI message [J. Chen, Y. Ye (Alibaba)]

The EOI SEI message was extended in TuC to support multiple object depth ranges. This contribution identifies several issues in the current design, and proposes to address them in the following aspects:

- Instead of signaling the near value and far value for each depth range, the near value and the difference between the far value and the near value are signaled to ensure that the far value is greater than or equal to the near value.

- Add constraints on the near value and the difference value to ensure that the depth ranges do not overlap.

- Add the value ranges of the near value and the difference value.

- Add text to further clarify the interpretation of each depth range according to the different values of eoi_depth_range_anchor_mode_idc.

- Adjust the quantization parameter ranges used for each depth range to align with the semantics of eoi_pic_quant_object_flag

Agreed for item 1.

Regarding item 2, it was questioned if it was harmful if that constraint did not exist. No action on item 2.

For item 3, it was suggested that for the idc equal to 2, the upper limit of the range should be 2^16 – 1, rather than 2^16. Agreed to item 3 with that change.

For item 4, it was suggested that the proposed text doesn’t handle the special case of 0.

It was suggested that the current semantics are clear.

No action on item 4.

It was noted that the current text includes “belonging not belonging” which should just be “belonging”.

Agreed item 5, with the fix to the above issue for the new text and the similar text above it.

JVET-AP0172 AHG9: On temporal sublayer limitation in EOI SEI message [X. Xu, S. Liu (Tencent)]

See notes for JVET-AP0067.

JVET-AP0173 AHG9: On temporal extrapolation semantics in EOI SEI message [X. Xu, S. Liu (Tencent)]

Some issues of semantics descriptions in current VSEI TuC document (JVET-AO2032) for temporal extrapolation in the EOI SEI message are identified in this contribution. Suggested changes are provided to improve the clarity of semantics descriptions:

1: semantics of eoi_temporal_extrapolation_dir_flag w.r.t. defining the reference picture for applying extrapolation operation. In v2, more details and completed solution on the issue#1 are presented.

2: missing semantics when eoi_temporal_clvs_flag=0

Discussed initially along with JVET-AP0165 and later in JVET-AP0292. See notes there.

JVET-AP0292 AHG9: On temporal interpolation and extrapolation indications in the EOI SEI message [M. M. Hannuksela (Nokia), J. Chen (Alibaba), X. Xu (Tencent)] [late]

Chaired on 15:50-16:00 CEST on 29 April 2026 by S. Deshpande.

This contribution results from the offline work related to JVET-AP0165 and JVET-AP0173. The proposed changes to the semantics are summarized as follows:

- An informative note is added to clarify the intent of eoi_temporal_extrapolation_flag equal to 0.

- The semantics of eoi_temporal_extrapolation_dir_flag are changed to avoid the phrasing to extrapolate between each pair of consecutive pictures in output order.

- The semantics of eoi_temporal_clvs_flag are changed to exclude consideration of temporal interpolation and to only indicate temporal extrapolation before the first picture of the current CLVS or after the last picture of the current CLVS in output order.

No changes to the syntax are proposed in this contribution.

Regarding aspect 2, it was commented that the wording “using the current cropped decoded picture as input” only specifies using a single decoded picture. It was agreed that the intention is not to include only the current picture and so the wording needs to be changed. Additionally using motion or other extra information is not precluded and should be also clarified in updated text. This is delegated to the editors.

Agreed, with comments noted above delegated to the editors.

### Modality information (MI) SEI message (2)

Contributions in this area were discussed during 1930–1950 on Sunday 26 April 2026 (chaired by J. Boyce).

JVET-AP0095 AHG9: On the modality information SEI message extension [J. Xu, Y.-K. Wang (Bytedance)]

This document proposes the following items to the modality information SEI message extension:

- Specify a constraint to disallow signalling multiple spectrum information when the associated picture is of the 4:0:0 colour format.

- Add syntax elements specifying a separate colour description to describe the colour domain where multispectral pictures are generated.

For item 1, it was suggested to use an interface variable to determine the colour format.

Agreed to the constraint in item 1, with the interface variable.

For item 2, it isn’t clear that the proposed information is useful.

JVET-AP0121 AHG9: On MI SEI Extension to Indicate Sample Value Usage[ S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

Multispectral data may represent either display-oriented content or physical measurements used for machine processing, which cannot be inferred from the bitstream alone. This contribution proposes to TuC an extension to the MI SEI message to signal the intended interpretation of decoded sample values.

It was noted that the EOI SEI message already has indications similar to what is proposed here.

No action.

### Digitally signed content (DSC) SEI messages (5)

Contributions in this area were discussed during 1800–1950 on Monday 27 April 2026 and 0900-1030 on Tuesday 28 April 2028 (chaired by J. Boyce and S. Deshpande).

JVET-AP0136 AHG9: Fixes to subpicture signing for digitally signed content SEI messages [M. Pettersson, R. Sjöberg, M. Damghanian, S. Harribey (Ericsson)]

This contribution proposes the following fixes to digitally signed content (DSC) SEI messages for subpicture/region-based signing in the VSEI TuC:

- Remove dsci_num_separate_picture_segments_minus1 from the syntax table. This syntax element appears not to be used.

- Change dsci_picture_segment_mode_flag to dscs_picture_segment_mode_flag in the semantics for the digitally signed content selection (DSCS) SEI message.

- Change dscs_picture_segment_id[ i ][ j ] to dsci_picture_segment_id[ i ][ j ] in the semantics for the 	digitally signed content initialization (DSCI) SEI message.

- Infer the values of dsci_picture_segment_mode_flag and dscs_picture_segment_mode_flag to be equal to 0 when not present.

- Set the constraints for the value range of dsci_num_picture_segments_minus1[ i ] and dsci_picture_segment_id[ i ][ j ] only when they are present. Alternatively, infer these values to be equal to 0 when not present.

- Change from past tense to present tense in the semantics of dscs_skip_flag by changing “indicated” to “indicates”.

Agreed to item 1. item 2, item 3. item 4, item 5, item 6.

JVET-AP0142 AHG9: On subpicture-based DSC SEI messages and subpicture extraction [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

In this contribution, it is asserted that when a digitally signed bitstream is inputtet into subpicture extraction process, some DSC SEI messages (i.e., DSCI, DSCS, and DSCV SEI messages) that are present in the bitstream may become invalid / incorrect. These SEI messages pose a risk such that decoder may try to run verification based on them and would definitely get a negative verification result.

To overcome the asserted problem, the following two options are proposed:

Option 1:

The following solution is proposed for handling DSC SEI messages during subpicture extraction:

- For picture-based DSC SEI messages, remove them from the output bitstream

- For subpicture-based DSC SEI message, rewrite the SEI message to correct information that become invalid / incorrect due to the subpicture extraction process.

Option 2:

The following solution is proposed for handling DSC SEI messages during subpicture extraction:

- Remove from the output bitstream all picture-based DSC SEI messages and subpicture-based DSC SEI messages that become invalid / incorrect due to the subpicture extraction process.

It is noted that the SPS and PPS are not signed when subpicture signing mode is used.

It was suggested in the current TuC implementation that verification substream ID values must range from 0 to the number of verification substreams minus 1.

In subpicture signing, for a single DSCI ID, does the extraction process need to extract all of the subpictures, or can a subset of the subpictures be extracted?

It was noted that verification substreams don’t require any special extraction when for example a highest temporal layer was removed. It was questioned why a special process should be required for subpicture extraction.

Check if VVC subpicture sub-bitstream extraction process should be changed for the purpose of the signing. That would require a change to VVC. It was agreed that the VVC sub-bitstream extraction process should not be changed, and a sub-bitstream extraction process should not be added to VSEI.

It was suggested that a note could be added to the SEI message.

JVET-AP0164 AHG9: Miscellaneous aspects of digitally signed content SEI messages in TuC [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

In this contribution, it is asserted that when a digitally signed bitstream is inputtet into subpicture extraction process, some DSC SEI messages (i.e., DSCI, DSCS, and DSCV SEI messages) that are present in the bitstream may become invalid / incorrect. These SEI messages pose a risk such that decoder may try to run verification based on them and would definitely get a negative verification result.

To overcome the asserted problem, the following two options are proposed:

Option 1:

The following solution is proposed for handling DSC SEI messages during subpicture extraction:

–	For picture-based DSC SEI messages, remove them from the output bitstream

–	For subpicture-based DSC SEI message, rewrite the SEI message to correct information that become invalid / incorrect due to the subpicture extraction process.

Option 2:

The following solution is proposed for handling DSC SEI messages during subpicture extraction:

–	Remove from the output bitstream all picture-based DSC SEI messages and subpicture-based DSC SEI messages that become invalid / incorrect due to the subpicture extraction process.

3	On whole picture signing

–	Specify that there shall be at least one verification system that is associated with the whole picture signing when subpicture-based signing is enabled.

4	On dependency between verification substreams

–	Option 1: specify that if two verification substreams are associated with any of different subpictures, those two verification substresms shall be independent.

–	Option 2: specify that verification substreams that contain subpictures that belong to the same pictures shall have no dependency to each other.

5	On verification substreams sharing same subpicture identifier(s)

–	Specify a constraint such that when two substreams in a subpicture-based digitally signed content systems include one or more same verification substream identifiers, DSCS SEI messages associated with the same subpictures shall not be present in PUs of the same temporal sublayer.

Agreed to item 1.

Further study for item 2.

For item 3, the proposal is that if subpicture signing is used, there either must be two DSCI SEIs present, or subpicture signing of all subpictures in the picture. For the second option, the parameter sets are not signed, which may be a disadvantage.

The first option would be backwards compatible with VSEI v4.

It is noted that option 1 might be a problem for extraction.

A question was raised if we want to require signing of subpictures without also signing the entire picture. Although we had previously agreed that it should be required, it is asserted that there is no spec language that imposes that constraint. It was suggested that a simper constraint can be made. Further discussion requested.

For item 4, option 1 is stricter than option 2.

Another suggestion is to add a note about dependencies between substreams.

Further discussed 1630 on Wednesday 29 April 2026 chaired by J. Boyce.

In the v2 of this document, option 2 for item 3 was added.

It was suggested that an alternative is to add a note rather than a constraint.

Agreed to add a note based on option 1 with the precise language delegated to the editors.

JVET-AP0188 AHG9: TuC DSC subpicture improvements [I. Sodagar (Dolby)]

Chaired by S. Deshpande from 09:10-10:10 CEST on 28 April 2026.

This contribution proposes a few improvements to the TuC subpicture DSC tool.

To analyse the TuC, the current solution in TuC and the proposed solution in AO0197 are compared in terms of functionality and efficiency, and then few tools are proposed to improve TuC bandwidth efficiency while maintaining its functionality.

The main goal of the proposal 1/2/3 is bit-rate efficiency.

Proposal 1, 2, and 3 are alternatives to each other.

Proposal 1: proposes one new flag

Proposal 2: has only semantics/ process change with no new syntax

Proposal 3: proposes two new syntax elements

Proposal 4.6: was related to JVET-AP0137 and was addressed in there. See notes there.

It was commented that if bit-savings is important, previously before unification, we had some variants which were also trying to be bit efficient and that in that case the method should be compared with those approach(es). It was answered that those approaches don’t contradict this and may be able to be combined.

This proposal is related to cross-PU aspect (proposal 3) of JVET-AP0231 and so they were discussed together. See notes under JVET-AP0231 proposal 3.

JVET-AP0231 AHG9: On DSC SEI support for subpictures [J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande from 09:28-10:30 CEST on 28 April 2026.

Several proposals are made to modify the design of the DSC SEI messages in the TuC in JVET-AO2032 for support of signaling of segments, which represent subpictures.

Proposal 1 addresses an asserted problem with backwards compatibility of the TuC design with VSEI v4 because SEI message extensions are ignored.

Multiple participants supported this. It was commented that this should be explained in a note in the specification to help v4 decoders.

Agreed (also add a note to the specification).

Proposal 2 allows a single DSCI SEI message be used for signing of both full pictures and segments.

It was commented by one participant that similar functionality is already supported. A similar proposal was presented in the last meeting and this has some modifications.

Proposal 3 reduces bitrate by enabling use of implicit mode and reducing the required signaling of DSCS SEI messages.

This is related to JVET-AP0188. The nalu pattern is signalled at DSCI level such that it does not need to be signaled in DSCS. The proposal does include change to DSCS as well. In JVET-AP0231 no change is proposed to DSCI.

This proposal is designed with primary assumption of picture signing (not subpicture signing).

Subpicture pattern can only change at CLVS boundary and there will be a DSCI there.

Verification period can not contain multiple CLVSes.

But you could have multiple slices in a subpicture and each slice could be in its own nal unit. However this is not very common.

Discussion regarding referencing DSCI versus previous AU:

The approach in JVET-AP0188 requires sending information at least one time in a DSCS compared to this proposal which can signal it only once in DSCI.

One other difference is JVET-AP0188 also handled non-VCL NAL units, but as proposed JVET-AP0231 only applies to VCL nal units. It was commented that for JVET-AP0231 we should not add non VCL nal units.

It was commented that in the repeat method design of JVET-AP0188 you could not easily add SEIs (or you have to insert another DSCS for the new pattern).

In this proposal also some existing signaling in DSCI is removed. It was commented that the removal may have the issue of not easily knowing at the receiver which subpictures are signed. It was commented that the removal is not require and that it was an oversight. Overall discussion (of JVET-AP0188 and JVET-AP0231 proposal 3):

Does bit-rate efficiency aspect needs to be handled? It was commented by multiple participants that if this can be done cleanly, for most common use cases and without losing any functionality that will be useful.

Discussed again after offline discussion. See notes under JVET-AP0293

JVET-AP0293 AHG9: TuC DSC picture segment improvements [I. Sodagar, J. Boyce, M. M. Hannuksela (Nokia)] [late]

Chaired by S. Deshpande from 1925-1945 on 29 April 2026.

This contribution proposes an improvements to the TuC subpicture DSC tool to increase its bandwidth efficiency.

Syntax included is mainly from JVET-AP0231, the process is based on JVET-AP0188 aspect 3 and JVET-AP0231.

It was commented that “for dscs_verficiation_substream_id equal to subIdVal” should be added.

A concern was raised to add this in AU scope and about if removal of the layers works. Further study on this aspect.

Agreed to add to TuC.

### Packed regions information (PRI) SEI message (1)

Contributions in this area were discussed during 1950–2000 on Sunday 26 April 2026 (chaired by J. Boyce).

JVET-AP0102 AHG9: On the PRI SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes to add the target picture colour description in the PRI SEI extension.

Other options proposed were to add a constraint disallowing different colour descriptions in the relevant layers or to add a note that warns encoders about the issue.

Agreed to add a note, with the specific language delegated to the editors.

### Film grain regions characteristics (FGRC) SEI message (5)

Contributions in this area were discussed during 1630–1730 on Tuesday 28 April 2026 (chaired by J. Boyce).

JVET-AP0055 AHG9: Editorial changes for the FGRC SEI message [Y.-K. Wang, J. Xu (Bytedance)]

- Use a global variable instead of a local variable when it is defined in the semantics and used in the syntax table (e.g., nbFgrModels ==> NumFgrModels).

- Use two spaces before and after two-symbol operators (e.g., "<=") and one space before and after one-symbol operators (e.g., "<").

- Remove spaces that should not be there, e.g., in "if (" and add missing spaces, e.g., like in "if(x_flag)".

- Make descriptions complete (e.g., add to second sentence in the semantics of the FGRC SEI message to mention that the SEI message also provides information on how to apply decoded alpha planes to the simulated film grain values).

- Try to use "specifies" and "indicates" as correctly as possible.

- For the semantics of a flag, write the semantics of the flag equal to 1 first, followed by the semantics of the flag equal to 0.

- Describe a syntax element as "may be" present instead of "is present" if the syntax element is sometimes present and sometimes not present under certain condition.

- Use the correct minor sign ('−'), which can be either copied and pasted from a right one, or by typing "2212" followed by typing the "Alt+X" combination simultaneously.

- Place the semantics of syntax elements in the order the syntax elements appear in the syntax table (e.g., the semantics of fgr_alpha_channel_adaptation_flag, fgr_alpha_layer_idx, and fgr_film_grain_enabled_flag[ i ] were not in the right order).

- If a syntax element or variable is actually a flag, then reflect that in its name (e.g., alphaBlending ==> alphaBlendingFlag).

- Avoid the name of one syntax element or variable to be the subset of the name of another syntax element or variable (e.g., fgrModelId and fgrModelidValue, which was the reason of the change for fgrModelidValue ==> fgrLocalModelId).

- Avoid specifying a value range constraint when that constraint is already implied by the coding method for the syntax element, e.g., 0..255 for the u(8)-coded fgr_active_regions_number_minus1.

- Try to fix various other minor editorial issues, like font issues (bold or not bold, font size), typos, etc.

Delegate to editors.

JVET-AP0056 AHG9: Bug fixes for the FGRC SEI message [Y.-K. Wang, J. Xu, K. Zhang (Bytedance)]

This contribution reports some asserted bugs for the film grain regions characteristics (FGRC) SEI message in JVET-AO2032-v2 and proposes changes for fixing those asserted bugs.

Below is a summary of the asserted bugs:

- Use of fgr_comp_model_present_flag[ ][ ] in a syntax condition

- Use of fgr_comp_model_value[ ][ ][ ][ ] in Equations 28, 30, and 33 and in NOTE 8

- Use of fgr_num_intensity_intervals_minus1[ ][ ] in a value range specification

- Use of fgr_num_intensity_intervals_minus1[ ][ ] for deriving intensityIntervalMax[ c ][ x ][ y ]

- On a superfluous square bracket in Equation 27

- On the regional film grain synthesis operation

Further discussion requested.

Further discussed at 1700 on Wednesday 29 Apri 2026 chaired by J. Boyce.

Agreed on item 1,2, 3, 4, 5, item 6 (exact wording delegated to editors.)

JVET-AP0057 AHG9: Some potential improvements for the FGRC SEI message [Y.-K. Wang, J. Xu (Bytedance)]

This contribution proposes some potential improvements for the film grain regions characteristics (FGRC) SEI message in JVET-AO2032-v2. Besides proposed changes, this contribution also raise some open discussions on some aspects of the FGRC SEI message.

Below is a summary of the proposed changes and the open discussions:

- On signalling of fgr_region_interval_max[ i ][ c ]

- On a constraint requiring the presence of the four VUI-type FGC parameters

- On interface text: inference of FGC picture width and height for VVC

- On one presence flag for FGC picture width and height and bit depth syntax elements

- On film grain parameters for chroma components

- Disallow any two explicitly indicated regions to be the same

- Disallow not using an entire set of modelling parameters for a film grain model

- Disallow not using the modelling parameters for any intensity interval

- Open discussion 1: Apply to decoded picture or cropped decoded picture?

- Open discussion 2: Use InPic or (cropped) decoded picture in the semantics?

It was questioned how important it is to align FGC with FGRC. It is noted that they have already drifted, and it was suggested that we can deviated.

Agreed to items 1, 2, 5. 7

Further discussion requested on item 3, 4.

For item 5, it was questioned if ChromaFormatIdc can be used instead of the proposed three components flag.

No action on item 6. 8

For item 9, it was agreed to apply to cropped decoded picture. Text is delegated to editors.

For item 10, it was agreed to use InPic in the semantics. Text is delegated to editors.

Further discussed at 1710 on Wednesday 29 April chaired by J. Boyce.

Further study on item 3.

Agreed on item 4 with the if condition moved to be the first set of syntax elements in the syntax table after the cancel flag.

JVET-AP0061 AHG9/AHG13: On Film Grain Regions Characteristics SEI message [S. Deshpande, J. Samuelsson-Allendes (Sharp)]

This contribution proposes modifications to film grain regions characteristics (FGRC) SEI message, which is included in the Technologies under consideration for future extensions of VSEI. Following is proposed:

- Proposal 1: It is proposed to signal the flag fgr_film_grain_enabled_flag[ i ] only when fgr_active_regions_number_minus1 is greater than 0 and infer its value otherwise.

- Proposal 2: It is proposed to normatively specify the processing logic for applying film grain at a sample location which falls within multiple regions.

- Proposal 3: An asserted simplification is proposed to specify the for loop which signals the film grain models using a ternary conditional operator instead of using a derived variable.

Agreed to proposal 1, proposal 2.

Proposal 3 is delegated to the editors.

JVET-AP0062 AHG9: On the FGRC SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

The v3 version proposes semantics update in which fgr_film_grain_enabled_flag[ i ] may be retained for certain use cases.

A constraint is imposed that at least one region have film grain specified.

It was proposed to rename  the fgr_active_regions_number_minus1 to fgr_regions_number_minus1.

Agreed to option 2 to impose the constraint, and to reword to remove “active” in the syntax and semantics.

### Constituent rectangles (CR) SEI message (2+1)

Contributions in this area were discussed during 1730–1750 on XXday 2X April 2026 (chaired by J. Boyce).

JVET-AP0092 AHG9: On redundant bits related to string signalling in the CR and NNPFA SEI messages in VSEI TuC [J. Xu, Y.-K. Wang (Bytedance)]

Also in 6.2.4

It was suggested to move the while (!byte aligned) syntax to be under the if condition. Agreed to this suggestion.

JVET-AP0099 AHG9: On the CR SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes the following changes to the CR SEI message in TuC.

- Avoid signaling sub-picture index, or rectangle position and size for an empty rectangle

- Propose constraint to avoid an empty group

- Propose text description for CR group

Agreed to item 1, 3.

The proposed text for item 2 is vague and unclear.

JVET-AP0159 AHG9: Miscellaneous aspects of constituent rectangles and enhanced colour format information SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

1	On signaling of subpicture index

–	Specify that the value of cr_subpic_idx[ ][ ] shall be less than the number of subpictures in the lIdx-th layer.

Agreed to item 1.

### Quality metrics (QM) SEI message (5)

Contributions in this area were discussed during 1840–2000 on Tuesday 28 April 2026 (chaired by J. Boyce).

JVET-AP0088 AHG9: On the QM SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes two alternative options to track the quality metric over multiple encoding stages or provide the source video quality for the decoder to assess the encoding quality indicated by the QM SEI message.

- Option 1, add text description to track the quality metric

- Option 2, signal the source CLVS or picture quality metric value of the current encoding system

v2 proposes to add sign syntax element to support negative metric gain value.

v3 modified the syntax elements proposed in Option 2

For option 1, it was noted that a text description is difficult to interpret.

For option 2, the proposed syntax allows signaling a source metric in addition to the quality metric of the coding. Both of these metrics are of the same metric type.

JVET-AP0179 is also related.

Further discussion requested on item 1.

Item 2 proposes adding a negative sign for the metric gain value.

Agreed to item 2.

JVET-AP0174 is related.

See notes under JVET-AP0139.

JVET-AP0135 AHG9: On circular reference in the QM SEI message [C. H. Demarty, A. Ak, R. James (InterDigital)]

It is asserted that the current specification of the Quality Metrics (QM) SEI message introduces a circular reference between the semantics of the syntax element qm_pic_metric_value[ i ][ c ] and the process described in subclause 8.42.2.1. In particular, subclause 8.42.2.1 refers to a derivation process that is itself defined in the same clause, resulting in an ambiguous and self-referential definition.

This contribution proposes to resolve this issue by making subclause 8.42.2.1 self-contained, such that all elements required for the derivation of picMetricValue[ i ][ c ] are explicitly defined without circular dependency.

In addition, minor editorial refinements are proposed to improve clarity and consistency of the text.

There is a problem with circular reference in the text.

The following text was suggested: “The value of picMetricValue[ i ][ 0 ] and, when  qm_three_component_flag[ i ] is equal to 1, the values of picMetricValue[ i ][ 1 ] and picMetricValue[ i ][ 2 ] are derived by a user-defined process.”

Further discussion requested.

In v2, a modified text as suggested by the group is proposed.

Agreed to the v2 version.

JVET-AP0139 AHG9: On the QM SEI message [C. H. Demarty, A. Ak, R. James (InterDigital), Yong He, Shuai Zhao, Louis Kerofsky, Marta Karczewicz (Qualcomm), Ying Gao, Ping Wu, Shaowei Xie, Shengjuan Wang, Yaxian Bai (ZTE Corporation)]

The current Quality Metrics (QM) SEI message enables signalling of quality metric values associated with a given encoding or processing step. In the context of concatenated encoding pipelines, such values may not accurately reflect the final quality when the input to the current step is already degraded.

This contribution proposes a simplified mechanism to signal source video quality within the QM SEI message. A single syntax element is introduced to indicate whether a metric value corresponds to the decoded picture quality or to the source quality of the content entering the concatenated encoding pipeline.

Corresponding semantic updates are proposed to define the interpretation and derivation of source quality metric values while maintaining consistency with the existing QM SEI framework.

In v2, some editorial changes were applied to the semantics.

Discussion about what the source represents, especially with multiple transcoding stages. The proponent says that it represents the source at the beginning of the transcoding chain.

The proponent indicated that if a transcoder’s input bitstream contained a QM SEI message, it would be removed, but its coded picture quality metric values would be inserted into the new bitstream’s source quality metric values.

Further discussion requested.

Further discussed 1715 on Wednesday 29 April 2026.

A v3 version was uploaded with new authors.

It was commented that the original source picture needs to be properly defined.

Agreed to add to TuC.

It was commented that there is text describing the transcoding operation and it should be considered whether that text is normative or informative and how to best phrase it. Further study on that aspect.

JVET-AP0154 AHG9: On the quality metrics SEI message [Y. Gao, P. Wu, S. Xie, S. Wang, Y. Bai (ZTE)]

This contribution presents a simplified syntax framework evolved from JVET-AO0116 for signaling source image quality information in the quality metrics (QM) SEI message.

Version 2 highlights the changes based on TuC(JVET-AO2032), with necessary textual revisions while keeping the essential content unchanged.

Is similar to JVET-AP0139 but with different signaling method.

See notes under JVET-AP0139.

JVET-AP0174 AHG9: On quality metrics SEI message [X. Xu, S. Wenger, S. Liu (Tencent)]

In current VSEI TuC document (JVET-AO2032) the following action items are proposed or identified in this contribution for QM SEI message.

1.	Add support forThe representation of perfect image quality is undefined for some metrics, such as PSNR

2.	Add support for Nnegative quality metric values cannot be expressed in current design.

3.	Assign Default default values for syntax elements related to QM value ranges are missing

4.	Assign IncreasingFlag[ * ] value when qm_metric_increasing_flag[ * ] is present.

In v2, issue#2 is adjusted to include a sign bit for QM values when negative values may be present; issues #4 is added to fixed a missing value assignment in the derivation process.

In v3, the location of qm_perfect_value_flag[ i ] syntax element is adjusted.For item 2, It was questioned if there was a future extensibility issue. It would likely be beneficial to be able to add additional metric types in a future version of the standard for which a v5 decoder would ignore those metrics but not have to discard the entire messages, so that use can use the metrics of types that are defined in v5.

It was suggested that the interpretation of a new metric could enable interpretation of the metric even for negative values, without requiring any change to the syntax.

No action on item 2.

For item 1, it was questioned why it is necessary to signal the perfect image quality, especially for known metrics, for which the definition could just be included in the specification of that metric type.

For item 4, a bug was identified that the IncreasingFlag[ ] variable is used but sometimes not defined.  Agreed to add the following semantics: “When qm_metric_increasing_flag[ i ] is present, the value of IncreasingFlag[qm_metric_type[ i ] ] is set equal to qm_metric_increasing_flag[ i ].”

### Lens optical correction (LOC) SEI message (1)

Contributions in this area were discussed during 1750–1755 on Tuesday 28 April 2026 (chaired by J. Boyce).

JVET-AP0075 AHG9: Proposed Lens Optical Correction SEI message for version 5 of VSEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou, S. Wenger (Tencent)]

This contribution proposes adding the lens optical correction (LOC) SEI message into the working draft of Version 5 of VSEI.  The LOC SEI message is meant to be available for HEVC and VVC.

Software implementing the LOC SEI message is available and will be provided for integration with the VTM if this message is adopted into a VSEIv5 draft.

### Display overlays information (DOI) SEI message (6)

Contributions in this area were discussed during 1755–XXXX on Tuesday 28 April 2026 (chaired by J. Boyce).

JVET-AP0103 AHG9: On the DOI SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes the following changes to the DOI SEI message in TuC.

1) Constrain DOI set to avoid an empty DOI set

2) Add value range of doi_num_sets_minus1

3) Condition target picture relevant syntax elements

4) Bug fix on the doi_target_init_present_flag signaling condition

5) Infer the target picture initial sample value to be equal to the sample of the first display overlay.

6) Remove one DOI constraint

No action on item 1.

Agreed Item 2 add a value range on doi_num_sets_minus1 of 0 to 15.

For item 3, JVET-AP0158 items 2 and 3 are related.

No action on item 4, 5.

Agreed item 6 to remove the constraint.

JVET-AP0104 AHG9: On the DOI alpha component [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes the following changes to the DOI alpha component in TuC.

1) doi_target_pic_alpha_flag semantic update

2) target picture derivation process update

3) condition the alpha partition position signaling

No action on item 1.

Further study on item 2.

On item 3, it was suggested to move the signaling of the alpha partition top left x and y to be under the !same size flag, and to rename the same size flag. Agreed as suggested.

JVET-AP0114 AHG9: On miscellaneous aspects of DOI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0115 AHG9: On viewer advisory overlay in the DOI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0131 AHG9: On display overlays information SEI message [K. Abe, T. Nishi (Panasonic)]

JVET-AP0158 AHG9: On signaling of target picture in display overlays information SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

### Enhanced colour format information (ECFI) SEI message (4+1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0245 in 4.3 is also related.

JVET-AP0106 AHG9: On signalling packed channels in ECFI SEI message [H. Tan, C. Kim, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0116 AHG9: On single-layer and chroma format support in the ECFI SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0159 AHG9: Miscellaneous aspects of constituent rectangles and enhanced colour format information SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Also in 6.2.11

JVET-AP0284 Issues with the Bitstream Conformance Requirements Added to the ECFI SEI Message [A. M. Tourapis, D. Podborski (Apple Inc)] [late]

JVET-AP0289 Editorial and Technical Issues in the ECFI SEI Message [A. M. Tourapis, D. Podborski (Apple Inc)] [late]

### Photosensitive content information (PCI) SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0129 AHG9: On PCI SEI [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande on 17:40-17:55 on 29 April 2026.

This contribution proposes adding new information in the photosensitive content information SEI to support post-processing use-cases preventing photosensitive epilepsy. The proposal includes support for explicit signaling of parameters used in the referenced PSE guidelines, enabling usage of the SEI by non-referenced guidelines (e.g. Japan broadcast system) and providing information for optimal post-processing.

It was asked about if peak luminance level will depend on display brightness and in that case how it can be known. The proponent commented that semantics may need to be improved for this. It was also commented that the syntax element signals a difference. It was asked if this should be a % instead.

It was asked in POC information is needed or the usual SEI mechanism of sending it when it starts and cancelling when it ends is sufficient.

Agreed to add without the 2 POC syntax element.

Further study about the aspect noted above for % luminance difference.

### Display rectangles (DR) SEI message (3)

Contributions in this area were discussed during 1800–1845 on Wednesday 28 April 2026 (chaired by J. Boyce and S. Deshpande).

JVET-AP0119 AHG9: On display rectangles SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

Chaired by S. Deshpande on 18:09-18:08 on 29 April 2026.

This contribution proposes the following changes for the display rectangles (DR) SEI message:

- Support resampling of cropped decoded picture for each display rectangle

- Add duplication filling method

Item 1 is related to JVET-AP0126.

It was commented by multiple participants that the resizing can happen in the display depending upon the actual display size without needing additional signaling.

It was commented that additional signaling may result in putting some requirement on the receiver to do resampling.

It was pointed out that we also have aspect ratio signalling in VUI and it was asked how that is handled.  This question applies to DR not specifically to this proposal. It was commented by a participant that encoder should account for this. Editors should consider adding a note about this in relation to VUI.

The assertion of the proposal is the equations don’t seem correct as they do sample by sample copy.

It was commented that software for this SEI is available.

Revisit after offline discussion on this equations aspect.

About item 2:

Additional filling methods of duplication proposed. The new signaling involves number of duplications and position signaling. Filling process is also changed.

It was asked if this aspect of duplication is orthogonal and why is it in this SEI. It was asked how popular this is.

JVET-AP0126 AHG9: On DR SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande on 17:55-18:08 on 29 April 2026.

The DR SEI provides receivers information on how to display the cropped decoded pictures on displays with different display aspect ratios, providing cropping and filling information for one or several display aspect ratios. When a receiver’s display aspect ratio differs from the aspect ratios described in the SEI, then it is unspecified what the receiver should do. The receiver may select a DR with a described aspect ratio close to its display aspect ratio, but this may lead to poor QoE. This contribution proposed to add optional signalling to support better this case.

JVET-AP0119 item 1 is related and proposes to use resizing instead of cropping.

It was asked if this is more an implementation aspect/ issue.

It was asked if any chroma sampling also needs to be different. e.g. if position will get affected depending upon 4:2:0. A clear specification would be useful. This applies to any rectangles signalled.

Multiple key points may be needed instead of a single one. But is not clear how to handle them. This may require further study.

It was asked if specification about how to use the signalled POI value should be used may need to be specified. The proponent thought this level of detail is not needed.

It was suggested to understand the point of referene for POI signalling:

“specifies the x and y coordinates of the point of interest of the cropped decoded picture.”

Further study.

JVET-AP0155 AHG9: On the display rectangles SEI message [S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)]

This contribution proposes improvements to the syntax and the semantics of the display rectangles SEI (DR SEI) in the VSEI TuC draft of JVET-AO2032. The proposed modifications are as follows:

Item 1: Fix and align syntax element names:

dr_display_aspect_ratio[ i ]  >>>  dr_display_aspect_ratio_idc[ i ]

Item 2: Fix bugs and correct some other typos:

Let Blur(OutPic, InPic, x0, y0, w0, h0, sigma0 ) be specified as ......   >>>  Let Blur(OutPic, InPic, x0, y0, w0, h0, DrKernelSize, sigma0 ) be specified as ......

Item 1 is delegated to editors.

Item 2 is agreed.

### Picture segmentation information (PSI) SEI message (1)

Contributions in this area were discussed during 1850–1855 on Wednesday 29 April 2026 (chaired by J. Boyce).

JVET-AP0245 in 4.3 is also related.

JVET-AP0089 AHG9: On the PSI SEI message [Y. He, S. Zhao, L. Kerofsky, M. Karczewicz (Qualcomm)]

- Define the codec interface variables

- Replace syntax element, psi_value_bit_depth_minus8, with variable BitDepthY

- Add psi_persistence_flag

Agreed to item 1.

No action on item 2 as it has a parsing dependency problem.

No action.

### Danmu information (DI) SEI message (6)

Contributions in this area were discussed during 1855–2000 on Wednesday 29 April 2026 (chaired by J. Boyce).

JVET-AP0097 in 6.5 is also related.

JVET-AP0117 AHG9: On danmu information SEI messages[ C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes changes that are asserted to improve various aspects for danmu information (DI) SEI message. The proposed items include:

- Item 1: Clarification of presentation precedence

- Item 2: Optional signaling of presentation priority

- Item 3: Signaling of semantic characteristics of danmu

- Item 4: Clarification of the semantic of di_default_params_present_flag

- Item 5: On the derivation of danmu start time

It was suggested that it isn’t needed to instruct the decoder to use start time to determine the display.

No action on item 1.

Agreed to item 2.

No action on item 3.

Item 4 is delegated to editors.

For item 5, it was questioned if pruning of Danmus would occur, which the proposed change would disallow

It was questioned if Danmus would be ordered by display start time. It would place a burden on the encoder to sort them to guarantee the order.

No action.

JVET-AP0127 AHG9: On Danmu SEI [T. Biatek, S. He, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AP0132 AHG9: On Danmu Information SEI [S. He, T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande on 19:45-20:00 on 29 April 2026.

Popular applications supporting danmus have a feature to merge multiple danmus into one, in order to optimize the display. It is proposed to add support for danmu replication to the danmu information SEI message.

A di_replication_enabled_flag is proposed and additional syntax elements are proposed based on this flag.

The flag allows encoder flexibility to enable this feature or not.

It was commented that for the proposed replication, the attributes should still allowed to be signalled. So replication should be only for the text and the language.

Generally supportive comments were made by mtuple participants, but some syntax changes were suggested.

Revisit after offline discussion.

JVET-AP0151 AHG9: On the default parameters in the DI SEI message [S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)]

JVET-AP0153 AHG9: Enable the use of segmentation planes with the DI SEI message [S. Xie, P. Wu, Y. Gao, W. Niu, Y. Bai (ZTE)]

### Colour mapping information (CMI) SEI message (4)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0071 AHG9: On the colour mapping information SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AP0096 AHG9: On the colour mapping information SEI message [J. Xu, Y.-K. Wang, K. Zhang (Bytedance)]

JVET-AP0122 AHG9: On CMI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AP0128 AHG9: On CMI SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

### Auxiliary sampling information SEI message (1)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0148 AHG9: On Auxiliary sampling alignment information SEI [S. Wang, Y. Gao, P. Wu, S. Xie, Y. Bai (ZTE)]

### Localization and mapping (LAM) SEI message (5)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0074 AHG9: Proposed Localization and Mapping SEI message for version 5 of VSEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou, S. Wenger (Tencent)]

JVET-AP0093 AHG9: On group cancellation for the localization and mapping SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AP0112 AHG9: On miscellaneous aspects of LAM SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0157 AHG9: On localization and mapping SEI message with multiple cameras [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AP0204 AHG9: Rotation precision in LAM SEI message [Y. Sanchez, R. Skupin, P. Helle, T. Hinz, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

### Sample interleaving SEI message (4)

Contributions in this area were discussed during XXXX–XXXX on XXday 2X April 2026 (chaired by XXX).

JVET-AP0077 AHG9: On Sample Interleaving [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

See the notes under JVET-AP0287.

JVET-AP0134 AHG9: On the Sample Interleaving SEI message [C. H. Demarty, A. Ak, R. James (InterDigital)]

See the notes under JVET-AP0287.

JVET-AP0193 AHG9: Syntax for the sample interleaving SEI message [J. Boyce, S. He, T. Biatek, M. M. Hannuksela (Nokia)]

See the notes under JVET-AP0287.

JVET-AP0287 AHG9: Unification of Sample Interleaving proposals [L. Kerofsky (Qualcomm), C. H. Demarty (InterDigital), J. Boyce (Nokia)] [late]

Chaired by S. Deshpande from 10:50-11:40 CEST on 28 April 2026.

Extensions to the Sample Interleaving SEI message are proposed as harmonization with AP0077 (Qualcomm), AP0134 (Interdigital) and AP0193 (Nokia) on syntax and semantics for Sample Interleaving SEI message.

From JVET-AP0077:

Extensions to the Sample Interleaving SEI message are proposed.  In the current TuC description, various values are inferred, and it is proposed to increase application of the Sample Interleaving SEI message by signalling various parameters rather than inferring a fixed set of values.

- Signal Color Filter Array size CFA_width and CFA_height

- Signal flag to indicate if decoded samples should be interleaved spatially for the CFA sampling pattern

- Signal flag to indicate sample range is remapped and additional parameters are signalled

  - Expanded bit depth

  - Expanded low value

  - Expanded high value

  - Flag for presence of nonlinearity

  - If present, signal a value for the nonlinearity

Software implementing the SI SEI message and these extensions can be made available.

From JVET-AP0134:

si_interleaving_flag[ i ] indicates whether re-interleaving is required prior to demosaicing or other post-processing operations.

From JVET-AP0193:

Syntax is proposed for the sample interleaving SEI message in the TuC to enable support for additional colour filter formats such as quad bayer and nonacell, as well as multispectral colour filters which represent more colours. The proposed syntax enables optional indication of the colour for locations in the decoded picture, to describe varying offsets of Bayer-type filters or to describe colours of multispectral colour filters.

Currrently the SEI message in TuC is empty with no syntax elements and some inference.

The harmonized version includes items from individual proposals with some overlap handled by using one option.

It was commented that camera manufacturers may use their own proprietary formats.

This proposed SEI includes syntax elements which are Bayer pattern related and some syntax elements which are compression related.

Regarding Bayer patter related syntax: Height, width, si_colour_params_present_flag and syntax conditioned on it  (basically the green highlighted syntax elements at the top of the SEI message) had support to be included. These had overlap in multiple proposals and were also inferred in current TuC and were in the first proposals.  Agreed to include those.

Further study the other items.

It was also agreed to change the name of the SEI. Delegated that exact name to use to the editors.

## Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (6)

Contributions in this area were discussed during 1140–XXXX on Tuesday 28 April 2026 (chaired by J. Boyce) and 01430-1300 on Wednesday 28 April.

JVET-AP0078 AHG9: Support of Infrared Auxiliary Picture (AUX_IR) and IR Information SEI for Sample Interpretation [S. Zhao, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)]

This contribution proposes enhancements to support consistent interpretation of infrared (IR) measurement domain data in video bitstreams. It considers three options:

- Propose an AUX_IR auxiliary picture type and an IR Information SEI for IR imagery carried as an auxiliary picture associated with a primary picture,

- Extending the Modality Information (MI) SEI to support interpretation of IR imagery only at the picture level, and

- Adopting both option 1 and option 2 to cover both use cases.

A use case was mentioned where a camera would simultaneously capture normal video plus infrared data.  The normal video picture and infrared picture might be of different resolutions. It was noted that the ASAI SEI message could potentially be used to align the two pictures.

In option 1, the SDI SEI is used to associate the infrared picture with the primary picture.

It was questioned if a CICP codepoint would be needed, similar to alpha and depth.

It was questioned why it is necessary to signal the temperature scale rather than just using Kelvin.

A scale and offset are proposed to be signaled. It was questioned if a linear mapping is appropriate.

It would be helpful to have some sample pictures to understand the what the data is. It is suggested to provide some coded pictures using the linear scaling method.

It was suggested that we have other messages that can indicate mapping.

It is probably undesirable to both add a new SEI message and extend an existing message if it is possible to avoid.

Further study.

JVET-AP0084 AHG9: Support of Gain Map Auxiliary Picture Type and Gain Map SEI Message [S. Zhao, L. Kerofsky, Y. He, M. Karczewicz (Qualcomm)]

This document proposes a new auxiliary picture type for signaling Gain Map information associated with a primary video layer. The proposed auxiliary picture type carries a Gain Map that enables reconstruction or approximation of an alternate dynamic-range representation from a baseline primary picture representation.

The proposal to TuC introduces:

- A new SDI auxiliary picture type AUX_GAIN_MAP for Gain Map Aux picture type.

- A new Gain Map information SEI message (gain_map_info ) describing the interpretation of the auxiliary picture samples

  - V2: updated syntax table.

The -v2 version of the document includes a different syntax approach from the -v1 version. In the -v2 version the ISO 21496 data. The ISO spec uses only fixed length coding.It was noted that HEIF embeds the ISO 21496 data, for images.

It was suggested to be useful to have a gain map that works for video.

It was suggested that the proposed for loop should start with index 2 rather than 8.

It was noted that the green MPEG metadata can be used for attenuation, which is similar, which uses alpha.

It was noted the SMPTE 2094-50 provides an alternative approach that doesn’t require a gain map layer.

Agreed to add to TuC.

JVET-AP0085 AHG9: Subject Recognition Information (SRI) SEI message [Q. Zhang (BOE)]

This contribution proposes to define a new Subject Recognition Information (SRI) SEI message to TuC for carrying data that specify subject recognition information for one or more pictures which generated during video recording or post-production.

The main goal of the proposed SEI is to provide a signaling mechanism to enable lightweight display devices to perform subject-type-specific image quality enhancement functions and subject-type-specific fast video classification/search functions.

It was suggested that the number of possible object types would need to be large. Using 5 bits is very limiting.

It was questioned if there is an existing list of objects with ID numbers that is used in the object detection and recognition space. It was suggested that Resnet may have such a list.

It was suggested that other SEI messages might address the use case, such as annotated regions, segmentation, object mask information, text descriptions.

The use case does not require location.

For the intended use case, it is asserted to be complicated to process text because of multiple languages and unknown length. It would be easier to identify from an ID value rather than from text.

The suggested use case is to enhance the video based on the object type.

The proposed syntax can only identify one object type.  The current persistence and cancellation syntax without an ID precludes the possibility of using a separate SEI message for each object type.

Is there any standard that includes a list of object types?

JVET-AP0177 AHG9: Transcoding history information SEI message [X. Xu, S. Wenger, S. Liu (Tencent), Johan Pardo, Alexander Karabutov, Hongjie You, Atanas Boev, Johannes Sauer, Timofey Solovyev, Elena Alshina (Huawei)]

Not requested to be presented.

JVET-AP0179 AHG9: Historical Information SEI message [J. Pardo, A. Karabutov, H. You, A. Boev, J. Sauer, T. Solovyev, E. Alshina (Huawei), X. Xu, S. Liu (Tencent)]

Following CfE and CfP sequences selection we can see that user generated content (UGC) plays a bigger role into the elaboration of the next video codec standard. It is proposed that UGC sequences characteristics should be also considered in the VSEI tracks. One of these characteristics that is proposed is to add transcoding information.

Aspect 1 proposes nesting of TDI SEI messages.

Aspect 2 proposes nesting of QM SEI messages.

Aspect 3 proposes nesting of EOI SEI messages.

Aspect 4 proposes nesting of user data unregistered SEI messages.

Aspect 5 proposes nesting of vui_parameters.

Aspect 6 proposes digitally_signed_content_initialization

Aspect 7 proposes multiple nested SEI messages.

It is noted that the scalable nesting and PON messages apply to the current bitstream and not to transcoding history.

It was suggested that the proposed syntax is lacking the payload size which would be needed.

For a new transcoder stage, additional nested SEI messages would be added at the end of this SEI message.

For aspect 7, it might be better to code nb_of_transcoding_minus1 using fixed length coding instead of ue(v) to make it easier to append messages without having to shift. u(7) would be byte aligned.

It was suggested to use the VVC payload type for the indicator value.

It was suggested to use the approach used in PON for indication of payload type.

It was questioned if there is demonstrated benefit for providing historical information. Some proponents considered there to be benefit for quality metrics.

It was suggested that having transcoding information might be useful for debugging purposes.

Further study encouraged.

JVET-AP0197 AHG9: Shot summarization information SEI message [J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande from 14:35-15:05 CEST on 28 April 2026.

A shot summarization information SEI message is proposed to describe video “shots” to enable generation of a video summarization or highlights video. The description includes a shot ID, syntax to identify a representative picture (sometimes referred to as a “key frame”) and start and end pictures of the shot, and an optional shot highlight priority order,

Proponents commented that the main use case may be for archival. The highlights could be generated in a live scenario.

Is the SEI only at the beginning or at associated picture: it is flexible but intended to be at a representative picture (based on the flag) inside the shot.

It was commented that for high value content, providers may not provide such a flexible viewing information directly to the viewer. Also the currently proposed SEI may not directly enable streaming only parts of the video. Some verification from content providers or use case validation may be useful to decide what exactly may be useful to be signaled. It was also asked if file format may be a good location for this information. It was commented that for VVC the SEI messages could be collected in their own separate track compared to the VCL data. There may be “low profile” sports (e.g. high school or division 3 sports) which may benefit from this.

It was asked if HEVC scene information SEI is used in real world scenarios. And was commented that it was not in VVC/ current VSEI. That SEI is clarified to be for a different use case.

Further study.

JVET-AP0263 Subpicture support in HEVC [A. M. Tourapis, D. Podborski, J. Kim, S. Paluri (Apple)] [late]

## Usage of SEI messages for Gaussian splatting technology (14)

Contributions in this area were discussed during 0900–1330 on Sunday 26 April 2026 in the meeting of the Joint AHG on Gaussian splats (chaired by J. Jung, W. Husak, Y. Liao, G. Bang). See further details in the report JVET-AP0019. Follow-up discussion was conducted in a joint meeting on Monday 27 April 1600-1800 where the yellow highlighted aspects were agreed.

JVET-AP0079 AHG9: Triplane video-based implicit Gaussian splatting with a scaffold-based anchor point framework [B. Kathariya, D. Y. Lee, T. Huang, F. Pu, G. Su, P. Yin, G. J. Sullivan, S. Oh, W. Husak (Dolby)]

Agreed to include in TuC

JVET-AP0100 AHG9: Gaussian splatting information SEI message [Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent), J. Xu, Y.-K. Wang, L. Zhang (Bytedance), M. M. Hannuksela, J. Boyce (Nokia), Jihoon Do, Hahyun Lee, Gun Bang (ETRI)]

V3 of JVET-AP0100 was presented in the joint meeting on Monday. It integrates with various other proposals, which was not agreed. It should only be combined with the elements from JVET-0108, JVET-AP0194, JVET-AP0195, JVET-AP0225. This shall be provided as V4, also correcting the list of authors. Agreed to include in TuC.

JVET-AP0101 AHG9: GSI SEI message workflow and packing examples [Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent), J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

For infomation

JVET-AP0108 AHG9: On Gaussian Splatting Information SEI message [J. Do, H. Lee, G. Bang (ETRI)]

Revisit: Further clarification about action about this

JVET-AP0109 AHG9: Enhancements to the Gaussian Splatting Information SEI message [H. Lee, J. Kang, J. Do, G. Bang (ETRI)]

Was partially integrated into JVET-AP0100

JVET-AP0194 AHG9: Common picture format for Gaussian splats coding [J. Boyce, M. M. Hannuksela (Nokia)]

Agreed to be integrated with JVET-AP0100

JVET-AP0195 AHG9: Splatting information SEI with patches [J. Boyce, M. M. Hannuksela (Nokia)]

Agreed to be integrated with JVET-AP0100

JVET-AP0205 AHG9: On the GSI SEI message for spatial random access [Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent)]

Agreed to be included in TuC (separate from AP0100)

JVET-AP0213 AHG9: On implicit representations with the Gaussian splatting information SEI message [S. Lee, S. Sasse, Y. Berendsohn, Y. Sanchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (HHI)]

Agreed to be included in TuC

JVET-AP0256 Crosscheck of JVET-AP0213 (AHG9: On implicit representations with the Gaussian splatting information SEI message) [Y.-G. Kim, R. Koo, E.-S. Ry (SKKU)] [late]

Not presented in JAhG (not 	necessary as it confirms results)

JVET-AP0220 AHG9: Showcase for Real-time Decoding using Video-based Implicit Representations for 4DGS [S. Lee, S. Sasse, Y. Berendsohn, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (HHI)]

For information, demo shown in JAhG on Sunday

JVET-AP0223 AHG9: Software implementation and results of the GSI SEI message with HM, HM-SCM, and VTM [J. Ricard, G. Teniou, S. Wenger (Tencent)]

JVET-AP0291 AHG9: Crosscheck of JVET-AP0223/m76700 Software implementation and results of the GSI SEI message with HM, HM-SCM, and VTM [P. Rondao Alface, J. Boyce (Nokia)] [late]

Confirms HM results, and points out some possible needs for improvement.

JVET-AP0225 AHG9: Extended Gaussian splatting information SEI message with SH non-uniform quantization and 2D transforms [J. Ricard, G. Teniou, S. Wenger (Tencent)]

Revisit: To be integrated in JVET-AP0100, or first studied in EE?

JVET-AP0227 AHG9: Content-adaptive packing order optimization for 3D Gaussian splatting video coding [J. Ricard, G. Teniou, S. Wenger (Tencent)]

Encoder optimization information

JVET-AP0288 AHG9: On pruning information indication for GSC [Y. Hu, X. Ma (MLS Lab), Y. Xu, K. Yang (SJTU)] [late]

Revisit: Studied in EE?

CTC also to be issued as a JVET document after definition in joint meetings

## SEI software and showcases (2)

Contributions in this area were discussed during 1950–2000 on Monday 27 April 2026 (chaired by J. Boyce).

JVET-AP0130 in 4.3 and JVET-AP0220 in 6.4 are also related.

JVET-AP0097 AHG9: Implementation and showcase for the danmu information SEI message [J. Xu (Bytedance)] [late]

JVET-AP0146 AHG9: Software implementation of the Green SEI message and demonstration of the use of attenuation map information metadata for display adaptation [C. H. Demarty, N. Caramelli, F. Aumont (InterDigital)]

This contribution informs of the implementation of the carriage of the display attenuation maps as auxiliary pictures in the bitstream, and of their usage at the decoder side for display adaptation, with the goal of reducing the energy consumption of the display.

This contribution also proposes a demonstration of the encoding/decoding of a green MPEG bitstream and its rendering result on a TV.

In v2 version, the number of a related contribution was added.

In v3, some typos were corrected and a slide deck was added.

A demo will be shown on Wednesday at lunch time in room C2.

A software implementation is available.

## Non-SEI HLS aspects (0)

Section kept as template for future use.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General

### Plenary on Friday 24 April, 1645-1745

The following topic was discussed in this JVET plenary: Consideration of possible VSEI version 5

Questions to be considered:

- Urgency of defining new SEI messages in terms of market relevance

- Maturity of those SEI messages, criteria such as showcase, implementation, expectation on usage, etc.

- Possible need of normative behaviour of SEI message processing

- Possible relation with elements of new video standard

- Possible timeline

SEI messages proposed in this meeting for a possible version 5 (in JVET-AP0067, JVET-AP0073, JVET-AP0074, JVET-AP0075, JVET-AP0097, JVET-AP0196, plus some verbal proposals)

- Shutter interval extension

- Lens optical correction

- Localization and mapping

- Sample interleaving

- Constituent rectangles

- Quality metrics

- Display overlay info

- Display rectangles

- Colour mapping info

- Danmu info

- Photosensitive content info

- Etc. …

None of those appears to overly urgent in terms of market needs, a timeline of finalizing by April 2028 (when SG21 meets) appears sufficient. At this moment, no need is seen to issue a separate working draft. For achieving further technical maturity, keeping all messages in TuC might be sufficient. In the review, highest priority should be given to be able to get through all documents, and potentially put notes in the TuC where open issues are left (rather than resolving every detail).

It was requested to provide a table (or similar) about the current content of TuC until Monday

- New SEI message, or extension of existing one

- History: How long has it been in TuC

- What is the new functionality

- Assessment of technical maturity

- Availability and implementation, a possible need of additional processing (for extraction and output), possible need for normative processing

- Availability of showcase, justification by application case and usage foreseen.

After availability of this table, further discussion should be conducted at plenary level about the advantage for JVET of maintaining a separate working draft document. At this moment, considering the long timeline and the potential danger of cancellation, no official request, or “official” WD should be issued by WG 5.

A joint meeting with parent bodies should also be held to discuss this (including the plan for timeline).

### Plenary on Monday 27 April 1400-1600

The following topics were discussed in this JVET plenary:

- Status of documents: Only two documents (JVET-AP0254, JVET-AP0268) still missing

- Liaison communication (section 7.5) JPEG m76981 – E. Alshina will draft a response

- Scheduling for the remaining week was discussed (further detail on scheduling is recorded in section 2.12)

- Joint meetings (different zoom link, see calendar)

  - Joint meeting with VCEG/WG 4/7 on Gaussian Splat (Monday 1600-1800)

  - Joint meeting with VCEG/AG 5 on Verification/quality testing and metrics (Monday 1800-1930)

  - Joint meeting with VCEG/WG 2/AG 5 on next gen requirements and CfP Wednesday 1800-1900

- Review status from main track

  - All EE1/EE2 reviewed

    - Adoption of RDO based frame-level switching for E2E AI intra interface, two more possible adoptions from EE1 when training successful

    - No new EE2

    - Two possible adoptions from EE2, dependent on issuing a new ECM

  - It was decided to issue another version ECM20.0, including the two candidates from EE, but from the next meeting (as EE2 is discontinued) likely not to further continue ECM development-

  - Tool assessment and hardware complexity categories reviewed – no new complexity reporting template from this meeting

- Gaussian splat status

  - JAhG report?

  - Status of results and cross-checking?

  - Dependency with JEEs on GS video packing improvements, CTC, anchor definition

  - …

- Review status from HLS track and discussion on potential open issues

  - Roughly 40% reviewed so far.

  - As it is likely that the time will run out without being able to review all documents, the following option was suggested: Give the AHG the authority to conduct further review and improvements on the TuC during interim AHG meeting(s), before issuing version 12. In this context, no new contributions should be allowed at the interim meeting, unless explicitly requested by the AHG. Otherwise, review of contributions might need to be shifted to the next meeting.

  - Gaussian splat SEI (if included) and new proposals from 6.3 should be definitely reviewed during the meeting.

  - It was requested that more discussion about criteria for including SEI messages in VSEI would be performed – Wednesday plenary.

- Review of documents at plenary level: Documents in 4.1

### Plenary on Wednesday 29 April 1045-1230

The following topics were discussed in this JVET plenary:

- Ballot results on 23888-3.2 available in m76996 – no technical comments, two small editorial changes. Revisit: According to ISO management, a DoCR should be issued.

- Status of documents: JVET-AP0254 still missing (crosscheck on EE2-4.1)

- Scheduling

  - Announcement of Demo by K. Sühring

- JPEG liaison review (either end of this session or Thursday morning)

- Reports and issues from tracks

  - Main track

    - Finalization  of draft CfP

    - Revisits on EE1

    - CICP to be reviewed

  - HLS track:

    - Status/delivery of AVC next version

    - General status of review / new messages in particular (in total roughly 30% still to be reviewed)

    - Gain map SEI was recommended for inclusion, two other new ones still need review

    - Review of GS SEI messages (to be included in TuC)

    - ..

  - Criteria for progression of SEI messages were discussed (see under JVET-AP0290).

- Output document planning (section 10)

- Review of documents at plenary level: sections 4.13 (see notes in that section)

## MPEG information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 27 April 0900–1300, Wednesday 29 April 0900–1030, and Friday 1 May 1400–1600.

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings

### Joint sessions XXXX-XXXX XXday 2X April on Gaussian splat coding: MPEG WG 4 / Video, MPEG WG 5 / JVET, MPEG WG 7 3D Graphics, and VCEG (ITU-T Q6/21)

The joint sessions was chaired by Lu Yu (MPEG WG 4 Convenor), Jens-Rainer Ohm (JVET Chair), Gary J. Sullivan (SC 29 Chair & Q6/21 Rapporteur), and M. Preda (MPEG WG / Convenor).

The session descriptions are based on notes by XXX (add notes from LY and GJS). Notes taken by JRO on decisions about documents in section 6.4 (from the session on Monday 27 April) can also be found in that section.

…

### Joint session 1815-1935 Tuesday 28 April on Verification tests, subjective viewing and metrics : MPEG WG 4 / Video, MPEG WG 5 / JVET, MPEG WG 7 3D Graphics, and VCEG (ITU-T Q6/21)

This session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Gary Sullivan (VCEG Rapporteur and SC 29 Chair), and Mathias Wien (AG 5 Convenor).

Review of documents in sections 4.5 and 4.4 (latter only JVET and AG 5) was conducted. See notes there.

### Joint session 1810-1915 Wednesday 29 April on next generation video standardization Call for Proposals: MPEG WG 2 / Requirements, MPEG WG 5 / JVET, MPEG AG 5 Visual Quality Assessment, and VCEG (ITU-T Q6/21)

This joint session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Gary Sullivan (VCEG Rapporteur and SC 29 Chair), Mathias Wien (AG 5 Convenor), and Igor Curcio (WG 2 Convenor).

This section is based on notes taken by XXX (add notes from GJS)

…

## BoGs (0)

Section kept as template for future use.

# Project planning (update)

## Software timeline

ECM 19.1 software was planned to be available 1 week after the meeting (31 Jan.).

The NNVC 16.0 codebase software was planned to be available 4 weeks after the meeting (20 Feb.), including all elements needed for CTC and EE1. Additional integration of various SADL aspects and harmonization with VTM may be deferred for version 16.1.

VTM23.14 software was planned to be released within one week after the meeting (31 Jan.). VTM 24.0 was planned to be released within two week after the meeting (6 Feb.), including full implementation of all SEI messages in JVET-AN1019 and JVET-AN2001. Additional versions may be released as appropriate.

Updates on top of HM18.0 and JM19.1 software were planned to be released within approximately 3 weeks after the meeting (integration and updates of SEI messages included in JVET-AN1018 and JVET-AN1017). Additional versions may be released as appropriate.

An update on top of SHM12.4 was planned to be released within approximately 2 weeks after the meeting (harmonization with HM, updates of build system, and other improvements). Additional versions may be released as appropriate.

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning (update)

An EE on neural network-based video coding was established, as recorded in output document JVET-AO2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AO2024.

Initial versions of these documents were presented and approved (see section 10).

JEE Gaussian splat?

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). In cases where combinations of different elements are planned to be tested, mutual cross-checking of the individual elements by other parties of the combination is discouraged. The combination must be cross-checked by an independent party. The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-AL2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow was previously described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

However, it was noted that the link doesn’t seem to exist anymore.

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Chairs of AHGs had been asked to send draft mandates to JRO before XXXX on XX April, preferably copy from the table below and sending with changemarks or yellow highlight of changes.

Review of AHG plans was conducted during the plenary on XXday XX April 2026 at XXXX–XXXX.

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents. - Conduct consultation with parent bodies on future JVET management structures. | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AO1016, JVET-AO1017). - Collect reports of errata for VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports. - Coordinate with AHG3 to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Consider plans for additional verification testing of VVC capability, particularly target conducting tests for VVC multi-layer features, and update the test plan accordingly. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing, as well as studying objective metrics in that context. - Coordinate with AHG17 on investigating sequences and making arrangements for viewing at the interim AHG17 meeting and at the next JVET meeting. - Coordinate with AHG18 on investigating visual impact of data losses. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AM1008, and further develop related conformance bitstreams. - Coordinate with AHG3 on implementation of the new HEVC multiview profiles. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-19.1 software version (and potential updates), corresponding VTM anchor, and the reference configuration encodings according to the ECM common test conditions. - Investigate encoder speedup and other software optimization such as reduction of memory consumption. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs) | N |
| Tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment, such as the aspects of memory access and bandwidth, number of maximum processing cycles, block decoding dependencies, number of context coded bins, pipeline and parallelization. - Study JVET-AO2040 and suggest improvements. - Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs). - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing as applicable. - Prepare reporting of tool assessment results as applicable. - Develop methodology of more reliable runtime measurement. - Collaborate with AHG16 on encoding complexity assessment. | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Discuss improvements on the evaluation framework, including evaluation procedures and methodologies. - Coordinate software development and experiments on optimization of encoders and receiving systems for machine analysis of coded video content, including combinations of proposed technologies. - Maintain the software implementation examples and develop tool combination examples in the repository, including sufficient documentation in terms of operation and performance. - Evaluate proposed technologies and their suitability for machine analysis applications. - Propose potential improvements to JVET-AO2030 on optimization of encoders and receiving systems for machine analysis of coded video content. - Study the potential of using SEI messages for the purpose of machine analysis in coordination with AHG9. - Investigate the impact of using different machine task models in the evaluation of the compression performance of tools optimized for machine analysis tasks. | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) | N |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AO1017, identify any issues and propose solutions as appropriate. - Study JVET-AO2032, including study of SEI messages with different options, when those are present, and propose improvements. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages. - Study SEI messages specified in HEVC and AVC for potential use in the VVC context and SEI messages in VSEI for potential use in the HEVC or AVC context. - Study the alignment of the same SEI messages in different standards. - Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM. - Coordinate with the joint AHG on Gaussian splat coding about aspects related to the usage of SEI messages. | J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and for low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, higher objective quality, or encoding with improved complexity/performance tradeoff, and coordinate such efforts with AHG3, AHG6, and AHG17. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Study potential improvements of the NNVC CTC document JVET-AJ2016, update a list of training materials. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. Promote the call for training materials, distribute it, and actively communicate with content owners. - Analyse complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology. - Discuss and propose improved metrics to perform complexity analysis of NN architectures, in particular also investigate bit-exact reproducibility of NN-based methods on various platforms in coordination with AhG14. (mention GPU or other dedicated NN processing devices more clearly) - Finalize and discuss the EE on neural network-based video coding. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. - Coordinate with AHG14 on items related to NNVC software development, in particular, evaluate the interface in the NNVC for end-to-end optimized AI coded reference pictures. | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) | Y (tel., 2 weeks notice), first on Feb. 20, second on March 20 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM19 algorithm description JVET-AN2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Establish a repository to make other codebases that implement enhanced compression available to interested JVET members. - Support AHG6 in generating anchors according to the test conditions in JVET-AI2017. - Analyse the results of exploration experiments described in JVET-AO2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Discuss and propose refinements to the draft of the TR 2nd ed. JVET-AN2020, and investigate for which elements described in the TR software might be attached. - Study alternative film grain models and their associated documentation. - Discuss and enumerate updates, improvements, and additions for the second edition of the technical report. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Study Film grain region characteristics information SEI in TuC and propose improvements as necessary - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on improving the plan for subjective quality testing of the FGC SEI message JVET-AJ2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, P. de Lagrange (co-chairs), S. Deshpande, A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-16.0 software version (and potential updates), based on updated VTM with adopted contributions and hybrid framework, and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AJ2016. Study the impact of the addition of new dataset on the already integrated models. - Continue to bridge the gap between NNVC and most recent VTM as necessary. - Continue to develop missing functionalities for hybrid end-to-end framework exploration. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AO2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. | F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs) | Y (tel., 2 weeks notice), first on Feb. 20, second on March 20 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AO2027, and provide test results at the next meeting. - Develop and maintain interfaces for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for efficient transporting these elements in the coded video bitstream. - Develop and maintain software for estimation of depth maps for gaming sequences, and evaluate the effect of the estimated maps against the original depth maps on coding efficiency where they are available using established accuracy metrics - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Investigate the possibility to estimate depth maps and camera parameters for those gaming sequences where they are not available. - Study conversion of depth maps using integer representation, and identifying efficient bit-depth resolution of depth maps to support identified use-cases that will be an input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs) | N |
| Hardware implementation complexity (AHG16) (jvet@lists.rwth-aachen.de) - Investigate hardware encoding complexity constraints in typical applications (e.g., mobile devices and hardware transcoding), and identify challenges and evaluate coding tools from hardware encoding implementation perspectives. - Solicit and develop hardware encoding complexity measurements (e.g., maximum number of RDO decisions per CU / CTU), and use them to analyze the performance of test models (e.g., VTM) and coding tools. - Consider the possibility of designing, developing and maintaining a hardware-encoding-mimicking simulation framework on top of test models, e.g., constraining RDO number per CU, constraining coding tree decision complexity (e.g., maximum tree depth, per-node split modes), developing rate estimation function interface, low-complexity CU/TU-level bit estimation function, etc. - Collaborate with AHG7 on aspects of hardware encoding and decoding complexity perspectives, and in improving JVET-AO2040. | Y. Zhao, J. Park, I. Moccagatta (co-chairs), H. Huang, T. Ikai, X. Li, K. Naser, N. Song, G. Verba (vice chairs) | Y (tel., 2 weeks notice) |
| Preparation of Call for Proposals (AHG17) (jvet@lists.rwth-aachen.de) - Review the set of categories, test sequences and rate points based on the results of the CfE, and propose updates for the test set to be used in the CfP. - Identify test sequences extending the represented range of applications and investigate the suitability for use in visual quality assessment. - Finalize the draft CfP text JVET-AO2026 based on the latest version contained in JVET-AO0290, and suggest updates towards the final draft CfP. - Coordinate the generation of VTM anchors and other bitstreams for the test cases defined in the updated test set, and make them available according to the timeline. - Prepare logistics and plans for conducting an AHG meeting during 25-27 February, and issue the announcement JVET-AO2041. - Coordinate with AG 5 on preparing logistics and organization of the visual assessment in the context of the CfP. | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs) | Y (tel., 2 weeks notice), first on 3 Feb. at 1500UTC Hybrid meeting in Aachen, DE, 25-27 February |
| Ultra-low latency and packet loss resilience (AHG18) (jvet@lists.rwth-aachen.de) - Study JVET-AN2039 common test conditions and propose improvements as appropriate. - Investigate creation of practical simulation software, including network transmission aspects, including consideration of network functionality that can aid timely video-coding based error correction, and conduct performance evaluation. - Identify potential requirements and feasibility of standard based technologies to support ultra-low delay requirements, including packet loss resilient decoding. - Investigate packet loss resilient technologies beyond VVC supporting ultra-low delay coding for interactive and live broadcasting scenarios. - Address comments and questions received during CfP proposals discussion. - Coordinate with AHG4 and AHG17 on investigating visual impact of data losses and appropriate evaluation procedure development. | S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |

It was suggested to discontinue AHG8 by the time of finalization of the TR (April or July 2026). For AHG8 and the previous AHG16 (generative face), it is necessary to find a persistent home for the software.

Related to this, it had been discussed during the 41st JVET meeting that it could be useful to generate a permanent repository (instead of AHG branches that may be forgotten) plus documentation for software parts that do extraction of SEI message parameters directly from video, and perform corresponding synthesis. Examples would be NNPF, FGC, GFV, … This was left for further study.

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N XXX) in order to make it easy to reference.

A Joint AHG group was installed with ISO/IEC JTC 1/‌SC 29/‌WG 4 and ISO/IEC JTC 1/‌SC 29/‌WG 7 as follows:

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Gaussian Splat Coding (mpeg-gsc@lists.aau.at) - Prepare use cases and requirements for Gaussian splat coding. - Collect and prepare data set for Gaussian splat coding. - Define common test conditions for Gaussian splat coding. - Prepare anchors for Gaussian splat coding. - Investigate new Gaussian splat representation and coding technologies. - Consolidate the software tools for Gaussian splat coding. - Coordinate the activity on Lightweight GS (1F). - Prepare subjective evaluation. - Coordinate development of GS technology and its standardization across JVET, WG4, and WG7. - Study encapsulation/high-level syntax technologies including those based on V-PCC, V3C, and VSEI. - Study GS technology that allows cross-encapsulation compatibility and definition of related conformance. | Y. Liao, G. Bang (co-chairs on behalf of WG 4), J. Jung, W. Husak (co-chairs on behalf of JVET), A. Zaghetto, J. Ricard (co-chairs on behalf of WG 7) | Y (tel., UTC) Feb 10 1400–1600 Feb 26 1400–1700 Mar 19 1400–1600 Apr 09 1500–1800 (optional) Apr 14 1200–1500 (optional) Apr 16 2200–0100 Apr 21 1400–1700 |

It is planned that formally only one instance of the joint AHG is installed over all MPEG WGs, and the responsibility for doing this is rotating among the involved WGs. At the current meeting, WG 7 was responsible, at the next meeting, WG 4 will take over (including management of documents submitted to the joint activity).

JVET members are invited to subscribe to the reflector of this AHG via https://lists.aau.at/mailman/listinfo/mpeg-gsc.

GS content is stored at https://content.mpeg.expert/data/Explorations/GSC/CTC/

The CTC is updated during each meeting period, and the version valid during the upcoming meeting cycle can be found as:

| MDS26077 | Common test conditions for Gaussian splat coding | B. Kroon (Philips), P. Rondao Alface (Nokia), J. Jung (Qualcomm), G. Sandri (InterDigital) |
| --- | --- | --- |

For accessing software, test materials and anchors, JVET members should contact the AHG chairs. Documents relevant for usage of SEI messages in Gaussian splat representation should be submitted both as WG 4 documents and JVET documents, such that JVET members who are participating via ITU also can get access.

It was anticipated to agree on a joint exploration experiment on topics related to the last three AHG mandates during the first interim AHG meeting on Feb 10. The result can be found in MDS26086, also described in section 8.2 of this report.

# Output documents (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N XXX, as noted in section 9.

A list of JVET-only output documents from the current meeting (including links to jvet-experts.org) was issued as a WG 5 document WG 5 N XXX.

JVET-AP1000 Meeting Report of the 42nd JVET Meeting [J.-R. Ohm] [WG 5 N XXX] (2026-05-29)

Initial versions of the meeting notes (d0 … d9) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne, K. Sharman, R. Sjöberg, G. J. Sullivan] [WG 5 N 103]

Primary editor: C. Rosewarne

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis]

Primary editor: G. J. Sullivan.

JVET-AP1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, J. Boyce, B. Bross, I. Moccagatta, C. Rosewarne, K. Sühring, G. J. Sullivan] (2026-02-27)

Primary editor: Y.-K. Wang.

Some issues found in latest editions of VSEI/VVC/HEVC (see list under 7.1.3).

Remains valid – not updated: JVET-AN1005 Future CICP extensions (Draft 2) [J. Boyce, E. Thomas, A. Tourapis] (2025-10-26)

Primary editor: J. Boyce.

Including update from …

JVET-AP1006 HEVC additional profiles (Draft 1) [Y.-K. Wang, B. Bross, G. J. Sullivan, A. Tourapis] (2026-XX-XX)

Primary editor: A. Tourapis.

New 4:4:4 10/12 profiles from JVET-AP0242

Remains valid – not updated: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)] [WG 11 M 32658]

May require merging with JVET-T2013. JCTVC-P1006 only defines 10-bit sequences for the cases on 4:4:4 and 4:2:2 chroma sampling, and 16-bit sequences for higher bit depth, no 12 bit cases. All sequences specified in this document should be already available in the JVET content server. It is also noted that JVET-AA1100 could be removed once such a merge is done.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

JVET-AP1008 Conformance testing for HEVC multiview extended and monochrome profiles [I. Moccagatta, T. Fu, S. Paluri, A. Tourapis] (2026-XX-XX)

Potential progression into H.265.1 at next meeting – would be premature to issue a request in WG 5.

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174]

Remains valid – not updated: JVET-AJ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien]

Update of bug tracking system description expected in the next meeting, as well as update on zoom meeting rules (no recording etc.).

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

JVET-AP1016 AVC with extensions and corrections (Draft 6) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2026-03-13)

Primary editor: K. Sühring.

Integrated text delivered for consent on next edition H.264, including changes from JVET-AO1017.

WG 5 FDIS + DoCR from N 384 with minor changes to be reviewed on Friday

Remains valid – not updated: JVET-AO1017 Support for additional SEI messages in AVC (Draft 5) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2026-02-27)

Primary editor: K. Sühring

A preliminary DoC WG 5 N 384 on the DAM was reviewed and approved on Thursday 22 Jan. at 2200-2210. Progression to FDAM/FDIS integrated version was planned to be done in July 2026 at latest.

Changes are editorial improvements, interface changes, constraints, and bug fixes agreed at this meeting, including elements from:

- JVET-AO0212 Extension mechanism for new AVC SEI messages

It is noted that the list above may not be complete, and some aspects relating to VSEI interface may first need a correction in VSEIv4 (see JVET-AO1004); if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AN1018 HEVC with extensions and corrections (Draft 3) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] (2025-10-17)

Primary editor: Y.-K. Wang.

This was submitted to ITU consent on H.265 11th ed. It is noted that this may not be identical with the final version published by ITU-T.

Remains valid – not updated: JVET-AN1019 VSEI with extensions and corrections (Draft 1) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-10-17)

Primary editor: J. Boyce.

This was submitted to ITU consent on H.274 4th ed. It is noted that this may not be identical with the final version published by ITU-T.

No output: JVET-Axx1020 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet. See note under JCTVC-P1006 above.

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AN2001 VVC with extensions and corrections (Draft 1) [Y.-K. Wang, B. Bross, M. M. Hannuksela, G. J. Sullivan] (2025-10-17)

Primary editor: Y.-K. Wang.

This was submitted to ITU consent on H.266 4th ed. It is noted that this may not be identical with the final version published by ITU-T

Remains valid – not updated: JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) [Y. Ye, A. Browne, S. Kim]

Primary editor: Y. Ye.

New elements from notes elsewhere in this report:

- RPR clarification (JVET-AO0136)

- MCTF (JVET-AO0136 and JVET-AO0288)

- BIM (JVET-AO0288)

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was suggested that editorial improvements would also be welcome.

Remains valid – not updated: JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

Remains valid – not updated: JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] (2025-08-01)

Primary editor: G. J. Sullivan.

Remains valid – not updated: JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-08-01)

Primary editor: J. Boyce.

Remains valid – not updated: JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li]

Might benefit from an update in the future, regarding some details of SADL implementation.

Remains valid – not updated: JVET-AN2008 Conformance testing for versatile video coding (Draft 8) [I. Moccagatta, F. Bossen, S. Iwamura, P. de Lagrange]

Primary editor: I. Moccagatta

This was submitted to ITU consent on H.266.1 3rd ed., and has been (pre-)published. An equivalent version was under ballot as ISO/IEC DIS 23090-15 3rd ed. A DoC WG 5 N 385 on the DIS was reviewed and approved on Thursday 22 Jan. at 2010-2020. The DIS text will be sent for publication as final standard.

Remains valid – not updated: JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)]

Software relating to H.266.2 and ISO/IEC 23090-16 can be found at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/2nd-edition.

Primary editor: F. Bossen.

[kept for future meetings] A request for next edition(s) of reference software(s) could be issued in the next meeting, and also CD(s) could be produced by the same meeting, as SC 29 is meeting beginning of February and could approve such projects without issuing a ballot. Issuing CD(s) by the January meeting (if mature enough) could avoid that an ISO version would lag behind an ITU version which might be submitted for consent in July 2026. Another consideration could be to start the process for ISO early enough in 2026, such that FDIS and ITU consent could be synchronized in April 2027.

Remains valid – not updated: JVET-AL2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

Remains valid – not updated: JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [E. François, W. Husak, S. Iwamura, D. Rusanovskyy] (2026-01-31)

Links to test sequences need to be updated due to the change of the content server.

New version including removal of metrics, and clarification for metrics to be used in HLG and PQ, as suggested in JVET-AO0184

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

Links to test sequences need to be updated due to the change of the content server.

JVET-AP2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall] (2026-XX-XX)

Updates on training sets (see JVET-AP0228 and JVET-AP0229)

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye]

This document contains test conditions relating to HDR as well as SDR, although ECM testing does not typically include HDR results. Note that a separate output document was issued for HDR test conditions in JVET-AO2011.

Add note about status – see under AHG12 report

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

Links to test sequences need to be updated due to the change of the content server.

JVET-AP2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC) [F. Galpin, Yue Li, Yun Li, D. Rusanovskyy, T. Shao, J. Ström, L. Wang] (2026-XX-XX)

New elements in text and software from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AP2020 Film grain synthesis technology for video applications ed. 2 (Draft 5) [W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis] (2026-XX-XX)

See notes under JVET-AP0187

Issue request Nxxx for second edition in WG 5 (W. Husak drafts document)

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8) [O. Chubach, P. de Lagrange, M. Wien] (2026-02-06)

No output after JVET-AP2033, or section on multiview retained?

No output: JVET-Ax2022

JVET-AP2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)] (2026-02-06)

An initial draft of this document was reviewed and approved at 1310-1335 on Friday 23 January.

This round of EE1 tests includes:

…

No output: JVET-Ax2024

JVET-AP2025 Algorithm description of Enhanced Compression Model 20 (ECM 20) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang] (2026-XX-XX)

New elements from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AP2026 Final draft Joint Call for Proposals on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, F. Bossen] [WG 5 N XXX] (2026-05-15)

Developed from latest version of JVET-AO2026 in JVET-AP0047

Remains valid – not updated: JVET-AO2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud]

Add Wukong2 and Wukong3 to class G2 (see discussion under JVET-AO0207). Furthermore, Level1 and Darktree (HDR versions) were removed from class G3 (still retained as SDR versions), as the difference in terms of performance was minor, and rendering of these is somewhat outdated.

Information about availability of depth maps is also updated.

It was further agreed to remove deltaE100 and PSNRE100 (same as for JVET-AO2011).

JVET-Ax2028 CTC on Gaussian splat technology [add editors] (2026-xx-xx)

Remains valid – not updated: JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC [F. Bossen, J.-R. Ohm, M. Wien] [AG 5 N 180] (2025-12-19)

Remains valid – not updated: JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10) [S. Liu, J. Chen, C. Hollmann, J. Ström] [WG 5 2nd DTR N 387]

Primary editor: S. Liu.

A DoC WG 5 N XXX on the 2nd DTR was reviewed and approved on XXday XX April at XXXX-XXXX.

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann]

JVET-AP2032 Technologies under consideration for future extensions of VSEI (version 12) [S. Deshpande, J. Boyce, J. Chen, M. M. Hannuksela, K. Sühring, H. Tan, J. Xu] (2026-XX-XX)

Primary editor: S. Deshpande

New elements from notes elsewhere in this report (list reviewed approved in JVET plenary on XXday XX April):

…

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AP2033 Report of verification test on VVC multi-layer coding [O. Chubach, S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N XXX)] (2026-xx-xx)

New version developed from JVET-AP0268, adding new section on dual-layer spatial scalability.

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)]

Remains valid – not updated: JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen]

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)]

Remains valid – not updated: JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)]

Remains valid – not updated: JVET-AM2038 White paper on VSEI [J. Boyce, S. McCarthy, S. Deshpande, G. J. Sullivan, Y. Sanchez, Y.-K. Wang] [AG 3 N209]

An update of the white paper on VVC (from 2021) should be done in a future meeting, removing the section on VSEI and referencing to JVET-AM2038.

Remains valid – not updated: JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience [S. Ikonin, S. Deshpande, V. Zakharchenko] (2025-10-31)

Remains valid – not updated: JVET-AO2040 Complexity reporting template for coding algorithms and tools [X. Li, E. Alshina, I. Moccagatta]

The changes suggested by the BoG JVET-AO0287 were reviewed and approved by JVET on Friday 23 Jan. 1615UTC. It was agreed to remove the ECM analysis from the examples (only retain VTM), as ECM may be an untypical example about complexity expected by proposals.

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting (update)

The draft of the WG 5 recommendations (see Annex C) was reviewed and approved in JVET at XXXX–XXXX on Friday 1 May.

Future meeting plans were established with the following general guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the Monday, Tuesday or Wednesday of the first week and closing it on the Wednesday of the second week of the SG21 meeting – a total of 8-10 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Wednesday, Thursday or Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8–10 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined, or AHG meetings might be scheduled prior to the meeting. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans were established as follows:

- During 7 – 15 July 2026, 43rd meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 17 – 23 October 2026, 44th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN, to be conducted as hybrid meeting,

- During 13 – 22 January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU, to be conducted as hybrid meeting,

- During 17 – 28 April 2027, 46th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 7 – 16 July 2027, 47th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI, to be conducted as hybrid meeting,

- During 20 – 29 October 2027, 48th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN, to be conducted as hybrid meeting,

- During January 2028, 49th meeting under ISO/IEC JTC 1/‌SC 29, date and location t.b.d,,

- During 25 March – 5 April 2028, 50th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During July 2028, 51st meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.,

- During October 2028, 52nd meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The agreed document deadline for the 43rd JVET meeting was planned to be Tuesday 30 June 2026.

XXX were thanked for offering new video test sequences.

Marius Preda was thanked for the service of managing and maintaining the document site jvet-experts.org. Institut Mines-Télécom was thanked for hosting the sites.

Thank meeting host and organizer …

The 42nd JVET meeting was closed at approximately XXXX hours CEST on Friday 1 May 2026.

# Annex A to JVET report:
List of documents

Dates and times in the table below are in Paris/Geneva time (2 hr. ahead of UTC by the current meeting). It is noted that, if title or authorship of a document deviates from the title or author list in the body of the report, the list of documents in this annex contains the correct title and authors.

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the thirty-ninth meeting of the JVET, according to a sign-in sheet circulated in the JVET meeting rooms (approximately XXX people in total), were as follows:

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the forty-second meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total, not including those who had attended the meeting in person at least part-time (see annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

# Annex C to JVET report:
Recommendations of the 23rd meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG21

ISO/IEC JTC 1/SC 29/WG 5 N XXX