	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG

(ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6)

6th Meeting: Awaji, Island, JP, 5-13 December, 2002
	Document:  JVT-F026
Filename: JVT-F026.doc


	Title:
	Chroma Intra Prediction Text

	Status:
	Input Document to JVT

	Purpose:
	Proposal

	Author(s) or
Contact(s):
	Eric Viscito
20450 Stevens Creek Blvd
Cupertino, CA 95014
USA
	
Tel:
Email:
	
1-408-861-7087
eric.viscito@conexant.com

	Source:
	Conexant


_____________________________
Introduction

Edits provided during the post-Geneva editing process were somehow not incorporated.

Proposal

Use the following text as the basis for Section 8.3.3 in Awaji
.  

8.3.3 Intra prediction process for chroma blocks

This process is invoked for I and SI macroblock types. It is specified how the Intra predicted chroma samples for the current macroblock are derived.
Input to this process are the syntax elements intra_chroma_pred_mode and decoded samples prior to the deblocking process from neighboring luma blocks (if available).
Output of this process are Intra predicted chroma samples for the current macroblock.

[Ed.Note: what about MB-AFF ???]

The chroma in intra macroblocks is predicted in a manner very similar to the luma block in Intra_16x16 macroblock type (subclause 8.3.2), using one of four prediction modes.  The same prediction mode is applied to both chroma blocks, but it is independent of the prediction mode used for the luma.

NOTE - If any portion of the luma macroblock is coded in intra mode, the entire chroma macroblock is coded intra.

Let P(-1,y), y=-1,0,...,7 and P(x,-1), with x=-1,0,…,7 denote the 17 neighbouring samples immediately above and/or to the left of the 8x8 chroma block.  Let Pred(x,y) with x,y = 0..7 denote the prediction for the 8x8 chroma block samples.  There are four different prediction modes as specified in subclauses 8.3.3.1 to 8.3.3.4.  Only Mode 3 uses P(-1,-1).

Samples P(x,-1) or P(-1,y) shall be considered not available if they are outside the picture or outside the current slice, or if they are in a non-intra macroblock and constrained_intra_pred is 1.  If none of the neighbouring samples are available, Mode 2 (DC prediction) is the only possible 8x8 chroma prediction mode.

8.3.3.1 Mode 0: DC prediction process

If all samples P(-1,n) and P(n,-1) used in Equation 8-21 are available, the prediction is formed as:


Pred(x, y) = 
[image: image1.wmf](

)

4

8

)

1

,

(

)

,

1

(

7

0

>>

÷

÷

ø

ö

ç

ç

è

æ

+

÷

ø

ö

ç

è

æ

-

+

-

å

=

n

n

P

n

P

    with x, y = 0..7,
(8-21)

If the 8 samples P(-1,n) are not available but the 8 samples P(n,-1) are available, the prediction is formed as:

Pred(x,y) = 
[image: image2.wmf]ú

û

ù

ê

ë

é

+

÷

ø

ö

ç

è

æ

-

å

=

4

)

1

,

(

7

0

n

n

P

>> 3
with x, y = 0..7,
(8-22)

If the 8 samples P(n,-1) are not available but the 8 samples P(-1, n) are available, the prediction is formed as:

Pred(x,y) = 
[image: image3.wmf]ú

û

ù

ê

ë

é

+

÷

ø

ö

ç

è

æ

-

å

=

4

)

,

1

(

7

0

n

n

P

>> 3
with x, y = 0..7,
(8-23)

If none of the neighbouring samples P(x,-1) and P(-1,y) are available, the prediction for all chroma samples in the macroblock shall be 128.

8.3.3.2 Mode 1: horizontal prediction process

This mode shall be used only if all neighbouring samples P(-1, y), with y = 0..7 are available:

Pred(x, y) = P(-1, y), with x, y = 0..7

(8-24)

8.3.3.3 Mode 2: vertical prediction process

This mode shall be used only if all neighbouring samples P(x, -1), with x = 0..7 are available:

Pred(x, y) = P(x, -1), with x, y = 0..7

(8-25)

8.3.3.4 Mode 3: plane prediction

For the plane mode, the prediction is formed as:


Pred(x,y) = Clip1( (a + b·(x-3) + c·(y-3) +16) >> 5 ),  with x, y = 0,…,7
(8-26)

where:

a = 16·(P(-1,7) + P(7,-1))

(8-27)

b = (17*H+16)>>5

(8-28)

c = (17*V+16)>>5

(8-29)

and H and V are specified as:


[image: image4.wmf](

)

[

]

å

=

-

-

-

-

+

×

=

4

1

)

1

,

3

(

1

,

3

x

x

P

x

P

x

H


(8-30)


[image: image5.wmf](

)

[

]

å

=

-

-

-

+

-

×

=

4

1

)

3

,

1

(

3

,

1

y

y

P

y

P

y

V


(8-31)
JVT Patent Disclosure Form

	International Telecommunication Union
Telecommunication Standardization Sector
	International Organization for Standardization
	International Electrotechnical Commission  

	[image: image6.wmf]
	[image: image7.png]1S0
NS




	[image: image8.png]





Joint Video Coding Experts Group - Patent Disclosure Form
(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

	Submitting Organization or Person:

	Organization name
	
	

	Mailing address
	
	

	Country
	
	

	Contact person
	
	

	Telephone
	
	

	Fax
	
	

	Email
	
	

	Place and date of submission
	
	

	Relevant Recommendation | Standard and, if applicable, Contribution:

	Name (ex: “JVT”)
	
	

	Title
	
	

	Contribution number
	
	

	
	
	


(Form continues on next page)

	Disclosure information – Submitting Organization/Person  (choose one box)

	
	

	
[image: image9.wmf] 


	2.0
The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.

or,

	The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.  In which case,



	[image: image10.wmf]
	2.1
The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation | Standard.

	
	

	[image: image11.wmf]
	2.2
The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation | Standard.


Such negotiations are left to the parties concerned and are performed outside the ITU | ISO/IEC.

	
	

	[image: image12.wmf]
	2.2.1
The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same.

	
	

	[image: image13.wmf]
	2.3
The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above.  In this case, the following information must be provided as part of this declaration:

· patent registration/application number;
· an indication of which portions of the Recommendation | Standard are affected.
· a description of the patent claims covering the Recommendation | Standard;

	In the case of any box other than 2.0 above, please provide the following:

	Patent number(s)/status
	
	

	Inventor(s)/Assignee(s)
	
	

	Relevance to JVT
	
	

	Any other remarks:
	
	

	(please provide attachments if more space is needed)




(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

	Disclosure information – Third Party Patents (choose one box)

	
	

	
[image: image14.wmf] 


	3.1
The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation | Standard or Contribution.



	[image: image15.wmf]
	3.2
The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.



	For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information:



	3rd party name(s)
	
	

	Mailing address
	
	

	Country
	
	

	Contact person
	
	

	Telephone
	
	

	Fax
	
	

	Email
	
	

	Patent number/status
	
	

	Inventor/Assignee
	
	

	Relevance to JVT
	
	

	
	
	


	Any other comments or remarks:




�  The WYSISYG formatting here is not intended to resemble the draft exactly, but it can be copied in.





File:JVT-F026.doc
Page: 1
Date Saved: 2002-11-27

_1090441517.unknown

_1093950594.unknown

_1099913356.doc



_1099913374.doc



_1097422426.unknown

_1093950513.unknown

_1090421544.unknown

