	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG

(ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6)

6th Meeting: Awaji, Island, JP, 5-13 December, 2002
	Document:  JVT-F050

Filename: JVT-F050.doc


	Title:
	POC Recovery in Random Access Point SEI Message

	Status:
	Input Document to JVT

	Purpose:
	Proposal

	Author(s) or
Contact(s):
	Ye-Kui Wang
TICSP
Tampere Univ. of Technology
P.O. Box 553
FIN-33101 Tampere
Finland

Miska M. Hannuksela
Nokia Mobile Software
P.O. Box 88
33721 Tampere
Finland
	
Tel:
Email:
	
+358 3 3115 3972
wyk@ieee.org


+358 7180 73151
miska.hannuksela@nokia.com




	Source:
	Tampere University of Technology and Nokia Corporation


_____________________________
1. summary

The latest draft standard JVT-E146d37 allows using random access point SEI messages to indicate gradual random access points, i.e. gradual decoder refresh points. However, since picture order count (POC) for each picture is calculated relative to the latest IDR picture in decoder order, POC values cannot be figured out when the decoding starts from a gradual random access point. This contribution proposes a solution for the problem by modifying the random access point SEI message.

2. Proposal

2.1 Random Access Point SEI message

The random access point SEI message syntax is modified as follows, where the modified parts are highlighted in yellow color.

	random_access_point( PayloadSize ) {
	C
	Descriptor

	
recovery_frame_cnt
	7
	ue(v)

	
exact_match_flag
	7
	u(1)

	
broken_link_flag
	7
	u(1)

	
if(pic_order_cnt_type = = 0)
	
	

	

PictureOrderCount
	7
	u(32)

	
else if(pic_order_cnt_type = = 1 || pic_order_cnt_type = = 2)
	
	

	

FrameNumPeriodCount
	7
	u(v)

	}
	
	


PictureOrderCount equals to the picture order count of the SEI target picture. If the picture is a frame, then PictureOrderCount is equal to FramePicOrderCnt. If the picture is a top field, PictureOrderCount is equal to TopPicOrderCnt. If the picture is a bottom field, PictureOrderCount is equal to BottomPicOrderCnt. The value of PictureOrderCount shall be in the range from 0 to 232-1, inclusive.

FrameNumPeriodCount equals to FrameNumOffset/MaxFrameNum, and FrameNumOffset is for the SEI target picture as specified in subclause 8.2.2.2. The size of FrameNumPeriodCount is (28-log2_max_frame_num_minus4) bits. The value of FrameNumPeriodCount shall be in the range from 0 to 2(28-log2_max_frame_num_minus4)-1, inclusive.

Note that the modification above for POC type 2 is for the new coding method proposed in JVT-F044. Please see JVT-F044 for the intention to modify POC type 2 and the new coding method.

2.2 Picture Order Count Calculation

When the decoding is started from a random access point SEI message, the picture order count of a picture is calculated as follows.

If pic_order_cnt_type equals 0, the picture order count of the target picture is equal to PictureOrderCount conveyed within the random access point SEI message. The picture order count values of subsequent pictures are calculated as specified in subclause 8.2.2.1. 

If pic_order_cnt_type equals 1, FrameNumOffset is first calculated as

FrameNumOffset = FrameNumPeriodCount x MaxFrameNum

Then POC is calculated according to Eqs. 8-7 to 8-9 as specified in subclause 8.2.2.2.

If pic_order_cnt_type equals 2, FrameNumOffset is first calculated as above. Next,  AbsFrameNum is calculated according to:


  AbsFrameNum = FrameNumOffset + frame_num

Then FramePicOrderCnt is calculated according to:


  FramePicOrderCnt = AbsFrameNum << 1

If the top field precedes the bottom field in decoding order, TopPicOrderCnt equals FramePicorderCnt and BottomPicOrderCnt equals FramePicorderCnt+1. Else if the bottom field precedes the top field in decoding order, BottomPicOrderCnt equals FramePicorderCnt and TopPicOrderCnt equals FramePicorderCnt+1. Otherwise, both TopPicOrderCnt and BottomPicOrderCnt equal FramePicorderCnt.

(Append for Proposal Documents)

JVT Patent Disclosure Form

	International Telecommunication Union
Telecommunication Standardization Sector
	International Organization for Standardization
	International Electrotechnical Commission  

	[image: image1.wmf]
	[image: image2.png]1S0
NS




	[image: image3.png]





Joint Video Coding Experts Group - Patent Disclosure Form
(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

	Submitting Organization or Person:

	Organization name
	Nokia Corporation
	

	Mailing address
	
	

	Country
	
	

	Contact person
	
	

	Telephone
	
	

	Fax
	
	

	Email
	
	

	Place and date of submission
	
	

	Relevant Recommendation | Standard and, if applicable, Contribution:

	Name (ex: “JVT”)
	JVT
	

	Title
	POC Recovery in Random Access Point SEI Message
	

	Contribution number
	JVT-F050
	

	
	
	


(Form continues on next page)

	Disclosure information – Submitting Organization/Person  (choose one box)

	
	

	[image: image4.wmf]
	2.0
The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.

or,

	The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.  In which case,



	[image: image5.wmf]
	2.1
The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation | Standard.

	
	

	[image: image6.wmf]
	2.2
The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation | Standard.


Such negotiations are left to the parties concerned and are performed outside the ITU | ISO/IEC.

	
	

	[image: image7.wmf]
	2.2.1
The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same.

	
	

	[image: image8.wmf]
	2.3
The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above.  In this case, the following information must be provided as part of this declaration:

· patent registration/application number;
· an indication of which portions of the Recommendation | Standard are affected.
· a description of the patent claims covering the Recommendation | Standard;

	In the case of any box other than 2.0 above, please provide the following:

	Patent number(s)/status
	
	

	Inventor(s)/Assignee(s)
	
	

	Relevance to JVT
	
	

	Any other remarks:
	
	

	(please provide attachments if more space is needed)




(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

	Disclosure information – Third Party Patents (choose one box)

	
	

	[image: image9.wmf]
	3.1
The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation | Standard or Contribution.



	[image: image10.wmf]
	3.2
The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.



	For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information:



	3rd party name(s)
	
	

	Mailing address
	
	

	Country
	
	

	Contact person
	
	

	Telephone
	
	

	Fax
	
	

	Email
	
	

	Patent number/status
	
	

	Inventor/Assignee
	
	

	Relevance to JVT
	
	

	
	
	


	Any other comments or remarks:




X





X








File:JVT-F050.doc
Page: 1
Date Saved: 2002-11-29

