	Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG

(ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6)

10th Meeting: 8-12 December 2003, Waikoloa
	Document: JVT-J036

Filename: JVT-J036.doc


	Title:
	SEI messages and DPB operation to support loop filter as reference filter

	Status:
	Input Document to JVT

	Purpose:
	Proposal

	Author(s) or
Contact(s):
	Jiuhuai Lu
100 Universal City Plaza, 3153-PHL

Universal City, CA 91608 USA

Shinya Kadono

1006 Kadoma, Kadoma

Osaka 573-8501, Japan

Yoshiichiro Kashiwagi

100 Universal City Plaza, 3153-PHL

Universal City, CA 91608 USA


	
Tel:
Email:


	
+1 818 777 8147
Jiuhuai@research.panasonic.com

kadono.shinya@jp.panasonic.com

kashiwagiy@research.panasonic.com 

	Source:
	Panasonic 


_____________________________
1. Introduction

At last meeting, we proposed as a reference filtering and its implementation by simply switching the loop filter out of the display path [1]. By that, we achieved greater inter-prediction efficiency for video pictures in the presence of uncorrelated textures and film grains. This document defines the syntax structure and DPB operation to support the decoding operation when the deblocking filter is set to serve as reference filter. The loop filter SEI message is structured to minimize the bits necessary to signal the reference picture filtering mode. 

2. SEI messages for switching between reference filter and deblocking filter mode

2.1 Loop Filter Mode SEI Syntax

The syntax of the SEI is described here for controlling loop filter modes to switch between deblocking and reference filtering. A MPEG-4 AVC encoder would transmit these SEI messages to the decoder to control the switch that determine which loop filter should be used.

	ref_pict_filtering_present (payloadsize) {
	C
	Descriptor

	
num_res_frames_in_dpb
	5
	u(4)

	
currLPFmode = Deblocking Mode
	
	

	}
	
	


	loop_filter_switching_in_pict(payloadsize)  {
	
	

	
num_mode_changes_minus1
	5
	u(8)

	
If (num_mode_changes_minus1 = = 0)
	
	

	

mode_for_all_pict
	5
	u(1)

	
else
	
	

	

mode_for_all_pict = 0
	
	

	
If (mode_for_all_pict = = 0)
	
	

	

For ( mbIdx = 0; mbIdx < num_mode_changes_minus1+1; mbIdx ++) 
	
	

	


mb_with_mode_switch [mbIdx]
	5
	ue(v)

	}
	
	


2.2 Loop Filter Mode SEI Semantics

The SEI message is to control the modes of the loop filter (currently only serving deblocking filter) to be either a deblocking filter or a reference filter.

ref_pict_filter_present: This message is only assigned to an IDR picture to indicate the reference filtering mode is used in some slices or some pictures or in all pictures following the IDR picture. When simple “bumping” process is used for DPB removal and output, it is required that this flag should be present in the first IDR picture in the sequence.

num_res_frames_in_dpb: When the reference filtering mode is used, both filtered reference pictures and the non-filtered copy of the references must be saved in DPB. This flag indicate how much memory must be reserved for the use of reference filtering modes.

loop_filter_switching_in_pict: This is a SEI message indicating that there is at least one loop filter mode switching in the current picture corresponding to this SEI message.

num_mode_changes_minus1 specifies the number of mode changes. It must be from 0-255. 

mode_for_all_pict specifies whether the new mode shall be applied to the whole picture. When mode_for_all_pict equals to 1, the new mode is applied to the entire picture.

mb_with_mode_switch: the index of the macroblock where this currLPFmode should switch to the other one of the two loop filter modes prior to decoding the macroblock. Each mb_with_mode_switch should be the first macroblock in a slice. mb_with_mode_switch [] should be arranged in the order that their coded block appears in the bitstream. The earlier the coded blocks of the macroblock, the lower their mbIdx value is. Once the current macroblock mb_with_mode_switch. The loop filter mode currLPFmode will be kept until the decoder reaches the next macroblock that signals a mode change. The loop filter parameters slice_alpha_c0_offset_div2 and slice_beta_offset_div2 should be derived from the slice that contains mb_with_mode_switch [].

3. Decoded Picture Buffer (DPB) operations

3.1 Decoding process associated with reference filtering mode

When a decoder finds that the SEI message  ref_pict_filtering_present_idr_cycle is present, the decoder shall make necessary preparation for the DPB operations defined here. A variable DPB bumping size DPBbumpingSize should be set to 

DPBbumpingSize = max_dec_frame_buffering - num_res_frames_in_dpb

It is used to determine if one or more frames should be removed from the DPB. When decoder finds that the SEI message  loop_filter_switching_in_pict is present and nal_ref_idc not equal to 0, following decoding the picture associated with the SEI message, the decoder shall insert two copies of the picture into DPB, one is the picture resulting from the inverse transform, motion compensation, and the other is the same picture with additional loop filter filtering. The latter picture is labeled as “used_as_reference”, while the former one is labeled as “non-reference”. They shall be assigned the same value of Picture Order Count (POC), depending on the POC type. After that, the decoder updates DPBbumpingSize by incrementing it by 1.


DPBbumpingSize = DPBbumpingSize + 1

3.2 DPB bumping process associated with reference filtering mode

When the decoder finds the SEI message message  ref_pict_filtering_present_idr_cycle  is present, it shall use the DPBbumpingSize to control the removal of pictures from DPB. When the DPB frame account exceeds the DPBbumpingSize, the lowest POC pictures labeled “unused for reference” should be removed. When there are two pictures with the same lowest POC value, one reference picture labeled with “unused for reference” and another with “non-reference”. Both will be removed. But only the picture labeled as “non-reference” will be used for output to display. After the removal, the following operation is executed:


DPBbumpingSize = DPBbumpingSize-1

4. Reference:

1. Jiuhuai Lu, Yoshiichiro Kashiwagi, Masayuki Kozuka, “Inter Prediction efficiency improvement in the presence of film grains and fine details,” JVT-I040, 9th JVT Meeting 205 Sept. 2003, San Diego. 

2. “Draft ITU-T Recommendation and Final Draft International Standard of Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC), May 2003.

 (Append for Proposal Documents)
JVT Patent Disclosure Form

	International Telecommunication Union
Telecommunication Standardization Sector
	International Organization for Standardization
	International Electrotechnical Commission  

	[image: image1.wmf]
	[image: image2.png]1S0
NS




	[image: image3.png]





Joint Video Coding Experts Group - Patent Disclosure Form
(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

	Submitting Organization or Person:

	Organization name
	Panasonic Hollywood Laboratory
	

	Mailing address
	100 Universal City Plaza, Bldg 3153-PHL

Universal City, CA 91608
	

	Country
	USA
	

	Contact person
	Jiuhuai Lu
	

	Telephone
	+1 818 777 8147
	

	Fax
	+1 818 733 1425
	

	Email
	Jiuhuai@research.panasonic.com
	

	Place and date of submission
	Waikoloa JVT Meeting 8-12 December 2003
	

	Relevant Recommendation | Standard and, if applicable, Contribution:

	Name (ex: “JVT”)
	H.264/AVC
	

	Title
	SEI messages and DPB operation to support loop filter as reference filter
	

	Contribution number
	JVT-J036
	

	
	
	


(Form continues on next page)

	Disclosure information – Submitting Organization/Person  (choose one box)

	
	

	[image: image4.wmf]
	2.0
The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.

or,

	The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.  In which case,



	[image: image5.wmf]
	2.1
The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation | Standard.

	
	

	x
	2.2
The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation | Standard.


Such negotiations are left to the parties concerned and are performed outside the ITU | ISO/IEC.

	
	

	
	2.2.1
The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same.

	
	

	[image: image6.wmf]
	2.3
The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above.  In this case, the following information must be provided as part of this declaration:

· patent registration/application number;
· an indication of which portions of the Recommendation | Standard are affected.
· a description of the patent claims covering the Recommendation | Standard;

	In the case of any box other than 2.0 above, please provide the following:

	Patent number(s)/status
	1 patent


	

	Inventor(s)/Assignee(s)
	Jiuhuai Lu, Shinya Kadono, Kashiwagi Kashiwagi
	

	Relevance to JVT
	
	

	Any other remarks:
	
	

	(please provide attachments if more space is needed)




(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

	Disclosure information – Third Party Patents (choose one box)

	
	

	x
	3.1
The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation | Standard or Contribution.



	[image: image7.wmf]
	3.2
The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.



	For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information:



	3rd party name(s)
	
	

	Mailing address
	
	

	Country
	
	

	Contact person
	
	

	Telephone
	
	

	Fax
	
	

	Email
	
	

	Patent number/status
	
	

	Inventor/Assignee
	
	

	Relevance to JVT
	
	

	
	
	


	Any other comments or remarks:






















































































Page: 2


