ITU - Telecommunication Standardization Sector
Temporary Document XX(PLEN)
STUDY GROUP 16
Geneva, 13-17 November 2000

Question(s):
11

SOURCE
:
Rapporteur Q11 (T. Geary)

TITLE:
Draft revised Implementor's Guide for the ITU-T H.324 Recommendation series
__________________

Introduction

This document is a text revision of the H.324 Implementor’s Guide approved in February 2000. The changes from the previous version 4 of the Implementor’s Guide are clarifying procedures as they appeared and were agreed in Q11-L-09r1 (Osaka).

Changes from the previous published version appear in the following sections:

1. 7.2.1 Section C.2 - used to be 7.2.2  (Q11L09r1 + editor)

2. 7.2.2 Section C.4 - used to be 7.2.3  (editor)

3. 7.2.3 Section C.5  (Q11L09r1)

4. 7.2.4.1 Section C.7  (Q11L09r1 + editor)

5. 7.2.4.2 Section C.7 - level change procedure text that used to be in 7.2.1 (Q11L09r1 + editor)

6. 7.3.1 Section D.5.1.2  (Q11L09r1)

7. 7.3.2 Section D.5.4  (Q11L09r1)

8. 7.4 and 7.4.1 Annex F - used to be 7.3 and 7.3.1 and incorrectly numbered 7.2 and 7.2.1 in IG version 4  (editor)

Implementor's Guide for the ITU-T H.324 Recommendation series Version 2 - Terminal for low bit-rate multimedia communication
Abstract

This document is a compilation of reported defects identified with the 1997-2000 editions of the ITU-T H.324-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.324-series Recommendations.

Contact information

ITU-T Study Group 16/Question 11 Rapporteur


Tom Geary

Conexant Systems Inc. 

4311 Jamboree Road

Newport Beach, CA 92660-3095
USA 


Tel:
+1 949 483 4092

Fax:
+1 949 483 6511

Email:
tom.geary@connexant.com

Implementor's Guide Editor


Patrick Luthi

PictureTel Corp.

100 Minuteman Road

Andover, MA 01810

USA


Tel:
+1 978 292 5000
Fax: 
+1 978 292 3309
Email: 
luthip@pictel.com

ITU-T Recommendation H.324 Editor
Mickey Nasiri
Ericsson Telecom
S-125 25 Stockholm
Sweden
Tel:
+46 8 404 5591
Fax:
+46 8 18 7620
Email:
mickey.nasiri@ecs.ericsson.com

Document history

Revision
Date
Description

1
8-11 June 1998
Initial version - Reviewed at the Q.11/SG 16 meeting.

2
22 September 1998
Second version - Completed at the ITU-T Study Group 16 Rapporteurs meeting.

3
24 May 1999
Third version - Completed at the ITU-T Study Group 16 Rapporteurs meeting.

4
7-18 February 2000
Fourth version - Completed at the ITU-T Study Group 16 Rapporteurs meeting.

5
13-17 November 2000
Fifth version - Completed at the ITU-T Study Group 16 Rapporteurs meeting.

1.  Introduction

This document is a compilation of reported defects identified with the 1997-2000 editions of the ITU-T H.324-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected H.324-series Recommendations.

The first version of the guide was produced following the September 1998 ITU-T Study Group 16 meeting. Wide distribution of this document is expected and encouraged.

2.  Scope

This guide resolves defects in the following categories:

•
editorial errors;

•
technical errors such as omissions or inconsistencies;

•
ambiguities.

In addition the Guide may include explanatory text found necessary as a result of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories. Proposals for new features should be made in the normal way through contributions to the ITU-T.

3.  Policies for updating this document

This document is managed by the ITU-T Study Group 16 Question 11 Rapporteur's Group. It can be revised at any recognized Q.11/16 Rapporteur's Group meeting provided the proposed revisions are unanimously accepted by the members of the group. A revision history cataloguing the evolution of this document is included.

4.  Defect resolution procedure

Upon discovering technical defects with any components of the H.324 Recommendations series, please provide a written description directly to the editors of the affected Recommendations with a copy to the Q.11/16 Rapporteur. The template for a defect report is enclosed. Contact information for these parties is included in this document. Return contact information should also be supplied so a dialogue can be established to resolve the matter and an appropriate reply to the defect report can be conveyed. This defect resolution process is open to anyone interested in H.324-series Recommendations. Formal membership in the ITU is not required to participate in this process.

5.  References

This document refers to the following H.324-series Recommendations:

· ITU-T Recommendation H.324 (1998), Terminal for low bit‑rate Multimedia Communication.

· ITU-T Recommendation H.324 Annex F (1998), Multilink operation
–
ITU-T Recommendation H.223 (1996), Multiplexing protocol for low bit rate multimedia communication
–
ITU-T Recommendation H.223 Annex D (1999), Optional multiplexing protocol for low bit rate multimedia mobile communication over highly error-prone channels
6.  Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

Symbol
Description

[Begin Correction]
Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described.

[End Correction]
Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described.

...


Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity.

--- SPECIAL INSTRUCTIONS --- {instructions}
Indicates a set of special editing instructions to be followed. 

7.  Technical and editorial corrections

7.1 Additions to H.324

In order to allow the use of G.722.1 in H.324 systems, a new section 6.7.4 is being added with the following text:
[Begin Correction]
6.7.4   Use of Recommendation G.722.1 for wideband audio
Recommendation G.722.1 may be used for wideband audio applications. G.722.1 frames shall be sent using AL2.  Audio frame boundaries within each AL-SDU shall be implied by the ratio of AL-SDU size to G.722.1 frame size at the currently selected G.722.1 bitrate.

[End Correction]
7.2  Technical and editorial corrections to H.324 Annex C

7.2.1  Additions  to Section C.2 - General

A clarification to the level change procedure and a reference to H.223 Annex D are being added to the H.223 Level 3 description as follows:

[Begin Correction]
C.2
General
…

· H.223 Level 1: Described in Annex A/H.223. The HDLC flag in H.223 used to delimit MUX-PDUs in Level 0 is replaced with a longer flag that leads to improved MUX-PDU synchronization. HDLC bit stuffing is not used. The Control Channel Segmentation and Reassembly Layer (CCSRL) is introduced for the transmission of the control channel.

The transmitter side shall take necessary precaution to prevent possible flag emulation for the control channel.  Flag emulation can be prevented, e.g. by detecting N 16-bit flags in each MUX-SDU for the channel and by breaking the SDU into N+1 segments. This may also apply to data channels.
· H.223 Level 2: Described in Annex B/H.223. Includes the features of Annex A/H.223. In addition, the header describing the MUX-PDU contents includes error protection.

· H.223 Level 3: Described in H.223/Annex C. Includes the features of H.223/Annex B. In addition, error protection and other features are provided to increase the protection of the AL-PDUs. Described in H.223/Annex D as an optional definition of H.223/Annex C.
In addition to the hierarchy offered by the level structure, some of the multiplex levels contain options.
If the terminal is connected to an octet oriented network interface, the transmitter shall align the first transmitted bit to the network octet timing. Note that the MUX-PDU of H.223 level 1 and higher levels have an octet-aligned structure. Therefore, the receiver may use the octet timing information from the network interface to detect the start of a MUX-PDU to reduce the mis-synchronisation.
Mobile terminals shall support the NSRP and the SRP mode of H.324/Annex A. If both terminals start the session in level 0 initially the SRP mode shall be used. Otherwise both terminals shall start with NSRP mode. 

If both terminals in a session support level 3 of H.223, then adaptation layers AL1M, AL2M, and AL3M as defined in level 3 of H.223 may also be used in H.223, H.223 Annex A and B (levels 1 and 2). However, bidirectional channels shall use either the H.223 adaptation layers, or the adaptation layers for level 3 of H.223, but not a mixture of the two.

…

[End Correction]
7.2.2  Additions to Section C.4 – Interworking

A reference to Annex D is being added to Section C.4 as follows:

[Begin Correction]
C.4
Interworking
Since all mobile terminals support H.223 level 0, no interworking function is needed when communicating with an H.324 terminal that does not support any of the robust multiplexing annexes (Annexes A, B, C and D of Recommendation H.223).
[End Correction]
7.2.3  Clarifications to Section C.5 - Terminal procedures

Clarifying text to allow H.324 Annex D terminals to support Annex C mode is being added to section C.5 as follows:

[Begin Correction]
C.5
Terminal procedures

…

•
Phase D: The value of timer T401 shall be defined using the procedures in Annex E. The transmission of 16 consecutive HDLC flags is replaced with the level setup procedure defined in section C.6
…

[End Correction]
7.2.4  Clarifications to Section C.7 - Dynamic change of level or option during session

7.2.4.1  "mobileOperationTransmitCapability"

Text describing the usage of the H.245 mobileOperationTransmitCapability in H.324 Annex C is being added as follows:

[Begin Correction]
C.7
Dynamic change of level or option during session

The procedure described below for changing multiplex options during a session assumes that a capabilities exchange has taken place between a receiver (Terminal A) and a transmitter (Terminal B), and that an H.245 command for a level change will be sent by the receiver terminal to the transmitter terminal. A terminal, which has the capability to change H.223 multiplex levels or options during a session, shall set modeChangeCapability in mobileOperationTransmitCapability to True. Levels and options supported by the terminal are indicated using the codepoint in mobileOperationTransmitCapability.
A terminal, which has the above capability and has received the H.245 message with modeChangeCapability of True, may start the H.223 mode change procedure illustrated in Figure C.1. Only levels supported by both terminals may be commanded. Note that the "replacementFor" procedure described in Recommendation H.245 may be used when changing from a mobile adaptation layer (ALXM) to a regular H.223 adaptation layer (ALX) or vice versa.

…

[End Correction]
7.2.4.2  Clarifications to level change procedure

This section describes the editorial corrections to clarify the level change procedure in the H.324 Annex C. The following text is being added at the end of section C.7.

[Begin Correction]

Note that after changing from level 0 to some higher levels, MUX-PDU octet alignment shall be preserved. Therefore, the transmitter shall add as many "0" bits after the level change sequence to octet align the first synchronization flag of the new level. In the transmitter, the reference for the octet alignment is the first bit of the first transmitted synchronization flag. In the receiver, the reference for the octet alignment is the first bit of the first detected synchronization flag in the initial level set-up procedure.

[End Correction]

7.3  Technical and editorial corrections to H.324 Annex D

7.3.1  Clarifications to Section D.5.1.2 - Received signals

Clarifying text to allow H.324 Annex D terminals to support Annex C mode is being added to section D.5.1.2 as follows:
[Begin Correction]
D.5.1.2 Received signals

While executing the V.140 Phase 1 procedure, the H.324/I terminal shall search the received data for signals conforming to:

•
V.140 signature;

•
HDLC flags followed by an H.223 MUX_PDU on the digital channel or, if Annex C is supported, all possible stuffing sequences defined in Table C.1;

•
H.320 (search for H.221 FAS and BAS signalling);

•
If V.8 bis is supported, V.8 bis (search for initial V.8 bis messages in G.711 audio);

•
V.8 (search for initial V.8 messages in G.711 audio).

Additionally, bits 1-6 of each octet may be decoded as audio according to Recommendation G.711 and delivered to the user while executing this procedure, so that voice telephony is established immediately upon connection of the circuit, if the far-end terminal supports voice telephony.

[End Correction]

7.3.2  Clarifications to Section D.5.4 - Phases D through G

Clarifying text to allow H.324 Annex D terminals to support Annex C mode is also being added to section D.5.4 as follows:
[Begin Correction]
All remaining phases (D through G) shall proceed as specified in the body of this Recommendation or, as defined in section C.5 if Annex C is supported.
[End Correction]

7.4  Technical and editorial corrections to H.324 Annex F

7.4.1  Clarification to Section F.5.2.2.1 – Responder request to add additional connections

To clarify the order of the procedures of Annex F, a reference to Section F.5.2.1 is being added to section F.5.2.2.1 as follows:

[Begin Correction]
At any time after exchange of call information according to Section F.5.2.1, the responder may request that the initiator add physical connections. This shall be done using the MultilinkRequest.addConnection message in H.245.

…

[End Correction]
8.  Implementation clarifications

This section describes the procedures for using the supplementary services Call Hold and Explicit Call Transfer in H.324/ISDN. Implementation of these procedures is optional.

8.1  Procedures for Call Hold (CH)

The two procedures as described below should be used if a terminal supports the Call Hold supplementary service.

8.1.1  Invocation procedure for CH

Initial situation:
Terminal A is connected to terminal B. Either Terminal A or terminal B has established the call.

Objective:
Terminal A wishes to put terminal B on hold.

1) In case Multilink is used, terminal A should remove all but one B-channel connections from the H.Multilink Channel Set according to the Multilink procedures.

2) Terminal A should proceed with phase F of Annex D/H.324. The EndSessionCommand message should indicate to the far end that the terminal will be put on hold by signalling terminalOnHold in isdnOptions.

3) Terminal A should invoke the CH supplementary service by D-channel signalling, requesting the network to put all B-channel connections with terminal B on hold.

8.1.2  Retrieval after invocation of CH

Initial situation:
Terminal A has terminal B on hold. 

Objective: 
Terminal A wishes to retrieve the call with terminal B.

1) Terminal A should apply D-channel signalling to retrieve all the B-channel connections with terminal B.

2) Terminal A should initiate phase A of Annex D/H.324 starting with the execution of H.Dispatch, because the channel is already established.

3) Terminal A should add the additional B-channel connections to the H.Multilink Channel Set using the Multilink procedures.

NOTE - The CH procedures should only be used if both terminals A and B are H.324/I terminals.

8.2  Procedures for Explicit Call Transfer (ECT)

The procedure as described below should be used if a terminal supports the invocation of ECT.

Initial situation:
Terminal A is connected to terminal B. Either terminal A or terminal B has established the call.

Objective: 
Terminal A wishes to put terminal B on HOLD, make a call to terminal C and then connect terminal B to terminal C.

8.2.1  Invocation procedure for ECT

1) In case Multilink is used, terminal A should disconnect all but one B-channel connections with terminal B according to the Multilink procedures defined in Annex F/H.324.

2) Terminal A should put terminal B on hold according to the procedures of the CH supplementary service.

3) Terminal A should establish a call with terminal C. 

4) ECT should not be activated when terminal A does not succeed in establishing a call with terminal C or when terminal C is not a H.324/I terminal; appropriate indications should be given to the user(s).

5) In case Multilink is used, terminal A should disconnect all but one B-channel connections with terminal C according to the Multilink procedures defined in Annex F/H.324.

6) Terminal A should put terminal C on hold according to the procedures of the CH supplementary service.

7) Terminal A should invoke the ECT supplementary service by D-channel signalling, requesting the network to connect terminal B to C.

NOTE 1 - The procedure for ECT should only be used if all terminals A, B and C are H.324/I terminals. The implementation of ECT in case not all the terminals A, B and C are H.324/I terminals is left for further study.

NOTE 2 - The method used for addressing phone numbers in H.Multilink in case calls are transferred is left for further study.

NOTE 3 - The network provider may restrict the invocation of the ECT supplementary service to either the calling or the called terminal.

H.324 Recommendation Series Defect Report Form

DATE:


CONTACT INFORMATION

NAME:

COMPANY:

ADDRESS:

TEL:

FAX:

EMAIL:


AFFECTED RECOMMENDATIONS:


DESCRIPTION OF PROBLEM:


SUGGESTIONS FOR RESOLUTION:


NOTE - Attach additional pages if more space is required than is provided above.

� Contact:	Patrick Luthi (Editor)	Phone: +1 978-292-4354


		Fax: +1 978-292-3309


		E-mail: luthip@pictel.com





1
1

