

Q1-A08


H.324 Recommendation Series Defect Report Form

DATE:
24-Nov-2000

CONTACT INFORMATION

NAME:

COMPANY:

ADDRESS:

TEL:

FAX:

EMAIL:
Ari Hourunranta
Nokia Mobile Phones
P.O. Box 83
Fin-33721 Tampere, Finland

Ari.hourunranta@nokia.com

AFFECTED RECOMMENDATIONS:
H.324 version 2 (02/98)

DESCRIPTION OF PROBLEM:
In H.324 it is specified (secton 6.4.3):

When the entire H.223 multiplex is limited by the FlowControlCommand, or when the terminal has no information to send, the terminal shall send HDLC flags in place of logical channel information. The limit applies to the entire multiplex output, including opening flags, header octets, and inserted zero bits, but not including idle flags. 

However, in Annex C it is not mentioned what should be done if H.324/M terminal receives a FlowControlCommand. Since HDLC flags are not used for stuffing in levels 1 and above, it does not make sense to use them in flow control either.

SUGGESTIONS FOR RESOLUTION:
Clarify the use of flow control in mobile terminals in H.324 Annex C. For example, there could be a sentence in section C.6.1:

The stuffings sequences are used also when the entire H.223 multiplex is limited by the FlowControlCommand.

1
1

