- v -

	Question(s):
	1
	Meeting, date:
	Geneva, 20-30 January 2004

	Study Group:
	16
	Working Party:
	2
	Intended type of document (R-C-D-TD):
	TD

	Source:
	Editor (P. Luthi)

	Title: 
	Draft revised Implementor’s Guide for T.120-series Recommendations

	Contact:
	Patrick Luthi

Tandberg

Norway
	Tel:
+47 67 125 125

Fax:
+47 67 125 234

Email:
patrick.luthi@tandberg.net

	Contact:
	
	Tel:

Fax:

Email:

	Please don’t change the structure of this table, just insert the necessary information.


Introduction

This document is a text revision of the Implementor’s Guide for T.120 series recommendations (version 9), last approved in February 2002. This Implementer’s Guide supersedes the ITU-T T.120 Recommendation series Implementer’s Guide approved in February 2002.

The changes from the previous published version appear in the following sections:

7.12.1 Corrections to Section 9.2 (TD43/2)

	[image: image1.wmf]
	INTERNATIONAL  TELECOMMUNICATION  UNION

	
	

	ITU-T
	T.120
Implementors' Guide

	TELECOMMUNICATION
STANDARDIZATION  SECTOR
OF  ITU
	(30 January 2004)   

	
	SERIES T: TERMINALS FOR TELEMATIC SERVICES


	
	Data protocols for multimedia conferencing

Implementors' Guide




Summary

This document is a compilation of reported defects identified with the 1993-1999 editions of the ITU-T T.120-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected T.120-series Recommendations. This Implementer’s Guide supersedes the ITU-T T.120 Recommendation series Implementer’s Guide approved in February 2002.

Document History

	Version
	Date
	Description

	1
	15 February 1996
	Initial version - completed at the ITU-T Study Group 8 meeting, Geneva, 6‑15 February 1996

	2
	26 April 1996
	Updated at the Q.10/8 Rapporteur's meeting, Boston, 21-26 April 1996
Sections added: 7.5.4 and 7.5.5

	3
	9 August 1996
	Updated at the Q.10/8 Rapporteur's meeting, Santa Rosa, 5-9 August 1996
Sections added: 7.5.6

	4
	4 October 1996
	Updated at the Q.10/8 Rapporteur's meeting, Ismaning, 30 September - 4 October 1996

	5
	17 January 1997
	Updated at the Q.10/8 Rapporteur's meeting, Newport Beach, 
13‑17 January 1997

	6
	29 January 1998
	Updated at the ITU-T SG 16 meeting, Geneva, 26 January - 6 February 1998 Sections added: 6.7.1.1

	7
	20 May 1999
	Updated at the ITU-T SG 16 meeting, Santiago, 17-28 May 1999
Added sections 6.8.6.1 and 6.8.9.1

	8
	8 June 2001
	Eighth version – Completed at the ITU-T Study Group 16 Rapporteurs meeting.

	9
	15 February 2002
	Ninth version – Completed at ITU-T Study Group 16 Rapporteurs meeting.

	10
	30 January 2004
	Tenth version – Completed at ITU-T Study Group 16 Rapporteurs meeting.


Contact information

	ITU-T Study Group 16/Question 1 Rapporteur
	Patrick Luthi

Tandberg

Philip Pedersens vei 22

1366 Lysaker

Norway
	Tel:
+47 67 125 125
Fax: 
+47 67 125 234
Email: 
patrick.luthi@tandberg.net

	ITU-T Recommendation T.120-series Implementor’s Guide Editor
	Patrick Luthi
	Tel:
+47 67 125 125
Fax: 
+47 67 125 234
Email: 
patrick.luthi@tandberg.net


Table of Contents
11.  Introduction


12.  Scope


13.  Policies for updating this document


14.  Defect Resolution Procedure


15.  References


26.  Nomenclature


37.  Technical and Editorial Corrections


37.1  Technical and Editorial Corrections to ITU-T Recommendation T.120


37.1.2  Addition of new MCS static channel assignments


37.2  Technical and Editorial Corrections to ITU-T Recommendation T.120, Annex C


37.3  Technical and Editorial Corrections to ITU-T Recommendation T.121


37.4  Technical and Editorial Corrections to ITU-T Recommendation T.122


37.5  Technical and Editorial Corrections to ITU-T Recommendation T.123


37.5.1  Clarifications to Section B.9 – Connection Negotiation Protocol (CNP)


47.6  Technical and Editorial Corrections to ITU-T Recommendation T.124


47.6.1  Addition of Unicode Support for Conference Names and Passwords


57.7  Technical and Editorial Corrections to ITU-T Recommendation T.125


57.8  Technical and Editorial Corrections to ITU-T Recommendation T.126


57.9  Technical and Editorial Corrections to ITU-T Recommendation T.127


57.9.1  ASN.1 Errata


57.10  Technical and Editorial Corrections to ITU-T Recommendation T.128


57.11  Technical and Editorial Corrections to ITU-T Recommendation T.134


57.12
Technical and Editorial Corrections to ITU-T Recommendation T.135


57.12.1 Corrections to Section 9.2 - URST ASN.1 Module


67.13  Technical and Editorial Corrections to ITU-T Recommendation T.136


67.14  Technical and Editorial Corrections to ITU-T Recommendation H.282


7Annex: T.120 Recommendation Series Defect Report Form





Implementor's Guide for the ITU-T T.120 Recommendation series - Data Protocols for Multimedia Conferencing

1.  Introduction

This document is a compilation of reported defects identified with the 1993-1999 editions of the ITU-T T.120-series Recommendations. It is intended to be read in conjunction with the Recommendations to serve as an additional authoritative source of information for implementors. The changes, clarifications and corrections defined herein are expected to be included in future versions of affected T.120-series Recommendations.

The first version of the guide was produced following the February 1996 ITU-T Study Group 8 meeting. Wide distribution of this document is expected and encouraged.

2.  Scope

This Guide resolves defects in the following categories:

•
Editorial errors

•
Technical errors such as omissions or inconsistencies

•
Ambiguities.

In addition, the Guide may include explanatory text found necessary because of interpretation difficulties apparent from the defect reports.

This Guide will not address proposed additions, deletions, or modifications to the Recommendations that are not strictly related to implementation difficulties in the above categories. Proposals for new features should be made in the normal way through contributions to the ITU-T.

3.  Policies for updating this document

This document is managed by the ITU-T Study Group 16 Question 1 Rapporteur's Group. It can be revised at any recognized Q.1/16 Rapporteur's Group meeting provided the proposed revisions are unanimously accepted by the members of the group. A revision history cataloguing the evolution of this document is included.

4.  Defect Resolution Procedure

Upon discovering technical defects with any components of the T.120 Recommendations series, please provide a written description directly to the Q.1/16 Rapporteur.

5.  References

This document refers to the following T.120-series Recommendations:

•
ITU-T Recommendation T.120 (07/96) - Data Protocols for Multimedia Conferencing.

•
ITU-T Recommendation T.120, Annex C (02/98) - Lightweight Profiles for the T.120 Architecture.

•
ITU-T Recommendation T.121 (07/96) - Generic Application Template.

•
ITU-T Recommendation T.122 (02/98) - Multipoint Communication Service - Service Definition.

•
ITU-T Recommendation T.123 (05/99) - Network Specific Data Protocol Stacks for Multimedia Conferencing.

•
ITU-T Recommendation T.124 (02/98) - Generic Conference Control.

•
ITU-T Recommendation T.125 (02/98) - Multipoint Communication Service - Protocol Specification.

•
ITU-T Recommendation T.126 (07/97) - Multipoint Still Image and Annotation Protocol.

•
ITU-T Recommendation T.127 (08/95) - Multipoint Binary File Transfer Protocol.

•
ITU-T Recommendation T.128 (02/98) - Multipoint Application Sharing.

•
ITU-T Recommendation T.134 (02/98) - Text Chat Application Entity.

•
ITU-T Recommendation T.135 (02/98) - User-To-Reservation System Transactions within T.120 Conferences.

•
ITU-T Recommendation T.136 (05/99) - Remote Device Control Application Protocol.

•
ITU-T Recommendation H.282 (05/99)- A Far End Camera Control Protocol for Video Conferencing Using H.224.

6.  Nomenclature

In addition to traditional revision marks, the following marks and symbols are used to indicate to the reader how changes to the text of a Recommendation should be applied:

	Symbol
	Description

	[Begin Correction]
	Identifies the start of revision marked text based on extractions from the published Recommendations affected by the correction being described.

	[End Correction]
	Identifies the end of revision marked text based on extractions from the published Recommendations affected by the correction being described.

	...


	Indicates that the portion of the Recommendation between the text appearing before and after this symbol has remained unaffected by the correction being described and has been omitted for brevity.

	--- SPECIAL INSTRUCTIONS --- {instructions}
	Indicates a set of special editing instructions to be followed. 


7.  Technical and Editorial Corrections

7.1  Technical and Editorial Corrections to ITU-T Recommendation T.120

7.1.2  Addition of new MCS static channel assignments

Description:
T.120 Annex A1.1 contains a table of MCS Static Channel ID numbers. Two new Application Protocol Entities have been defined, each with a new Channel ID. The table is updated to add Channel IDs 11 and 12.

[Begin Correction]
A1.1
Static Channels

	Symbolic Name
	MCS Channel ID
	Description
	Recommendation

	GCC-CHANNEL-0
	1
	GCC Broadcast Channel
	T.124

	GCC-CHANNEL-1
	2
	Convenor Channel
	T.124

	SI-CHANNEL-0
	8
	MSIA Communications Channel
	T.126

	MBFT-CHANNEL-0
	9
	Control Channel
	T.127

	MBFT-CHANNEL-1
	10
	Data Channel
	T.127

	AS-CHANNEL-0
	11
	Application Sharing Channel
	T.128

	CHAT-CHANNEL-0
	12
	Chat Data Channel
	T.134


 [End Correction]

7.2  Technical and Editorial Corrections to ITU-T Recommendation T.120, Annex C

-None-

7.3  Technical and Editorial Corrections to ITU-T Recommendation T.121

-None-

7.4  Technical and Editorial Corrections to ITU-T Recommendation T.122

-None-

7.5  Technical and Editorial Corrections to ITU-T Recommendation T.123

7.5.1  Clarifications to Section B.9 – Connection Negotiation Protocol (CNP)

Text to clarify the usage of TPDU identifiers is being added to the overview of Section B.9 as follows:

[Begin Correction]
B.9.1
Overview

The PDUs defined in this protocol are based on their X.224/Class 0 counterparts. Both X.224 and CNP provide Connect Request, Connect Confirm, Disconnect Request, Data, and Error TPDUs. In addition, CNP provides a NonStandardPDU for protocol extensibility.
The CNP TPDUs defined in this Annex shall not be used outside the context of T.120 communications.
[End Correction]

7.6  Technical and Editorial Corrections to ITU-T Recommendation T.124

7.6.1  Addition of Unicode Support for Conference Names and Passwords

Description:
In order to better support these users of Unicode languages the definitions of ConferenceName and Password shall be modified with additional optional fields past the extension markers. These fields will allow the full support of Unicode strings for the identification of conference names and for conference passwords. The following are the proposed modifications to the ASN.1.

[Begin Correction]
ConferenceName ::= SEQUENCE

{


numeric

SimpleNumericString,


text


SimpleTextString OPTIONAL,


...,


unicode_text

TextString OPTIONAL
}

ConferenceNameSelector ::= CHOICE

{


numeric

SimpleNumericString,


text


SimpleTextString,


...,


unicode_text

TextString
}

Password ::= SEQUENCE

{


numeric

SimpleNumericString,


text


SimpleTextString OPTIONAL,


...,


unicode_text

TextString OPTIONAL
}

PasswordSelector ::= CHOICE

{


numeric

SimpleNumericString,


text


SimpleTextString,


...,


unicode_text

TextString
}

[End Correction]

7.7  Technical and Editorial Corrections to ITU-T Recommendation T.125

-None-

7.8  Technical and Editorial Corrections to ITU-T Recommendation T.126

-None-

7.9  Technical and Editorial Corrections to ITU-T Recommendation T.127

7.9.1  ASN.1 Errata

Description:
The following additions to the ASN.1 for the File-OfferPDU were omitted from the published text, as documented in the editor's errata sheet issued at the Paris Rapporteur meeting of 10/95.




The ASN.1 text shown below shall be added.

[Begin Correction]
File-OfferPDU ::= SEQUENCE

{


file-header


FileHeader,


data-channel-id

ChannelID,


file-handle


Handle,


roster-instance

INTEGER (0..65535) OPTIONAL,


file-transmit-token

TokenID OPTIONAL,


file-request-token

TokenID OPTIONAL,


file-request-handle

Handle OPTIONAL,


mbftID


UserID OPTIONAL,

compression-specifier
CompressionSpecifier OPTIONAL,


compressed-filesize

INTEGER OPTIONAL,


ack-flag


BOOLEAN,


-- True if acknowledgements










-- required


...

}

[End Correction]

7.10  Technical and Editorial Corrections to ITU-T Recommendation T.128

-None-

7.11  Technical and Editorial Corrections to ITU-T Recommendation T.134

-None-

7.12 Technical and Editorial Corrections to ITU-T Recommendation T.135

7.12.1 Corrections to Section 9.2 - URST ASN.1 Module

GCC-PROTOCOL is usually not associated with and object identifier (OID) except in T.135. Therefore, to be consistent with the other T.120-series documents, the OID reference in Section 9.2 is removed as shown below.

[Begin Correction]
…

-- Imports definitions from T.124

IMPORTS


TextString, SimpleTextString, DialingString, ExtraDialingString, SubAddressString,


H221NonStandardIdentifier, Key, NonStandardParameter, ChallengeResponse, ChallengeRequest,


Privilege

FROM GCC-PROTOCOL;

[End Correction]

7.13  Technical and Editorial Corrections to ITU-T Recommendation T.136

-None-

7.14  Technical and Editorial Corrections to ITU-T Recommendation H.282

-None-

Annex: T.120 Recommendation Series Defect Report Form

	DATE:
	

	CONTACT INFORMATION

NAME:

COMPANY:

ADDRESS:

TEL:

FAX:

EMAIL:
	

	AFFECTED RECOMMENDATIONS:
	

	DESCRIPTION OF PROBLEM:
	

	SUGGESTIONS FOR RESOLUTION:
	


Note - Attach additional pages if more space is required than is provided above.

                   
Attention: This is not a publication made available to the public, but an internal ITU-T Document intended only for use by the Member States of the ITU, by ITU-T Sector Members and Associates, and their respective staff and collaborators in their ITU related work. It shall not be made available to, and used by, any other persons or entities without the prior written consent of the ITU-T.

