	ITU - Telecommunications Standardization Sector

STUDY GROUP 16 Question 6

Video Coding Experts Group (VCEG)

_________________

Twelfth Meeting: Eibsee, Germany, 9-12 January, 2001
	Document  VCEG-L38
Filename: VCEG-L38.doc

Generated: 10.Jan ’01


	Question:
	Q.6/SG16 (VCEG)

	Source:
	Gisle Bjontegaard
Telenor Satellite Services
P.O.Box 6914  St.Olavs plass
N-0130 Oslo,  Norway
	
Tel:
Fax:
Email:
	
+47 23 13 83 81
+47 22 77 79 80
gisle.bjontegaard@telenor.com

	Title:
	Recommended Simulation Conditions for H.26L

	Purpose:
	Information


_____________________________
1 Introduction

The document is derived from the previous test condition document: Q15-I-62.  The main changes are:

· Reduction of number of QP values from 6 to 4 to ease the workload of testing and better match the most suitable range of bitrates.

· Reference to how subjective comparisons should be made based on D1 is removed since it is felt less relevant as PC based comparisons become more and more used.

This document contains a set of suggested common conditions for H.26L experiments. Those submitting contributions toH.26L are strongly encouraged to provide material using the conditions specified in this document t o ease comparison and with that to make their contribution more valuable.  These are considered as a minimum recommended set of sequences, bit-rates, frame rates, and spatial resolutions that each submission demonstration should contain. Those making a contribution  are free to provide additional demonstration material as appropriate. It is also expected that all results are derived from decoded bit-streams.

2 Conditions for coding of anchors and methods to be tested

The tests are performed using objective comparisons employing measures such as PSNR and bit-rate as well as subjective comparisons.

Anchor:

The exact conditions described in this document shall be used when encoding anchor material.  The anchor sequences are encoded according to the most recent TML software.

Those making a submission are strongly encouraged to down-load these bitstreams and to decode them to provide the anchor for both objective and subjective comparisons.

The quantizer value is held constant throughout the entire sequence (including the first frame). Also the frame rate is kept constant over the complete sequence. These settings have been chosen so as to minimize the impacts of the various parameters involved.

Method to be tested:

For coding of the first frame two methods could be used:

Preferably use the decoded first frame from the anchor if appropriate.

When other intra coding is used, the same number of bits should be used for the first frame as for the first frame of that anchor bitstream which comes closest to the method to be tested concerning overall bitrate.

3 Conditions for coding of anchors and methods to be tested

For objective comparisons, parameterized rate distortion curves shall be plotted with the corresponding tables given in the documents as well. These curves show luma PSNR versus bit-rate measured of the complete bit-stream. PSNR is measured as the arithmetic mean of the PSNR values of each frame - including the first INTRA frame. Bit-rate is measured as the average of all bits per frame - including the first INTRA frame -multiplied by 30/frameskip (kbit/s).  The plots shall have 0.5 dB grid lines on the PSNR axis.

Anchor bitstreams will be available for sequences, picture formats and frame skips indicated in the table.  The picture format (QCIF or CIF) and the number of frames in the sequence is indicated together with each sequence.  For all entries in the table bitstreams using QUANT of  16,20,24,28 are available.  Bitstreams with the QUANT in parenthesis shall be used for subjective comparisons.  The bitstreams used for subjective comparisons is a subset of the total number of bitstreams.  It is intended that this shall apply also in the future, but different subsets may be chosen.  Notice that all sequences are assumed to have 30 frames/second (even if some of the sequences were produced at a different frame rate).

	Frame Skip
	2
	1
	0

	Sequences
	Container, QCIF,300 (20)

News, QCIF,300 (24)

Foreman, QCIF,300 (24)
	Paris, CIF,300(16)

Silent Voice, QCIF,300(20)


	Mobile CIF 300(20)

Tempete CIF 260(28)


Together with the anchor bitstreams, tables with the following statistical data for each bitstreams will be made available:

· PSNR (separate for luma and chroma) for the whole sequence.

· PSNR (separate for luma and chroma) for the first frame.

· Bitrate for the whole sequence

· Number of bits for the first frame.

4 Display of sequences

For subjective comparisons the sequence under test shall be presented side-by-side with a  sequence  generated by the Test Model.  

File:VCEG-L38.doc
Page: 1
Date Printed: 20.01.2001

