	ITU - Telecommunications Standardization Sector
STUDY GROUP 16
Video Coding Experts Group (VCEG)
_________________
13th meeting, Austin, U. S., April 2-4, 2001
	Document  VCEG-M19
Filename:   VCEG-M19.doc
Generated: 27 March 2001


	Question:
	Q6/SG16 (VCEG)

	Source:
	Shijun Sun
Sharp Laboratories of America
5750 NW Pacific Rim Blvd.
Camas, WA 98607, U.S.A.

Louis Kerofsky
Sharp Laboratories of America
5750 NW Pacific Rim Blvd.
Camas, WA 98607, U.S.A.
	
Tel:
Fax:
Email:

Tel:
Fax:
Email:
	
+1 360 817 7531
+1 360 817 8436
ssun@sharplabs.com
+1 360 817 7644
+1 360 817 8436
lkerofsky@sharplabs.com

	Title:
	Reference Frame Prediction for Fast Motion Estimation

	Purpose:
	Information


_____________________________

1. Introduction

Motion estimation is the most demanding task of a video encoder. In the current H.26L testing model, multiple reference frames are allowed for the motion search. This improves the video coding efficiency by better motion compensation; however, it also significantly increases the computational burden on the encoder. For example, a motion search with 5 reference frames, which is a typical configuration in H.26L, could take up to 95% of the total computational power. 

Our solution to the complexity issue is to add dynamic constraints from encoder side so that only a subset of the reference frames is searched. The technique is based on the reference-frame prediction, where the motion search of each image block will follow the predictions by its neighbouring blocks. The advantage of the encoder-side approach is that we can keep the flexibility in the current H.26L decoder, which is going to be standardized, and still we can reduce the computation requirement for the motion estimation.

The description of our fast search technique is organized as the following. First, the method for reference-frame prediction is introduced. Then, we will show the experimental results followed by conclusions. This contribution is prepared in response to VCEG-L36, the “Proposed Investigations on Coding Performance and Complexity in H.26L.”

2. Reference-frame Prediction

It has been observed in our tests that when n (with n>1) reference frames are used in the motion search for an image block (the shaded area in Fig. 1) of a current frame, the probability of selecting the optimal reference block from a reference frame temporally close to the current frame (for example, f0) is much higher than the probability of choosing the reference block from a reference frame far away from the current frame (for example, fn-1). Therefore, while consuming a great deal of computational time, search through all the n reference frames for all the image blocks in the current frame will not give significant benefit in most cases. One straightforward solution to this computational complexity issue is to use less reference frames; however, the tradeoff is that many reference frames will not be used at that time and the video coding quality may have to suffer. We believe that the reference frame of a image block is highly correlated to that of its neighbouring blocks. To take advantage of this understanding, the reference-frame prediction technique is developed to adaptively control the number of reference frames during motion search.

[image: image1.wmf]current frame

……

……

f

0

f

1

f

2

f

n

-

2

f

n

-

1

Time flow

n

reference frames

current frame

……

……

f

0

f

1

f

2

f

n

-

2

f

n

-

1

Time flow

current frame

current frame

……

……

f

0

f

1

f

2

f

n

-

2

f

n

-

1

Time flow

n

reference frames


Figure 1. Motion search with multiple reference frames.

[image: image2.wmf]E

B

A

D

C

E

B

A

D

C


Figure 2. Image block E in the current image frame and its neighbouring image blocks.

Table 1. Experimental results of reference-frame prediction


[image: image3.wmf]sequences

number 

of frames

qp

relative 

computation 

savings

avsnry gain 

(sweet spot) 

dB

avsnry gain 

dB

16

56.15%

20

56.92%

24

57.88%

28

58.62%

16

57.80%

20

58.36%

24

58.75%

28

58.81%

16

53.03%

20

54.49%

24

55.65%

28

56.30%

16

57.95%

20

58.26%

24

58.42%

28

58.61%

16

55.97%

20

56.09%

24

56.52%

28

57.95%

16

43.15%

20

43.64%

24

46.02%

28

50.83%

16

44.18%

20

46.65%

24

50.29%

28

54.40%

container_qcif

100

news_qcif

100

foreman_qcif

100

silent_qcif

150

paris_cif

150

mobile_cif

300

tempete_cif

260

-0.0396

-0.0265

-0.0377

-0.0437

-0.1296

-0.1181

-0.1607

-0.1895

-0.1674

-0.163

-0.0651

-0.0567

-0.2085

-0.2184


In Fig. 2, block E represents an image macroblock in the current inter frame for which a motion search is going to be conducted; blocks A, B, C, and D are the possible image blocks adjacent to block E in the current frame whose reference image blocks have been chosen from reference frames fa, fb, fc, and fd, respectively. A prediction fp can be derived with p as the following


[image: image4.wmf]))

,

,

,

max(

,

1

min(

0

d

c

b

a

p

n

p

+

-

=

                                                                   (1)

where p0 is a pre-fixed positive integer parameter. And the motion search for image block E can be conducted within frames f0 to fp, which is a subset of all the n reference frames, so that the total computational burden can be significantly released. The prediction for fp can be very flexible. It can be made from various sets of the neighbouring image blocks. 

In a special case, if any of the neighbouring blocks used for prediction are out of the image frame or coded as intra blocks, the value of p will be set to (n-1), which means all of the reference frames will be considered during the motion search.

3. Experiments

Experiments have been conducted with the following parameters: n = 5 (corresponding to 5 reference frames); p0 = 1; ( = ½; w = 4; and w1 = 1. The testing condition follows the “Recommended Simulation Conditions for H.26L” in VCEG-L38.

Table 1 shows the testing results with the reference-frame prediction. The relative computational saving for motion search is estimated based on the number of searches, where one search corresponds to the motion search for one macroblock with one block mode within one reference frame. The reference-prediction method gives computation saving of 43-58% while causing very limited video quality degradation in terms of avsnry’s. Intuitively, setting p0 to 1 is a reasonable solution for the occlusion problems if fast motions are presented in the video sequence.

Detailed results of the experiments are available in VCEG-M19.xls. Experiments are all based on the H.26L TML-5.0 codec. Since there has been no significant change in the motion compensation section, we believe the similar results should be observed with the most up-to-date codec.

4. Conclusions

We have shown a technique that can significantly reduce the computations for motion search with almost no quality degradation by reducing the number of reference frames dynamically during motion search. Our fast motion estimation method can be very flexible by adjusting the control parameter p0. Our method can be combined with many traditional single-frame fast motion search methods to further reduce the computation requirement. 

We have shown a way to reduce the complexity for motion search and still maintain the flexibility for possible maximization of compression efficiency. Thus, we suggest that the reference frame selection be remained as an encoder-side issue for H.26L. We also suggest a consideration of adopting the proposed fast motion search into TML.
5. Legal Statement

Sharp has intellectual property related to this contribution and has chosen to subscribe to subclause 2.2 of the ITU-T patent policy under the condition of reciprocity.

References

[1] Michael J. Horowitz, “Proposed Investigation on Coding Performance and Complexity in H.26L,” document VCEG-L36, ITU- T Video Coding Experts Group (VCEG) Meeting, Eibsee, Germany, 9-12 January, 2001. 

[2] Gisle Bjontegaard, “Recommended Simulation Conditions for H.26L,” document VCEG-L38, ITU- T Video Coding Experts Group (VCEG) Meeting, Eibsee, Germany, 9-12 January, 2001. 

[3] Gisle Bjontegaard, "H.26L Test Model Long Term Number 5 (TML-5) draft0",  document Q15-K-59, ITU-T Video Coding Experts Group (Question 15) Meeting, Oregon, USA, 22-25 August, 2000.

[4] Gisle Bjontegaard, "H.26L Test Model Long Term Number 6 (TML-6) draft0",  document VCEG-L45, ITU-T Video Coding Experts Group (VCEG) Meeting, Eibsee, Germany, 9-12 January, 2001.

Appendix: RD Curves

[image: image5.emf]container_qcif

26

28

30

32

34

36

38

0 5000 10000 15000 20000 25000 30000 35000

TML

reference-frame

prediction


[image: image6.emf]news_qcif

26

28

30

32

34

36

38

40

0 10000 20000 30000 40000 50000 60000 70000

TML

reference-frame

prediction


[image: image7.emf]foreman_qcif

26

28

30

32

34

36

38

0 20000 40000 60000 80000 100000 120000

TML

reference-frame

prediction


[image: image8.emf]silent_qcif

26

28

30

32

34

36

38

0 10000 20000 30000 40000 50000 60000 70000 80000

TML

reference-frame

prediction


[image: image9.emf]paris_cif

26

28

30

32

34

36

38

0 50000 100000 150000 200000 250000 300000 350000 400000 450000

TML

reference-frame

prediction


[image: image10.emf]mobile_cif

24

26

28

30

32

34

36

0 500000 1000000 1500000 2000000 2500000

TML

reference-frame

prediction


[image: image11.emf]tempete_cif

26

28

30

32

34

36

38

0 200000 400000 600000 80000010000001200000140000016000001800000

TML

reference-frame

prediction

















PAGE  
6

_1044881537.xls
Sheet1

		sequences		number of frames		qp		savings		snr gain (sweet)		avsnr gain

		container_qcif		100		16		63.56%		-0.805		-0.7687

						20		65.12%

						24		66.16%

						28		66.98%

		news_qcif		100		16		58.15%		-0.5715		-0.5792

						20		58.15%

						24		60.43%

						28		62.48%

		foreman_qcif		100		16		35.36%		-0.2301		-0.2253

						20		42.29%

						24		49.22%

						28		53.43%

		silent_qcif		150		16		55.40%		-0.4806		-0.4435

						20		58.93%

						24		62.66%

						28		64.46%

		paris_cif		150		16		68.32%		-0.6485		-0.6428

						20		68.14%

						24		69.21%

						28		72.26%

		mobile_cif		300		16		64.89%		-0.462		-0.4727

						20		60.56%

						24		65.63%

						28		68.79%

		tempete_cif		260		16		46.29%		-0.4191		-0.4044

						20		59.25%

						24		66.84%

						28		70.59%





Sheet2

		sequences		number of frames		qp		relative computation savings		avsnry gain (sweet spot) dB		avsnry gain dB

		container_qcif		100		16		56.15%		-0.0396		-0.0265

						20		56.92%

						24		57.88%

						28		58.62%

		news_qcif		100		16		57.80%		-0.0437		-0.0377

						20		58.36%

						24		58.75%

						28		58.81%

		foreman_qcif		100		16		53.03%		-0.1181		-0.1296

						20		54.49%

						24		55.65%

						28		56.30%

		silent_qcif		150		16		57.95%		-0.1895		-0.1607

						20		58.26%

						24		58.42%

						28		58.61%

		paris_cif		150		16		55.97%		-0.0567		-0.0651

						20		56.09%

						24		56.52%

						28		57.95%

		mobile_cif		300		16		43.15%		-0.2184		-0.2085

						20		43.64%

						24		46.02%

						28		50.83%

		tempete_cif		260		16		44.18%		-0.163		-0.1674

						20		46.65%

						24		50.29%

						28		54.40%





Sheet3

		






_1036915456.unknown

