ITU - Telecommunications Standardization Sector
STUDY GROUP 16
Video Coding Experts Group (VCEG)
_________________
13th meeting, Austin, Texas, USA, April 2-4, 2001
Document  VCEG-M22
Filename:   VCEG-M22.doc
Generated: 27 Mar 2001

Question:
Q6/SG16

Source:
Jani Lainema, Marta Karczewicz
Nokia Research Center
6000 Connection Drive
Irving, TX 75039, U.S.A.

Tel:
Fax:
Email:

+1 972 894 4045
+1 972 894 4589
jani.lainema@nokia.com

Title:
Further Improvements on TML Loop Filtering

Purpose:
Proposal

_____________________________

1 Introduction

This document proposes modifications to the TML loop filter in order to improve the visual quality of decoded sequences.

1.1 Proposed Modifications

In low bitrates TML 5.9 produces occasional visual artefacts due to coarse quantization in transform coding, limited block size and rate-distortion optimizations. It turns out that with small adjustments to the current filter we can reduce these artefacts without sacrificing image sharpness. Proposed modifications to the TML filter are:

1. Increasing the Alpha parameter (to allow more pixels to be filtered)

2. Increasing the Beta parameter (to filter areas with higher frequencies)

3. Allow the maximum of two pixels to be filtered on one side of the block boundary regardless of the strength parameter (exception: for chroma this is allowed only if strength parameter on that side of the boundary is equal to 3). 

Proposed Alpha and Beta parameters are listed below and their relationship to the old ones is demonstrated in the Figures 1 and 2.

QP
0
1
2
3
4
5
6
7
8
9
10
11
12
13
14
15
16
17
18
19
20
21
22
23
24
25
26
27
28
29
30
31

(
0
0
0
0
0
0
0
0
6
6
6
14
14
14
23
33
33
44
55
67
80
93
120
134
163
178
224
273
306
374
426
498

(
0
0
0
0
0
0
0
0
3
3
4
5
5
6
7
7
8
9
10
11
12
13
14
15
16
17
18
19
20
21
22
23

Figure 1: The relationship between the proposed and TML 5.9 Alpha parameter.


[image: image1.wmf]0

100

200

300

400

500

7

9

11

13

15

17

19

21

23

25

27

29

31

QP

Alpha

Alpha TML 5.9

Alpha Proposed


Figure 2: The relationship between the proposed and TML 5.9 Beta parameter.


[image: image2.wmf]0

5

10

15

20

25

7

9

11

13

15

17

19

21

23

25

27

29

31

QP

Beta

Beta TML 5.9

Beta Proposed


In the results below proposed modifications are integrated to the proposed output of the Low Complexity Loop Filtering Core Experiment (VCEG-M21) and compared against TML 5.9 results. More exact simulation results and PSNR curves can be found in VCEG-M22.xls.

Table 1: Loop filter complexity with proposed changes (operations per Macroblock)
QP
Cont
Fore
News
Sile
Pari
Mobi
Temp
Average
Worst case

16
2193
6276
2536
2728
2592
5171
5318
3831
25984

20
1392
5843
2266
2510
2318
4603
4396
3333
25984

24
991
5390
1926
2224
1978
3513
3281
2758
25984

28
774
5061
1695
1886
1673
2672
2560
2332
25984

Average
1338
5643
2106
2337
2140
3990
3889
3063
25984

Table 2: Complexity relative to TML 5.9
QP
Cont
Fore
News
Sile
Pari
Mobi
Temp
Average
Worst case

16
0.98
0.89
0.90
0.91
0.83
0.82
0.84
0.88
0.91

20
0.96
0.88
0.89
0.91
0.84
0.77
0.83
0.87
0.91

24
0.94
0.88
0.87
0.94
0.86
0.72
0.82
0.86
0.91

28
0.95
0.88
0.87
0.92
0.84
0.66
0.79
0.84
0.91

Average
0.96
0.88
0.88
0.92
0.84
0.74
0.82
0.86
0.91

Table 3: PSNR differences with respect to TML 5.9

Cont
Fore
News
Sile
Pari
Mobi
Temp
Average

Luma Sweet Spot
-0.02
0.12
0.09
0.13
0.10
0.04
0.02
0.07

Luma Total
-0.02
0.11
0.07
0.13
0.09
0.05
0.02
0.06

Chroma Sweet Spot
N/A
0.12
N/A
0.14
0.21
0.16
0.14
0.16

Chroma Total
N/A
0.15
N/A
0.11
0.18
0.15
0.13
0.14

Table 4: Summary of results
Average Complexity Reduction
14%

Luma PSNR improvement (sweet spot)
0.07

Luma PSNR improvement
0.06

Chroma PSNR improvement (sweet spot)
0.16

Chroma PSNR improvement
0.14

2 Examples

Following figures demonstrate visual improvements in two extreme cases with proposed modifications to the TML 5.9 filter.

Figure 3: Part of the News sequence, QP = 24, 1 reference frame. TML 5.9 on the left, TML 5.9 with proposed changes on the right.

[image: image3.png]Yy




Figure 4: Part of the News sequence, QP = 24, 5 reference frames. TML 5.9 on the left, TML 5.9 with proposed changes on the right.

[image: image4.png]p— .!.





3 Summary

Visual comparison of the TML 5.9 with and without proposed changes will be provided in the Austin meeting. Results obtained suggest that the compression efficiency and visual quality can be improved by introducing small modifications to the TML loop filter. Given the results, authors of this contribution would like to ask Video Coding Experts Group to adopt the proposed changes to the next version of TML.

Patent Statement and Licensing Declaration

(Typically one per ITU-T Recommendation)

This declaration does not represent an implied license grant

Please return to:
Director
Place des Nations 

Telecommunication Standardization Bureau
CH-1211 Geneva 20, Switzerland

International Telecommunication Union
Fax: +41 22 730 5853

Patent Holder/Organization:

Legal Name
Nokia Inc.


Contact for license application:

Name & Department



Address







Tel.



Fax



E-mail



ITU-T Recommendation:

Number
H.26L


Title



3.1.1 Licensing declaration

The Patent Holder believes to hold granted patents and/or pending applications, whose use would be required to implement the above ITU-T Recommendation and hereby declares, in accordance with the Statement on ITU-T Patent Policy (see ITU-T web site), that (check one box only).




[image: image5.wmf]
1
The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation.





[image: image6.wmf] X


2
The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation.

Such negotiations are left to the parties concerned and are performed outside the ITU-T.




[image: image7.wmf]
3
The Patent Holder is unwilling to grant licenses according to the provisions of either 1 or 2 above. In this case, the following information must be provided as part of this declaration:

· patent registration/application number;
· an indication of which portions of the Recommendation are affected.
· a description of the patent claims covering the Recommendation;

Signature

Organization



Name of authorized person



Title of authorized person



Signature



Place, Date






















_1046783302.doc


 X












_1046784740.xls
Chart2

		7		7

		8		8

		9		9

		10		10

		11		11

		12		12

		13		13

		14		14

		15		15

		16		16

		17		17

		18		18

		19		19

		20		20

		21		21

		22		22

		23		23

		24		24

		25		25

		26		26

		27		27

		28		28

		29		29

		30		30

		31		31



Beta TML 5.9

Beta Proposed

QP

Beta

0

0

3

3

3

3

3

4

4

5

4

5

4

6

6

7

6

7

6

8

7

9

8

10

8

11

9

12

9

13

10

14

10

15

11

16

11

17

12

18

12

19

13

20

13

21

14

22

14

23



PSNR differences

		

				Originals:				Proposed

				alpha		beta		alpha		beta

		0		0		0		0		0

		1		0		0		0		0

		2		0		0		0		0

		3		0		0		0		0

		4		0		0		0		0

		5		0		0		0		0

		6		0		0		0		0

		7		0		0		0		0

		8		3		3		6		3

		9		3		3		6		3

		10		3		3		6		4

		11		7		4		14		5

		12		7		4		14		5

		13		7		4		14		6

		14		12		6		23		7

		15		17		6		33		7

		16		17		6		33		8

		17		22		7		44		9

		18		28		8		55		10

		19		34		8		67		11

		20		40		9		80		12

		21		47		9		93		13

		22		60		10		120		14

		23		67		10		134		15

		24		82		11		163		16

		25		89		11		178		17

		26		112		12		224		18

		27		137		12		273		19

		28		153		13		306		20

		29		187		13		374		21

		30		213		14		426		22

		31		249		14		498		23





PSNR differences

		



Alpha TML 5.9

Alpha Proposed

QP

Alpha



		



Beta TML 5.9

Beta Proposed

QP

Beta




_1046782232.xls
Chart1

		7		7

		8		8

		9		9

		10		10

		11		11

		12		12

		13		13

		14		14

		15		15

		16		16

		17		17

		18		18

		19		19

		20		20

		21		21

		22		22

		23		23

		24		24

		25		25

		26		26

		27		27

		28		28

		29		29

		30		30

		31		31



Alpha TML 5.9

Alpha Proposed

QP

Alpha

0

0

3

6

3

6

3

6

7

14

7

14

7

14

12

23

17

33

17

33

22

44

28

55

34

67

40

80

47

93

60

120

67

134

82

163

89

178

112

224

137

273

153

306

187

374

213

426

249

498



PSNR differences

		

				Originals:				Proposed

				alpha		beta		alpha		beta

		0		0		0		0		0

		1		0		0		0		0

		2		0		0		0		0

		3		0		0		0		0

		4		0		0		0		0

		5		0		0		0		0

		6		0		0		0		0

		7		0		0		0		0

		8		3		3		6		3

		9		3		3		6		3

		10		3		3		6		4

		11		7		4		14		5

		12		7		4		14		5

		13		7		4		14		6

		14		12		6		23		7

		15		17		6		33		7

		16		17		6		33		8

		17		22		7		44		9

		18		28		8		55		10

		19		34		8		67		11

		20		40		9		80		12

		21		47		9		93		13

		22		60		10		120		14

		23		67		10		134		15

		24		82		11		163		16

		25		89		11		178		17

		26		112		12		224		18

		27		137		12		273		19

		28		153		13		306		20

		29		187		13		374		21

		30		213		14		426		22

		31		249		14		498		23





PSNR differences

		



Alpha TML 5.9

Alpha Proposed

QP

Alpha



		



Beta TML 5.9

Beta Proposed

QP

Beta




