ITU - Telecommunications Standardization Sector

STUDY GROUP 16 Question 6

Video Coding Experts Group (VCEG)

_________________

15th Meeting: Pattaya, Thailand, 4-6 Dec., 2001
Document  VCEG-O32
Filename: VCEG-O32.doc

Generated: 26 November ’01

Question:
Q.6/SG16 (VCEG)

Source:
Mathias Wien 
Institut und Lehrstuhl für Nachrichtentechnik 
RWTH Aachen 
52074 Aachen, Germany

Ulrich Benzler
Robert Bosch GmbH
FV/SLM, PO Box 777777
31132 Hildesheim, Germany

Tel:
Fax:
Email:

++49-241-807681
++49-241-8888196
wien@ient.rwth-aachen.de



++49-5121-49-3087
++49-5121-49-173087
ulrich.benzler@de.bosch.com

Title:
Interlaced Coding with Frame Motion Compensation and Field Update Coding

Purpose:
Proposal

_____________________________
Introduction

Interlaced coding was proposed for H.26L at the September 2001 meeting in Santa Barbara. In the output document VCEG-N85, a core experiment was set up to evaluate several techniques for interlaced coding for H.26L. These techniques include frame based coding (ignoring the interlaced structure of the source as a reference), macroblock based indication of frame-based or field-based coding (Inter-MFFA), and a Ref-for-each-MV technique, which allows for signaling the appropriate reference frame or field separately for each motion vector.

Here, we propose another possibility of interlaced coding with H.26L, that combines field and frame based coding. While the motion compensation remains unchanged, update coding is performed on a field basis. In doing so, the motion is efficiently coded using the techniques already incorporated into the TML. Since frame based transform coding of an interlaced signal might introduce artificial high frequency signal components into the prediction error, the update could be efficiently encoded on a field basis. The proposal is to have frame-based motion compensation and Field Transform Coding (FTC) for the update.

At this early stage, the proposed technique is employed for the luma component of motion compensated inter macroblocks only. 

The performance of the interlaced coding scheme can be increased by the proposed technique. Especially, in combination with the concept of Adaptive Block Transforms, a remarkable gain of 10% bitrate savings or 0.5dB PSNR gain on average can be achieved.

Field Transform Coding

Only few changes have to be incorporated into the existing TML scheme in order to apply the field transform coding (FTC) for update encoding. For motion estimation the calculation of the cost criterion has to be revisited. For encoding the update, the prediction error has to be reassembled according to the field structure of the video material. The de-quantized coefficients are interleaved again for reconstruction of the decoded signal.

For encoding, the prediction error is de-interleaved into top and bottom field. This reordering is carried out on a macroblock basis. The proper handling of the subblocks of the prediction error is provided by a remapping of the macroblock modes for encoding of the prediction error. The reordering of the macroblock is depicted in Figure 1. The remapping for encoding of the prediction error is given in Table 1. Some frame based macroblock modes lead to transform blocks in the fields that contain boundaries of two MC blocks (e.g. frame 4x4 blocks). Therefore some of the TML macroblock modes are excluded for interlaced coding. The motion vectors are coded according to the TML macroblock mode, While the prediction error is coded according to the mapped mode.

[image: image11.wmf]Tabel 1 : Mapping of the TML motion compensation macroblock modes to the new field macroblock modes used for interlaced transform coding of the prediction error.

16x16
16x8

16x8
16x4

8x16
8x8

8x8
8x4

8x4
not allowed

4x8
4x4

4x4
not allowed

Motion Estimation

In the TML, the SATD is used as a cost criterion for motion estimation. The SATD simulates the effect of the succeeding transform coding stage and generally gives an improved performance for motion estimation. 

The field structure of the macroblock has to be taken into account for SATD calculation. The smallest block size used for motion compensation is 4x8 pixels. I.e., the SATD can be calculated on two interleaved 4x4 blocks. The other block sizes are aggregations of frame 4x8 blocks and can be are treated accordingly.

Transform Coding

For transform coding, the full macroblock is reordered as depicted in Figure 1. Then, the mapped macroblock modes for prediction error coding are applied to the prediction error signal. The prediction error is transform coded using the TML 4x4 transform. The CBP is applied to the de-interleaved macroblock.

Application of Adaptive Block Transforms

The concept of Adaptive Block Transforms (ABT) can easily be incorporated into the proposed technique. For frame based inter coding with ABT, the transform block size is connected to the block size used for motion compensation. Here, the interlaced macroblock mode mapping an the reordering of the coefficients before transform coding which were introduced for the 'TML case' in the paragraph above are used as well. 

For ABT, macroblock mode 2 is additionally excluded from the set of possible macroblock modes to avoid encoding of 16x4 blocks. The macroblock mode mapping for ABT is given in Table 2 below:

Tabel 2 : Mapping of the motion compensation macroblock modes to the new modes used for  ABT transform coding of the prediction error. 

16x16
16x8

16x8
not allowed

8x16
8x8

8x8
8x4

8x4
not allowed

4x8
4x4

4x4
not allowed

Simulation Results

The proposed technique was integrated into the TML-8.0 software. In this preliminary test, FTC was employed only for the luma component of motion compensated macroblocks. Three different settings were evaluated. As proposed in [VCEG-N85], the non-modified TML coder was used as a reference:

TML 
: the original frame based TML-8.0 coder

TML-IL
: FTC with the TML 4x4 transform

ABT-IL
: FTC with Adaptive Block Transforms 

The VQEG test sequences Canoa Valesia (src5), F1 Car (src6), Rugby (src9), and Football  (src19) were used in the experiments. The original CbYCrY 422 sequences were converted to YCbCr 420 using Peter Borgwards conversion tool 422_to_420_fullres.

The following settings were used throughout the experiments. The restrictions on search range motion resolution etc were imposed to reduce the simulation time. 

MV resolution
¼ pel

UseHadamard
ON

Reference Frames
3

Search Range
16

Symbol Mode
CABAC

Restrict Search Range
2

RD optimization
ON

QP
16,20,24,28

Frame Rate
30Hz

Frame Structure
IPPP…

Table 3 gives the difference in terms of rate and PSNR as calculated using the Bjontegaard measurement method [VCEG-M33]. Both, TML-IL and ABT-IL are compared to the performance of the original TML coder. The corresponding RD plots are given in the Appendix.

Test Sequences
ABT-IL average % 
bit savings (avsnr)
ABT-IL average dB
PSNR gain (avsnr)
TML-IL average % 
bit savings (avsnr)
TML-IL average dB
PSNR gain (avsnr)

Canoa SRC5
8.80
0.326
-1.91
-0.068

F1 Car SRC6
8.49
0.370
-1.04
-0.047

Rugby SRC9
12.01
0.605
3.56
0.167

Football SRC19
13.92
0.634
3.74
0.159

average 
10.81
0.48
1.09
0.05

Table 1: Objective comparison between the two interlaced proposals and TML intra coding.

Discussion

The RD results reveal a comparable performance of TML and TML-IL. On some sequences, FTC results in a gain, on some sequences, there is a slight drop in terms of RD performance. Combining FTC with ABT leads to a largely improved performance resulting in rate savings of about 10% on average or an increased PSNR of almost 0.5 dB.

Conclusion

The concept of Field Transform Coding for update coding of frame motion compensated video was presented. FTC facilitates interlaced video coding without increasing the number of transmitted motion vectors. Preliminary results show a partially very promising performance, especially in combination with ABT coding of the update. 

We propose to incorporate FTC into the upcoming Interlaced Core Experiments and to evaluate further application of ABT for field based coding. Additionally, Intra FTC has to be addressed.

References

[VCEG-N10]
Gisle Bjontegaard, "H.26L Test Model Long Term Number 8 (TML-8) draft0," 
document VCEG-N10, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[VCEG-N49]
M. Wien and A. Dahlhoff, 'New ABT Results using CABAC', ITU-T Q6/16 VCEG, 
Doc. VCEG-N49r1, Santa Barbara, CA, Sept. 2001.

[VCEG-N85]
P. Borgwardt, 'Core Experiment on Interlaced Video Coding', ITU-T Q6/16
VCEG, Doc. VCEG-N85, Santa Barbara, CA, Sept. 2001.

[VCEG-M33]
Gisle Bjontegaard, 'Calculation of average PSNR differences between RD 
curves,' document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

Appendix – Rate/Distortion Plots

[image: image12.wmf]1000

2000

3000

4000

5000

6000

7000

8000

9000

10000

27

27.5

28

28.5

29

29.5

30

30.5

31

31.5

32

32.5

33

33.5

34

34.5

35

Rate [kbit/s]

PSNR [dB]

canoa src5 420

TML80 prog

ABT-IL

TML-IL


canoa src5 420 TML80 prog

 QP     Rate
PSNR

 28  1311710
27.09

 24  2743004
29.41

 20  5030387
31.64

 16  9046848
33.90

canoa src5 420 ABT-IL

 QP     Rate
PSNR

 28  1518604
27.83

 24  2873366
29.95

 20  5119868
32.05

 16  9204045
34.07

canoa src5 420 TML-IL

 QP     Rate
PSNR

 28  1368155
27.13

 24  2798088
29.39

 20  5013850
31.58

 16  8866126
33.79

[image: image13.wmf]0

2000

4000

6000

8000

10000

12000

27

27.5

28

28.5

29

29.5

30

30.5

31

31.5

32

32.5

33

33.5

34

34.5

35

35.5

36

Rate [kbit/s]

PSNR [dB]

f1 car src6 420

TML80 prog

ABT-IL

TML-IL


f1 car src6 420 TML80 prog

 QP     Rate
PSNR

 28  1253856
27.23

 24  3305553
30.03

 20  6243582
32.77

 16 10381769
35.57

f1 car src6 420 ABT-IL

 QP     Rate
PSNR

 28  1542982
27.84

 24  3255121
30.30

 20  5853608
32.96

 16  9478993
35.59

f1 car src6 420 TML-IL

 QP     Rate
PSNR

 28  1286520
27.25

 24  3338698
29.98

 20  6173459
32.70

 16 10089770
35.48

[image: image14.wmf]1000

2000

3000

4000

5000

6000

7000

8000

9000

10000

27

27.5

28

28.5

29

29.5

30

30.5

31

31.5

32

32.5

33

33.5

34

34.5

35

35.5

36

Rate [kbit/s]

PSNR [dB]

rugby src9 420

TML80 prog

ABT-IL

TML-IL


rugby src9 420 TML80 prog

 QP     Rate
PSNR

 28  1803159
27.33

 24  3430522
29.85

 20  5908684
32.44

 16  9652751
35.21

rugby src9 420 ABT-IL

 QP     Rate
PSNR

 28  1852278
27.99

 24  3341713
30.32

 20  5714555
32.90

 16  9050441
35.48

rugby src9 420 TML-IL

 QP     Rate
PSNR

 28  1756527
27.45

 24  3301606
29.85

 20  5666296
32.39

 16  9251848
35.08

[image: image15.wmf]0

1000

2000

3000

4000

5000

6000

28

28.5

29

29.5

30

30.5

31

31.5

32

32.5

33

33.5

34

34.5

35

35.5

36

36.5

37

Rate [kbit/s]

PSNR [dB]

football src19 420

TML80 prog

ABT-IL

TML-IL


football src19 420 TML80 prog

 QP     Rate
PSNR

 28  1016905
28.82

 24  1916658
31.20

 20  3299696
33.50

 16  5469200
35.91

football src19 420 ABT-IL

 QP     Rate
PSNR

 28  1039183
29.52

 24  1872663
31.72

 20  3229650
34.06

 16  5196875
36.32

football src19 420 TML-IL

 QP     Rate
PSNR

 28   987237
29.00

 24  1832071
31.21

 20  3163234
33.42

 16  5251860
35.77

JVT Patent Disclosure Form

International Telecommunication Union
Telecommunication Standardization Sector
International Organization for Standardization
International Electrotechnical Commission  

[image: image1.wmf]
[image: image2.png]1S0
NS




[image: image3.png]




Joint Video Coding Experts Group - Patent Disclosure Form
(Typically one per contribution and one per Standard | Recommendation)

Please send to:


JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA


Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

Submitting Organization or Person:

Organization name
Institut und Lehrstuhl für Nachrichtentechnik, RWTH Aachen


Mailing address
Institut und Lehrstuhl für Nachrichtentechnik

RWTH Aachen

53074 Aachen, Germany


Country
Germany


Contact person
Mathias Wien


Telephone
+49-241-8027681


Fax
+49-241-8888196


Email
wien@ient.rwth-aachen.de


Place and date of submission
Pattaya, December 4th 2001


Relevant Recommendation | Standard and, if applicable, Contribution:

Name (ex: “JVT”)
JVT


Title
Interlaced Coding with Frame Motion Compensation and Field Update Coding


Contribution number
VCEG-O32






(Form continues on next page)

Disclosure information – Submitting Organization/Person  (choose one box)




[image: image4.wmf]
2.0
The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.

or,

The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.  In which case,



[image: image5.wmf]
2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation | Standard.




[image: image6.wmf]
2.2
The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation | Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation | Standard.


Such negotiations are left to the parties concerned and are performed outside the ITU | ISO/IEC.





[image: image7.wmf]
2.2.1
The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same.




[image: image8.wmf]
2.3
The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above.  In this case, the following information must be provided as part of this declaration:

· patent registration/application number;
· an indication of which portions of the Recommendation | Standard are affected.
· a description of the patent claims covering the Recommendation | Standard;

In the case of any box other than 2.0 above, please provide the following:

Patent number(s)/status



Inventor(s)/Assignee(s)



Relevance to JVT



Any other remarks:



(please provide attachments if more space is needed)



(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

Disclosure information – Third Party Patents (choose one box)




[image: image9.wmf]
3.1
The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation | Standard or Contribution.



[image: image10.wmf]
3.2
The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation | Standard or Contribution.



For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information:



3rd party name(s)



Mailing address



Country



Contact person



Telephone



Fax



Email



Patent number/status



Inventor/Assignee



Relevance to JVT







Any other comments or remarks:



Figure 1  Reordering of the pixels of a 16x16 macroblock from frame into field structure. The subblock tiling of the current macroblock mode is applied at the transform stage.











File:VCEG-O32.doc
Page: 12
Date Printed: 06.12.01

_1069141495.doc



