| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: 19–26 June 2015, Warsaw, Poland | Document VCEG-AZ05 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | E.Alshina, A.Alshin, J.-H. Min, K. Choi, A.Saxena, M. Budagavi Samsung Electronics | Tel: Email: | +821030261305 elena_a.alshina@samsung.com |
| Title: | Known tools performance investigation for next generation video coding |  |  |
| Purpose: | proposal |  |  |

_____________________________

Abstract

In this contribution, several coding tools are evaluated on top of the HEVC version 1 together with tools proposed for HM-KTA. All these coding tools have been studied during HEVC development. Minor modification of tools is needed for efficient work together with already existed in HM-KTA tools. With the investigated coding tools, the coding performance of HM-KTA reaches 14% and 10%, on average for random access and all intra configurations respectively.

- Introduction

Considering industry requests for necessity of further coding efficiency improvements, the Study Group 16 established three "ad hoc groups" (AHGs) related to next generation video coding standard during Strasbourg 2014 meeting. As part of the AHG on Coding Efficiency Improvement activity the technical work on coding performance improvement over HEVC version 1 started at Geneva 2015 meeting. Proposal from Qualcomm [1] becomes the starting point for HM-KTA (rc1).

Proposal [1] contains straight-forward extensions of HEVC design such as:

- Enlarged CU sizes (largest supported CTU is 512×512),

- Enlarged transform sizes (largest supported TU is 64×64);

and several tools studied during HEVC or HEVC standard extensions development:

- Adaptive loop filter (ALF),

- Advanced Temporal Motion Vector Prediction,

- Cross Component Prediction (also known as LM-Chroma mode),

- Overlapped Block Motion Compensation (OBMC),

- Adaptive Multiple Transform (AMT).

In this contribution one more straight-forward extensions of HEVC design:

- Finer granularity arbitrary directional intra prediction,

and 4 coding tools proposed for HEVC before:

- Bi-directional optical flow [2]

- Adaptive secondary transform [3],

- Multi-parameter Intra prediction [4],

- Multi-parameter CABAC probability up-date [5]

are applied. By adding mentioned tools HM-KTA reaches significant gain over HEVC version 1: 10%, 14%, 10% and 13% for all intra, random access, low-delay and low-delay-P configurations respectively.

- Technology description

The majority of tools under this study is targeting for Intra coding improvement. The main gain from multi-parameter CABAC probability up-date is also observed in I-slices. One tool (Bi-directional optical flow) is applied in random access test scenario only.

  - Finer granularity arbitrary directional intra prediction

Intra predictions set in HEVC consist of Planar, DC and ADI – Arbitrary Direction Intra (as it called in [4]). ADI is generic way to predict samples inside the block referring to the reconstructed boundary. Boundary signal is copied along direction. Intra prediction directions in HEVC are almost uniformly split each quarter into 8 sectors. Here we extend amount of angles and use 16 sectors in each quarter (Fig. 1). Performance impact of this tool is roughly 0,4% in all-intra test. This straight-forward extension of HEVC design makes sense together with block size enlarging.

Fig. 1. Finer granularity of arbitrary directional intra prediction.

  - Bi-directional optical flow

This tool is sample level motion refinement along optical flow trajectory under assumption of steady motion. It is applied only for “truly” bi-predicted PUs (only if POC information indicates that current picture is between L0 and L1 Reference pictures). Motion vector refinement {vx, vy} is calculated on a fly (not signalled) to minimize discrepancy between values A and B (Fig. 2). With {vx, vy} bi-prediction

is modified as follows:

,

.

where  are gradients of predictions  coming from L0 and L1 and is regularization parameter. More details about this tool can be found in [2].

Fig. 2. Bi-directional optical flow.

If this tool is applied then the portion of large-size PUs becomes higher. The fine motion inside large bi-predicted PU can be handled by BIO. This results in saving partitioning and MC information and provides ~2% performance gain in random access test scenario.

  - Adaptive secondary transform

Secondary transform (also known as Rotational transform ROT) is applied between core full-size transform and quantization [3] and partially redistributes energy between transform coefficients. One of 4 variants of predetermined secondary transforms which provides the best energy compaction in CU is applied and signalled to decoder with 2 bits. It also can be seen as adaptive quantization. Tool is applied together with HEVC core transform and together with Adaptive Multiple Transform from [1].  The only difference with [3] is secondary transform size. In tested version it is 44 for all TU sizes (not 88 for large TUs). This restriction is applied for better adjustment with 44 coefficients sub-groups coding in HEVC (Fig.3). Gain provided by this tool in all-intra test is ~1,2%.

invTransform

invROT

invQuant

Fig. 3. Secondary transform at decoder.

  - Multi-parameter Intra prediction

This tool is post-processing for HEVC Intra prediction which invokes additional smoothing with decoded boundary [4]:

,

where outside of the block  is equal to reconstructed signal (Fig. 4)

if  .

The strength of this post-processing (parameters) is controlled on CU level and signalled with up to 2 bits. Gain provided by this tool in all-intra test is ~1,4%.

Fig. 4. Multi-parameter Intra prediction.

  - Multi-parameter CABAC probability up-date

Probability up-date in HEVC CABAC uses exponential smoothing:

.

Here p is current probability, y is recently decoded bin, is up-dated probability, W is “window size” (amount of previously decoded affected current bin probability up-date). In HEVC CABAC window-size W19. A longer window results is more accurate probability up-date. For quicker probability adjustment in case of trend change shorter window is preferable.

Multi-parameter probability up-date for CABAC [5] is analog of bi-prediction where two probability up-dates with long and with short window are combined into one:

; ; .

For simplicity of implementation window sizes were selected as power of 2: .

This tool is efficient for all test scenarios contributing 0.8% for all-intra and 0.5-0.6% gain in motion compensation test scenarios.

- Experimental results

Mentioned tools were implemented on top of HM-KTArc1 and tested vs HM 14.0. Simulations were conducted following the JCT-VC common test condition used during HEVC development [6].

Table 1: Coding performance improvement over HM14

| HM-14.0 | HM-KTArc1 |  |  |  |  | "+investigated tools" |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main |  |  |  |  | All Intra Main |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -8,8% | -23,8% | -18,8% |  |  | -12,3% | -28,2% | -23,9% |
| Class B | -6,4% | -11,7% | -8,6% |  |  | -9,9% | -16,5% | -14,3% |
| Class C | -6,5% | -11,8% | -13,9% |  |  | -9,6% | -16,2% | -18,5% |
| Class D | -4,8% | -10,1% | -10,4% |  |  | -7,2% | -13,1% | -13,9% |
| Class E | -7,3% | -12,3% | -14,2% |  |  | -11,0% | -18,3% | -20,0% |
| Overall | -6,7% | -13,9% | -12,9% |  |  | -10,0% | -18,4% | -17,8% |
| Class F | -7,0% | -11,6% | -11,1% |  |  | -8,8% | -14,1% | -13,4% |
|  | Random Access Main |  |  |  |  | Random Access Main |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class A | -12,5% | -29,5% | -24,7% |  |  | -15,3% | -32,6% | -27,9% |
| Class B | -10,4% | -15,4% | -11,5% |  |  | -14,2% | -19,7% | -16,1% |
| Class C | -9,1% | -12,4% | -14,8% |  |  | -12,8% | -16,4% | -18,6% |
| Class D | -9,6% | -11,0% | -11,5% |  |  | -13,7% | -14,9% | -15,1% |
| Overall | -10,4% | -17,0% | -15,4% |  |  | -14,0% | -20,8% | -19,2% |
| Class F | -6,6% | -12,6% | -12,0% |  |  | -8,6% | -14,8% | -14,3% |
|  | Low delay B Main |  |  |  |  | Low delay B Main |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class B | -8,9% | -10,2% | -8,5% |  |  | -9,8% | -11,3% | -9,6% |
| Class C | -8,6% | -7,9% | -8,6% |  |  | -9,3% | -8,7% | -9,5% |
| Class D | -8,1% | -6,2% | -6,4% |  |  | -8,4% | -6,7% | -7,0% |
| Class E | -10,7% | -13,4% | -17,5% |  |  | -12,0% | -16,8% | -20,1% |
| Overall | -9,0% | -9,2% | -9,7% |  |  | -9,8% | -10,5% | -10,9% |
| Class F | -5,5% | -9,8% | -9,1% |  |  | -6,2% | -10,3% | -10,0% |
|  | Low delay P Main |  |  |  |  | Low delay P Main |  |  |
|  | Y | U | V |  |  | Y | U | V |
| Class B | -13,7% | -14,2% | -11,3% |  |  | -14,6% | -15,3% | -12,4% |
| Class C | -10,1% | -9,4% | -10,6% |  |  | -10,8% | -10,3% | -11,5% |
| Class D | -9,4% | -7,6% | -7,9% |  |  | -9,9% | -8,5% | -8,2% |
| Class E | -14,4% | -16,3% | -20,0% |  |  | -15,7% | -19,8% | -22,3% |
| Overall | -11,9% | -11,8% | -11,9% |  |  | -12,7% | -13,2% | -13,0% |
| Class F | -11,9% | -11,8% | -11,9% |  |  | -12,7% | -13,2% | -13,0% |

Table 1 shows the performance improvement over HEVC version 1 provided by HM-KTArc1 [1] together with tools under this study. As mentioned above tested tools mainly provide improvement in all-intra and random access test cases.

- Conclusion

In this contribution further improvement of coding efficiency is demonstrated. Together with investigated tools HM-KTA reaches significant gain over HEVC version 1: 10%, 14%, 10% and 13% for all intra, random access, low-delay and low-delay-P configurations respectively.

- Reference

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao , Coding tools investigation for next generation video coding COM16-C 806-E, Jan. 2015, Geneva

- A. Alshin, E. Alshina, “Bi-directional optical flow” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C204, Guangzhou, China, 10-15 October, 2010.

- E. Alshina, A. Alshin, F. Fernandes, A. Saxena, V. Seregin, Z. Ma, W.-J. Han (Samsung), “CE7: Experimental results of ROT by Samsung” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E389, Geneva, CH, 16-23 March, 2011.

- K. McCann, W.-J. Han, I.-K. Kim, J.-H. Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y.-M. Hong, M.-S. Cheon, N. Shlyakhov, “Video coding technology proposal by Samsung (and BBC)” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-A124, Dresden, Germany, 15-23 April, 2010.

- A. Alshin, E. Alshina, “Multi-parameter probability up-date for CABAC” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F254, Torino, Italy, 14-22 July, 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Doc. JCTVC-L1100, 12th Meeting.

________________________