| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: 19–26 June 2015, Warsaw, Poland | Document VCEG-AZ07_v2 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Qualcomm Incorporated | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Title: | Further improvements to HMKTA-1.0 |  |  |
| Authors: | Jianle Chen, Wei-Jung Chien, Marta Karczewicz, Xiang Li, Hongbin Liu, Amir Said Li Zhang, Xin Zhao |  |  |
| Purpose: | Proposal |  |  |

_____________________________

Abstract

In this contribution, several new coding tools, as well as improvements of the existing tools, are proposed on top of HMKTA-1.0. Compared to HMKTA-1.0, the proposed tools and improvements achieve 2.9%, 6.7%, and 4.9% luma BD rate reduction, on average for all intra, random access and low delay configurations, respectively. The overall coding gain reaches 16.5% on average for random access when compared to HM14.0.

# Introduction

The HMKTA-1.0 [1][2] was released after last VCEG meeting. On top of HEVC, HMKTA-1.0 integrates a few well-known tools, including larger Coding Tree Blocks (CTB) and larger transforms, Adaptive Loop Filter (ALF), Cross-Component Linear Model (CCLM) prediction, Overlapped Block Motion Compensation (OBMC) and Adaptive Multiple Transform (AMT). In addition, Advanced Temporal Motion Vector Prediction (ATMVP) that extends the current Temporal Motion Vector Prediction (TMVP) design in HEVC and applies the TMVP-like motion prediction on a sub-Prediction Unit (sub-PU) level is also included.

In this contribution, we introduce several new coding tools, which further improve the performance of motion compensation, intra prediction, as well as arithmetic entropy coding of HMKTA-1.0.

# Technical description

Inter prediction is improved in this contribution by applying the following four techniques: Local Illumination Compensation (LIC), Adaptive Motion Vector Resolution (AMVR), Extension of ATMVP and Frame Rate Up-Conversion (FRUC) mode. For intra coding, the intra prediction modes are extended to support 63 directional intra prediction, and the intra mode signalling is improved by modifying the design of most probable mode selection, also intra prediction is enhanced by using 4-tap interpolation filter and an additional boundary filter. Furthermore, entropy coding is improved by employing a new arithmetic engine and a new context modeling for transform coefficients. The description of Local Illumination Compensation and Extension of ATMVP can be found in document VCEG-AZ06 [3] and VCEG-AZ10 [4], respectively. The description of the other methods is provided in the next sub-sections.

# Adaptive motion vector resolution

In Advanced Motion Vector Resolution (AMVR) mode, Motion Vector Difference (MVD) (between the motion vector and predicted motion vector of a PU) can be coded with either quarter-pel resolution or integer-pel resolution. The MVD resolution is controlled at coding unit (CU) level and an integer MVD resolution flag is conditionally signaled for each CU that has at least one non-zero MVD components.

When the integer MVD resolution flag is false, or not coded for a CU, the default quarter-pel MV resolution is used for all PUs belonging to the CU. Otherwise (when integer MV resolution flag is true for a CU), all PUs coded with AMVP mode belonging to the CU use integer MV resolution, while the PUs coded with merge mode still use quarter-pel MV resolution. When a PU uses integer MV resolution, the AMVP candidate list is filled with integer MV by rounding quarter-pel MVs to integer-pel MVs.

# Frame rate up-conversion mode

Frame Rate Up-Conversion (FRUC) mode is a special merge mode based on frame-rate up conversion techniques. With this mode, motion information of the block is derived at decoder side.

A FRUC flag is signaled for a CU or PU when the merge flag is true. When the FRUC flag is false, a merge index is signaled and the regular merge mode is used. When the FRUC flag is true, an additional FRUC mode flag is signaled to indicate which method (bilateral matching or template matching) is to be used to derive motion information for the block.

During the motion derivation process, an initial motion vector is first derived for the whole PU based on bilateral matching or template matching. First, the merge list of the PU is checked and the candidate which leads to the minimum matching cost is selected as the starting point. Then a local search based on bilateral matching or template matching around the starting point is performed and the MV results in the minimum matching cost is taken as the MV for the PU. Then the motion information is further refined at sub-block level with the derived CU/PU motion vectors as the starting points.

As shown in the Figure 1, the bilateral matching is used to derive motion information of the current block by finding the best match between two blocks along the motion trajectory of the current block in two different reference pictures. Under the assumption of continuous motion trajectory, the motion vectors MV0 and MV1 pointing to the two reference blocks shall be proportional to the temporal distances, i.e., TD0 and TD1, between the current picture and the two reference pictures. As a special case, when the current picture is temporally between the two reference pictures and the temporal distance from the current picture to the two reference pictures is the same, the proposed bilateral matching becomes mirror based bi-directional MV [5].

Figure 1: Bilateral matching

As shown in Figure 2, template matching is used to derive motion information of the current block by finding the best match between a template (top and/or left neighboring blocks of the current block) in the current picture and a block (same size to the template) in a reference picture.

Figure 2: Template matching

# Intra coding improvement

### Intra mode coding with 67 intra prediction modes

To capture finer edge directions presented in natural videos, in this contribution, it is proposed to extend the directional intra modes from 33, as defined in HEVC, to 65. The new directional modes are indicated as red dotted arrows in Figure 3, and the Planar and DC modes remain the same. These proposed denser directional intra prediction modes apply for all PU sizes and both luma and chroma intra predictions.

Figure 3: Proposed 67 intra prediction modes

To accommodate the increased number of directional Intra modes, an improved Intra mode coding method is proposed, using 6 Most Probable Modes (MPMs). Two major technical aspects are involved: 1) the derivation of 6 MPMs, and 2) entropy coding of 6 MPMs.

When deriving the new set of 6 MPMs we first changed the definition of the left and above neighboring Intra modes. Instead of using the Intra modes from top and left neighboring blocks directly as in HEVC, the most frequently used Intra mode along the top neighboring row and along the left neighboring column are computed, and then used as the left and above neighboring modes, respectively.

Next, to derive the actual 6 MPMs, we first employ the same set of rules used in HEVC to obtain 3 MPMs (but using the new values of left and above modes, as explained above), and then define the remaining 3 MPMs as the 3 Intra modes closest to (in terms of prediction direction) the angular modes which have already been included in the list of MPMs. More detailed descriptions of deriving the 6 MPMs using the left and above neighboring modes are listed in Table 1.

Table 1: Derivation of 6 MPMs using the (L)eft and (A)bove neighboring modes

| Conditions |  |  | MPM0 | MPM1 | MPM2 | MPM3 | MPM4 | MPM5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L=A | L≠Planar and L≠DC |  | L | Planar | L+1 | L-1 | L+2 | DC |
|  | Otherwise |  | Planar | DC | 26 (Ver) | 10 (Hor) | 2 | 18 |
| L≠A | L≠Planar and R≠Planar | L=DC or A=DC | L | A | Planar | Max-1 | Max+1 | Max+2 |
|  |  | otherwise | L | A | Planar | DC | Max+1 | Min-1 |
|  | otherwise | L+A<2 | L | A | 26 (Ver) | 10 (Hor) | 2 | 18 |
|  |  | otherwise | L | A | DC | Max-1 | Max+1 | Max+2 |

* “Max” and “Min” indicates the maximum and minimum of “L” and “A”, respectively

For entropy coding of 6 MPMs, a truncated unary binarization of the MPMs is used. The first three bins are coded with contexts which depend on the left and above neighboring Intra mode, and the remaining bits are bypass coded.

### Four-tap intra interpolation filter

Four-tap intra interpolation filters are utilized to improve the directional intra prediction accuracy. In HEVC, a two-tap linear interpolation filter has been used to generate the intra prediction block in the directional prediction modes (i.e., excluding Planar and DC predictors). In this proposal, two types of four-tap interpolation filters are used: Cubic interpolation filters for 4x4 and 8x8 blocks, and Gaussian interpolation filters for 16x16 and larger blocks. The parameters of the filters are fixed according to block size, and the same filter is used for all predicted pixels, in all directional modes.

### Boundary prediction filters

In HEVC, after the intra prediction block has been generated for VER and HOR intra modes, the left-most column and top-most row of the prediction samples are further adjusted, respectively. In this contribution, as originally proposed by several proponents in previous JCTVC meetings [6], this method has been further extended to several diagonal intra modes, and boundary samples up to four columns or rows are further adjusted using a two-tap (for intra mode 2 & 34) or a three-tap filter (for intra mode 3-6 & 30-33). Examples of the boundary prediction filters for intra mode 34 and 30~33 are shown in Figure 4, and the boundary prediction filters for intra mode 2 and 3~6 are similar.

Figure 4: Examples of boundary prediction filters for intra mode 30 – 34

# Enhanced CABAC design

The proposed Enhanced CABAC (eCABAC) contains the following three major changes compared to the design in HEVC:

- Modified context model selection for transform coefficient levels

- Advanced binary arithmetic coder with context-dependent probability updating speed

- Adaptive initialization for context models

### Context model selection for transform coefficient levels

In HEVC, transform coefficients of one transform unit (TU) are coded with non-overlapped coefficient groups (CG), and each CG contains the coefficients of a 4x4 block of a TU. The CGs inside a TU, and the transform coefficients within a CG, are coded according to pre-defined scan orders. The coding of transform coefficient levels is separated into multiple scan passes. In the first pass, the first bin (denoted by bin0, also referred as significant_coeff_flag, which indicates the magnitude of the coefficient is larger than 0) is coded. Next, two scan passes for context coding the second/third bins (denoted by bin1 and bin2, respectively, also referred as coeff_abs_greater1_flag and coeff_abs_greater2_flag) may be applied. Finally, two scan passes for coding the sign information and the remaining values (also referred as coeff_abs_level_remaining) of coefficient levels are invoked, if necessary. Note that only bins in the first three scan passes are coded in a regular mode and named regular bins in the following descriptions.

In this proposal, the context model selection method of regular bins are modified. When coding bini in the i-th scan pass (i being 0, 1, 2), the context index is dependent on the values of the i-th bins of previously coded coefficients in the neighborhood covered by a local template. More specifically, the context index is determined based on the sum of the i-th bins of neighboring coefficients. The template is the same as the one used in [7], and depicted in Figure 5. To capture the characteristics of transform coefficients at different frequencies, one TU may be split into up to three regions and the splitting method is fixed regardless of the TU sizes. For example, when coding bin0 of luma transform coefficients, as depicted in Figure 5, one TU is split into three regions marked with different colors, and the context index assigned to each region is listed. Luma and chroma components are treated in a similar way but with separate sets of context models. Moreover, the context model selection for bin0 (i.e., significant flags) of the luma component is further dependent on transform size. For the bypass coded bins of the remaining values of coefficient levels, the coding method of coeff_abs_level_remaining in [7] is applied.

Figure 5: Definition of template used in transform coefficient context modeling

### Advanced binary arithmetic coder

In the proposed method, the binary arithmetic coder with the following probability update model [8] is applied.

(1)

where Pold and Pnew represent the probabilities (i.e., state associated with one context model) before and after coding a bin; respectively, Mi is a parameter which controls the probability updating speed; and k represents the precision of probabilities (here it is equal to 15).

We propose a method for changing the probability updating speed where for each context model we assign different values of parameter Mi used in equation (1). The value of Mi ranges from 4 to 7, and a 2-bit variable is used to indicate the probability updating speed for a specific context model.

Since the optimal probability update speed for each context model is determined at the encoder side and signaled in the bitstream, the additional decoder complexity is very small.

### Adaptive initialization for context models

Instead of using fixed tables for context initialization, in the proposed method, the initial probability states of context models for inter-coded slices can be initialized by copying states from previously coded pictures. More specifically, after coding a centrally-located Coding Tree Block (CTB) of each picture, the probability state of all context models are stored and optionally used as the initial state of each corresponding context model on later pictures. In this contribution, the set of initial states for each inter-coded slice is copied from the saved states of a previously coded picture that has the same slice type, and the same slice level QP as the current slice.

# Experimental results

The aforementioned tools were implemented on top of HMKTA-1.0 and simulations were conducted following the JCT-VC common test condition used during HEVC development [9].

As shown in Table 2, compared to HMKTA-1.0, the reported tools provide 2.9%, 6.7%, and 4.9% luma BD rate reduction, for all intra, random access and low delay configurations, respectively.

Table 2 Coding performance improvement over HMKTA-1.0

|  | All Intra Main |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -2.6% | -2.0% | -1.6% | -2.6% | -2.1% | -2.1% |
| Class B | -2.9% | -1.6% | -1.8% | -2.9% | -1.7% | -1.9% |
| Class C | -3.1% | -2.4% | -2.5% | -3.1% | -2.1% | -2.3% |
| Class D | -2.3% | -1.3% | -1.5% | -2.3% | -1.5% | -1.8% |
| Class E | -4.1% | -2.9% | -2.5% | -4.2% | -3.8% | -3.3% |
| Overall | -2.9% | -2.0% | -1.9% | -2.9% | -2.1% | -2.2% |
| Class F | -2.4% | -1.8% | -1.7% | -2.3% | -1.5% | -1.2% |
|  | Random Access Main |  |  | Random Access Main10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -6.6% | -4.3% | -3.5% | -6.5% | -4.1% | -3.1% |
| Class B | -6.6% | -5.1% | -4.9% | -6.6% | -5.2% | -5.4% |
| Class C | -7.2% | -6.5% | -6.9% | -7.2% | -6.9% | -7.0% |
| Class D | -6.5% | -5.3% | -5.6% | -6.5% | -5.7% | -6.0% |
| Class E |  |  |  |  |  |  |
| Overall | -6.7% | -5.3% | -5.2% | -6.7% | -5.5% | -5.4% |
| Class F | -6.5% | -5.5% | -5.3% | -6.7% | -5.8% | -5.6% |
|  | Low delay B Main |  |  | Low delay B Main10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -4.6% | -3.7% | -3.8% | -4.5% | -3.3% | -3.3% |
| Class C | -5.0% | -4.7% | -5.2% | -5.0% | -4.5% | -4.8% |
| Class D | -4.6% | -5.1% | -4.7% | -4.6% | -4.4% | -3.8% |
| Class E | -5.9% | -6.3% | -6.7% | -5.8% | -5.1% | -4.4% |
| Overall | -4.9% | -4.8% | -4.9% | -4.9% | -4.2% | -4.0% |
| Class F | -7.9% | -6.9% | -7.1% | -8.1% | -7.9% | -7.6% |

Table 3 provides the results using HM14.0 as anchor. It is shown that the proposed tools together with the coding tools HMKTA-1.0 achieves 9.5%, 16.5%, and 13.4% luma BD rate reduction, for all intra, random access and low delay configurations in Main Profile, respectively. It was also found that slightly higher gain is achieved in Main10 profile. The highest coding gain is observed for class A under random access configuration in Main10 Profile, where 18.5%, 39.3%, and 34.2% BD rate reduction is achieved for Y, Cb and Cr components, respectively.

Table 3 Coding performance improvement over HM14.0

|  | All Intra Main |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -11.2% | -25.3% | -20.3% | -11.3% | -28.8% | -24.1% |
| Class B | -9.2% | -13.1% | -10.3% | -9.2% | -15.2% | -12.5% |
| Class C | -9.4% | -13.9% | -15.9% | -9.4% | -15.5% | -17.8% |
| Class D | -6.9% | -11.3% | -11.7% | -7.0% | -12.5% | -13.4% |
| Class E | -11.1% | -14.8% | -16.3% | -11.2% | -16.7% | -18.8% |
| Overall | -9.5% | -15.6% | -14.6% | -9.5% | -17.7% | -17.0% |
| Class F | -9.3% | -13.2% | -12.6% | -9.3% | -14.5% | -14.1% |
|  | Random Access Main |  |  | Random Access Main10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -18.3% | -33.0% | -27.9% | -18.5% | -39.3% | -34.2% |
| Class B | -16.4% | -19.7% | -15.9% | -16.9% | -23.7% | -19.8% |
| Class C | -15.7% | -18.1% | -20.6% | -16.2% | -21.0% | -23.3% |
| Class D | -15.5% | -15.6% | -16.5% | -15.8% | -17.2% | -17.9% |
| Class E |  |  |  |  |  |  |
| Overall | -16.5% | -21.5% | -19.9% | -16.9% | -25.2% | -23.6% |
| Class F | -12.7% | -17.4% | -16.7% | -13.2% | -19.4% | -19.3% |
|  | Low delay B Main |  |  | Low delay B Main10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -13.0% | -13.5% | -11.9% | -13.7% | -18.4% | -17.0% |
| Class C | -13.2% | -12.1% | -13.2% | -14.0% | -16.3% | -18.2% |
| Class D | -12.3% | -10.9% | -10.7% | -12.9% | -12.9% | -12.0% |
| Class E | -16.0% | -18.8% | -23.0% | -17.4% | -23.4% | -27.3% |
| Overall | -13.4% | -13.5% | -14.0% | -14.3% | -17.5% | -18.0% |
| Class F | -13.0% | -16.0% | -15.5% | -14.2% | -20.4% | -19.9% |

# References

- J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, X. Zhao, “Coding tools investigation for next generation video coding”, ITU-T SG16 Doc. COM16–C806, Feb. 2015.

- VCEG HM KTA-1.0, https://vceg.hhi.fraunhofer.de/svn/svn_HMKTASoftware/tags/HM-14.0-KTA-1.0/.

- H. Liu, Y. Chen, J. Chen, L. Zhang, M. Karczewicz, “Local illumination compensation”, VCEG-AZ06, Jun. 2015.

- W.-J. Chien, M. Karczewicz, “Extension of Advanced Temporal Motion Vector Predictor (ATMVP)”, VCEG-AZ10, Jun. 2015.

- Y.-J. Chiu, L. Xu, W. Zhang, H. Jiang, “Decoder-side Motion Estimation and Wiener filter for HEVC”, Visual communications and Image Processing (VCIP), 2013.

- M. Guo, X. Guo, X. Zhao, et al. “CE6 Subtest d: Intra Prediction with Secondary Boundary”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Doc. JCTVC-G280, 7th Meeting: Geneva, CH, 21-30 Nov., 2011.

- T. Nguyen, D. Marpe, T. Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding”, JCTVC-H0228, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- A. Alshin, E. Alshina, J.-H. Park, “CE1 (subset B): Multi-parameter probability up-date for CABAC”, JCTVC-G764, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- F. Bossen, “Common HM test conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Doc. JCTVC-L1100, 12th Meeting.