| ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) 52nd Meeting: 19–26 June 2015, Warsaw, Poland | Document VCEG-AZ10 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Qualcomm Incorporated | Tel: Email: | +1-858-845-1795 wchien@qti.qualcomm.com martak@qti.qualcom.com |
| Title: | Extension of Advanced Temporal Motion Vector Predictor (ATMVP) |  |  |
| Authors: | Wei-Jung Chien and Marta Karczewicz |  |  |
| Purpose: | Proposal |  |  |

_____________________________

Abstract

In this contribution, advanced temporal motion vector predictor (ATMVP) is extended to consider spatial motion vector predictors. As an additional merge candidate, the spatio-temporal motion vector predictor together with ATMVP achieve 1.6% and 1.7% BD rate reduction for random access and low delay configurations, respectively.

# Introduction

In [1], advanced temporal motion vector prediction (ATMVP) was proposed to allow each PU to obtain multiple sets of motion information (including motion vectors and reference frames). The motion information in ATMVP comes though only from the reference pictures. In this contribution, an additional merge candidate is introduced by extending sub-block concept of ATMVP. The spatio-temporal motion vector predictor (SPMVP) considers not only the temporal motion vector predictors but also the spatial motion vector predictors. By averaging motion information of two spatial motion vector predictor and one temporal motion vector predictor, an additional merge candidate is generated for each sub-PU to achieve further BD rate reduction.

# Technical description

The PUs are split into square NxN sub-PUs (in this contribution N is set to 4). The motion vectors of the sub-PUs are derived recursively, following raster scan order. Figure 1 illustrates this concept. Let us consider an 8x8 PU which contains four 4x4 sub-PUs A, B, C, and D. The neighboring NxN blocks in the current frame are labeled as a, b, c, and d.

The motion derivation for sub-PU A starts by identifying its two spatial neighbors. The first neighbor is NxN block above sub-PU A (block c). If this block c is not available or is intra coded the other NxN blocks above sub-PU A are checked (from left to right, starting at block c). The second neighbor is a block to the left of the sub-PU A (block b). If block b is not available or is intra coded other blocks to the left of sub-PU A are checked (from top to bottom, staring at block b).  The motion information obtained from the neighboring blocks for each list is scaled to the first reference frame for a given list. Next, temporal motion vector predictor (TMVP) of sub-block A is derived by following the same procedure of TMVP derivation as specified in HEVC. The motion information of the co-located block at location D is fetched and scaled accordingly. At last, after retrieving and scaling the motion information, all available motion vectors (up to 3) are averaged separately for each reference list. The averaged motion vector is assigned as the motion vector of the current sub-PU.

A

B

C

D

c

d

b

a

Figure 1 Example of one PU with four sub-blocks (A-D) and its neighbouring blocks (a-d).

The STMVP mode is enabled as an additional merge candidate and there is no additional syntax element required to signal this mode. Up to seven merge candidates are used, if the sequence parameter set indicates that both ATMVP and STMVP are enabled. Before the STMVP mode is inserted to merge candidate list, a pruning process is applied. The STMVP mode is inserted into candidate list if motion information of at least one sub-PU is different than motion information obtained for this sub-PU using other merge candidates or ATMVP.

# Experimental results

The proposed additional merge candidate was implemented on top of HMKTA-1.0 [1] and simulations were conducted following the JCT-VC common test condition [2]. The anchor used here is HMKTA-1.0 without ATMVP with max_num_merge_cand equal to 5. Table 1 shows the BD-Rate reduction of HMKTA-1.0 with ATMVP enabled (max_num_merge_cand equal to 6).  Table 2 shows the BD-Rate reduction when both ATMVP and the proposed merge candidate are enabled (max_num_merge_cand equal to 7).

|  | Random Access Main |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.3% | -1.2% | -1.6% | -1.4% | -2.0% | -2.1% |
| Class B | -1.1% | -0.9% | -1.0% | -1.1% | -0.7% | -0.9% |
| Class C | -0.9% | -0.9% | -1.0% | -0.9% | -0.9% | -0.9% |
| Class D | -1.2% | -1.0% | -1.2% | -1.2% | -1.3% | -1.3% |
| Class E |  |  |  |  |  |  |
| Overall | -1.1% | -1.0% | -1.2% | -1.2% | -1.2% | -1.3% |
|  | -1.1% | -1.0% | -1.2% | -1.2% | -1.2% | -1.3% |
| Class F | -0.4% | -0.4% | -0.5% | -0.3% | -0.3% | -0.3% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 109% |  |  | 108% |  |  |
|  | Low delay B Main |  |  | Low delay B Main10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | -0.8% | -0.6% | -0.9% | -1.2% | -1.5% |
| Class C | -1.0% | -1.2% | -1.0% | -1.0% | -1.1% | -1.0% |
| Class D | -1.1% | -1.2% | -1.4% | -1.1% | -1.0% | -0.8% |
| Class E | -2.7% | -2.4% | -3.3% | -2.7% | -3.3% | -2.9% |
| Overall | -1.3% | -1.3% | -1.4% | -1.3% | -1.5% | -1.4% |
|  | -1.3% | -1.4% | -1.4% | -1.3% | -1.6% | -1.5% |
| Class F | -0.5% | -0.7% | -0.7% | -0.4% | -0.6% | -0.4% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |

Table 1 Coding performance improvement of ATMVP

|  | Random Access Main |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.8% | -1.7% | -1.9% | -1.7% | -2.4% | -2.4% |
| Class B | -1.6% | -1.3% | -1.5% | -1.6% | -1.2% | -1.4% |
| Class C | -1.5% | -1.5% | -1.7% | -1.4% | -1.4% | -1.5% |
| Class D | -1.5% | -1.4% | -1.6% | -1.5% | -1.5% | -1.9% |
| Class E |  |  |  |  |  |  |
| Overall | -1.6% | -1.5% | -1.6% | -1.6% | -1.6% | -1.8% |
|  | -1.6% | -1.5% | -1.7% | -1.6% | -1.6% | -1.8% |
| Class F | -0.5% | -0.7% | -0.7% | -0.5% | -0.7% | -0.6% |
| Enc Time[%] | 111% |  |  | 109% |  |  |
| Dec Time[%] | 119% |  |  | 118% |  |  |
|  | Low delay B Main |  |  | Low delay B Main10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.1% | -0.7% | -0.7% | -1.1% | -1.0% | -1.4% |
| Class C | -1.3% | -1.5% | -1.4% | -1.3% | -1.3% | -1.2% |
| Class D | -1.4% | -1.1% | -1.3% | -1.3% | -1.3% | -1.0% |
| Class E | -3.5% | -2.9% | -3.9% | -3.6% | -4.1% | -4.0% |
| Overall | -1.7% | -1.4% | -1.6% | -1.7% | -1.7% | -1.7% |
|  | -1.7% | -1.5% | -1.6% | -1.7% | -1.8% | -1.7% |
| Class F | -0.6% | -0.8% | -1.3% | -0.5% | -0.8% | -0.8% |
| Enc Time[%] | 110% |  |  | 106% |  |  |
| Dec Time[%] | 114% |  |  | 114% |  |  |

Table 2 Coding performance improvement of ATMVP and the proposed merge candidate.

# Conclusion

This contribution introduces an additional merge candidate and evaluates the coding performance benefits based on the common test condition of HEVC version 1. The results show that 0.5% additional performance improvement over HMKTA-1.0 can be achieved with similar complexity as the existing coding tool, ATMVP.

# References

- Qualcomm Incorporated, “Coding tools investigation for next generation video coding”, ITU-T SG16 CONTRIBUTION 806, Jan., 2015.

- F. Bossen, “Common HM test conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Doc. JCTVC-L1100, 12th Meeting.

________________________