| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: November, 14-22 July, 2011 | Document: JCTVC-G737 WG11 Doc: m22306 |
| --- | --- |

| Title: | CE10: Full Factorization Core Transforms for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin, Wonjae Lee JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea Kulbhushan Pachauri 560 093 -Bagmane Lakeview B Block, #66/1, Bagmane Tech Park, C V Raman Nagar, Byrasandra Bangalore Karnataka India 560093 Pankaj Topiwala 5840 Banneker Rd., #230, Columbia, MD 21044 USA 750 N. Atlantic Ave., #1504, Cocoa Beach, FL 32931 USA | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.com alexander_b.alshin@samsung.com ellio.lee@samsung.com jeonghoon@samsung.com kb.pachauri@samsung.com pankajtva@gmail.com |
| Source: | Samsung Electronics, Ltd, FastVDO LLC |  |  |

_____________________________

# Abstract

This is the CE10 submission by Samsung and FastVDO, follow-on to F251. All transform components have been retained from F251 (no updates). Since the transform design has a simple matrix multiplication definition, it is so presented, in the Appendix. It is claimed that the proposed scheme supports all three forms of implementation: matrix multiplication, partial butterfly and full-factorization, which are identical in performance. This reported equivalence allows choosing the implementation according to platform: i.e. matrix multiplication form may be preferred for SIMD implementations while full-factorization may be preferred for platforms with strict limitations for number of multiplications. By utilizing Loeffler’s factorization in the 32pt transform, it is reported that a 4X reduction in the number of multiplications compared to partial butterfly, and a 12X reduction compared to matrix multiplication, is achieved. Performance testing under common test conditions appears to show no performance change compare to both HM3.0 and HM4.0 for Luma component (0.0%) with small gain for Chroma component (-0.1%). Scalar quantization is reportedly unchanged compared to HM design for all transform sizes. This proposal also verifies h/w test performed by independent expert STMicro; who reported 33-61% G/C reduction for proposed transform compare to HM4.0. SIMD implementation for proposed core transform was done by Samsung. It was implemented in 4 different equivalent forms: naive and fast matrix multiplication, partial and full butterfly. Our comparison claims that for small block sizes fast matrix multiplication is fastest but for large transforms full factorization provides the best speed: reportedly 30% faster for 16 pt and 50% faster for 32 pt transform (compare to partial butterfly).

# Introduction

HM4.0 uses core transforms introduced by [1]. This transform is an approximation of the DCT. It is implemented in two equivalent forms in HM4.0: as matrix multiplications and as so-called partial butterfly. This convertibility allows a variety of implementations according to platform. Partial butterfly is fast transform implementation providing about 3 times reduction for number of multiplications compared to matrix multiplications form. Quantization process introduced in [1] uses scalar quantization which essentially reduces memory for quantization matrices.

In this proposal we keep all these advantages but use different approximations for DCT transform which additionally allows full Loeffler’s factorization. Briefly, proposed changes of core transforms compared to [1] can be described in 2 steps. At first step transform matrix elements were multiplied by 64. Each 1D transform in HM3.0 uses its own de-scaling left shift (in order to keep output in predetermined range). Values of all these shifts were increased by 6 in order to compensate transform matrix element increment. So dynamic range of each buffer for temporal data storage is not changed compare to HM3.0. First step of our modifications does not change performance. At second step scaled by 64 transform matrix elements were modified in absolute values in order to allow higher degree of factorization compare to partial butterfly. These modifications lead to performance change.

Modified approximations of DCT matrices keeps fundamental properties of it’s symmetry. In particular 4x4 transform matrix is embedded into 8x8, 16x16 and 32x32 matrices, 8x8 is embedded into 16x16 and 32x32 and 16x16 transform matrix is part of 32x32 transform.

Transform matrix for 4x4 is not modified compared to [1] in our proposal.

The proposed transforms can be implemented in three ways:

- matrix multiplications,

- partial butterfly,

- full-factorization form.

First and second forms are also supported by HM4.0, while the third one becomes available after proposed changes for transform matrices. The code for matrix multiplication and partial butterfly is similar to HM4.0 but uses different values of transform matrices elements. The HM4.0 transform matrix elements are represented in 8 bits. For this proposal, transform matrix elements are represented in 14 bits.

All equivalent forms use multiplications. In matrix multiplications and partial butterfly forms first multiplier is transform matrix element (14 bits in our design) and another multiplier is de-quantized coefficient or content of temporal buffer after 1st inverse transform. Both are supposed to be within 16 bits. If proposed here transform is implemented as matrix multiplication or partial butterfly then both multipliers don’t exceed 16 bits.

The third form (full-factorization) is targeting for h/w platforms with strict restrictions for number of multiplications (~4 times smaller than partial butterfly). The full-factorization form may have up to two cascade multiplications. One multiplier is 8 bits number in proposed design. Another multiplier is 16 bits number (as well as for partial butterfly) for first multiplication in a path. But for second multiplications in a path one multiplier can be over 16 bits.

Our transform design remain un-changed since Torino meeting, please refer to [2] for more details.

# HW test

Hardware test is the part of CE10 activity. As 32 pt inverse transform is the bottleneck of the current HM transform design this size was the main target of h/w test. Independent expert (not CE10 proponent) STMicro performed h/w test in CE10 together with Samsung and TI.

As it was agreed in Torino in order to equalize the level of code optimization automatic RTL generator was used. Input of automatic RTL generator which is c-code was open to all CE10 members in August. Each proponent had enough time to prepare, correct and optimize their code. Basically no specific optimization was applied except un-rolling all loops (without this automatic RTL generator fails to generate the code with low latency).

Then STMciro and Samsung started automatic RTL generation. STMicro used SynthonyCC, Samsung used CATAPULT C. Settings for both tools were the same except STMicro and Samsung used different 45 nm each company proprietary libraries. Input parameter for both automatic RTL generator is frequency (both tester cannot control latency which is output parameter).

After RTL generation Synopsys Design Compiler was used both by Samsung and STMicro for synthesis and G/C measurement. Synopsys Design Compiler constrain is also cannot not be disclosed due to each company security regulations. So even with the same RTL code we didn’t expect perfect match in resulting G/C measurement.

It was agreed in CE10 to represent test data in the form of table (G/C depends on latency L=2,3,…and frequency F=150-500 MHz). Test results provided by STMicro are shown in the Table 1: HM4.0 and F-251 core transform G/C and ration between them. Empty cells mean that this combination of frequency/latency was not tested.

RTL generated by SynthonyCC was shared by STMicro to all CE10 members and Samsung has cross-checked them. First we have checked that RTL and c-code realizing transform provide identical outputs. Then we synthesized provided RTL using our constrain of Synopsys Design Compiler. As it is mentioned above G/C in our measurement cannot be the same with reported by STMicro (due to the difference in Design Compiler settings); but difference is not big. Cross-check results by Samsung is reported in the left part of Table 1.

Table 1. G/C measured by STMicro using automatically generated by SynthonyCC RTL (and cross-check by Samsung).

HM 4.0 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7; F ¦ L; 2; 3; 4; 5. 150; 108364; 134553; 147701; 182063; 150; 172404. 200; 110269; 134615; 147711; 182016; 200; 172402. 250; 123155; 143120; 151092; 182387; 250; 172404. 300; 126322; 143793; 159014; 204536; 300; 175013. 350; 129150; 153378; 168162; 204752; 350; 186522. 400; 165196; 178534; 216075; 400; 195681. 450; 450; 208838. 500; 500; 211648.

F-251 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7; F ¦ L; 2; 3; 4; 5. 150; 49647; 57657; 67446; 77805; 81256; 150; 73963. 200; 57870; 70142; 68929; 78830; 82065; 200; 85011. 250; 71707; 74467; 83120; 84958; 92561; 250; 91854. 300; 77493; 81463; 84758; 96416; 98556; 300; 108377. 350; 86402; 87763; 89053; 100879; 100297; 350; 109918. 400; 103183; 99331; 90470; 101509; 103581; 400; 116369. 450; 450; 112543. 500; 500; 114422.

G/C ration between F-251 and HM4.0 32 pt 1D inverse core transform F ¦ L; 2; 3; 4; 5; 6; 7; F ¦ L; 2; 3; 4; 5. 150; 46%; 39%; 45%; 150; 43%. 200; 52%; 47%; 45%; 200; 49%. 250; 58%; 49%; 51%; 250; 53%. 300; 61%; 51%; 48%; 300; 62%. 350; 67%; 52%; 49%; 350; 59%. 400; 56%; 48%; 400; 59%. 450; 450; 54%. 500; 500; 54%.

Based on this measurement we can conclude that area required for full-factorized 32 pt transform is only 39%...67% of are required for partial butterfly (E243). With frequency increment the difference between full-factorized and partial butterfly transform becomes smaller.

Samsung generated RTL using CATAPULT C and then use Synopsys Design Compiler and measured G/C by the same way with STMicro. Test results will be up-dated by start of the meeting.

# SIMD implementation study

In order to study s/w perspectives of full-factorized transform application SIMD implementation was done. This is stand alone application realizes 2D inverse transform (both vertical and horizontal). We run it in a loop. Loop size is 10 000 000. We measured run-time and computed clock cycles per one 2D transform.

We studied 4 forms of transform implementation:

“Mat Normal”: Normal (naive) matrix multiplication

“Mat Fast”: Fast matrix multiplication

“PB”: Partal Buterfly

“FB”: Full Butterfly

“Mat Normal”and “Mat Fast”can be applied to any kind of matrix multiplication, in particular to E243 or F251 (both with the same speed)

PB utilizes symmetry and anti-symmetry of DCT coefficients, once again there is no difference between  E243 and if both are realized in PB form.

FB supported by F-251.

Test results are summarized in the Table 2.

For 16pt and 32 pt FB is the fastest one in our test. For smaller sizes (like 4 and 8) we have not found big difference between PB and FB.

"C" means that not SIMD but regular c-code was used; we show this data just to illustrate how much SIMD speeds-up the process. SSEX indicates the version of SSE instructions we used.

Both implementations use the same level of optimization (at least no specific optimization applied in FB implementation compare to PB). PC which was used for this test specification is Ubutnu 11.10 (64 bit), Intel® Core™ i5  @2.66GHz , 4GB.

Table 2. S/w test results.

|  | Time Taken(Secs) | Clock cycles/2D tr |  |
| --- | --- | --- | --- |
| 8pt |  |  |  |
| C Mat | 14.4874 |  | 3854 |
| C PB | 3.9302 |  | 1045 |
| C FB | 3.0351 |  | 807 |
| SSE Mat Normal | 2.5762 |  | 685 |
| SSE Mat Fast | 1.0952 |  | 291 |
| SSE PB | 0.8688 |  | 231 |
| SSE FB | 0.8675 |  | 231 |
| 16pt |  |  |  |
| C Mat | 104.9058 |  | 27905 |
| C PB | 23.9085 |  | 6360 |
| C FB | 16.5632 |  | 4406 |
| SSE Mat Normal | 12.6481 |  | 3364 |
| SSE Mat Fast | 6.7924 |  | 1807 |
| SSE PB | 8.2895 |  | 2205 |
| SSE FB | 5.3295 |  | 1418 |
| 32pt |  |  |  |
| C Mat | 793.3536 |  | 211032 |
| C PB | 161.5157 |  | 42963 |
| C FB | 93.7578 |  | 24940 |
| SSE Mat Normal | 66.8819 |  | 17791 |
| SSE Mat Fast | 56.0049 |  | 14897 |
| SSE PB | 54.6074 |  | 14526 |
| SSE FB | 27.2221 |  | 7241 |

Now we are working on further optimization for both PB and FB and we expect up-date for SIMD implementations by start of the meeting and plan to up-date for this table with smaller numbers.

# Number of sequential operations

As mentioned in chapters 2-3 there is no more than 2 multiplications in the longest execution path. In the worst case execution path uses 2 multiplications and 5 additions. There are no left (de-scaling) shifts at any intermediate stages.

# Quantization/dequantization matrices

There is no change of quantization process compare to HM4.0:  6 scalar values for quantization and 6 scalar values for de-quantization.

# Performance test

According to CE10 requirements [3] all proposal should be tested using implementation based on HM3.0. Since our design and code was not changed performance is un-changed compare to reported in [2]. Additionally we provide HM4.0 based implementation (attached to this contribution). It was tested under common test conditions [4] (including class F and LD(P), which are optional). Table 4 provides test results for normal QP range

Table 4. Normal QP test for HM4.0 based implementation.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.1% | -0.1% | 0.3% | 0.0% | -0.1% |
| Class B | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | 0.0% |
| Class F | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.2% | -0.3% | 0.1% | 0.2% | 0.0% |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.4% | 0.0% | -0.1% | 0.1% |
| Class C | 0.1% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.4% | -0.3% |
| Class E | 0.1% | 0.5% | 0.4% | 0.0% | -0.1% | -0.3% |
| Class F | -0.1% | -0.4% | -0.3% | 0.1% | 0.2% | 0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | -0.4% | -0.8% | 0.0% | -0.1% | -0.4% |
| Class E | 0.0% | 1.1% | -0.5% | 0.0% | -0.5% | -0.5% |
| Class F | 0.0% | -0.2% | -0.3% | 0.0% | 0.3% | 0.1% |
| Overall | 0.0% | 0.1% | -0.4% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.1% | -0.4% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Performance is pretty similar to the reported in [2] for HM3.0 based implementation: 0.0% in avg. for Luma, with some improvement for Chroma (0.1% gain in avg).

# Description for Working Draft

10.1. Basis vectors

The transform matrices TrMatrix the 4x4, 8x8, 16x6 and 32x32 transforms are shown in Appendix A.

10.2 Intermediate scaling in inverse transform and dequantization.

Definitions

B	= internal bit depth (as specified by InternalBitDepth in the common config files)

W, H 	= transform size (horizontal and vertical)

Q	= f(QP%6), 	where f(x)   =   {26214,23302,20560,18396,16384,14564}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

Inverse transform consists of 3 steps: dequantizer, 1st (column) and second (row) inverse transforms. Output of each step is storied in TU size temporal buffer of 16 bits size. Intermediate calculation requires 32 bits precision.

The dequantizer is specified as follows

coeffQ 	=  ((level*IQ << (QP/6)) + offset)>>(M-1+(B-8)),  offset = 1<<(M-2+(B-8)),

coeffQ = min(32767,max(-32768,coeffQ)).

Here level is qunatized coefficents, M is

M	= log2(W*H)/2

The clipping ensures that the value of the quantized transform coefficients, coeffQ, can be represented with 16 bits.

The inverse transform is specified as a matrix multiplication in two stages, horizontal and vertical.

The first stage is column transform:

register1=TrMatrix(H)T×coeffQ.

After the first stage, the resulting transform coefficients are de-scaled as follows

temp  = (register1 + offset1)>>shift1)

shift1 = 13; offset1 = 1<<( shift1-1).

The first stage is row transform:

register2=temp ×TrMatrix(W).

After the second stage, the resulting transform coefficients are scaled as follows

rec_residual  =  (register2 + offset2)>>shift2;

shift2=(18-(B-8)),offset2 = 1<<( shift2-1).

# Conclusion

# Based on test results:

# 0.0%(Y) 0.0% (U) -0.1%(V) (average performance across all 8 test cases in normal QP tests)

# we would like to recommend adoption for proposed core transform modification for HM5.0 and WD.

# This will keep possibility of 2 equivalent implementations:

# matrix multiplication

# partial butterfly

# which are available in HM4.0 and add one more equivalent implementation

# full-factorized form

# which uses 4 times smaller number of multiplications in 32 pt transform compare to default HM4.0 configuration (partial butterfly) and so roughly twice faster in SIMD implementation (by Samsung’s measurement) and requires twice smaller G/C in h/w implementation (according to independent experts test).

# References

[1] A. Fuldseth, G. Bjøntegaard (Cisco), M. Sadafale, M. Budagavi (TI), “Transform design for HEVC with 16 bit intermediate data representation”, JCTVC-E243, 5th. JCTVC, Meeting, Geneva, Mar. 2011

[2] E.Alshina, A.Alshin, I-K. Kim, P. Topiwala “CE10: Full-factorized core transform proposal by Samsung/FastVDO”, JCTVC-F251, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

[3] P. Topiwala, M. Budagavi, A. Fuldseth, R. Joshi, I.-K. Kim, “CE10: Core Transform Design”JCTVC-F910, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

[4] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

# Patent rights declaration(s)

Samsung Electronics, Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fast VDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

APPENDIX  A

Transform matrices for proposed core transform.

DST: TrMatrix(4) =

{

{1856,   3520,    4736,   5376},

{4736,   4736,       0,  -4736},

{5376,  -1856,   -4736,   3520},

{3520,  -5376,    4736,  -1856}};

DCT: TrMatrix(4) =

{

{ 4096, 4096, 4096, 4096},

{ 5312, 2304,-2304,-5312  },

{ 4096,-4096,-4096, 4096},

{ 2304,-5312, 5312,-2304  }

};

TrMatrix(8)  =

{

{ 4096, 4096, 4096, 4096, 4096, 4096, 4096, 4096,},

{ 5696, 4800, 3264, 1088,-1088,-3264,-4800,-5696,},

{ 5312, 2304,-2304,-5312,-5312,-2304, 2304, 5312,},

{ 4770,-1080,-5670,-3240, 3240, 5670, 1080,-4770,},

{ 4096,-4096,-4096, 4096, 4096,-4096,-4096, 4096,},

{ 3240,-5670, 1080, 4770,-4770,-1080, 5670,-3240,},

{ 2304,-5312, 5312,-2304,-2304, 5312,-5312, 2304,},

{ 1088,-3264, 4800,-5696, 5696,-4800, 3264,-1088,},};

TrMatrix(16)  =

{

{	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	},

{	5760,	5504,	5120,	4480,	3712,	2688,	1664,	512,	-512,	-1664,	-2688,	-3712,	-4480,	-5120,	-5504,	-5760,	},

{	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	},

{	5543,	3568,	619,	-2802,	-5096,	-5747,	-4506,	-1639,	1639,	4506,	5747,	5096,	2802,	-619,	-3568,	-5543,	},

{	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	},

{	5167,	474,	-4379,	-5528,	-1806,	3773,	5728,	2591,	-2591,	-5728,	-3773,	1806,	5528,	4379,	-474,	-5167,	},

{	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	},

{	4410,	-2700,	-5490,	540,	5760,	1710,	-5040,	-3690,	3690,	5040,	-1710,	-5760,	-540,	5490,	2700,	-4410,	},

{	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	},

{	3690,	-5040,	-1710,	5760,	-540,	-5490,	2700,	4410,	-4410,	-2700,	5490,	540,	-5760,	1710,	5040,	-3690,	},

{	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	},

{	2591,	-5728,	3773,	1806,	-5528,	4379,	474,	-5167,	5167,	-474,	-4379,	5528,	-1806,	-3773,	5728,	-2591,	},

{	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	},

{	1639,	-4506,	5747,	-5096,	2802,	619,	-3568,	5543,	-5543,	3568,	-619,	-2802,	5096,	-5747,	4506,	-1639,	},

{	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	},

{	512,	-1664,	2688,	-3712,	4480,	-5120,	5504,	-5760,	5760,	-5504,	5120,	-4480,	3712,	-2688,	1664,	-512,	},

};

TrMatrix(32)  =

{

{	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	4096,	},

{	5760,	5696,	5632,	5440,	5248,	4992,	4608,	4288,	3904,	3456,	2944,	2432,	1984,	1408,	832,	256,	-256,	-832,	-1408,	-1984,	-2432,	-2944,	-3456,	-3904,	-4288,	-4608,	-4992,	-5248,	-5440,	-5632,	-5696,	-5760,	},

{	5760,	5504,	5120,	4480,	3712,	2688,	1664,	512,	-512,	-1664,	-2688,	-3712,	-4480,	-5120,	-5504,	-5760,	-5760,	-5504,	-5120,	-4480,	-3712,	-2688,	-1664,	-512,	512,	1664,	2688,	3712,	4480,	5120,	5504,	5760,	},

{	5675,	5223,	4301,	3000,	1350,	-308,	-1935,	-3425,	-4650,	-5445,	-5806,	-5600,	-4925,	-3927,	-2464,	-875,	875,	2464,	3927,	4925,	5600,	5806,	5445,	4650,	3425,	1935,	308,	-1350,	-3000,	-4301,	-5223,	-5675,	},

{	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	5696,	4800,	3264,	1088,	-1088,	-3264,	-4800,	-5696,	-5696,	-4800,	-3264,	-1088,	1088,	3264,	4800,	5696,	},

{	5622,	4249,	2002,	-933,	-3378,	-5246,	-5760,	-4953,	-3039,	-270,	2478,	4710,	5727,	5456,	3893,	1332,	-1332,	-3893,	-5456,	-5727,	-4710,	-2478,	270,	3039,	4953,	5760,	5246,	3378,	933,	-2002,	-4249,	-5622,	},

{	5543,	3568,	619,	-2802,	-5096,	-5747,	-4506,	-1639,	1639,	4506,	5747,	5096,	2802,	-619,	-3568,	-5543,	-5543,	-3568,	-619,	2802,	5096,	5747,	4506,	1639,	-1639,	-4506,	-5747,	-5096,	-2802,	619,	3568,	5543,	},

{	5449,	2865,	-759,	-4329,	-5772,	-4570,	-1521,	2553,	5204,	5553,	3560,	-334,	-3842,	-5753,	-4990,	-1877,	1877,	4990,	5753,	3842,	334,	-3560,	-5553,	-5204,	-2553,	1521,	4570,	5772,	4329,	759,	-2865,	-5449,	},

{	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	5312,	2304,	-2304,	-5312,	-5312,	-2304,	2304,	5312,	},

{	5261,	1318,	-3377,	-5786,	-3986,	904,	4905,	5420,	2055,	-3015,	-5722,	-4188,	153,	4719,	5601,	2353,	-2353,	-5601,	-4719,	-153,	4188,	5722,	3015,	-2055,	-5420,	-4905,	-904,	3986,	5786,	3377,	-1318,	-5261,	},

{	5167,	474,	-4379,	-5528,	-1806,	3773,	5728,	2591,	-2591,	-5728,	-3773,	1806,	5528,	4379,	-474,	-5167,	-5167,	-474,	4379,	5528,	1806,	-3773,	-5728,	-2591,	2591,	5728,	3773,	-1806,	-5528,	-4379,	474,	5167,	},

{	4957,	-300,	-5225,	-4676,	878,	5450,	4275,	-1419,	-5642,	-3825,	1900,	5736,	3447,	-2475,	-5725,	-2981,	2981,	5725,	2475,	-3447,	-5736,	-1900,	3825,	5642,	1419,	-4275,	-5450,	-878,	4676,	5225,	300,	-4957,	},

{	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	4770,	-1080,	-5670,	-3240,	3240,	5670,	1080,	-4770,	-4770,	1080,	5670,	3240,	-3240,	-5670,	-1080,	4770,	},

{	4626,	-1887,	-5808,	-1417,	4966,	4362,	-2538,	-5747,	-821,	5184,	3834,	-2978,	-5621,	-330,	5451,	3452,	-3452,	-5451,	330,	5621,	2978,	-3834,	-5184,	821,	5747,	2538,	-4362,	-4966,	1417,	5808,	1887,	-4626,	},

{	4410,	-2700,	-5490,	540,	5760,	1710,	-5040,	-3690,	3690,	5040,	-1710,	-5760,	-540,	5490,	2700,	-4410,	-4410,	2700,	5490,	-540,	-5760,	-1710,	5040,	3690,	-3690,	-5040,	1710,	5760,	540,	-5490,	-2700,	4410,	},

{	4230,	-3420,	-4950,	2430,	5400,	-1440,	-5670,	270,	5760,	810,	-5580,	-1980,	5220,	2970,	-4590,	-3870,	3870,	4590,	-2970,	-5220,	1980,	5580,	-810,	-5760,	-270,	5670,	1440,	-5400,	-2430,	4950,	3420,	-4230,	},

{	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	4096,	-4096,	-4096,	4096,	},

{	3870,	-4590,	-2970,	5220,	1980,	-5580,	-810,	5760,	-270,	-5670,	1440,	5400,	-2430,	-4950,	3420,	4230,	-4230,	-3420,	4950,	2430,	-5400,	-1440,	5670,	270,	-5760,	810,	5580,	-1980,	-5220,	2970,	4590,	-3870,	},

{	3690,	-5040,	-1710,	5760,	-540,	-5490,	2700,	4410,	-4410,	-2700,	5490,	540,	-5760,	1710,	5040,	-3690,	-3690,	5040,	1710,	-5760,	540,	5490,	-2700,	-4410,	4410,	2700,	-5490,	-540,	5760,	-1710,	-5040,	3690,	},

{	3452,	-5451,	-330,	5621,	-2978,	-3834,	5184,	821,	-5747,	2538,	4362,	-4966,	-1417,	5808,	-1887,	-4626,	4626,	1887,	-5808,	1417,	4966,	-4362,	-2538,	5747,	-821,	-5184,	3834,	2978,	-5621,	330,	5451,	-3452,	},

{	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	3240,	-5670,	1080,	4770,	-4770,	-1080,	5670,	-3240,	-3240,	5670,	-1080,	-4770,	4770,	1080,	-5670,	3240,	},

{	2981,	-5725,	2475,	3447,	-5736,	1900,	3825,	-5642,	1419,	4275,	-5450,	878,	4676,	-5225,	300,	4957,	-4957,	-300,	5225,	-4676,	-878,	5450,	-4275,	-1419,	5642,	-3825,	-1900,	5736,	-3447,	-2475,	5725,	-2981,	},

{	2591,	-5728,	3773,	1806,	-5528,	4379,	474,	-5167,	5167,	-474,	-4379,	5528,	-1806,	-3773,	5728,	-2591,	-2591,	5728,	-3773,	-1806,	5528,	-4379,	-474,	5167,	-5167,	474,	4379,	-5528,	1806,	3773,	-5728,	2591,	},

{	2353,	-5601,	4719,	-153,	-4188,	5722,	-3015,	-2055,	5420,	-4905,	904,	3986,	-5786,	3377,	1318,	-5261,	5261,	-1318,	-3377,	5786,	-3986,	-904,	4905,	-5420,	2055,	3015,	-5722,	4188,	153,	-4719,	5601,	-2353,	},

{	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	2304,	-5312,	5312,	-2304,	-2304,	5312,	-5312,	2304,	},

{	1877,	-4990,	5753,	-3842,	334,	3560,	-5553,	5204,	-2553,	-1521,	4570,	-5772,	4329,	-759,	-2865,	5449,	-5449,	2865,	759,	-4329,	5772,	-4570,	1521,	2553,	-5204,	5553,	-3560,	-334,	3842,	-5753,	4990,	-1877,	},

{	1639,	-4506,	5747,	-5096,	2802,	619,	-3568,	5543,	-5543,	3568,	-619,	-2802,	5096,	-5747,	4506,	-1639,	-1639,	4506,	-5747,	5096,	-2802,	-619,	3568,	-5543,	5543,	-3568,	619,	2802,	-5096,	5747,	-4506,	1639,	},

{	1332,	-3893,	5456,	-5727,	4710,	-2478,	-270,	3039,	-4953,	5760,	-5246,	3378,	-933,	-2002,	4249,	-5622,	5622,	-4249,	2002,	933,	-3378,	5246,	-5760,	4953,	-3039,	270,	2478,	-4710,	5727,	-5456,	3893,	-1332,	},

{	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	1088,	-3264,	4800,	-5696,	5696,	-4800,	3264,	-1088,	-1088,	3264,	-4800,	5696,	-5696,	4800,	-3264,	1088,	},

{	875,	-2464,	3927,	-4925,	5600,	-5806,	5445,	-4650,	3425,	-1935,	308,	1350,	-3000,	4301,	-5223,	5675,	-5675,	5223,	-4301,	3000,	-1350,	-308,	1935,	-3425,	4650,	-5445,	5806,	-5600,	4925,	-3927,	2464,	-875,	},

{	512,	-1664,	2688,	-3712,	4480,	-5120,	5504,	-5760,	5760,	-5504,	5120,	-4480,	3712,	-2688,	1664,	-512,	-512,	1664,	-2688,	3712,	-4480,	5120,	-5504,	5760,	-5760,	5504,	-5120,	4480,	-3712,	2688,	-1664,	512,	},

{	256,	-832,	1408,	-1984,	2432,	-2944,	3456,	-3904,	4288,	-4608,	4992,	-5248,	5440,	-5632,	5696,	-5760,	5760,	-5696,	5632,	-5440,	5248,	-4992,	4608,	-4288,	3904,	-3456,	2944,	-2432,	1984,	-1408,	832,	-256,	},

};