| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0292 |
| --- | --- |

| Title: | Non-CE1: SAO syntax relocation for interleaving mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi, Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 yasugi.yukinobu@sharp.co.jp, ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a change of syntax elements location associated with SAO unit for interleave mode. It is intended that the order of parsing parameters will be the same as that of encoding process. In proposal, the syntax elements associated with SAO unit are moved just after the coding tree data. Experimental results show that the proposed method does not affect the coding efficiency.

Introduction

In the current HEVC[1], SAO unit is encoded before coding tree as Table 1 and Table 2shows. However, at the encoding side, the LCU coding tree data have to be re-encoded so that the SAO unit is inserted before the coding tree.

Proposed method

Relocation of SAO unit syntax

In order to resolve the encoder side problem, we propose to relocate the SAO unit just after the coding tree. Table 3 and Table 4 show the proposed syntax of slice data. The syntax elements of coding tree (highlighted by yellow) are put before SAO unit. The order of parsing and reconstructing data will be unified by this relocation of syntax elements.

Table 1.  Slice data syntax ( HM6 )

| slice_data( ) { | Descriptor |
| --- | --- |
| ... |  |
| do { |  |
| ... |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| ... |  |
| } while( moreDataFlag ) |  |
| } |  |

Highlighting: SAO unit, coding tree

Table 2.  Coding tree syntax ( HM6 )

| coding_tree( x0, y0, log2CbSize, cbDepth ) { | Descriptor |
| --- | --- |
| ... |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| ... |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if( !( ( x0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && !( ( y0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && NumPCMBlock = = 0 ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

Table 3.  Slice data syntax ( Proposal )

| slice_data( ) { | Descriptor |
| --- | --- |
| ... |  |
| do { |  |
| ... |  |
| moreDataFlag = coding_tree( xCtb, yCtb, Log2CtbSize, 0 ) |  |
| ... |  |
| } while( moreDataFlag ) |  |
| } |  |

Table 4.  Coding tree syntax ( Proposal )

| coding_tree( x0, y0, log2CbSize, cbDepth ) { | Descriptor |
| --- | --- |
| ... |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| ... |  |
| } else { |  |
| if(adaptive_loop_filter_flag && alf_cu_control_flag ) |  |
| AlfCuFlag[ x0 ][ y0 ] = alf_cu_flag[ AlfCuFlagIdx ] |  |
| if( NumPCMBlock = = 0 ) |  |
| coding_unit( x0, y0, log2CbSize ) |  |
| else |  |
| pcm_sample( x0, y0, log2CbSize ) |  |
| if( !( ( x0 + 1 << log2CbSize ) % CtbSize ) && !( ( y0 + 1 << log2CbSize ) % CtbSize ) && slice_loop_filter_across_slice_enabled_flag && ((CtbAddrRS << SliceGranularity) < SliceCtbAddrRS) && NumPCMBlock = = 0 ) { |  |
| if( slice_sao_interleaving_flag ) { |  |
| if(slice_sample_adaptive_offset_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 0 ) |  |
| if(sao_cb_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 1 ) |  |
| if(sao_cr_enable_flag) |  |
| sao_unit_cabac( xCtb, yCtb, 2 ) |  |
| } |  |
| if( !( ( x0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && !( ( y0 + 1 << log2CbSize ) % ( 1 << SliceGranularity ) ) && NumPCMBlock = = 0 ) { |  |
| end_of_slice_flag | ae(v) |
| moreDataFlag = !end_of_slice_flag |  |
| } else |  |
| moreDataFlag = 1 |  |
| } |  |
| return moreDataFlag |  |
| } |  |

Problem with fine granularity slices

According to the HEVC draft, a single set of SAO parameters is used for SAO filtering process at each LCU regardless of whether the LCU includes multiple slices or not. However, it is considered from the syntax table that the SAO parameters for a mixed-slice LCU are encoded multiple times and able to take different values for each slice.

In order to avoid this ambiguity, the proposed software encodes the SAO parameters only at the bottom-right end of an LCU. In addition, when slice_loop_filter_across_slices_enabled_flag is false, it is inferred that sao_type_idx is zero (“Not applied”) for mixed-slice LCUs and the SAO parameters are not coded.

The proposed method also modified the semantic of sao_type_idx as follows:

7.4.2.8 Sample adaptive offset VLC semantics

sao_type_idx[ cIdx ][ rx ][ ry ] indicates the offset type as specified in Table 7-6 of current coding treeblock at position rx and ry for the colour component cIdx.

When sao_type_idx[ cIdx ][ rx ][ ry ] is not present, it is inferred as follows.

- If aps_sao_interleaving_flag is equal to 1 and both sao_merge_up_flag and sao_merge_left_flag are not present, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to 0.

- If sao_merge_up_flag is equal to 1, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx ][ ry − 1 ].

- Otherwise, sao_type_idx[ cIdx ][ rx ][ ry ] is set equal to sao_type_idx[ cIdx ][ rx − 1 ][ ry ].

Table 7-6 – Specification of the edge type for SAO

| sao_type_idx[ cIdx ][ rx ][ ry ] | Edge type (informative) |
| --- | --- |
| 0 | Not applied |
| 1 | 1D 0-degree edge |
| 2 | 1D 90-degree edge |
| 3 | 1D 135-degree edge |
| 4 | 1D 45-degree edge |
| 5 | Band |

Experimental results

Table 5 is the summary of the experimental results under the CE1 test conditions. We implemented the modification on top of CE1 anchor software, which is also used as the anchor. Note that the experiment was carried out on our PC cluster environment, and the results about runtimes may not be accurate. The results show that the modification does not impact the BD-rate performance. The results were confirmed by MediaTek (JCTVC-I0210).

Table 5. BD-rate results of the proposed modification compared to CE1 anchor (64x64 LCU)

Experimental results with fine granularity slices

We also tested the proposal with fine granularity slices. The software was implemented on top of HM-6.1 rev.2197 (includes CE1 anchor modification).

Base software (anchor):	HM-6.1 rev.2197 + CE1 anchor modification

Tested conditions:	IO_Main, RA_Main, LB_Main and LP_Main

Test A1)	Proposed method vs. Anchor

Encoder options:	--SAOInterleaving=1  --SliceMode=2  --SliceArgument=1500  
--SliceGranularity=1  -dqd=1  -- DFCrossSliceBoundaryFlag =1

Test A2)	Proposed method vs. Anchor

Encoder options:	--SAOInterleaving=1  --SliceMode=2  --SliceArgument=1500  
--SliceGranularity=1  -dqd=1  -- DFCrossSliceBoundaryFlag =0

Table 6 and Table 7 show the results of Test A1 and Test A2, respectively. As shown in Table 6, there are differences in BD-rate at maximum -0.1%. The reason comes from the implementation described above. The proposed software encodes SAO parameters only once for each LCU even if it is a mixed-slice LCU. On the other hand, HM-6.0/6.1 encodes it twice (or more) for a mixed-slice LCU.

Table 6. BD-rate results of Test A1

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  | Low delay P Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | (To be updated) |  |  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| Class D | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | -0.1% | -0.1% |  |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.3% |
| Overall |  |  |  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
|  |  |  |  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] |  |  |  | 98% |  |  | 99% |  |  | 100% |  |  |
| Dec Time[%] | 85% |  |  | 100% |  |  | 101% |  |  | 101% |  |  |

Table 7. BD-rate results of Test A2

(To be updated)

Conclusion

This contribution proposes change of syntax elements location associated with SAO unit for interleave mode. It is intended that the order of parsing parameters is the same as that of encoding process. In proposal, the syntax elements associated with SAO unit are moved to the end of the coding tree data. Experimental results show that the proposed method does not affect the coding efficiency.

Reference

[1] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 6, JCTVC-H1003,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 8th Meeting: San José, USA, 1-10 February, 2012

Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).