| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0089 |
| --- | --- |

| Title: | Non SCE4: simplified design of cross-color inter-layer (test 4.2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin Yongjin Cho 416, Maetan 3-Dong, Gyeonggi-do, Suwon, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, alexander_b.alshin, yongjin9.cho} @samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

- Abstract

In order to simplify samples processing and reduce the latency on decoder side 2 modifications are of very promising cross-color inter-layer filter tested in SCE4 are proposed in this contribution. Variable de-scaling shift in original design was replaced by fixed left shift. This modification simplifies samples processing and it is almost lossless: cross-color inter-layer filter provides 0,4% Luma and 7,3%(IntraBL)/7,8% (RefIdx) Chroma BD-rate gain in average. Instead up-sampled Luma signal we suggest to use reconstructed base-layer Luma for cross-color inter-layer filtering. In this case decoder doesn’t need to wait until Luma will be up-sampled and inter-layer prediction for Luma and Chroma are independent. This modification reduces the gain from cross-color inter-layer filter to 0,3% Luma and 6,1%(IntraBL)/6,6% (RefIdx) but makes design implementation friendly.

Introduction

Cross-color inter-layer filtering (proponents call it also “Chroma enhancement”) was proposed by Inter Digital in [JCTVC-L0059]. This tool was included into SCE4 and provided 0,4% (Luma) and 7,6% (Chroma) average BD-rate gain.

Tool works both for IntraBL and RefIdx frameworks and requires access to only reconstructed collocated base-layer picture (which is any-way needed for inter-layer texture prediction).

Cross-color inter-layer filtering is implemented as 2D no-separable 4x3 filter, which refers to up-sampled Luma signal in order to reduce the difference between up-sampled and original Chroma signal (as shown on Figure 1).

Separable filters are very friendly for SIMD implementation. 2D non-separable filter from Fig. 1 is equivalent to 3 times application of 1D 4 taps filter. So compare to 2D separable up-sampling filter in SHM1.0 (which is 4 taps) amount of operations will be ~1.5 times higher.

Variable filter coefficients are not friendly for implementation. But this tools was proposed as additional inter-layer filter (not up-sampling filter which is key part of SHVC design), which could be switched on and off in SPS. In this case variable coefficients are acceptable.

Fig. 1. Cross-color inter-layer filter: circles – Luma, stars –Chroma, red star – current Chroma sample, red dots surround Luma samples taken into account for current Chroma sample processing.

Proposed modification A.

The goal of this modification is sample processing and slice header simplification.

In original design of cross-color inter-layer filter sample processing is performed as follows:

Chx,y=ClipChx,y+Qj=v_startv_endi=h_starth_endfi,jLuma2x+i,2y+j≫S  .                                           (1)

Values Q, S and f(i,j) are signaled in slice-header for Cb and Cr (separately). Additionally tool can be switched On and Off Cb and Cr separately (with corresponding flag encoding in slice header).

Variable shift is not friendly for implementation. According to statistics collected using bit-streams from test 4.2.4 in SCE4 in majority of cases S=16 was chosen. We propose to fix this value equal to 16. This simplifies implementation and saves 10 bits in slice header (in original design S was encoded using 5 bits fixed length code).

If slice level on/off flag for this tool is false then Chroma samples will not be modified. The same will happen if Q in (1) is equal to 0. In order to avoid duplicated functionality of syntax elements in a slice header we suggest modifying coding for Q:

- In original design

  - Q≤1023 is encoded using 10-bits fixed length code; if this value is not zero the sign of Q was signaled (1 bit flag).

- In proposed design

  - -1024≤Q<0∪0<Q≤1024 is encoded using 11-bits fixed length code.

In terms of specification text proposed modification looks as follows shown in Tables 1-2. Highlighted in yellow lines (which include condition check) are removed.

Table 1. Slice header specification text for original design.

| cross_color_filter_cb_flag | u(1) |
| --- | --- |
| if (cross_color_filter_cb _flag) { |  |
| cb_ cross_color _filter_ mult_abs | u(10) |
| for( k = 0; k < 12; k++ ) |  |
| cb_ cross_color _filter_coeff_plus8[k] | u(4) |
| if (cb_ cross_color _filter_ mult_abs){ |  |
| cb_ cross_color _filter_ mult_sign | u(1) |
| cb_ cross_color _filter_ shift | u(5) |
| } |  |
| } |  |
| cross_color_filter_cr_flag | u(1) |
| if (cross_color_filter_cr _flag) { |  |
| cr_ cross_color _filter_ mult_abs | u(10) |
| for( k = 0; k < 12; k++ ) |  |
| cr_ cross_color _filter_coeff_plus8[k] | u(4) |
| if (cr_ cross_color _filter_ mult_abs){ |  |
| cr_ cross_color _filter_ mult_sign | u(1) |
| cr_ cross_color _filter_ shift | u(5) |
| } |  |
| } |  |

Table 2. Slice header specification text for proposed design

| cross_color_filter_cb_flag | u(1) |
| --- | --- |
| if (cross_color_filter_cb _flag) { |  |
| cb_ cross_color _filter_no_zero_mult | u(11) |
| for( k = 0; k < 12; k++ ) |  |
| cb_ cross_color _filter_coeff_plus8[k] | u(4) |
| } |  |
| cross_color_filter_cr _flag | u(1) |
| if (cross_color_filter_cr flag) { |  |
| cr_ cross_color _filter_no_zero_mult | u(11) |
| for( k = 0; k < 12; k++ ) |  |
| cr_ cross_color_filter _filter_coeff_plus8[k] | u(4) |
| } |  |

Proposed modification B.

The goal of this modification is resolving the latency.

SHM1.0 uses inter-layer texture prediction which is independent for Luma and Chroma.

The block diagram for original design of cross-color inter-layer filter is shown Fig. 2. In order to start cross-color filtering of Chroma decoder shall wait until up-sampling for Luma component will be finished.

Fig. 2. Original design

Fig. 3. Proposed design.

We would suggest to use reconstructed Luma samples from base-layer (before up-sampling) instead up-sampled Luma signal. In this case Luma and Chroma inter-layer texture prediction can be done in parallel. And there is no need for decoder to wait until Luma up-sampling is finished during inter-layer Chroma texture prediction.

Performance test results

All tests were performed following SCE4 description: all intra, random access and low-delay-P tests were performed for x2, x1,5 and SNR scalability tests. Tool was tested in both IntraBL and RefIdx frameworks. Optional low-delay –B test results are also provided, but excluded from average in order to simplify comparison with SCE output.

Brief summary of test results is shown in Table 3: “ALL” means average BD-rate counted across all mandatory tests; “AI” – “all intra” ; “MC” – random access and low-delay-P spatial scalability tests ; “SNR” – SNR scalability tests. Test 4.2.4 was released in 2 variants. Our implementation and tests are based on v1 (which was available earlier).

Table 3. Performance test summary.

| Tool | ALL Luma | ALL Chroma | AI Luma | AI Chroma | MC Luma | MC Chroma | SNR Luma | SNR Chroma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraBL framework |  |  |  |  |  |  |  |  |
| 4.2.4 v1 | -0,38% | -7,57% | -0,77% | -7,63% | -0,23% | -8,23% | -0,27% | -6,18% |
| Modification A | -0,37% | -7,25% | -0,74% | -7,19% | -0,23% | -7,80% | -0,27% | -6,21% |
| Modification A&B | -0,30% | -6,12% | -0,60% | -5,47% | -0,16% | -6,40% | -0,27% | -6,21% |
| RefIdx framework |  |  |  |  |  |  |  |  |
| 4.2.4 v1 | -0,40% | -8,14% | -0,79% | -8,31% | -0,25% | -8,73% | -0,28% | -6,79% |
| Modification A | -0,39% | -7,80% | -0,76% | -7,84% | -0,26% | -8,29% | -0,29% | -6,77% |
| Modification A&B | -0,32% | -6,57% | -0,61% | -6,02% | -0,19% | -6,75% | -0,29% | -6,77% |

Details of these tests can be found in section 7 and excel spread-sheets attached to this contribution.

Complexity assessment

Complexity assessment was done according to [2]. There are no changes compare to original design in the worst case. Average numbers for memory access and computations will be up-dated in later revision.

Conclusions

Based on following summary:

- 0,3% average Luma BD-rate gain

- 6,1%(IntraBL)/6,6% (RefIdx) average Chroma BD-rate gain

- No latency on decoder side

- Inter-layer texture prediction can be done for Luma and Chroma in parallel

- Constant de-scaling shift

- Simplified compare to original version syntax in slice-header

Samsung suggests adoption of cross-color inter-layer filter to the next version for SHVC test model with proposed simplifications.

References

[1] J. Dong, Y. He, Y. Ye, Chroma enhancement for ILR picture JCTVC-L0059 document, January, 2013.

[2] E. François, A. Tabatabai, E. Alshina, BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC, JCTVC-L0440 document, January, 2013.

Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Details of tests

Table 4. Performance test summary for modification A in IntraBL framework.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0,9% | -7,3% | -5,7% |  |  |  |  |  |  |
| Class B | -0,8% | -6,1% | -8,0% | -0,7% | -7,4% | -9,3% |  |  |  |
| Overall (Test vs Ref) | -0,8% | -6,4% | -7,3% | -0,7% | -7,4% | -9,3% |  |  |  |
| Overall (Test vs single layer) | 11,5% | 6,6% | 5,6% | 9,5% | 2,4% | 0,4% |  |  |  |
| EL only (Test vs Ref) | -1,5% | -7,6% | -8,4% | -2,2% | -10,3% | -12,4% |  |  |  |
| Enc Time[%] | 102,1% |  |  | 101,7% |  |  |  |  |  |
| Dec Time[%] | 106,5% |  |  | 104,5% |  |  |  |  |  |
| Enc Mem[%] | 102,6% |  |  | 103,7% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,4% | -10,6% | -7,1% |  |  |  | -0,4% | -8,4% | -4,9% |
| Class B | -0,3% | -7,7% | -9,0% | -0,2% | -9,5% | -10,8% | -0,3% | -6,6% | -7,4% |
| Overall (Test vs Ref) | -0,3% | -8,5% | -8,5% | -0,2% | -9,5% | -10,8% | -0,3% | -7,1% | -6,7% |
| Overall (Test vs single layer) | 18,9% | 20,6% | 22,3% | 16,3% | 16,7% | 17,7% | 14,5% | 19,7% | 24,1% |
| EL only (Test vs Ref) | -0,6% | -9,2% | -9,0% | -0,7% | -10,9% | -12,1% | -0,6% | -8,0% | -7,4% |
| Enc Time[%] | 100,8% |  |  | 100,6% |  |  | 100,4% |  |  |
| Dec Time[%] | 108,7% |  |  | 107,8% |  |  | 108,7% |  |  |
| Enc Mem[%] | 103,2% |  |  | 104,2% |  |  | 112,2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,2% | -9,1% | -5,2% |  |  |  | -0,3% | -7,1% | -4,3% |
| Class B | -0,2% | -4,7% | -5,9% | -0,1% | -7,4% | -9,6% | -0,2% | -4,8% | -5,9% |
| Overall (Test vs Ref) | -0,2% | -6,0% | -5,7% | -0,1% | -7,4% | -9,6% | -0,2% | -5,5% | -5,4% |
| Overall (Test vs single layer) | 25,8% | 28,0% | 30,5% | 22,4% | 23,6% | 23,5% | 22,1% | 26,9% | 31,6% |
| EL only (Test vs Ref) | -0,3% | -6,3% | -5,8% | -0,5% | -8,1% | -10,2% | -0,3% | -6,0% | -5,8% |
| Enc Time[%] | 100,7% |  |  | 100,4% |  |  | 100,2% |  |  |
| Dec Time[%] | 109,3% |  |  | 107,2% |  |  | 108,9% |  |  |
| Enc Mem[%] | 103,3% |  |  | 104,4% |  |  | 112,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 5. Performance test summary for modification A in RefIdx framework.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0,8% | -6,9% | -5,5% |  |  |  |  |  |  |
| Class B | -0,8% | -5,7% | -7,5% | -0,7% | -8,1% | -10,2% |  |  |  |
| Overall (Test vs Ref) | -0,8% | -6,1% | -6,9% | -0,7% | -8,1% | -10,2% |  |  |  |
| Overall (Test vs single layer) | 12,0% | 8,2% | 7,1% | 9,7% | 1,2% | -0,8% |  |  |  |
| EL only (Test vs Ref) | -1,4% | -7,1% | -8,0% | -2,2% | -11,2% | -13,5% |  |  |  |
| Enc Time[%] | 104,3% |  |  | 104,1% |  |  |  |  |  |
| Dec Time[%] | 104,8% |  |  | 105,0% |  |  |  |  |  |
| Enc Mem[%] | 104,4% |  |  | 105,8% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,4% | -9,9% | -6,6% |  |  |  | -0,4% | -9,0% | -5,4% |
| Class B | -0,3% | -7,2% | -8,7% | -0,3% | -10,5% | -12,0% | -0,3% | -7,7% | -8,5% |
| Overall (Test vs Ref) | -0,3% | -8,0% | -8,1% | -0,3% | -10,5% | -12,0% | -0,3% | -8,1% | -7,6% |
| Overall (Test vs single layer) | 19,7% | 22,2% | 23,6% | 17,0% | 14,9% | 15,8% | 15,4% | 19,4% | 24,7% |
| EL only (Test vs Ref) | -0,6% | -8,6% | -8,6% | -0,8% | -11,9% | -13,3% | -0,6% | -8,9% | -8,4% |
| Enc Time[%] | 101,4% |  |  | 101,2% |  |  | 100,9% |  |  |
| Dec Time[%] | 108,7% |  |  | 108,3% |  |  | 107,5% |  |  |
| Enc Mem[%] | 103,5% |  |  | 104,4% |  |  | 112,2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,2% | -8,1% | -4,4% |  |  |  | -0,3% | -7,3% | -4,3% |
| Class B | -0,2% | -4,1% | -5,3% | -0,2% | -7,5% | -10,0% | -0,2% | -5,2% | -6,1% |
| Overall (Test vs Ref) | -0,2% | -5,2% | -5,1% | -0,2% | -7,5% | -10,0% | -0,3% | -5,8% | -5,6% |
| Overall (Test vs single layer) | 26,4% | 30,1% | 32,1% | 22,6% | 23,1% | 22,5% | 23,1% | 26,6% | 31,6% |
| EL only (Test vs Ref) | -0,3% | -5,5% | -5,2% | -0,6% | -8,3% | -10,8% | -0,4% | -6,3% | -6,0% |
| Enc Time[%] | 101,4% |  |  | 101,1% |  |  | 100,9% |  |  |
| Dec Time[%] | 109,8% |  |  | 108,9% |  |  | 108,1% |  |  |
| Enc Mem[%] | 103,7% |  |  | 104,8% |  |  | 112,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Table 6. Performance test summary for modifications A&B in IntraBL framework.

|  | AI HEVC 2x |  |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |  |  |
| Class A | -1,1% | -9,0% | -6,4% |  |  |  |  |  |  |  |
| Class B | -0,3% | -1,4% | -2,6% |  | -0,7% | -6,6% | -8,1% |  |  |  |
| Overall (Test vs Ref) | -0,5% | -3,5% | -3,6% |  | -0,7% | -6,6% | -8,1% |  |  |  |
| Overall (Test vs single layer) | 11,8% | 9,9% | 9,5% |  | 9,6% | 3,3% | 1,6% |  |  |  |
| EL only (Test vs Ref) | -1,0% | -4,3% | -4,3% |  | -2,1% | -9,2% | -10,8% |  |  |  |
| Enc Time[%] | 102,2% |  |  |  | 101,7% |  |  |  |  |  |
| Dec Time[%] | 108,0% |  |  |  | 106,4% |  |  |  |  |  |
| Enc Mem[%] | 104,0% |  |  |  | 103,9% |  |  |  |  |  |
| BL Match | Matched |  |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U |  | V | Y | U | V | Y | U | V |
| Class A | -0,5% | -13,1% |  | -7,8% |  |  |  | -0,4% | -8,4% | -4,9% |
| Class B | 0,0% | -2,0% |  | -3,4% | -0,2% | -8,6% | -9,7% | -0,3% | -6,7% | -7,4% |
| Overall (Test vs Ref) | -0,2% | -5,2% |  | -4,7% | -0,2% | -8,6% | -9,7% | -0,3% | -7,2% | -6,7% |
| Overall (Test vs single layer) | 19,1% | 24,8% |  | 27,0% | 16,3% | 17,8% | 19,0% | 14,4% | 19,6% | 24,1% |
| EL only (Test vs Ref) | -0,3% | -5,5% |  | -5,0% | -0,6% | -9,9% | -10,8% | -0,6% | -8,0% | -7,4% |
| Enc Time[%] | 101,0% |  |  |  | 100,7% |  |  | 100,7% |  |  |
| Dec Time[%] | 110,3% |  |  |  | 109,1% |  |  | 109,9% |  |  |
| Enc Mem[%] | 103,2% |  |  |  | 104,3% |  |  | 112,2% |  |  |
| BL Match | Matched |  |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U |  | V | Y | U | V | Y | U | V |
| Class A | -0,3% | -11,2% |  | -5,9% |  |  |  | -0,3% | -7,2% | -4,2% |
| Class B | -0,1% | -1,4% |  | -2,3% | -0,1% | -6,8% | -8,7% | -0,2% | -4,8% | -6,0% |
| Overall (Test vs Ref) | -0,1% | -4,2% |  | -3,3% | -0,1% | -6,8% | -8,7% | -0,2% | -5,5% | -5,5% |
| Overall (Test vs single layer) | 25,9% | 30,1% |  | 33,6% | 22,4% | 24,3% | 24,7% | 22,1% | 26,9% | 31,5% |
| EL only (Test vs Ref) | -0,2% | -4,5% |  | -3,5% | -0,4% | -7,5% | -9,3% | -0,4% | -6,0% | -5,9% |
| Enc Time[%] | 100,9% |  |  |  | 100,5% |  |  | 100,5% |  |  |
| Dec Time[%] | 110,6% |  |  |  | 109,9% |  |  | 109,6% |  |  |
| Enc Mem[%] | 103,3% |  |  |  | 104,2% |  |  | 112,3% |  |  |
| BL Match | Matched |  |  |  | Matched |  |  | Matched |  |  |

Table 7. Performance test summary for modifications A&B in RefIdx framework.

|  | AI HEVC 2x |  |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  |  |  |
| Class A | -1,1% | -9,5% | -6,9% |  |  |  |  |  |  |  |
| Class B | -0,3% | -1,5% | -2,8% |  | -0,7% | -7,3% | -9,0% |  |  |  |
| Overall (Test vs Ref) | -0,5% | -3,8% | -3,9% |  | -0,7% | -7,3% | -9,0% |  |  |  |
| Overall (Test vs single layer) | 12,2% | 10,9% | 10,5% |  | 9,8% | 2,1% | 0,4% |  |  |  |
| EL only (Test vs Ref) | -1,0% | -4,6% | -4,6% |  | -2,1% | -10,1% | -11,9% |  |  |  |
| Enc Time[%] | 103,9% |  |  |  | 103,6% |  |  |  |  |  |
| Dec Time[%] | 105,7% |  |  |  | 105,7% |  |  |  |  |  |
| Enc Mem[%] | 103,7% |  |  |  | 105,4% |  |  |  |  |  |
| BL Match | Matched |  |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U |  | V | Y | U | V | Y | U | V |
| Class A | -0,6% | -13,7% |  | -8,4% |  |  |  | -0,4% | -9,0% | -5,4% |
| Class B | 0,0% | -2,0% |  | -3,6% | -0,2% | -9,5% | -10,7% | -0,3% | -7,7% | -8,5% |
| Overall (Test vs Ref) | -0,2% | -5,4% |  | -5,0% | -0,2% | -9,5% | -10,7% | -0,3% | -8,1% | -7,6% |
| Overall (Test vs single layer) | 19,9% | 25,5% |  | 27,7% | 17,1% | 16,1% | 17,3% | 15,4% | 19,4% | 24,7% |
| EL only (Test vs Ref) | -0,4% | -5,7% |  | -5,3% | -0,7% | -10,8% | -11,9% | -0,6% | -8,9% | -8,4% |
| Enc Time[%] | 101,2% |  |  |  | 100,9% |  |  | 100,7% |  |  |
| Dec Time[%] | 111,5% |  |  |  | 110,4% |  |  | 109,7% |  |  |
| Enc Mem[%] | 103,5% |  |  |  | 104,6% |  |  | 112,2% |  |  |
| BL Match | Matched |  |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V |  | Y | U | V | Y | U | V |
| Class A | -0,3% | -11,3% | -5,9% |  |  |  |  | -0,3% | -7,3% | -4,3% |
| Class B | 0,0% | -1,4% | -2,0% |  | -0,2% | -7,0% | -9,1% | -0,2% | -5,2% | -6,1% |
| Overall (Test vs Ref) | -0,1% | -4,3% | -3,1% |  | -0,2% | -7,0% | -9,1% | -0,3% | -5,8% | -5,6% |
| Overall (Test vs single layer) | 26,5% | 31,3% | 34,8% |  | 22,7% | 23,7% | 23,8% | 23,1% | 26,6% | 31,6% |
| EL only (Test vs Ref) | -0,2% | -4,5% | -3,2% |  | -0,5% | -7,7% | -9,7% | -0,4% | -6,3% | -6,0% |
| Enc Time[%] | 101,3% |  |  |  | 101,0% |  |  | 100,8% |  |  |
| Dec Time[%] | 110,9% |  |  |  | 110,4% |  |  | 110,8% |  |  |
| Enc Mem[%] | 103,7% |  |  |  | 104,7% |  |  | 112,4% |  |  |
| BL Match | Matched |  |  |  | Matched |  |  | Matched |  |  |