    - Traverse scan order array initialization process

Input to this process is a block size blkSize.

Output of this process is the array travScan[ sPos ][ sComp ]. The array index sPos specifies the scan position ranging from 0 to ( blkSize * blkSize ) − 1, inclusive. The array index sComp equal to 0 specifies the horizontal component and the array index sComp equal to 1 specifies the vertical component. Depending on the value of blkSize, the array travScan is derived as follows:

–	If blkSize is smaller than 64, the array travScan is derived as follows:

i = 0
for( y = 0; y  < blkSize; y++ )
	if( y % 2  = =  0 )
		for( x = 0; x < blkSize; x++ ) {
			travScan[ i ][ 0 ] = x
			travScan[ i ][ 1 ] = y
			i++
		}
	else					(6-14)
		for( x = blkSize − 1; x  >=  0; x− − ) {
			travScan[ i ][ 0 ] = x
			travScan[ i ][ 1 ] = y
			i++
		}

–	Otherwise (if blkSize is equal to 64), the array travScan is derived as follows:

i = 0

for(blkIdx = 0; blkIdx < 4; blkIdx++){
  x_offset = (blkIdx/2)*32
  y_offset = (blkIdx%2)*32
  for( y = 0; y  < 32; y++ )
	if( y % 2  = =  0 )
		for( x = 0; x < 32; x++ ) {
			travScan[ i ][ 0 ] = x + x_offset
			travScan[ i ][ 1 ] = y + y_offset
			i++
		}
	else				 (6-xx)
		for( x = 31; x  >=  0; x− − ) {
			travScan[ i ][ 0 ] = x+ x_offset
			travScan[ i ][ 1 ] = y+ y_offset
			i++
		}

}