

22 May, 1996



SOURCE :    IMTC



CONTACT:   Gary Thom, H.323 Editor

		Phone:	+1-215-657-5270

		Fax: 		+1-215-657-5273

		Email:		gthom@interramp.com



		

TITLE  :Draft Recommendation H.323: VISUAL TELEPHONE SYSTEMS AND     EQUIPMENT FOR LOCAL AREA  NETWORKS  WHICH  PROVIDE  A  NON-GUARANTEED  QUALITY  OF  SERVICE



This document reflects changes indicated in the 22 May 1996 Proposed Modifications to COM15-245 (H.323).



���

INTERNATIONAL  TELECOMMUNICATION  UNION��

ITU-T	DRAFT H.323

TELECOMMUNICATION	January 30May 22, 1996�STANDARDIZATION  SECTOR                                                   Determined November 1995�OF  ITU





LINE  TRANSMISSION  OF  NON-TELEPHONE�SIGNALS

�





VISUAL  TELEPHONE  SYSTEMS  AND   EQUIPMENT FOR  LOCAL  AREA  NETWORKS  WHICH  PROVIDE  A  NON-GUARANTEED  QUALITY  OF  SERVICE ��

DRAFT  ITU-T  Recommendation  H.323

		





�







FOREWORD



The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Tele�com�munication Union.  The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation H.323 was prepared by the ITU-T Study Group 15 (199x-199x) and was approved by the WTSC (Place, Month xx-xx, 199x).

___________________











ã  ITU  199x

All rights reserved.  No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

�

SUMMARY



Recommendation H.323 describes terminals, equipment, and services for multimedia communication over Local Area Networks (LAN) which do not provide a guaranteed Quality of Service.  H.323 terminals and equipment may carry real-time voice, data, and video, or any combination, including videotelephony.

The LAN over which H.323 terminals communicate, may be a single segment or ring, or it may be multiple segments with complex topologies.  It should be noted that operation of H.323 terminals over the multiple LAN segments (including the Internet) may result in poor performance.  The possible means by which quality of service might be assured on such types of LANs/internetworks is beyond the scope of this recommendation.

H.323 terminals may be integrated into personal computers or implemented in stand-alone devices such as videotelephones.  Support for voice is mandatory, while data and video are optional, but if supported, the ability to use a specified common mode of operation is required, so that all terminals supporting that media type can interwork.  H.323 allows more than one channel of each type to be in use.  Other Recommendations in the H.323 series include H.225.0 packet and synchronization, H.245 control, H.261 and H.263 video codecs, G.711, G.722, G.728, G.729, and G.723 audio codecs, and the T.120 series of multimedia communications protocols.

H.323 makes use of the logical channel signalling procedures of Recommendation H.245, in which the content of each logical channel is described when the channel is opened.  Procedures are provided for expression of receiver and transmitter capabilities, so transmissions are limited to what receivers can decode, and so that receivers may request a particular desired mode from transmitters.  Since the procedures of H.245 are also used by Recommendation H.310 for ATM networks, Recommendation H.324 for GSTN, and V.70, interworking with these systems should not require H.242 to H.245 translation as would be the case for H.320 systems.

H.323 terminals may be used in multipoint configurations, and may interwork with H.310 terminals on B-ISDN, H.320 terminals on N-ISDN, H.321 terminals on B-ISDN, H.322 terminals on Guaranteed Quality of Service LANs, H.324 terminals on GSTN and wireless networks, and V.70 terminals on GSTN.

�Table of Contents

�toc \o �SUMMARY	� GOTOBUTTON _Toc356953936  � PAGEREF _Toc356953936 �iiii��

1 Scope	� GOTOBUTTON _Toc356953937  � PAGEREF _Toc356953937 �11��

2 Normative references	� GOTOBUTTON _Toc356953938  � PAGEREF _Toc356953938 �22��

3 Definitions	� GOTOBUTTON _Toc356953939  � PAGEREF _Toc356953939 �44��

4 Symbols and abbreviations	� GOTOBUTTON _Toc356953940  � PAGEREF _Toc356953940 �88��

5 Conventions	� GOTOBUTTON _Toc356953943  � PAGEREF _Toc356953943 �1010��

6 System description	� GOTOBUTTON _Toc356953944  � PAGEREF _Toc356953944 �1010��

6.1 Information Streams	� GOTOBUTTON _Toc356953945  � PAGEREF _Toc356953945 �1010��

6.2 Terminal Characteristics	� GOTOBUTTON _Toc356953946  � PAGEREF _Toc356953946 �1111��

6.2.1 Terminal elements outside the scope of H.323	� GOTOBUTTON _Toc356953947  � PAGEREF _Toc356953947 �1212��

6.2.2 Terminal elements within the scope of H.323	� GOTOBUTTON _Toc356953948  � PAGEREF _Toc356953948 �1313��

6.2.3 LAN Interface	� GOTOBUTTON _Toc356953949  � PAGEREF _Toc356953949 �1313��

6.2.4 Video Codec	� GOTOBUTTON _Toc356953950  � PAGEREF _Toc356953950 �1414��

6.2.4.1 Terminal Based Continuous Presence	� GOTOBUTTON _Toc356953951  � PAGEREF _Toc356953951 �1414��

6.2.5 Audio Codec	� GOTOBUTTON _Toc356953952  � PAGEREF _Toc356953952 �1515��

6.2.5.1 Audio Mixing	� GOTOBUTTON _Toc356953953  � PAGEREF _Toc356953953 �1515��

6.2.5.2 Maximum Audio-Video Transmit Skew	� GOTOBUTTON _Toc356953954  � PAGEREF _Toc356953954 �1515��

6.2.6 Receive Path Delay	� GOTOBUTTON _Toc356953955  � PAGEREF _Toc356953955 �1515��

6.2.7 Data Channel	� GOTOBUTTON _Toc356953956  � PAGEREF _Toc356953956 �1616��

6.2.7.1 T.120 Data Channels	� GOTOBUTTON _Toc356953957  � PAGEREF _Toc356953957 �1616��

6.2.8 H.245 Control Function	� GOTOBUTTON _Toc356953958  � PAGEREF _Toc356953958 �1818��

6.2.8.1 Capabilities exchange	� GOTOBUTTON _Toc356953959  � PAGEREF _Toc356953959 �1919��

6.2.8.2 Logical channel signalling	� GOTOBUTTON _Toc356953960  � PAGEREF _Toc356953960 �2020��

6.2.8.3 Mode preferences	� GOTOBUTTON _Toc356953961  � PAGEREF _Toc356953961 �2121��

6.2.8.4 Master Slave Determination	� GOTOBUTTON _Toc356953962  � PAGEREF _Toc356953962 �2121��

6.2.8.5 Timer and counter values	� GOTOBUTTON _Toc356953963  � PAGEREF _Toc356953963 �2222��

6.2.9 RAS Signalling Function	� GOTOBUTTON _Toc356953964  � PAGEREF _Toc356953964 �2222��

6.2.10 Call Signalling Function	� GOTOBUTTON _Toc356953965  � PAGEREF _Toc356953965 �2222��

6.2.11 H.225.0 Layer	� GOTOBUTTON _Toc356953966  � PAGEREF _Toc356953966 �2323��

6.2.11.1 Logical channel numbers	� GOTOBUTTON _Toc356953967  � PAGEREF _Toc356953967 �2323��

6.2.11.2 Logical Channel Bitrate Limits	� GOTOBUTTON _Toc356953968  � PAGEREF _Toc356953968 �2323��

6.3 Gateway Characteristics	� GOTOBUTTON _Toc356953969  � PAGEREF _Toc356953969 �2323��

6.4 Gatekeeper Characteristics	� GOTOBUTTON _Toc356953970  � PAGEREF _Toc356953970 �2626��

6.5 Multipoint Controller Characteristics	� GOTOBUTTON _Toc356953971  � PAGEREF _Toc356953971 �2727��

6.6 Multipoint Processor Characteristics	� GOTOBUTTON _Toc356953972  � PAGEREF _Toc356953972 �2929��

6.7 Multipoint Control Unit Characteristics	� GOTOBUTTON _Toc356953973  � PAGEREF _Toc356953973 �2929��

6.8 Multipoint Capability	� GOTOBUTTON _Toc356953974  � PAGEREF _Toc356953974 �3030��

6.8.1 Centralized Multipoint Capability	� GOTOBUTTON _Toc356953975  � PAGEREF _Toc356953975 �3030��

6.8.2 Decentralized Multipoint Capability	� GOTOBUTTON _Toc356953976  � PAGEREF _Toc356953976 �3030��

6.8.3 Hybrid Multipoint - Centralized Audio	� GOTOBUTTON _Toc356953977  � PAGEREF _Toc356953977 �3030��

6.8.4 Hybrid Multipoint - Centralized Video	� GOTOBUTTON _Toc356953978  � PAGEREF _Toc356953978 �3131��

6.8.5 Establishment of Common Mode	� GOTOBUTTON _Toc356953979  � PAGEREF _Toc356953979 �3131��

6.8.6 Multipoint rate matching	� GOTOBUTTON _Toc356953980  � PAGEREF _Toc356953980 �3232��

6.8.7 Multipoint Lip Synchronization	� GOTOBUTTON _Toc356953981  � PAGEREF _Toc356953981 �3232��

6.8.8 Multipoint Encryption	� GOTOBUTTON _Toc356953982  � PAGEREF _Toc356953982 �3232��

6.8.9 Cascading Multipoint Control Units	� GOTOBUTTON _Toc356953983  � PAGEREF _Toc356953983 �3232��

7 Call Signalling	� GOTOBUTTON _Toc356953984  � PAGEREF _Toc356953984 �3232��

7.1 Addresses	� GOTOBUTTON _Toc356953985  � PAGEREF _Toc356953985 �3232��

7.1.1 LAN Address	� GOTOBUTTON _Toc356953986  � PAGEREF _Toc356953986 �3232��

7.1.2 TSAP Identifier	� GOTOBUTTON _Toc356953987  � PAGEREF _Toc356953987 �3333��

7.1.3 Alias Address	� GOTOBUTTON _Toc356953988  � PAGEREF _Toc356953988 �3333��

7.2 Registration, Admissions, and Status (RAS) Channel	� GOTOBUTTON _Toc356953989  � PAGEREF _Toc356953989 �3434��

7.2.1 Gatekeeper Discovery	� GOTOBUTTON _Toc356953990  � PAGEREF _Toc356953990 �3434��

7.2.2 Endpoint Registration	� GOTOBUTTON _Toc356953991  � PAGEREF _Toc356953991 �3535��

7.2.3 Endpoint Location	� GOTOBUTTON _Toc356953992  � PAGEREF _Toc356953992 �3737��

7.2.4 Admissions, Bandwidth Change, Status, Disengage	� GOTOBUTTON _Toc356953993  � PAGEREF _Toc356953993 �3737��

7.3 Call Signalling Channel	� GOTOBUTTON _Toc356953994  � PAGEREF _Toc356953994 �3737��

7.3.1 Call Signalling Channel Routing	� GOTOBUTTON _Toc356953995  � PAGEREF _Toc356953995 �3838��

7.3.2 Control Channel Routing	� GOTOBUTTON _Toc356953996  � PAGEREF _Toc356953996 �3939��

7.4 Call Reference Value	� GOTOBUTTON _Toc356953997  � PAGEREF _Toc356953997 �4040��

7.5 Conference ID and Conference Goal	� GOTOBUTTON _Toc356953998  � PAGEREF _Toc356953998 �4141��

8.  Call Signalling Procedures	� GOTOBUTTON _Toc356953999  � PAGEREF _Toc356953999 �4242��

8.1 Phase A  - Call set-up	� GOTOBUTTON _Toc356954000  � PAGEREF _Toc356954000 �4242��

8.1.1 Basic Call Setup - Neither Endpoint Registered	� GOTOBUTTON _Toc356954001  � PAGEREF _Toc356954001 �4242��

8.1.2 Both Endpoints Registered to the Same Gatekeeper	� GOTOBUTTON _Toc356954002  � PAGEREF _Toc356954002 �4343��

8.1.3 Only Calling Endpoint has Gatekeeper	� GOTOBUTTON _Toc356954003  � PAGEREF _Toc356954003 �4545��

8.1.4 Only Called Endpoint has Gatekeeper	� GOTOBUTTON _Toc356954004  � PAGEREF _Toc356954004 �4646��

8.1.5 Both Endpoints Registered to Different Gatekeepers	� GOTOBUTTON _Toc356954005  � PAGEREF _Toc356954005 �4848��

8.1.6 Call Set-up via Gateways	� GOTOBUTTON _Toc356954006  � PAGEREF _Toc356954006 �5555��

8.1.6.1 Gateway Inbound Call Set-up	� GOTOBUTTON _Toc356954007  � PAGEREF _Toc356954007 �5555��

8.1.6.2 Gateway Outbound Call Set-up	� GOTOBUTTON _Toc356954008  � PAGEREF _Toc356954008 �5555��

8.1.7 Call Setup with an MCU	� GOTOBUTTON _Toc356954009  � PAGEREF _Toc356954009 �5656��

8.1.8 Call Forwarding	� GOTOBUTTON _Toc356954010  � PAGEREF _Toc356954010 �5656��

8.1.9 Broadcast Call Setup	� GOTOBUTTON _Toc356954011  � PAGEREF _Toc356954011 �5656��

8.2 Phase B - Initial communication and capability exchange	� GOTOBUTTON _Toc356954012  � PAGEREF _Toc356954012 �5656��

8.3 Phase C - Establishment of audiovisual communication	� GOTOBUTTON _Toc356954013  � PAGEREF _Toc356954013 �5757��

8.3.1 Mode changes	� GOTOBUTTON _Toc356954014  � PAGEREF _Toc356954014 �5757��

8.3.2 Exchange of video by mutual agreement	� GOTOBUTTON _Toc356954015  � PAGEREF _Toc356954015 �5757��

8.3.3 Media Stream Address Distribution	� GOTOBUTTON _Toc356954016  � PAGEREF _Toc356954016 �5858��

8.4 Phase D: Call Services	� GOTOBUTTON _Toc356954017  � PAGEREF _Toc356954017 �5858��

8.4.1 Bandwidth Changes	� GOTOBUTTON _Toc356954018  � PAGEREF _Toc356954018 �5858��

8.4.2 Status	� GOTOBUTTON _Toc356954019  � PAGEREF _Toc356954019 �6060��

8.4.3 Ad Hoc Conference Expansion	� GOTOBUTTON _Toc356954020  � PAGEREF _Toc356954020 �6060��

8.4.3.1 Direct Endpoint Call Signalling -  Conference Create	� GOTOBUTTON _Toc356954021  � PAGEREF _Toc356954021 �6161��

8.4.3.2 Direct Endpoint Call Signalling -  Conference Invite	� GOTOBUTTON _Toc356954022  � PAGEREF _Toc356954022 �6262��

8.4.3.3Direct Endpoint Call Signalling -  Conference Join	� GOTOBUTTON _Toc356954023  � PAGEREF _Toc356954023 �6464��

8.4.3.4 Gatekeeper Routed Call Signalling - Conference Create	� GOTOBUTTON _Toc356954024  � PAGEREF _Toc356954024 �6666��

8.4.3.5 Gatekeeper Routed Call Signaling - Conference Invite	� GOTOBUTTON _Toc356954025  � PAGEREF _Toc356954025 �6666��

8.4.3.6 Gatekeeper Routed Call Model - Conference Join	� GOTOBUTTON _Toc356954026  � PAGEREF _Toc356954026 �6767��

8.4.4 Supplementary Services	� GOTOBUTTON _Toc356954027  � PAGEREF _Toc356954027 �7575��

8.5 Phase E: Call termination	� GOTOBUTTON _Toc356954061  � PAGEREF _Toc356954061 �7575��

8.5.1 Call Clearing without a Gatekeeper	� GOTOBUTTON _Toc356954062  � PAGEREF _Toc356954062 �7676��

8.5.2 Call Clearing with a Gatekeeper	� GOTOBUTTON _Toc356954063  � PAGEREF _Toc356954063 �7676��

8.5.3 Call Clearing by Gatekeeper	� GOTOBUTTON _Toc356954064  � PAGEREF _Toc356954064 �7777��

8.6 Protocol Failure Handling	� GOTOBUTTON _Toc356954065  � PAGEREF _Toc356954065 �7878��

9 Interoperation with other terminal types	� GOTOBUTTON _Toc356954070  � PAGEREF _Toc356954070 �7979��

9.1 Speech only terminals	� GOTOBUTTON _Toc356954071  � PAGEREF _Toc356954071 �7979��

9.2 Visual telephone terminals over the ISDN (H.320)	� GOTOBUTTON _Toc356954072  � PAGEREF _Toc356954072 �7979��

9.3 Visual telephone terminals over GSTN (H.324)	� GOTOBUTTON _Toc356954073  � PAGEREF _Toc356954073 �8080��

9.4 Visual telephone terminals over Mobile Radio (H.324/M)	� GOTOBUTTON _Toc356954074  � PAGEREF _Toc356954074 �8080��

9.5 Visual telephone terminals over ATM (H.321)	� GOTOBUTTON _Toc356954075  � PAGEREF _Toc356954075 �8080��

9.6 Visual telephone terminals over Guaranteed Quality of Service LANs (H.322)	� GOTOBUTTON _Toc356954076  � PAGEREF _Toc356954076 �8080��

9.7 Simultaneous Voice and Data Terminals over GSTN (V.70)	� GOTOBUTTON _Toc356954077  � PAGEREF _Toc356954077 �8181��

9.8 T.120 Terminals on the LAN	� GOTOBUTTON _Toc356954078  � PAGEREF _Toc356954078 �8181��

10 Optional enhancements	� GOTOBUTTON _Toc356954079  � PAGEREF _Toc356954079 �8181��

10.1 Encryption	� GOTOBUTTON _Toc356954080  � PAGEREF _Toc356954080 �8181��

11 Maintenance	� GOTOBUTTON _Toc356954081  � PAGEREF _Toc356954081 �8383��

11.1 Loopbacks for maintenance purposes	� GOTOBUTTON _Toc356954082  � PAGEREF _Toc356954082 �8383��

11.2  Monitoring Methods	� GOTOBUTTON _Toc356954083  � PAGEREF _Toc356954083 �8484��

��1 Scope

This Recommendation, H.323, covers the technical requirements for narrow-band visual telephone services defined in H.200/AV.120-Series Recommendations, in those situations where the transmission path includes one or more Local Area Networks (LAN), which may not provide a guaranteed Quality of Service (QoS) equivalent to that of N-ISDN.  Examples of this type of LAN are:

	-Ethernet (IEEE 802.3)

	-Fast Ethernet (IEEE 802.10)

	-FDDI (non-guaranteed quality of service mode)

	-Token Ring (IEEE 802.5)

Recommendation H.322 covers the case of visual telephone services in those situations where the transmission path includes one or more Local Area Networks (LAN), which are configured and managed to provide a guaranteed Quality of Service (QoS) equivalent to that of N-ISDN such that no additional protection or recovery mechanisms beyond those mandated by Rec.  H.320 need be provided in the terminals.  Pertinent parameters are the data error and loss properties and variation of transit delay.  An example  of a suitable LAN is: Integrated Services (IS) LAN: IEEE 802.9A Isochronous services with Carrier Sense Multiple Access with Collision Detection (CSMA/CD) Media access control (MAC) service.

H.323 terminals may be used in multipoint configurations, and may interwork with H.310 terminals on B-ISDN, H.320 terminals on N-ISDN, H.321 terminals on B-ISDN, H.322 terminals on Guaranteed Quality of Service LANs, H.324 terminals on GSTN and wireless networks, and V.70 terminals on GSTN.  See Figure 1/H.323.

The scope of H.323 does not include the LAN itself, or the transport layer which may be used to connect various LANs.  Only elements needed for interaction with the Switched Circuit Network (SCN) are within the scope of H.323.  The combination of the H.323 Gateway, the H.323 terminal, and the out-of-scope LAN appears on the SCN as an H.320, H.310, H.324, or V.70 terminal.

This Recommendation describes the components of an H.323 system.  This includes Terminals, Gateways, Gatekeepers, Multipoint Controllers, Multipoint Processors, and Multipoint Control Units.  Control messages and procedures within this Recommendation define how these components communicate.  Detailed descriptions of these components are contained in Section 6.

The components described in this Recommendation consist of H.323 endpoints and H.323 entities.  The endpoints can call and are callable according to the call setup procedures of Section 8.  The entities are not callable, however, they can be addressed in order to perform their specific functions.  For example, a terminal cannot place a call to a Gatekeeper, however, the Gatekeeper is addressed as part of the call establishment procedures.



�embed MSDraw \* mergeformat ���

Figure 1/H.323 Interoperability of H.323 Terminals

2 Normative references

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation.  At the time of publication, the editions indicated were valid.  All Recommendations and other references are subject to revision;  all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.  A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation H.225.0 (199X): " Media Stream Packetization and Synchronization for Visual Telephone Systems on Non-Guaranteed Quality of Service LANs ".

[2]	ITU-T Recommendation H.245 (1995): "Control of communications between Visual Telephone Systems and Terminal Equipment".

[3]	ITU-T Recommendation G.711 (1988): "Pulse Code Modulation (PCM) of Voice Frequencies".

[4]	ITU-T Recommendation G.722 (1988): "7 kHz Audio-coding within 64 kbit/s".

[5]	ITU-T Recommendation G.723.1 (1995): "Dual Rate Speech codec for multimedia telecommunications transmitting at 6.4 and 5.3 kbit/s".

[6]	ITU-T Recommendation G.728 (1992): "Speech Coding at 16 kbit/s".

[7]	ITU-T Recommendation G.729 (1995):  "Speech codec for multimedia telecommunications transmitting at 8/13 kbit/s".

[8]	ITU-T  Recommendation H.261 (1993):  "Video CODEC for audiovisual services at p X 64 kbit/s"

[9]	ITU-T Recommendation H.263 (1995):  "Video CODEC for narrow telecommunications channels at < 64 kbit/s"

[10]	ITU-T Recommendation T.120  (1994):  "Transmission protocols for multimedia data"

[11]	ITU-T Recommendation H.320 (1995):  "Narrow-band ISDN visual telephone systems and terminal equipment".

[12]	ITU-T Recommendation H.321 (1995):  "Adaptation of H.320 Visual Telephone Terminals to B-ISDN Environments".

[13]	ITU-T Recommendation H.322 (1995):  "Visual Telephone Systems and Terminal Equipment for Local Area Networks which Provide a Guaranteed Quality of Service".

[14]	ITU-T Recommendation H.324 (1995):  Terminal for Low Bitrate Multimedia Communications".

[15]	ITU-T Recommendation H.310 (1996):  "Broadband and audio-visual communicationsNarrow-band ISDN visual telephone systems and terminal equipment".

[16]	ITU-T Recommendation Q.931 (1993): "Digital Subscriber Signalling System No.  1 (DSS 1) - ISDN User-Network Interface Layer 3 Specification for Basic Call Control".

[17]	ITU-T Recommendation Q.932 (1993): "Digital Subscriber Signalling System No.  1 (DSS 1) - Generic Procedures for the Control of ISDN Supplementary Services".

[18]	ITU-T Recommendation Q.950 (1993): "Digital Subscriber Signalling System No.  1 (DSS 1) - Supplementary Services Protocols, Structure, and General Principles".

[19]	Proposed ITU-T Recommendation E.164 (1991) “Numbering Plan for the ISDN Era”.H.24I (199x): "Interopeability between H.245 and H.242 Control Procedures".

[20]	ISO/IEC 10646-1 (1993): "Information Technology - Universal Multiple-Octet Coded Character Set (USC) -- Part I: Architecture and Basic Multilingual Plane".

3 Definitions

For the purposes of this Recommendation the definitions given in Clause 3 of both H.225.0 [1] and H.245 [2] apply along with those in this section.  These definitions apply to the LAN side only.  Other terms may be appropriate when refering to the Switched Circuit Network (SCN) side.



Active MC: An MC that has won the master slave determination procedure and is currently providing the multipoint control function for the conference.

Ad Hoc Multipoint Conference: An Ad Hoc Multipoint conference was a point-to-point conference that had been expanded into a multipoint conference at some time during the call.  This can be done if a one or more of the terminals in the initial point-to-point conference contains an MC, if the call is made using a Gatekeeper that includes MC functionality, or if the initial call is made through an MCU as a multipoint call between only two terminals.

Addressable: An H.323 entity on the LAN having a Transport Address.  Not the same as being callable.  A terminal or MCU is addressable and callable.  A gatekeeper is addressable but not callable.  An MC or MP is neither callable nor addressable but is contained within an endpoint or Gatekeeper that is.

Audio mute: Suppressing of the audio signal of a single or all source(s).  Send muting means that the originator of an audio stream mutes its microphone and/or does not transmit any audio signal at all.  Receive mute means that the receiving terminal ignores a particular incoming audio stream or mutes its loudspeaker.

Broadcast Conference: A Broadcast conference is one in which there is one transmitter of media streams and many receivers.  There is no bi-directional transmission of control or media streams.  Such conferences should be implemented using LAN transport multicast facilities, if available.  This conference type is for further study.

Broadcast Panel Conference: A Broadcast Panel conference is a combination of a Multipoint conference and a Broadcast conference.  In this conference, several terminals are engaged in a multipoint conference while many other terminals are only receiving the media streams.  There is bi-directional transmission between the terminals in the multipoint portion of the conference and no bi-directional transmission between them and the listening terminals.  This conference type is for further study.

Call: Point-to-point multimedia communication between two H.323 endpoints.  The call begins with the call setup procedure and ends with the call termination procedure.  The call consists of the collection of reliable and unreliable channels between the endpoints.  In case of interworking with some SCN endpoints via a gateway, all the channels terminate at the Gateway where they are converted to the appropriate representation for the SCN end system.

Call Signalling Channel: Reliable channel used to convey the call setup and teardown messages (following H.225.0) between two H.323 entities.

Callable: Capable of being called as described in Section 8.  Terminals, MCUs, and Gateways are callable, but Gatekeepers and MCs are not.

Centralized Multipoint Conference: A Centralized Multipoint conference is one in which all participating terminals communicate in a point-to-point fashion with an MCU.  The terminals transmit their control, audio, video, and/or data streams to the MCU.  The MC within the MCU centrally manages the conference.  The MP within the MCU processes the audio, video, and/or data streams, and returns the processed streams to each terminal.

Control and Indication (C&I): End-to-end signalling between terminals, consisting of Control, which causes a state change in the receiver, and Indication which provides for information as to the state or functioning of the system (see also H.245 [2] for additional information and abbreviations).

Data:  Information  stream other than audio, video, and control, carried in the logical data channel (see H.225.0 [1]).

Decentralized Multipoint Conference: A Decentralized Multipoint conference is one in which the participating terminals multicast their audio and video to all other participating terminals without using an MCU.  The terminals are responsible for (a) summing the received audio streams and (b) selecting one or more of the received video streams for display.  No audio or video MP is required in this case.  The terminals communicate on their H.245 Control Channels with an MC which manages the conference.  The data stream is still centrally processed by the MCS MCU which may be within an MP.

Endpoint: An H.323 terminal, Gateway, or MCU.  An endpoint can call and be called.  It generates and/or terminates information streams.

Gatekeeper: The Gatekeeper (GK) is an H.323 entity on the LAN that provides address translation and controls access to the local area network for H.323 terminals, Gateways, and MCUs.  The Gatekeeper may also provide other services to the terminals, Gateways, and MCUs such as bandwidth management and locating Gateways.

Gateway: An H.323 Gateway (GW) is an endpoint on the local area network which provides for real-time, two-way communications between H.323 Terminals on the LAN and other ITU Terminals on a wide area network, or to another H.323 Gateway.  Other ITU Terminals include those complying with Recommendations H.310 (H.320 on B-ISDN), H.320 (ISDN), H.321 (ATM), H.322 (GQOS-LAN), H.324 (GSTN), H.324M (Mobile), and V.70 (DSVD).

H.323 Entity: Any H.323 component, including terminals, Gateways, Gatekeepers, MCs, MPs, and MCUs. Not all H.323 entities are callable, but they are all addressable.

H.245 Control Channel: Reliable Channel used to carry the H.245 control information messages (following H.245) between two H.323 endpoints.

H.245 Logical Channel: Channel used to carry the information streams between two H.323 endpoints.  These channels are established following the H.245 OpenLogicalChannel procedures.  An unreliable channel is used for audio, audio control, video, and video control information streams.  A reliable channel is used for data and H.245 control information streams.  There is no relationship between a logical channel and a physical channel.

H.245 Session: the part of the call that begins with the establishment of an H.245 Control Channel, and ends with the receipt of the H.245 EndSessionCommand or termination due to failure.  Not to be confused with a call, which is delineated by the H.225.0 Setup and Release Complete messages.  

Hybrid Multipoint Conference - Centralized Audio: A Hybrid Multipoint - Centralized Audio conference is one in which terminals multicast their video to other participating terminals, and unicast their audio to the MP for mixing.  The MP returns a mixed audio stream to each terminal.  

Hybrid Multipoint Conference - Centralized Video: A Hybrid Multipoint - Centralized Video conference is one in which terminals multicast their audio to other participating terminals, and unicast their video to the MP for switching or mixing.  The MP returns a video stream to each terminal.  

Information Stream: A flow of information of a specific media type (e.  g.  audio) from a single source to one or more destinations.

LAN address: The network layer address of an H.323 entity as defined by the (inter)network layer protocol in use (e.  g.  an IP address).  This address is mapped onto the layer one address of the respective system by some means defined in the (inter)networking protocol.

Lip Synchronization: Operation to provide the feeling that speaking motion of the displayed person is synchronized with his speech.

Local Area Network (LAN): A shared or switched medium, peer-to-peer communications network that broadcasts information for all stations to receive within a moderate-sized geographic area, such as a single office building or a campus.  The network is generally owned, used, and operated by a single organization.  In the context of H.323 LANs also include internetworks composed of several LANs that are interconnected by bridges or routers.  

Mixed Multipoint Conference: A Mixed Multipoint conference (Figure 2/H.323) has some terminals (D,E, and F) participating in a centralized mode while other terminals (A, B, and C) are participating in a decentralized mode.  A terminal is not aware of the mixed nature of the conference, only of the type of conference it is participating in.  The MCU provides the bridge between the two types of conferences.

�embed Word.Picture.6 ���

Figure 2/H.323 Mixed Multipoint Conference

Multicast: A process of transmitting PDUs from one source to many destinations.  The actual mechanism (ie IP multicast, multi-unicast, etc) for this process may be different for different LAN technologies.

Multipoint Conference: A Multipoint conference is a conference between three or more terminals.  The terminals may be on the LAN or on the SCN.  The multipoint conference shall always be controlled by an MC.  Various multipoint conference types are defined in this section but they all require a single MC per conference.  They may also involve one or more H.231 MCUs on the SCN.  A terminal on the LAN may also participate in an SCN multipoint conference by connecting via a Gateway to an SCN MCU.  This does not require the use of an MC.

Multipoint Control Unit: The Multipoint Control Unit (MCU) is an endpoint on the local area network which provides the capability for three or more terminals and Gateways to participate in a multipoint conference.  May also connect two terminals in a point-to-point conference which may later develop into a multipoint conference.  The MCU generally operates in the fashion of an H.231 MCU, however, an audio processor is not mandatory.  The MCU consists of two parts: a mandatory Multipoint Controller and optional Multipoint Processors.  In the simplest case, an MCU may consist only of an MC with no MPs.

Multipoint Controller: The Multipoint Controller (MC) is an H.323 entity on the local area network which provides for the control of three or more terminals participating in a multipoint conference.  May also connect two terminals in a point-to-point conference which may later develop into a multipoint conference.  The MC provides for capability negotiation with all terminals to achieve common levels of communications.  It also may control conference resources such as who is multicasting video.  The MC does not perform mixing or switching of audio, video and data.

Multipoint Processor: The Multipoint Processor (MP) is an H.323 entity on the local area network which provides for the centralized processing of audio, video, and/or data streams in a multipoint conference.  The MP provides for the mixing, switching, or other processing of media streams under the control of the MC.  The MP may process a single media stream or multiple media streams depending on the type of conference supported.

Mult-Unicast: A process of transfering PDUs where an endpoint sends more than one copy of a media stream, but to different endpoints. This may be necessary in networks which do not support multicast.

Point-to-Point Conference: A Point-to-Point conference is a conference between two terminals.  May be either directly between two H.323 terminals or between an H.323 terminal and an SCN terminal via a gateway.  A call between two terminals (See Call).

RAS Channel: Unreliable channel used to convey the registration, admissions, bandwidth change, and status messages (following H.225.0) between two H.323 entities.

Reliable Channel: A transport connection used for reliable transmission of an information stream from its source to one or more destinations.  

Reliable Transmission: Transmission of messages from a sender to a receiver using connection-mode data transmission.  The transmission service guarantees sequenced, error-free, flow-controlled transmission of messages to the receiver for the duration of the transport connection.

RTP Session: For each participant, the session is defined by a particular pair of destination Transport Addresses (one LAN address plus a TSAP identifier pair for RTP and RTCP).  The destination Transport Address pair may be common for all participants, as in the case of IP multicast, or may be different for each, as in the case of individual unicast network addresses plus a common TSAP identifier pair.  In a multimedia session, the media audio and video are carried in separate RTP sessions with their own RTCP packets.  The multiple RTP sessions are distinguished by different transport addresses.

Switched Circuit Network (SCN): A public or private switched telecommunications network such as the GSTN, N-ISDN, or B-ISDN.

Terminal: An H.323 Terminal is an endpoint on the local area network which provides for real-time, two-way communications with another H.323 terminal, Gateway, or Multipoint Control Unit.  This communication consists of control, indications, audio, moving color video pictures, and/or data between the two terminals.  A terminal may provide speech only, speech and data, speech and video, or speech, data, and video.

Transport Address: The transport layer address of an addressable H.323 entity as defined by the (inter)network protocol suite in use.  The Transport Address of an H.323 entity is composed of the LAN address plus the TSAP identifier of the addressable H.323 entity.

Transport Connection: An association established by a transport layer between two H.323 entities for the transfer of data.  In the context of H.323, a transport connection provides reliable transmission of information.

TSAP Identifier: The piece of information used to multiplex several transport connections of the same type on a single H.323 entity with all transport connections sharing the same LAN address, (e.  g.  the port number in a TCP/UDP/IP environment).  TSAP identifiers may be (pre)assigned statically by some international authority or may be allocated dynamically during setup of a call.  Dynamically assigned TSAP identifiers are of transient nature, i.  e.  their values are only valid for the duration of a single call.

Unicast: A process of transmitting messages from one source to one destination.

Unreliable Channel: A logical communication path used for unreliable transmission of an information stream from its source to one or more destinations.

Unreliable Transmission: Transmission of messages from a sender to one or more receivers by means of connectionless-mode data transmission.  The transmission service is best-effort delivery of the PDU, meaning that  messages transmitted by the sender may be lost, duplicated, or received out of order by (any of) the receiver(s).

Well-known TSAP Identifier:  A TSAP identifier that has been allocated by an (international) authority that is in charge for the assignment of TSAP identifiers for a particular (inter)networking protocol and the related transport protocols -- (e.  g.  the IANA for TCP and UDP port numbers).  This identifier is guaranteed to be unique in the context of the respective protocol.

Zone: A Zone (Figure 3/H.323) is the collection of all terminals (Tx), Gateways (GW), and Multipoint Control Units (MCU) managed by a single Gatekeeper (GK).  A Zone includes at least one terminal, and may or may not include Gateways or MCUs.  A Zone has one and only one Gatekeeper.  A Zone may be independent of LAN topology and may be comprised of multiple LAN segments which are connected using routers (R) or other devices.

�embed Word.Picture.6 ���

Figure 3/H.323 Zone

4 Symbols and abbreviations

For the purposes of this �ref standard \* mergeformat �Recommendation Recommendation �, the following symbols and abbreviations apply.

4CIF	4 times CIF

16CIF	16 times CIF

ACF	Admission Confirmation

ARJ	Admission Reject

ARQ	Admission Request

BAS	Bitrate Allocation Signal

BCF	Bandwidth Change Confirmation

BCH	Bose, Chaudhuri, and Hocquengham

B-ISDN	Broadband - Integrated Services Digital Network

BRJ	Bandwidth Change Reject

BRQ	Bandwidth Change Request

C&I	Control and Indication

CID	Conference Identifier 

CIF	Common Intermediate Format

DCF	Disengage Confirmation

DID	Direct Inward Dialing

DRQ	Disengage Request

ECS	Encryption Control Signal

EIV	Encryption Initialization Vector

FAS	Frame Alignment Signal

FIR	Full Intra Request

GCF	Gatekeeper Confirmation

GK	Gatekeeper

GQOS	Guaranteed Quality of Service

GRJ	Gatekeeper Reject

GRQ	Gatekeeper Request

GSTN	General Switched Telephone Network

GW	Gateway

IANA	Internet Assigned Number Authority

IP	Internet Protocol

IPX	Internetwork Protocol Exchange

IRQ	Information Request

IRR	Information Request Response

ISDN	Integrated Services Digital Network

ITU-T	International Telecommunications Union- Telecommunications Standardization Sector

LAN	Local Area Network

LCF	Location ConfirmationRequest

LCN	Logical Channel Number

LRJ	Location Reject

LRQ	Location Request

MC	Multipoint Controller

MCS	Multipoint Communications System

MCU	Multipoint Control Unit

MP	Multipoint Processor

MSN	Multiple Subscriber Number

N-ISDN	Narrow Band-Integrated Services Digital Network

NACK	Negative Acknowledge

QCIF	Quarter CIF

RCF	Registration Confirmation

RRJ	Registration Reject

RRQ	Registration Request

RTP	Real Time Protocol

RTCP	Real Time Control Protocol

SBE	Single Byte Extension

SCM	Selected Communications Mode

SCN	Switched Circuit Network

SPX	Sequential Protocol Exchange

SQCIF	Sub QCIF

TCP	Transport Control Protocol

TSAP	Transport Layer Service Access Point

UCF	Unregister Confirmation

UDP	User Datagram Protocol

URJ	Unregister Reject

URQ	Unregister Request



5 Conventions

In this document the following conventions are used:

"Shall" indicates a mandatory requirement.

"Should" indicates a suggested but optional course of action.

"May" indicates an optional course of action rather than a recommendation that something take place.

References to Sections, Paragraphs, Annexes, and Appendices refer to those items within this Recommendation unless another document is explicitly listed.  For example, Section 1.4 refers to section 1.4 of this Recommendation; H.245 Section 6.4 refers to section 6.4 in H.245.

Where items exist on both the LAN and on the SCN, references to the SCN item will be explicit.  For example, an MCU is an H.323 MCU on the LAN, an SCN MCU is an MCU on the SCN.  

This Recommendation describes the use of three different message types: H.245, RAS, and Q.931. To distinguish between the different message types the following convension is followed. H.245 message and parameter names consist of multiple concatinated words highlighted in bold typeface (maximumDelayJitter).  RAS message names are represented by three letter abbreviations (ARQ). Q.931 message names consist of one or two words with the first letters capitalized (Call Proceeding).

6 System description

This Recommendation describes the elements of the H.323 components.  These are Terminals, Gateways, Gatekeepers, MCs, and MCUs.  These components communicate through the transmission of Information Streams.  The characteristics of these components are described in this section.

6.1 Information Streams

Visual telephone components communicate through the transmission of Information Streams.  These Information Streams are classified into video, audio, data, communications control, and call control as follows:

Audio signals contain digitized and coded speech.  In order to reduce the average bitrate of audio signals, voice activation may be provided.  The audio signal is accompanied by an audio control signal.

Video signals contain digitized and coded motion video.  Video is transmitted at a rate no greater than that selected as a result of the capability exchange.  The video signal is accompanied by a video control signal.

Data signals include still pictures, facsimile, documents, computer files, and other data streams.

Communications control signals pass control data between remote like functional elements and are used for capability exchange, opening and closing logical channels, mode control and other functions that are part of communications control.

Call control signals are used for call establishment, disconnect, and other call control functions.

The information streams described above are formatted and sent to the network interface as described in H.225.0.

6.2 Terminal Characteristics

An example of an H.323 terminal is shown in Figure 4/H.323.  The diagram shows the user equipment interfaces, video codec, audio codec, telematic equipment, H.225.0 layer, system control functions, and the interface to the LAN.  All H.323 terminals shall have a System Control Unit, H.225.0 layer, Network Interface, and an Audio Codec Unit.  The Video Codec Unit and User Data Applications are optional.

�embed MSDraw \* mergeformat ����

Figure 4/H.323 H.323 Terminal Equipment

6.2.1 Terminal elements outside the scope of H.323

The following elements are not within the scope of H.323, and are therefore not defined within this Recommendation:

·	Attached audio devices providing, voice activation sensing, microphone and loudspeaker, telephone instrument or equivalent, multiple microphones mixers, and acoustic echo cancellation.

·	Attached video equipment providing cameras and monitors, and their control and selection, video processing to improve compression or provide split screen functions.

·	Data applications and associated user interfaces which use T.120 or other data services over the data channel.  

·	Attached Network Interface, which provides the interface to the LAN, supporting appropriate signalling, and voltage levels, in accordance with national and international standards.

·	Human user system control, user interface and operation.

6.2.2 Terminal elements within the scope of H.323

The following elements are within the scope of H.323, and are therefore subject to standardization and are defined within this Recommendation:

·	The Video Codec (H.261, etc.) encodes the video from the video source (i.e.  camera) for transmission and decodes the received video code which is output to a video display.

·	The Audio Codec (G.711, etc.) encodes the audio signal from the microphone for transmission, and decodes the received audio code which is output to the loudspeaker.

·	The Data Channel supports telematic applications such as electronic whiteboards, still image transfer, file exchange, database access, audiographics conferencing, etc.  The standardized data application for real-time audiographics conferencing is T.120.  Other applications and protocols may also be used via H.245 negotiation as specified in Section 6.2.7.

·	The System Control Unit (H.245, H.225.0) provides signalling for proper operation of the H.323 terminal.  It provides for call control, capability exchange, signalling of commands and indications, and messages to open and fully describe the content of logical channels.

·	H.225.0 Layer (H.225.0) formats the transmitted video, audio, data and control streams into messages for output to the network interface, and retrieves the received video, audio, data, and control streams from messages which have been input from the network interface.  In addition, it performs logical framing, sequence numbering, error detection, and error correction as appropriate to each media type.

6.2.3 LAN Interface

The LAN interface is implementation specific and is outside the scope of this recommendation.  However, the LAN interface shall provide the services described in Recommendation H.225.0.  This includes the following: Reliable (e.g.  TCP, SPX)  end-to-end service is mandatory for the H.245 Control Channel, the Data Channels, and the Call Signalling Channel.  Unreliable (e.g.  UDP, IPX) end-to end service is mandatory for the Audio Channels, the Video Channels, and the RAS Channel.  These services may be duplex or simplex, unicast or multicast depending on the application, the capabilities of the terminals, and the configuration of the LAN.

6.2.4 Video Codec

The video codec is optional.  All H.323 terminals providing video communications shall be capable of encoding and decoding video according to H.261 QCIF.  Optionally, a terminal may also be capable of encoding and decoding video according H.261 CIF or H.263 SQCIF, QCIF, CIF, 4CIF, and 16CIF.  If a terminal supports H.263 with CIF or higher resolution, it shall also support H.261 CIF.  All terminals which support H.263 shall support H.263 QCIF.  The H.261 and H.263 codecs, on the LAN, shall be used without BCH error correction and without error correction framing.

Other video codecs, and other picture formats, may also be used via H.245 negotiation.  More than one video channel may be transmitted and/or received, as negotiated via the H.245 Control Channel.  The H.323 terminal may optionally send more than one video channel at the same time, for example, to convey the speaker and a second video source.  The H.323 terminal may optionally receive more than one video channel at the same time, for example, to display multiple participants in a distributed multipoint conference.

CIF and QCIF are defined in H.261.  SQCIF, 4CIF and 16CIF are defined in H.263.  For the H.261 algorithm, SQCIF is any active picture size less than QCIF, filled out by a black border, and coded in the QCIF format.  For all these formats, the pixel aspect ratio is the same as that of the CIF format.  NOTE - The resulting picture aspect ratio for H.263 SQCIF is different from the other formats.

The video bitrate, picture format and algorithm options that can be accepted by the decoder are defined during the capability exchange using H.245.  The encoder is free to transmit anything that is within the decoder capability set.  The decoder should have the possibility to generate requests via H.245 for a certain mode, but the encoder is allowed to simply ignore these requests if they are not mandatory modes.  Decoders which indicate capability for a particular algorithm option shall also be capable of accepting video bitstreams which do not make use of that option.

H.323 terminals shall be capable of operating in asymmetric video bitrates, frame rates, and, if more than one picture resolution is supported, picture resolutions.  For example, this will allow a CIF capable terminal to transmit QCIF while receiving CIF pictures.

When each video logical channel is opened, the maximum operating mode to be used on that channel is signaled to the receiver in the H.245 OpenLogicalChannel message.  The header within the video logical channel indicates which mode is actually used for each picture, within the stated maximum.  The maximum mode signaled includes maximum picture format, algorithm options, maximum codec bit rate, etc. The header within the video logical channel indicates which mode is actually used for each picture, within the stated maximum.  For example, a video logical channel opened for CIF format may transmit CIF, QCIF, or SQCIF pictures, but not 4CIF or 16CIF.  A video logical channel opened with only the unrestrictedVector and arithmeticCoding options may use neither, either, or both options, but shall not use options which were not signaled.

The video stream is formatted as described in H.225.0.  Each logical channel opened for video shall be accompanied by a logical channel opened for video control.  The video control channel is described in H.225.0.

6.2.4.1 Terminal Based Continuous Presence

H.323 terminals may receive more than one video channel, particularly for multipoint conferencing.  In these cases, the H.323 terminal may need to perform a video mixing or switching function in order to present the video signal to the user.  This function may include presenting the video from more than one terminal to the user.  The H.323 terminal shall use H.245 simultaneous capabilities to indicate how many simultaneous video streams it is capable of decoding.  The simultaneous capability of one terminal shouldshall not limit the number of video streams which are multicast in a conference (this choice is made by the MC). 

6.2.5 Audio Codec

All H.323 terminals shall have an audio codec.  All H.323 terminals shall be capable of encoding and decoding speech according to Recommendation G.711.  All terminals shall be capable of transmitting and receiving A-law and �symbol 109 \f "Symbol" \s 10��-law.  A terminal may optionally be capable of encoding and decoding speech using Recommendations G.722, G.728, G.729, MPEG1 audio, and G.723.  The audio algorithm used by the encoder shall be derived during the capability exchange using H.245.  The H.323 terminal should be capable of asymmetric operation for all audio capabilities it has declared within the same capability set, e.g.  it should be able to send G.711 and receive G.728 if it is capable of both.

The audio stream is formatted as described in H.225.0.  Each logical channel opened for audio shall be accompanied by a logical channel opened for audio control.  The audio control channel is described in H.225.0.  

The H.323 terminal may optionally send more than one audio channel at the same time, for example, to allow two languages to be conveyed.

Audio packets shouldshall be delivered to the transport layer periodically at an interval determined by the audio codec Recommendation in use (audio frame interval).  The delivery of each audio packet shall occur no later than 5 milliseconds after a whole multiple of the audio frame interval, measured from delivery of the first audio frame (audio delay jitter).  Audio coders capable of further limiting their audio delay jitter may so signal using the H.245 maximumDelayJitter parameter of the h2250Capability structure contained within a terminal capability set message, so that receivers may optionally reduce their jitter delay buffers.  This is not the same as the RTCP interarrival jitter field.  

Note: The testing point for the maximum delay jitter is at the input to network transport layer.  Network stack, network, driver, and interface card jitter is not included.

6.2.5.1 Audio Mixing

H.323 terminals may receive more than one audio channel, particularly for multipoint conferencing.  In these cases, the H.323 terminal may need to perform an audio mixing function in order to present a composite audio signal to the user.  The H.323 terminal shall use H.245 simultaneous capabilities to indicate how many simultaneous audio streams it is capable of decoding.  The simultaneous capability of one terminal shouldshall not limit the number of audio streams which are multicast in a conference.

6.2.5.2 Maximum Audio-Video Transmit Skew

To allow H.323 terminals to appropriately set their audio receive buffer size, H.323 terminals shall transmit the h2250MaximumSkewIndication message at the start of each call to indicate the maximum skew between the audio and video signals as delivered to the network transport. h2250MaximumSkewIndication shall be sent for each pair of associated audio and video logical channels. This is not required for audio only or hybrid conferences.  Lip synchronization, if desired, shall be achieved via use of timestamps.

6.2.6 Receive Path Delay

Receive path delay includes delay added to athe mediaaudio stream in order to maintain synchronization and to account for network packet arrival jitter.  MediaAudio streams may optionally be delayed in the receiver processing path to maintain synchronization with other mediathe video streams.  Further, athe mediaaudio stream may optionally be delayed to allow for network delays which cause packet arrival jitter.  An H.323 terminal shall not add delay for this purpose in its transmitting mediaaudio path.

Intermediate processing points such as MCUs or Gateways may alter the video and audio time tag information, and shall transmit appropriately modified audio and video time tags and sequence numbers, reflecting their transmitted signals.  Receiving endpoints may add appropriate delay in the audio path to achieve lip synchronization.

6.2.7 Data Channel 

One or more data channels are optional.  The data channel may be uni-directional or bi-directional depending on the requirements of the data application.

T.120 is the default basis of data interoperability between an H.323 terminal and other H.323, H.324, H.320, or H.310 terminals.  Where any optional data application is implemented using one or more of the ITU-T Recommendations which can be negotiated via H.245, the equivalent T.120 application, if any, shall be one of those provided.  A terminal that provides far-end camera control using H.281 and H.224 is not required to also support a T.120 far end camera control protocol.

Note that non-standard data applications (dataApplicationCapability = non-standard application, dataProtocolCapability = non-standard) and Transparent User Data (dataApplicationCapability = userData application, dataProtocolCapability = transparent) may be used whether the equivalent T.120 application is provided or not.

T.120 capability shall be signaled using dataApplicationCapability = t120 application, dataProtocolCapability = t123LANStack.  H.224 capability shall be signaled using dataApplicationCapability = h224 application, dataProtocolCapability = hdlcFrameTunneling.

The Data channel is formatted as described in H.225.0.

6.2.7.1 T.120 Data Channels

There are two ways of establishing a T.120 connection within the context of an H.323 call.  The first is the establishment of the T.120 connection during an H.323 call as an inherent part of the call, using the procedures of H.245 for opening logical channels.  The second is the establishment of the T.120 connection prior to placing the H.323 call.

In the case where the H.323 call is established first, the normal call setup procedures of Section 8.1 are followed.  The capability exchange takes place, and a bi-directional logical channel shall be opened for the T.120 connection according to the normal H.245 procedures indicating that a new connection shall be created as described below.(logicalChannelSetupType = create).  Note that up to four logical channels may be required for a T.120 connection.  The channel can now be used for T.120 data communications.

The opening of a bidirectional logical channel for T.120 may be initiated by the master using openLogicalChannel or by the slave requesting the master to open a bi-directional logical channel following the procedures of H.245.

To actually open the logical channel, the master shall send an openLogicalChannel command indicating that a T.120 data channel is to be opened in the forwardLogicalChannelParameters as well as in the reverseLogicalChannelParameters.  If the opening of the bi-directional channel was induced by an explicit request from the slave, the master shall include a transport address to be used for setup of the T.120 connection.  If not, the transport address shall be absent.  If the slave accepts this logical channel it shall confirm the opening of the logical channel using openLogicalChannelAck.  In the openLogicalChannelAck, the slave shall include a transport address to be used for setup of the T.120 connection if it did not receive a transport address from the master.  Otherwise, the transport address shall be absent.The transport address for the T.120 conncetion shall in both cases be carried in the separateStack parameter .

The end point transmitting the transport address shall be prepared to accept a T.120 connection on this transport address.  The entity receiving the transport address shall initiate a T.120 connection setup using the previously received transport address.

In both the openLogicalChannel and the openLogicalChannelAck commands, the associateConference parameter shall be set to false.

T.120 shall  follow the procedures for the LAN protocol stack or the separate protocol stack (if selected) as described in T.123 except that the transport addresses as described above shall be employed for connection setup.

In the case where the T.120 connection is established first, the H.323 call is placed following the normal call setup procedures of Section 8.1.  The capability exchange takes place, and it is desired to associate the T.120 connection with the H.323 call.  The procedures of H.245 are used to open a logical channel fordo the T.120 data as described below.  The openLogicalChannel message and OpenLogicalChannelAck message shall contain the Transport Addresses of the existing T.120 connection, and shall indicate that an association shall be made with an existing connection (logicalChannelSetupType = associate).  Note that up to four logical channels may be required for a T.120 connection.  The connections are then locally associated with the logical channels and treated as if they were established within the H.323 call.

The opening of a bidirectional logical channel for T.120 may be initiated by the master using openLogicalChannel or by the slave requesting the master to open a bi-directional logical channel following the procedures of H.245.  The initiator of the setup should include identification information for the already existing T.120 connection: the master in the openLogicalChannel, the slave in the openLogicalChannelAck part of the bi-directional open request to indicate to the peer which T.120 connection (if there are several) is to be associated.

To actually open the logical channel, the master shall send an openLogicalChannel command for a bi-directional logical channel indicating that a T.120 data channel is to be opened.  If the slave accepts this logical channel it shall return an openLogicalChannelAck message to the initiator.  In both messages the associateConference parameter shall be set to true.

As identification information the local (dynamically instanciated) transport address of the initial transport connection of the T.120 connection should be provided in the separateStack parameter.  In addition, the externalReference parameter may be used to provide further information on which T.120 connection is to be associated.

If any of this identification information is not available to the initiator, it may omit the respective value(s).

Note:	If the transport address is not specified and the two endpoints share more than one T.120 connection it may be ambiguous for the recipient which T.120 connection is referred to.  Implications of this ambiguity and steps to avoid it are for further study.

T.120 applications shall use the LAN protocol stack as described in T.123.

6.2.8 H.245 Control Function 

The H.245 Control Function uses the H.245 Control Channel to carry end-to-end control messages governing operation of the H.323 entity, including capabilities exchange, opening and closing of logical channels, mode preference requests, flow control messages, and general commands and indications.  

H.245 signalling is established between two endpoints, an endpoint and an MC, or an endpoint and a Gatekeeper.  The endpoint shall establish exactly one H.245 Control Channel in each direction, for each call that the endpoint is participating in.  This channel shall use the messages and procedures of Recommendation H.245.  Note that a terminal, MCU, Gateway, or Gatekeeper may support many calls, and thus many H.245 Control Channels.  The H.245 Control Channel shall be carried on logical channel 0.  Logical channel 0 shall be considered to be permanently open from the establishment of the H.245 Control Channel until the termination of this channel.  The normal procedures for opening and closing logical channels shall not apply to the H.245 Control Channel.

H.245 specifies a number of independent protocol entities which support endpoint to endpoint signalling.  A protocol entity is specified by its syntax (messages), semantics, and a set of procedures which specify the exchange of messages and the interaction with the user.  H.323 endpoints shall support the syntax, semantics, and procedures of the following protocol entities:

·	Master/slave determination

·	Capability Exchange

·	Logical Channel Signalling

·	Bi-directional Logical Channel Signalling

·	Close Logical Channel Signalling

·	Mode Request

·	Round Trip Delay Determination

·	Maintenance Loop Signalling



General commands and indications shall be chosen from the message set contained in H.245.  In addition, other command and indication signals may be sent which have been specifically defined to be transferred in-band within video, audio or data streams (see the appropriate Recommendation to determine if such signals have been defined).

H.245 messages fall into four categories: Request, Response, Command, and Indication.  Request and Response messages are used by the protocol entities.  Request messages require a specific action by the receiver, including an immediate response.  Response messages respond to a corresponding request.  Command messages require a specific action, but do not require a response.  Indication messages are informative only, and do not require any action or response.  H.323 terminals shall respond to all H.245 commands and requests as specified in Annex A, and shall transmit indications reflecting the state of the terminal.

H.323 terminals shall be capable of parsing all H.245 MmultimediaSystemControlPDUMessage messages, and shall send and receive all messages needed to implement required functions and those optional functions which are supported by the terminal.  Annex A contains a table showing which H.245 messages are mandatory, optional, or forbidden for H.323 terminals.  H.323 terminals shall send the functionNotSupported message in response to any unrecognized request, response, or command messages that it receives.

An H.245 indication, userInputIndication, is available for transport of user input alphanumeric characters from a keypad or keyboard, equivalent to the DTMF signals used in analog telephony, or SBE number messages in H.230.  This may be used to manually operate remote equipment such as voice mail or video mail systems, menu-driven information services, etc.  H.323 terminals shall support the transmission of user input characters 0-9, ‘*’, and ‘#’.  Transmission of other characters is optional.

NOTE - If the encryption procedures of this Recommendation are in use, the H.245 Control Channel will not be encrypted.  Users are therefore cautioned regarding the carriage of user data in the H.245 Control Channel, the use of non-standard messages, and the confidentiality risk from traffic analysis of the H.245 Control Channel.  

ThreeTwo H.245 request messages conflict with RTCP control packets.  The H.245 videoFastUpdatePicture, and videoFastUpdateGOB, and videoFastUpdateMB requests shouldshall be used instead of the RTCP control packets Full Intra Request (FIR) and Negative Acknowledgement (NACK). The ability to accept FIR and NACK is signalled during the H.245 capability exchange.

6.2.8.1	Capabilities exchange

Capabilities exchange shall follow the procedures of H.245, which provides for separate receive and transmit capabilities, as well as a method by which the terminal may describe its ability to operate in various combinations of modes simultaneously.

Receive capabilities describe the terminal’s ability to receive and process incoming information streams.  Transmitters shall limit the content of their transmitted information to that which the receiver has indicated it is capable of receiving.  The absence of a receive capability indicates that the terminal cannot receive (is a transmitter only).

Transmit capabilities describe the terminal’s ability to transmit information streams.  Transmit capabilities serve to offer receivers a choice of possible modes of operation, so that the receiver may request the mode which it prefers to receive.  The absence of a transmit capability indicates that the terminal is not offering a choice of preferred modes to the receiver (but it may still transmit anything within the capability of the receiver).  

The transmitting terminal assigns each individual mode the terminal is capable of operating in a number in a capabilityTable.  For example, G.723 audio, G.728 audio, and CIF H.263 video would each be assigned separate numbers.  

These capability numbers are grouped into alternativeCapabilitySet structures.  Each alternativeCapabilitySet indicates that the terminal is capable of operating in exactly one mode listed in the set.  For example, an alternativeCapabilitySet listing {G.711, G.723, G.728} means that the terminal can operate in any one of those audio modes, but not more than one.

These alternativeCapabilitySet structures are grouped into simultaneousCapabilities structures.  Each simultaneousCapabilities structure indicates a set of modes the terminal is capable of using simultaneously.  For example, a simultaneousCapabilities structure containing the two alternativeCapabilitySet structures {H.261, H.263} and {G.711, G.723, G.728} means that the terminal can operate either of the video codecs simultaneously with any one of the audio codecs.  The simultaneousCapabilities set { {H.261}, {H.261, H.263}, {G.711, G.723, G.728} } means the terminal can operate two video channels and one audio channel simultaneously: One video channel per H.261, another video channel per either H.261 or H.263, and one audio channel per either G.711, G.723, or G.728.

NOTE - The actual capabilities stored in the capabilityTable are often more complex than presented here.  For example, each H.263 capability indicates details including ability to support various picture formats at given minimum picture intervals, and ability to use optional coding modes.  For a complete description, see Recommendation H.245.

The terminal’s total capabilities are described by a set of capabilityDescriptor structures, each of which is a single simultaneousCapabilities structure and a capabilityDescriptorNumber.  By sending more than one capabilityDescriptor, the terminal may signal dependencies between operating modes by describing different sets of modes which it can simultaneously use.  For example, a terminal issuing two capabilityDescriptor structures, one { {H.261, H.263}, {G.711, G.723, G.728} } as in the previous example, and the other { {H.262}, {G.711} }, means the terminal can also operate the H.262 video codec, but only with the low-complexity G.711 audio codec.

Terminals may dynamically add capabilities during a communication session by issuing additional capabilityDescriptor structures, or remove capabilities by sending revised capabilityDescriptor structures.  All H.323 terminals shall transmit at least one capabilityDescriptor structure.

Non-standard capabilities and control messages may be issued using the nonStandardParameter structure defined in H.245.  Note that while the meaning of non-standard messages is defined by individual organizations, equipment built by any manufacturer may signal any non-standard message, if the meaning is known.

Terminals may reissue capability sets at any time, according to the procedures of H.245.

6.2.8.2	Logical channel signalling

Each logical channel carries information from a transmitter to one or morea receivers, and is identified by a logical channel number which is unique for each direction of transmission.

Logical channels are opened and closed using the openLogicalChannel and closeLogicalChannel messages and procedures of H.245.  When a logical channel is opened, the openLogicalChannel message fully describes the content of the logical channel, including media type, algorithm in use, any options, and all other information needed for the receiver to interpret the content of the logical channel.  Logical channels may be closed when no longer needed.  Open logical channels may be inactive, if the information source has nothing to send.

Most logical channels in H.323 are unidirectional, so asymmetrical operation, in which the number and type of information streams is different in each direction of transmission, is allowed.  However, if a receiver is capable only of certain symmetrical modes of operation, it may send a receive capability set that reflects its limitations, except where noted elsewhere in H.323.  Terminals may also be capable of using a particular mode in only one direction of transmission.  Certain media types, including data protocols such as T.120, inherently require a bi-directional channel for their operation.  In such cases a pair of unidirectional logical channels, one in each direction, may be opened and associated together to form a bi-directional channel using the bi-directional channel opening procedures of H.245.  Such pairs of associated channels need not share the same logical channel number, since logical channel numbers are independent in each direction of transmission.

The openLogicalChannelAck shall be used to return the Transport Address that the receiver has allocated for this logical channel.  The transmitter shall transmit the information associated with the logical channel to the assigned Transport Address.

Logical channels shall be opened using the following procedure:

The initiating terminal shall send an OpenLogicalChannel message as described in H.245.  If the logical channel is to carry a media type using RTP (audio or video), the OpenLogicalChannel message shall include the mediaControlChannel parameter containing the transport address for the reverse RTCP channel.

The responding terminal shall respond with an OpenLogicalChannelAck message as described in H.245.  If the logical channel is to carry a media type using RTP, the OpenLogicalChannelAck message shall include both the media transportChannel parameter containing the RTP transport address for the media channel and the mediaControlChannel parameter containing the transport address for the forward RTCP channel.

Media types (such as T.120 data) which do not use RTP/RTCP shall omit the mediaControlChannel parameters.

If a corresponding reverse channel is opened for a given existing RTP session (identified by the RTP sessionID), the mediaControlChannel transport addresses exchanged by the OpenLogicalChannel process shall be identical to those used for the forward channel.  Should a collision occur where both ends attempt to establish conflicting RTP sessions at the same time, the master endpoint shall reject the conflicting attempt as described in H.245.  The rejected OpenLogicalChannel attempt may then be retried at a later time.

6.2.8.3	Mode preferences

Receivers may request transmitters to send a particular mode using the H.245 requestMode message, which describes the desired mode.  Transmitters should comply if possible.

An endpoint receiving the multipointModeCommand from the MC shall then comply with all requestMode commands, if they are within its capability set.  Note that in a decentralized conference, as in a centralized conference, all terminal rRequestMode commands are directed to the MC.  The MC may grant the request or not; the basis for this decision is left to the manufacturer.

6.2.8.4 Master Slave Determination

The H.245 Master Slave Determination procedures are used to resolve conflicts between two endpoints which can both be the MC for a conference, or between two endpoints which are attempting to open a bi-directional channel.  In this procedure, two endpoints exchange random numbers in the H.245 masterSlaveDetermination message, to determine the master and slave endpoints.  H.323 endpoints shall be capable of operating in both master and slave modes.  The endpoints shall set terminalType to the value specified in Table 1/H.323 below and set statusDeterminationNumber to a random number in the range 0 to 224-1.  Only one random number shall be chosen by the endpoint for each call, except in the case of identical random numbers, as described in H.245.

TerminalType Value Table�H.323 Entity��Feature Set�Terminal�Gateway�Gatekeeper�MCU��Entity with No MC�50�60�NA�NA��Entity contains an MC but no MP�70�80�120�160��Entity contains MCU with Data MP�NA�90�130�170��Entity contains MCU with Data and audio MP�NA�100�140�180��Entity contains MCU with Data, Audio, and video MP�NA�110�150�190��

Table 1/H.323:  H.323 terminal types for H.245 master-slave determination

The Active MC in a conference shall use a value of 240. Cascaded MCs are for further study.

If a single H.323 entity can take part in multiple calls, then the value used for terminalType in the master-slave determination process shall be based on the features that the H.323 entity has assigned or will assign to the call in which it is being signalled.

An MC that is already acting as an MC shall always remain the active MC.  Therefore, once an MC has been selected as the active MC in a conference, it shall use the Active MC value for all subsequent connections to the conference. 

If no MC is active and the entities are of the same type, then the H.323 entity with the highest feature set (as shown in Table 1/H.323) shall win the master-slave determination.  If no MC is active and the entities are of different typeshave the same feature set, then an MC that is located in an MCU shall have priority over an MC that is located in a Gatekeeper, which shall have priority over an MC that is located in a Gateway, which in turn shall have priority over an MC located in a terminal.

If an H.323 entity can be associated with two or more of the classifications shown in Table 1/H.323, then it should use the highest value for which it qualifies.

6.2.8.5	Timer and counter values

All timers defined in H.245 should have periods of at least as long as the maximum data delivery time allowed by the data link layer carrying the H.245 Control Channel, including any retransmissions.

The H.245 retry counter N100 should be at least 3.

Procedures relating to H.245 protocol error handling are covered in Section 8.6.

6.2.9 RAS Signalling Function

The RAS signalling function uses H.225.0 messages to perform registration, admissions, bandwidth changes, status, and disengage procedures between endpoints and Gatekeepers.  The RAS Signalling Channel is independent from the Call Signalling Channel and the H.245 Control Channel.  H.245 open logical channel procedures are not used to establish the RAS Signalling Channel.  In LAN environments that do not have a Gatekeeper, the RAS Signalling Channel is not used.  In LAN environments which contain a Gatekeeper (a Zone), the RAS Signalling Channel is opened between the endpoint and the Gatekeeper.  The RAS Signalling Channel is opened prior to the establishment of any other channels between H.323 endpoints.  This channel is described in detail in Section 7 of this document.

6.2.10 Call Signalling Function

The call signalling function uses H.225.0 call signalling to establish a connection between two H.323 endpoints.  The Call Signalling Channel is independent from the RAS Channel and the H.245 Control Channel.  H.245 open logical channel procedures are not used to establish the Call Signalling Channel.  The Call Signalling Channel is opened prior to the establishment of the H.245 Channel and any other logical channels between H.323 endpoints.  In systems that do not have a Gatekeeper, the Call Signalling Channel is opened between the two endpoints involved in the call.  In systems which contain a Gatekeeper, the Call Signalling Channel is opened between the end point and the Gatekeeper, or between the endpoints themselves as chosen by the Gatekeeper.  This channel is described in detail in Section 7 of this document.

6.2.11 H.225.0 Layer

Logical channels of video, audio, data or control information are established according to the procedures of Recommendation H.245.  Logical channels are unidirectional, and are independent in each direction of transmission.  Some logical channels, such as for data, may be bi-directional, and are associated through the bi-directional open logical channel procedure of H.245.  Any number of logical channels of each media type may be transmitted, except for the H.245 Control Channel of which there shall be one per call.  In addition to the logical channels, H.323 endpoints use two signalling channels for call control, and Gatekeeper related functions.  The formatting used for these channels shall conform to Recommendation H.225.0.

6.2.11.1	Logical channel numbers

Each logical channel is identified by a logical channel number (LCN), in the range 0 to 65535, which serves only to associate logical channels with the transport connection.  Logical channel numbers are selected arbitrarily by the transmitter, except that logical channel 0 shall be permanently assigned to the H.245 Control Channel.  The actual Transport Address that the transmitter shall transmit to, shall be returned by the receiver in the openLogicalChannelAck message.

6.2.11.2	Logical Channel Bitrate Limits

A logical channel’s bandwidth shall have an upper limit specified by the minimum of the endpoint’s transmit capability (if present) and the receiving endpoint’s receive capability.  Based on this limit, an endpoint shall open a logical channel at a bitrate at or below this upper limit.  A transmitter shall transmit an information stream within the logical channel at any bitrate at or below the open logical channel bitrate. The limit applies to the information streams which are the content of the logical channel(s), not including RTP headers, RTP payload headers and LAN H.225.0 and LAN headers and other overhead.

The transmit bitrate shall be further reduced by an H.245 flowControlCommand message from the receiving endpoint which specifies a lower bitrate.  H.323 endpoints shall obeyrespond to the flowControlCommand message of H.245, which commands a limit to the bitrate of a logical channel or the aggregate bitrate of all logical channels.  H.323 endpoints that want to limit the bitrate of a logical channel, or the aggregate bitrate of all logical channels should send the flowControlCommand message to the transmitting endpoint.

When the terminal has no information to send in a given channel, the terminal shall send no information.  Fill data shall not be sent on the LAN in order to maintain a specific data rate.

6.3 Gateway Characteristics

The Gateway shall provide the appropriate translation between transmission formats (for example H.225.0 to/from H.221) and between communications procedures (for example H.245 to/from H.242; this conversion is described in H.24I).  The Gateway shall also perform call setup and clearing on both the LAN side and the SCN side.  Translation between video, audio, and data formats may also be performed in the Gateway.  In general, the purpose of the Gateway (when not operating as an MCU) is to reflect the characteristics of a LAN endpoint to an SCN endpoint, and the reverse, in a transparent fashion.

An H.323 endpoint may communicate with another H.323 endpoint on the same LAN directly and without involving a Gateway.  The Gateway may be omitted if communications with SCN terminals (terminals not on the LAN) is not required.  It may also be possible for a terminal on one segment of the LAN to call out through one Gateway and back onto the LAN through another Gateway in order to bypass a router or a low bandwidth link.  

The Gateway has the characteristics of an H.323 Terminal or MCU on the LAN, and of the SCN terminal or MCU on the SCN.  The choice of terminal or MCU is left to the manufacturer.  The Gateway provides the necessary conversion between the different terminal types.  Note that the Gateway may initially operate as a terminal, but later using H.245 signalling begin to operate as an MCU for the same call that was initially point-to-point.  Gatekeepers are aware of which terminals are Gateways since this is indicated when the terminal/Gateway registers with the Gatekeeper.

A Gateway which passes T.120 data between the SCN and the LAN may contain a T.120 MCS Provider which connects the T.120 MCS Providers on the LAN to the T.120 MCS Providers on the SCN.  This will allow the Gateway to manage flow control and routing of the T.120 stream between the SCN and the LAN.

Four examples of an H.323 Gateway are shown in Figure 5/H.323.  The diagrams show the H.323 terminal or MCU function, the SCN terminal or MCU function, and the conversion function.  The H.323 terminal function has the characteristics described in Section 6.2.  The H.323 MCU function has the characteristics described in Section 6.5.  The Gateway appears to the other H.323 terminals on the LAN as one or more H.323 terminals, or an H.323 MCU.  It communicates with the other H.323 terminals using the procedures in this Recommendation.

The SCN terminal or MCU function has the characteristics described in the appropriate Recommendation (H.310, H.320, H.321, H.322, H.324, V.70, GSTN or ISDN speech only terminals).  The Gateway appears to the terminals on the SCN as one or more of the same terminal types or MCUs.  It communicates to another terminal on the SCN using the procedures described in the appropriate Recommendation for that terminal.  SCN signalling procedures are beyond the scope of H.323, including such topics as whether the H.323 Gateway appears as a terminal or a network to the SCN.  Note that a Gateway may convert H.323 directly to H.324 or H.310 without going to H.320.

Gateways supporting interworking with speech only terminals on GSTN or ISDN should generate and detect DTMF signals corresponding to H.245 userInputIndications for 0-9, *, and #.

�embed MSDraw \* mergeformat ���

Figure 5/H.323 H.323 Gateway Configurations

The conversion function provides the necessary conversion of transmission format, control, audio, video, and/or data streams between the different terminal recommendations.  At a minimum, the Gateway shall provide a conversion function for the transmission format, call setup signals and procedures, and communications control signals and procedures.  When required, the Gateway shall provide for H.242 to H.245 conversiondescribed in H.24I.  The Gateway performs the appropriate conversion between the H.225.0 Call Signalling and the SCN signalling system (Q.931, Q.2931,  etc.). The conversion between Q.931 messaages on the LAN and Q.931 messages on the SCN is described in Appendix A.

All Q.931 signalling received by the Gateway from an ISDN endpoint and not applicable to the Gateway shouldshall be passed through to the LAN endpoint, and vice versa.  This signalling includes Q.932 and Q.950 series messages.  This will allow H.323 endpoints to implement the Supplementary Services defined in those Recommendations.  The handling of other SCN call signalling systems is for further study.

This recommendation describes the connection of one H.323 terminal on the LAN to one external terminal on the SCN through the Gateway.  The actual number of H.323 terminals that can communicate through the Gateway is not subject to standardization.  Similarly, the number of SCN connections, number of simultaneous independent conferences, audio/video/data conversion functions, and inclusion of multipoint functions is left to the manufacturer.  If the Gateway includes an MCU function on the LAN side, that function shall be an H.323 MCU on the LAN side.  If the Gateway includes an MCU function on the SCN side, it may appear as an H.231/H.243 MCU, or as an MCU for H.310 or H.324 systems (these MCUs are indicated as for further study in the respective Recommendations) on the SCN side.

A Gateway may be connected via the SCN to other Gateways to provide H.320 communication between H.323 terminals which are not on the same LAN.

Equipment which provides transparent interconnection between LANs without using H.320 (such as routers and remote dial in units) are not Gateways as defined within the scope of this Recommendation.  

6.4 Gatekeeper Characteristics

The Gatekeeper, which is optional in an H.323 system, provides call control services to the H.323 endpoints.  More than one Gatekeeper may be present and communicate with each other in an unspecified fashion.  The Gatekeeper is logically separate from the endpoints, however, its physical implementation may coexist with a terminal, MCU, Gateway, MCserver, or other non-H.323 LAN device.

When it is present in a system, the Gatekeeper shall provide the following services:

·	Address Translation - The Gatekeeper shall perform alias address to Transport Address translation.  This shouldshall be done using a translation table which is updated using the Registration messages described in Section 7. Other methods of updating the translation table are also allowed.

·	Admissions Control - The Gatekeeper shall authorize LAN access using ARQ/ACF/ARJ H.225.0 messages.  This may be based on call authorization, bandwidth, or some other criteria which is left to the manufacturer.  It may also be a null function which admits all requests.

·	Bandwidth Control - The Gatekeeper shall support BRQ/BRJ/BCF messages.  This may be based on bandwidth management.  It may also be a null function which accepts all requests for bandwidth changes.

·	Zone Management - The Gatekeeper shall provide the above functions for terminals, MCUs, and Gateways which have registered with it as described in Section 7.2.  

The Gatekeeper may also perform other optional function such as:

·	Call Control Signalling - The Gatekeeper may choose to complete the call signalling with the endpoints and may process the call signalling itself.  Alternatively, the Gatekeeper may direct the endpoints to connect the Call Signalling Channel directly to each other.  In this manner, the Gatekeeper can avoid handling the H.225.0 call control signals. The Gatekeeper may have to act as the network as defined in Q.931 in order to support supplementary services. This operation is for further study.

·	Call Authorization - Through the use of the H.225.0 signalling, the Gatekeeper may reject calls from a terminal due to authorization failure.  The reasons for rejection may include, but are not limited to, restricted access to/from particular terminals or Gateways, and restricted access during certain periods of time.  The criteria for determining if authorization passes or fails is outside the scope of this Recommendation.

·	Bandwidth Management - Control of the number of H.323 terminals permitted simultaneous access to the LAN.  Through the use of the H.225.0 signalling, the Gatekeeper may reject calls from a terminal due to bandwidth limitations.  This may occur if the Gatekeeper determines that there is not sufficient bandwidth available on the network to support the call.  The criteria for determining if bandwidth is available is outside the scope of this Recommendation.  Note that this may be a null function, i.e.  all terminals are granted access.  This function also operates during an active call when a terminal requests additional bandwidth.

·	Call Management - For example, the Gatekeeper may maintain a list of ongoing H.323 calls.  This information may be necessary to indicate that a called terminal is busy, and to provide information for the Bandwidth Management function.

·	Gatekeeper management information data structure.  For further study.

�symbol 183 \f "Symbol" \s 10 \h��	Bandwidth reservation for terminals not capable of this function.  For further study.

�symbol 183 \f "Symbol" \s 10 \h��	Directory services.  For further study.

In order to support Ad Hoc Multipoint Conferences, the Gatekeeper may choose to receive the H.245 Control Channels from the two terminals in a point-to-point conference.  When the conference switches to a multipoint conference, the Gatekeeper can redirect the H.245 Control Channel to an MC.  The Gatekeeper need not process the H.245 signalling, it only needs to pass it between the terminals or the terminals and the MC.

LANs which contain Gateways should also contain a Gatekeeper in order to translate incoming E.164 addresses into Transport Addresses.

H.323 entities that contain a Gatekeeper shallshould have a mechanism to disable the internal Gatekeeper so that when there are multiple H.323 entities that contain a Gatekeeper on a LAN, the H.323 entities can be configured into the same Zone. 

6.5 Multipoint Controller Characteristics

The MC provides control functions to support conferences between three or more endpoints in a multipoint conference.  The MC carries out the capabilities exchange with each endpoint in a multipoint conference.  The MC sends a capability set to the endpoints in the conference indicating the operating modes in which they may transmit.  The MC may revise the capability set that it sends to the terminals as a result of terminals joining or leaving the conference, or for other reasons.

In this manner, the MC determines the Selected Communication Mode (SCM) for the conference.  The SCM may be common for all endpoints in the conference.  Alternatively, some endpoints may have a different SCM than other endpoints in the conference.  The manner in which the MC determines a SCM is not within the scope of this Recommendation.

As part of multipoint conference setup, an endpoint will become connected to an MC on its H.245 Control Channel.  This connection may occur:

- via an explicit connection with an MCU.

- via an implicit connection to the MC within a Gatekeeper.

- via an implicit connection to the MC within another terminal or Gateway in the multipoint conference.

- via an implicit connection through a Gatekeeper to an MCU.

The choice of conference mode (e.g.  decentralized or centralized) occurs after connection with the MC using H.245 signalling.  The choice of conference mode may be limited by the capability of the endpoints or the MC.  

The MC may be located within a Gatekeeper, Gateway, terminal, or MCU.  See Figure 6/H.323.  

�embed MSDraw \* mergeformat ���

Figure 6/H.323 Possible locations of MC and MP in H.323 system 

An MC within a terminal is not callable.  It can be included in the call in order to process the H.245 signalling to support Ad Hoc multipoint conferences.  In this case, there may be no distinction between the  MC and the H.245 Control Function  (Section 6.2.8) of the terminal.  They may be the same entity or separate entities.  If separate, Ccommunications between them is outside the scope of this recommendation.

An MC located with the Gatekeeper is not callable, however, an MCU located with a Gatekeeper is callable.  An MCU located with a Gatekeeper functions as an independent MCU.  An MC located with a Gatekeeper may be used to support Ad Hoc multipoint conferences when the Gatekeeper receives the H.245 Control Channels from the endpoints.  In this manner, the Gatekeeper can route the H.245 Control Channels to the MC at the start of the call or when the conference switches to multipoint.

The Gateway can function as a terminal or an MCU.  When functioning as a terminal, the Gateway may contain an MC.  This has the same characteristics as described above for an MC within a terminal.

An MCU always contains an MC.  The MCU is callable and the MC processes the H.245 Control channel from all of the endpoints.

When two or more endpoints are in a decentralized multipoint conference contain an MC, the endpoints shall use the master slave resolution procedure of H.245 to determine the MC that will control the conference.

After the capability exchange and master/slave determination, the MC may first assign a terminal number to a new endpoint using the terminalNumberAssign. The MC shall then notify the other endpoints of the new endpoint in the conference using terminalJoinedConference. The new endpoint may request a list of other endpoints in the conference using the terminalListRequest.

6.6 Multipoint Processor Characteristics

The MP receives audio, video, and/or data streams from the endpoints involved in a centralized or hybrid multipoint conference.  The MP processes these media streams, and returns them to the endpoints.  

Communications between the MC and the MP are not subject to standardization.

The MP may process one or more media stream types.  When the MP processes video, it shall process the video algorithms and formats as described in 6.2.4.  When the MP processes audio, it shall process the audio algorithms as described in 6.2.5.  When the MP processes data, it shall process data streams as described in 6.2.7.

An MP which processes video shall provide either video switching or video mixing.  Video switching is the process of selecting the video that the MP outputs to the terminals from one source to another.  The criteria used to make the switch may be determined through detection of a change in speaker (sensed by the associated audio level) or through H.245 control.  Video mixing is the process of formatting more than one video source into the video stream that the MP outputs to the terminals.  An example of video mixing is combining four source pictures into a two by two array in the video output picture.  The criteria for which sources and how many are mixed is determined by the MC until other controls are defined.  The use of the T.120AVCseries for these control functions is for further study.

An MP which processes audio shall prepare N audio outputs from M audio inputs by switching, mixing, or a combination of these.  Audio mixing requires decoding the input audio to linear signals (PCM or analog), performing a linear combination of the signals, and recoding the result to the appropriate audio format.  The MP may eliminate or attenuate some of the input signals in order to reduce noise and other unwanted signals.  Each audio output may have a different mix of input signals providing for private conversations.  The terminals shallmay assume that their audio is not present in the audio stream returned to them.  Terminal removal of its own audio from the MP audio output is for further study.

An MP which processes T.120 data shall be capable of acting as a non-leaf MCS provider and should be capable of acting as the Top MCS Provider.  An MP may also process non-standard data, transparent user data, and/or other types of data.

The MP may provide algorithm and format conversion, allowing terminals to participate in a conference at different SCMs.

The MP is not callable, the MCU which it is a part of is callable.  The MP terminates and sources the media channels.

6.7 Multipoint Control Unit Characteristics

The MCU is an endpoint which provides support for multipoint conferences.  The MCU shall consist of an MC and zero or more MPs.  The MCU uses H.245 messages and procedures to implement features similar to those found in H.243.  More information on H.245 MCU operation can be found in H.MCU245 (proposed).  This is for further study.

A typical MCU that supports centralized multipoint conferences consists of an MC and an audio, video, and data MP.  A typical MCU that supports decentralized multipoint conferences consists of an MC and a data MP supporting T.120.  It relies on decentralized audio and video processing.

The LAN side of a Gateway may be an MCU.  A Gatekeeper may also include an MCU.  In either case they are independent functions that happen to be co-located.

The MCU shall be callable by other endpoints using the procedures of Section 8.

6.8 Multipoint Capability

6.8.1 Centralized Multipoint Capability

All terminals shall have centralized multipoint capability.  A Gateway which appears as a terminal on the LAN shall also have centralized multipoint capability.  In this mode of operation they communicate with the MC of the MCU in a point-to-point manner on the control channel and with the MP on the audio, video, and data channels.  In this mode, the MC performs H.245 multipoint control functions, while the MP performs video switching or mixing, audio mixing, and T.120 multipoint data distribution.  The MP transmits the resulting video, audio, and data streams back to the terminals.  The MP may have the capability to convert between different audio, video, and data formats and bitrates, allowing the terminals to participate in the conference using different communications modes.

The MCU may use multicast to distribute the processed video if the terminals in the conference can receive multicast transmissions.  Multicast distribution of processed audio is for further study.

This mode is signalled by the following H.245 capabilities: centralizedControl, centralizedAudio, centralizedVideo, and centralizedData.

6.8.2 Decentralized Multipoint Capability

If the terminals have decentralized multipoint capability, they communicate with the MC of an MCU, Gateway, Gatekeeper, or terminal in a point-to-point mode on the H.245 Control Channel and optionally with an MP on data channels.  The terminals shall have the capability to multicast their audio and video channels to all other endpoints in the conference.  The MC may control which terminal or terminals are actively multicasting audio and/or video (for example by using the flowControlCommand on either channel).

The terminals receive multicast video channels and select one or more of the available channels for display to the user.  The terminals receive the multicast audio channels and perform an audio mixing function in order to present a composite audio signal to the user.

The MC may provide conference control functions such as chair control, video broadcast and video selection.  This shall be done by receiving H.245 from a terminal and then sending the appropriate control to other terminals to enable or disable their video multicast.  T.120 commands may optionally provide the same functions.

This mode is signalled by the following H.245 capabilities: centralizedControl, decentralizedAudio, decentralizedVideo, and centralizedData.

6.8.3 Hybrid Multipoint - Centralized Audio

If the terminals and MCU have hybrid multipoint-centralized audio capability, they may use distributed multipoint for video and centralized multipoint for audio.  In this mode, the terminals communicate with the MC in a point-to-point mode on the H.245 Control Channel and optionally with an MP on data channels.

The terminals shall have the capability to multicast their video channels to all other endpoints in the conference.  The MC may control which terminal or terminals are actively multicasting video.  The terminals receive multicast video channels and select one or more of the available channels for display to the user.  

All of the terminals in the conference transmit their audio channels to the MP.  The MP performs the audio mixing function and outputs the resulting audio streams to the terminals.  The MP may produce an exclusive audio sum for each terminal in the conference. Multicast distribution of processed audio is for further study.

This mode is signalled by the following H.245 capabilities: centralizedControl, centralizedAudio, decentralizedVideo, and centralizedData.

6.8.4 Hybrid Multipoint - Centralized Video

If the terminals and MCU have hybrid multipoint-centralized video capability, they may use distributed multipoint for audio and centralized multipoint for video.  In this mode, the terminals communicate with the MC in a point-to-point mode on the H.245 Control Channel and optionally with an MP on data channels.

The terminals shall have the capability to multicast their audio channels to all other endpoints in the conference.  The MC may control which terminal or terminals are actively multicasting audio.  The terminals receive multicast audio channels and perform a mixing function in order to present a composite audio signal to the user.  

All of the terminals in the conference transmit their video channels to the MP.  The MP performs the video switching,  mixing, or format conversion functions and outputs the resulting video streams to the terminals.  The MP may produce an exclusive video stream for each terminal in the conference, or it may multicast a video stream to all participating terminals, in order to minimize the bandwidth used on the LAN.

This mode is signalled by the following H.245 capabilities: centralizedControl, decentralizedAudio, centralizedVideo, and centralizedData.

6.8.5 Establishment of Common Mode

The MC shall coordinate a common communications mode between the terminals in the multipoint conference.  The MC may force terminals into a particular common mode of transmission (as allowed by their capability sets) by sending to the terminal a receive capability set listing only the desired mode of transmission, or the MC may rely on multipointModeCommand and mode preference commands to enforce mode symmetry.  The later approach should be used since it allows the terminals to know the full range of conference capabilities available that can be requested.

If the MCU has the capability to convert audio and/or video formats, it may not be necessary to force all terminals into the same communications mode.

6.8.6 Multipoint rate matching

Since the terminals on each link in a multipoint configuration may attempt to operate at different bit rates,  the MC shall send H.245 flowControlCommand messages to limit the transmitted bit rates to those which can be sent to receivers.

6.8.7 Multipoint Lip Synchronization

An MP which is providing audio mixing in either the Centralized or Hybrid multipoint conferences shall modify the time tags of the audio and video streams, taking into account its own time base, in order to maintain audio and video synchronization.  Further, when the MP processes the audio and/or video to generate a new stream sourced from the MP, the MP shall generate its own sequence numbers in the audio and video packets.

When mixing audio, the MP shouldshall synchronize each of the incoming audio streams to its own timing, mix the audio streams, and then shall generate a new audio stream based on its own timing with its own sequence numbers.  If the MP is also switching video, the switched stream shall have its original time stamp replaced with the MP time base to synchronize it with the mixed audio stream and shall have a new sequence number representing the stream from the MP.

In the case of distributed multipoint conferences, the receiving terminal may be able to maintain lip synchronization by aligning the selected video stream and its associated audio by using the RTP time tags.  Alignment of the other audio streams may not be necessary.  If multiple video streams are displayed, the associated audio streams should be aligned.

It may not be possible to guarantee lip synchronization in hybrid multipoint conferences.

6.8.8 Multipoint Encryption

In a centralized multipoint configuration the MP is considered to be a trusted entity.  Each port of the MP decrypts the information streams from each of the H.323 terminals and encrypts the information streams to each terminal in accordance with Section 10.1.  Operation of an untrusted MCU is for further study.  

Encryption in decentralized and hybrid multipoint conferences is for further study.

6.8.9 Cascading Multipoint Control Units

The multipoint control function may be distributed between several MCU entities.  Such operations are for further study. 

7 Call Signalling

Call signalling is the messages and procedures used to establish a call, request changes in bandwidth of the call, get status of the endpoints in the call , and disconnect the call.  Call signalling uses messages defined in H.225.0 and the procedures described in Section 8.  This section describes some call signalling concepts.

7.1 Addresses

7.1.1 LAN Address

Each H.323 entity shall have at least one LAN Address.  This address uniquely identifies the H.323 entity on the LAN.  Some entities may share a LAN Address (ie.  a terminal and a co-located MC).  This address is specific to the LAN environment in which the endpoint is located.  Different LAN environments may have different LAN Address formats.  

An endpoint may use different LAN addresses for different channels within the same call.

7.1.2 TSAP Identifier

For each LAN Address, each H.323 entity may have several TSAP Identifiers.  These TSAP Identifiers allow multiplexing of several channels sharing the same LAN Address.  

Terminals, Gateways, and MCUEndpoints have onetwo well known TSAP Identifiers defined: the RAS Channel TSAP Identifier and the Call Signalling Channel TSAP Identifier.  Gatekeepers have one well known TSAP Identifier defined: the RAS Channel TSAP Identifier and one well known multiast address defined: Discovery Multicast Address.  These TSAP Identifiers are defined in H.225.0 Appendixnnex D.

Endpoints and H.323 entities should use dynamic TSAP Identifiers for the H.245 Control Channel, Audio Channels, Video Channels, and Data Channels.  The Gatekeeper should use a dynamic TSAP Identifier for Call Signalling Channels. The RAS Channels and Signalling Channels may be redirected to dynamic TSAP Identifiers during the registration procedure.

7.1.3 Alias Address

An endpoint may also have one or more alias addresses associated with it.  The alias addresses provide an alternate method of addressing the endpoint.  These address include E.164 addresses (network access number, telephone number, etc.), H.323 IDs (name, e-mail like address, etc.), and any others defined in H.225.0.  Alias addresses shall be unique within a Zone.  Gatekeepers, MCs, and MPs shall not have alias addresses.

When there is no Gatekeeper in the system, the calling endpoint shall address the called endpoint directly using the Call Signalling Channel Transport Address of the called endpoint.  When there is a Gatekeeper in the system, the calling endpoint may address the called endpoint by its Call Signalling Channel Transport Address, or alias address.  The latter shall be translated into a Call Signalling Channel Transport Address by the Gatekeeper.

The called endpoint’s E.164 address may consist of an optional access code followed by the E.164 address.  The access code consists of n digits from the set of 0 to 9, *, and #.  The number of digits and their meaning is left to the discretion of the manufacturer.  One purpose of such an access code might be to request access to a Gateway.  When this access code is used, it shall be the responsibility of the Gatekeeper to remove it from the destination addresses prior to the Setup message being sent to the Gateway. The Gatekeeper may alter this address prior to sending it to the destination.

The H.323 ID consists of a string of up to 64 ISO/IEC 10646-1 characters as defined in H.225.0terminated with a null.  The H.323 ID must be unique within a Zone.  It may be a user name, email name, or other identifier.

An endpoint may have more than one alias address (including more than one of the same type) which is translated to the same Transport Address. The endpoint’s alias addresses shall be unique within a Zone.

7.2 Registration, Admissions, and Status (RAS) Channel

The RAS Channel shall be used to carry messages used in the Gatekeeper discovery and endpoint registration processes which associate an endpoint’s alias address with its Call Signalling Channel Transport Address.  The RAS Channel shall be an unreliable channel.

7.2.1 Gatekeeper Discovery

Gatekeeper discovery is the process an endpoint uses to determine which Gatekeeper to register with.  This may be done manually or automatically.  Manual discovery relies on methods outside the scope of this Recommendation to determine which Gatekeeper an endpoint is associated with.  The endpoint is configured with the Transport Address of the associated Gatekeeper.  For example, it may be entered at endpoint configuration, or it may be entered into a initialization file.  In this way, the endpoint knows a-priori which Gatekeeper it is associated with.  The endpoint can now register with that Gatekeeper.

The Automatic method allows the endpoint - Gatekeeper association to change over time.  The endpoint may not know who its Gatekeeper is, or may need to identify another Gatekeeper due to a failure.  This may be done through auto discovery.  Auto discovery allows for lower administrative overhead in configuring individual endpoints and additionally allows replacement of an existing Gatekeeper without manually reconfiguring all of the affected endpoints.

The endpoint may multicast (or use other methods as described in H.225.0 Appendix D) a Gatekeeper Request (GRQ) message, asking “Who is my Gatekeeper?”.  This is sent to the Gatekeeper’s well known Discovery Multicast AddressRAS Channel TSAP Identifier.  One or more Gatekeepers may respond with the Gatekeeper Confirmation (GCF) message indicating “I can be your Gatekeeper.”, and returns the Transport Address of the Gatekeeper’s RAS Channel.  If a Gatekeeper does not want the endpoint to register to it, it shall return Gatekeeper Reject (GRJ).  See Figure 7/H.323.  If more than one Gatekeeper responds, the endpoint may choose the Gatekeeper it wants to use.  At this point, the endpoint knows which Gatekeeper to register with.  The endpoint can now register with that Gatekeeper.

�embed MSDraw \* mergeformat ���

Figure 7/H.323 Auto Discovery

If no Gatekeeper responds within a timeout, the endpoint may retry the GRQ.  An endpoint shall not send a GRQ within 5 seconds after sending a previous one.  If no response is received, the endpoint mayshall use the manual discovery method.

If at any time an endpoint determines it has an invalid registration with its Gatekeeper, it must re-discover its Gatekeeper.  The invalid registration may be detected by either receiving an RRJ message from a Gatekeeper in response to an RRQ from the endpoint, or not receiving any response to an RRQ from the endpoint within a timeout.

The GRQ may be repeated periodically (i.e., at endpoint power up), so the Gatekeeper shall be able to handle multiple requests from the same endpoint.

7.2.2 Endpoint Registration

Registration is the process by which an endpoint joins a Zone, and informs the Gatekeeper of its Transport Address and alias addresses.  As part of their configuration process, all endpoints shall register with the Gatekeeper identified through the discovery process.  Registration shall occur before any calls are attempted, and may occur periodically as necessary (for example, at endpoint power up).

An endpoint shall send a Registration Request (RRQ) to athe Gatekeeper which they are registered to.  This is sent to the Gatekeeper’s RAS Channel Transport Address.  The endpoint has the LAN Address of the Gatekeeper from the Gatekeeper discovery process and uses the well known RAS Channel TSAP Identifier.  The Gatekeeper shall respond with either a Registration Confirmation (RCF) or a Registration Reject (RRJ). See Figure 8/H.323.  An endpoint shall only register with a single Gatekeeper.

The RRQ may be repeated periodically (ie, at terminal power up), so the Gatekeeper shall be able to handle multiple requests from the same endpoint. If a Gatekeeper receives an RRQ having the same alias address and the same Transport address as a previous RRQ, it shall respond with RCFshould ignore the registration. If a Gatekeeper receives an RRQ having the same alias address as a previous RRQ and a different Transport address, it should reject the registration indicating a duplicate registration. The endpoint should then unregister the alias address, and re-register it with the new Transport Address.  If the Gatekeeper receives an RRQ having the same Transport Address as a previous RRQ and a different alias address, it should replace theadd a new translation table entryies for the new alias.  The Gatekeeper may have a method to authenticate these changes which is for further studyoutside the scope of this Recommendation.

The Gatekeeper shall ensure that each alias address translates uniquely to a single Transport Address.  Ambiguous registrations shall be rejected by the Gatekeeper.  The Gatekeeper may reject the registration for other reasons, such as changes in discovery or security issues.

If the endpoint does not include an alias address in the RRQ message, the Gatekeeper may assign one.  The Gatekeeper shall return the assigned alias address to the terminal in the RCF message.

�embed MSDraw \* mergeformat ����

Figure 8/H.323 Registration

An endpoint may cancel its registration by sending an Unregister Request message (URQ) to the Gatekeeper.  The Gatekeeper shall respond with an Unregister Confirmation message (UCF).  This allows endpoints to change the alias address associated with its Transport Address, or vice versa.  If the endpoint was not registered with the Gatekeeper, it shall return an Unregister Reject message (URJ) to the endpoint.

A Gatekeeper may cancel the registration of an endpoint by sending an Unregister Request message (URQ) to the endpoint.  The endpoint shall respond with an Unregister Confirmation message (UCF).  The endpoint shall re-register with a Gatekeeper prior to initiating any calls.  This may require the endpoint to register with a new Gatekeeper.

An endpoint which is not registered with a Gatekeeper is called an unregistered endpoint.  This type of endpoint does not request admission permission from a Gatekeeper and so cannot participate in admissions control, bandwidth control, address translation, and other functions performed by the Gatekeeper.

7.2.3 Endpoint Location

An endpoint or Gatekeeper which has an alias address for an endpoint and would like determine its Transport Address may issue a Location Request (LRQ) message. This message may be sent to a specific Gatekeeper, or may be multicast like the GRQ message.  This is sent to the Gatekeeper’s well known RAS Channel TSAP Identifier, or if multicast, is sent to the Gatekeeper’s well known Discovery Multicast Addressr.  The Gatekeeper with which the requested endpoint is registered, shall respond with the Location Confirmation (LCF) message containing the Transport Address of the endpoint’s Call Signalling Channel or the endpoint’s Gatekeeper’s Call Signalling Channel.  All Gatekeepers with which the requested endpoint is not registered, shall return Location Reject (LRJ) if they received the LRQ on the RAS Channel. Any Gatekeeper with which the requested endpoint is not registered, shall not respond to the LRQ, if it received the LRQ on the Discovery Multicast address,.

7.2.4 Admissions, Bandwidth Change, Status, Disengage

The RAS Channel is also used for the transmission of Admissions, Bandwidth Change, Status, and Disengage messages.  These messages take place between an endpoint and a Gatekeeper and are used to provide admissions control and bandwidth management functions.  The detailed use of these messages is described in Section 8.

The Admissions Request (ARQ) message specifies the requested Call Bandwidth.  This is an upper limit on the aggregate bitrate for all transmitted and received, audio and video channels excluding any RTP headers, RTP payload headers, LAN headers, and other overhead.  Data and control channels are not included in this limit.  The Gatekeeper may reduce the requested Call Bandwidth in the Admissions Confirm (ACF) message.  An endpoint shall assure that the aggregate bitrate, averaged over one second, for all transmitted and received, audio and video channels is at or below the Call Bandwidth.  An endpoint or the Gatekeeper may attempt to modify the Call Bandwidth during a call using the Bandwidth Change Request (BRQ) message.

7.3 Call Signalling Channel

The Call Signalling Channel shall be used to carry H.225.0Q.931 call control messages.  The Call Signalling channel shall be a reliable channel.

In networks that do not contain a Gatekeeper, call signalling messages are passed directly between the calling and called endpoints using the Call Signalling Transport Addresses.  In these networks, it is assumed that the calling endpoint knows the Call Signalling Transport Address of the called endpoint and thus can communicate directly.

In networks that do contain a Gatekeeper, the initial admission message exchange takes place between the calling endpoint and the Gatekeeper using the Gatekeeper's RAS Channel Transport Address.  Within the initial admissions message exchange, the Gatekeeper indicates in the ACF message whether to send the call signalling directly to the other endpoint or to route it through the Gatekeeper.  The call signalling messages are sent to either the endpoint’s Call Signalling Transport Address or the Gatekeeper’s Call Signalling Transport Address.

H.225.0 specifies the mandatory Q.931 messages that are used for call signalling in H.323.  Section 8 specifies the procedures for using them.  All other Q.931 messages are optional.

7.3.1 Call Signalling Channel Routing

Call signalling messages may be passed in two ways.  The first method is Gatekeeper Routed Call Signalling (Figure 9/H.323).  In this method, call signalling messages are routed through the Gatekeeper between the endpoints.  The second method is Direct Endpoint Call Signalling (Figure 10/H.323).  In this method, the call signalling messages are passed directly between the endpoints.  The choice of which methods is used is made by the Gatekeeper.

Both methods use the same kinds of connections for the same purposes, and the same messages.  Admissions messages are exchanges on RAS channels with the Gatekeeper, followed by an exchange of call signalling messages on a Call Signalling channel.  This is then followed by the establishment of the H.245 Control Channel.  The actions of the Gatekeeper in response to the admission messages determine which call model is used, this is not under the control of the endpoint, although the endpoint can specify a preference.

For the Gatekeeper Routed method, the Gatekeeper may choose to close the Call Signalling Channel after the call setup is completed, or it may choose to keep it open for the duration of the call to support supplementary services.  Only the Gatekeeper shall close the Call Signalling Channel and it should not be closed when a Gateway is involved in the call.

The symmetrical signalling method of Q.931 Annex D shall be used for all mandatory call signalling procedures.  This does not address the role that a Gateway might play on the SCN side using Q.931 or other call signalling protocols.

The Gatekeeper Clouds in Figure 9/H.323 thru Figure 12/H.323 contain one or more Gatekeepers which may or may not communicate with each other.  The endpoints may be connected to the same Gatekeeper or to different Gatekeepers.



�embed MSDraw \* mergeformat ���

Figure 9/H.323 Gatekeeper Routed Call Signalling

�embed MSDraw \* mergeformat ���

Figure 10/H.323 Direct Endpoint Call Signalling

7.3.2 Control Channel Routing

When Gatekeeper Routed call signalling is used, there are two methods to route the H.245 Control Channel.  In the first method, the H.245 Control Channel is established directly between the endpoints.  See Figure 11/H.323. This method is for further study. In the second method, the H.245 Control Channel is routed between the endpoints through the Gatekeeper.  See Figure 12/H.323.  This method allows the Gatekeeper to redirect the H.245 Control Channel to an MC when an Ad Hoc multipoint conference switches from a point-to-point conference to a multipoint conference.  This choice is made by the Gatekeeper.  When Direct Endpoint call signalling is used, the H.245 Control Channel can only be connected directly between the endpoints.

�embed MSDraw \* mergeformat ���

Figure 11/H.323: Direct H.245 Control Channel Connection Between Endpoints

�embed MSDraw \* mergeformat ���

Figure 12/H.323 Gatekeeper Routed H.245 Control

7.4 Call Reference Value

All call signalling and RAS messages contain a Call Reference Value (CRV).  Refer to H.225.0. This is used to associate all messages related to the same call.  The same CRV shall be used in all admissions, call setup, supplementary service, bandwidth change, and call termination messages relating to the same call, including multiple Setup messages within the same procedure defined in Section 8.1.  A new CRV shall be used for new calls.  A second call from an endpoint to invite another endpoint into the same conference shall use a new CRV.  The CRV is not the same as the Conference ID (CID).  The CRV relates messages within the same call, the CID relates calls within the same conference.

7.5 Conference ID and Conference Goal

The Conference ID (CID) is a unique non-zero value created by the originating endpoint and passed in various H.225.0 messages. The CID identifies the conference with which the message is associated. The CID shall be formed from 16 octets as follows:

CID octet�15�14�13�12�11�10�9�8�7�6�5�4�3�2�1�0��value�N5�N4�N3�N2�N1�N0�C1�C0�H1�AV�M1�M0�L3�L2�L1�L0��where: 

N5:0 is 48 bits of physical LAN address

C1:0 is 16 bits of counter incremented per conference if the clock cannot be guaranteed to be monotonic.

H1, A,M1:0,L3:0 is 60 bits of 100 nanosecond clock since Oct 15, 1582

V is 4 bits of version number = 0001.

The conferenceGoal indicates the intention of the call. Choices are: Create - to create a new conference, Join - to join an existing conference, and Invite - to invite a new endpoint into an existing conference.

The Conference ID (CID) is a value passed in the Setup and Connect message.  The CID is used to indicate if a call setup exchange is to create a new conference or to join an existing conference.

To create a new conference, either point-to-point or multipoint, the calling endpoint sends a Setup message containing a CID = 0 to the called endpoint.

To join an existing conference, the calling endpoint sends a Setup message, containing the CID of that conference, to the endpoint containing the MC.

To invite an endpoint to join an existing conference, the endpoint containing the MC sends a Setup message, containing the CID of that conference, to the invited endpoint.

When call signalling is routed through the Gatekeeper, the Gatekeeper forwards the Setup and Connect messages between the calling and called endpoints, without modifying the CID.

An endpoint receiving a Setup message containing CID = 0, is being asked to enter a new conference. If the called endpoint wishes to enter the new conference, it assigns a unique non-zero CID to this new conference.  The new CID is returned to the calling endpoint in the Connect message. If the called endpoint is in another conference, it may reject the call by sending Release Complete indicating destination busy, or it may invite the calling endpoint to join the other conference by sending a Release Complete message indicating the CID of the conference and the Transport Address of the endpoint containing the MC.  The calling endpoint may then join the conference by sending a new Setup message to the endpoint containing the MC with the new CID.

An endpoint receiving a Setup message with a non-zero CID must determine if the calling endpoint is attempting to join the called endpoints conference, or asking the called endpoint to join another conference.

If the called endpoint is not in another conference or if the received CID does not match the CID of the called endpoints current conference, then the called endpoint is being invited to join another conference. If the called endpoint wishes to enter the new conference, it returns the received CID to the calling endpoint in the Connect message. If the called endpoint is in another conference, it may reject the call by sending Release Complete indicating destination busy, or it may invite the calling endpoint to join the other conference by sending a Release Complete message indicating the CID of the conference and the Transport Address of the endpoint containing the MC. Note that only an endpoint that contains an MC can invite another endpoint to join a conference. If neither the calling endpoint nor the called endpoint contain an MC, the call will connect as a point-to-point conference that will not be able to be expanded.

If the called endpoint is in another conference and the received CID matches the CID of the called endpoint’s current conference, then the calling endpoint is asking to join the current conference. If the called endpoint contains an MC and wishes to allow the calling endpoint to join, it returns the CID to the calling endpoint in the Connect message. If the called endpoint does not wish to allow the calling endpoint to join, it may reject the call by sending Release Complete indicating destination busy. If the called endpoint does not contain an MC, it may invite the calling endpoint to join through the MC by sending a Release Complete message indicating the CID of the conference and the Transport Address of the endpoint containing the MC.

The CID shall consist of the called endpoints Call Signalling Channel Transport Address plus a sequence number.  The endpoint shall guarantee that the sequence number is unique for each conference that it is participating in.

8.  Call Signalling Procedures

The provision of the communication is made in the following steps:

-	phase A: Call set-up (subclause 8.1);

-	phase B: Initial communication and capability exchange (subclause 8.2);

-	phase C: Establishment of audio visual communication (subclause 8.3);

-	phase D: Call Services (subclause 8.4);

-	phase E: Call termination (subclause 8.5)



8.1 Phase A  - Call set-up

Call Set-up takes place using the call control messages defined in H.225.0 according to the call control procedures defined below. Requests for bandwidth reservation from services such as RSVP should take place at the earliest possible prior to the procedures of Phase A.

If both the alias address and the transport address are specified, preference shall be given to the alias address.

8.1.1 Basic Call Setup - Neither Endpoint Registered

In the scenario shown in Figure 13/H.323 neither endpoint is registered to a Gatekeeper.  The two endpoints communicate directly.  Endpoint 1 (calling endpoint) sends the Setup (1) message to the well known Call Signalling Channel TSAP Identifier of endpoint 2.  Endpoint 2 responds with the Connect (4) message which contains an H.245 Control Channel Transport Address for use in H.245 signalling.  Messages (2), (3) and (5) are optional.

�embed MSDraw \* mergeformat ����

Figure 13/H.323 Basic Call Setup, no Gatekeepers

8.1.2 Both Endpoints Registered to the Same Gatekeeper

In the scenario shown in Figure 14/H.323, both endpoints are registered to the same Gatekeeper, and the Gatekeeper has chosen Direct Call Signalling.  Endpoint 1 (calling endpoint) initiates the ARQ (1)/ACF (2) exchange with that Gatekeeper.  The Gatekeeper shall return the Call Signalling Channel Transport Address of endpoint 2 (called endpoint) in the ACF.  Endpoint 1 then sends the Setup (3) message to endpoint 2 using that Transport Address.  If endpoint 2 wishes to accept the call, it initiates an ARQ (5)/ACF (6) exchange with the Gatekeeper.  It is possible that an ARJ (6) is received by endpoint 2, in which case it sends Release Complete to endpoint 1.  Endpoint 2 responds with the Connect (4) message which contains an H.245 Control Channel Transport Address for use in H.245 signalling.  Messages (4), (7) and (9) are optional.



�embed MSDraw \* mergeformat ����

Figure 14/H.323 Both Endpoints Registered, Same Gatekeeper - Direct Call Signalling

In the scenario shown in Figure 15/H.323, both endpoints are registered to the same Gatekeeper, and the Gatekeeper has chosen to route the call signalling.  Endpoint 1 (calling endpoint) initiates the ARQ (1)/ACF (2) exchange with that Gatekeeper.  The Gatekeeper shall return a Call Signalling Channel Transport Address of itself in the ACF.  Endpoint 1 then sends the Setup (3) message using that Transport Address.  The Gatekeeper then sends the Setup (4) message to endpoint 2.  If endpoint 2 wishes to accept the call, it initiates an ARQ (6)/ACF (7) exchange with the Gatekeeper.  It is possible that an ARJ (7) is received by endpoint 2, in which case it sends Release Complete to the Gatekeeperendpoint 1.  Endpoint 2 responds with the Connect (9) message which contains an H.245 Control Channel Transport Address for use in H.245 signalling.  The Gatekeeper sends the Connect (10) message to endpoint 1 which may contain the endpoint 2 H.245 Control Channel Transport Address, or a Gatekeeper (MC) H.245 Control Channel Transport Address, based on whether the Gatekeeper chooses to route the H.245 Control Channel or not.  Messages (5), (8) and (11) are optional.

�embed MSDraw \* mergeformat ����

Figure 15/H.323 Both Endpoints Registered, Same Gatekeeper - Gatekeeper Routed Call Signalling

8.1.3 Only Calling Endpoint has Gatekeeper 

In the scenario shown in Figure 16/H.323, endpoint 1 (calling endpoint) is registered with a Gatekeeper, endpoint 2 (called endpoint) is not registered with a Gatekeeper, and the Gatekeeper has chosen Direct Call Signalling.  Endpoint 1 initiates the ARQ (1)/ACF (2) exchange with the Gatekeeper.  Endpoint 1 then sends the Setup (3) message to endpoint 2 using the well known Call Signalling Channel Transport Address.  If endpoint 2 wishes to accept the call, it responds with the Connect (4) message which contains an H.245 Control Channel Transport Address for use in H.245 signalling.  Messages (4), (5) and (7) are optional.



�embed MSDraw \* mergeformat ����

Figure 16/H.323 Only Calling Endpoint Registered - Direct Call Signalling

In the scenario shown in Figure 17/H.323, endpoint 1 (calling endpoint) is registered with a Gatekeeper, endpoint 2 (called endpoint) is not registered with a Gatekeeper, and the Gatekeeper has chosen to route the call signalling.  Endpoint 1 (calling endpoint) initiates the ARQ (1)/ACF (2) exchange with that Gatekeeper.  The Gatekeeper shall return a Call Signalling Channel Transport Address of itself in the ACF(2).  Endpoint 1 then sends the Setup (3) message using that Transport Address.  The Gatekeeper then sends the Setup (4) message to the well known Call Signalling Channel Transport Address of endpoint 2.  If endpoint 2 wishes to accept the call, it responds with the Connect (7) message which contains an H.245 Control Channel Transport Address for use in H.245 signalling.  The Gatekeeper sends the Connect (9) message to endpoint 1 which may contain the endpoint 2 H.245 Control Channel Transport Address, or a Gatekeeper (MC) H.245 Control Channel Transport Address, based on whether the Gatekeeper chooses to route the H.245 Control Channel or not.  Messages (5), (6) and (9) are optional.

�embed MSDraw \* mergeformat ����

Figure 17/H.323 Only Calling Endpoint Registered - Gatekeeper Routed Call Signalling

8.1.4 Only Called Endpoint has Gatekeeper

In the scenario shown in Figure 18/H.323, endpoint 1 (calling endpoint) is not registered with a Gatekeeper, endpoint 2 (called endpoint) is registered with a Gatekeeper, and the Gatekeeper has chosen Direct Call Signalling.  Endpoint 1 sends the Setup (1) message to endpoint 2 using the well known Call Signalling Channel Transport Address.  If endpoint 2 wishes to accept the call, it initiates an ARQ (3)/ACF (4)  exchange with the Gatekeeper.  It is possible that an ARJ (4) is received by endpoint 2, in which case it sends Release Complete to endpoint 1.  Endpoint 2 responds with the Connect (6) message which contains an H.245 Control Channel Transport Address for use in H.245 signalling.  Messages (2), (5) and (7) are optional.

�embed MSDraw \* mergeformat ����

Figure 18/H.323 Only Called Endpoint Registered - Direct Call Signalling

In the scenario shown in Figure 19/H.323, endpoint 1 (calling endpoint) is not registered with a Gatekeeper, endpoint 2 (called endpoint) is registered with a Gatekeeper, and the Gatekeeper has chosen to route the call signalling.  Endpoint 1 (calling endpoint) sends a Setup (1) message to the well known Call Signalling Channel Transport address of endpoint 2.  If endpoint 2 wishes to accept the call, it initiates the ARQ (32)/ACF (43) exchange with that Gatekeeper.  It is possible that an ARJ (3) is received by endpoint 2, in which case it sends Release Complete to endpoint 1.  If acceeptable, tThe Gatekeeper shall return a Call Signalling Channel Transport Address of itself in the ARJCF(43) with a cause code of routeCallToGatekeeper.  Endpoint 2 replies to endpoint 1 with a Release CompleteFacility (54) message containing the Call Signalling Transport Address of its Gatekeeper.  Endpoint 1 then sends the Release Complete (6) message to endpoint 2. Endpoint 1 sends a Setup (75) message to theat Gatekeeper’s Call Signalling Channel Transport Address.  The Gatekeeper sends the Setup (84) message to endpoint 2.  Endpoint 2 initiates the ARQ (9)/ACF (10) exchange with that Gatekeeper. Endpoint 2 then responds with the Connect (129) message which contains its H.245 Control Channel Transport Address for use in H.245 signalling.  The Gatekeeper sends the Connect (130) message to endpoint 1 which may contain the endpoint 2 H.245 Control Channel Transport Address, or a Gatekeeper (MC) H.245 Control Channel Transport Address, based on whether the Gatekeeper chooses to route the H.245 Control Channel or not.  Messages (5), (8) and (11) are optional.

�embed MSDraw \* mergeformat ����embed MSDraw \* mergeformat ���

Figure 19/H.323 Only Called Endpoint Registered - Gatekeeper Routed Call Signalling

8.1.5 Both Endpoints Registered to Different Gatekeepers

In the scenario shown in Figure 20/H.323, both endpoints are registered to different Gatekeepers, and both Gatekeepers choose Direct Call Signalling.  Endpoint 1 (calling endpoint) initiates the ARQ (1)/ACF (2) exchange with Gatekeeper 1.  Gatekeeper 1 may return the Call Signalling Channel Transport Address of endpoint 2 (called endpoint) in the ACF if Gatekeeper 1 has a method of communicating with Gatekeeper 2.  Endpoint 1 then sends the Setup (3) message to either the Transport Address returned by the Gatekeeper (if available) or to the well known Call Signalling Channel Transport Address of endpoint 2.  If endpoint 2 wishes to accept the call, it initiates an ARQ (5)/ACF (6) exchange with Gatekeeper 2.  It is possible that an ARJ (6) is received by endpoint 2, in which case it sends Release Complete to endpoint 1.  Endpoint 2 responds with the Connect (8) message which contains an H.245 Control Channel Transport Address for use in H.245 signalling.  Messages (4), (7) and (9) are optional.

�embed MSDraw \* mergeformat ����

Figure 20/H.323 Both Endpoints Registered - Both Gatekeepers Direct Call Signalling

In the scenario shown in Figure 21/H.323, both endpoints are registered to different Gatekeepers, the calling endpoint’s Gatekeeper chooses Direct Call Signalling, and the called endpoint’s Gatekeeper chooses to route the call signalling.  Endpoint 1 (calling endpoint) initiates the ARQ (1)/ACF (2) exchange with Gatekeeper 1.  Gatekeeper 1 may return the Call Signalling Channel Transport Address of endpoint 2 (called endpoint) in the ACF (2) if Gatekeeper 1 has a method of communicating with Gatekeeper 2.  Endpoint 1 then sends the Setup (3) message to either the Transport Address returned by the Gatekeeper (if available) or to the well known Call Signalling Channel Transport Address of endpoint 2.  If endpoint 2 wishes to accept the call, it initiates the ARQ (5)/ACF (6) exchange with Gatekeeper 2.  It is possible that an ARJ (6) is received by endpoint 2, in which case it sends Release Complete to endpoint 1.  If acceptable, Gatekeeper 2 shall return a Call Signalling Channel Transport Address of itself in the ARJCF(6) with a cause code of routeCallToGatekeeper.  Endpoint 2 replies to endpoint 1 with a Release CompleteFacility (7) message containing the Call Signalling Transport Address of Gatekeeper 2.  Endpoint 1 then sends the Release Complete (8) message to endpoint 2. Endpoint 1 shall send a DRQ (9) to Gatekeeper 1 which responds with DCF (10). Endpoint 1 then initiates a new ARQ(11)/ACF(12) exchange with Gatekeeper 1. Endpoint 1 sends a Setup (138) message to theat Gatekeeper’s Call Signalling Channel Transport Address.  Gatekeeper 2 sends the Setup (149) message to endpoint 2.  Endpoint 2 initiates the ARQ (15)/ACF (16) exchange with Gatekeeper 2. Endpoint 2 then responds with the Connect (182) message which contains its H.245 Control Channel Transport Address for use in H.245 signalling.  Gatekeeper 2 sends the Connect (193) message to endpoint 1 which may contain the endpoint 2 H.245 Control Channel Transport Address, or a Gatekeeper 2 (MC) H.245 Control Channel Transport Address, based on whether the Gatekeeper chooses to route the H.245 Control Channel or not.  Messages (4), (10), (11)  and (14) are optional.  

�embed MSDraw \* mergeformat ����embed MSDraw \* mergeformat ���

Figure 21/H.323 Both Endpoint Registered - Direct / Routed Call Signalling

In the scenario shown in Figure 22/H.323, both endpoints are registered to different Gatekeepers, the calling endpoint’s Gatekeeper chooses to route the call signalling, and the called endpoint’s Gatekeeper chooses Direct Call Signalling.  Endpoint 1 (calling endpoint) initiates the ARQ (1)/ACF (2) exchange with Gatekeeper 1.  Gatekeeper 1 shall return a Call Signalling Channel Transport Address of itself in the ACF(2).  Endpoint 1 then sends the Setup (3) message using that Transport Address.  Gatekeeper 1 then sends the Setup (4) message containing its Call Signalling Channel Transport Address to the well known Call Signalling Channel Transport Address of endpoint 2.  If endpoint 2 wishes to accept the call, it initiates the ARQ (6)/ACF (7) exchange with Gatekeeper 2.  It is possible that an ARJ (7) is received by endpoint 2, in which case it sends Release Complete to endpoint 1.  Endpoint 2 responds to Gatekeeper 1 with the Connect (9) message which contains its H.245 Control Channel Transport Address for use in H.245 signalling.  Gatekeeper 1 sends the Connect (10) message to endpoint 1 which may contain the endpoint 2 H.245 Control Channel Transport Address, or a Gatekeeper 1 (MC) H.245 Control Channel Transport Address, based on whether the Gatekeeper chooses to route the H.245 Control Channel or not.  Messages (5), (8)  and (11) are optional.



�embed MSDraw \* mergeformat ����

Figure 22/H.323 Both Endpoint Registered - Routed / Direct Call Signalling

In the scenario shown in Figure 23/H.323, both endpoints are registered to different Gatekeepers, and both Gatekeepers choose to route the call signalling.  Endpoint 1 (calling endpoint) initiates the ARQ (1)/ACF (2) exchange with Gatekeeper 1.  Gatekeeper 1 shall return a Call Signalling Channel Transport Address of itself in the ACF(2).  Endpoint 1 then sends the Setup (3) message using that Transport Address.  Gatekeeper 1 then sends the Setup (4) message to the well known Call Signalling Channel Transport Address of endpoint 2.  If endpoint 2 wishes to accept the call, it initiates the ARQ (6)/ACF (7) exchange with Gatekeeper 2.  It is possible that an ARJ (7) is received by endpoint 2, in which case it sends Release Complete to Gatekeeper 1.  If acceptable, Gatekeeper 2 shall return a Call Signalling Channel Transport Address of itself in the ARJCF(7) with a cause code of routeCallToGatekeeper.  Endpoint 2 replies to Gatekeeper 1 with a FacilityRelease Complete (8) message containing the Call Signalling Transport Address of Gatekeeper 2.  Gatekeeper 1 then sends the Release Complete (9) message to endpoint 2. Gatekeeper 1 sends a Setup (108) message to thatGatekeeper 2’s Call Signalling Channel  Transport Address.  Gatekeeper 2 sends the Setup (110) message to endpoint 2.  Endpoint 2 initiates the ARQ (12)/ACF (13) exchange with Gatekeeper 2. Endpoint 2 then responds to Gatekeeper 2 with the Connect (153) message which contains its H.245 Control Channel Transport Address for use in H.245 signalling.  Gatekeeper 2 sends the Connect (164) message to Gatekeeper 1 which may contain the endpoint 2 H.245 Control Channel Transport Address, or a Gatekeeper 2 (MC) H.245 Control Channel Transport Address, based on whether the Gatekeeper 2 chooses to route the H.245 Control Channel or not.  Gatekeeper 1 sends the Connect (175) message to endpoint 1 which may contain the H.245 Control Channel Transport Address sent by Gatekeeper 2, or a Gatekeeper 1 (MC) H.245 Control Channel Transport Address, based on whether the Gatekeeper 1 chooses to route the H.245 Control Channel or not.  Messages (5), (11), (12)  and (16) are optional.

�embed MSDraw \* mergeformat ����embed MSDraw \* mergeformat ���

Figure 23/H.323 Both Endpoint Registered - Both Gatekeepers Routing Call Signalling



8.1.6 Call Set-up via Gateways

8.1.6.1 Gateway Inbound Call Set-up

When an external terminal calls a LAN endpoint via the Gateway, call set-up between the Gateway and the LAN endpoint proceeds the same as the endpoint to endpoint call setup.  The Gateway may need to issue Call Proceeding messages to the external terminal while establishing the call on the network.

A Gateway which cannot directly route an incoming SCN call to an H.323 endpoint shall be able to accept two stage dialing.  For Gateways to H.320 networks (also H.321, H.322, and H.310 in H.321 mode), the Gateway shall accept SBE numbers from the H.320 terminal.  For Gateways to H.310 native mode and H.324 networks, the Gateway shall accept H.245 userInputIndication messages from the H.324 terminal.  In these two cases, support of DTMF is optional.  For Gateways to speech only terminals, the Gateway shall accept DTMF numbers from the speech only terminal.  These numbers will indicate a second stage dialing number to access the individual endpoint on the LAN.

8.1.6.2 Gateway Outbound Call Set-up

When a LAN endpoint calls an external terminal via the Gateway, call set-up between the LAN endpoint and the Gateway proceeds the same as the endpoint to endpoint call setup.  The Gateway will receive the destination E.164 address in the Setup message.  It will then use this address to place the outbound call.  The Gateway may issue Call Proceeding messages to the LAN endpoint while establishing the outgoing call.

The Progress Indicator information element is used to indicate that inter-networking is occuring.  The Gateway shall issue a Progress indicator information element within the Alerting, Call Proceeding, or Connect messages.  This information may also be sent in a Progress message.

The LAN endpoint shall send alla separate Setup message for each E.164 addresses that it is calling in the Setup message. For exampleTherefore, a sixmultiple B channel call on the ISDN will require sixmultiple E.164 addresses in the Setup messages. The Gateway shall respond to theeach Setup message with a Connect or Release Complete message as well as optional Alerting, Call Proceding, or Progress messages. Failure of the SCN call shall be reported to the LAN endpoint in the Release Complete message. The use of multiple CRV values and multiple Setup messages is for further study. Addition of channels on the SCN during a call is for further study.Each Setup message shall use a unique CRV. All other messages (including status and call termination messages) associated with a specific Setup message shall use the CRV from that Setup message.

A LAN endpoint which is registered with a Gatekeeper should request sufficient call bandwidth in the ARQ message for the aggregate of all SCN calls.  If sufficient call bandwidth was not requested in the ARQ message, the procedures of section 8.4.1, Bandwidth Changes, shall be followed in order to obtain additional call bandwidth. 

A Gateway which is registered with a Gatekeeper shall request sufficient call bandwidth in the ARQ message, as indicated in the Setup message received from the LAN endpoint.  If sufficient call bandwidth was not requested in the ARQ message, the procedures of section 8.4.1, Bandwidth Changes, shall be followed in order to obtain additional call bandwidth. 

The GatewayLAN endpoint may advance to Phase B after placing the first call onto the SCNGateway.  Additional calls for the additional SCN E.164 numbers may be placed after the capability exchange with the Gateway and establishment of audio communications with the SCN endpoint. 

8.1.7 Call Setup with an MCU

For Centralized Multipoint Conferences, all endpoints exchange call signalling with the MCU.  Call set-up between an endpoint and the MCU proceeds the same as the endpoint to endpoint call setup scenarios of Sections 8.1.1 thru 8.1.5.  The MCU may be the called endpoint or the calling endpoint.

In a Centralized Multipoint Conference, the H.245 Control Channel is opened between the endpoints and the MC within the MCU. The audio, video, and data channels are opened between the endpoints and the MP within the MCU.  In a Decentralized Multipoint Conference, the H.245 Control Channel is open between the endpoint and the MC (there may be many such H.245 Control Channels, one for each callendpoint).  The Audio and Video Channels shouldwill be multicast to all endpoints in the conference.  The Data Channel shall be opened with the  Data MP.

In an Ad Hoc Multipoint Conference where there is no MC within the endpoints, the H.245 Control Channel shallmust be routed through the Gatekeeper.  Initially the H.245 Control Channel will be routed between the endpoints through the Gatekeeper.  When the conference switches to multipoint, the Gatekeeper may connect the endpoints to an MC associated within the Gatekeeper, or to an MC external to the Gatekeeper.  No additional call setup is required for the MC within the Gatekeeper.  For the MC external to the Gatekeeper, the Gatekeeper shall exchange call signalling with the MCU that contains the MC on behalf of each endpoint.  The Gatekeeper shall then establish an H.245 Control Channel with the MCU for each endpoint, and route the H.245 messages between the endpoints and the MCU through the Gatekeeper.

In an Ad Hoc Multipoint Conference where one or both of the endpoints contains an MC, the normal call setup procedures defined in Sections 8.1.1 thru 8.1.5 are used. The master slave determination procedure is used to determine which MC will be the active MC for the conference.

8.1.8 Call ForwardingTransfer

An endpoint wishing to transfer or forward a call to another endpoint may issue a FacilityRelease Complete message indicating the Transport Address of the new endpoint. The endpoint receiving this FacilityRelease Complete indication should send a Release Complete and then restart the Phase A procedures with the new endpoint.

8.1.9 Broadcast Call Setup

This section is for further study.

8.2 Phase B - Initial communication and capability exchange

Once both sides have exchanged call setup messages from Phase A, each knows the H.245 Control Channel Transport Address of the other endpoint.  Tthe endpoints shall then establish the H.245 Control Channel.  The procedures of H.245 are used over the H.245 Control Channel for the capability exchange and to open the media channels. Note: Optionally, the H.245 Control Channel may be set up by the called endpoint on receipt of Setup, and by the calling endpoint on receipt of Alerting or Call Proceeding. In the event that Connect does not arrive, or an endpoint sends Release Complete, the H.245 Control Channel shall be closed.

Endpoint system capabilities are exchanged by transmission of the H.245 terminalCapabilitySet message.  This capability message shall be the first H.245 message sent.

The Audio Channel may optionally be opened, using the H.245 open logical channel procedure, prior to capability exchange, providing the capability to have speech communications at the earliest possible opportunity and before proceeding to visual telephony.  If the Audio Channel is opened prior to the capability exchange, the audio channel shall use G.711 audio at 64 kbps.

The master/slave determination procedure of H.245 shall take place as described in Section 6.2.8.4.  In cases where both endpoints in a call have MC capability, the master slave determination is used for determining which MC will be the active MC for the conference. The active MC shall then send the  mcLocationIndication message. The procedure also provides master slave determination for opening bi-directional channels for data.

If the initial capability exchange or master/slave determination procedures fail, these should be retried at least two additional times before the terminal abandons the connection attempt and proceeds to phase E.  

Following this exchange of capabilities, the endpoints shall proceed directly to the desired operating mode i.e.  Phase C.

8.3 Phase C - Establishment of audiovisual communication

Following the exchange of capabilities and master slave determination, the procedures of H.245 shall then be used to open logical channels for the various information streams.  The audio and video streams, which are transmitted in the logical channels setup in H.245, are transported over dynamic TSAP Identifiers using an unreliable protocol (see H.225.0).  Audio and video streams require the use of two logical channels each, one for the media and one for the RTCP media control. Data communications which is transmitted in the logical channels setup in H.245, are transported over dynamic TSAP Identifiers using a reliable protocol (see H.225.0).

The openLogicalChannelAck returns the Transport Address that the receiving endpoint has assigned to that logical channel.  The transmitting channel shall then send the information stream associated with the logical channel to that Transport Address.

Following the opening of logical channels for audio and video, one h2250MaximumSkewIndication message shall be sent by the transmitter for each associated audio and video pairs.

8.3.1 Mode changes

During a session the procedures for changing channel structure, capability, receive mode etc.  shall be carried out as defined in Recommendation  H.245.

8.3.2 Exchange of video by mutual agreement

The indication, videoIndicateReadyToActivate, is defined in H.245.  Its use is optional, but when used the procedure shall be as follows.

Endpoint 1 has been set so that video is not transmitted unless, and until, endpoint 2 has also indicated readiness to transmit video.  Endpoint 1 shall send the indication videoIndicateReadyToActivate when the initial capability exchange has been completed, but shall not transmit a video signal until it has received either videoIndicateReadyToActivate or incoming video from endpoint 2.

An endpoint which has not been set in this optional way is not obliged to wait until receipt of videoIndicateReadyToActivate or video before initiating its video transmission.

8.3.3 Media Stream Address Distribution

In unicast, the endpoint shall open logical channels to the MCU or other endpoint. Addresses are passed in the openLogicalChannel and openLogicalChannelAck.

In multicast, the multicast addresses are assigned by the MC and distributed to the endpoints in the communicationsModeCommand. It is the responsibility of the MC to allocate and assign unique multicast addresses. The endpoint shall signal an open logical channel to the MC with a multicasst address in the openLogicalChannel. The MC shall forward the openLogicalChannel to each receiving endpoint. 

In multi-unicast, the endpoint must open logical channels to each of the other endpoints. The openLogicalChannel is sent to the MC and shall contain the terminal number of the endpoint for which the channel is intended. The endpoint can match a openLogicalChannelAck by the forwardLogicalChannelNumber.

8.4 Phase D: Call Services

8.4.1 Bandwidth Changes

Call bandwidth is initially established and approved by the Gatekeeper during the admissions exchange.  An endpoint shall assure that the aggregate for all transmitted and received audio and video channels, excluding any RTP headers, RTP payload headers, LAN headers, and other overhead, is within this bandwidth.  Data and control channels are not included in this limit.

At any time during a conference, the endpoints or Gatekeeper may request an increase or decrease in the call bandwidth.  An endpoint may change the bitrate of a logical channel without requesting a bandwidth change from the Gatekeeper if the aggregate bitrate of all transmitted and received channels does not exceed the current call bandwidth.  If the change will result in a aggregate bitrate that exceeds the current call bandwidth, the endpoint shall request a change in the call bandwidth from its Gatekeeper and await confirmation prior to actually increasing any bitrate.  A bandwidth change request is recommended when an endpoint will use a reduced bandwidth for an extended period of time, thus freeing up bandwidth for other calls.

An endpoint wishing to change its call bandwidth sends a Bandwidth Change Request (BRQ) message (1)to the Gatekeeper.  The Gatekeeper determines if the request is acceptable.  The criteria for this determination is outside the scope of this Recommendation.  If the Gatekeeper determines that the request is not acceptable, it returns a Bandwidth Change Reject (BRJ) message (2)  to endpoint.  If the Gatekeeper determines that the request is acceptable, it returns a Bandwidth Change Confirm (BCF) message (2).  

If endpoint 1 wishes to increase its transmitted bitrate on a logical channel, it first determines if the call bandwidth will be exceeded.  See Figure 24/H.323.  If it will, endpoint 1 shall request a bandwidth change (1 and 2) from Gatekeeper 1.  When the call bandwidth is sufficient to support the change, endpoint 1 sends a closeLogicalChannel (3) message to close the logical channel.  It then re-opens the logical channel using the openLogicalChannelRequest (4) specifying the new bitrate.  If the receiving endpoint wishes to accept the channel with the new bitrate, it must first assure that its call bandwidth is not exceeded by the change.  If it is, the endpoint shall request a call bandwidth change (5 and 6) with its Gatekeeper.  When the call bandwidth is sufficient to support the channel, the endpoint replies with an openLogicalChannelAck (7), otherwise, it responds with an openLogicalChannelReject indicating unacceptable bitrate.

�embed MSDraw \* mergeformat ���

Figure 24/H.323 Bandwidth Change Request - Transmitter Change

If the endpoint 1 wishes to increase its transmitted bitrate on a logical channel from endpoint 2 which it previously flow controlled to a lower bitrate, endpoint 1 first determines if the call bandwidth will be exceeded.  See Figure 25/H.323.  If it will, endpoint 1 shall request a bandwidth change from Gatekeeper 1.  When the call bandwidth is sufficient to support the change, endpoint 1 sends a flowControlCommand (3) to indicate the new upper limit on bitrate for the channel.  If endpoint 2 decides to increase the bitrate on the channel, it must first assure that its call bandwidth is not exceeded by the change.  If it is, endpoint 2 shall request a call bandwidth change (4 and 5) with its Gatekeeper.  When the call bandwidth is sufficient to support the channel, endpoint 2 will send the closeLogicalChannel (6) message to close the logical channel.  It then re-opens the logical channel using the openLogicalChannelRequest (7) specifying the new bitrate.  Endpoint 1 should then accept the channel with the new bitrate, and it replies with an openLogicalChannelAck (6). 

�embed MSDraw \* mergeformat ���

Figure 25/H.323 Bandwidth Change Request - Receiver Change

A Gatekeeper wishing to change the transmitted bitrate of endpoint 1 sends a BRQ message to endpoint 1.  If the request is for a decrease in bitrate, endpoint 1 shall always comply by reducing its aggregate bitrate and returning a BCF.  Endpoint 1 may initiate the appropriate H.245 signalling to inform endpoint 2 that bitrates have changed.  This will allow endpoint 2 to inform its Gatekeeper of the change.  If the request isf for an increase, the endpoint may increase its bitrate when desired.

8.4.2 Status

In order for the Gatekeeper to determine if an endpoint is turned off, or has otherwise enter a failure mode, the Gatekeeper may use the Information Request (IRQ) / Information Request Response (IRR) message sequence (see H.225.0) to poll the endpoints on active calls at an interval decided by the manufacturer.  The endpoint shall respond within 5 seconds, and the polling interval shall be greater than 10 seconds.  This message may also be used by a diagnostic device as described in Section 11.2.

The Gatekeeper may want an endpoint to periodically send an unrequested IRR message.  The Gatekeeper may indicate this to the endpoint by specifying the rate that this IRR is sent within the irrFrequency field of the Admission Confirm (ACF) message.  An endpoint receiving this irrFrequency rate shall send an IRR message at that rate for the duration of the call.  While this rate is in effect, the Gatekeeper may still send IRQ messages to the endpoint which shall respond as described above.

During the duration of a call, an endpoint or Gatekeeper may periodically request call status from another endpoint.  The requesting endpoint or Gatekeeper issues a Status Enquiry message.  The Endpoint receiving the Status Enquiry message shall respond with a Status message indicating the current call state.  This procedure may be used by the Gatekeeper in order to periodically check if a call is still active.  Note that this is a H.225.0Q.931 message sent on the Call Signalling Channel, and should not be confused with IRR which is a RAS message sent on the RAS Channel.

8.4.3 Ad Hoc Conference Expansion

The following procedures are optional for terminals and Gateways and mandatory for MCs.

An Ad Hoc Multipoint conference is one that can be expanded from a point-to-point conference involving  an MC to a multipoint conference. First, a point-to-point conference is created between two endpoints (endpoint 1 and endpoint 2). At least one endpoint, or the Gatekeeper must contain an MC. Once the point-to-point conference has been created, the conference  may be expanded to multipoint conference in two different ways.  The first way is when any endpoint in the conference invites another endpoint (endpoint 3) into the conference by calling that endpoint through the MC.  The second way is for an endpoint (endpoint 3) to join an existing conference by calling an endpoint in the conference.

Ad Hoc Conference expansion can take place when using either the Direct Call Signalling model or the Gatekeeper Routed Call Signalling model. The H.245 Control Channel topology for the Direct Call Signalling model appears as:

�EMBED Word.Picture.6���

The H.245 Control Channel topology for the Gatekeeper Routed Call Signaling model appears as:

�EMBED Word.Picture.6���

In either case an MC must be present in the conference at the time of expansion to any number greater than 2 endpoints. 

The procedures required to create a point-to-point conference and then expand the conference through invite and join, for each call model, is covered in the following sub-sections.

It should be noted that the call is ended by a failure of the entity that is providing the MC.

8.4.3.1 Direct Endpoint Call Signalling -  Conference Create

Endpoint 1 creates a conference with endpoint 2 as follows:

A1) Endpoint 1 sends a Setup message to endpoint 2 containing a globally unique CID = N and conferenceGoal = create according to the procedure in Section 8.1 

A2) Endpoint 2 has the following options:

A2a) If it wants to join the conference, it sends a Connect message with CID = N to endpoint 1. In this case it is either 1) not participating in another conference or 2) it is participating in another conference, it is capable of participating in multiple conferences at the same time, and the received CID = N does not match the CID of any of the conferences in which it is currently participating.

A2b) If it is in another conference with CID = M and can participate in only one conference at a time it either 1) rejects the call by sending Release Complete indicating in-conference or 2) it can request endpoint 1 to join the conference with CID = M by sending a Facility message indicating routeCallToMC with the Call Signalling Channel Transport Address of the endpoint containing the MC and CID = M of the conference.

A2c) If it does not wish to join this conference it rejects the call by sending Release Complete indicating destinationBusy.

A3) If endpoint 2 enters the conference, endpoint 1 uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 2.

A4) The H.245 messages are then exchanged as described below:

A4a) TerminalCapabilitySet messages are exchanged between the endpoints to determine the version number of the H.245 used in order to parse the remaining received messages correctly.

A4b) Using H.245 master/slave determination procedure, it is determined that Endpoint 2 is the master.  In the Gatekeeper-Routed model the master could be in the Gatekeeper’s MC. If the master has an MC, it becomes the Active MC. It may then send the MCLocationIndication to the other endpoint(s). The MC may be active in the conference now, or when the user initiates the multipoint conference function, at the choice of the manufacturer.

A4c) The master may send  the terminalNumberAssign message to the endpoints. The endpoints shall use the 8 bit terminal number, and not use the 8 bit MCU number, from the 16 bit number assigned  as the low 8 bits of the SSRC field in the RTP header. These low 8 bits in SSRC then identify the streams from a particular endpoint.

A4d) Since the capabilities of the receiver are known from the TerminalCapabilitySet message, the transmitter opens the logical channels. It shall send one h2250MaximumSkewIndication for each pair of audio and video transmitted.

8.4.3.2 Direct Endpoint Call Signalling -  Conference Invite

There are two cases of the conference invite.  First, the endpoint which contains the active MC wishes to invite another endpoint into the conference.  Second, an endpoint which does not contain the active MC wishes to invite another endpoint into the conference.

After a point-to-point conference has been established using procedures A1 to A4,  an endpoint (endpoint 2) containing the active MC wishing to add another endpoint to the conference, shall use the following procedure:

B1) Endpoint 2 sends a Setup message to endpoint 3 with CID = N and conferenceGoal = invite according to the procedures in Section 8.1. See Figure 26/H.323.

B2) Endpoint 3 has the following options:

B2a) If it wishes to accept the invitation to join the conference, it sends a Connect message with CID = N to endpoint 2.

B2b) If it wishes to reject the invitation to join the conference, it sends a Release Complete message indicating destinationBusy to endpoint 2.

B2c) If it is in another conference with CID = M, it can request endpoint 2 to join the conference with CID = M by sending a Facility message indicating routeCallToMC with the Call Signalling Channel Transport Address of the endpoint containing the MC and CID = M of the conference.

B2d) If the received CID matches the CID of a conference that endpoint 3 is currently participating in, it shall reject the call by sending Release Complete indicating that it is already in the conference.

B3) If endpoint 3 accepts the invitation, endpoint 2 uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 3.

B4) The H.245 messages are then exchanged as described below:

C1) TerminalCapabilitySet messages are exchanged between the MC and endpoint 3.

C2) Using H.245 master/slave determination procedure, it is determined that endpoint 2 is already the active MC.  The MC may then send the MCLocationIndication to the endpoint 3.

C3)The MC shall send multipointModeCommand at this time to all the three endpoints.

C4) The MC may send the terminalNumberAssign message to endpoint 3. If received, the endpoints shall use the 8 bit terminal number, and not use the 8 bit MCU number, from the 16 bit number assigned  as the low 8 bits of the SSRC field in the RTP header. These low 8 bits in SSRC then identify the streams from a particular endpoints.

C5) An endpoint can get the list of the other endpoints in the conference by sending the terminalListRequest message to the MC. The MC responds with the terminalListResponse.

C6) Whenever a new endpoint joins the conference, the MC sends the terminalNumberAssign message to endpoint 4 and terminalJoinedConference message to endpoints 1, 2, and 3.

C7) Whenever an endpoint leaves the conference, the MC sends terminalLeftConference to the remaining endpoints.

C8) The MC shall send the communicationModeCommand to all the endpoints in the conference.

C9) Endpoint 1 and endpoint 2 will close their logical channels that were created during the point-to-point conference if they are inconsistent with the information contained in the communicationModeCommand. 

C10) The logical channels can now be opened between the MC and the endpoints. 

�EMBED Word.Picture.6���

Figure 26/H.323 MC Invite Signalling

After a point-to-point conference has been established using procedures A1 to A4,  an endpoint (endpoint 1) that does not contain the active MC wishing to add another endpoint to the conference, shall use the following procedure:

B1)Endpoint 1 sends a Setup message to the MC (endpoint 2) with a new CRV indicating a call to endpoint 3 by providing the transport address of endpoint 3, CID = N, and conferenceGoal = invite.  See Figure 27/H.323.

B2) Endpoint 2 sends a Setup message to endpoint 3 with CID = N and conferenceGoal = invite according to the procedures in Section 8.1.

B3) During call signalling with endpoint 3, endpoint 2 shall pass Call Signalling messages received from endpoint 3, including Connect, to endpoint 1 (the original inviter).

B4) Endpoint 3 has the same options, described previously, of either accepting or rejecting the invitation.

B5) At some time after the completion of the call setup procedure between endpoint 2 and endpoint 3, endpoint 2 shall send a Release Complete message to endpoint 1.

B6) If endpoint 3 accepts the invitation, endpoint 2 uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 3.

B7) The H.245 messages are then exchanged as previously described in procedures C1 to C10.

�EMBED Word.Picture.6���

Figure 27/H.323 Non-MC Invite Signalling

8.4.3.3Direct Endpoint Call Signalling -  Conference Join

There are two cases of the conference join.  First, an endpoint calls the endpoint which contains the active MC.  Second, an endpoint calls an endpoint which is not the active MC.  

After a point-to-point conference has been established using procedures A1 to A4, an endpoint (endpoint 3) wishing to join a conference may attempt to connect with the endpoint containing the active MC in the conference.  In this case, the following procedure shall be used.

B1) Endpoint 3 sends a Setup message to endpoint 2 with CID = N, and conferenceGoal = join according to the procedure in Section 8.1.  See Figure 28/H.323.

B2) If the CID matched the CID of an active conference in the MC, endpoint  2 (MC) has the following options:

B2a) If it decides that endpoint 3 should be allowed to join the conference, it sends the Connect message with CID = N.

B2b) If it decides that endpoint 3 should not be allowed to join the conference, it sends the Release Complete message with destinationBusy.

B3) If the CID does not match the CID of an active conference in the MC, endpoint 2 shall send Release Complete indicating a bad CID.

B4) If endpoint 2 allows the join, endpoint 2 opens the Control Channel with endpoint 3.

B5) The H.245 messages are then exchanged as previously described in procedures C1 to C10.

�EMBED Word.Picture.6���



Figure 28/H.323 MC Join Signalling

After a point-to-point conference has been established using procedures A1 to A4, an endpoint (endpoint 3) wishing to join a conference may attempt to connect with an endpoint that does not contain the active MC in the conference.  In this case, the following procedure shall be used.

B1) Endpoint 3 sends a Setup message to endpoint 1 with CID = N, and conferenceGoal = join according to the procedure in Section 8.1.  See Figure 29/H.323.  

B2) Endpoint 1 returns a Facility message indicating routeCallToMC with the Call Signalling Channel Transport Address of endpoint 2 (containing the active MC) and the CID = N of the conference.

B3) Endpoint 3 then sends a Setup message to endpoint 2 (MC) with CID = N and conferenceGoal = join as described in the previous conference join procedure.

�EMBED Word.Picture.6���

Figure 29/H.323 Non-MC Join Signalling

8.4.3.4 Gatekeeper Routed Call Signalling - Conference Create

In cases where the Gatekeeper routes the Call Signalling Channel and the H.245 Control Channel, the Gatekeeper should contain (or have access to) an MC or MCU. Procedures A1 to A4  are used to establish the point-to-point call. During Master/Slave determination (A4b), if the Gatekeeper’s terminalType is greater than the terminalType received from an endpoint in the masterSlaveDetermination message, the Gatekeeper may replace the endpoint’s terminalType value with its own before forwarding the message to the destination endpoint. If the Gatekeepers terminalType is not greater than the terminaltype of the endpoint, the Gatekeeper shall not modify the terminalType value. In effect, the Gatekeeper is performing the master slave determination procedure with each endpoint. If the Gatekeeper wins the master slave determination with both endpoints, the MC associated with the Gatekeeper shall be the active MC, otherwise, one of the endpoints shall be the active MC.

8.4.3.5 Gatekeeper Routed Call Signaling - Conference Invite

After a point-to-point conference has been established using procedures A1 to A4 as modified above,  an endpoint (endpoint 1 or 2) that does not contain the active MC wishing to add another endpoint to the conference, shall use the following procedure:

B1)Endpoint 1 sends a Setup message through the Gatekeeper directed to endpoint 3 with a new CRV, CID = N, and conferenceGoal = invite.  See Figure 30/H.323.

B2) The Gatekeeper (MC) sends a Setup message to endpoint 3 with CID = N and conferenceGoal = invite according to the procedures in Section 8.1.

B3) During call signalling with endpoint 3, the Gatekeeper shall pass Call Signalling messages received from endpoint 3, including Connect, to endpoint 1 (the original inviter).

B4) Endpoint 3 has the same options, described previously, of either accepting or rejecting the invitation.

B5) At some time after the completion of the call setup procedure between the Gatekeeper and endpoint 3, the Gatekeeper shall send a Release Complete message to endpoint.

B6) If endpoint 3 accepts the invitation, the Gatekeeper uses the transport address of the Control Channel provided in the Connect message to open the Control Channel with endpoint 3.

B7) The H.245 messages are then exchanged as previously described in procedures C1 to C10 with the Gatekeeper taking part in all master slave determination procedures as the active MC (C2). At this time, the Control Channels from the endpoints should be connected to the MC, and the MC should be in control of the conference.



�EMBED Word.Picture.6���

Figure 30/H.323 Gatekeeper Routed invite Signalling

8.4.3.6 Gatekeeper Routed Call Model - Conference Join

After a point-to-point conference has been established using procedures A1 to A4 as modified above,  an endpoint (endpoint 3), wishing to join a conference may attempt to connect with an endpoint that does not contain the active MC in the conference.  In this case, the following procedure shall be used.

B1) Endpoint 3 sends a Setup message through the Gatekeeper directed to endpoint 1 with CID = N, and conferenceGoal = join according to the procedure in Section 8.1.  See Figure 31/H.323.

B2) If the CID matched the CID of an active conference in the MC, the Gatekeeper (MC) has the following options:

B2a) If it decides that endpoint 3 should be allowed to join the conference, it sends the Connect message with CID = N to endpoint 3.

B2b) If it decides that endpoint 3 should not be allowed to join the conference, it sends the Release Complete message with destinationBusy.

B2c) The Gatekeeper may forward the Setup message to endpoint 1. Endpoint 1 may respond with a Facility message indicating routeCallToMC or it it may respond with a release complete.

B3) If the CID does not match the CID of an active conference in the MC, the Gatekeeper shall send Release Complete indicating a bad CID.

B4) If the Gatekeeper allows the join, the Gatekeeper uses the transport address of the Control Channel provided in the Setup message to open the Control Channel with endpoint 3.

B5) The H.245 messages are then exchanged as previously described in procedures C1 to C10 with the Gatekeeper taking part in all master slave determination procedures as the active MC (C2). At this time, the Control Channels from the endpoints should be connected to the MC, and the MC should be in control of the conference.



�EMBED Word.Picture.6���

Figure 31/H.323 Gatekeeper Routed Join Signalling





8.4.3 Ad Hoc Conference Expansion

Ad Hoc Multipoint conferences may be expanded in two different ways.  The first way is when any endpoint in the conference invites another endpoint into an existing conference by calling that endpoint.  The second way is for an endpoint to join an existing conference by calling an endpoint in the conference.  It should be noted that the call is ended by a failure of the entity that is providing the MC.  In all cases an MC/MCU must be present in the conference at the time of expansion to any number greater than 2 endpoints.  

8.4.3.1 Direct Endpoint Call Signalling -  Conference Invite

There are two cases of the conference invite.  First, the endpoint which contains the active MC wishes to invite another endpoint into the conference.  The second case is one in which an endpoint which does not contain the active MC wishes to invite another endpoint into the conference.

All of the following procedures assume that an initial call was established as follows:

1) Endpoint 1 calls endpoint 2 with CID = 0 according to the procedure in Section 8.1 and endpoint 2 responds to endpoint 1 with the CID = N.

2) Using H.245 master/slave determination procedure, it is determined that Endpoint 2 is the master, and hence has the active MC.  This determination must occur before the expansion is initiated.  The MC shall send the MCLocationCommand to the other endpoint after this determination.

3) The MC may be active in the conference now, or when the user initiates the multipoint conference function, at the choice of the manufacturer.

In all cases the H.245 Control Channel topology appears as:

�embed Word.Picture.6 ���

If an endpoint containing the active MC wishes to add another endpoint to the conference, the following procedure is used:

4) Endpoint 2 calls endpoint 3 with CID = N according to the procedures in Section 8.1.  Endpoint 3’s H.245 Control Channel is connected to the MC in endpoint 2.  See Figure 26/H.323.

5) Using H.245 master/slave determination procedure, it is determined that endpoint 2 is the active MC, and hence remains the active MC.  The MC shall send the MCLocationCommand to the endpoint 3 after this determination.

6)The MC shall send multipointModeCommand at this time.

�embed Word.Picture.6 ���

Figure 26/H.323 MC Invite Signalling

If the endpoint that does not contain the active MC wishes to add another endpoint to the conference, the following procedure is used:

4)Endpoint 1 sends a Setup message to endpoint 2 indicating a call to endpoint 3 with CID = N.  See Figure 27/H.323.

5) Endpoint 2’s MC recognizes CID = N and the endpoint 3 destination.  

6) Endpoint 2 calls endpoint 3 with CID = N according to the procedures in Section 8.1.  Endpoint 3’s H.245 Control Channel is connected to the MC in endpoint 2.

7) During this call to endpoint 3, endpoint 2 may issue Call Proceeding and or Call Alerting messages to endpoint 1 (the original inviter).

8) At the completion of the call setup procedure between endpoint 2 and endpoint 3, endpoint 2 shall send a Release Complete message to endpoint 1 to close the Call Signalling Channel for the invitation.  (note that the Q.931 connection possibly existing from the original endpoint 1 - endpoint 2  call may be considered completely separate).  Thus, endpoint 2 remains at the center of the conference.  In effect it is acting as a dial-out MCU.

9) Using H.245 master/slave determination procedure, it is determined that endpoint 2 is the active MC, and hence remains the active MC.  The MC shall send the MCLocationCommand to the endpoint 3 after this determination.

10)The MC shall send multipointModeCommand at this time.

�embed Word.Picture.6 ���

Figure 27/H.323 Non-MC Invite Signalling

8.4.3.2 Direct Endpoint Call Signalling -  Conference Join

There are three cases of the conference join.  First, an endpoint calls the endpoint which contains the active MC.  The second case is one in which an endpoint calls an endpoint which is not the active MC.  In the third case, an endpoint unknowingly calls another endpoint which is already involved in a conference.

All of the following procedures assume that an initial call was established following steps 1 thru 3 of Section 8.4.3.2.

In all cases, the H.245 Control Channel topology appears as:

�embed Word.Picture.6 ���

If an endpoint attempts to connect with the endpoint containing the active MC in the conference  the standard connect procedures in Section 8.1 are followed.

4) Endpoint 3 calls endpoint 2 with CID = N according to the procedure in Section 8.1 and endpoint 2 responds to endpoint 3.  See Figure 28/H.323.

5) Using H.245 master/slave determination procedure, it is determined that endpoint 2 is the active MC, and hence remains the active MC.  The MC shall send the MCLocationCommand to the endpoint 3 after this determination.

6)The MC shall send multipointModeCommand at this time.

�embed Word.Picture.6 ���



Figure 28/H.323 MC Join Signalling

If an endpoint attempts to connect with an endpoint that does not contain the active MC in the conference,  the following connect procedures are followed.

4) Endpoint 3 attempts to call endpoint 1 with CID = N according to the procedure in Section 8.1.  See Figure 29/H.323.  

5) Endpoint 1 returns a ReleaseComplete with CID = N and the Call Signalling Channel Transport Address of endpoint 2 (containing the active MC).

6) Endpoint 3 then calls endpoint 2 with CID = N according to the procedure in Section 8.1.

7) Using H.245 master/slave determination procedure, it is determined that endpoint 2 is the active MC, and hence remains the active MC.  The MC shall send the MCLocationCommand to the endpoint 3 after this determination.

8)The MC shall send multipointModeCommand at this time.

�embed Word.Picture.6 ���

Figure 29/H.323 Non-MC Join Signalling

If an endpoint attempts to connect with another endpoint, not knowing that the endpoint is already in a conference,  the following connect procedures are followed.

4) Endpoint 3 attempts to call endpoint 1 or 2 with CID = 0 according to the procedure in Section 8.1.  See Figure 30/H.323.  

5) If endpoint 1 or 2 wishes to allow endpoint 3 to join the conference, it returns a ReleaseComplete with CID = N and the Call Signalling Channel Transport Address of endpoint 2 (containing the active MC).

6) If Endpoint 3 wishes to join the existing conference, it then calls endpoint 2 with CID = N according to the procedure in Section 8.1.

7) Using H.245 master/slave determination procedure, it is determined that endpoint 2 is the active MC, and hence remains the active MC.  The MC shall send the MCLocationCommand to the endpoint 3 after this determination.

8)The MC shall send multipointModeCommand at this time.

�embed Word.Picture.6 ���

Figure 30/H.323 Non-MC Join Signalling

8.4.3.3 Gatekeeper Routed Call Signalling

In cases where the Gatekeeper routes the Call Signalling Channel, the Gatekeeper should contain (or have access to) an MC or MCU.  The Gatekeeper shall not win the Master/Slave determination if it does not contain this functionality (or have access to it by some proprietary means).  An example of a Gatekeeper routed Call Signalling Channel for the conference invite is shown below.  In this case, endpoint 1 attempts to add endpoint 3 to create a 3-point conference.  Endpoint 1 and endpoint 2 are already connected in the conference through the MC in the Gatekeeper.

�embed Word.Picture.6 ���

An example of a Gatekeeper routed Call Signalling Channel for the conference join is shown below.  In this case, endpoint 3 attempts to join an existing conference.  Endpoint 1 and endpoint 2 are already connected in the conference through the MC in the Gatekeeper.

�embed Word.Picture.6 ���



In all cases, the H.245 Control Channel topology appears as:

�embed Word.Picture.6 ���

8.4.4 Supplementary Services

Provisions have been made within this Recommendation to allow endpoints to the use the Supplementary Services as defined in Q.931, Q.932, and the Q.95X series of Recommendations.  These services shall use the Call Signalling Channel andwithin Q.931 messages.  These services may provide features such as hold, transfer, forward, and redirection.

Other methods of providing supplementary services are for further study.

8.5 Phase E: Call termination

Either endpoint may terminate a call by the following procedure:

1)	It shouldshall discontinue transmission of video at the end of a complete picture, and then close all logical channels for video.

2)	It shouldshall discontinue transmission of data and then close all logical channels for data.

3)	It shouldshall discontinue transmission of audio and then close all logical channels for audio.

4)	It shall transmit the H.245 endSessionCommand message in the H.245 Control Channel, indicating to the far end that it wishes to disconnect the callline and then discontinue H.245 message transmission.  

5)	It shall then wait to receive the endSessionCommand message from the other endpoint and then shall close the H.245 Control Channel.

56)	If the Call Signalling Channel is open, a Release Complete message shall be sent and the channel closed.  Optionally, a Release message may be sent which requires a Release Complete in response.  This channel may have been left open since call setup, or it may be re-opened for call clearing.

67)	It shall clear the call by using the procedures defined below.

A endpoint receiving endSessionCommand without first having transmitted it, shall carry out steps 1) to 57) above, except that in step 45, it shall not wait forsend the endSessionCommand fromback to the first endpoint.

Terminating a call may not terminate a conference; a conference may be explicitly terminated using H.245 messages. In this case, the endpoints shall wait for the MC to terminate the calls as described above.

8.5.1 Call Clearing without a Gatekeeper

In networks that do not contain a Gatekeeper, after steps 1 to 6 abovetransmitting the endSessionCommand, the call is terminated.  No further action is required.

8.5.2 Call Clearing with a Gatekeeper

In networks that contain a Gatekeeper, the Gatekeeper needs to know about the release of bandwidth.  After performing steps 1 to 6 abovetransmitting the endSessionCommand (1) and if necessary, the Release Complete message (2), each endpoint shall transmit an H.225.0 Disengage Request (DRQ) message (3) to its Gatekeeper.  The Gatekeeper shall respond with a Disengage Confirm (DCF) message (4).  After sending the DRQ message, the endpoints shall not send further unsolicited IRR messages to the Gatekeeper.  See Figure 321/H.323.  At this point the call is terminated. This figure shows the direct call model, a similar procedure is followed for the Gatekeeper routed model.

The DRQ and DCF messages shall be sent on the RAS Channel.



�embed MSDraw \* mergeformat ����

Figure 321/H.323 Endpoint Initiated Call Clearing

8.5.3 Call Clearing by Gatekeeper

The Gatekeeper may terminate any callconference by sending a DRQ to an endpoint.  See Figure 332/H.323.  The endpoint shall immediately follow steps 1 through 56 from above and then reply to the Gatekeeper with DCF.  The other endpoint, upon receiving endSessionCommand shall follow the procedure described above. This figure shows the direct call model, a similar procedure is followed for the Gatekeeper routed model.

If the conference is a multipoint conference, the Gatekeeper shouldmust sendt a DRQ tofor each point-to-endpoint in the conferenceconnection to the MC, in order to close the entire conference.

�embed MSDraw \* mergeformat ����

Figure 332/H.323 Gatekeeper Initiated Call Clearing



8.6 Protocol Failure Handling

The underlying reliable protocol of the H.245 Control Channel uses appropriate effort to deliver or receive data on the channel before reporting a protocol failure.  Therefore, if a protocol failure is reported on the channel, the H.245 Control Channel, and all associated logical channels shall be closed.  This shall be done following the procedures of Phase E, as if the other endpoint had issued the H.245 endSessionCommand.  This includes transmission of the DRQ message to the Gatekeeper and termination of the Call Signalling Channel.  In the case where the failure is detected by the MC in a multipoint conference, the MC shall send terminalLeftConference messages to the remaining terminals.  It is up to the implementation whether or not to try to re-establish the call without user intervention.  In any case, this would appear to the other endpoint (and the Gatekeeper) as a new call.

The Call Signalling Channel also uses an underlying reliable protocol.  Depending on the routing of the Call Signalling Channel, either the Gatekeeper or an endpoint may detect the protocol failure.  If the Gatekeeper detects the failure, it shall attempt to re-establish the Call Control Channel.  This implies that the endpoint shall always have the ability to establish a channel on its Call Signalling Channel Transport Address.  Failure of the Call Signalling channel shall not change the Q.931 call state.  After re-establishment of the Call Signalling Channel, the Gatekeeper may send a Status message to request the call state of the endpoint to assure that they are in synchronization.

If the endpoint detects the failure, the endpoint may choose to terminate the call as described in Phase E, or it may attempt to re-establish the Call Signalling Channel as described above.

If during a call, an endpoint wants to determine if the other endpoint is still functioning and connected, is may send the H.245 roundTripDelayRequest.  Since H.245 Control Channel is carried on a reliable channel, this will result in a response from the other endpoint or an error from the transport interface.  In the latter case, the procedures described above shall be used.  An endpoint in a multipoint conference may use the same mechanism; however it will learn only whether it still has a connection to the MC.  Note that it is possible for an endpoint to have an error-free connection with the MC but still be receiving no audio or video from the rest of the terminals in the conference.

9 Interoperation with other terminal types

Interoperation with other terminals shall be accomplished through the Gateway.  See Section 6.3.

9.1 Speech only terminals

Interoperation with speech only terminals (telephony) over the ISDN or GSTN can be provided by:

	1) using a H.323-ISDN speech Gateway.

	2) using a H.323-GSTN speech Gateway.



The Gateway shouldmust consider the following issues:

- Audio code conversion.

	ISDN: If desired, since ISDN uses G.711

	GSTN: from analog to G.711

- Bitstream conversion.

	ISDN: H.225.0 to/from unframed

	GSTN: generate H.225.0

- Control conversion.  (Generate H.245)

- Call Control Signalling conversion.

- DTMF tone conversion to/from H.245 userInputIndication message.



9.2 Visual telephone terminals over the ISDN (H.320)

Interoperation with visual telephone terminals over the ISDN (H.320) can be provided by:

1) using a H.323-H.320 Gateway.



The Gateway shouldmust consider the following issues:

- Video format conversion.  (if desired, H.261 is mandatory for both terminal types.)

- Audio code conversion.  (if desired, G.711 is mandatory for both terminal types.)

- Data protocol conversion.

- Bitstream conversion.  (H.225.0 to/from H.221)

- Control conversion.  (H.245 to/from H.242)

- Call Control Signalling conversion.

- SBE Number conversion to/from H.245 userInputIndication message.





9.3 Visual telephone terminals over GSTN (H.324)

Interoperation with visual telephone terminals over the GSTN (H.324) can be provided by two methods:

1) using a H.323-H.324 Gateway.

2) using a H.323-H.320 Gateway assuming that there exists an ISDN/GSTN Interworking Unit in the network.



The Gateway shouldmust consider the following issues:

- Video format conversion.  (if desired, H.261 is mandatory for both terminal types.)

- Audio code conversion.  (G.711 is mandatory for H.323 terminal, G.723 is mandatory for H.324 terminal.)

- Data protocol conversion.

- Bitstream conversion.  (H.225.0 to/from H.223)

- Call Control Signalling conversion.



9.4 Visual telephone terminals over Mobile Radio (H.324/M)

For further study.

9.5 Visual telephone terminals over ATM (H.321)

Interoperation with visual telephone terminals over ATM networks (H.321) can be provided by two methods:

1) using a H.323-H.321 Gateway.

2) using a H.323-H.320 Gateway assuming that there exists an I.580 ISDN/ATM Interworking Unit in the network.



The Gateway shouldmust consider the following issues:

- Video format conversion.  (if desired, H.261 is mandatory for both terminal types.)

- Audio code conversion.  (if desired.  G.711 is mandatory for both terminal types.)

- Data protocol conversion.

- Bitstream conversion.  (H.225.0 to/from H.221)

- Control conversion.  (H.245 to/from H.242)

- Call Control Signalling conversion.



9.6 Visual telephone terminals over Guaranteed Quality of Service LANs (H.322)

Interoperation with visual telephone terminals over Guaranteed Quality of Service LANs (H.322) can be provided by:

1) using a H.323-H.320 Gateway assuming that there exists a GQOS LAN-ISDN Gateway in the network.



The Gateway shouldmust consider the following issues:

- Video format conversion.  (if desired, H.261 is mandatory for both terminal types.)

- Audio code conversion.  (if desired, G.711 is mandatory for both terminal types)

- Data protocol conversion.

- Bitstream conversion.  (H.225.0 to/from H.221)

- Control conversion.  (H.245 to/from H.242)

- Call Control Signalling conversion.



9.7 Simultaneous Voice and Data Terminals over GSTN (V.70)

Interoperation with Simultaneous Voice and Data Terminals over GSTN (V.70) can be provided by:

1) using a H.323-V.70 Gateway.



The Gateway shouldmust consider the following issues:

- Audio code conversion.  (G.711 to/from G.729 Annex A)

- Data protocol conversion.

- Bitstream conversion.  (H.225.0 to/from V.76/V.75)

- Control conversion.  (both terminals use H.245)

- Call Control Signalling conversion.



9.8 T.120 Terminals on the LAN

An H.323 terminal that has T.120 capability should be capable of being configured as a T.120 Only terminal which listens and transmits on the standard T.120 well known TSAP Identifier.  This will allow the T.120 capable H.323 terminal to participate in T.120 only conferences.

A T.120 only terminal on the LAN shall be able to participate in the T.120 portion of multipoint H.323 conferences by connecting to the MCS MCU. See Section 6.2.7.1.

10	Optional enhancements

10.1	Encryption

For further study.

Encryption may optionally be used by H.323 terminals.  Encryption, including selection of algorithm and key exchange, shall be according to the procedures of Recommendations H.233 and H.234 with the following modifications to the procedures defined in H.233.  The ability to support encryption shall be signaled by the presence of the h233EncryptionTransmitCapability and h233EncryptionReceiveCapability parameters of the capability message of H.245.

In H.233, specific reference is made to H.221 in describing how encryption takes place.  In applying H.233 to H.323 terminals, references to H.221, FAS, and BAS channels therein shall be ignored and appropriate substitute recommendations be taken from this section.  Messages referred to as carried in the H.221 ECS channel shall be re-interpreted as being carried within the encryptionSE parameter of the H.245 encryptionCommand or encryption initialization vector (EIV) logical channel, as specified below.

10.1.1	EncryptionSE messages

H.233 session exchange (SE) messages shall be carried in the encryptionSE parameter of the H.245 encryptionCommand message.  Since the H.245 Control Channel is carried on a reliable data link layer using retransmission of errored frames, the error protection bits described in H.233 shall not be applied to SE messages.

If the 8th bit is set in the media identifier, the following encodings are indicated.  The normal tuple of 3 bytes containing the media identifier, algorithm identifier, and parameter identifier (each with one byte respectively) will be replaced by a tuple of 5 bytes containing media identifier, algorithm identifier, parameter identifier, and a 16 bit channel identifier.

NOTE - Non-standard encryption algorithms may be referenced in SE messages after associating a non-standard algorithm with a H.233 Algorithm Identifier value using the encryptionAlgorithmID parameter of the encryptionCommand message.

10.1.2	Encryption initialization vector (EIV) channel

The encryption initialization vector (EIV) logical channel is used for the transmission of H.233 initialization vector (IV) messages.  

To ensure accurate synchronization of the IV messages with the H.225.0 media streams, the EIV channel is an independent logical channel which shall be non-segmentable and use a reliable transport mechanism.  The entire IV message, exactly as defined in H.233, including error protection bits, shall be placed in a single H.225.0 message.

Messages carried within the EIV channel shall retain the error protection mechanism of H.233.

10.1.3	Encryption procedure

The encryptor shall produce an independent pseudo-random bitstream (cipher stream) for each media stream being encrypted.

When encryption is activated according to H.233, the media stream shall be exclusive-ORed with the pseudo-random bitstream generated by the encryptor.  However, the exclusive-OR procedure shall not be applied to the RTP header octets and all octets belonging to the H.245 Control Channel or EIV channel, which shall all be output unencrypted.

For each transmitted RTP header octet, eight bits shall be discarded from the pseudo-random bitstream generated by the encryptor.  The receiver shall apply the reverse procedure.

10.1.4	Encryption initialization vectors

Once an encrypted session is in progress the transmitter should periodically send new IV messages in order to limit the duration of repeated pseudo-random bitstreams in the event of a collision with a previously used state of the pseudo-random bitstream generator.  The frequency of these messages is left to the discretion of the implementor.

The new initialization vectors (IVs)  will be synchronized with the media packets utilizing RTP sequence numbers.  This implies having the H.233 implementation extract this information from the RTP header in order to provide the encryption synchronization.

10.1.5	Error recovery

In the event that the receiver suspects that it has lost encryption synchronization it shall send an encryptionIVrequest command, except that it should not re-send such commands at intervals less than the maximum expected round-trip response time.

Upon receipt of an encryptionIVrequest command, the transmitter shall, at its earliest opportunity, send a new IV message, except that it should ignore encryptionIVrequest commands received within the minimum expected round-trip response time since sending the last IV message.

11	Maintenance 

11.1	Loopbacks for maintenance purposes

Some loopback functions are defined in H.245 to allow verification of some functional aspects of the terminal, to ensure correct operation of the system and satisfactory quality of the service to the remote party.

The systemLoop request and logicalChannelLoop request shall not be used.  The mediaLoop request is optional.  An endpoint that receives the maintenanceLoopOffCommand shall turn off all loopbacks currently in effect.

For the purpose of loopbacks, two modes are defined:

a)	Normal operation mode: no loopback.  Indicated in (a) of Figure 343/H.323.  This shall be the default mode, and the mode entered when the maintenanceLoopOffCommand is received.

b)	Media loop mode: loopback of media stream at the analog I/O interface: Upon receiving the mediaLoop request as defined in H.245, loopback of the content of the selected logical channel shall be activated as close as possible to the analog interface of the video/audio codec towards the video/audio codec, so that decoded and re-coded media content is looped, as indicated in (b) Figure 343/H.323/H.324.  This loopback is optional.  It should be used only when a single logical channel containing the same media type is opened in each direction.  Operation when multiple channels are opened in the return direction is undefined.

�embed Word.Picture.6 ���

FIGURE 343/H.323 - Loop back

A Gateway to H.324/H.310, which receives an H.245 systemLoop request, H.245 logicalChannelLoop request, or a Gateway to H.320, which receives an H.230 Dig-Loop command from an SCN endpoint may perform the appropriate loopback function within the Gateway.  The Gateway shall not pass these requests to the LAN endpoint.  A Gateway to H.324/H.310, receiving H.245 mediaLoop from an SCN endpoint shall pass the request to the LAN endpoint.  A Gateway  to H.320, receiving H.230 Vid-loop or Au-loop command from an SCN endpoint shall convert it to the appropriate H.245 mediaLoop request and send it to the LAN endpoint.

A Gateway to H.320, which receives an H.245 mediaLoop request from a LAN endpoint shall convert it to the appropriate H.230 Vid-loop or Au-loop command and send it to the SCN endpoint.

A Gateway to H.324/H.310, may send an H.245 systemLoop request or H.245 logicalChannelLoop request to the SCN endpoint.  A Gateway to H.320, may send an H.230 Dig-Loop command to the SCN endpoint.  If a LAN endpoint is in a call to the SCN endpoint, the audio and video sent to the LAN endpoint may be the looped back audio or video, pre-recorded audio or video message indicating the loopback condition, or no audio or video.

11.2  Monitoring Methods

All terminals shall support the Information Request/Information Request Response (IRQ/IRR) message of H.225.0.  The Information Request Response message contains the TSAP Identifier of all channels currently active on the call, including T.120 and H.245 control, as well as audio and video.  This information can be used by third party maintenance devices to monitor H.323 conferences to  verify system operation.  



�ANNEX A

H.245 MESSAGES USED BY H.323 ENDPOINTS

The following rules apply to the use of H.245 messages by H.323 endpoints:

An endpoint shall not malfunction or otherwise be adversely affected by receiving H.245 messages that it does not recognize.  An endpoint receiving an unrecognized request, response, or command shall return  “function not supported”.  (This is not required for indications)

The following abbreviations are used in the table: M = Mandatory, O = Optional, F = Forbidden to transmit.

A message marked as mandatory for the receiving endpoint indicates that the endpoint shall accept the message and take the appropriate action. A message marked as mandatory for the transmitting endpoint indicates that the endpoint shall generate the message under the appropriate circumstances.

A.  Master Slave Determination Messages



Message�Receiving Endpoint 

Status �Transmitting Endpoint

Status��Determination�M�M��Determination Acknowledge�M�M��Determination Reject�M�M��Determination Release�M�M��

B.  Terminal Capability Messages



Message�Receiving Endpoint 

Status �Transmitting Endpoint

Status��Capability Set�M�M��Capability Set Acknowledge�M�M��Capability Set Reject�M�M��Capability Set Release�M�M��

C.  Logical Channel Signalling Messages



Message�Receiving Endpoint 

Status �Transmitting Endpoint

Status��Open Logical Channel�M�M��Open Logical Channel Acknowledge�M�M��Open Logical Channel Reject�M�M��Open Logical Channel Confirm�M�M������Close Logical Channel�M�M��Close Logical Channel Acknowledge�M�M������Request Channel Close�M�O��Request Channel Close Acknowledge�MO�O��Request Channel Close Reject�MO�M��Request Channel Close Release�M�O��

D.  Multiplex Table Signalling Messages



Message�Status��Multiplex Entry Send�F��Multiplex Entry Send Acknowledge�F��Multiplex Entry Send Reject�F��Multiplex Entry Send Release�F��

E.  Request Multiplex Table Signalling Messages



Message�Status��Request Multiplex Entry�F��Request Multiplex Entry Acknowledge�F��Request Multiplex Entry Reject�F��Request Multiplex Entry Release�F��

F.  Request Mode Messages



Message�Receiving Endpoint Status�Transmitting Endpoint Status ��Request Mode�M�O��Request Mode Acknowledge�O�O��Request Mode Reject�O�M��Request Mode Release�O�O��

G.  Round Trip Delay messages



Message�Receiving Endpoint Status�Transmitting Endpoint Status ��Round Trip Delay Request�O�O��Round Trip Delay Response�O�O��

H.  Maintenance Loop Messages



Message�Receiving Endpoint Status�Transmitting Endpoint Status ��Maintenance Loop Request����	System Loop�F�F��	Media Loop�O(Note 1)�O(Note 1)��	Logical Channel Loop�F�F��Maintenance Loop Acknowledge�O�O��Maintenance Loop Reject�O�M��Maintenance Loop Command Off�M�O��Note 1: Mandatory in Gateways.

I.  Commands



Message�Receiving Endpoint Status�Transmitting Endpoint Status ��Send Terminal Capability Set�M�M��Encryption�O�O��Flow Control�M�O��End Session�M�M��Miscellaneous Commands����	Equalize Delay�O�O��	Zero Delay�O�O��	Multipoint Mode Command�M�O ��	Cancel Multipoint Mode Command�M�O ��	Video Freeze Picture�M�O��	Video Fast Update Picture�M�O��	Video Fast Update GOB�M�O��	Video Fast Update MB�M�O��	Video Temporal Spatial Trade Off�O�O��	Video Send Sync Every GOB�O�O��	Video Send Sync Every GOB Cancel�O�O��MCLocationIndicationCommand�MO�O������	Terminal ID Request�O�O��	Terminal List Request�O�O��	broadcast me�O�O��	cancel Broadcast Me�O�O��	Make Terminal Broadcaster�O�O��	Send This Source�O�O��	Cancel Send This Source�O�O��	Drop Terminal�O�O��	Make Me Chair�O�O��	Cancel Make Me Chair�O�O��	Drop Conference�O�O��	Enter H.243 Password�O�O��	Enter H.243 Terminal Id�O�O��	Enter H.243 Conference ID�O�O��	Request Terminal ID�O�O��	Terminal ID Response�O�O��	Terminal List Response�O�O��	Video Command Reject�O�O��	Make ME Chair  Response�O�O��

J.  Conference Mode Commands



Message�Receiving Endpoint Status�Transmitting Endpoint Status ��Communication Mode Command�M�OM��Communication Mode Request�OM�OM��Communication Mode Response�OM�OM��

K.  Indications



Message�Receiving Endpoint Status�Transmitting Endpoint Status ��Function Not Supported�M�M��Miscellaneous Indication����	Logical Channel Active�OM�OM��	Logical Channel Inactive�OM�OM��	Multipoint Conference �M�OM��	Cancel  Multipoint Conference�M�OM��	Multipoint Zero Comm�OM�OM��	Cancel Multipoint Zero Comm�OM�OM��	Multipoint Secondary Status�OM�OM��	Cancel Multipoint Secondary Status�OM�OM��	Video Indicate Ready to Activate�OM�O��	Video Temporal Spatial Trade Off�OM�MO��	SBE Number�O�O��	Terminal Number Assign�MO�O��	Terminal Joined Conference�O�O��	Terminal Left Conference�O�O��	Seen By At Least One Other�O�O��	Cancel Seen By At Least One Other�O�O��	Seen By All�O�O��	Cancel Seen By All�O�O��	Terminal You Are Seeing�O�O��	Request For Floor�O�O������Jitter Indication�O�O��H.223 Skew Indication�F�F��H2250MaximumSkewIndication�O�M��New ATM Virtual Channel Indication�F�F��User Input�M (for 0-9, * and #)�M (for 0-9, * and #)��

non-standard commands, requests, etc are allowed.

�APPENDIX A

INFORMATIVE

PROCESSING OF Q.931 MESSAGES IN GATEWAYS



The Gateway shall terminate the Q.931 Call Signalling Channel between an H.323 endpoint and the Gateway on one hand and the call signaling channel (if any) between the Gateway and the SCN endpoint on the other.  The following applies only if the SCN side supports a call signalling protocol such as Q.931 or Q.2931.



The Gateway shall conform to the call signalling procedures recommended for the SCN side independent from the LAN side. The Gateway shall conform to the call signalling procedures of this Recommendation for the LAN side independent from the SCN side.



In addition, call signalling messages received from one side (LAN/SCN) may require forwarding to the other side (SCN/LAN). Some forwarded messages may contain information elements or parts of information elements which are unmodified or uninterpreted by the Gateway.  Other forwarded messages may contain inforation elements or parts of information elements may be added or removed  by the Gateway, as needed.



In the following, an overview of the actions to be taken by the gateway in response to Q.931 messages and the information elements, is provided.  Messages and information elements that are forbidden in H.225.0 are not considered.  Messages for supplementary services shall not be forwarded.



Q.931 messages originating on the H.323 side:



·	A SETUP message side shall lead to initiation of call setup procedure for the SCN side.

·	A RELEASE COMPLETE shall lead to initiation of the call disconnect as defined for the SCN side.

·	A CALL PROCEEDING message shall be forwarded to the SCN side.  This shall not be done if a CALL PROCEEDING has been sent before to the SCN in compliance to the respective SCN specification (Q.931 in the ISDN case).

·	A CONNECT message shall be forwarded to the SCN side upon receipt from an H.323 endpoint.  This message shall not be generated autonomously by the gateway.

·	A CONNECT ACKNOWLEDGE message shall be forwarded to the SCN side upon receipt.  This shall not be done if CONNECT ACKNOWLEDGE has been sent before to the SCN in compliance to the respective SCN specification.  If the gateway has sent a CONNECT to an H.323 endpoint and has not received the corresponding CONNECT ACKNOWLEDGE from the H.323 endpoint within the time frame required for successful completion of the call, it shall generate the CONNECT ACKNOWLEDGE to the SCN side as appropriate to comply to the SCN call setup procedures.

The INFORMATION message shall be forwarded to the SCN side if the contents of  the INFORMATION message indicate that it is destined for the SCN side rather than for the Gateway itself.

·	Messages for supplementary services (FACILITY, HOLD, HOLD ACKNOWELDGE, HOLD REJECT, RETRIEVE, RETRIEVE ACKNOWLEDGE, RETRIEVE REJECT) and the INFORMATION messages that are not processed by the Gateway, shall be forwarded to the SCN side.

·	All messages forbidden to be originated from an H.323 endpoint shall be generated by the Gateway autonomously as required by the SCN protocol.

The information elements of the respective messages are to be converted as follows:



·	The contents of connection specific information elements (such as Call Reference Value) shall be adapted as required by the SCN protocol.

·	Information elements that are not in use on the H.323 side shall be generated by the Gateway as required by the SCN protocol.

·	Translation of other information elements shall be done as required by the SCN protocols and procedures.  Where interoperability is not an issue, conversion is left to the discretion of the manufacturer.

·	Only the user-data part of the user-user information element shall be forwarded to the SCN side.  It shall be re-encoded following Figure 4-36/Q.931 and Table 4-26/Q.931.



All Q.931 messages originating on the SCN side are forwarded to the H.323 endpoint without modification except for the following:



·	Messages forbidden by H.225.0 shall not be passed to the H.323 side.

·	The call reference value is mapped to the appropriate value for the H.323 side.

·	The user data field is copied into the corresponding ASN.1 user-user information element structure.

·	The user-user information element structure shall be generated according to the specification in H.225.0.













�page  �50v�





Page �page �50v�

Draft H.323, 22 May 30 Jan 1996








