| INTERNATIONAL TELECOMMUNICATION UNION |  | NGN-GSI |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 454 (NGN-GSI) |
|  |  | English only Original: English |
| Question(s): | 12/11 | Geneva, 6-16 September 2010 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Q.12/11 Rapporteur |  |
| Title: | Output of Q.nid-test-arch, “Testing architecture for tag-based identification systems and functions” |  |

This output has been made as an update of the draft Recommendation, Q.nid-test-arch, “Testing architecture for tag-based identification systems and functions,” based on agreements reached as follows regarding the contribution, NGN-GSI C 714 submitted to and discussed at Q.12/11 during its meeting in Geneva, 6 – 16 September 2010.

| C. No. | Contribution title and proposals | Agreements |
| --- | --- | --- |
| C 714 | Proposal for update of Clause 9 of Q.nid-test-arch |  |
|  | - Proposed that the contributed text of Clause 9 be adopted into Q.nid-test-arch. | - Accepted with a few typographical corrections and addition of a note. |

Draft Recommendation ITU-T Q.nid-test-arch

Testing architecture for tag-based identification systems and functions

Summary

[Editor’s note] the summary will be written after the skeleton of this Recommendation are prepared and agreed.

Keywords

Identifier; Tag-based identification; RFID; Bar code; ID tag; ID terminal; and Identifier resolution

Introduction

[Editor’s note] the introduction will be written when the contents of this Recommendation are finalized because it will depend on the contents.

Table of Contents

Page

1	Scope	5

2	References	5

3	Definitions	6

3.1	Terms defined elsewhere	6

3.2	Terms defined in this Recommendation	7

4	Abbreviations and acronyms	7

5	Conventions	8

6	Compatibility issues	8

7	Overview of tag-based identification applications and services	8

7.1	Application and service use cases	8

7.2	Reference service architectures	9

8	Classification of tag-based identification systems and functions to be tested	10

8.1	Classification of tag-based identification systems to be tested	10

8.1.1	ID terminals	11

8.1.2	Identifier resolution systems	11

8.1.3	OID resolution systems	11

8.1.4	Service broker	11

8.2	Classification of tag-based identification functions to be tested	11

8.2.1	Application functions	11

8.2.2	Service functions	12

8.2.3	Network service functions	12

8.3	Conformance of tag-based identification functions to tag-based identification systems to be tested	13

9	Testing architecture for tag-based identification systems and functions	13

9.1	Purpose of model network	13

9.2	Model network for testing	13

9.3	Testing procedure	14

9.3.1	Service functions testing process	14

9.3.2	Network service functions testing process	14

9.4	Testing requirements	15

9.4.1	Model network configuration requirements	15

9.4.2	Methodology of model networks testing	16

Bibliography	17

Draft Recommendation ITU-T Q.nid-test-arch

Testing architecture for tag-based identification systems and functions

# Scope

A set of standards and relevant implementation systems are necessary to enable tag-based identification applications and services over the NGN and other communications networks. The implementation systems are recommended to be verified according to implemented standards to support their conformance and interoperability.

This Recommendation specifies the testing architecture that describes target systems, system configurations in terms of model network, general procedures and testing requirements.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T F.771]	Recommendation ITU-T F.771 (2008), Service description and requirements for multimedia information access triggered by tag-based identification

[ITU-T H.621]	Recommendation ITU-T H.621 (2008), Architecture of a system for multimedia information access triggered by tag-based identification

[ITU-T Y.2016]	Recommendation ITU-T Y.2016 (2009), Functional requirements and architecture of the NGN for applications and services using tag-based identification

[ITU-T Y.2213]	Recommendation ITU-T Y.2213 (2008), NGN service requirements and capabilities for network aspects of applications and services using tag-based identification

[ITU-T H.IRP]	Draft Recommendation ITU-T H.IRP, ID resolution protocols for multimedia information access triggered by tag-based identification

[ITU-T X.oid-res | ISO/IEC 29168]	Draft Recommendation ITU-T X.oid-res | ISO/IEC 29168, Information technology — Open Systems Interconnection — Object Identifier resolution protocol and associated architecture, requirements and guidance

[Editor’s note] On-going works such as these draft Recommendations cannot be referred to in published Recommendations. Thus, these references shall be removed if they are still on-going works when Q.nid-test-arch is consented.

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	associated information [ITU-T Y.2213]: the information which is associated with an identifier 
NOTE: Example associated information instances are URL, URN, IP address, E.164 number, etc.

3.1.2	B2B tag-based identification applications and services: This term refers to tag-based identification applications and services based on business relationships which involve exchanges of identification information between business partners.

3.1.3	B2B2C tag-based identification applications and services: This term refers to tag-based identification applications and services based on integrated business relationships of B2B and B2C which involve exchanges of identification information.

3.1.4	B2C tag-based identification applications and services: This term refers to tag-based identification applications and services based on business relationships which involve exchanges of identification information between business and consumer.

3.1.5	C2C tag-based identification applications and services: This term refers to tag-based identification applications and services based on business relationships which involve exchanges of identification information between consumers.

3.1.6	identifier [b-ITU-T Y.2091]: a series of digits, characters and symbols or any other form of data used to identify subscriber(s), user(s), network element(s), function(s), network entity(ies) providing services/applications, or other entities (e.g. physical or logical objects). Identifiers can be used for registration or authorization. 
NOTE: Identifiers can be either public to all networks, shared between a limited number of networks or private to a specific network (private identifiers are normally not disclosed to third parties.)

3.1.7	forward identifier resolution [ITU-T Y.2213]: A function to resolve an identifier into an associated information.

3.1.8	identifier resolution [ITU-T Y.2213]:	A function to resolve an identifier into associated information (see “Forward identifier resolution”) and vice versa (see “Reverse identifier resolution”).

3.1.9	identifier scheme [ITU-T Y.2213]: It is a numbering scheme that specifies the format and structure of the identifiers used within that scheme.

3.1.10	ID tag [ITU-T Y.2213]: A physical object which stores one or more identifiers and optionally application data such as name, title, price, address, etc. 
NOTE: It may have a communication capability with an ID terminal depending on implementations.

3.1.11	ID terminal [ITU-T Y.2213]: A device with a data reading and optional writing capability which reads (and optionally writes) identifier(s) and optionally application data from/into an ID tag. 
NOTE: The data reading (and optionally writing) capability depends on implementations.

3.1.12	model network [b-ITU-T Q.3900]: Network which simulates the capabilities similar to those available in present telecommunication networks, has a similar architecture and functionality and uses the same telecommunication technical means.

3.1.13	Object Identifier (OID) resolution [ITU-T X.oid-res | ISO/IEC 29168]: process which translates an Object Identifier (OID) into associated information with the OID

3.1.14	reverse identifier resolution (or backward identifier resolution) [ITU-T Y.2213]: A function to resolve an associated information into a corresponding identifier. It is the reverse operation of the forward identifier resolution.

3.1.15	tag-based identification [ITU-T Y.2213]: The process of specifically identifying a physical or logical object from other physical or logical objects by using identifiers stored on an ID tag.

3.1.16	tag-based identification applications and services [ITU-T Y.2213]: Applications and services which use tag-based identification.

3.1.17	tag-terminal interface [ITU-T Y.2213]: 	A communication interface between ID tag and ID terminal. The ID terminal reads identifier(s) and optionally application data from the ID tag and/or writes them into the ID tag. 
NOTE: The interface medium may be infrared, RF, camera, optical scanner, and galvanic current. Communication techniques depend on the interface medium.

## Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	tag-based identification system: all relevant equipment to providing tag-based identification applications and services

3.2.2	tag data: whole data encoded in a certain format standard and stored in an ID tag or to be written in an ID tag 
NOTE – tag data may consist of an identifier and optional application data.

[TBD]

[Editor’s note] “[TBD]” hereafter means something more will be added later by contributions.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

1D	1-Dimension

2D	2-Dimension

B2B	Business-to-Business

B2B2C	Business-to-Business-to-Consumer

B2C	Business-to-Consumer

C2C	Consumer-to-Consumer

DNS	Domain Name Service

ENUM	Telephone Number Mapping

HTTP	Hypertext Transfer Protocol

ID	Identification

IP	Internet Protocol

LDAP	Lightweight Directory Access Protocol

NID	Network aspects of Identification systems (including RFID)

OID	Object Identifier

RF	Radio Frequency

RFID	Radio Frequency Identification

SCM	Supply Chain Management

SOAP	Simple Object Access Protocol

TCP	Transmission Control Protocol

TTI	Tag Terminal Interface

URI	Uniform Resource Identifier

URN	Uniform Resource Name

URL	Uniform Resource Locator

# Conventions

None.

# Compatibility issues

None.

# Overview of tag-based identification applications and services

Tag-based identification applications and services are defined by [ITU-T Y.2213] as applications and services which use tag-based identification and are required to incorporate three mandatory elements, identifier, ID tag and ID terminal, and one optional element, associated information. Table II.1 of [ITU-T Y.2213] shows how to evaluate applications and services in the context of the key elements.

## Application and service use cases

The Appendix III of [ITU-T Y.2213] describes several use cases of tag-based identification applications and services. It also classified them into:

- Closed-domain tag-based identification applications and services

    - Inventory management

    - Parking management

- B2B tag-based identification applications and services

    - Supply chain management

    - Manufacturing and production management

- B2C tag-based identification applications and services

    - Bus and subway route search

    - Mobile shopping

    - Tag-based payment

- B2B2C tag-based identification applications and services

    - Food chain information service

    - Home delivery service

- C2C tag-based identification applications and services

    - Business card exchange

## Reference service architectures

Figure 1 depicts a reference service architecture to support tag-based identification applications and services. [ITU-T Y.2213] and [ITU-T H.621] also describe relevant architecture models but [ITU-T Y.2213] focuses on operating over the NGN and [ITU-T H.621] focuses on general and functional operation models independent of the network environment. Figure 1 does not depend on a specific network type but describes the whole functional configuration consisting of key functional equipment.

Figure 1 – Reference service architecture for tag-based identification applications and services

Figure 1 includes lots of entities which are described as follows:

- tag – an ID tag;

- id – an identifier encoded in a format standard and contained in the tag. It is treated as tag data in this Recommendation;

- reader 1 – one of devices of reading tag data (e.g. camera, RFID interrogator, etc.);

- reader 2 – one of devices of reading tag data (e.g. camera, RFID interrogator, etc.);

- r-id – reader identifier to be identified among readers as a local matter within an ID terminal;

- writer – one of devices of writing tag data on an ID tag. 
NOTE – reader and writer may be separate devices or a combined device. An RFID interrogator works as both reader and writer but a camera works only as a reader;

- w-id – writer identifier to be identified among writers as a local matter within an ID terminal;

- network attachment – network connectivity-supporting function (e.g. TCP/IP) between an ID terminal and a peer communication entity such as service broker, contents server, identifier resolution system and OID resolution system;

- application functions – functions to invoke service functions and network service functions as well as to retrieve information from a contents server and present the information to an end-user.
NOTE – the multimedia information handling function (MIHF) of [ITU-T H.621] is one of application functions;

- service functions – functions to access a reader and/or a writer built in an ID terminal and to execute additional handling of information retrieved from a contents server according to service requirements (e.g. media transformation, compression/decompression, etc.). 
NOTE – the beginning part of the multimedia information discovery function (MIDF) of [ITU-T H.621] corresponds to a service function;

- network service functions – functions to execute identifier resolutions through an identifier resolution system or a service broker, and to execute OID resolutions through an OID resolution system or a service broker. 
NOTE – the later part of the multimedia information discovery function (MIDF) of [ITU-T H.621] corresponds to a network service function;

- service broker – a functional entity to invoke service functions and network service functions on behalf of the application functions of an ID terminal and to be triggered by the application functions of the ID terminal;

- identifier resolution system – a functional entity to provide identifier resolutions;

- OID resolution system – a functional entity to provide OID resolutions. 
NOTE – the OID resolution server of [ITU-T X.oid-res | ISO/IEC 29168] corresponds to an OID resolution system in this Recommendation; and

- contents server – a functional entity for storing, handling and providing information contents. 
NOTE – the multimedia information manager (MIM) of [ITU-T H.621] corresponds to the contents server.

Figure 1 shows two external interfaces: Tag-Terminal Interface and Network Interface. Their specific operation relationships and procedures are the same with what [ITU-T H.621], [ITU-T H.IRP] and [ITU-T X.oid-res | ISO/IEC 29168] describe.

# Classification of tag-based identification systems and functions to be tested

According to the reference service architecture, following tag-based identification systems and functions are specified to be tested. The tag-based identification systems are presented as a physical entity and the tag-based identification functions are presented as functions of physical entities.

## Classification of tag-based identification systems to be tested

Figure 1 depicts six physical entities: ID tag, ID terminal, service broker, identifier resolution system, OID resolution system and contents server. The Tag-Terminal Interface is required to be out of scope because its interface standards and relevant testing standards have been covered by ISO/IEC JTC 1. But some of functions of the ID terminal are related to the Network Interface. The contents server also is required to be out of scope because it is established usually with using Web technologies and not involved with network interfaces of tag-based identification systems.

### ID terminals

As shown in Figure 1, the ID terminal includes various functions. Some of them related to the Tag-Terminal Interface are out of scope of this Recommendation but the other functions related to the network interfaces of tag-based identification systems are required to be covered by this Recommendation. Thus ID terminals are the targets to be tested.

### Identifier resolution systems

Identifier resolution systems are the peer parts of ID terminals in terms of identifier resolutions. A specific communication protocol is performed between identifier resolution system and ID terminal (e.g. Domain Name System (DNS), X.500, Lightweight Directory Access Protocol (LDAP), etc.) The communication protocol is a network interface of tag-based identification systems.

### OID resolution systems

OID resolution systems are the peer parts of ID terminals in terms of OID resolutions. A specific communication protocol is performed between OID resolution system and ID terminal (e.g. Domain Name System (DNS), X.500, Lightweight Directory Access Protocol (LDAP), etc.) The communication protocol is a network interface of tag-based identification systems.

### Service broker

The service broker intervenes between ID terminal and other tag-based identification systems. Its functions depend on service requirements and implementations. An interface between service broker and ID terminal is needed. On behalf of an ID terminal, the service broker may access network interfaces to other tag-based identification systems and operation results are finally delivered to the ID terminal. Involved network interfaces depend on implementation of supporting functions of tag-based identification systems.

## Classification of tag-based identification functions to be tested

Tag-based identification systems classified by Clause 8.1 contain various application functions, service functions and network service functions. Some of them are out of scope of this Recommendation because relevant standards have already been developed or they take advantage of existing technologies (e.g. TCP/IP, HTTP, SOAP, DNS, X.500, LDAP, etc.). This Recommendation covers application functions, service functions and network service functions newly specified for tag-based identification applications and services below.

### Application functions

There are no application functions to be tested. From an implementation point of view, there may be at least two application functions provided through: an application entity to invoke service functions and network service functions (e.g. a Tag-Terminal Interface is executed to read tag data from an ID tag; a network interface of identifier resolution is executed to translate an identifier into associated information via identifier resolution systems; and a network interface is executed to translate an OID into associated information.); and another application entity to retrieve information content from a contents server (e.g. a World Wide Web browser is a typical example.). The former application entity does not embed any function to communicate with tag-based identification systems but just utilizes service functions and network service functions. The later application entity is an existing one which has already been tested against its supporting functionalities.

Table 1 – Application functions to be tested

| Application functions | Testing target |
| --- | --- |
| - An application function to invoke service functions and network service functions | No |
| - An application function to retrieve information content from a contents server | No |

### Service functions

There may be four service functions as described in Table 2. The service function 1) and 2) are required to follow existing Tag-Terminal Interface relevant standards and to be out of scope. The service function 3) corresponds to application-specific functionalities which are required to follow existing relevant standards and to be out of scope. If a service broker intervenes between ID terminal and other tag-based identification systems, a communication interface is required between ID terminal and service broker because the service broker works on behalf of the ID terminal with other tag-based identification systems. The communication interface is specified by service broker-relevant standards.

Table 2 – Service functions to be tested

| Service functions | Testing target |
| --- | --- |
| - A service function to access a reader or a writer to read tag data from an ID tag or write tag data into an ID tag | No |
| - A service function to manipulate tag data returned from a reader or to be written by a writer | No |
| - A service function to manipulate information content retrieved from a contents server (e.g. media transformation, compression/decompression, etc.) | No |
| - A service function for communication interfaces between ID terminal and service broker | Yes |

NOTE – No communication interfaces between ID terminal and service broker are specified yet, which means there is no testing items yet.

### Network service functions

There may be two network service functions and all the network service functions are required to be tested. The identifier resolutions are performed between ID terminal and identifier resolution system and the OID resolutions are performed between ID terminal and OID resolution system. Relevant functions embedded in ID terminals, identifier resolution systems and OID resolution systems are required to be tested.

Table 3 – Network service functions to be tested

| Service functions | Testing target |
| --- | --- |
| - A network service function to perform an identifier resolution | Yes |
| - A network service function to perform an OID resolution | Yes |

NOTE – The network interface of identifier resolution is specified by [ITU-T H.IRP] and the network interface of OID resolution is specified by [ITU-T X.oid-res | ISO/IEC 29168].

## Conformance of tag-based identification functions to tag-based identification systems to be tested

Clause 8.2 identified three tag-based identification functions to be tested as follows:

- A service function for communication interfaces between ID terminal and service broker;

- A network service function to perform an identifier resolution; and

- A network service function to perform an OID resolution.

These functions run at ID terminals as well as service brokers, identifier resolution systems and OID resolution systems. Thus conformance of relevant interface standards is required to be tested against the service functions and network service functions. Interoperability also is recommended to be tested between ID terminal and service broker; between ID terminal and identifier resolution system; between ID terminal and OID resolution system; between service broker and identifier resolution system; and between service broker and OID resolution system.

# Testing architecture for tag-based identification systems and functions

## Purpose of model network

The testing architecture provides a reference model network to describe target systems and system configurations in terms of model network, testing procedure and testing requirements.

The model network is a prototype configuration of tag-based identification systems. It supports system testing in order to evaluate functioning and compatibility of the specific functions of target systems.

The model network can be used for testing the full list of functions of tag-based identification systems according to Clause 9.3. The target systems to be tested should include all the technical functions defined in Clause 8.2.

## Model network for testing

Clause 8.1 specifies four testing targets, i.e. ID terminal, service broker, identifier resolution system, and OID resolution system. The basic configuration of the model network is shown in Figure 2. The service broker looks located in different places but it may work as a single physical entity and provide two different functions, i.e. service functions and network service functions described in Clause 7.2.

NOTE – there are conformance issues for the testing targets as described in Clause 8.3.

Identifier resolution and OID resolution services in Figure 2 may be provided to the ID terminal directly or indirectly through the service broker by identifier resolution system and OID resolution system. It is an implementation issue and a decision point of end users or service providers to take which operation way.

Figure 2 – Basic configuration of the model network

The two network service interfaces between ID terminal and identifier resolution system and between ID terminal and service broker may be identical or different. But the two network service interfaces between ID terminal and identifier resolution system and between service broker and identifier resolution system are required to be identical, which means the identifier resolution system provides the only single interface to either ID terminal or service broker. The same associations apply to ID terminal, OID resolution system and service broker.

## Testing procedure

### Service functions testing process

Following the classification of tag-based identification functions to be tested as specified in Clause 8.2, the testing procedure is classified into two types: service functions testing process and network service functions testing process.

Following target systems are required to be tested at this testing process:

- ID terminal; and

- Service broker

The tests to be applied to ID terminals are the following ones:

- Client communication interface

The tests to be applied to service brokers are the following ones:

- Server communication interface

Specific communication interfaces will be defined by an application interfaces standard and depend on service features.

### Network service functions testing process

Following target systems are required to be tested at this testing process:

- ID terminal;

- Service broker;

- Identifier resolution system; and

- OID resolution system

The tests to be applied to ID terminals are the following ones:

- Client communication interface for identifier resolutions with the identifier resolution system

- Client communication interface for identifier resolutions with the service broker

- Client communication interface for OID resolutions with the OID resolution system

- Client communication interface for OID resolutions with the service broker

The network service interfaces for item 1) and 2) above may be identical or different as described in Clause 9.2.

The tests to be applied to service brokers are the following ones:

- Server communication interface for identifier and OID resolutions to the ID terminal

- Client communication interface for identifier resolutions with the identifier resolution system

- Client communication interface for OID resolutions with the OID resolution system

The test to be applied to identifier resolution systems is the following one:

- Server communication interface for identifier resolutions to the ID terminal and the service broker

The test to be applied to identifier resolution systems is the following one:

- Server communication interface for OID resolutions to the ID terminal and the service broker

## Testing requirements

### Model network configuration requirements

The protocols scheme of the model network is required to be realized in accordance with the scheme illustrated in Figure 3.

Figure 3 – Model network configuration

### Methodology of model networks testing

The methodology of testing on the model network is required to provide all the tests and accordingly allow checking the two types of service functions which are described in Clause 9.3. The testing scheme is shown in Figure 3 and specific test sets and testing procedures are specified by separate standards. The testing results of each service function have to be delivered to the database of which structure and data format also have to be specified separately.

# Bibliography

[b-ITU-T Q.3900]	Recommendation ITU-T Q.3900 (2006), Methods of testing and model network architecture for NGN technical means testing as applied to public telecommunication networks

[b-ITU-T Y.2091]	Recommendation ITU-T Y.2091 (2006), Terms and Definitions for Next Generation Networks

________________