| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4385 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.RTCPPROF: Update proposal clause 8.5 (Reporting aspects) |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes updates to clauses 8.5 concerning reporting point related requirements.

Discussion and conclusions

Existing text in clause 8.5 was only addressing content of reporting, but not the various H.248 capabilities for reporting control. It is suggest to add explicit requirements at least for the three major H.248 command related capabilities.

Clause 8.5 should be then divided in two sub-clauses.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 23 R1 (WP 1/16) from the January 2013 Q.3/16 meeting.

Change proposal:

[Begin Proposed Correction]

## 8.5	H.248 MG as reporting point at H.248 interface (RPH.248)

### 8.5.1	Reporting content

This reporting capability is tightly coupled to the H.248 signalling capability according to individual H.248 packages:

R-8.5.1/1:	The MG is required to support H.248 statistics according to the RTP package [ITU-T H.248.1].

R-8.5.1/2:	The MG is required to support H.248 statistics according to the RTCP extended performance metrics package [ITU-T H.248.30].

R-8.5.1/3:	The MG is required to support H.248 statistics according to the RTCP XR Block Reporting package [ITU-T H.248.48].

R-8.5.1/4:	The MG is required to support H.248 statistics according to the RTP application data package [ITU-T H.248.58].

R-8.5.1/5:	The MG is required to support H.248 statistics according to the Received RTCP package [ITU-T H.248.71].

### 8.5.2	Reporting control

The principal H.248 control capabilities for statistics reporting are summarized in Appendix IV/[ITU-T H.248.1]. Following three major practices may be considered from H.248 command perspective:

R-8.5.2/1:	The MG is required to support H.248 statistics reporting at the end of communication phases only (i.e., related to SUBTRACT command).

R-8.5.2/2:	The MG is required to support H.248 statistics reporting during active communication phases on request by the MGC (i.e., related to AUDITVALUE command).

R-8.5.2/3:	The MG is required to support H.248 statistics reporting during active communication phases dependent on configured reporting conditions (i.e., related to [ITU-T H.248.47] and NOTIFY command).

[End Proposed Correction]

____________________