| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4467 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | Q5 | Oslo, 17 - 21 June 2013 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | Modified high-level architecture of telepresence system |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

The contribution proposes the modified high-level functional architecture of telepresence system for section 6 of F/H.TPS-Arch.

# Discussion

Section 6 of F/H.TPS-Arch describes a high level architecture and components of the telepresence system. The goal of this section is to describe the overall functional architecture of telepresence system which shows its functional components and interfaces between them in an implementation independent way. There are several editors’ notes that should be considered in definition of the high-level architecture. To solve these editor’s notes, our approaches are as follows.

- We consider that a high-level architecture of telepresence system needs to be generic and independent of any implementations. For this, a set of main components according to logical functions and relationships between different components are identified.

- Use the same interface labels to align section 6 with section 8.

- Instead of “Gatekeeper”, we propose to use the term “Call Controller (CC)”.

# Proposal

Based on the discussion above, we propose the following changes.

- Add abbreviation of Call Controller (CC) in section 4

- Modify the current text of section 6 by the proposed text, and

- Modify the current text of section 7.3

________________________ begin for proposals _____________________________

Proposal (1): Add abbreviation of Call Controller (CC) in section 4

# 4	Abbreviations and acronyms

CC	Call Controller

Proposal (2): Modify the current text of section 6 by the proposed text

# High level architecture

IP Network

PLMN

Figure 2: High level Telepresence architecture network model

A high-level telepresence architecture includes: telepresence endpoints, legacy video conferencing endpoints, legacy audio endpoints, Multipoint Control Units (MCU), Call Controller (CC), Gateways (GW), the management system, etc. There may be multiple forms of telepresence endpoints, such as three-screen endpoint, single-screen endpoint and etc. The architecture supports inter-operation of heterogeneous telepresence endpoints and inter-operation between telepresence endpoints and legacy video conferencing and audio endpoints.

There may be multiple MCUs and CCs in the architecture.

The architecture may have multiple GWs that connect to other types of networks, such as fixed and mobile telephone networks.

Figure x illustrates a high-level functional architecture for telepresence system. It identifies a set of functional components and relationships to show how these components are connected to each other. The main components are telepresence endpoint, multipoint control unit, call controller, gateway, and management system. The detailed characteristics and functions of these components are described in the sequent sub-clauses. In figure x, there are each singles of these components however there can be multiple MCUs, Call controllers, and Gateways in the telepresence system. These functional components can support multiple functions on the same device or separately on different devices. A relationship in the figure indicates that two components involved can interact and exchange messages through the given interfaces. The interfaces between multiple components (e.g. between Call Controllers, MCUs) are also shown in the functional architecture.

Figure x. High-level functional architecture of telepresence system

The main interfaces identified in the functional architecture of telepresence system are described as follows.

- A: Interface between legacy conferencing endpoint and MCU, for transmitting conference call and control information.

- A1: Interface between telepresence endpoint and MCU for transmitting conference call and control information.

- B: Interface between legacy conferencing endpoint and Call Controller for transmitting registration, authentication and call process information.

- B1: Interface between telepresence endpoint and Call Controller for transmitting registration, authentication and call process information.

- C: Interface between MCU and management system for transmitting conference scheduling information, creating and ending conferences.

- D: Interface between MCU and Call Controller for registration, authentication, call process of MCU and transmitting resolving information of user addresses.

- E: Interface between multiple Call Controller for transmission of resolving information of user addresses.

- F: Interface between Call Controller and management system, for transmitting Call Controller’s management and configuration information, user information, billing data, etc.

- G: Interface between legacy videoconferencing endpoints for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication.

- G1: Interface between telepresence endpoints for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication.

- G2: Interface between legacy videoconferencing endpoint and telepresence endpoint for transmitting call signalling, media streams and related control information when a telepresence endpoint has a peer-to-peer communication with a legacy videoconferencing endpoint.

- H: Interface between MCUs for transmitting media streams and information of MCU cascade connection and inter-operation.

- I: Interface between MCU and Gateway for converting media provided in one type of network to another required type of network.

## Description of sub-system components

### Telepresence Endpoint

The Telepresence Endpoint is the logical point of final termination, and may consist of more than one physical device. The main functions of the telepresence endpoint include:

- Setting up call sessions through capability negotiation,

- Capturing, encoding, packetizing and sending local media streams,

- Receiving, de-packetizing, decoding and rendering remote media streams.

In contrast to a telepresence endpoint, an MCU may also send and receive media streams, but it cannot initiate or terminate media streams through capturing and/or rendering. There may be multiple forms of telepresence endpoints, such as: 3 camera/screen endpoint, single camera/screen endpoint, audio-only endpoint and etc.

Endpoint characteristics include:

- The number of main cameras, spatial relationships of cameras and capture capabilities, such as: resolution, frame rate and aspect ratio.

- Viewpoint and field of view for main cameras, including overlap regions, missing regions or whether images of cameras can be composited to a seamless image in over the entire depth range. The number of presentation cameras and capture capabilities, such as: resolution, frame rate and aspect ratio.

- The video stream capabilities for data sharing such as: resolution, frame rate and aspect ratio.

- The number, type, size of main screens, spatial relationships of screens and screen capabilities, such as: resolution, frame rate, aspect ratio, and screen border width.

- The number and type of presentation screens, spatial relationship of presentation screens and screen capabilities.

- Spatial relationships of audio signals of microphones and capture capabilities of microphones.

- Spatial relationships of loudspeakers and parameters of loudspeakers.

- Display strategies for multipoint conference, such as the static or dynamic mapping of camera-to-display.

In general endpoint characteristics are transmitted through signalling streams in session initialization and used for capability negotiation in inter-operation of different telepresence endpoints.

Note: [ITU-T H.245] defines a terminal as “any endpoint and may be a user's terminal or some other communication system such as an MCU or an information server”. As such the term H.323 Telepresence Terminal is equivalent to the term “Telepresence Endpoint”.

### Multipoint Control Unit

The MCU is a device that connects two or more telepresence endpoints together to one single telepresence conference. An MCU can setup and control multiple videoconferencing call sessions. After receiving media streams from multiple endpoints or cascading MCUs and GWs, an MCU processes the media streams and sends the processed media stream to an endpoint, MCU or GW.

An MCU must support video and audio mixing and media adaptation for different types of endpoints and should also support control functions, for example, auxiliary stream token control, voice switch and control based on endpoint screens, etc.

### Call Controller (CC)

A CC is the resource management device for video conferencing and can provide resource management for endpoints, MCUs and GWs. The functions of a CC include: address resolution, access control and bandwidth management. CC corresponds to Gate Keeper on H.323 system.

### 6.1.4	Gateway

A GW can connect various types of networks and provide interworking of signalling and media streams, such as interworking H.323 and SIP. It may also provide adaptation for media streams that have different characteristics e.g. codec format.

### 6.1.5	Management System

A management system mainly contains control and management functions that provide specific telepresence services, such as device management, conference scheduling and user authentication, etc.

### 6.1.6	Legacy conferencing endpoints

A legacy conferencing endpoint can be legacy H.323 or non H.323 conferencing endpoint. In case of legacy H.323 endpoint, the main capabilities and functions are identical to the definition in [ITU-T H.323]. In case of non H.323 endpoint, it can be a legacy endpoint which uses non H.323 open standards (i.e. SIP). This kind of endpoints can be a SIP hard phone and SIP soft phone (i.e. PC or Laptop). The legacy non H.323 endpoints can connect to other components (e.g. Call Controller) through a gateway which performs protocol conversion and stream adaptation.

Proposal (3): Modify the current text of section 7.3

## 7.3	Call Controller (CC)

The main functions of CC are identical to the definitions in [ITU-T H.323].

__________________end of proposals _________________________