| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4612 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Sup.13 (Rel. 2): Terminology related to media translation |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes initial proposals for terminology related to media translation.

1	Motivation

The control of modes of operation at H.248 gateway control interfaces is typically subject of SDP information embedded in the H.248 Stream Descriptor. SDP usage in H.248 profile specifications is subject of a dedicated template clause. Existing H.248 profile specifications provide mainly the isolated usage of supported SDP elements, but does not address "SDP combinations across a H.248 stream endpoint pair" or lower levels of details as such.

Alcatel-Lucent is currently preparing the proposal of a new work item related to the "H.248 profile guidelines for media translation configurations", which could be a new self-contained Recommendation or finally added either to

- H.248.39 ("due to SDP-based control of media translation modes of operation")?, or

- H.248.79 ("due to packet processing guidelines")?, or

- H.248.88 ("due to origination in RTP topologies")?

Prequisite of such a work item is terminology related to media translation.

The primary purpose of this contribution is the preparation of such a terminology framework, which would belong to H.Sup13 due to its H.248 wide scope.

The secondary motiviation is related to the "media plane B2BUA" discussion (see AVD-4599), a subject which is also tightly coupled to the concept of media (translation) type of interworking functions.

2	Discussion

Terminology related to media translation could be derived from existing work, such as the ETSI H.248 Ia profile. This contribution is therefore proposing the addition of two new Appendices to H.Sup13, related to

- H.248 IP-to-IP media gateways – Modes of operation related to application and transport layer protocols

- H.248 IP-to-IP media gateways – Modes of operation related to media translation

The development of good terminology requires usually multiple discussion cycles. This contribution provides initial material for the first review and discussion.

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against H.Sup.13 (Release 1; 07/2014).

#1:

[Begin Proposal]

# 1	Scope

In scope of this Supplement are GCP related terms that relate to multiple H.248 documents.

Out of scope of this Supplement are

- terms not related to H.248 technology,

- terms which affect a single H.248.x Recommendation.

## 1.1	Release 2

Release 2 adds further terms, in particular related to following areas:

- application (media) and transport related modes of operation in H.248 IP-to-IP media gateways;

[End Proposed Correction]

#2:

[Begin Proposal]

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.x]	The ITU-T H.248.x-series of Recommendations (Gateway Control Protocol).

[ETSI TR 183068]	ETSI TR 183 068 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

[End Proposed Correction]

#3:

[Begin Proposal]

# 3	Definitions

## 3.1	Terms defined elsewhere

In addition to the basic GCP terms defined by clause 3.2/[ITU-T H.248.1], this Supplement uses the following terms defined elsewhere.

None.

## 3.2	Terms defined in this Supplement

NOTE - The terms in this clause are tabulated in order to maintain the usual alphabetical order when adding additional terms in future releases of this Supplement.

This Supplement defines the following terms (Table 1):

Table 1 – Terms defined in this Supplement

| Term | Definition |
| --- | --- |
| H.248 master-slave control relationship | The H.248 decomposed gateway follows a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship, with the MGC as master and MG as the slave entity. NOTE 1 - [b-ITU-T Q.Sup31] Annexes C.2 or C.3 provide examples of functional-to-physical mapping models according to the H.248 master-slave control relationship. |
| MG autonomous mode | For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only. |
| MGC-strictly controlled mode | For a particular function, the control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| stream endpoint tuple (SEPT) | the generalization of SEPP towards multiple associated Stream Endpoints (SEP) within the same Context. All SEPs share the same StreamID value. The stream topology is given by the topology descriptor settings. |

### FIXTHIS: add terms (from clauses #3#.3.2.1 and  #4#.3.4) to table ###

[End Proposed Correction]

#4:

[Begin Proposal]

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| CT | Content Translation |
| --- | --- |
| GCP | Gateway Control Protocol |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MT | Media Translation |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |
| TF | Transparent Forwarding |
| TT | Transport Translation |

[End Proposed Correction]

#5:

[Begin Proposal]

Appendix #3#

H.248 IP-to-IP media gateways – Modes of operation 
related to application and transport layer protocols

## #3#.1	Introduction

H.248 IP-to-IP media gateways are also known as (session) border gateways, transition gateways, etc. There is a long history of correspondent H.248 profiles for such gateways, which are using a well-established (but still not explicitly) defined terminology concerning the concept of "mode of operation" for packet-to-packet interworking in the IP media/data/bearer path.

Furthermore, the initially introduced modes of operation were further refined and detailed, which demands also for a more detailed terminology in that area.

## #3#.2	Inventory of baseline terminology for H.248 gateways

All H.248 profiles (as defined by various SDOs) for such gateway types are using the terminology as introduced by ETSI TISPAN for the so called H.248 Ia profile, which is documented in [ETSI TR 183068].

## #3#.3.1	References

The framework is outlined in [ETSI TR 183068], Annex G "Illustration of BGF modes of operation". The control of the mode of operation is tightly coupled with the SDP usage in the H.248 stream descriptor for the media description (see also clause 7.1.8.1.1.2/[ITU-T H.248.1]), see Figure #3#.1.

Figure #3#.1 – Copy of Figure G.3/[ETSI TR 183068] "BGF modes - H.248 Local and Remote Descriptor: SDP "m="-line specification combinations"

The baseline terminology is documented in [ETSI TR 183068], Annex I "Illustration of "Protocol Layer Lx"-based Packet Processing BGF modes". Table #3#.1 provides a copy from [ETSI TR 183068]/Figure I.1 "BGF modes - Control framework - Major SDP "m="-line specification combinations":

Table #3#.1 – Control framework - Major SDP "m="-line specification combinations

| Media type | L4 port | L4 (& L4+) protocol | Media format | H.248 IP-to-IP context "mode of operation" |  |
| --- | --- | --- | --- | --- | --- |
| X | X | X | X | media aware |  |
| X | X | X | - | media format agnostic, media framing aware, transport aware |
| - | X | X | - | media agnostic, transport aware |
| - | X | - | - | media agnostic, L4-port aware, transport-protocol agnostic |
| - | - | - | - | media agnostic, transport agnostic |

The SDP "m="-line is based on four information elements. Symbol 'X' indicates the specification of a concrete value for the SDP "m="-line parameter, and symbol '-' represents a particular H.248 wildcard type (see clause 5.2.2.1/[ITU-T H.248.39]).

The concerned mode of operation related to an H.248 IP-to-IP context relates actually to the level of IP SEPPs (IP-enabled stream endpoint pairs). Every IP SEPP is constituted by four IP connection endpoints, which recalls again Figure #3#.2:

Figure #3#.2 – Copy of Figure I.2/[ETSI TR 183068] "BGF modes - Control Framework - H.248 Local and Remote Descriptor: SDP "m="-line and H.248 IP-to-IP Context"

For more information see [ETSI TR 183068].

### #3#.3.2	Explicit definitions for baseline terminology

### #3#.3.2.1	Definitions

The most frequently used terms are:

| media aware | # a fully specified media description which contains (at least?) the information related to media type , L4 port, transport protocol (stack) and media format. |
| --- | --- |
| media agnostic | # a media description with unspecified information (via '-' tagging) for the two media parameters media type and media format. |
| media format agnostic | # a media description with unspecified information (via '-' tagging) for media parameter media format., but an explicit specification of media type (and transport protocol (stack)). |
| media framing aware | # media format agnostic associated with full specified transport protocol (stack) information. |
| transport agnostic | # a media description with unspecified information (via '-' tagging) for the two transport parameters L4 port and transport protocol (stack). |
| # transport aware | # a media description with fully specified information for the two transport parameters L4 port and transport protocol (stack). |
| transport-protocol agnostic | a media description with unspecified information (via '-' tagging) for transport parameter transport protocol (stack), but information about the L4 port. |

#### #3#.3.2.2	Qualification

Above terms, - with respect to the H.248 IP-to-IP context "mode of operation" characteristic -, are typically used to qualify other terms, such as:

- <term> SDP "m="-line,

- <term> H.248 SEP, under the condition that both IP connection endpoints (source and destination) share the same mode of operation,

- <term> traffic direction, under the condition that the media description is identical for a particular H.248 stream direction, or

- <term> H.248 SEPP, under the condition that all four IP connection endpoints  share the same mode of operation.

#### #3#.3.2.3	Term media versus application

The term 'media' originates from the SDP (media description, media line), but may be synonymously replaced in general by term 'application' in above definitions.

[End Proposed Correction]

#6:

[Begin Proposal]

Appendix #4#

H.248 IP-to-IP media gateways – Modes of operation 
related to media translation

## #4#.1	Introduction

Appendix #3# summarizes the core terminology used for the qualification of media/transport related modes of operation as known for H.248 IP-to-IP media gateways.

There is a need to further detail these terms, primarily driven by media type specific packet operations beyond the high level category of media format awareness and it's associated media translation functionality. There are a number of specific functions behind that category.

## #4#.2	Reminder: media processing in context of RTP topologies

The concept of "RTP topologies" (see [IETF RFC 5117]) describes such modes of operation in case of "media-over-RTP-over-IP" transport (or when application data is carried via the application framing protocol 'RTP'). The subject of RTP topologies in context of H.248 gateways is the scope of [ITU-T H.248.88], Figure #3#.4 recalls the basic RTP topology models as relevant for H.248 IP-to-IP media gateways:

Figure #3#.4 – H.248 IP-to-IP media gateways – RTP topology model [ITU-T H.248.88]

Some of these RTP topology types (see clause 7.2/[ITU-T H.248.88]) correlate directly, e.g., "RTP transparent forwarding" with "media agnostic, transport aware" or "RTP media translation" with "media aware, transport aware" modes of operation. However, it is evident that there are already terminology gaps for major RTP topologies (e.g., "RTP transport translation"?), but also at the more detailed level (see e.g., RTP media translation with and without media format change (RTPMTlight and RTPMTfull in Table 2/[ITU-T H.248.88]))

## #4#.3	Problem statement

There are three terminology areas which would benefit from clarification:

- Define terms, - which were originally introcuded by RTP topologies -, in a transport protocol independent manner (i.e., not RTP specific) and correlate them with the original "mode of operation" terminology (from Appendix #3#).

- Consideration of more detailed levels as used in H.248 IP-to-IP media gateways.

- Terminology should distinguish between the functional level and (physical/logical/virtual) resource level.

The three items are addressed in below sub-clauses.

## #4#.4	Solution proposals

### #4#.3.1	Packet path processing models related to media translation

Figure #3#.5 indicates the three major modes of operation:

Figure #3#.5 – H.248 IP-to-IP media gateways – RTP-independent model for the three major modes of operation (from media processing perspective)

Table #3.2 summarizes the terminology correlation:

Table #3#.2 – Correlation between "packet path topology" and "mode of operation"

| "generic 'RTP' topology" | H.248 IP-to-IP context "mode of operation" |  |
| --- | --- | --- |
| transparent forwarding (TF) | media agnostic, transport agnostic |  |
| transport translation (TT) | media agnostic, transport aware |
| media translation (MT) | media aware, transport aware |

{Editor's note (2014-11): the TF mode might be still subject of discussion.}

### #4#.3.2	Media translation path in more detail

The previous model (and associated terminology) is sufficient for "media agnostic" only H.248 profiles. However, "media aware" oriented H.248 profile with support of many media specific processing functions demand for a more detailed terminology. Figure #3#.6 indicates the next lower level behind the media translation path:

Figure #3#.6 – H.248 IP-to-IP media gateways – the media translation path in more detail

The detailed media translation (MT) model acknowledges firstly the fact that the egress media format could be equal to or different to the ingress media format. The two MT variants (3.1 and 3.2) are known as media modification and media transcoding. Furthermore, besides the inherent MT functions of decoding and encoding stages, might be an optional interim stage related to content translation (CT).

Conclusions from terminology perspective:

- term "media transcoding" could be correlated with "identical media types, but different media formats" in the SEPP-related media descriptions (with scope on one traffic direction); hence, still purely SDP "m="-line based;

- term "media modification" is related to "identical media types and identical media formats", but different media format configurations, as given by additional attributes (i.e., SDP "a="-line(s)).

### #4#.3.3	Functional versus resource level

The execution of a network function (such as media translation) implies the usage of a resource (which might be of type physical, logical, virtual). There is typically also a correlation at terminology level given by following general rule: the resource level term is constituted by nominalization of the functional level term, some examples:

- (media) translation  (media) translator;

- (media) transcoding  (media) transcoder;

- etc.

Furthermore, there are more terminology aspects when considering both levels from H.248 context point of view: e.g., an H.248 context could be configured for an interworking function which leads to the reservation and allocation of a e.g., resource "(media) transcoder", but the function itself could be still disabled on that resource, leading to a "(media) transcoding-free" operation from packet processing perspective. The same MG behaviour "(media) transcoding-free" would be effectively the result of a "(media) transcoder-free" context configuration. Thus, there are two options at resource level behind the same functional level behaviour of "(media) transcoding-free", but both options are characterized by significant differences in terms of cost factors (capacity, performance, power dissipation, etc).

Conclusions from terminology perspective:

- the two levels of functions and resources should be more precisely differentiated in terminology in order to avoid confusions.

### #4#.3.4	Explicit definitions for baseline terminology

### #4#.3.4.1	Functional level definitions

The most frequently used terms are:

| content translation | the transformation of content from one representation to another representation. 'Content' relates to the service data unit (i.e., "packet payload") at application protocol layer in general. Examples for media type 'audio': - automatic level gain control - noise reduction in voice signals Examples for media type 'video': - change of image size change - change of mage orientation - change of image resolution - etc |
| --- | --- |
| media modification | any kind of media processing where the egress media format is still unchanged, hence identical to the ingress media format. |
| media transcoding | any kind of media processing where the egress media format is different versus the ingress media format. NOTE – 'Media formats' corelate with the term 'codec' for some media types (such as audio or video), hence the notion of 'transcoding'. |
| media translation | any kind of "media protocol layer" related interworking function (i.e., formally a (L7)-IWF) related to media processing such as media modification or media transcoding. NOTE – Media translation relates thus to a particular category of Layer-(7)-aware H.248 service interworking functions. |
| transport translation | any kind of "transport protocol layer" related interworking function (i.e., formally a (L4)-IWF), which covers additional layer specific interworking functions at lower protocol layers. |

Thus, a media translator implies the full termination of the "media protocol", which relates actually to the formal concatenation of (media)-connection-endpoints (see Appendix #1#) and the actual (media)-IWF.

A media relay could be the defined as:

| media relay | a lightweight media translation function (given by the formal concatenation of (media)-relay-endpoints and inserted (media)-IWF). |
| --- | --- |

#### #4#.3.4.2	Resource level definitions

The resource (physical/logical/virtual) level terms may be then derived from the functional level terminology. For instance, following terms might be derived for the example of media transcoding:

| (media) transcoder | a resource component equipped with a media transcoding function. |
| --- | --- |
| (media) transcoding(-full) mode | a (media) transcoder with an enabled media transcoding function. |
| (media) transcoding-free mode | a (media) transcoder with a disabled media transcoding function. NOTE – The resource is allocated for media transcoding hence not available for other purposes (even if not used). |
| (media) transcoder free mode | a media translator without any resource consumption related to media transcoding. NOTE – The media translator may result in a transport translator or even in a transparent forwarder in case that there are not any other media- or transport-related functions used. |

[End Proposed Correction]

_________________