| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4616 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.WEBRTC: Differentiated Services (DiffServ) and Real-time (RT) Communication |  |  |  |
| Purpose: | Discussion |  |  |  |

Summary

This contribution proposes a new living list item for H.248.WEBRTC.

Discussion

The topic is related to following technology,

IETF WG Dart Status Pages: DiffServ Applied to Real-time Transports

"Differentiated Services (DiffServ) and Real-time Communication"
http://tools.ietf.org/html/draft-york-dart-dscp-rtp-02

which is considered for WEBRTC. This is a pending item, we'd like to suggest to document it in our living list. The proposed text contains an initial, high-level evaluation, see below.

Proposal

It is proposed to accept the marked up changes below. Changes are marked up against the input draft of this meeting (AVD-4594).

Change proposal:

[Begin Proposed Correction]

Appendix L3
Living List – DiffServ for WebRTC multimedia traffic – Discussion of IETF proposal "DiffServ for Real-time communication"

(This appendix does only contain working material.)

{Editor's note (2014-11): this item is relevant for H.248 WEBRTC gateways, but documented as living list item because 1st the requirement situation is still unclear and 2nd the technology (DiffServ extensions) are yet in discussion (IETF WG DART).}

## L3.1	Item of discussion

H.248 WEBRTC gateway is required to support "QoS marking" for WEBRTC bearer traffic. QoS marking relates to capabilities as defined by [ITU-T H.248.52].

For the DiffServ-based QoS marking method are discussions ongoing how WEBRTC could benefit from DiffServ, under the specific conditions of

Condition 1: WEBRTC = multiple application traffic flows multiplexed on IP transport connections;

Condition 2: WEBRTC = different application traffic flows benefit from different DSCPs (DiffServ codepoint);

Such a DiffServ model is not yet supported.

## L3.2	DiffServ models

Figure L3.1 illustrates three DiffServ models:

Figure L3.1 – DiffServ models

The existing DiffServ technology, as well as H.248.52, was designed for model (I), but would be also applicable for model (II).

Model (III) is in scope of IETF draft draft-york-dart-dscp-rtp, and considered for WEBRTC.

Draft "draft-york-dart-dscp-rtp" considers the case of a) multiplexed applications as transported via b) RTP/UDP (or RTP/TCP), at the example of WebRTC (i.e., audio/RTP and video/RTP share the same IP transport connection, but would benefit from different DS PHB). There would be consequently different DSCP assignments for the same IP transport connection.

Further information, see:

[b-IETF dscp-rtp]	IETF draft-york-dart-dscp-rtp (2014), Differentiated Services (DiffServ) and Real-time Communication.
URL: http://tools.ietf.org/html/draft-york-dart-dscp-rtp-02

## L3.3	Impact on H.248 WebRTC gateways

There are various roles of H.248 IP media gateways concerning support and handling of DiffServ, see Appendix IV/[ITU-T H.248.52]. Thus, the possible set of "DiffServ" roles is primarily limited by the "IP mode of operation", see [ITU-T H.248.64]: Back-to-Back IP Host (B2BIH) mode or IP router (IPR) mode. True DS PHB is associated to the IPR mode only!

Status H.248 WebRTC gateways:

Existing H.248 profiles for IP-IP gateways, which are subject of WEBRTC support (such as 3GPP eIMS-AGW), enforce the Back-to-Back IP Host (B2BIH) mode.

Such B2BIH configured H.248 border gateways act in a DiffServ environment either

- in DSCP transparent forwarding mode (see H.248.52, § 7.6.1.2 Copying DSCP values from the ingress to the egress);

- as DS boundary node (see H.248.52, § IV.3.2  H.248 MG as "DS boundary node"); or

- as DSCP translator (see H.248.52, § IV.3.3  H.248 MG between two DS domains).

Thus, such H.248 border gateways don't provide any kind of actual DS PHB.

## L3.4	Conclusions

On a first glance, when the B2BIH mode would be maintained for H.248 WEBRTC gateways, then:

- there seems to be no real impact from the extended DiffServ model (draft-york-dart-dscp-rtp) on existing H.248 border gateway profiles;

- if there are multiple DSCPs used per IP transport connection, then it would also not affect the DSCP translator function.

- H.248.52 may firstly need to be extended for that DiffServ model;

However, a final evaluation is dependent on the finally required H.248 WEBRTC gateway interworking models for WEBRTC RTP traffic components (as dependent on draft RTCWEB-RTP-USAGE).

[End Proposed Correction]

____________________

DiffServ: There's the basic assumption that the IP L4 payload contains unmultiplexed application data traffic, i.e., a single DS codepoint is assignment per "application IP flow".