| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4654a |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SCTP: Appendix II - Update of signalling example for SCTP via DTLS/UDP |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to update the signalling syntax in Appendix II in order to align with the latest updates of the correspondent IETF drafts.

Contributor's note (2014-10-27): there are a number of Editor's Notes requesting editorial updates of the Figures in Appendix II. This aspect is not yet addressed by this contribution, but still planned for a late revision.

Contributor's note (2014-10-30): AVD-4654R1 provides above mentioned editorial enhancements

There is also a new living list added related to the discussion of "SDP vs H.248" based bearer control procedures.

1	Introduction

The signalling example in Appendix II is based on some SDP elements, which are still work in progress items in the IETF. There were some updates on these IETF drafts since the last SG16 meeting.

2	Discussion

Following drafts are primarily affected by Appendix II:

- IETF draft on "Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP"

was recently updated (draft-ietf-mmusic-sctp-sdp-07), inclusive one major SDP syntax change:

SDP attribute

- a=sctpmap:… <application usage>

is replaced by

- a=fmtp: <application usage> …

E.g., application usage value "webrtc-datachannel" is intended for WebRTC and Telepresence services.

- IETF draft on "SDP-based "SCTP over DTLS" data channel negotiation"

was recently updated in order to align with the latest SDP syntax for SCTP (see above). The latest draft is:

- draft-ejzak-mmusic-data-channel-sdpneg-01
(http://tools.ietf.org/html/draft-ejzak-mmusic-data-channel-sdpneg-01 )

with following changes:

- Revisions to identify difference between internal and external negotiation and their usage.

- Introduction of more generic terminology, e.g. "application" instead of "browser".

- Clarification of how max_retr and max_time affect the usage of unreliable and reliable data channels.

- Updates of examples to take into account the SDP syntax changes introduced with draft-ietf-mmusic-sctp-sdp-07.

- Removal of the SCTP port number from the a=dcmap and a=dcsa attributes as this is now contained in the a=sctp-port attribute, and as draft-ietf-mmusic-sctp-sdp-07 supports only one SCTP association on top of the DTLS connection.

- IETF draft on "MSRP over SCTP/DTLS data channels"

was recently updated in order to align with the latest SDP syntax for SCTP (see above):

- draft-ejzak-mmusic-msrp-usage-data-channel 
(http://tools.ietf.org/html/draft-ejzak-mmusic-msrp-usage-data-channel-00 )

replaces

- draft-ejzak-dispatch-msrp-data-channel 
(http://tools.ietf.org/html/draft-ejzak-dispatch-msrp-data-channel-00 )

because the responsible IETF WG (MMUSIC?, DISPATCH?, other?) is still undecided.

3	Discussion of H.248 signalling options for SCTP transport modes

This item is a follow-up topic to the SDP discussion. Some initial material was added fairly late to this contribution, covered as new Appendix L1 "Living List – Discussion of H.248 signalling options for SCTP transport modes".

That living list material needs to be carefully reviewed and updated by the meeting.

4	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against TD 172R2 (WP 1/16) from the Sapporo, Japan, (30 June – 11 July 2014) meeting.

Change proposal #1:

[Begin Proposed Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.4]	Recommendation ITU-T H.248.4 (12/2009), Gateway contro protocol: Transport over Stream Control Transmission Protocol (SCTP).

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (07/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[IETF RFC 4960]	IETF RFC 4960 (2007), Stream Control Transmission Protocol.

[IETF RFC 5061]	IETF RFC 5061 (2007), Stream Control Transmission Protocol (SCTP) Dynamic Address Reconfiguration.

[IETF RFC 6525]	IETF RFC 6525 (2012), Stream Control Transmission Protocol (SCTP) Stream Reconfiguration.

{Editor's note (update 2014-11, 2013-10): following references might be needed

- draft-ietf-mmusic-sctp-sdp

- draft-ietf-tsvwg-sctp-dtls-encaps

- draft-ejzak-mmusic-msrp-usage-data-channel [b-IETF data-channel-msrp]

- draft-ietf-rtcweb-data-protocol

- H.248.93

- others

}

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

# 7	Models

…

## 7.3	Protocol stack model for mapping of SCTP streams to applications

The SCTP layer represents a SCTP association which offers multiple SCTP streams for application data traffic transport (Figure x1). The SCTP stream identifier could be considered as service access point for the upper-layer protocol (or more precisely: the SCTP stream ID represents the (SCTP)-service-access-point-address according to [ITU-T X.200]).

Figure x1 – Generic protocol stack model for mapping of SCTP streams to applications
(here at the example of SCTP transport mode "tunneled SCTP via DTLS/UDP")

Figure x2 illustrates example SCTP stream configurations for communication services WebRTC and telepresence:

Figure x2 – Example protocol stack model for communication services WebRTC and Telepresence

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix II
Example signalling scenarios for tunneled SCTP via DTLS/UDP
and interworking to non-SCTP based data transport

(This appendix does not form an integral part of this Recommendation)

## II.1	Example application for tunneled SCTP via DTLS/UDP: generic data channel

### II.1.1	Network model for interworking to non-SCTP based data transport

Figure II.1 illustrates a "half call" model with focus on an H.248 gateway and a peered user equipment (UE). An H.248 IP-IP gateway is located between access and core network level. The data channel (DC) uses the protocol stack segment SCTP/DTLS/UDP/IP as transport. The data channel is terminated in the H.248 MG (see Figure II.1), which implies the termination of protocols 'DTLS' and 'SCTP'.

Figure II.1 – Network model for H.248 IP access gateways (for half call only) and user plane stacks

Both protocols, DTLS and SCTP, are of type connection-oriented, hence there are correspondent bearer control protocol (BCP) procedures for the establishment of firstly a DTLS session and then subsequently a SCTP association.

NOTE – It is expected that there will be only a primary SCTP path used, thus, not any secondary SCTP path prepared (due to the fact of the underlying DTLS/UDP tunnel).

There will be initial application control signalling (such as SIP based call control signalling), which triggers the MGC to request DTLS and SCTP bearer establishment from the MG (see next sub-clauses).

### II.1.2	Example signalling at SIP and H.248 interfaces

A typical SIP-level SDP offer is illustrated in Table II.1:

Table II.1 – Example command encoding – Step (1) – (SIP) SDP Offer

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... m=application <UE_A_port_data> DTLS/SCTP webrtc-datachannel c=IN IP4 <UE_A_IP_addr_data> a=fmtp:webrtc-datachannel max-message-size=… # a=sctp-port:<UE_A_port_SCTP> a=dcmap:0 label="chat" ;subprotocol="MSRP” a=dcsa:0 accept-types:message/cpim text/plain text/html a=dcsa:0 path:msrps://<UE_A_authority>/<UE_A_session>;dc … a=setup:actpass a=connection:new a=fingerprint:SHA-256 <UE_A_DTLS_fingerprint_data> a=sendrecv… … | NOTE – Same SDP offer as in see Table II.4. SIP level signalling is always application aware (independent of H.248 application agnostic or application aware context configurations). The application here is indicated as "MSRP"-over-"webrtc-datachannel", using SCTP transport over a DTLS tunnel. |

{Editor’s note (2014-11): There's a pending syntax question with attribute a=sctp-port from [b-IETF sctp-sdp] concerning the characater between attribute and transport port value:
a) a=sctp-port:<UE_A_port_SCTP> ?
b) a=sctp-port=<UE_A_port_SCTP> ?
c) a=sctp-port <UE_A_port_SCTP> ?
The examples here are based on the assumption of the ":" delimiter symbol.}

{Editor's note (2014-07): the generic data channel (DC) case is interesting from H.248 perspective with regards to the indication of "application-agnostic":
Background - [b-IETF data-channel-sdpneg]:

##### 5.1.1.3.  subprotocol parameter

The 'subprotocol' parameter indicates which protocol the client

expects to exchange via the channel.  Subprotocol is a mandatory

parameter.

subprotocol-attr       =  "subprotocol=" labelstring

labelstring            =  text

text                   =  byte-string

[ACTION ITEM] The IANA registry to be used for the subprotocol

parameter is still to be determined.  It also needs to be determined

what the relationship is to existing registries and how to reference

already-existing protocols.

Below H.248 signalling examples uses the "-" (subprotocol="-”)… but such signallig needs still confirmation.}

The SIP-level SDP offer stimulates an H.248 transaction request from MGC to MG (Table II.2). The media description is application-agnostic (i.e., the MG will be not aware about the IP application protocol 'MSRP', as served user of the data channel):

Table II.2 – Example command encoding– Step (2) – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { Add = ip/$/$/$ { Media { Stream = 1 { LocalControl { ipdc/realm = <access realm>, Mode = Inactive, ... }, Local { v=0 c=IN IP6 $ m=application $ DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… a=sctp-port:- a=dcmap:0 … }, Remote { v=0 c=IN IP6 <UE_A_IP_addr_data> m=application <UE_A_port_data> DTLS/SCTP \ webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… a=sctp-port:<UE_A_port_SCTP> a=dcmap:0 } … } | Value "-" " in SDP attribute "a=sctp-port:" indicates to the MG, that the local SCTP port is set to be equal to the chosen local UDP port. {Editor's note (2014-07): Alternatives would have been - Local SCTP port resource management on MGC, that is MGC already fills the SCTP port values before sending the Add command request to the MG, or - Local SCTP port resource management on MG, that is MGC uses H.248.39’s new line wildcard to request MG to choose one SCTP port value in all related places in the m=, a=sctpmap, and a=data-channel lines. But the "-" seems to be preferable (as also documented in H.248.39 Rev} FIXTHIS |

{Editor's note (2014-07): to be fixed – provide description of above H.248 Command.request. Descripe usage of H.248.93 (DTLS).}

A possible example MG reply is indicated in Table II.3:

Table II.3 – Example command encoding – Step (3) – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [11.9.19.27]:4646 Transaction = 1 { Context = 101 { Add = ip/1/2/3 { Media { Stream = 1 { Local { v=0 c=IN IP6 <MGW_A_IP_addr_data> m=application <MGW_A_port_data> DTLS/SCTP \ webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… ## a=dcmap:0 }, Remote { v=0 c=IN IP6 <UE_A_IP_addr_data> m=application <UE_A_port_data> DTLS/SCTP \ webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… a=sctp-port:<UE_A_port_SCTP> a=dcmap:0 } … } | The MG confirms the request for an application-agnostic WebRTC data channel. |

The application-agnostic WebRTC data channel "SCTP stream/SCTP association/DTLS/UDP/IP" might be mapped on an application agnostic TCP bearer connection. The MG provides then an interworking function between SCTP messages and TCP segments. This is a feasible IWF, but conditional, e.g., dependent on further default parameter values of the SCTP layer ("parameters which were not signalled in-band or/and out-of-band"), single SCTP stream usage ("SCTP Stream 0 mapped to TCP", therefore SDP "a=dcmap:0" required at H.248 interface) and the actuall IP application protocol (e.g., not possible for MRSP, but feasible for legacy IP data applications).

## II.2	Example application for tunneled SCTP via DTLS/UDP: application-aware data channel service for WEBRTC

### II.2.1	Network model for interworking to non-SCTP based data transport and example application MSRP

Figure II.1 summarizes again the half-call model and bearer plane protocol stacks. The MG will be now aware about the the IP application protocol 'MSRP'. The data channel is terminated by the MG and the application data is using native, TCP-based transport as defined for MSRP. There might be an optional 'TLS' based transport security on top of TCP.

Figure II.2 – Network model for H.248 IP access gateways (for half call only) and user plane stacks – application-aware data channel service for WEBRTC for application 'MSRP'

The interworking between MSRP/DC/SCTP/DTLS/UDP and MSRP/(TLS/)TCP implies a fragmentation and reassembly service (see FIXTHIS_Reference).

### II.2.2	Example signalling at SIP and H.248 interfaces

#### II.2.2.1	Communication stacks overview and control plane building blocks

The scope is on SCTP in this Recommendation, hence the WebRTC data channel part of an overall WebRTC multimedia communcation service (see [b-ITU-T H.248.WEBRTC]). Figure II.3 outlines the overall layered protocol architecture and provides the references to relevant IETF technologies:

Figure II.3 – WebRTC data channel end-to-end communication stacks in general
(and for WEBRTC for application 'MSRP' in particular)

Explanations to Figure II.3:

| (1) | [b-IETF sctp-sdp] proposes how to negotiate SCTP-over-DTLS transport and number of SCTP streams. Example SDP offer (offering 16 data channel instances): m=application 54111 DTLS/SCTP webrtc-datachannel c=IN IP4 79.97.215.79 a=sctp-port=5000 a=fmtp:webrtc-datachannel max-message-size=100000 |
| --- | --- |
| (2) |  | [b-IETF data-channel-sdpneg] specifies how to use SDP offer/answer procedures for external negotiation of data channels, sub-protocols and sub-protocol specific attributes. It introduces the related "a=dcmap" and "a=dcsa" SDP attributes. |
| (3) |  | [b-IETF msrp-over-webrtc] specifies how to negotiate data channels for MSRP usage based on the data channel and sub-protocol external SDP negotiation introduced in [b-IETF data-channel-sdpneg]. |
| (4) |  | DC Control [b-IETF rtcweb-data-channel] describes two basic alternatives how to add and close data channel instances. These two alternatives are: - "In-band open": This is specified in [b-IETF rtcweb-data-protocol]. This alternative is based on the DATA_CHANNEL_OPEN and DATA_CHANNEL_ACK messages to open a channel on an unused SCTP stream. - "External negotiation": Applications use a signaling channel (which may also be a dedicated data channel instance) to declare/negotiate data channel instantiation, intended sub-protocol and sub-protocol related attributes; the DATA_CHANNEL_OPEN message is not used. |
| (5) |  | SCTP Stream Usage - One WebRTC data channel instance maps to one pair of unidirectional (incoming and outgoing) SCTP streams. - The number of SCTP streams may initially be negotiated out-of-band ("external negotiation") but may also be extended via in-band request (based on SCTP association reconfiguration procedure). |
| (6) |  | SCTP Association - [b-IETF sctp-dtls-encaps] describes "DTLS Encapsulation of SCTP Packets". - Single homed SCTP association endpoints. |

The WebRTC data channel handling from perspective of an H.248 gateway is outlined in Figure II.4.

Figure II.4 – Relevant H.248.x-series Recommendations in case of an WebRTC H.248 gateway

{Editor's note (2014-07): the data channel (DC) assignment to a particular H.248.x Recommendation is still open.}Editor’s note: The use of H.248.STGROUP in this context is an open issue that requires further study.

#### II.2.2.2	"WebRTC call" originating at user equipment

It has to be noted that all SDP and H.248 signalling structures are incomplete, only relevant information is indicated.

The following message sequence diagram (Figure II.5) shows the messages and message numbering scheme which is used by all example message flows in this example, which are related to UE A being the communication session originator.

Figure II.5 – MGC-MG H.248 Procedures – UE origination

The UE-embedded WEBRTC client originates a MSRP session across a WEBRTC data channel (Table II.4).

Table II.4 – Example command encoding – Step (1) – (SIP) SDP Offer

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... m=application <UE_A_port_data> DTLS/SCTP webrtc-datachannel c=IN IP4 <UE_A_IP_addr_data> a=sctp-port:<UE_A_port_SCTP> a=fmtp:webrtc-datachannel max-message-size=... a=dcmap:0; \ label="<UE_A_label>"; subprotocol="MSRP” a=dcsa:0 \ accept-types:message/cpim text/plain text/html a=dcsa:0 \ path:msrps://<UE_A_authority>/<UE_A_session>;dc … a=setup:actpass a=connection:new a=fingerprint:SHA-256 <UE_A_DTLS_fingerprint_data> a=sendrecv… … | {The basic WebRTC data channel is requested based on SDP elements from - [b-IETF sctp-sdp] and - [b-IETF data-channel-sdpneg]. The usage of this WebRTC data channel for the IP application protocol "MSRP" is achieved by profiling of the two SDP attributes "a=dcmap" and "a=dcsa" according to - [b-IETF data-channel-msrp]. |

{Editor's note (2014-07): to be fixed – describe the "call context" of such an example(!) SDP offer.}

The MGC prepares the MG #########, see Table II.5:

Table II.5 – Example command encoding– Step (2) – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { Add = ip/$/$/$ { Media { Stream = 1 { LocalControl { ipdc/realm = <access realm>, Mode = Inactive, ... }, Local { v=0 c=IN IP6 $ m=application $ DTLS/SCTP webrtc-datachannel a=sctp-port:- a=fmtp:webrtc-datachannel max-message-size=… a=dcmap:0; \ label="<UE_A_label>";subprotocol="MSRP” // Local descriptor does not contain further MSRP session related attributes (tbc) a=fingerprint:$ $ a=setup:passive }, Remote { v=0 c=IN IP6 <UE_A_IP_addr_audio_rtp> m=application <UE_A_port_data> DTLS/SCTP webrtc-datachannel a=sctp-port:<UE_A_port_SCTP> a=fmtp:webrtc-datachannel max-message-size=… a=dcmap:0; \ label="<UE_A_label>";subprotocol="MSRP” a=dcsa:0 path:msrps://<UE_A_authority>/\ <UE_A_session>;dc a=fingerprint:SHA-256 \ <UE_A_DTLS_fingerprint_data> } … Add = ip/$/$/$ { Media { Stream = 1 { ; NOTE 1 LocalControl { ipdc/realm = <core realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP4 $ m=message $ TCP/MSRP – a=setup:active } } } } }}} | {Editor's note (2014-07):to be fixed - comment important signalling elements.} {Editor's note (2014-07):to be fixed - comment important signalling elements – here the example of "TCP" as non-SCTP based data transport.} |
| NOTE 1 – A simple one stream per data channel approach is chosen for MSRP (or known IMS bearer protocol interworking) due to simplicity and to map it closely to equivalent procedures in RCS (Rich Communication Suite) world, where any new RCS service (chat, file transfer, image share) uses a new SIP dialog rather than using existing "audio + video" dialog or other RCS dialog. It may be noted that the one stream per data channel mapping would simplify the H.248 protocol usage greatly. However, there are other cases such as BFCP floor control along with "audio + video". If BFCP is done as data channel then also a separate data channel with just one stream for data channel and seperate audio/video streams could be created. So, even here one data channel one stream rule holds good. IMS is not planning to support bundling of data channel and audio/video (or audio, video bundling itself) into the same single DTLS association for other QoS reasons anyway. If multiple streams are needed then per data channel then it is most likely a non-IMS interworking then having an end to end SCTP tunnelled data channel makes sense in that case (see II.2 case above). |  |

{Editor's note (2014-07): to be fixed – provide description of above H.248 Command.request. 
a) Describe usage of H.248.93 (DTLS). 
b) Word smithing on Table NOTE 1 => (2014-11) a couple of electronic comments added}

A possible example MG reply is indicated in Table II.6:

Table II.6 – Example command encoding – Step (3) – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 1 { Context = <C1> { Add = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP6 <UE_A_IP_addr_data> m=application <UE_A_port_data> DTLS/SCTP \ webrtc-datachannel a=sctp-port:- a=fmtp:webrtc-datachannel max-message-size=… a=dcmap:0; \ label="<UE_A_label>";subprotocol="MSRP” // Local descriptor does not contain further MSRP session related attributes (tbc) a=fingerprint: SHA-256 \ <IP_A_DTLS_ fingerprint_data > a=setup:passive }, Remote { v=0 c=IN IP6 <UE_A_IP_addr_audio_rtp> m=application <UE_A_port_data> DTLS/SCTP webrtc-datachannel a=sctp-port:<UE_A_port_SCTP> a=fmtp:webrtc-datachannel max-message-size=… a=dcmap:0; \ label="<UE_A_label>";subprotocol="MSRP” a=dcsa:0 path:msrps://<UE_A_authority>/\ <UE_A_session>;dc a=fingerprint:SHA-256 \ <UE_A_DTLS_fingerprint_data> } } } }, Add = <IP_C> { Media { Stream = 1 { Local { v=0 c=IN IP4 <IP_C_IP_addr_data> m=message <IP_C_port_data> TCP/MSRP – a=setup:active } } } } }} } | {Editor's note (2014-07):to be fixed - comment important signalling elements.} |

{Editor's note (2014-07): to be fixed – provide description of above H.248 Command.reply.}

The remaining signalling steps (4) to (8) do not add substantial new information and are therefore omitted.

#### II.2.2.3	"WebRTC call" terminating at user equipment

The following message sequence diagram (Figure II.6) shows the messages and message numbering scheme which is used by all example message flows in this example, which are related to UE A being the communication session terminator.

Figure II.6 – MGC-MG H.248 Procedures – UE termination

## II.3	Example application for tunneled SCTP via DTLS/UDP: application-aware data channel service for TELEPRESENCE

<placeholder>

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Bibliography

[b-IETF RFC 4347]	IETF RFC 4347 (2006), Datagram Transport Layer Security.

[b-IETF sctp-sdp]	IETF draft-ietf-mmusic-sctp-sdp (2014), Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP).

[b-IETF data-channel-sdpneg]	IETF draft-ejzak-mmusic-data-channel-sdpneg (2014), SDP-based "SCTP over DTLS" data channel negotiation.

[b-IETF data-channel-msrp]	IETF draft-ejzak-mmusic-msrp-usage-data-channel (2014), MSRP over SCTP/DTLS data channels.

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

Appendix L1
Living List – Discussion of H.248 signalling options 
for SCTP transport modes

(This appendix does only contain working material.)

{Editor's note (2014-11 meeting): some initial material is drafted for the discussion of this topic.}

## L1.1	Problem statement

Similar situation as in case of bearer types "TCP" (H.248.89), "TLS" (H.248.90) and "DTLS" (H.248.93), whether the bearer control procedures (BCP) related to

- bearer establishment and

- bearer release

are based on

- pure SDP information elements (i.e., so-called package-less H.248 procedures) or

- usual H.248 signalling elements (i.e., package-defined properties, events and signals) or

- a mix of both.

The evaluation is also similar, but even more complicated due to multiple SCTP transport mode options.

## L1.2	Requirements

### L1.2.1	Requirements for pure SCTP protocol layer

See Appendix IV concerning requirements for the parameterization and usage of the SCTP layer as such.

### L1.2.2	Requirements for SCTP-based protocol stack segments

There are the combined protocol layers of "DTLS-over-SCTP" and "SCTP-over-DTLS-over-UDP" in case of the SCTP transport modes "transport secured SCTP" and "tunneled SCTP via DTLS/UDP".

Such protocol stack segments could be either handled as monolithic "super-bearer" connection from H.248 control perspective, or each protocol layer individually as usual. There are pros and cons for each approach (from H.248 point of view), and there are valid use cases for each approach. The consideration of an "atomic model" could simplify H.248 control concerning the establishment process, but might be nasty concerning protocol failure scenarios at individual layers due to side effects from one layer to the other layer. The individual control of protocol layers allows also to decouple the timelines of establishment and release procedures between the various layers.

### L1.2.3	Requirements for MG level interworking functions

Similar as for other bearer technologies, there are requirements to support at least the two fundamental IWF types of

- application-agnostic interworking (i.e., the MG is not aware of the application protocol on top of the particular SCTP transport mode), and

- application-aware interworking.

The IWF type requirement impacts the applicability of SDP signalling elements. E.g., in case of the "pure SDP approach" (1) would be wildcarding "-" required for application-agnostic ("and the resulting question whether such SDP wildcarding is already supported").

## L1.3	H.248 signalling options versus SCTP transport mode

Figure L1.1 recalls again the SCTP transport modes as in scope of this Recommendation:

Figure L1.1 – = copy of Figure 3 "SCTP transport modes"

### L1.3.1	Transport mode "native SCTP"

See Table L.1 with scope on a) establishment procedure only and focus on b) separated protocol layer model:

Table L.1 – Transport mode "native SCTP" – Signalling elements

| Protocol layer | H.248 package defined elements | defined SDP elements | Element which triggers the Lx-specific bearer establishment procedure |
| --- | --- | --- | --- |
| SCTP | H.248.SCTP: sctpbcc package? (still work in progress) | [b-IETF sctp-sdp]: - "m="-line proto=SCTP | H.248.93: - sctpbcc package: signal / event for outgoing / incoming SCTP bearer establishment procedure SDP: - combination of "m="-line proto codepoint plus SDP attribute "a=setup:"? (to be discussed) |

### L1.3.2	Transport mode "transport secured SCTP"

See Table L.2 with scope on a) establishment procedure only and focus on b) separated protocol layer model:

Table L.2 – Transport mode "transport secured SCTP" – Signalling elements

| Protocol layer | H.248 package defined elements | defined SDP elements | Element which triggers the Lx-specific bearer establishment procedure |
| --- | --- | --- | --- |
| DTLS | H.248.93: - tlsbcc package (reuse for DTLS) - other (D)TLS packages in case of additional DTLS control options. | [b-IETF sctp-sdp]: - "m="-line proto="SCTP/DTLS" | H.248.93: - tlsbcc package: signal / event for outgoing / incoming DTLS bearer establishment procedure SDP: - ???? |
| SCTP | H.248.SCTP: sctpbcc package? (still work in progress) | Already part of above "m="-line. [b-IETF sctp-sdp]: - additional SDP SCTP parameters? (TBD) | H.248.93: - sctpbcc package: signal / event for outgoing / incoming SCTP bearer establishment procedure SDP: - combination of "m="-line proto codepoint plus SDP attribute "a=setup:"? (to be discussed) |

### L1.3.3	Transport mode "tunneled SCTP via DTLS/UDP"

See Table L.3 with scope on a) establishment procedure only and focus on b) separated protocol layer model:

Table L.3 – Transport mode "tunneled SCTP via DTLS/UDP" – Signalling elements

| Protocol layer | H.248 package defined elements | defined SDP elements | Element which triggers the Lx-specific bearer establishment procedure |
| --- | --- | --- | --- |
| SCTP | H.248.SCTP: - sctpbcc package? (still work in progress) | [b-IETF sctp-sdp]: - "m="-line proto="DTLS/SCTP" [b-IETF data-channel-sdpneg]: - ??? [b-IETF data-channel-msrp]: - ??? | H.248.93: - sctpbcc package: signal / event for outgoing / incoming SCTP bearer establishment procedure SDP: - combination of "m="-line proto codepoint plus SDP attribute "a=setup:"? (to be discussed) |
| DTLS | H.248.93: - tlsbcc package (reuse for DTLS) - other (D)TLS packages in case of additional DTLS control options. | Already part of above "m="-line (NOTE 1) | H.248.93: - tlsbcc package: signal / event for outgoing / incoming DTLS bearer establishment procedure SDP: - ???? |
| UDP | None (due to connectionless nature) | - "m="-line proto=… (value with UDP-based transport indication) | not required |
| NOTE 1 – SDP codepoint "m="-line proto="UDP/DTLS" (see H.Sup14 / H.248.93) would not used for this SCTP transport mode. |  |  |  |

## L1.4	Observations / conclusions

FIXTHIS

[End Proposed Correction]

____________________