| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4705 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 2/16, 5/16 | Guilin, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.245 Logical Channel Update |  |  |  |
| Purpose: | Discussion |  |  |  |

1.	Introduction

This contribution discusses possible procedures for updating the WebRTC data channels on an existing logical channel.

2.	Discussion

At the February Joint Q2 and Q5/16 meeting text was introduced into H.TPS-SIG regarding sending an updated OpenLogicalChannel message on an existing logical channel to remove “CLUE” from the DataApplicationCapability. It was assumed that by resending an OpenLogicalChannel message for an already open logical channel that the characteristics of the channel would be updated. This would allow webrtc data channel to be added or removed from an existing DTLS/SCTP association without the need to re-establish the DTLS/SCTP association. However during the discussions it was noted that neither H.323 nor H.245 discussed or allowed this type of behaviour.

WebRTC allows an endpoint to add or remove data channels without the need to re-establish a DTLS/SCTP association. Therefore in order to support data channels in H.323 equivalent functionality is needed.

The closest procedure to the required behaviour is defined in H.245 Appendix V “The replacementFor procedure”. This allows an endpoint to signal that a codec change is occurring by sending a new OpenLogicalChannel with new parameters indicating that it is a replacement for an existing logical channel. The scope of this appendix appears only to cover codec changes and it indicates only one of the logical channels is active at a time. The new channel replaces the old one. There it seems the use of replacementFor is unsuitable for WebRTC datachannels as the existing data channels should be undisturbed whilst adding or removing datachannels.

Given that there are no existing procedures that can be used, a new procedure needs to be defined. There are two main possibilities:

- A generic approach where a mechanism is created that allows an endpoint to modify logical channels’ characteristics beyond those associated with WebRTC data channels.

- A specific approach where a mechanism is created only for updating WebRTC data channels.

These approaches could be signalled by a number of methods including using existing H.245 control messages or creating new Generic Messages, additional elements etc. The sub-sections below explore some possibilities:

## 2.1 Generic Approach Option 1 – A new “logicalChannelUpdate” element

An element could be introduced into the OpenLogicalChannel message either by updating the ASN.1 or via a GenericInformation parameter that indicates that the OpenLogicalChannel message is an update to an existing logical channel. Specific procedural text would need to be added to indicate the required behaviour from the endpoints. This would be mainly to indicate that sending an OpenLogicalChannel message on an existing logical channel is allowed where the “logicalChannelUpdate” element is used. The advantage of such a mechanism is that it could be used to change a myriad of parameters, however that is also a disadvantage as there may be unspecified behaviours or consequences. To minimise the possible changes it could be specified that an endpoint should not use this mechanism to change any elements that would result in a change of transport addresses or that it shouldn’t be used to change the DataType. Existing messages would be used perhaps with a new error code to indicate cases where updating was not possible.

## 2.2 Generic Approach Option 2 – A new updateLogicalChannel message

A new updateLogicalChannel message could be defined either via ASN.1 or via a H.245 Generic Message that is essentially a copy of the OpenLogicalChannel structure. Existing OpenLogicalChannel ASN.1 structures that are unlikely to be used (e.g. transport related structures) could be removed. Acknowledgement and Rejection messages would also be needed. As per clause 2.1 this could be quite powerful however it would take significant work to ensure the correct structures and procedures were added.

## 2.3 Specific Approach Option 1 –  Specific WebRTC datachannel update procedure

Rather than adding new elements or messages to support the addition or deletion of data channels the procedures around the support of WebRTC data channel could be updated to indicate that if an endpoint supports WebRTC data channel it must also support receiving an updated OpenLogicalChannel on an existing logical channel. The updated OpenLogicalChannel MUST be identical except for the DataChannel element which may be updated to add or remove DataChannelProfiles. This would avoid additional syntax to what has already been defined and narrows the scope of the update.

## 2.4 Specific Approach Option 2 –  Specific WebRTC datachannel update element

A new element could be defined to indicate that data channels are added or removed. This would require the same behaviour as in clause 2.3 but also introduce an update to the OpenLogicalChannel message to indicate that it is updating WebRTC datachannels. It doesn’t appear to offer any advantage over clause 2.3 as the change would still be signalled via an OpenLogicalChannel.

## 2.5 Specific Approach Option 3 –  Specific WebRTC datachannel message

A new H.245 message could be defined either via ASN.1 update or generic message definition that indicates that a DTLS/SCTP association for WebRTC data channel has changed. This message would contain a reference to the applicable logical channel number and an updated DataChannel element. Acknowledgement and Rejection messages would also need to be defined. This has the advantage of being specific to WebRTC. It would not alter existing OpenLogicalChannel procedures and a receiving endpoint would not have to parse any more data than is necessary.

3.	Proposal

Given the above options the Contributions believe that options in clause 2.3 or 2.5 would be the most suitable.

In order to incorporate the changes necessary to support addition/deletion of data channels according to the selected option would be needed to H.245 Annex U and H.TPS-SIG.

__________________________

____________________