| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4729 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.SIPREC: update of RTP terminology |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal related to RTP terms.

1	Discussion and conclusions

H.248.SIPREC uses the two RTP terms of an RTP stream and RTP session. Both terms were clarified in the meanwhile by the IETF and could be refered by H.248 Recommendations. See parallel AVD-4684 related to H.248.RTPMUX. This contribution is based on that work, i.e. reuses the proposals of AVD-4684.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4667 as input draft to this meeting.

Change proposal #1:

[Begin Proposed Correction]

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Communication Session (CS) [b-IETF RFC 6341]: A session created between two or more SIP User Agents (UAs) that is the subject of recording.

3.1.2	Recording-aware User Agent (UA) [b-IETF RFC 7245]: A SIP User Agent that is aware of SIP extensions associated with the CS.  Such extensions may be used to notify the recording-aware UA that a session is being recorded, or by a recording-aware UA to express preferences as to whether a recording should be started, paused, resumed, or stopped.

3.1.3	Recording Metadata [b-IETF RFC 7245]: The metadata describing the CS that is required by the SRS. This will include, for example, the identities of users that participate in the CS and dialog state. Typically, this metadata is archived with the Replicated Media at the SRS. The recording metadata is delivered in real-time to the SRS.

3.1.4	Recording Session (RS) [b-IETF RFC 6341]: The SIP session created between an SRC and SRS for the purpose of recording a Communication Session.

3.1.5	Recording-unaware User Agent (UA) [b-IETF RFC 7245]: A SIP User Agent that is unaware of SIP extensions associated with the CS. Such a recording- unaware UA will be notified that a session is being recorded or will express preferences as to whether a recording should be started, paused, resumed, or stopped via some other means that is out of scope for the SIP media recording architecture.

3.1.6	Replicated Media [b-IETF RFC 7245]: A copy of the media that is associated with the CS, was created by the SRC, and was sent to the SRS. It may contain all the media associated with the CS (e.g., audio and video) or just a subset (e.g., audio). Replicated Media is part of the Recording Session.

3.1.x1	RTP session (clause 2.2.2/[b-IETF draft taxonomy]|[b-ITU-T H.248.RTPMUX]): An RTP Session is an association among a group of Participants communicating with RTP.  It is a group communications channel which can potentially carry a number of RTP Streams. Within an RTP Session, every Participant can find meta-data and control information (over RTCP) about all the RTP Streams in the RTP Session.  The bandwidth of the RTCP control channel is shared between all Participants within an RTP Session.

NOTE – [b-ITU-T H.248.57], clause 3.1.3 provides following definition:

RTP session: An RTP session comprises a single RTP flow and an optional RTCP flow.

It could be concluded that the "H.248.57 RTP session" is synonym to RTP stream in general..

3.1.x2	RTP stream (clause 2.1.10/[b-IETF draft taxonomy]|[b-ITU-T H.248.RTPMUX]): A stream of RTP packets containing media data, source or redundant. The RTP stream is identified by an SSRC belonging to a particular RTP session.

3.1.7	Session Recording Client (SRC) [b-IETF RFC 6341]: A Session Recording Client (SRC) is a SIP User Agent (UA) that acts as the source of the recorded media, sending it to the SRS. An SRC is a logical function. Its capabilities may be implemented across one or more physical devices.  In practice, an SRC could be a personal device (such as a SIP phone), a SIP Media Gateway (SMG), a Session Border Controller (SBC), or a SIP Media Server (SMS) integrated with an Application Server (AS). This specification defines the term "SRC" such that all such SIP entities can be generically addressed under one definition. The SRC provides metadata to the SRS.

NOTE – The abbreviations for SIP media gateway and SIP media server were extended in order not to be confused with the correspondent H.248 entities.

3.1.8	Session Recording Server (SRS) [b-IETF RFC 6341]: A Session Recording Server (SRS) is a SIP User Agent (UA) that is a specialized media server or collector that acts as the sink of the recorded media. An SRS is typically implemented as a multi-port device that is capable of receiving media from multiple sources simultaneously. An SRS is the sink of the recorded session metadata.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	SRC-capable H.248 gateway: an ITU-T H.248 gateway with partial or complete support of the Session Recording Client (SRC) function.

NOTE – The notion of 'partial' because SIP-based call control is streactly speaken out of scope of the definition of H.248 gateways.

3.2.2	SRS-capable H.248 gateway: an ITU-T H.248 gateway with partial or complete support of the Session Recording Server (SRS) function.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

### 7.1.2	H.248 context model with SRC-to-SRS interface

SIP-based media recording covers multi-party, multimedia communication services in its entirety. Figure 4 illustrates the H.248 context model for the example of a two-party, dual media (audio, video) communication service between parties A and B.

H.248 modeling of the communication session:

The two RTP streams (definition see clause 3.1.x2) for audio and video are mapped on two H.248 streams S1 and S2, i.e., the communication session is given by the two dual-stream-per-termination H.248 terminations TA and TB. Each H.248 stream contains two H.248 flow components for RTP and RTCP packet traffic.

H.248 modeling of the recording session:

Only the media plane traffic component of the recording session is considered here. There are two options for modelling the SRC-to-SRS interface: either the media traffic of both traffic directions could be merged or should be forwarded separately to the SRS?

The merge option might be only feasible for media type 'audio' but not for other media types. Thus, the general case would be separated forwarding, as illustrated in Figure 4. There are then two further dual-stream-per-termination H.248 terminations TC and TD. However, there is only egress traffic on these H.248 terminations and an RTCP flow in reverse direction, from SRS-to-SRC could be excluded.

Figure 4 – H.248 context model with SRC-to-SRS interface (example)

It has to be noted that above H.248 context model does only indicate an example (out of multiple options) and does not define the SRC-to-SRS interface in general.

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

Appendix I

Use case specific capability sets – Two examples

(This appendix does not form an integral part of this Recommendation)

## I.1	Overview

Profile content is mainly use case dependent as outlined in clause 1. In order to demonstrate profile specification guidelines in clause 9, this Recommendation considers two exemplary use cases:

- Capability set 'A' (CSA):

  - H.248 MG type: an IP-to-IP media gateway,

  - use cases [b-IETF RFC 6341] in scope:

    - use case (1) "full-time recording" service,

  - communication session:

    - two party conversational service,

    - two media types (audio, video),

    - bidirectional RTP sessions (definition see clause 3.1.x1) with RTCP,

    - RTP multiplexing: none,

    - RTP topologies: B2BRE and RTP translator types

  - recording session "media component",

    - "stereo mode", i.e., separate RTP sessions (i.e., UDP transport connections) per media type and per communication party,

    - RTCP: only in SRC-to-SRS direction,

    - performance monitoring: H.248 statistics with regards to the IP layer traffic volume,

  - media analytics support: none.

- Capability set 'B' (CSB):

  - FIXTHIS …,

  - …

[End Proposed Correction]

Change proposal #4:

[Begin Proposed Correction]

Bibliography

[b-ITU-T H.248.RTPMUX]	Recommendation ITU-T H.248.RTPMUX (##/2016), Gateway control protocol: H.248 support for RTP multiplexing.

[b-IETF RFC 6341]	IETF RFC 6341 (08/2011), Use Cases and Requirements for SIP-Based Media Recording (SIPREC).

[b-IETF RFC 7245]	IETF RFC 7245 (05/2014), An Architecture for Media Recording Using the Session Initiation Protocol.

[b-IETF siprec-prot]	IETF draft-ietf-siprec-protocol (201#), Session Recording Protocol. 
<http://tools.ietf.org/wg/siprec/draft-ietf-siprec-protocol/>

[b-IETF siprec-meta]	IETF draft-ietf-siprec-metadata (201#), Session Initiation Protocol (SIP) Recording Metadata. 
<http://tools.ietf.org/wg/siprec/draft-ietf-siprec-metadata/>

[b-IETF siprec-flow]	IETF draft-ietf-siprec-callflows (201#), Session Initiation Protocol (SIP) Recording Call Flows. 
<http://tools.ietf.org/wg/siprec/draft-ietf-siprec-callflows/>

[b-IETF siprec-msrp]	IETF draft-yan-siprec-msrp-recording (201#), Overview for MSRP Recording based on SIPREC. 
<http://tools.ietf.org/id/draft-yan-siprec-msrp-recording-01.txt>

[IETF draft taxonomy]	IETF draft-ietf-avtext-rtp-grouping-taxonomy (XX/2015), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

[End Proposed Correction]

____________________