| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 347 R2 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 23 May – 3 June 2016 |
| TD |  |  |
| Source: | Editor H.248.77 |  |
| Title: | H.248.77 "Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures" (Rev.): Output draft (Geneva, 23 May – 3 June 2016) |  |

1.	Introduction

This document contains the Editor's output draft for H.248.77 "Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures" (Revised).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.77rev Ed. 0.1 | Initial baseline text for revised H.248.77 based on ITU-T H.248.77 (09/2010) and AVD-4619 from the Seoul, 3 – 7 November 2014 meeting. |
| H.248.77rev Ed. 0.2 | Input draft to the Geneva, 9-20 February 2015 meeting. TD 229/WP1: http://itu.int/md/T13-SG16-150209-TD-WP1-0229 |
| H.248.77rev Ed. 0.3 | Input draft for the Guilin, 8 - 12 June 2015 meeting. |
| H.248.77rev Ed. 0.4 | Output draft from the Chengdu, 8 - 12 June 2015 meeting. Changes due to: AVD-4676 |
| H.248.77rev Ed. 0.5 | Input draft for the Geneva, 12-23 October 2015 meeting. TD 287/WP1 – http://itu.int/md/T13-SG16-151012-TD-WP1-0287 |
| H.248.77rev Ed. 0.6 | Output draft from the Geneva, 12-23 October 2015 meeting. Changes due to: C.913. |
| H.248.77rev Ed. 0.7 | Input draft for the Geneva, 23 May – 3 June 2016 meeting. TD 347/R1 - http://www.itu.int/md/T13-SG16-160523-TD-WP1-0347 |
| H.248.77rev Ed. 0.8 | Output draft from the Geneva, 23 May – 3 June 2016 meeting. Changes due to: C.1062 and C.1126r1. |

CONTENTS

Page

1 Scope 1 1.1 Connection model 2 2 References 2 3 Definitions 3 3.1 Terms defined elsewhere 3 3.2 Terms defined in this Recommendation 3 4 Abbreviations and acronyms 4 5 Conventions 4 6 Secure RTP package 5 6.1 Properties 5 6.1.1 Supported Encryption Transforms 5 6.1.2 Supported Authentication Transforms 5 6.1.3 Key Management Scheme 6 6.1.4 Key Lifetime Expiry Behaviour 6 6.2 Events 7 6.2.1 Master Key Expiry 7 6.3 Signals 8 6.4 Statistics 8 6.4.1 Number of Replayed Packets 8 6.4.2 Number of Authentication Failures 8 6.4.3 Sent SRTP Packets Protected by Master Key 9 6.4.4 Sent SRTCP Packets Protected by Master Key 9 6.4.5 Received SRTP Packets Protected by Master Key 9 6.4.6 Received SRTCP Packets Protected by Master Key 9 6.5 Error codes 10 6.6 Procedures 10 6.6.1 Determining cryptographic capabilities 10 6.6.2 Key management 10 6.6.3 Master key lifetime 11 6.6.4 Logging of security violations 12 7 Key management using SDP security descriptions 12 7.1 Overspecification of SRTP parameters and multiple keys 13 7.2 Wildcarding of SRTP parameters 13 7.3 Interoperability with offer/answer-based implementations 14 7.4 SDES and SRTP cryptographic contexts 14 7.5 Mapping of master keys for sent packets and received packets statistics 15 8 Key management using DTLS-SRTP 16 8.1 Key management indication 16 8.2 Bearer plane connection for "DTLS-SRTP" 16 8.2.1 Protocol stack and DTLS usage 17 8.2.2 Reuse of DTLS connection by other applications 17 8.2.3 Lifetime of DTLS session and DTLS connection 17 8.2.4 Lifetime of DTLS master secret and SRTP master key 18 8. 3 Multi key management operation 18 9 Security considerations 18 9.1 Relation to key management scheme "SDES" 19 9.2 Relation to key management scheme "DTLS-SRTP" 19 Appendix I Example call flows for key management scheme “SDES” 20 I.1 Initial session setup using SDP security descriptions 20 I.2 MG1's key is about to expire 23 I.3 Auditing SRTP capabilities 24 I.4 Auditing of SRTP Statistics 24 Appendix II Sample use-cases of SRTP bearer encryption 26 II.1 Use-case #1: ITU-T H.248 MG for peering IP and circuit-switched networks 26 II.2 Use case #2: ITU-T H.248 MG for peering IP networks 26 II.3 Use case #3: Transparent SRTP forwarding 27 Appendix III Example call flows for key management scheme "DTLS-SRTP" 1 III.1 Initial session setup using DTLS-SRTP based key exchange 1 Bibliography 1

List of Tables

Page

Table 1 – Wildcarding of SDP security descriptions 14

List of Figures

Page

Figure 1 – Two-termination context with a SRTP termination 2 Figure II.1 – Use case #1: SRTP to circuit-switched ITU-T H.248 MG 26 Figure II.2 – Use case #2: SRTP to RTP ITU-T H.248 MG 27 Figure II.3 – Use case #3: ITU-T H.248 MG with transparent SRTP forwarding 28

Draft revised Recommendation ITU-T H.248.77

Gateway control protocol: Secure real-time transport protocol (SRTP)
package and procedures

AAP Summary

[To be provided before Consent]

Summary

Recommendation ITU-T H.248.77 defines a new ITU-T H.248 package, the secure RTP package. In addition, this Recommendation covers a set of procedures related to SRTP key management. The combination of package and procedures allows a MGC to control the use of secure RTP (SRTP) by a MG.

This revision …

Keywords

[Mandatory material]

# 1	Scope

The secure real-time transport protocol (SRTP) is an RTP profile that provides confidentiality, message authentication and replay protection to RTP and RTCP sessions. The secure RTP package allows a MGC to control the use of SRTP by a MG. This package is defined in detail in clause 6.

By itself, the secure RTP package is incomplete, as it does not provide procedures for key management. Instead, it is designed to rely on existing key-management schemes (see also [b-IETF RFC 7202]). [b-IETF RFC 5479] provides an example selection of key-management protocol options for SRTP in "SIP networks".

Clause 7 provides procedures for the use of one such key-management scheme: SDP security descriptions.

Clause 8 provides procedures for the use of one such key-management scheme: DTLS-SRTP.

Several reasons exist why this Recommendation is required, in addition to the existing (usually SDP-based) SRTP key-management schemes. The most significant of which are listed below:

–	Most existing SDP key-management schemes rely on the SDP offer/answer model (see [b-IETF RFC 3264]). However, the offer/answer model is not used in ITU-T H.248 as it does not fit the nature of the connection between an ITU-T H.248 MGC and a MG.

–	Existing SDP key-management schemes do not contain procedures relating to parameter overspecification and wildcarding, which are unique to ITU-T H.248.

–	The limited lifetime of SRTP master keys calls for mechanisms for handling master key expiry. The existing mechanisms cannot be used in ITU-T H.248.

–	The SRTP package allows explicit control over the key-management scheme employed, allowing easy interoperability with, and migration to future schemes.

–	The SRTP package allows an MGC to audit the SRTP capabilities of an MG through the use of the packages descriptor and the properties of the new package.

–	The SRTP package allows an MGC to collect statistics regarding the number of security violations encountered by the MG, and the volume of SRTP traffic it processed.

The scope of the (09/2010) -edition of this Recommendation is limited to use-cases in which a MG applies SRTP procedures, as described in clause 3.3 of [IETF RFC 3711], to the SRTP packets it sends and receives. Use-cases in which the MG handles SRTP packets without using those procedures (e.g., transparent forwarding, storage in encrypted form, etc.) are intentionally left out of this Recommendation.

This revision (##/2016) extends the scope for further SRTP key management schemes, such as DTLS-SRTP according to [b-IETF RFC 5763] and [IETF RFC 5764].

## 1.1	Connection model

All protocol elements and procedures described in this Recommendation are limited to the extent of a single ITU-T H.248 termination. In addition, no assumptions are made regarding either the lower layer protocols beneath the SRTP level or the upper layer protocols/codecs being carried by the SRTP. This allows the use of the Recommendation's procedures in various connection models and use-cases (e.g., a SRTP enabled announcement server, a SRTP to RTP translator, etc.).

Figure 1 details the generic connection-model where a SRTP-enabled termination is connected to a single other termination (either SRTP-enabled or not). The generalization to any number of terminations is trivial.

Figure 1 – Two-termination context with a SRTP termination

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.8]	Recommendation ITU-T H.248.8 (03/2013), Gateway control protocol: Error code and service change reason description.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (07/2008), Gateway control protocol: Statistic conditional reporting package.

[ITU-T H.248.49]	Recommendation ITU-T H.248.49 (08/2007), Gateway control protocol: Session description protocol RFC and capabilities packages.

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (10/2014), Gateway control protocol: ITU-T H.248 packages for control of transport security using transport layer security (TLS).

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (10/2014), Gateway control protocol: ITU-T H.248 support for control of transport security using the datagram transport layer security (DTLS) protocol.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 3711]	IETF RFC 3711 (2004), The Secure Real-time Transport Protocol (SRTP).

[IETF RFC 4568]	IETF RFC 4568 (2006), Session Description Protocol (SDP) Security Descriptions for Media Streams.

[IETF RFC 4572]	IETF RFC 4572 (2006), Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP).

[IETF RFC 5246]	IETF RFC 5246 (2008), The Transport Layer Security (TLS) Protocol Version 1.2.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 6347]	IETF RFC 6347 (2012), Datagram Transport Layer Security Version 1.2.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	RTP session (clause 2.2.2/[b-IETF RFC 7656]|[b-ITU-T H.248.95]): An association among a group of participants communicating with RTP. It is a group communications channel which can potentially carry a number of RTP Streams. Within an RTP Session, every Participant can find meta-data and control information (over RTCP) about all the RTP Streams in the RTP Session. The bandwidth of the RTCP control channel is shared between all Participants within an RTP Session.

3.1.2	SRTP cryptographic context [IETF RFC 3711]: The set of cryptographic state information that an SRTP sender or receiver must maintain per SRTP session participant.

NOTE 1 – This term is often abbreviated as "crypto context".

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	decrypting master key: A SRTP master key used to decrypt and authenticate SRTP packets received by the MG.

3.2.2	encrypting master key: A SRTP master key used to encrypt and authenticate SRTP packets sent by the MG.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AES | Advanced Encryption Standard |
| --- | --- |
| DTLS | Datagram Transport Layer Security |
| HMAC | Keyed-Hash Message Authentication Code |
| IP | Internet Protocol |
| IPSec | IP Security |
| ISDN | Integrated Services Digital Network |
| L1 | Layer 1 (of the Open Systems Interconnection model – the physical layer) |
| L2 | Layer 2 (of the Open Systems Interconnection model – the data link layer) |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| MKI | Master Key Identifier |
| NAPT | Network Address and Port Translation |
| NAT | Network Address Translation |
| PSTN | Public Switched Telephone Network |
| ROC | Rollover Counter |
| RTCP | RTP Control Protocol |
| RTP | Real-time Transport Protocol |
| SDES | Session Description Protocol Security Descriptions |
| SDP | Session Description Protocol |
| SHA1 | Secure Hash Algorithm 1 |
| SRTCP | Secure RTCP |
| SRTP | Secure RTP |
| SSRC | Synchronization Source |
| TDM | Time Division Multiplexing |
| TLS | Transport Layer Security |
| UDP | User Datagram Protocol |
| WebRTC | Web Real-Time Communications |

# 5	Conventions

The names of ITU-T H.248 descriptors are always capitalized, for example, Streams and Local Descriptor.

The names of ITU-T H.248 properties, events, signals and parameters appear in the text in italics, for example ReserveValue.

All error codes appearing in this Recommendation are described in [ITU-T H.248.8] and [ITU-T H.248.49].

# 6	Secure RTP package

|  | Package Name: | Secure RTP |
| --- | --- | --- |
|  | Package ID: | srtp (0x0107) |
|  | Description: | This package defines elements that allow the MGC to control an MG's use of the SRTP profile. Version 2 adds support for key management via DTLS for SRTP [IETF RFC 5764]. |
|  | Version: | 2 |
|  | Extends: | None |

## 6.1	Properties

### 6.1.1	Supported Encryption Transforms

|  | Property Name: | Supported Encryption Transforms |
| --- | --- | --- |
|  | Property ID: | set (0x0001) |
|  | Description: | This property declares the set of encryption transforms that can be used by SRTP sessions. |
|  | Type: | Sub-list of Enumeration |
|  | Possible values: | Each item in the list can be one of: "NULL" (0x0000): The NULL Cipher "AES_CM_128" (0x0001): AES in counter mode with a 128-bit key "AES_CM_192" (0x0002): AES in counter mode with a 192-bit key "AES_CM_256" (0x0003): AES in counter mode with a 256-bit key "AES_F8_128" (0x0004): AES in f8 mode with a 128-bit key "AES_F8_192" (0x0005): AES in f8 mode with a 192-bit key "AES_F8_256" (0x0006): AES in f8 mode with a 256-bit key |
|  | Default: | Provisioned |
|  | Defined in: | TerminationState |
|  | Characteristics: | ReadOnly |

### 6.1.2	Supported Authentication Transforms

|  | Property Name: | Supported Authentication Transforms |
| --- | --- | --- |
|  | Property ID: | sat (0x0002) |
|  | Description: | This property declares the set of authentication transforms that can be used by SRTP sessions. |
|  | Type: | Sub-list of Enumeration |
|  | Possible values: | Each item in the list can be one of: "NULL" (0x0000):The NULL authentication algorithm "HMAC_SHA1_80" (0x0001):HMAC-SHA1 with an 80-bit tag "HMAC_SHA1_32" (0x0002):HMAC-SHA1 with a 32-bit tag |
|  | Default: | Provisioned |
|  | Defined in: | TerminationState |
|  | Characteristics: | ReadOnly |

### 6.1.3	Key Management Scheme

|  | Property Name: | Key Management Scheme |
| --- | --- | --- |
|  | Property ID: | km (0x0003) |
|  | Description: | This property controls the key management scheme that will be used for supplying the SRTP parameters and keys |
|  | Type: | Enumeration |
|  | Possible values: | "None" (0x0000): No key management will be used. "SDES" (0x0001):SDP security descriptions [IETF RFC 4568] "DTLS-SRTP" (0x0002):DTLS-SRTP [IETF RFC 5764] |
|  | Default: | "None", unless provisioned otherwise |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

### 6.1.4	Key Lifetime Expiry Behaviour

|  | Property Name: | Key Lifetime Expiry Behaviour |
| --- | --- | --- |
|  | Property ID: | kleb (0x0004) |
|  | Description: | This property indicates which actions should be taken upon the expiry of the encrypting master key. The MG triggers key lifetime expiry when it determines that it has used the SRTP master key for the maximal number of packets allowed (by default 248 SRTP and/or 231 SRTCP packets; this value can be lowered through key-management). See clause 6.6.3 and clause 9.2 of [IETF RFC 3711] for further information regarding master key lifetime expiry. |
|  | Type: | Enumeration |
|  | Possible values: | "DROP" (0x0000):Do not close SRTP session, drop all packets. If the property srtp/km is set to DTLS-SRTP then the DTLS connection is kept open. "BYE" (0x0001):Close SRTP session, send SRTCP BYE. If the srtp/km property is set to DTLS-SRTP then the DTLS connection shall be closed with a fatal alert. "RENEW-KEYS" (0x0002): This value is only applicable if the srtp/km property is set to DTLS-SRTP. The related DTLS procedure is triggered in the MG to renew the DTLS master secret and derive new SRTP/SRTCP master keys. For DTLS version 1.2 or lower a renegotiation (term see [b-IETF tls-terms]) will be used. |
|  | Default: | "DROP", unless provisioned otherwise |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

## 6.2	Events

{Editor's note (2014-11): any impact on events by DTLS-SRTP based key management?}

### 6.2.1	Master Key Expiry

{Editor's note (2015-06): analyis of DTLS-SRTP applicability (see also discussion/framework in AVD-4744), - results & comments:

Item#1: wrt "watermark threshold crossed" => might not be relevant for the MGC in case of DTLS-SRTP, as renegotiating new SRTP-master-keys does not involve the MGC. That's just an observation, event can be kept applicable for DTLS-SRTP as well?

Status: open, to be clarified.

}

|  | Event Name: | Master Key Expiry |
| --- | --- | --- |
|  | Event ID: | mke (0x0001) |
|  | Description: | This event allows the MGC to be notified when the encrypting SRTP master key is about to expire (watermark threshold crossed) or has already expired. As the lifetime is media-stream specific, when multiple streams are defined on a termination, this event shall be notified on a specific stream only. NOTE – If the watermarks are set to 0, notification will be sent only upon master key expiration. |

#### 6.2.1.1	EventsDescriptor Parameters

##### 6.2.1.1.1	SRTP Watermark

|  | Parameter Name: | SRTP Watermark |
| --- | --- | --- |
|  | Parameter ID: | rtpw (0x0001) |
|  | Description: | The number of SRTP packets that the master key can still support when the event is first notified. |
|  | Type: | Double (Note) |
|  | Optional: | Yes |
|  | Possible Values: | Any non-negative value. |
|  | Default: | 0, unless provisioned otherwise |

NOTE – The maximal master key lifetime is 248 SRTP packets and 231 SRTCP packets. Therefore, the SRTP Watermark and SRTCP Watermark parameters are of type Double and Unsigned Integer, respectively.

##### 6.2.1.1.2	SRTCP Watermark

|  | Parameter Name: | SRTCP Watermark |
| --- | --- | --- |
|  | Parameter ID: | rtcpw (0x0002) |
|  | Description: | The number of SRTCP packets that the master key can still support when the event is first notified. |
|  | Type: | Unsigned Integer (Note) |
|  | Optional: | Yes |
|  | Possible Values: | Any non-negative value. |
|  | Default: | 0, unless provisioned otherwise |

NOTE – The maximal master key lifetime is 248 SRTP packets and 231 SRTCP packets. Therefore, the SRTP Watermark and SRTCP Watermark parameters are of type Double and Unsigned Integer, respectively.

#### 6.2.1.2	ObservedEventsDescriptor Parameters

##### 6.2.1.2.1	Key Expired

|  | Parameter Name: | Key Expired |
| --- | --- | --- |
|  | Parameter ID: | ke (0x0001) |
|  | Description: | The parameter indicates whether, at the time of notification, the master key is still valid or has already expired. |
|  | Type: | Boolean |
|  | Optional: | Yes |
|  | Possible Values: | True:The number of SRTP and SRTCP packets has met the master key lifetime, i.e., the key has already expired. False: The number of SRTP and SRTCP packets is still within the master key's lifetime. |
|  | Default: | False |

## 6.3	Signals

None.

## 6.4	Statistics

### 6.4.1	Number of Replayed Packets

|  | Statistic Name: | Number of Replayed Packets |
| --- | --- | --- |
|  | Statistic ID: | replay (0x0001) |
|  | Description: | This statistic logs the number of received packets that have been judged to be replayed and discarded, according to clause 3.3 of [IETF RFC 3711], since the instantiation of the statistic. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 6.4.2	Number of Authentication Failures

|  | Statistic Name: | Number of Authentication Failures |
| --- | --- | --- |
|  | Statistic ID: | authfail (0x0002) |
|  | Description: | This statistic logs the number of packets that have failed authentication and been discarded, according to clause 3.3 of [IETF RFC 3711], since the instantiation of the statistic. |
|  | Type: | Double |
|  | Possible values: | Any non-negative value |
|  | Level: | Either |

### 6.4.3	Sent SRTP Packets Protected by Master Key

|  | Statistic Name: | Sent SRTP Packets Protected by Master Key |
| --- | --- | --- |
|  | Statistic ID: | srpk (0x0003) |
|  | Description: | This statistic logs the number of sent SRTP packets that were protected by each of the current master key(s). |
|  | Type: | Sub-list of Double. Each element in the list corresponds to one of the encrypting master keys. The mapping between keys and list positions depends on the key management scheme employed. |
|  | Possible values: | Any non-negative value |
|  | Level: | Stream |

### 6.4.4	Sent SRTCP Packets Protected by Master Key

|  | Statistic Name: | Sent SRTCP Packets Protected by Master Key |
| --- | --- | --- |
|  | Statistic ID: | scpk (0x0004) |
|  | Description: | This statistic logs the number of sent SRTCP packets that were protected by each of the current master key(s). |
|  | Type: | Sub-list of Unsigned Integer. Each element in the list corresponds to one of the encrypting master keys. The mapping between keys and list positions depends on the key management scheme employed. |
|  | Possible values: | Any non-negative value |
|  | Level: | Stream |

### 6.4.5	Received SRTP Packets Protected by Master Key

|  | Statistic Name: | Received SRTP Packets Protected by Master Key |
| --- | --- | --- |
|  | Statistic ID: | rrpk (0x0005) |
|  | Description: | This statistic logs the number of received SRTP packets that were protected by each of the current master key(s). |
|  | Type: | Sub-list of Double. Each element in the list corresponds to one of the decrypting master keys. The mapping between keys and list positions depends on the key management scheme employed. |
|  | Possible values: | Any non-negative value |
|  | Level: | Stream |

### 6.4.6	Received SRTCP Packets Protected by Master Key

|  | Statistic Name: | Received SRTCP Packets Protected by Master Key |
| --- | --- | --- |
|  | Statistic ID: | rcpk (0x0006) |
|  | Description: | This statistic logs the number of received SRTCP packets that were protected by each of the current master key(s). |
|  | Type: | Sub-list of Unsigned Integer. Each element in the list corresponds to one of the decrypting master keys. The mapping between keys and list positions depends on the key management scheme employed. |
|  | Possible values: | Any non-negative value |
|  | Level: | Stream |

## 6.5	Error codes

None.

## 6.6	Procedures

{Editor's note (2014-11): any impact on procedures by DTLS-SRTP based key management? For instance: 
procedure: 	IF srtp/km = "DTLS-SRTP 
	THEN srtp/set = only {NULL, AES_CM_128} }

{Editor's note (2015-06): analysis of DTLS-SRTP applicability (see also discussion/framework in AVD-4744), - results & comments: 
=> YES, If srtp/km = DTLS-SRPT, then other properties or propertiy values might not be applicable!}

{Editor's note (2015-10): Proposal: srtp/set and srtp/sat are only applicable for srtp/km =  "SDES". For srtp/km = "DTLS-SRTP" the H.248.93(DTLS) poperty dtlscn/dssp is used.}

### 6.6.1	Determining cryptographic capabilities

The MGC can determine the cryptographic transforms that an MG supports for the “SDES” key management scheme by auditing the value of the Supported Encryption Transforms (set) and Supported Authentication Transforms (sat) properties. Usually, these properties are only available on the Root termination, and convey the cryptographic capabilities of the MG as a whole. However, it is possible that some use-cases will call for the support of these properties on non-Root terminations. One example would be a case where different terminations have different cryptographic capabilities.

The Supported Encryption Transforms and Supported Authentication Transforms properties declare a value for the NULL cipher and the NULL authentication algorithm respectively. By including these values, a MG explicitly indicates that its policy allows the use of unencrypted and/or unauthenticated SRTP and SRTCP packets. Note that, in accordance with [IETF RFC 3711], SRTCP packets must be authenticated using a non-NULL algorithm, regardless of the declared support of the NULL authentication algorithm.

To determine the cryptographic capability for the "DTLS-SRTP" key management scheme, the MGC may audit the DTLS-SRTP protection profiles (dtlscn/dspp) property from the dtlscn package in [ITU-T H.248.90].

### 6.6.2	Key management

The SRTP package does not define protocol elements for performing SRTP key management. Instead, the Key Management Scheme (km) property allows the MGC to indicate the use of one of several, already established, key management schemes.

The two key management schemes supported by version 2 of the package is the use of SDP security descriptions (see [IETF RFC 4568]) in the Local and Remote Descriptors and the use of DTLS-SRTP. Further details about the adaptation of these security descriptions for use in ITU-T H.248 are provided in clause 7 and 8.

NOTE –Future versions of this package may allow the use of additional key management schemes, for example, SDP key management extensions (see [b-IETF RFC 4567]).

By default, the value of the Key Management Scheme property is "None"; indicating that no key management is used, and therefore SRTP is not employed. This prevents an MGC that is unaware of this package from inadvertently "turning on" SRTP through the careless inclusion of SDP parameters in the Local and Remote Descriptors.

### 6.6.3	Master key lifetime

SRTP master keys have a limited lifetime, measured in the number of SRTP and SRTCP packets that may be protected using the same key. The Master Key Expiry (mke) event allows the MGC to be notified when the master key is close to being, or has already been exhausted. The MG notifies this event based on the expiry status of the key used to encrypt and authenticate sent packets. No notification is generated regarding the expiry of the decrypting master key used for handling received packets.

The SRTP Watermark (rtpw) and SRTCP Watermark (rtcpw) event parameters allow the MGC to control how long before key exhaustion the mke event is first notified. If the value of the relevant watermark is different from 0, the MG shall generate the event when the master key has been used for (lifetime – rtpw) SRTP packets or (lifetime – rtcpw) SRTCP packets (whichever happens first). For example, if the key lifetime is 220, and rtpw and rtcpw are both equal to 216, the event will be notified after (220–216 = 983040) SRTP or SRTCP packets have been protected by that key.

Regardless of the value of the rtpw and rtcpw parameters, the Master Key Expiry event shall be notified by the MG when the master key has fully expired and can no longer be used. The MGC can differentiate whether the master key has already expired or only the SRTP/SRTCP watermark was crossed through the Key Expired parameter.

When several master keys can be used by the MG (only applicable if the Key Management Scheme property is set to "SDES"), the gateway shall generate the mke event only when all master keys are about to be, and/or have already been exhausted. For example, if three keys are used in series, each with a lifetime of X packets, the MG shall first send a notification only after the third (and last) key has been used for X-rtpw SRTP packets or X-rtcpw SRTCP packets. An additional notification will be sent when the third key has been fully exhausted.

The mke event may be configured on a specific stream or on the complete termination. Configuring the mke event on a termination is equivalent to configuring it, with the same parameter values, on each of these termination's streams (other than streams that already have the event explicitly enabled). As different streams may exhaust the key at different times, an mke notification shall always be associated with a specific Stream.

#### 6.6.3.1	MG behaviour at key exhaustion

The operations taken by the MG when the last encrypting master key is exhausted are controlled through the Key Lifetime Expiry Behaviour (kleb) property. Setting this property to "BYE" will cause the MG to send an SRTCP BYE packet, hence leaving the RTP session or closing it (if it is the session's sole sender) as soon as the master-key expires. If DTLS-SRTP has been used for the Key Management Scheme property, the MG shall enforce the closure of the DTLS connection as well by sending a fatal alert.

NOTE – The MG must ensure that it is able to send the SRTCP BYE packet using a valid master key. This means that when kleb is set to "BYE", the encrypting master key will expire when it can still protect one additional SRTCP packet.

If the property Key Management Scheme is set to "SDES" and if the MGC installs a new master key after the MG has sent an SRTCP BYE packet, the MG will rejoin the RTP session or create a new one (if it was closed). This, in turn, will have whatever effects such rejoining or creating a session entails. For example, the SRTP rollover counter (ROC) will be reset, and the MG may start using a new SSRC value.

{Editor’s note: Report an error if srtp/kleb = "RENEW-KEYS" and tlscn/srsc = "false"}

Regardless of the value of the kleb property, the MG shall neither receive nor send SRTP packets using an expired key (i.e., all such packets shall be discarded).

The MG's behaviour at key-exhaustion is completely independent of the expiry notification, and remains the same regardless of whether the Master Key Expiry event is configured at the stream level, termination level, or not at all.

### 6.6.4	Logging of security violations

Once an SRTP stream is established on a MG, sent and received packets are processed according to clause 3.3 of [IETF RFC 3711]. Received packets are authenticated and decrypted. During this process, received packets may be judged to have been replayed or may fail authentication and be discarded. In order to log these events, the MGC shall set the Number of Replayed Packets (srtp/replay) and the Number of Authentication Failures (strp/authfail) statistics. The MGC may audit these statistics when it wishes to know the number of replayed packets and/or authentication failures detected.  If the MGC requires notification of such events, it shall use the Statistic Conditional Reporting package (see [ITU-T H.248.47]) with an appropriate reporting threshold.

# 7	Key management using SDP security descriptions

The MGC indicates that SDP security descriptions will be used for key management by setting the value of the Key Management Scheme property to "SDES". Under this scheme, the MGC and MG negotiate a Stream's SRTP parameter by placing a "crypto" SDP attribute in the Local and Remote Descriptors. The "crypto" attribute and its use for negotiating SRTP parameters is described in [IETF RFC 4568]. This clause provides additional details regarding the adaptation of those procedures for use with ITU-T H.248.

Naturally, this scheme is only applicable when SDP is used for the Local and Remote Descriptors. If the binary encoding of the protocol is used, the "crypto" SDP attribute can be carried using the SDP equivalents of clause C.11 of [ITU-T H.248.1]. The MG shall use error code 473 (Conflicting Property Values) when the Local and Remote Descriptors cannot carry a "crypto" SDP attribute and the Key Management Scheme is set to "SDES".

The Local Descriptor controls the SRTP parameters of the flow(s) sent by the MG. Similarly, the Remote Descriptor controls the SRTP parameters of the flow(s) received by the MG. A Local or Remote Descriptor indicates that the MG shall use SRTP to protect sent or received packets if both:

1)	The media description ("m=" line) uses an SRTP-based profile as the transport protocol (e.g., "RTP/SAVP" or "RTP/SAVPF").

2)	The SDP contains one or more "crypto" attributes.

If either of these conditions is not met, the MG shall not apply SRTP procedures to the packets. This Recommendation does not imply any special meaning to descriptors that match only one or none of these conditions.

The MG shall use error code 474 (Invalid SDP Syntax) if the above procedures indicate that the MG shall protect flows using SRTP but the "crypto" attribute does not match the SRTP-specific format, as described in clause 6 of [IETF RFC 4568].

In the following clauses, words appearing in fixed-font are references to specific ABNF rules from clause 9 of [IETF RFC 4568].

## 7.1	Overspecification of SRTP parameters and multiple keys

There are two possible ways for the MGC to specify more than one set of SRTP parameters within one SDP group:

1)	Overspecify the "crypto" attribute by including more than one such attribute in the SDP group.

2)	Include more than one key-param in one "crypto" attribute.

These two methods can be combined (i.e., include several "crypto" attributes in the SDP group, each including more than one key-param).

According to clause 7.1.5 of [ITU-T H.248.1], the behaviour of the MG when the "crypto" attribute is overspecified depends on the value of the ReserveValue property. If this value is false, the MG shall choose only one of the included "crypto" attributes and remove all others from the SDP group. Conversely, if ReserveValue is true, the MG shall reserve enough resources to support as many of the included "crypto" attributes as it can, and keep all those supported attributes in the descriptor.

A Remote Descriptor with more than one "crypto" attribute and/or more than one key-param within a "crypto" attribute indicates that the MG shall be prepared to accept packets protected using any of the master keys contained in the Descriptor. To achieve this, each key shall include a master key identifier (MKI) value and that value shall be unique. Any command resulting in one MKI value being mapped to more than one master key shall be rejected using error 473 (Conflicting Property Values).

A Local Descriptor with more than one "crypto" attribute indicates that the MG has reserved resources for all these attributes; however, only the first "crypto" attribute is used by the MG for protecting sent packets. As the MG does not use any of the other "crypto" attributes, different attributes may include identical MKI values (or not include MKI at all). Such configurations are often transient and exist while the session is being set up. An example for such a scenario is provided in item 1 of clause I.1.

NOTE 1 – Mandating the use of the first "crypto" attribute in the Local Descriptor allows re-keying an existing session. The MGC would:

a)	Overspecify the Local Descriptor of the sender, adding a second, new "crypto" attribute.

b)	Overspecify the Remote Descriptor of the receiver, adding the new "crypto" attribute.

c)	Remove the first "crypto" attribute from the sender's Local Descriptor, leaving only the new attribute there.

A "crypto" attribute with more than one key-param appearing first in the Local Descriptor indicates that the different key-param sub-fields shall be used sequentially. The MG shall use the first key-param whose master key has not yet expired for protecting sent packets. Once a master key expires (due to the number of either SRTP or SRTCP packets sent), the MG shall start using the next key-param in the attribute. As, over the course time, all key-param sub-fields might be used, each shall include a MKI value and that value shall be unique.

NOTE 2 – The above procedures allow for the "automatic" re-keying of a stream upon key exhaustion, without the need for additional signalling messages.

## 7.2	Wildcarding of SRTP parameters

In addition to overspecification, many sub-fields of the "crypto" attribute may be wildcarded using the CHOOSE ("$") wildcard. When a sub-field is wildcarded, the MG shall choose a value for it based on the MG capabilities and local configuration. The exact procedures for doing so are outside the scope of this Recommendation.

Table 1 summarizes the guidelines for the sub-fields that may be wildcarded. Sub-fields that do not appear in the table cannot be wildcarded.

Table 1 – Wildcarding of SDP security descriptions

| Sub-Field |  | Guidelines |
| --- | --- | --- |
| crypto-suite |  | Wildcarding this sub-field mandates that key-salt is also wildcarded, as the MGC cannot know in advance the required key length. |
| key-info |  | Each part of key-info is wildcarded separately |
|  | key-salt | Can be wildcarded. It is impossible to wildcard only the key or the salt. |
|  | lifetime | Can be wildcarded. |
|  | mki | Only mki-value can be wildcarded (i.e., mki-length cannot). The MG shall choose the mki-value so that it is different from any other MKI appearing in the descriptor. |
| kdr |  | Can be wildcarded, using the form "KDR=$" |
| fec-order |  | Can be wildcarded, using the form "FEC_ORDER=$" |
| fec-key |  | The key-params part of the sub-field can be wildcarded, using the procedures for key-info above. |
| wsh |  | Can be wildcarded, using the form "WSH=$" |

The MGC may combine overspecification and wildcarding, i.e., include in a descriptor multiple "crypto" attributes, where some of the attribute's subfields contain the CHOOSE wildcard.

## 7.3	Interoperability with offer/answer-based implementations

Under the SDP offer/answer procedures of [IETF RFC 4568], some of the SRTP parameters are considered "negotiated", meaning that the same parameter value must be used for both the sent and received RTP packets. The list of these parameters is:

1)	crypto-suite

2)	UNENCRYPTED_SRTCP

3)	UNENCRYPTED_SRTP

4)	UNAUTHENTICATED_SRTP

To increase interoperability with such offer/answer based implementations, whenever the MG needs to choose a value for one of those parameters (i.e., when overspecification or wildcarding is employed), it shall ensure that the same value is used in both the Local and Remote Descriptors. Using different values in the Local and Remote Descriptors for a "negotiated" parameter is only allowed when the request sent by the MGC explicitly prevents the use of the same value.

## 7.4	SDES and SRTP cryptographic contexts

With regard to the initialization and maintenance of SRTP crypto contexts, the MGC and MG shall follow the procedures of clauses 6.4 and 6.5 of [IETF RFC 4568]. In addition, the MGC and MG shall follow the procedures of clause 7 of [IETF RFC 4568], adapted to ITU-T H.248's use of SDP (which is different from the offer/answer model covered by that document). The following list highlights the points of those clauses that have the most significant impact on the MGC's and MG's behaviour.

1)	The ROC of any newly created crypto context shall be initialized to zero.

2)	The MG should choose an initial sequence number in the range of 0..215–1 for any RTP stream associated with a newly created SRTP crypto context.

3)	The MG shall choose different SSRC values for different RTP streams sharing the same master key.

4)	The MG shall remove crypto-contexts using the same procedures as for SSRC removal from the member table, as described in [IETF RFC 3550].

5)	If the MGC has wildcarded a master key, the MG shall choose a master key different from all other master keys it is currently using. In particular, a master key chosen for the Local or Remote Descriptor shall be different from any other master key appearing in the Local or Remote Descriptor of the same stream.

6)	A command that changes the first "crypto" SDP attribute in the Local or Remote Descriptor shall create a new SRTP crypto context, which will be used by the MG for sending or receiving packets respectively. In particular, such a command shall reset the relevant ROC counter (Note 1).

A change of a master key that does not involve a new "crypto" attribute (e.g., when multiple key-param sub-fields exist) shall not cause a new crypto context to be created, and the existing context shall be used (Note 2).

NOTE 1 – A change of the first "crypto" attribute is considered as equivalent to sending a new master-key in a SDP offer/answer procedure. Therefore, the MG shall follow the requirements of clause 7.1.4 of [IETF RFC 4568]:

"… the offerer MUST include a new master key with the offer (and in so doing, it will be creating a new crypto context where the ROC is set to zero)."

NOTE 2 – A change of the master-key that does not involve a new "crypto" attribute is equivalent to re-keying the SRTP session without using an offer/answer exchange. Therefore the MG shall follow the requirements of clause 3.3.1 of [IETF RFC 3711]:

"After a re-keying occurs (changing to a new master key), the rollover counter always maintains its sequence of values, i.e., it MUST NOT be reset to zero."

7)	The MGC should apply a new "crypto" SDP attribute to the Local Descriptor (and hence create a new local crypto context) whenever it changes the address or port used in that Descriptor.

## 7.5	Mapping of master keys for sent packets and received packets statistics

When SDP security descriptions are used for key management, each entry in the Sent SRTP Packets Protected by Master Key (srpk) and Sent SRTCP Packets Protected by Master Key (scpk) shall correspond to one of the master keys appearing in the first "crypto" SDP attribute of the Local Descriptor. The order of entries in the statistics shall match the order of keys in the "crypto" attribute.

In a similar manner, each entry in the Received SRTP Packets Protected by Master Key (rrpk) and Received SRTCP Packets Protected by Master Key (rcpk) statistics shall correspond to one of the master keys appearing in the first "crypto" attribute of the Remote Descriptor.

Changing the first "crypto" attribute of the Local or Remote Descriptors will cause the MG to discard the appropriate statistics values, and to start maintaining new ones.

# 8	Key management using DTLS-SRTP

{Editor's note (2015-06): analyis of DTLS-SRTP applicability (see also discussion/framework in AVD-4744), - results & comments:

Item#1: The procedures handling the SDP "a=fingerprint" attribute should added. May be a reference to H.248.90 (TLS), clause 13.2 can be made.
Status: to be fixed.

}

{Editor’s note: The interworking between SDES and DTLS-SRTP may also need to be considered.}

The DTLS-SRTP [IETF RFC 5764] key management scheme exchanges the peers' certificates via the signalling path and then establishes a DTLS connection on the bearer path. The DTLS master secret is used to derive the SRTP master key. SRTP packets are then exchanged between the endpoints on the same transport protocol connection as used for the DTLS connection.

The packages from [ITU-T H.248.90] (TLS) and [ITU-T H.248.93] (DTLS) are used for the establishment of the DTLS connection.

## 8.1	Key management indication

The MGC indicates the use of DTLS-SRTP key management by setting the Key Management Scheme (srtp/km) property to the value "DTLS-SRTP".

Using this scheme a DTLS-SRTP connection is setup which is controlled by two sets of properties, one set is related to DTLS and the other is related to SRTP. Both sets of properties are orthogonal.

The DTLS related properties and procedures are covered by [ITU-T H.248.90] (TLS) and by [ITU-T H.248.93] (DTLS).

This clause provides additional details which are specific for the DTLS-SRTP key management scheme.

{Editor's note: m-line in RD and LD?}

## 8.2	Bearer plane connection for "DTLS-SRTP"

The MG will establish the DTLS-SRTP connection based on the following properties:

- TLS Domain Profile Identifier (H.248.90: tlscn/dpid)
This property identifies the (D)TLS profile to be used to setup the DTLS connection.

- TLS Versions (H.248.90: tlscn/tlsv)
This property defines the set of DTLS protocol versions allowed establish the DTLS connection.

- Chipher Suites (H.248.90: tlscn/cs)
This property defines the set of ciphersuites used to negotiate a DTLS session.

- Compression Methods (H.248.90: tlscn/cm)
This property defines the set of compression methods to be used for the negotiation with the remote DTLS endpoint. 
NOTE: DTLS-SRTP [IETF RFC 5764] itself does not consider any application data to be protected and therefore potentially compressed by the DTLS stack. Nevertheless there might be use cases which will use DTLS-SRTP for the SRTP key management scheme and additionally for the protection of application data (e.g. WebRTC data channel).

- Client Authentication Required (H.248.90: tlscn/car)
This property indicates that – if the MG is acting as a DTLS-server – authentication of the client is requested.

- DTLS-SRTP protection profiles (H.248.93: dtlcns/dspp)
This property indicates the set of SRTP cypto suites that will be used for the negotiation with the peer endpoint.

- Master Key Identifier usage (H.248.93:dtlcns/mkiu)
This property indicates the use or not use of the DTLS-SRTP specific Master Key Identifier (MKI) (according to [IETF RFC 5764], clause 4.1.3 "srtp_mki value").

Prior the establishment of the DTLS connection the SDP "a=fingerprint:" attribute (according to [IETF RFC 4572]) will be used to ensure the integrity of the self-signed certificates of both DTLS connection endpoints. The detailed procedures on how the fingerprints are exchanged between the MGC and the MG are covered in [ITU-T H.248.90] (TLS).

Based on the events of the TLS basic session control package (tlsbsc) [ITU-T H.248.90] (TLS) the MG will start the setup of a DTLS connection.

If the MG takes the DTLS client role, a DTLS handshake is initiated and the extension "use_srtp" [IETF RFC 5764] is included into the "ClientHello" message. This extension will be used according to the value of the properties DTLS-SRTP protection profiles (dtlcns/dspp) and Master Key Identifier usage (dtlcns/mkiu).

If the MG takes the DTLS server role the SRTP crypto suite is negotiated according to the received "use_srtp" extension from the "ClientHello" and the MG's property DTLS-SRTP protection profiles (dtlcns/dspp). In case the received "ClientHello" does not include the "use_srtp" extension the DTLS connection is terminated and an error is reported to the MGC.

{Editor’s note: Which H.248 error?}

The handshake to negotiate a DTLS session follows the procedures of [IETF RFC 5246]. Once a DTLS session is negotiated and the DTLS master secret is determined the SRTP master keys are derived according to clause 4.2 / [IETF RFC 5764]. From this point in time SRTP/SRTCP packets may be exchanged between the MG and its peer connection endpoint.

### 8.2.1	Protocol stack and DTLS usage

{Editor’s note: TBD}

### 8.2.2	Reuse of DTLS connection by other applications

DTLS-SRTP as specified by [IETF RFC 5764] does not consider the protection of DTLS application data. The DTLS connection is used as a key exchange mechanism for SRTP and once the SRTP master key is determined the video and/or audio flow is protected by SRTP.

However there may be the requirement to send different audio, video or data streams secured over the same transport connection (e.g. SRTP and WebRTC Data Channel using the same DTLS connection). In such a case the data stream will correspond to the DTLS application and the data stream packets will be secured by the DTLS record layer.

### 8.2.3	Lifetime of DTLS session and DTLS connection

For a DTLS session used to derive the SRTP master key no special rule applies for its lifetime. Thus the session used might be a transient DTLS session or a semi-permanent DTLS session (i.e. resumable).

The lifetime of the DTLS connection is coupled as follows to the lifetime of the (SRTP)-connection: If an event occurs which leads to the termination of the DTLS connection then the related (SRTP) and (SRTCP) connections must be terminated as well (NOTE 2).

NOTE 1 – An SRTCP-BYE is not sent in this case. The rationale behind this is that any usage of the DTLS master secret and respectively the derived SRTP master key is not allowed anymore.

NOTE 2 – See [IETF RFC 5246] (TLS version 1.2), clause 7.2.2 on error alerts: "… Servers and clients MUST forget any session-identifiers, keys, and secrets associated with a failed connection.    …". The SRTP master key represents a "secret associated with a failed connection", hence the conclusion that the DTLS fatal alert will lead to a termination of the SRTP connection.

### 8.2.4	Lifetime of DTLS master secret and SRTP master key

In case the DTLS connection is reused by another application (e.g. WebRTC Data Channel) application data records as well as SRTP packets might be sent in parallel using the same transport address. This potentially may lead either to the expiration of the DTLS master secret (the connection state sequence number may not exceed 264-1) or to the expiration of the SRTP master key (at maximum 248 SRTP or 231 SRTCP packets may be sent using the same master key).

In case the DTLS master key expires...

{Editor's note: H.248.90 does not define an MG-autonomous renegotiation in case a watermark (=number of packets) is reached. Only a renegotiation period is defined, which might not be appropriate). Needs further consideration.}

In case the values as defined by the SRTP Watermark (srtp/rtpw) property or by the SRTCP Watermark (srtp/rtcpw) property are crossed the MG's behavior depends on the Key Lifetime Expiry Behaviour (srtp/kleb) property:

- srtp/kleb = "DROP": All SRTP/SRTCP packets are dropped. DTLS application data sent or received over the DTLS record layer are not impacted.

- srtp/kleb = "BYE": Close the (SRTP)-connection and send an SRTCP-BYE. DTLS application data sent or received over the DTLS record layer are not impacted. E.g. for WebRTC this allows to close the audio/video session but to continue the data transfer.

- srtp/kleb = "RENEW-KEYS": This will trigger a renegotiation. A DTLS handshake will be initiated to determine a new DTLS master secret and derive a new SRTP master key from it. During the renegotiation SRTP/SRTCP packets may still be sent (if the watermarks are greater than zero) as well as DTLS application data using the old keying material. Once the handshake is completed (Finished message sent and received) the new keying material will be used for SRTP/SRTCP packets as well as for DTLS application data.
NOTE – Future (D)TLS versions might drop the support for renegotiation but may provide another procedure for renewing the (D)TLS master secret.

## 8.3	Multi key management operation

{Editor’s note: TBD}

# 9	Security considerations

{Editor's note (2015-06): analysis of DTLS-SRTP applicability (see also discussion/framework in AVD-4744), - results & comments:

Item#1: It's worth to provide a separate security consideration for DTLS-SRTP, e.g. integrity protection of the signalled SDP a=fingerprint attribute.
Status: to be fixed.

}

## 9.1	Relation to key management scheme "SDES"

SDP security descriptions do not provide any inherent authentication or encryption of the SRTP parameters carried in the Local and Remote Descriptors. Therefore, use of this key-management scheme is only appropriate when the ITU-T H.248 channel is secured through some other means (e.g., IPSec).

## 9.2	Relation to key management scheme "DTLS-SRTP"

For the "DTLS-SRTP" key management scheme the security considerations as described by the related IETF RFCs [IETF RFC 5246], [IETF RFC 5764], [IETF RFC 6347] apply. An additional aspect for the establishment of a DTLS connection is the integrity protection of the self-signed certificates. Fingerprint hashes are calculated for the certificates and are communicated via out-of-band means (SDP "a=fingerprint" attribute). This attribute must be protected against tampering, thus the communication channel used to exchange the fingerprints must be integrity protected.

Appendix I

Example call flows for key management scheme “SDES”

(This appendix does not form an integral part of this Recommendation)

## I.1	Initial session setup using SDP security descriptions

In the following examples, tokens such as <key1> and <key2> indicate sequences of 240 bits, encoded as 40 base64 characters.

1)	The MGC ADDs a new, SRTP-enabled, termination to MG1.

The Key Management Scheme is set to SDES, the transport protocol is RTP/SAVP and a "crypto" attribute appear in the Local Descriptor, indicating that the packets sent by MG1 should be protected using SRTP.

The Local Descriptor contains two "crypto" attributes and ReserveValue is true, meaning that MG1 should reserve resources for both, but only use the first. Note that the MKI value "1" is shared between the two attributes, which is allowed.

MGC to MG1:

MEGACO/3 [123.123.123.4]:55555

Transaction = 10003 {

Context = $ {

Add = $ {

Media {

TerminationState {

srtp/km = SDES

},

Stream = 1 {

LocalControl {

Mode = RecvOnly,

ReservedValue = ON

},

Local {

v=0

c=IN IP4 $

m=audio $ RTP/SAVP 4

a=ptime:30

a=crypto:1 AES_CM_128_HMAC_SHA1_80 inline:$|2^20|1:4;inline:$|2^20|2:4

a=crypto:2 F8_128_HMAC_SHA1_80 inline:$|$|1:4

}

}

}

}

}

}

2)	MG1 returns the key-salt sub-fields it has chosen as well as the lifetime of the AES_F8 key (these sub-fields were wildcarded in the request). The <key1>, <key2>, <key3> values must all be different from one another.

MG1 to MGC

MEGACO/3 [124.124.124.222]:55555

Reply = 10003 {

Context = 2000 {

Add = A4445 {

Media {

Stream = 1 {

Local {

v=0

o=- 2890844526 2890842807 IN IP4 124.124.124.222

s=-

t=0 0

c=IN IP4 124.124.124.222

m=audio 2222 RTP/SAVP 4

a=ptime:30

a=crypto:1 AES_CM_128_HMAC_SHA1_80 inline:<key1>|2^20|1:4;inline:<key2>|2^20|2:4

a=crypto:2 F8_128_HMAC_SHA1_80 inline:<key3>|2^30|1:4

}

}

}

}

}

}

3)	The MGC ADDs a new, SRTP-enabled termination to MG2.

The Remote Descriptor includes the crypto attributes returned by MG1. ReserveValue is false, indicating that MG2 should choose the first "crypto" SDP attribute it supports.

The crypto-suite, key-salt, lifetime and mki-value of the Local Descriptor are all wildcarded.

The Master Key Expiry event is enabled on the complete termination, indicating that the MGC should be first notified of an imminent master key exhaustion when the encryption key can only protect 10'000 additional SRTP packets or 50 additional SRTCP packets. Another notification will be made when the key is completely exhausted.

MGC to MG2:

MEGACO/3 [123.123.123.4]:55555

Transaction = 50003 {

Context = $ {

Add = $ {

Media {

TerminationState {

srtp/km = SDES

},

Stream = 1 {

LocalControl {

Mode = SendRecv

; ReserveValue is false by default

},

Local {

v=0

c=IN IP4 $

m=audio $ RTP/SAVP 4

a=ptime:30

a=crypto:1 $ inline:$|$|$:4

},

Remote {

v=0

c=IN IP4 124.124.124.222

m=audio 2222 RTP/SAVP 4

a=ptime:30

a=crypto:1 AES_CM_128_HMAC_SHA1_80 inline:<key1>|2^20|1:4;inline:<key2>|2^20|2:4

a=crypto:2 F8_128_HMAC_SHA1_80 inline<key3>|2^30|1:4

}

}

},

Events = 1234 {

srtp/mke { rtpw=10000, rtcpw=50 }

}

}

}

}

4)	MG2 chooses the first "crypto" attribute in the Remote Descriptor. In accordance with clause 7.3, it chooses for the Local Descriptor the same crypto-suite as the one now used in the Remote Descriptor.

MG2 to MGC:

MEGACO/3 [125.125.125.111]:55555

Reply = 50003 {

Context = 5000 {

Add = A5556{

Media {

Stream = 1 {

Local {

v=0

o=- 7736844526 7736842807 IN IP4 125.125.125.111

s=-

t=0 0

c=IN IP4 125.125.125.111

m=audio 1111 RTP/SAVP 4

a=ptime:30

a=crypto:1 AES_CM_128_HMAC_SHA1_80 inline:<key4>|2^20|1:4

},

Remote {

v=0

o=- 7736849782 7736858112 IN IP4 125.125.125.111

s=-

t=0 0

c=IN IP4 124.124.124.222

m=audio 2222 RTP/SAVP 4

a=ptime:30

a=crypto:1 AES_CM_128_HMAC_SHA1_80 inline:<key1>|2^20|1:4;inline:<key2>|2^20|2:4

}

}

}

}

}

}

5)	The MGC updates MG1's Local and Remote Descriptors according to the choices made by MG2. The Master Key Expiry event is configured on the termination, using the same parameter values as the ones used at 3).

MGC to MG1:

MEGACO/3 [123.123.123.4]:55555

Transaction = 60006 {

Context = 2000 {

Modify = A4445 {

Media {

Stream = 1 {

LocalControl {

Mode = SendRecv

},

Local {

v=0

c=IN IP4 124.124.124.222

m=audio 2222 RTP/SAVP 4

a=ptime:30

a=crypto:1 AES_CM_128_HMAC_SHA1_80 inline:<key1>|2^20|1:4;inline:<key2>|2^20|2:4

},

Remote {

v=0

c=IN IP4 125.125.125.111

m=audio 1111 RTP/SAVP 4

a=ptime:30

a=crypto:1 AES_CM_128_HMAC_SHA1_80 inline:<key4>|2^20|1:4

},

}

},

Events = 5678 {

srtp/mke { rtpw=10000, rtcpw=50 }

}

}

}

}

6)	MG1 acknowledges the MODIFY request.

MG1 to MGC

MEGACO/3 [124.124.124.222]:55555

Reply = 60006 {

Context = 2000 {

Modify = A4445

}

}

## I.2	MG1's key is about to expire

1)	MG1 sends a Master Key Expiry NOTIFY request to the MGC. The Key Expired parameter is missing and its default value (false) is used, indicating that the key has not yet expired, but that either the SRTP or SRTCP watermarks has been crossed.

MG1 to MGC:

MEGACO/3 [124.124.124.222]:55555

Transaction = 76819 {

Context = 2000 {

Notify = A4445 {

ObservedEvents = 5678 {

20091201T07450122:srtp/mke ;ke is false by default

}

}

}

}

2)	The MGC acknowledges the NOTIFY request.

MGC to MG1:

MEGACO/3 [123.123.123.4]:55555

Transaction = 76819 {

Context = 2000 {

Notify = A4445

}

}

## I.3	Auditing SRTP capabilities

1)	The MGC audits all SRTP properties on MG1's Root termination.

MGC to MG1

MEGACO/3 [123.123.123.4]:55555

Transaction = 87395 {

Context = – {

AuditValue = Root {

Audit { TerminationState { srtp/* } }

}

}

}

2)	MG1 answers with the lists of encryption and authentication transforms that it supports. These lists are missing the NULL value, meaning that MG1's security policy does not allow the use of unencrypted or unauthenticated SRTP packets.

MG1 to MGC

MEGACO/3 [124.124.124.222]:55555

Reply = 87395 {

Context = – {

AuditValue = Root {

TerminationState { srtp/set=[AES_CM_128, AES_CM_192, AES_CM_256],

srtp/sat=[HMAC_SHA1_32, HMAC_SHA1_80]

}

}

}

}

## I.4	Auditing of SRTP Statistics

1)	The MGC audits the SRTP statistics of stream 1 on MG1:

MGC to MG1:

MEGACO/3 [123.123.123.4]:2944

Transaction = 91903 {

Context = 2000 {

AuditValue = A4445 {

Audit {

Media {

Stream = 1 {

Statistics { srtp/* }

}

}

}

}

}

}

2)	MG1 returns the current SRTP statistics. According to the values returned, MG1 has discarded 7 packets due to authentication failure and considered 3 packets as replays. In addition (assuming that MG1 is using the keys negotiated in item 1 of clause I.1) the MG has protected 220 SRTP packets and 4'086 SRTCP packets using <key1>, and 37'112 SRTP packets and 941 SRTCP packets using <key2>. It received 519'733 SRTP packets and 2080 SRTCP packets protected by <key4>.

MG1 to MGC:

MEGACO/3 [124.124.124.222]:55555

Reply = 91903 {

Context = 2000 {

AuditValue = A4445 {

Media {

Stream = 1 {

Statistics {

srtp/replay = 3,

srtp/authfail = 7,

srtp/srpk = [1048576, 37112],

srtp/scpk = [4086, 941],

srtp/rrpk = 519733,

srtp/rcpk = 2080

}

}

}

}

}

}

Appendix II

Sample use-cases of SRTP bearer encryption

(This appendix does not form an integral part of this Recommendation)

This appendix illustrates some network level scenarios that employ SRTP bearer encryption.

## II.1	Use-case #1: ITU-T H.248 MG for peering IP and circuit-switched networks

Figure II.1 illustrates an ITU-T H.248 connection model of (IP, physical). This model is often employed for peering a circuit-switched and an IP network at a residential, access or trunking MG. The RTP session is terminated by the ITU-T H.248 MG. The MG is consequently behaving as a RTP end system (see clause 3 of [IETF RFC 3550]).

Any application of SRTP as a means for media security implies the termination of the SRTP session by the corresponding ITU-T H.248 stream.

Figure II.1 – Use case #1: SRTP to circuit-switched ITU-T H.248 MG

## II.2	Use case #2: ITU-T H.248 MG for peering IP networks

ITU-T H.248 IP-IP MGs are widely used as, e.g., border routers, border gateways, policy enforcement points, firewalls with session-dependent filter rules, NAT devices, media transcoders, etc.

Figure II.2 outlines a scenario, where such a gateway is located between two IP domains: one domain without any media security and another domain using SRTP encrypted media. The ITU-T H.248 MG behaves as two, back-to-back RTP end systems due to the termination of SRTP in one ITU-T H.248 stream.

Figure II.2 – Use case #2: SRTP to RTP ITU-T H.248 MG

## II.3	Use case #3: Transparent SRTP forwarding

It is possible for an MG to transparently forward SRTP packets, treating them as unprotected UDP or RTP packets. Such a scenario is illustrated in Figure II.3.

As stated in clause 1, this use-case is outside the scope of this Recommendation. It is presented here for the sake of completeness.

Figure II.3 – Use case #3: ITU-T H.248 MG with transparent SRTP forwarding

Appendix III

Example call flows for key management scheme "DTLS-SRTP"

(This appendix does not form an integral part of this Recommendation)

## III.1	Initial session setup using DTLS-SRTP based key exchange

{Editor’s note: TBD}

Bibliography

[b-ITU-T H.248.95]	Recommendation ITU-T H.248.95 (11/2015), Gateway control protocol: H.248 support for RTP multiplexing.

[b-IETF RFC 3264]	IETF RFC 3264 (2002), An Offer/Answer Model with Session Description Protocol (SDP).

[b-IETF RFC 4567]	IETF RFC 4567 (2006), Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP).

[b-IETF RFC 5479]	IETF RFC 5479 (04/2009), Requirements and Analysis of Media Security Management Protocols.

[b-IETF RFC 5763]	IETF RFC 5763 (05/2010), Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS).

[b-IETF RFC 7202]	IETF RFC 7202 (04/2014), Securing the RTP Framework: Why RTP Does Not Mandate a Single Media Security Solution.

[b-IETF RFC 7656]	IETF RFC 7656(11/2015), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

[b-IETF tls-terms]	IETF draft-guballa-tls-terminology-03 (2015), Terminology related to TLS and DTLS.

__________________

____________________