| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4784 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | H.VSMprot: Proposal to add real-time video/audio acquisition to clause 6.1 and recording and playback to clause 6.2 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposed to add real-time video/audio acquisition to clause 6.1 and recording and playback to clause 6.2 to draft H.VSMprot: "Signalling and Protocols for mobile visual surveillance".

# Introduction

The Recommendation H.VSMprot focuses on the Signalling and Protocols of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

# Discussion

In order to push forward the progress of the work item: H.VSMprot “Signalling and Protocols for mobile visual surveillance”, The clause 6.1 and 6.2 are about real-time video/audio acquisition and recording and playback.

# Conclusion

It is proposed to add clause 6.1 and clause 6.2 to draft H.VSMprot: "Signalling and Protocols for mobile visual surveillance".

Attachment
Proposal text

[Begin proposed text]

Signalling and protocols for mobile visual surveillance

# 6	Signalling and protocols

The following is some illustrative items:

- The SIP message format should comply with the definition in RFC3261;

- The RTSP message format should comply with the definition in RFC2326;

- The HTTP message format should comply with HTTP1.1 defined in RFC2616;

- RTP/RTCP signaling and message refers to RFC3550.

## 6.1	Real-time video/audio acquisition

The Real-time Video/Audio Acquisition is used to view the real-time surveillance video/audio data by mobile terminals. After MSP performs authentication and returns the VAU information, the M_CU chooses and requests to the VAU for real-time video/audio data.

Figure 6-1: High level procedural flows for real-time media acquisition

- The M_CU sends  RTSP DESCRIBE (PUID-ChannelNo)  to the VAU.

- The VAU returns response message to the M_CU according to parameter configuration.

- The M_CU sends RTSP SETUP request to the VAU.

- The VAU distributes a channel and returns RTSP 200OK response message to the M_CU.

- The M_CU sends RTSP PLAY request to the VAU for real-time video surveillance.

- The VAU chooses the CMU according to PUID.

- The VAU starts session request to the CMU.

- The CMU returns session response to the VAU. If the VAU provides video format message body, the CMU chooses codec that the PU supports and returns the message body to the VAU. If the VAU doesn’t offer message body info, the CMU returns the message body carrying all capacities the PU supports to the VAU.

- The VAU judges the media data format and decides whether to start the transcoding process. The VAU starts the connection between PUID-ChannelNo and CallID , and creates a RTSP Session NO.

- The VAU sends session control response message to the M_CU.

## 6.2	Recording and playback

The playback function is used to play audio and video which has been previously recorded and stored.

### 6.2.1	Playback in platform

When users want to playback the surveillance video using mobile phones, they must log into the

MSP by the M_CU. The MSP performs authentication and returned the playback surveillance list.

After the users choose one surveillance point name, the M_CU sends a request for this surveillance

video to the VAU, and then VAU returns the media stream to the M_CU to play.

Figure 6-2-1Video Playback from the platform procedural flows

- The M_CU sends  http video search request(PUID-ChannelNo) to the MSP.

- The MSP sends http video search request(PUID-ChannelNo) to the CMU.

- The CMU returns videos list info to the MSP.

- The MSP returns the videos list info to the M_CU. The videos gets the unique Record-No in the CMU.

- The M_CU sends RTSP DESCRIBE (PUID-ChannelNo,Record-No)  to the VAU.

- The VAU returns response message to the M_CU according to parameter configuration.

- The M_CU sends RTSP SETUP request to the VAU.

- The VAU distributes a channel and returns RTSP 200OK response message to the M_CU.

- The M_CU sends RTSP PLAY request to the VAU for videos record (PUID-ChannelNo, Record-No).

- The VAU chooses the CMU according to PUID and Record-No.

- The VAU starts session request to the CMU. The request message carries message body including video format the VAU supports.

- The CMU returns session response to the VAU. If the VAU provides video format message body, the CMU chooses codec the PU supports and returns the message body to the VAU. If the VAU doesn’t offer message body info, the CMU returns the message body carrying all capacities the PU supports to the VAU.

- The VAU decides whether to start the transcoding process according to the media data format. The VAU starts the connection between PUID-ChannelNo ,RecordNo and CallID, and creates a RTSP Session NO.

- The VAU sends session response message to the M_CU .

- The M_CU sends RTSP  TEARDOWN to the VAU in order to remove the real-time session.

- The VAU sends the end request to the CMU to stop session.

- The CMU sends the end response to the VAU to stop session.

18. The VAU sends the end response to the M_CU to stop session.

### 6.2.2	Playback in PU

Figure 6-2-2 Video Playback from the PU procedural flows

- The M_CU sends  http video search request(PUID-ChannelNo) to the MSP.

- The MSP sends http video search request(PUID-ChannelNo) to the CMU.

- The CMU sends SIP(INFO) request to the PU for videos list info(PUID-ChannelNo).

- The PU returns videos list info to the CMU.

- The CMU returns videos list info to the MSP.

- The MSP returns the videos list info to the M_CU. The videos get the unique Record-No in the CMU.

- The M_CU sends RTSP DESCRIBE (PUID-ChannelNo, Record-No) to the VAU.

- The VAU returns response message to the M_CU according to parameter configuration.

- The M_CU sends RTSP SETUP request to the VAU.

- The VAU distributes a channel and returns RTSP 200OK response message to the M_CU.

- The M_CU sends RTSP PLAY request to the VAU for videos record (PUID-ChannelNo,  Record-No).

- The VAU chooses the CMU according to PUID and  Record-No.

- The VAU starts session request to the PU. The request message carries message body including videos format the VAU supports.

- The PU returns session response to the VAU. If the VAU provides video format message body, the CMU chooses codec the PU supports and returns the message body to the VAU. If the VAU doesn’t offer message body info, the CMU returns the message body carrying all capacities the PU supports to the VAU.

- The VAU decides whether to start the transcoding process according to the media data format. The VAU starts the connection between PUID-ChannelNo, RecordNo and CallID, and creates a RTSP Session NO.

- The VAU sends session response message to the M_CU.

- The M_CU sends RTSP  TEARDOWN to the VAU in order to disconnect the real-time session.

- The VAU sends the disconnect request to the PU to stop session.

- The PU sends the disconnect response to the VAU to stop session.

20. The VAU sends the disconnect response to the M_CU to stop session.

_____________