| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4791 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.VCDN.1-NV: proposed content of loose coupled SDN-based network virtualization |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to provide context about the loose coupled SDN-based network virtualization for vCDN service.

# Introduction

In the last meeting, the group created a new draft Recommendation to describe the network virtualization in the vCDN service. As one approach, tight coupled SDN-based network virtualization in vCDN service was introduced. In that approach, it shows how the CDN function is integrated with a conceptual SDN functions. The major principle of tight coupled SDN-CDN is integrating CDN functions into SDN functions so that SDN device can reserve the network resource based on the request from CDN functions. This scenario is used for enhancing CDN Node with SDN-based equipments. In the other scenario, if the CDN functions can be separated from hardware/software, its logical functionality can be implemented in a general hardware/software platform. The benefit is CDN Node does not rely on the special firmware. It can be deployed flexibly in a general firmware as virtualized resource so that the delivery and distribution capability can be enhanced.

This contribution is proposed to introduce another approach as loose coupled SDN-based network virtualization for vCDN service

# Proposal

- Proposes to add the new clause “loose coupled SDN-based network virtualization” in H.VCDN.1-NV

# Start proposal

# 8.1.2	Loose coupled SDN-based network virtualization

The loose coupled network virtualization means the major functions of CDN, i.e. the application, distribution and delivery, can be migrated into vCDN and only a few functions are still remained in CDN functions, e.g. user request redirect. When vCDN receives the request redirected from CDN, it implements its own service logic and control the resource usage directly under vCDN software environment. In this case, the vCDN node is almost independent from the real CDN node. The benefit of adopting the loose coupled network virtualization is to improve the CDN functions deploy in a flexible way. CDN nodes can be created as a software module in a general hardware and operation system, where is running many other application program in parallel. It is easily managed by Operators and can reduce the hardware and software investment.

Figure 8-2 shows how CDN functions are deployed in the SDN functional architecture under the vCDN scope.

vCDN

SDN control layer

SDN application layer

CDN functions

CDN application

SDN Resource layer

Data transport and processing

Function migration

Media stream

Content Distribution Functions

Content Delivery Functions

Orchestrator

CDN Function (SDN application)

Content Cache/Storage Functions

Request Function

Operation Function

Control

Orchestrate

Loading balance

User terminal device

Service request

Request message

Resource request

Resource reservation

Orchestrate feedback

Media access

Content Distribution Functions

Figure 8-2: tight coupled SDN-based network Virtualization solutions

- SDN Application layer –CDN Function (SDN application)

CDN logical functions could be migrated into SDN application layer as a SDN application. To implement CDN service logical operations, CDN Function needs to implement at least 2 functional modules:

- Request Function

This functional block is used to add the integrated CDN service logical functions into SDN application layer. Moreover, it receives the CDN service request and then sends the request of resource requiring and service orchestrating to Orchestrate Function in Orchestrator in SDN control layer.

- Operation Function

The major CDN functions, e.g. CDN application, content distribution and delivery, content storage and load balance, can be migrated into that logical functional module, The Operation Function handles the logical procedure of CDN service. When the feedback information of resource reservation is received, the Operation Function implements the logical operation based on the content distribution or media accessing request.

- SDN control layer – Orchestrator functions

The Orchestrator provides control and management of the network resource, as well as coordination of request from SDN application layer (Referenced from [ITU-T Y.3300]). To process the request from SDN application layer, the Orchestrator is recommended to implement 2 functional modules:

- Orchestrate Function

The Orchestrate Function can be further composed of 2 functional blocks:

- Recourse coordination: It is used to receive the request of resource requiring and service orchestrating and to process the service orchestrating according to the SDN network information. Then it sends the request of required resource reservation to the Control Function.

- Orchestrate feedback: when the resource reservation is completed, it returns the feedback to CDN Function with the resource reservation information.

- Control Function

The Control Function is used to allocate the resource required by CDN service, according to the resource reservation request. After the resource allocation, the Control Function will determine the routing path for content distribution and delivery, combining with the condition of load balance and routing status in the network. It also creates the data link layer/network layer (L2/L3) forwarding tables based on the control policy of SDN controller layer and then sends those tables to Data transport and process Function. Moreover, it also needs to return the information of resource reservation to the Orchestrate Function.

- SDN Resource layer – data transport and processing functions

The data transport and processing function entity involves many network devices which support SDN transport protocols. They are used for storing data based on the allocated resource related to the CDN Function. Furthermore, it executes data transporting correspond with the logical operation in CDN Function, according to the L2/L3 forwarding tables determined by Control Function.

[TBD]

# End Proposal

__________________